3GPP TSG-RAN WG5 Meeting #95-e	R5-222610r2

Electronic Meeting, 9th – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2330 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of scheduling information for positioning system information blocks |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The scheduling information for positioning system information blocks is missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added scheduling information for positioning system information blocks |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The scheduling for positioning system information blocks will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.3.1.3, 4.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Added a description of the conditions for the existence of different positioning SI messages. R2: Updated the posSIB scheduling information |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

4.4.3.1.3	Scheduling of system information blocks

The scheduling configurations for combinations of system information blocks are defined in the following tables. There is no scheduling information for combination NR-1.

Table 4.4.3.1.3-1: Scheduling for combination NR-2

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |

Table 4.4.3.1.3-2: Scheduling for combination NR-3

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 64 | SIB3 |

Table 4.4.3.1.3-3: Scheduling for combination NR-4

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 64 | SIB4 |

Table 4.4.3.1.3-4: Scheduling for combination NR-5

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 64 | SIB3 |
| 3 | 64 | SIB4 |

Table 4.4.3.1.3-5: Scheduling for combination NR-6

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 64 | SIB5 |

Table 4.4.3.1.3-6: Scheduling for combination NR-7

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 64 | SIB4, SIB5 |

Table 4.4.3.1.3-7: Scheduling for combination NR-8

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB6 |
| 2 | 32 | SIB7 |

Table 4.4.3.1.3-8: Scheduling for combination NR-9

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB8 |

Table 4.4.3.1.3-9: Scheduling for combination NR-10

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
|  | 32 | SIB6 |

Table 4.4.3.1.3-10: Scheduling for combination NR-11

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB7 |

Table 4.4.3.1.3-11: Scheduling for combination NR-12

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 64 | SIB10 |

Table 4.4.3.1.3-12: Scheduling for combination NR-13

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 64 | SIB10 |

Table 4.4.3.1.3-13: Scheduling for combination NR-14

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 64 | SIB4 |
| 3 | 64 | SIB12 |

Table 4.4.3.1.3-14: Scheduling for combination NR-15

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | posSibType1-1, posSibType1-2, posSibType1-3 (Note 1) |
|  | 32 | posSibType4-1 (Note 2) |
|  | 32 | posSibType5-1 (Note 3) |
|  | 32 | posSibType6-1 (Note 4) |
| 2 | 32 | posSibType2-1, posSibType2-3, posSibType2-6, posSibType2-7, posSibType2-8, posSibType2-9 (Note 4) |
| Note 1: Content of SI message 1 when UE supports GNSS as defined in TS 37.571-2 [20] Table 8.1-1: Sub-test 25. Note 2: Content of SI message 1 when UE supports Sensor as defined in TS 37.571-2 [20] Table 8.1-1: Sub-test 24. Note 3: Content of SI message 1 when UE supports MBS as defined in TS 37.571-2 [20] Table 8.1-1: Sub-test 23. Note 4: Content of SI message 2, only present when UE supports GNSS as defined in TS 37.571-2 [20] Table 8.1-1: Sub-test 25. If UE supports N GNSS systems, SI message 2 needs to be broadcasted N times, once per each GNSS system. |  |  |

Table 4.4.3.1.3-15: Scheduling for combination NR-16

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 32 | posSibType6-1 |

Table 4.4.3.1.3-16: Scheduling for combination NR-17

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 64 | SIB4 |
| 3 | 32 | posSibType6-1 |

# < Unchanged sections omitted >

–	SystemInformation

Table 4.6.1-29: SystemInformation

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SystemInformation ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| systemInformation SEQUENCE { |  |  |  |  |
| sib-TypeAndInfo SEQUENCE (SIZE (1..maxSIB)) OF CHOICE |  | See subclause 4.4.3.1.3 |  |  |
| sibX |  | "X" denotes the SIB number |  |  |
| ... |  |  |  |  |
| sibY |  | "Y" denotes the SIB number |  |  |
| } |  |  |  |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| criticalExtensionsFuture-r16 CHOICE { |  |  |  | posSIB |
| posSystemInformation-r16 |  | PosSystemInformation-r16-IEs |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| posSIB | For test cases using posSIBs in system information. |

# < Unchanged sections omitted >

–	PosSystemInformation-r16-IEs

Table 4.6.2a-1: PosSystemInformation-r16-IEs

| Derivation Path: TS 38.331 [6], clause 6.3.1a |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PosSystemInformation-r16-IEs ::= SEQUENCE { |  |  |  |
| posSIB-TypeAndInfo-r16 SEQUENCE (SIZE (1..maxSIB)) | n entries | The number of entries depends on the sub-test, as specified in TS 37.571-2 [40] |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |

–	PosSI-SchedulingInfo

Table 4.6.2a-2: PosSI-SchedulingInfo

| Derivation Path: TS 38.331 [6], clause 6.3.1a |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PosSI-SchedulingInfo-r16 ::= SEQUENCE { |  |  |  |
| posSchedulingInfoList-r16 SEQUENCE (SIZE (1..maxSI-Message)) OF PosSchedulingInfo-r16 { | n entries | The size of PosSchedulingInfo-r16 is depending on the sub-tests, as specified in TS 37.571-2 [40] |  |
| PosSchedulingInfo-r16[n] SEQUENCE { |  | entry n |  |
| offsetToSI-Used-r16 | Not present |  |  |
| posSI-Periodicity-r16 | rf32 |  |  |
| posSI-BroadcastStatus-r16 | broadcasting |  |  |
| posSIB-MappingInfo-r16 SEQUENCE (SIZE (1..maxSIB)) OF PosSIB-Type-r16 { | n entries | The number of entries depends on the sub-test, as specified in TS 37.571-2 [40] |  |
| PosSIB-Type-r16[n] |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| posSI-RequestConfig-r16 | Not present |  |  |
| posSI-RequestConfigSUL-r16 | Not present |  |  |
| } |  |  |  |

–	SIBpos

Table 4.6.2a-3: SIBpos

| Derivation Path: TS 38.331 [6], clause 6.3.1a |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIBpos-r16 ::= SEQUENCE { |  |  |  |
| assistanceDataSIB-Element-r16 | OCTET STRING containing AssistanceDataSIBelement as spepcified in TS 37.571-2 [40] |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |