3GPP TSG-RAN5 Meeting #95-e	R5-222755r1

# Electronic Meeting, 09th – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0388 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR IMS TC 7.1-MO Voice Call with 503 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The description that steps 10-12 of generic procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 [21] are performed, is missing at step 14a1-14b8. But the corresponding behavior has been implemented in TTCN code. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add steps corresponding to steps 10-12 of generic procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description of test body is not aligned with the implemetation of TTCN code. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.508-1... CR 2380... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  | 1st revision: - Steps 14a1 and 14a2-14a5 could be combined into 14a1-14a5 - Revise steps 14a6-14a15 to make sure that UPDATE and UL Information Transfer can happen in parallel. - Correct typos - Update the steps in parallel to align with CR 2380. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

7.1	MTSI MO Voice Call / 503 Service Unavailable / 5GS

7.1.1	Test Purpose (TP)

(1)

with { UE being registered to IMS and UE having sent an initial INVITE request for MO Voice call }

ensure that {

when { UE receiving a 503 Service Unavailable response containing a Retry-After header indicating a period of 20 seconds }

then { UE does not reattempt the request until after the indicated period }

}

(2)

Void

7.1.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-15 requirements.

[TS 24.229, clause 5.1.3.1]:

Upon receiving a 503 (Service Unavailable) response to an initial INVITE request containing a Retry-After header, then the originating UE shall not automatically reattempt the request until after the period indicated by the Retry-After header contents.

[TS 24.229, clause 6.1.2]:

An INVITE request generated by a UE shall contain a SDP offer and at least one media description. This SDP offer shall reflect the calling user's terminal capabilities and user preferences for the session.

…

NOTE 2:	If the originating UE does not use the precondition mechanism (see subclause 5.1.3.1), it will not include any precondition information in the SDP message body.

7.1.3	Test description

7.1.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	The UE contains either ISIM and USIM applications or only USIM application on UICC.

-	The UE is configured to register for IMS after switch on.

-	The UE is either configured to use preconditions or to not use preconditions or does not support preconditions..

Preamble:

-	UE is in state 1N-A and registered to IMS

7.1.3.2	Test procedure sequence

Table 7.1.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is made to attempt an IMS voice call | - | - | - | - |
| 2-7 | Steps 2-7 of generic procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| - | EXCEPTION: In parallel with Step 8, parallel behaviour defined in table 7.1.3.2-2 takes place | - | - | - | - |
| - | EXCEPTION: Steps 8a to 8b describe behaviour that depends on UE configuration; the “lower case letter” identifies a step sequence that takes place if such configuration was conducted. | - | - | - | - |
| 8a | IF the UE is configured to use preconditions THEN step1 of Annex A.4.1 takes place. | --> | INVITE | - | - |
| 8b | ELSE step 1 of Annex A.4.2 takes place. | --> | INVITE | - | - |
| 9 | SS sends 503 (Service Unavailable) with Retry-After header indicating a period of 20 seconds. | <-- | 503 Service Unavailable | - | - |
| 10 | UE acknowledges the reception of 503 (Service Unavailable) message. | --> | ACK | - | - |
| 11 | The SS starts timer t_Waits=20s. | - | - | - | - |
| 12 | Check: Does the UE reattempt the INVITE request? | --> | INVITE | 1 | F |
| 13 | The SS waits for expiry of t_Waits. | - | - | - | - |
| - | EXCEPTION: Steps 14a1 to 14b8 happen if the UE choses to reattempt the INVITE request, i.e., these steps are optional. The SS waits at most 30 more seconds for such INVITE request before it terminates the test case. | - | - | - | - |
| - | EXCEPTION: Steps 14a1 to 14b11 describe behaviour that depends on UE configuration; the “lower case letter” identifies a step sequence that takes place if such configuration was conducted. | - | - | - | - |
| 14a1-14a5 | IF the UE is configured to use preconditions THEN steps 1 to 5 of Annex A.4.1 take place. | - | - | - | - |
| 14a6 | Step 10 of generic procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 [21] is performed. | - | - | - | - |
| - | EXCEPTION: In parallel to steps 14a7 and 14a8 below, step 14a9 occurs. | - | - | - | - |
| 14a7-14a8 | Steps 11-12 of generic procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| 14a9-14a15 | steps 6 to 12 of Annex A.4.1 take place | - | - | - | - |
| 14b1-14b5 | ELSE steps 1 to 5 of Annex A.4.2 takes place | - | - | - | - |
| 14b6-14b8 | Steps 10-12 of generic procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| 14b9-14b11 | Steps 6 to 8 of Annex A.4.2 take place | - | - | - | - |

Table 7.1.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |

7.1.3.3	Specific message contents

Table 7.1.3.3-1: 503 Service Unavailable (step 9, table 7.1.3.2-1)

| Derivation path: TS 34.229-1 [2], Annex A.4.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Retry-After |  |  |  |  |
| delta-seconds |  | 20 |  |  |

<End of modified section 1>