3GPP TSG-RAN5 Meeting #95-e	R5-222821r1

# Electronic Meeting, 09th – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2347 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update SIB1 for RedCap test |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1, There is no Redcap related parameters in SIB1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Update Redcap related parameters into SIB1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Redcap related parameters in SIB1 is not configured |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | 1st revision: (1)Change the RedCap condition to pc_RedCap_r17. (2)Delete the eDRX condition, if will be configured in specific test cases. (3)fix the overlap with R5-223290 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

Table 4.6.1-28: SIB1

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |  |
| q-RxLevMin |  | -70 | -140 dBm | RF OR RRM |
|  |  | -55 | -110 dBm | SIG AND FR1 |
|  |  | ROUND((-110+Delta(NRfs))/2) | -110+Delta(NRfs) | SIG AND FR2 |
| q-RxLevMinOffset |  | Not present |  |  |
| q-RxLevMinSUL |  | Not Present |  |  |
|  |  | -70 | -140 dBm | SUL AND (RF OR RRM) |
|  |  | -55 | -110 dBm | SUL AND SIG |
| q-QualMin |  | -20 | -20dB | QBASED |
|  |  | Not present |  |  |
| q-QualMinOffset |  | Not present |  |  |
| } |  |  |  |  |
| cellAccessRelatedInfo |  | CellAccessRelatedInfo |  |  |
| connEstFailureControl |  | ConnEstFailureControl |  |  |
| si-SchedulingInfo |  | Not present |  | NR_1 |
|  |  | SI-SchedulingInfo |  |  |
| servingCellConfigCommon |  | ServingCellConfigCommonSIB |  |  |
| ims-EmergencySupport |  | Not present |  |  |
|  |  | true | Indicates the cell supports IMS emergency bearer services for UEs in limited service mode. | SIG |
| eCallOverIMS-Support |  | Not present |  |  |
|  |  | true | Support of eCall over IMS services | eCalloverIMSforNR |
| ue-TimersAndConstants |  | UE-TimersAndConstants |  |  |
| uac-BarringInfo |  | Not present |  |  |
| useFullResumeID |  | Not present |  |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | posSIB |
| idleModeMeasurementsEUTRA-r16 |  | Not present |  |  |
| idleModeMeasurementsNR-r16 |  | Not present |  |  |
| posSI-SchedulingInfo-r16 |  | PosSI-SchedulingInfo-r16 |  |  |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | pc_RedCap_r17 |
| uac-BarringInfo-v1630 |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| hsdn-Cell-r17 |  | Not present |  |  |
| ue-TimersAndConstants-RemoteUE-r17 |  | Not present |  |  |
| uac-BarringInfo-v1700 |  | Not present |  |  |
| sdt-ConfigCommon-r17 |  | Not present |  |  |
| featurePriorities-r17 |  | Not present |  |  |
| si-SchedulingInfo-v1700 |  | Not present |  |  |
| hyperSFN-r17 |  | A valid value as defined in TS 38.331 [6] |  |  |
| eDRX-Allowed-r17 |  | Not present |  |  |
| halfDuplexRedCapAllowed-r17 |  | Not present |  |  |
| cellBarredRedCap-r17 |  | Not present |  |  |
| intraFreqReselectionRedCap-r17 |  | allowed |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| NOTE 1: Delta(NRfs) is derived based on calibration procedure defined in the clause 6.1.3.3. NRfs is NR frequency on which SIB1 is broadcasted. NOTE 2: ROUND is rounded off to the nearest integer. As an example, '1 to 1.49' set to '1' while '1.5 to 2' to '2' and '-2.0 to 1.5' set to '-2' while '-1.49 to -1' set to '-1'. |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| SUL | For test cases using SUL frequency for the serving cell, Qrxlevmin is obtained from q-RxLevMinSUL. |
| QBASED | This condition applies to Quality based signalling test cases. |
| NR_1 | System information combination NR-1 according table 4.4.3.1.2-1 is applied. |
| SIG | Used for signalling test cases. |
| eCalloverIMSforNR | Used for eCall over IMS test cases (TS 38.523-1[12], TS 34.229-5[47]) |
| posSIB | For test cases using posSIBs in system information. |

<End of modified section 1>