3GPP TSG-RAN5 Meeting #95-e	R5-222837r2

# Online, , 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1704 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Reference sensitivity TC for RedCap |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The redcap requirement was introduced at R5#94 mmeting. The refsens test case for redcap is still missing in TS 38.521-1. Therefore, the reference sensitivity TC for RedCap need to be added to align with core spec TS 38.101-1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of new TC 7.3I |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The reference sensitivity TC for RedCap will be still missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3I(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1:Updated editor’s note to indicate the message and generic procedure are not yet determined. Updated a typo of table number in minimum conformance requirement Updated test description Updated test requirement r2: Added editor’s note to indicated the PICS for HD-FDD RedCap UE in TS 38.508-2 is FFS |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

7.3I	Reference sensitivity for RedCap

7.3I.1	General

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports for all UE categories, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

7.3I.2	Reference sensitivity power level

Editor’s Note: The test case is not completed due to the following aspects are not yet determined:

- Generic procedure for RedCap UE in TS 38.508-1 is FFS

- Default message configuration for RedCap UE in TS 38.508-1 is FFS.

- PICS for HD-FDD ReDCap UE in TS 38.508-2 is FFS.

7.3I.2.1	Test purpose

The test purpose is to verify the ability of the UE to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise.

The test purpose is to verify the ability of the UE to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise.

7.3I.2.2	Test applicability

This test case applies to all types of NR UE release 17 and forward that support NR RedCap.

7.3I.2.3	Minimum conformance requirements

For a RedCap UE equipped with 2 Rx antenna ports, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.2.3-1a and Table 7.3.2.3-1b for the applicable operating bands. The reference sensitivity (REFSENS) requirement specified for a RedCap UE equipped with 2 Rx antenna ports shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3.2.3-3 and, for FDD bands, with the Tx-Rx separation as defined in clause 5.4.4 for the applicable band and UE channel bandwidth.

For a RedCap UE equipped with 1 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3.2.3-1a and in Table 7.3.2.3-1b shall be modified by the amount given in ΔR1R in Table 7.3I.2.3-1 for the applicable operating bands. The reference sensitivity (REFSENS) requirement specified for a RedCap UE equipped with 1 Rx antenna ports shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3.2.3-3 and, for FDD bands, with the Tx-Rx separation as defined in clause 5.4.4 for the applicable band and UE channel bandwidth.

Table 7.3I.2.3-1: Single antenna port reference sensitivity allowance ΔR1R

| Operating band | Channel bandwidth (MHz) | ΔR1R (dB) |
| --- | --- | --- |
| TDD band | 5, 10, 15, 20 | 2,5 |
| FDD band | 5 | 2.5 |
| FDD band | 10, 15, 20 | 3 |

For a RedCap UE equipped with 2 Rx antenna ports operating in HD-FDD mode, reference sensitivity for 2Rx antenna ports in Table 7.3I.2.3-2 shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3I.2.3-5.

Table 7.3I.2.3-2: HD-FDD RedCap UE with 2 Rx antenna port reference sensitivity

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| n1 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
| n2 | 15 | -98.8 | -95.6 | -93.8 | -92.5 |
|  | 30 |  | -96.0 | -94.0 | -92.7 |
|  | 60 |  | -96.3 | -94.2 | -93.0 |
| n3 | 15 | -97.8 | -94.6 | -92.8 | -91.5 |
|  | 30 |  | -95.0 | -93.0 | -91.7 |
|  | 60 |  | -95.3 | -93.2 | -92.0 |
| n5 | 15 | -98.8 | -95.6 | -93.8 | -92.5 |
|  | 30 |  | -96.0 | -94.0 | -92.7 |
| n7 | 15 | -98.8 | -95.6 | -93.8 | -92.5 |
|  | 30 |  | -96.0 | -94.0 | -92.7 |
|  | 60 |  | -96.3 | -94.2 | -93.0 |
| n8 | 15 | -97.8 | -94.6 | -92.8 | -91.5 |
|  | 30 |  | -95.0 | -93.0 | -91.7 |
| n12 | 15 | -97.8 | -94.6 | -92.8 |  |
|  | 30 |  | -95.0 | -93.0 |  |
| n13 | 15 | -97.8 | -94.6 |  |  |
|  | 30 |  | -95.0 |  |  |
| n14 | 15 | -97.8 | -94.6 |  |  |
|  | 30 |  | -95.0 |  |  |
| n18 | 15 | -100.0 | -96.8 | -95.0 |  |
|  | 30 |  | -97.2 | -95.2 |  |
| n20 | 15 | -97.8 | -94.6 | -92.8 | -91.5 |
|  | 30 |  | -95.0 | -93.0 | -91.7 |
| n24 | 15 | -100.0 | -96.8 |  |  |
|  | 30 |  | -97.2 |  |  |
|  | 60 |  | -97.5 |  |  |
| n25 | 15 | -97.3 | -94.1 | -92.3 | -91.0 |
|  | 30 |  | -94.5 | -92.5 | -91.2 |
|  | 60 |  | -94.8 | -92.7 | -91.5 |
| n26 | 15 | -98.3 | -95.1 | -93.3 | -92.0 |
|  | 30 |  | -95.5 | -93.5 | -92.2 |
| n28 | 15 | -99.3 | -96.1 | -94.3 | -93.0 |
|  | 30 |  | -96.5 | -94.5 | -93.2 |
| n30 | 15 | -99.5 | -96.3 |  |  |
|  | 30 |  | -96.7 |  |  |
| n65 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
| n66 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
| n70 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
| n71 | 15 | -98.0 | -94.8 | -93.0 | -91.7 |
|  | 30 |  | -95.2 | -93.2 | -91.9 |
| n74 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
| n85 | 15 | -97.8 | -94.6 | -92.8 |  |
|  | 30 |  | -95.0 | -93.0 |  |
| n91 | 15 | -100.0 |  |  |  |
| n92 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |
| n93 | 15 | -100.0 |  |  |  |
| n94 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |

For a RedCap UE equipped with 1 Rx antenna ports and operating in HD-FDD mode, reference sensitivity for 1Rx antenna ports in Table 7.3I.2.3-4 shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3I.2.3-5.

Table 7.3I.2.3-4: HD-FDD RedCap UE with 1 Rx antenna port reference sensitivity

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| n1 | 15 | -97.5 | -94.3 | -92.5 | -91.2 |
|  | 30 |  | -94.7 | -92.7 | -91.4 |
|  | 60 |  | -95.0 | -92.9 | -91.7 |
| n2 | 15 | -96.3 | -93.1 | -91.3 | -90.0 |
|  | 30 |  | -93.5 | -91.5 | -90.2 |
|  | 60 |  | -93.8 | -91.7 | -90.5 |
| n3 | 15 | -95.3 | -92.1 | -90.3 | -89.0 |
|  | 30 |  | -92.5 | -90.5 | -89.2 |
|  | 60 |  | -92.8 | -90.7 | -89.5 |
| n5 | 15 | -96.3 | -93.1 | -91.3 | -90.0 |
|  | 30 |  | -93.5 | -91.5 | -90.2 |
| n7 | 15 | -96.3 | -93.1 | -91.3 | -90.0 |
|  | 30 |  | -93.5 | -91.5 | -90.2 |
|  | 60 |  | -93.8 | -91.7 | -90.5 |
| n8 | 15 | -95.3 | -92.1 | -90.3 | -89.0 |
|  | 30 |  | -92.5 | -90.5 | -89.2 |
| n12 | 15 | -95.3 | -92.1 | -90.3 |  |
|  | 30 |  | -92.5 | -90.5 |  |
| n13 | 15 | -95.3 | -92.1 |  |  |
|  | 30 |  | -92.5 |  |  |
| n14 | 15 | -95.3 | -92.1 |  |  |
|  | 30 |  | -92.5 |  |  |
| n18 | 15 | -97.5 | -94.3 | -92.5 |  |
|  | 30 |  | -94.7 | -92.7 |  |
| n20 | 15 | -95.3 | -92.1 | -90.3 | -89.0 |
|  | 30 |  | -92.5 | -90.5 | -89.2 |
| n24 | 15 | -97.5 | -94.3 |  |  |
|  | 30 |  | -94.7 |  |  |
|  | 60 |  | -95.0 |  |  |
| n25 | 15 | -94.8 | -91.6 | -89.8 | -88.5 |
|  | 30 |  | -92.0 | -90.0 | -88.7 |
|  | 60 |  | -92.3 | -90.2 | -89.0 |
| n26 | 15 | -95.8 | -92.6 | -90.8 | -89.5 |
|  | 30 |  | -93.0 | -91.0 | -89.7 |
| n28 | 15 | -96.8 | -93.6 | -91.8 | -90.5 |
|  | 30 |  | -94.0 | -92.0 | -90.7 |
| n30 | 15 | -97.0 | -93.8 |  |  |
|  | 30 |  | -94.2 |  |  |
| n65 | 15 | -97.5 | -94.3 | -92.5 | -91.2 |
|  | 30 |  | -94.7 | -92.7 | -91.4 |
|  | 60 |  | -95.0 | -92.9 | -91.7 |
| n66 | 15 | -97.5 | -94.3 | -92.5 | -91.2 |
|  | 30 |  | -94.7 | -92.7 | -91.4 |
|  | 60 |  | -95.0 | -92.9 | -91.7 |
| n70 | 15 | -97.5 | -94.3 | -92.5 | -91.2 |
|  | 30 |  | -94.7 | -92.7 | -91.4 |
|  | 60 |  | -95.0 | -92.9 | -91.7 |
| n71 | 15 | -95.5 | -92.3 | -90.5 | -89.2 |
|  | 30 |  | -92.7 | -90.7 | -89.4 |
| n74 | 15 | -97.5 | -94.3 | -92.5 | -91.2 |
|  | 30 |  | -94.7 | -92.7 | -91.4 |
|  | 60 |  | -95.0 | -92.9 | -91.7 |
| n85 | 15 | -95.3 | -92.1 | -90.3 |  |
|  | 30 |  | -92.5 | -90.5 |  |
| n91 | 15 | -97.5 |  |  |  |
| n92 | 15 | -97.5 | -94.3 | -92.5 | -91.2 |
|  | 30 |  | -94.7 | -92.7 | -91.4 |
| n93 | 15 | -97.5 |  |  |  |
| n94 | 15 | -97.5 | -94.3 | -92.5 | -91.2 |
|  | 30 |  | -94.7 | -92.7 | -91.4 |

Table 7.3I.2.3-5: Uplink configuration for HD-FDD reference sensitivity

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| n1 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n2 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n3 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n5 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n7 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  |  |  | 10 | 18 | 24 |
| n8 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n12 | 15 | 25 | 50 | 75 |  |
|  | 30 |  | 24 | 36 |  |
| n13 | 15 | 25 | 50 |  |  |
|  | 30 |  | 24 |  |  |
| n14 | 15 | 25 | 50 |  |  |
|  | 30 |  | 24 |  |  |
| n18 | 15 | 25 | 50 | 75 |  |
|  | 30 |  | 24 | 36 |  |
| n20 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n24 | 15 | 25 | 50 |  |  |
|  | 30 |  | 24 |  |  |
|  |  |  | 10 |  |  |
| n25 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n26 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n28 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n30 | 15 | 25 | 50 |  |  |
|  | 30 |  | 24 |  |  |
| n65 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  |  |  | 10 | 18 | 24 |
| n66 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  |  |  | 10 | 18 | 24 |
| n70 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  |  |  | 10 | 18 | 24 |
| n71 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n74 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  |  |  | 10 | 18 | 24 |
| n85 | 15 | 25 | 50 | 75 |  |
|  | 30 |  | 24 | 36 |  |
| n91 | 15 | 25 |  |  |  |
| n92 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n93 | 15 | 25 |  |  |  |
| n94 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |

The normative reference for this requirement is TS 38.101-1 [2] clause 7.3I.2.

7.3I.2.4	Test description

7.3I.2.4.1	Initial conditions

For RedCap UE with 1Rx or 2 Rx antenna ports, same initial conditions as in 7.3.2.4.1 with following exception:

- The test channel bandwidth are specified in TS 38.508-1 [5] subclause 4.3.1 for RedCap.

For HD-FDD RedCap UE with 1 Rx or 2 Rx antenna ports, same initial conditions as in 7.3.2.4.1 with following exception:

- The test channel bandwidth are specified in TS 38.508-1 [5] subclause 4.3.1 for RedCap.

- The RB allocation for uplink configuration in Table 7.3.2.4.1-1 refers to Table 7.3I.2.4.1-1 for each SCS, channel BW and NR band.

Table 7.3I.2.4.1-1: Uplink configuration for reference sensitivity of HD-FDD RedCap UE, LCRB @ RBstart format

| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- |
| n1 | 15 | 25@0 | 50@2 | 75@4 | 100@6 | HD-FDD |
|  | 30 |  | 24@0 | 36@2 | 50@1 |  |
|  | 60 |  | 10@1 | 18@0 | 24@0 |  |
| n2 | 15 | 25@0 | 50@2 | 75@4 | 100@6 | HD-FDD |
|  | 30 |  | 24@0 | 36@2 | 50@1 |  |
|  | 60 |  | 10@1 | 18@0 | 24@0 |  |
| n3 | 15 | 25@0 | 50@2 | 75@4 | 100@6 | HD-FDD |
|  | 30 |  | 24@0 | 36@2 | 50@1 |  |
|  | 60 |  | 10@1 | 18@0 | 24@0 |  |
| n5 | 15 | 25@0 | 50@2 | 75@4 | 100@6 | HD-FDD |
|  | 30 |  | 24@0 | 36@2 | 50@1 |  |
| n7 | 15 | 25@0 | 50@2 | 75@4 | 100@6 | HD-FDD |
|  | 30 |  | 24@0 | 36@2 | 50@1 |  |
|  | 60 |  | 10@1 | 18@0 | 24@0 |  |
| n8 | 15 | 25@0 | 50@2 | 75@4 | 100@6 | HD-FDD |
|  | 30 |  | 24@0 | 36@2 | 50@1 |  |
| n12 | 15 | 25@0 | 50@2 | 75@4 |  | HD-FDD |
|  | 30 |  | 24@0 | 36@2 |  |  |
| n14 | 15 | 25@0 | 50@0 |  |  | HD-FDD |
|  | 30 |  | 24@0 |  |  |  |
| n20 | 15 | 25@0 | 50@0 | 75@0 | 100@0 | HD-FDD |
|  | 30 |  | 24@0 | 36@0 | 50@0 |  |
| n24 | 15 | 25@0 | 50@0 |  |  | HD-FDD |
|  | 30 |  | 24@0 |  |  |  |
|  | 60 |  | 10@0 |  |  |  |
| n25 | 15 | 25@0 | 50@2 | 75@4 | 100@6 | HD-FDD |
|  | 30 |  | 24@0 | 36@2 | 50@1 |  |
|  | 60 |  | 10@1 | 18@0 | 24@0 |  |
| n26 | 15 | 25@0 | 50@2 | 75@4 | 100@6 | HD-FDD |
|  | 30 |  | 24@0 | 36@2 | 50@1 |  |
| n28 | 15 | 25@0 | 50@2 | 75@4 | 100@6 | HD-FDD |
|  | 30 |  | 24@0 | 36@2 | 50@1 |  |
| n30 | 15 | 25@0 | 50@2 |  |  | HD-FDD |
|  | 30 |  | 24@0 |  |  |  |
| n65 | 15 | 25@0 | 50@2 | 75@4 | 100@6 | HD-FDD |
|  | 30 |  | 24@0 | 36@2 | 50@1 |  |
|  | 60 |  | 10@1 | 18@0 | 24@0 |  |
| n66 | 15 | 25@0 | 50@2 | 75@4 | 100@6 | HD-FDD |
|  | 30 |  | 24@0 | 36@2 | 50@1 |  |
|  | 60 |  | 10@1 | 18@0 | 24@0 |  |
| n70 | 15 | 25@0 | 50@2 | 75@4 | 100@6 | HD-FDD |
|  | 30 |  | 24@0 | 36@2 | 50@1 |  |
|  | 60 |  | 10@1 | 18@0 | 24@0 |  |
| n71 | 15 | 25@0 | 50@0 | 75@0 | 100@0 | HD-FDD |
|  | 30 |  | 24@0 | 36@0 | 50@0 |  |
| n74 | 15 | 25@0 | 50@2 | 75@4 | 100@6 | HD-FDD |
|  | 30 |  | 24@0 | 36@2 | 50@1 |  |
|  | 60 |  | 10@1 | 18@0 | 24@0 |  |

7.3I.2.4.2	Test procedure

Same test procedure as steps 1~4 of clause 7.3.2.4.2 with the following exceptions of step 3.

- Set the Downlink signal level to the appropriate REFSENS value defined in Tables 7.3I.2.5-1, 7.3I.2.5-2 and 7.3I.2.5-5 if 2Rx antennas connected or Tables 7.3I.2.5-3, 7.3I.2.5-4 and 7.3I.2.5-6 if 1Rx antennas connected.

7.3I.2.4.3	Message contents

FFS

7.3I.2.5	Test requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A3.2 with reference receive power level specified in Table 7.3I.2.5-1 and Table 7.3I.2.5-2 for RedCap UE with 2 Rx antenna port, Tables 7.3I.2.5-3 and Table 7.3I.2.5-4 for RedCap UE with single antenna port, Table 7.3I.2.5-5 for HD-FDD RedCap UE with 2 Rx antenna port, Table 7.3I.2.5-6 for HD-FDD RedCap UE single antenna port, and parameters specified Table 7.3.2.4.1-1, Table 7.3.2.4.1-2 and Table 7.3I.2.4.1-1.

Table 7.3I.2.5-1: Two antenna port Reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth / REFSENS / Duplex Mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| n1 | 15 | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.8 +TT | FDD |
|  | 30 |  | -97.1 +TT | -95.1 +TT | -94.0 +TT |  |
|  | 60 |  | -97.5 +TT | -95.4 +TT | -94.2 +TT |  |
| n2 | 15 | -98.0 +TT | -94.8 +TT | -93.0 +TT | -91.8 +TT | FDD |
|  | 30 |  | -95.1 +TT | -93.1 +TT | -92.0 +TT |  |
|  | 60 |  | -95.5 +TT | -93.4 +TT | -92.2 +TT |  |
| n3 | 15 | -97.0 +TT | -93.8 +TT | -92.0 +TT | -90.8 +TT | FDD |
|  | 30 |  | -94.1 +TT | -92.1 +TT | -91.0 +TT |  |
|  | 60 |  | -94.5 +TT | -92.4 +TT | -91.2 +TT |  |
| n5 | 15 | -98.0 +TT | -94.8 +TT | -93.0 +TT | -90.8 +TT | FDD |
|  | 30 |  | -95.1 +TT | -93.1 +TT | -91.0 +TT |  |
| n7 | 15 | -98.0 +TT | -94.8 +TT | -93.0 +TT | -91.8 +TT | FDD |
|  | 30 |  | -95.1 +TT | -93.1 +TT | -92.0 +TT |  |
|  | 60 |  | -95.5 +TT | -93.4 +TT | -92.2 +TT |  |
| n8 | 15 | -97.0 +TT | -93.8 +TT | -92.0 +TT | -90.0 +TT | FDD |
|  | 30 |  | -94.1 +TT | -92.1 +TT | -90.2 +TT |  |
| n12 | 15 | -97.0 +TT | -93.8 +TT | -84.0 +TT |  | FDD |
|  | 30 |  | -94.1 +TT | -84.1 +TT |  |  |
| n14 | 15 | -97.0 +TT | -93.8 +TT |  |  | FDD |
|  | 30 |  | -94.1 +TT |  |  |  |
| n20 | 15 | -97.0 +TT | -93.8 +TT | -91.0 +TT | -89.8 +TT | FDD |
|  | 30 |  | -94.1 +TT | -91.1 +TT | -90.0 +TT |  |
| n24 | 15 | -100.0 +TT | -96.8 +TT |  |  | FDD |
|  | 30 |  | -97.1 +TT |  |  |  |
|  | 60 |  | -97.5 +TT |  |  |  |
| n25 | 15 | -96.5 +TT | -93.3 +TT | -91.5 +TT | -90.3 +TT | FDD |
|  | 30 |  | -93.6 +TT | -91.6 +TT | -90.5 +TT |  |
|  | 60 |  | -94.0 +TT | -91.9 +TT | -90.7 +TT |  |
| n26 | 15 | -97.5 +TT | -94.5 +TT | -92.7 +TT | -87.6 +TT | FDD |
|  | 30 |  | -94.8 +TT | -92.7 +TT | -87.7 +TT |  |
| n28 | 15 | -98.5 +TT | -95.5 +TT | -93.5 +TT | -90.8 +TT | FDD |
|  | 30 |  | -95.6 +TT | -93.6 +TT | -91.0 +TT |  |
| n30 | 15 | -99.0 +TT | -95.8 +TT |  |  | FDD |
|  | 30 |  | -96.1 +TT |  |  |  |
| n65 | 15 | -99.5+TT | -96.3+TT | -94.5+TT | -93.3+TT | FDD |
|  | 30 |  | -96.6+TT | -94.6+TT | -93.5+TT |  |
|  | 60 |  | -97.0+TT | -94.9+TT | -93.7+TT |  |
| n66 | 15 | -99.5 +TT | -96.3 +TT | -94.5 +TT | -93.3 +TT | FDD |
|  | 30 |  | -96.6 +TT | -94.6 +TT | -93.5 +TT |  |
|  | 60 |  | -97.0 +TT | -94.9 +TT | -93.7 +TT |  |
| n70 | 15 | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.8 +TT | FDD |
|  | 30 |  | -97.1 +TT | -95.1 +TT | -94.0 +TT |  |
|  | 60 |  | -97.5 +TT | -95.4 +TT | -94.2 +TT |  |
| n71 | 15 | -97.2 +TT | -94.0 +TT | -91.6 +TT | -86.0 +TT | FDD |
|  | 30 |  | -94.3 +TT | -91.9 +TT | -87.4 +TT |  |
| n74 | 15 | -99.53 +TT | -96.33 +TT | -94.53 +TT | -93.33 +TT | FDD |
|  | 30 |  | -96.63 +TT | -94.63 +TT | -93.53 +TT |  |
|  | 60 |  | -97.03 +TT | -94.93 +TT | -93.73 +TT |  |
| NOTE 1: Void NOTE 2: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 NOTE 3: 3 indicates that the requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9-1510.9 MHz. NOTE 4: TT for each frequency and channel bandwidth is specified in Table 7.3I.2.5-7. |  |  |  |  |  |  |

Table 7.3I.2.5-2:  Two antenna port reference sensitivity QPSK PREFSENS for TDD bands

| Operating band / SCS / Channel bandwidth / REFSENS / Duplex Mode |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm)8 | Duplex Mode |
| n34 | 15 | 5, 10, 15 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15 | -97.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15 | -97.5 + 10log10(NRB/11)+TT |  |
| n381 | 15 | 5, 10, 15, 20 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15, 20, | -97.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20 | -97.5 + 10log10(NRB/11)+TT |  |
| n39 | 15 | 5, 10, 15, 20 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15, 20 | -97.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20, | -97.5 + 10log10(NRB/11)+TT |  |
| n40 | 15 | 5, 10, 15, 20 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15, 20 | -97.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20 | -97.5 + 10log10(NRB/11)+TT |  |
| n411 | 15 | 10, 15, 20 | -94.8 + 10log10(NRB/50)+TT | TDD |
|  | 30 | 10, 15, 20 | -95.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20 | -95.5 + 10log10(NRB/11)+TT |  |
| n481 | 15 | 5, 10, 15, 20, | -99 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15, 20, | -96.1 + 10log10(NRB/24) +TT |  |
|  | 60 | 10, 15, 20, | -96.5 + 10log10(NRB/11)+TT |  |
| n50 | 15 | 5, 10, 15, 20 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15, 20 | -97.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20 | -97.5 + 10log10(NRB/11)+TT |  |
| n51 | 15 | 5 | -100+TT | TDD |
| n53 | 15 | 5, 10 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10 | -97.1+TT |  |
|  | 60 | 10 | -97.5+TT |  |
| n771,4 | 15 | 10, 15, 20, | -95.3 + 10log10(NRB/50)+TT | TDD |
|  | 30 | 10, 15, 20 | -95.6 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20 | -96.0 + 10log10(NRB/11)+TT |  |
| n781 | 15 | 10, 15, 20 | -95.8 + 10log10(NRB/50) +TT | TDD |
|  | 30 | 10, 15, 20 | -96.1 + 10log10(NRB/24) +TT |  |
|  | 60 | 10, 15, 20 | -96.5 + 10log10(NRB/11)+TT |  |
| n791 | 15 | 10, 20, | -89.6 + 10log10(NRB/216)+TT | TDD |
|  | 30 | 10, 20, | -89.7 + 10log10(NRB/106) +TT |  |
|  | 60 | 10, 20 | -89.9 + 10log10(NRB/51) +TT |  |
| NOTE 1: Void. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 3: Void NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: Void NOTE 6: Void NOTE 7: Void NOTE 8: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. NOTE 9: TT for each frequency and channel bandwidth is specified in Table 7.3I.2.5-7. |  |  |  |  |

Table 7.3I.2.5-3: Single antenna port Reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| n1 | 15 | -100.0+2.5 +TT | -96.8 +3 +TT | -95.0 +3 +TT | -93.8 +3 +TT | FDD |
|  | 30 |  | -97.1 +3 +TT | -95.1 +3 +TT | -94.0 +3 +TT |  |
|  | 60 |  | -97.5 +TT | -95.4 +TT | -94.2 +TT |  |
| n2 | 15 | -98.0 +2.5+TT | -94.8 +3 +TT | -93.0 +3 +TT | -91.8 +3 +TT | FDD |
|  | 30 |  | -95.1 +3 +TT | -93.1 +3 +TT | -92.0 +3 +TT |  |
|  | 60 |  | -95.5 +3 +TT | -93.4 +3 +TT | -92.2 +3 +TT |  |
| n3 | 15 | -97.0+2.5 +TT | -93.8 +3 +TT | -92.0 +3 +TT | -90.8 +3 +TT | FDD |
|  | 30 |  | -94.1 +3 +TT | -92.1 +3 +TT | -91.0 +3 +TT |  |
|  | 60 |  | -94.5 +3 +TT | -92.4 +3 +TT | -91.2 +3 +TT |  |
| n5 | 15 | -98.0+2.5 +TT | -94.8 +3 +TT | -93.0 +3 +TT | -90.8 +3 +TT | FDD |
|  | 30 |  | -95.1 +3 +TT | -93.1 +3 +TT | -91.0 +3 +TT |  |
| n7 | 15 | -98.0+2.5 +TT | -94.8 +3 +TT | -93.0 +3 +TT | -91.8 +3 +TT | FDD |
|  | 30 |  | -95.1 +3 +TT | -93.1 +3 +TT | -92.0 +3 +TT |  |
|  | 60 |  | -95.5 +3 +TT | -93.4 +3 +TT | -92.2 +3 +TT |  |
| n8 | 15 | -97.0+2.5 +TT | -93.8 +3 +TT | -92.0 +3 +TT | -90.0 +3 +TT | FDD |
|  | 30 |  | -94.1 +3 +TT | -92.1 +3 +TT | -90.2 +3 +TT |  |
| n12 | 15 | -97.0 +2.5+TT | -93.8 +3 +TT | -84.0 +3 +TT |  | FDD |
|  | 30 |  | -94.1 +3 +TT | -84.1 +3 +TT |  |  |
| n14 | 15 | -97.0+2.5 +TT | -93.8 +3 +TT |  |  | FDD |
|  | 30 |  | -94.1 +3 +TT |  |  |  |
| n20 | 15 | -97.0 +2.5+TT | -93.8 +3 +TT | -91.0 +3 +TT | -89.8 +3 +TT | FDD |
|  | 30 |  | -94.1 +3 +TT | -91.1 +3 +TT | -90.0 +3 +TT |  |
| n24 | 15 | -100.0 +2.5+TT | -96.8 +3 +TT |  |  | FDD |
|  | 30 |  | -97.1 +3 +TT |  |  |  |
|  | 60 |  | -97.5 +3 +TT |  |  |  |
| n25 | 15 | -96.5 +2.5+TT | -93.3 +3 +TT | -91.5 +3 +TT | -90.3 +3 +TT | FDD |
|  | 30 |  | -93.6 +3 +TT | -91.6 +3 +TT | -90.5 +3 +TT |  |
|  | 60 |  | -94.0 +3 +TT | -91.9 +3 +TT | -90.7 +3 +TT |  |
| n26 | 15 | -97.5+2.5 +TT | -94.5 +3 +TT | -92.7 +3 +TT | -87.6 +3 +TT | FDD |
|  | 30 |  | -94.8 +3 +TT | -92.7 +3 +TT | -87.7 +3 +TT |  |
| n28 | 15 | -98.5 +2.5+TT | -95.5 +3 +TT | -93.5 +3 +TT | -90.8 +3 +TT | FDD |
|  | 30 |  | -95.6 +3 +TT | -93.6 +3 +TT | -91.0 +3 +TT |  |
| n30 | 15 | -99.0 +2.5+TT | -95.8 +3 +TT |  |  | FDD |
|  | 30 |  | -96.1 +3 +TT |  |  |  |
| n65 | 15 | -99.5 +2.5+TT | -96.3 +3+TT | -94.5 +3+TT | -93.3 +3+TT | FDD |
|  | 30 |  | -96.6 +3 +TT | -94.6 +3 +TT | -93.5 +3 +TT |  |
|  | 60 |  | -97.0 +3+TT | -94.9 +3+TT | -93.7 +3+TT |  |
| n66 | 15 | -99.5 +2.5+TT | -96.3 +3+TT | -94.5 +3 +TT | -93.3 +3 +TT | FDD |
|  | 30 |  | -96.6 +3 +TT | -94.6 +3 +TT | -93.5 +3+TT |  |
|  | 60 |  | -97.0 +3 +TT | -94.9 +3 +TT | -93.7 +3 +TT |  |
| n70 | 15 | -100.0 +2.5 +TT | -96.8 +3 +TT | -95.0 +3+TT | -93.8 +3 +TT | FDD |
|  | 30 |  | -97.1 +3 +TT | -95.1 +3 +TT | -94.0 +3 +TT |  |
|  | 60 |  | -97.5 +3 +TT | -95.4 +3 +TT | -94.2 +3 +TT |  |
| n71 | 15 | -97.2+2.5 +TT | -94.0 +3 +TT | -91.6 +3 +TT | -86.0 +3 +TT | FDD |
|  | 30 |  | -94.3 +3 +TT | -91.9 +3 +TT | -87.4 +3 +TT |  |
| n74 | 15 | -99.53 +2.5 +TT | -96.33 +3 +TT | -94.53 +3 +TT | -93.33 +3+TT | FDD |
|  | 30 |  | -96.63 +3 +TT | -94.63 +3 +TT | -93.53 +3 +TT |  |
|  | 60 |  | -97.03 +3 +TT | -94.93 +3 +TT | -93.73 +3+TT |  |
| NOTE 1: Void NOTE 2: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 NOTE 3: 3 indicates that the requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9-1510.9 MHz. NOTE 4: TT for each frequency and channel bandwidth is specified in Table 7.3I.2.5-7. |  |  |  |  |  |  |

Table 7.3F.2.5-4:  Single antenna port reference sensitivity QPSK PREFSENS for TDD bands

| Operating band / SCS / Channel bandwidth / REFSENS/Duplex Mode |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm)8 | Duplex Mode |
| n34 | 15 | 5, 10, 15 | -100 + 10log10(NRB/25) +2.5+TT | TDD |
|  | 30 | 10, 15 | -97.1 + 10log10(NRB/24) +2.5+TT |  |
|  | 60 | 10, 15 | -97.5 + 10log10(NRB/11) +2.5+TT |  |
| n381 | 15 | 5, 10, 15, 20, | -100 + 10log10(NRB/25) +2.5+TT | TDD |
|  | 30 | 10, 15, 20, | -97.1 + 10log10(NRB/24) +2.5+TT |  |
|  | 60 | 10, 15, 20 | -97.5 + 10log10(NRB/11) +2.5+TT |  |
| n39 | 15 | 5, 10, 15, 20 | -100 + 10log10(NRB/25) +2.5+TT | TDD |
|  | 30 | 10, 15, 20, | -97.1 + 10log10(NRB/24) +2.5+TT |  |
|  | 60 | 10, 15, 20, | -97.5 + 10log10(NRB/11) +2.5+TT |  |
| n40 | 15 | 5, 10, 15, 20 | -100 + 10log10(NRB/25) +2.5+TT | TDD |
|  | 30 | 10, 15, 20 | -97.1 + 10log10(NRB/24) +2.5+TT |  |
|  | 60 | 10, 15, 20 | -97.5 + 10log10(NRB/11) +2.5+TT |  |
| n411 | 15 | 10, 15, 20 | -94.8 + 10log10(NRB/50) +2.5+TT | TDD |
|  | 30 | 10, 15, 20 | -95.1 + 10log10(NRB/24) +2.5+TT |  |
|  | 60 | 10, 15, 20 | -95.5 + 10log10(NRB/11) +2.5+TT |  |
| n481 | 15 | 5, 10, 15, 20, | -99 + 10log10(NRB/25) +2.5+TT | TDD |
|  | 30 | 10, 15, 20, | -96.1 + 10log10(NRB/24) +2.5+TT |  |
|  | 60 | 10, 15, 20, | -96.5 + 10log10(NRB/11) +2.5+TT |  |
| n50 | 15 | 5, 10, 15, 20 | -100 + 10log10(NRB/25) +2.5+TT | TDD |
|  | 30 | 10, 15, 20 | -97.1 + 10log10(NRB/24) +2.5+TT |  |
|  | 60 | 10, 15, 20 | -97.5 + 10log10(NRB/11) +2.5+TT |  |
| n51 | 15 | 5 | -100 +2.5+TT | TDD |
| n53 | 15 | 5, 10 | -100 + 10log10(NRB/25) +2.5+TT | TDD |
|  | 30 | 10 | -97.1 +2.5+TT |  |
|  | 60 | 10 | -97.5 +2.5+TT |  |
| n771,4 | 15 | 10, 15, 20, | -95.3 + 10log10(NRB/50) +2.5+TT | TDD |
|  | 30 | 10, 15, 20 | -95.6 + 10log10(NRB/24) +2.5+TT |  |
|  | 60 | 10, 15, 20 | -96.0 + 10log10(NRB/11) +2.5+TT |  |
| n781 | 15 | 10, 15, 20 | -95.8 + 10log10(NRB/50) +2.5+TT | TDD |
|  | 30 | 10, 15, 20 | -96.1 + 10log10(NRB/24) +2.5+TT |  |
|  | 60 | 10, 15, 20 | -96.5 + 10log10(NRB/11) +2.5+TT |  |
| n791 | 15 | 10, 20, | -89.6 + 10log10(NRB/216) +2.5+TT | TDD |
|  | 30 | 10, 20, | -89.7 + 10log10(NRB/106) +2.5+TT |  |
|  | 60 | 10, 20 | -89.9 + 10log10(NRB/51) +2.5+TT |  |
| NOTE 1: Void. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 3: Void NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: Void NOTE 6: Void NOTE 7: Void NOTE 8: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. NOTE 9: TT for each frequency and channel bandwidth is specified in Table 7.3I.2.5-7. |  |  |  |  |

Table 7.3I.2.5-5: Two antenna port reference sensitivity QPSK PREFSENS for HD-FDD operation

| Operating band / SCS / Channel bandwidth/ REFSENS / Duplex Mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| n1 | 15 | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.7 +TT | HD-FDD |
|  | 30 |  | -97.2 +TT | -95.2 +TT | -93.9 +TT |  |
|  | 60 |  | -97.5 +TT | -95.4 +TT | -94.2 +TT |  |
| n2 | 15 | -98.8 +TT | -95.6 +TT | -93.8 +TT | -92.5 +TT | HD-FDD |
|  | 30 |  | -96.0 +TT | -94.0 +TT | -92.7 +TT |  |
|  | 60 |  | -96.3 +TT | -94.2 +TT | -93.0 +TT |  |
| n3 | 15 | -97.8 +TT | -94.6 +TT | -92.8 +TT | -91.5 +TT | HD-FDD |
|  | 30 |  | -95.0 +TT | -93.0 +TT | -91.7 +TT |  |
|  | 60 |  | -95.3 +TT | -93.2 +TT | -92.0 +TT |  |
| n5 | 15 | -98.8 +TT | -95.6 +TT | -93.8 +TT | -92.5 +TT | HD-FDD |
|  | 30 |  | -96.0 +TT | -94.0 +TT | -92.7 +TT |  |
| n7 | 15 | -98.8 +TT | -95.6 +TT | -93.8 +TT | -92.5 +TT | HD-FDD |
|  | 30 |  | -96.0 +TT | -94.0 +TT | -92.7 +TT |  |
|  | 60 |  | -96.3 +TT | -94.2 +TT | -93.0 +TT |  |
| n8 | 15 | -97.8 +TT | -94.6 +TT | -92.8 +TT | -91.5 +TT | HD-FDD |
|  | 30 |  | -95.0 +TT | -93.0 +TT | -91.7 +TT |  |
| n12 | 15 | -97.8 +TT | -94.6 +TT | -92.8 +TT |  | HD-FDD |
|  | 30 |  | -95.0 +TT | -93.0 +TT |  |  |
| n13 | 15 | -97.8 +TT | -94.6 +TT |  |  | HD-FDD |
|  | 30 |  | -95.0 +TT |  |  |  |
| n14 | 15 | -97.8 +TT | -94.6 +TT |  |  | HD-FDD |
|  | 30 |  | -95.0 +TT |  |  |  |
| n18 | 15 | -100.0 +TT | -96.8 +TT | -95.0 +TT |  | HD-FDD |
|  | 30 |  | -97.2 +TT | -95.2 +TT |  |  |
| n20 | 15 | -97.8 +TT | -94.6 +TT | -92.8 +TT | -91.5 +TT | HD-FDD |
|  | 30 |  | -95.0 +TT | -93.0 +TT | -91.7 +TT |  |
| n24 | 15 | -100.0 +TT | -96.8 +TT |  |  | HD-FDD |
|  | 30 |  | -97.2 +TT |  |  |  |
|  | 60 |  | -97.5 +TT |  |  |  |
| n25 | 15 | -97.3 +TT | -94.1 +TT | -92.3 +TT | -91.0 +TT | HD-FDD |
|  | 30 |  | -94.5 +TT | -92.5 +TT | -91.2 +TT |  |
|  | 60 |  | -94.8 +TT | -92.7 +TT | -91.5 +TT |  |
| n26 | 15 | -98.3 +TT | -95.1 +TT | -93.3 +TT | -92.0 +TT | HD-FDD |
|  | 30 |  | -95.5 +TT | -93.5 +TT | -92.2 +TT |  |
| n28 | 15 | -99.3 +TT | -96.1 +TT | -94.3 +TT | -93.0 +TT | HD-FDD |
|  | 30 |  | -96.5 +TT | -94.5 +TT | -93.2 +TT |  |
| n30 | 15 | -99.5 +TT | -96.3 +TT |  |  | HD-FDD |
|  | 30 |  | -96.7 +TT |  |  |  |
| n65 | 15 | -100.0 +TT | -96.8+TT | -95.0 +TT | -93.7 +TT | HD-FDD |
|  | 30 |  | -97.2 +TT | -95.2 +TT | -93.9 +TT |  |
|  | 60 |  | -97.5 +TT | -95.4 +TT | -94.2 +TT |  |
| n66 | 15 | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.7 +TT | HD-FDD |
|  | 30 |  | -97.2 +TT | -95.2 +TT | -93.9 +TT |  |
|  | 60 |  | -97.5 +TT | -95.4 +TT | -94.2 +TT |  |
| n70 | 15 | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.7 +TT | HD-FDD |
|  | 30 |  | -97.2 +TT | -95.2 +TT | -93.9 +TT |  |
|  | 60 |  | -97.5 +TT | -95.4 +TT | -94.2 +TT |  |
| n71 | 15 | -98.0 +TT | -94.8 | -93.0 +TT | -91.7 +TT | HD-FDD |
|  | 30 |  | -95.2 +TT | -93.2 +TT | -91.9 +TT |  |
| n74 | 15 | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.7 +TT | HD-FDD |
|  | 30 |  | -97.2 +TT | -95.2 +TT | -93.9 +TT |  |
|  | 60 |  | -97.5 +TT | -95.4 +TT | -94.2 +TT |  |
| n85 | 15 | -97.8 +TT | -94.6 +TT | -92.8 +TT |  | HD-FDD |
|  | 30 |  | -95.0 +TT | -93.0 +TT |  |  |
| n91 | 15 | -100.0 +TT |  |  |  | HD-FDD |
| n92 | 15 | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.7 +TT | HD-FDD |
|  | 30 |  | -97.2 +TT | -95.2 +TT | -93.9 +TT |  |
| n93 | 15 | -100.0 +TT |  |  |  | HD-FDD |
| n94 | 15 | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.7 +TT | HD-FDD |
|  | 30 |  | -97.2 +TT | -95.2 +TT | -93.9 +TT |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 7.3I.2.5-7. |  |  |  |  |  |  |

Table 7.3I.2.5-6: Single antenna port Reference sensitivity QPSK PREFSENS for HD-FDD operation

| Operating band / SCS / Channel bandwidth/ REFSENS / Duplex Mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| n1 | 15 | -97.5 +TT | -94.3 +TT | -92.5 +TT | -91.2 +TT | HD-FDD |
|  | 30 |  | -94.7 +TT | -92.7 +TT | -91.4 +TT |  |
|  | 60 |  | -95.0 +TT | -92.9 +TT | -91.7 +TT |  |
| n2 | 15 | -96.3 +TT | -93.1 +TT | -91.3 +TT | -90.0 +TT | HD-FDD |
|  | 30 |  | -93.5 +TT | -91.5 | -90.2 |  |
|  | 60 |  | -93.8 +TT | -91.7 +TT | -90.5 +TT |  |
| n3 | 15 | -95.3 +TT | -92.1 +TT | -90.3 +TT | -89.0 +TT | HD-FDD |
|  | 30 |  | -92.5 +TT | -90.5 +TT | -89.2 +TT |  |
|  | 60 |  | -92.8 +TT | -90.7 +TT | -89.5 +TT |  |
| n5 | 15 | -96.3 +TT | -93.1 +TT | -91.3 +TT | -90.0 +TT | HD-FDD |
|  | 30 |  | -93.5 +TT | -91.5 +TT | -90.2 +TT |  |
| n7 | 15 | -96.3 +TT | -93.1 +TT | -91.3 +TT | -90.0 +TT | HD-FDD |
|  | 30 |  | -93.5 +TT | -91.5 +TT | -90.2 +TT |  |
|  | 60 |  | -93.8 +TT | -91.7 +TT | -90.5 +TT |  |
| n8 | 15 | -95.3 +TT | -92.1 +TT | -90.3 +TT | -89.0 +TT | HD-FDD |
|  | 30 |  | -92.5 +TT | -90.5 +TT | -89.2 +TT |  |
| n12 | 15 | -95.3 +TT | -92.1 +TT | -90.3 +TT |  | HD-FDD |
|  | 30 |  | -92.5 +TT | -90.5 +TT |  |  |
| n13 | 15 | -95.3 +TT | -92.1 +TT |  |  | HD-FDD |
|  | 30 |  | -92.5 +TT |  |  |  |
| n14 | 15 | -95.3 +TT | -92.1 +TT |  |  | HD-FDD |
|  | 30 |  | -92.5 +TT |  |  |  |
| n18 | 15 | -97.5 +TT | -94.3 +TT | -92.5 +TT |  | HD-FDD |
|  | 30 |  | -94.7 +TT | -92.7 +TT |  |  |
| n20 | 15 | -95.3 +TT | -92.1 +TT | -90.3 +TT | -89.0 +TT | HD-FDD |
|  | 30 |  | -92.5 +TT | -90.5 +TT | -89.2 +TT |  |
| n24 | 15 | -97.5 +TT | -94.3 +TT |  |  | HD-FDD |
|  | 30 |  | -94.7 +TT |  |  |  |
|  | 60 |  | -95.0 +TT |  |  |  |
| n25 | 15 | -94.8 +TT | -91.6 +TT | -89.8 +TT | -88.5 +TT | HD-FDD |
|  | 30 |  | -92.0 +TT | -90.0 +TT | -88.7 +TT |  |
|  | 60 |  | -92.3 +TT | -90.2 +TT | -89.0 +TT |  |
| n26 | 15 | -95.8 +TT | -92.6 +TT | -90.8 +TT | -89.5 +TT | HD-FDD |
|  | 30 |  | -93.0 +TT | -91.0 +TT | -89.7 +TT |  |
| n28 | 15 | -96.8 +TT | -93.6 +TT | -91.8 +TT | -90.5 +TT | HD-FDD |
|  | 30 |  | -94.0 +TT | -92.0 +TT | -90.7 +TT |  |
| n30 | 15 | -97.0 +TT | -93.8 +TT |  |  | HD-FDD |
|  | 30 |  | -94.2 +TT |  |  |  |
| n65 | 15 | -97.5 +TT | -94.3 +TT | -92.5 +TT | -91.2 +TT | HD-FDD |
|  | 30 |  | -94.7 +TT | -92.7 +TT | -91.4 +TT |  |
|  | 60 |  | -95.0 +TT | -92.9 +TT | -91.7 +TT |  |
| n66 | 15 | -97.5 +TT | -94.3 +TT | -92.5 +TT | -91.2 +TT | HD-FDD |
|  | 30 |  | -94.7 +TT | -92.7 +TT | -91.4 +TT |  |
|  | 60 |  | -95.0 +TT | -92.9 +TT | -91.7 +TT |  |
| n70 | 15 | -97.5 +TT | -94.3 +TT | -92.5 +TT | -91.2 +TT | HD-FDD |
|  | 30 |  | -94.7 +TT | -92.7 +TT | -91.4 +TT |  |
|  | 60 |  | -95.0 +TT | -92.9 +TT | -91.7 +TT |  |
| n71 | 15 | -95.5 +TT | -92.3 +TT | -90.5 +TT | -89.2 +TT | HD-FDD |
|  | 30 |  | -92.7 +TT | -90.7 +TT | -89.4 +TT |  |
| n74 | 15 | -97.5 +TT | -94.3 +TT | -92.5 +TT | -91.2 +TT | HD-FDD |
|  | 30 |  | -94.7 +TT | -92.7 +TT | -91.4 +TT |  |
|  | 60 |  | -95.0 +TT | -92.9 +TT | -91.7 |  |
| n85 | 15 | -95.3 +TT | -92.1 +TT | -90.3 +TT |  | HD-FDD |
|  | 30 |  | -92.5 +TT | -90.5 +TT |  |  |
| n91 | 15 | -97.5 +TT |  |  |  | HD-FDD |
| n92 | 15 | -97.5 +TT | -94.3 +TT | -92.5 +TT | -91.2 +TT | HD-FDD |
|  | 30 |  | -94.7 +TT | -92.7 +TT | -91.4 +TT |  |
| n93 | 15 | -97.5 +TT |  |  |  | HD-FDD |
| n94 | 15 | -97.5 +TT | -94.3 +TT | -92.5 +TT | -91.2 +TT | HD-FDD |
|  | 30 |  | -94.7 +TT | -92.7 +TT | -91.4 +TT |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 7.3I.2.5-7. |  |  |  |  |  |  |

Table 7.3I.2.5-7: Test Tolerance (TT) for RX sensitivity level for RedCap UE

| f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0 GHz |
| --- | --- |
| 0.7 dB | 1.0 dB |

<End of Changes >