3GPP TSG-RAN5 Meeting #95-e	R5-223031r3

# Online, , 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.509 | CR | 0060 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test function to limit Pcell power |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Portugal |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This new conformance test function allows limiting PCell power during UL-CA test cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduces new test function to limit PCell Power |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UL-CA tests blocked from execution due to SCell Drop Issue |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.11, 6.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Resubmission of endorsed draftCR R5-221921 with Editors Note added |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1 includes a minor editorial change and correction to octet values to avoid overlap with other test functions. Also the CR impact tick box for ME is set. r2 modifies octet values to a more recognizable set. Some information element details are updated. CR CAT updated to A on cover page. r3 adds more clarity to Editor’s notes and incorporates generic formula for the power backoff; WIC updated on cover page, editorial corrections |  |  |  |  |  |  |  |  |

<Unchanged sections skipped>

## 5.11	UE Power Limit Function (UPLF)

Editor’s note:	The power limit request message values set by the UPLF test mode is currently applicable to equal PSD (equal channel bandwidths on all component carriers) only. Message values are pending for unequal channel bandwidths

### 5.11.1	General

The UE Power Limit Function is intended for the SS to communicate to the UE to apply a backoff of transmitted power on the NR primary component carrier when in FR2 carrier aggregation mode.  On activation of this test function, the UE shall apply a configured power backoff on the primary component carrier to provide sufficient power head room for the other (secondary) component carrier(s). The UE power limit function is mandatory for applicable UEs operating in Frequency Range 2 (FR2) and supporting UL CA from Rel-16.

The SS uses the UE Power Limit test mode activation procedure to send a power limit request to the UE. The transmission from the SS  shall be a vector of values, where the number of the reported values equals the back-off or power limit to be applied by the UE.

Figure 5.11.1-1:  Activate Power Limit test mode procedure

Figure 5.11.1-2:  Deactivate Power Limit test mode procedure

### 5.11.2	Activate Power Limit Procedure

### 5.11.2.1	Initiation

The SS requests the UE to apply the power limit by transmitting an ACTIVATE POWER LIMIT REQUEST message. The UE confirms that the power limit function is activated by sending an ACTIVATE POWER LIMIT RESPONSE.

#### 5.11.2.2	Reception of ACTIVATE POWER LIMIT REQUEST by UE

#### When the UE receives the ACTIVATE POWER LIMIT REQUEST message, then the UE shall:

1>	if the UE is operating in FR2 AND is in RRC_CONNECTED state:

2>	if POWER LIMIT REQUEST = 0010

3>	UE limits power on PCell corresponding to NR FR2 2CA

2>	elseif POWER LIMIT REQUEST = 0011

3>	UE limits power on PCell corresponding to NR FR2 3CA

2>	elseif POWER LIMIT REQUEST = 0100

3>	UE limits power on PCell corresponding to NR FR2 4CA

2>	elseif POWER LIMIT REQUEST = 0101

3>	UE limits power on PCell corresponding to NR FR2 5CA

2>	elseif POWER LIMIT REQUEST = 0110

3>	UE limits power on PCell corresponding to NR FR2 6CA

2>	elseif POWER LIMIT REQUEST = 0111

3>	UE limits power on PCell corresponding to NR FR2 7CA

2>	elseif POWER LIMIT REQUEST =1000

3>	UE limits power on PCell corresponding to NR FR2 8CA

2>	Transmit POWER LIMIT RESPONSE MESSAGE

1>else:

2>	the UE behavior is unspecified

The ACTIVATE POWER LIMIT REQUEST and ACTIVATE POWER LIMIT RESPONSE Message Octets are defined below in Section 6.11.

### 5.11.3	Deactivate Power Limit Procedure

#### 5.11.3.1	Initiation

The SS requests the UE to deactivate from the power limit state by transmitting a DEACTIVATE POWER LIMIT REQUEST message. The SS should do this when the UE is in RRC_CONNECTED state.

#### 5.11.3.2	Reception of DEACTIVATE POWER LIMIT REQUEST message by UE

When UE receives DEACTIVATE POWER LIMIT REQUEST message then the UE shall:

1>	if the UE is operating in FR2 AND is in RRC_CONNECTED state AND the UE Power Limit test function is active:

2>	Remove UE power limit on PCell and transmit DEACTIVATE POWER LIMIT RESPONSE message.

1> else:

2>	the UE behaviour is unspecified.

#### 5.11.3.3	Removal of power limits by UE

When the UE leaves the RRC_CONNECTED state, the UE shall:

1>	if the UE is operating in FR2 AND the UE Power limit test Function is active

2>	remove UE power limit on PCell;

<Unchanged sections skipped>

## 6.11	Power Limit messages

### 6.11.1	ACTIVATE POWER LIMIT REQUEST

The values sent the SS as part of this test function maps to the values defined in Table 6.11.1-1.

For intra-band contiguous FR2 UL-CA scenarios where it is critical to prevent drop of the secondary component carrier(s) due to priorization rules in section 7.5 of TS 38.213, a parameter Xmax,f,PCell  is defined for the Pcell such that

Xmax,i,PCell  = 10 log10(i) for i=1…n component carriers

Xmax,i,PCell  is the back-off in transmit power applied on the primary component carrier fom PCMAX,f,c. This ensure sufficient power allocation is possible on the secondary component carrier(s)

Table 6.11.1-1: Mapping of Xmax,i,PCell  to number of NR FR2 Component Carriers

| Number of FR2 component carriers | POWER LIMIT REQUEST value | Xmax,i,PCell (dB) |
| --- | --- | --- |
| 2 | 0 0 1 0 | 3.0103 |
| 3 | 0 0 1 1 | 4.771213 |
| 4 | 0 1 0 0 | 6.0206 |
| 5 | 0 1 0 1 | 6.9897 |
| 6 | 0 1 1 0 | 7.781513 |
| 7 | 0 1 1 1 | 8.45098 |
| 8 | 1 0 0 0 | 9.0309 |

This message is only sent in the direction SS to UE.

| Information Element | Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- |
| Protocol discriminator | TS 24.007 [7], subclause 11.2.3.1.1 | M | V | ½ |
| Skip indicator | TS 24.007 [7], subclause 11.2.3.1.2 | M | V | ½ |
| Message type |  | M | V | 1 |
| Power Limit Request |  | M | V | 1 |

where message type is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | octet 1 |

where POWER LIMIT REQUEST is a single octet IE:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | X1 | X2 | X3 | X4 | octet 1 |

The Power Limit Request values are as below:

X1, X2, X3, X4 = 0010 for NR FR2 2CA

X1, X2, X3, X4 = 0011 for NR FR2 3CA

X1, X2, X3, X4 = 0100 for NR FR2 4CA

X1, X2, X3, X4 = 0101 for NR FR2 5CA

X1, X2, X3, X4 = 0110 for NR FR2 6CA

X1, X2, X3, X4 = 0111 for NR FR2 7CA

X1, X2, X3, X4 = 1000 for NR FR2 8CA

### 6.11.2	ACTIVATE POWER LIMIT RESPONSE

This message is only sent in the direction UE to SS.

| Information Element | Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- |
| Protocol discriminator | TS 24.007 [5], sub clause 11.2.3.1.1 | M | V | ½ |
| Skip indicator | TS 24.007 [5], sub clause 11.2.3.1.2 | M | V | ½ |
| Message Type |  | M | V | 1 |

where message type is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 0 | 1 | 1 | 0 | 1 | octet 1 |

### 6.11.3	DEACTIVATE POWER LIMIT REQUEST

This message is only sent in the direction SS to UE

| Information Element | Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- |
| Protocol discriminator | TS 24.007 [5], sub clause 11.2.3.1.1 | M | V | ½ |
| Skip indicator | TS 24.007 [5], sub clause 11.2.3.1.2 | M | V | ½ |
| Message Type |  | M | V | 1 |

where message type is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | octet 1 |

### 6.11.4	DEACTIVATE POWER LIMIT RESPONSE

This message is only sent in the direction UE to SS.

| Information Element | Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- |
| Protocol discriminator | TS 24.007 [5], sub clause 11.2.3.1.1 | M | V | ½ |
| Skip indicator | TS 24.007 [5], sub clause 11.2.3.1.2 | M | V | ½ |
| Message Type |  | M | V | 1 |

where message type is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | octet 1 |

<End of changes>