3GPP TSG-RAN5 Meeting #95-e	R5-223033r2

# Online, , 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2377 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add new messages and procedure for test function to limit Pcell Power |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Portugal |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduce the outline of necessary messages and procedures to allow limiting of PCell power via a conformance test function |  |  |  |  |  |  |  |  |
| Summary of change: |  | New test function messages added to limit PCell power in UL-CA test cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SCell drop in UL-CA tests due to expected power prioritization rules |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7A, 4.9.32, 4.9.33 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Resubmission of endorsed draftCR R5-221920 with Editor’s notes added. CR introduces message contents for UE Power Limit Function (UPLF) applicable to Release 16 and forward UEs. Therefore, it was allowed to use TEl16_Test WIC for this CR. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1 aligns the message values with R5-220032r3. Updates clause numbers to align with available clause numbers in the test spec r2 updates Other Comments section to explain WIC used on cover page. |  |  |  |  |  |  |  |  |

<Unchanged Sections skipped>

#### NSSAI DELETE RESPONSE

This message is only sent in the direction UE to SS, embedded in a RRC ULInformationTransfer message.

Table 4.7A.6-2: NSSAI DELETE RESPONSE

| Derivation Path: 38.509 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol discriminator | 1 1 1 1 |  |  |
| Skip indicator | 0 0 0 0 |  |  |
| Message type | 1 0 1 0 0 1 1 1 |  |  |

### 4.7A.7	UE Power Limit Messages

Editor’s note:	The power limit request message values set by the UPLF test mode is currently applicable to equal PSD (equal channel bandwidths on all component carriers) only. Message values are pending for unequal channel bandwidths

#### -	ACTIVATE POWER LIMIT REQUEST

This Message is only sent in the direction SS to UE, embedded in a RRC DLInformationTransfer message.

- Table 4.7A.7-1: ACTIVATE POWER LIMIT REQUEST

| Derivation Path: 38.509 clause 6.11.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol discriminator | 1 1 1 1 |  |  |
| Skip indicator | 0 0 0 0 |  |  |
| Message type | 1 0 1 0 0 1 1 1 |  |  |
| POWER LIMIT REQUEST | 1 0 1 0 0 0 1 0 |  | NR FR2 2CA |
| POWER LIMIT REQUEST | 1 0 1 0 0 0 1 1 |  | NR FR2 3CA |
| POWER LIMIT REQUEST | 1 0 1 0 0 1 0 0 |  | NR FR2 4CA |
| POWER LIMIT REQUEST | 1 0 1 0 0 1 0 1 |  | NR FR2 5CA |
| POWER LIMIT REQUEST | 1 0 1 0 0 1 1 0 |  | NR FR2 6CA |
| POWER LIMIT REQUEST | 1 0 1 0 0 1 1 1 |  | NR FR2 7CA |
| POWER LIMIT REQUEST | 1 0 1 0 1 0 0 0 |  | NR FR2 8CA |

| Condition | Explanation |
| --- | --- |
| NR FR2 2CA | Power Limit Request corresponding to NR FR2 2CA only |
| NR FR2 3CA | Power Limit Request corresponding to NR FR2 3CA only |
| NR FR2 4CA | Power Limit Request corresponding to NR FR2 4CA only |
| NR FR2 5CA | Power Limit Request corresponding to NR FR2 5CA only |
| NR FR2 6CA | Power Limit Request corresponding to NR FR2 6CA only |
| NR FR2 7CA | Power Limit Request corresponding to NR FR2 7CA only |
| NR FR2 8CA | Power Limit Request corresponding to NR FR2 8CA only |

#### ACTIVATE POWER LIMIT RESPONSE

This message is only sent in the direction UE to SS, embedded in a RRC ULInformationTransfer message.

- Table 4.7A.7-2: ACTIVATE POWER LIMIT RESPONSE

| Derivation Path: 38.509 clause 6.11.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol discriminator | 1 1 1 1 |  |  |
| Skip indicator | 0 0 0 0 |  |  |
| Message type | 1 0 1 0 1 1 0 1 |  |  |

#### DEACTIVATE POWER LIMIT REQUEST

This message is only sent in the direction SS to UE, embedded in a RRC DLInformationTransfer message.

- Table 4.7A.7-3: DEACTIVATE POWER LIMIT REQUEST

| Derivation Path: 38.509 clause 6.11.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol discriminator | 1 1 1 1 |  |  |
| Skip indicator | 0 0 0 0 |  |  |
| Message type | 1 0 1 0 1 1 1 0 |  |  |

#### DEACTIVATE POWER LIMIT RESPONSE

This message is only sent in the direction UE to SS, embedded in a RRC ULInformationTransfer message.

- Table 4.7A.7-3: DEACTIVATE POWER LIMIT RESPONSE

| Derivation Path: 38.509 clause 6.11.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol discriminator | 1 1 1 1 |  |  |
| Skip indicator | 0 0 0 0 |  |  |
| Message type | 1 0 1 0 1 1 1 1 |  |  |

<Unchanged sections skipped>

### 4.9.32	Test procedure to activate UE Power Limit Function (UPLF)

Editor’s note:	The power limit request message values set by the UPLF test mode is currently applicable to equal PSD (equal channel bandwidths on all component carriers) only. Message values are pending for unequal channel bandwidths

4.9.32.1	Initiation

UE is operating in FR2 in RRC_CONNECTED state with UE power limit test function activated.

4.9.32.2	Procedure

Table 4.9.32.2-1: Test procedure Sequence

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| 1 | SS request UE to activate UE Power Limit Function | <-- | ACTIVATE POWER LIMIT REQUEST | - | - |
| 2 | UE confirms that UE Power Limit Function is activated | --> | ACTIVATE POWER LIMIT RESPONSE | - | - |

4.9.32.3	Specific Message contents

- Table 4.9.32.3-1: ACTIVATE POWER LIMIT REQUEST

| Derivation Path: 38.509 clause 6.11.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol discriminator | 1 1 1 1 |  |  |
| Skip indicator | 0 0 0 0 |  |  |
| Message type | 1 0 1 0 0 0 0 0 |  |  |
| POWER LIMIT REQUEST | 1 0 1 0 0 0 0 0 |  | NR FR2 2CA |
| POWER LIMIT REQUEST | 1 0 1 0 0 0 0 1 |  | NR FR2 3CA |
| POWER LIMIT REQUEST | 1 0 1 0 0 0 1 0 |  | NR FR2 4CA |
| POWER LIMIT REQUEST | 1 0 1 0 0 0 1 1 |  | NR FR2 5CA |
| POWER LIMIT REQUEST | 1 0 1 0 0 1 0 0 |  | NR FR2 6CA |
| POWER LIMIT REQUEST | 1 0 1 0 0 1 0 1 |  | NR FR2 7CA |
| POWER LIMIT REQUEST | 1 0 1 0 0 1 1 0 |  | NR FR2 8CA |

| Condition | Explanation |
| --- | --- |
| NR FR2 2CA | Power Limit Request corresponding to NR FR2 2CA only |
| NR FR2 3CA | Power Limit Request corresponding to NR FR2 3CA only |
| NR FR2 4CA | Power Limit Request corresponding to NR FR2 4CA only |
| NR FR2 5CA | Power Limit Request corresponding to NR FR2 5CA only |
| NR FR2 6CA | Power Limit Request corresponding to NR FR2 6CA only |
| NR FR2 7CA | Power Limit Request corresponding to NR FR2 7CA only |
| NR FR2 8CA | Power Limit Request corresponding to NR FR2 8CA only |

- Table 4.9.32.3-2: ACTIVATE POWER LIMIT RESPONSE

| Derivation Path: 38.509 clause 6.11.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | 1 1 1 1 |  |  |
| Skip indicator |  | 0 0 0 0 |  |  |
| Message type |  | 1 0 1 0 1 1 0 1 |  |  |

### 4.9.33	Test procedure to deactivate UE Power Limit Function (UPLF)

Editor’s note:	The power limit request message values set by the UPLF test mode is currently applicable to equal PSD (equal channel bandwidths on all component carriers) only. Message values are pending for unequal channel bandwidths

4.9.33.1	Initiation

UE is operating in FR2 in RRC_CONNECTED state.

4.9.33.2	Procedure

Table 4.9.33.2-1: Test procedure Sequence

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| 1 | SS request UE to deactivate UE Power Limit Function | <-- | DEACTIVATE POWER LIMIT REQUEST | - | - |
| 2 | UE confirms that UE Power Limit Function is deactivated | --> | DEACTIVATE POWER LIMIT RESPONSE | - | - |

4.9.33.3	Specific Message contents

Table 4.9.33.3-1: DEACTIVATE POWER LIMIT REQUEST

| Derivation Path: 38.509 clause 6.11.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol discriminator | 1 1 1 1 |  |  |
| Skip indicator | 0 0 0 0 |  |  |
| Message type | 1 0 1 0 1 1 1 0 |  |  |

Table 4.9.33.3-2: DEACTIVATE POWER LIMIT RESPONSE

| Derivation Path: 38.509 clause 6.11.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol discriminator | 1 1 1 1 |  |  |
| Skip indicator | 0 0 0 0 |  |  |
| Message type | 1 0 1 0 1 1 1 1 |  |  |

<End of changes>