3GPP TSG RAN Meeting #95-e	                          R5-223040

Electronic Meeting, 9th May – 22nd May 2022

Agenda item:	5.3.39.1

Source:   Apple

Title:       Inputs to RF AP#94e.22

Document for:	DISCUSSION AND ENDORSEMENT

# 1	Introduction

RAN5 has started work on adopting the outcome of TR 38.884 study item into RAN5 test specifications. One of the key category of tests is those impacted by the High DL power and Low UL power issue. In this context, an Action Point was proposed dat RAN5#95

This paper proposed to provide an input to the above action point.

# 2	Discussion

## 2.1     Background

Current FR2 test methods are IFF (CATR) based. Overcoming testability issues (TE unable to set signal levels due to previous hardware limitations) has two options

- State-of-the-art enhancements to existing test methodology (Indirect Far Field | CATR)

- New test methodology (Near Field based test methods/systems proposed in TR 38.884)

As recommended by RAN4 and also agreed in RAN5, the approach in RAN5 is to analyze any and all improvements/enhancements to existing permitted test methodologies. If there is no feasible way to overcome the testability issues using enhancements to permitted test methods, thereafter based on analysis and comparison, the new test methodologies can be looked at for adoption

Specific to the Receiver Spurious emissions test, the low UL power issue has been well demonstrated.

The below figure shows the core requirements

The table below shows the status in RAN5 test specifications where significant relaxation si required in order to test these requirements for Receiver spurious emissions

## 2.2 Observations on the action point

Observation 1: FR2 Tx Spurious emissions are testable without any core requirement relaxation. On the other hand the Additional spurious emissions test require core requirement relaxation.

Proposal 1: Correct the action point to refer to FR2 Additional Tx Spurious Emissions. The updated Action Point would be

With reference to the question raised in the action point, we would like to present two views

Observation 2: From a theoretical standpoint, it is known that coupling or leakage within the device hardware and antenna panels can cause the spurs to be emanating from an antenna panel that is not corresponding to the one which generates the in-band beam peak.

Observation 3: In general, any spurs that originate in digital domain can be picked up by any antenna panel coupled to the same.

Proposal 2: Consider the input that it is not possible to assume that the origin of the additional Tx Spurious/receive spurious emissions (regardless of frequency) is always collocated within the antenna panel that generates the in-band beam peak

# 3	Summary

Below are the observations and propoals in this discussion paper

Observation 1: FR2 Tx Spurious emissions are testable without any core requirement relaxation. On the other hand, the Additional spurious emissions test require core requirement relaxation.

Proposal 1: Correct the action point to refer to FR2 Additional Tx Spurious Emissions. The updated Action Point would be

Observation 2: From a theoretical standpoint, it is known that coupling or leakage within the device hardware and antenna panels can cause the spurs to be emanating from an antenna panel that is not corresponding to the one which generates the in-band beam peak.

Observation 3: In general, any spurs that originate in digital domain can be picked up by any antenna panel coupled to the same.

Proposal 2: Consider the input that it is not possible to assume that the origin of the additional Tx Spurious/receive spurious emissions (regardless of frequency) is always collocated within the antenna panel that generates the in-band beam peak

# 4	References

TS 38.521-2, FR2 RF Test Specification