3GPP TSG-RAN5 Meeting #96-e	R5-223940

# Electronic Meeting, 15th - 26th August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5102 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to NB-IoT RRC TC 22.4.14a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, NB_IOTenh2-UEConTest |  |  |  |  |  | Date: |  |  | 2022-07-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Definition of the SystemInformationBlockType22-NB is not required neither by the test purpose nor by conformance requirements and can be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To reduce complexity removed reference to SystemInformationBlockType22-NB in Pre-test conditions clause and relevant message contents in tables Table 22.4.14a.3.3-1 and Table 22.4.14a.3.3-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary configuration will remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.4.14a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

### 22.4.14a	NB-IoT / RRC Connection Establishment / Multi-Carrier/ Mixed Operation Mode

#### 22.4.14a.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {
  when { UE receives an RRCConnectionSetup-NB message including IE CarrierConfigDedicated }

then { UE transmits an RRCConnectionSetupComplete-NB message }

}

(2)

with { UE has received an RRCConnectionSetup-NB message including IE CarrierConfigDedicated }

ensure that {

when { UE continues with initial registration }

then { RRC signalling happens on non-anchor carrier }

}

#### 22.4.14a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.10.6.

[TS 36.331, clause 5.3.10.6]

...

For NB-IoT, the UE shall:

1>	if the carrierConfigDedicated is not included in the received physicalConfigDedicated:

2>	if the UE is configured with a carrier configuration previously received in carrierConfigDedicated:

3>	use the carrier configuration received in carrierConfigDedicated;

2>	else:

3>	use the carrier configuration received for the anchor carrier in the system information;

1>	else:

2>	use the carrier configuration received in carrierConfigDedicated;

2>	start to use the new carrier immediately after the last transport block carrying the RRC message has been acknowledged by the MAC layer, and any subsequent RRC response message sent for the current RRC procedure is therefore sent on the new carrier;

1>	reconfigure the physical channel configuration in accordance with the received physicalConfigDedicated.

#### 22.4.14a.3	Test description

##### 22.4.14a.3.1	Pre-test conditions

System Simulator:

-	Ncell 1.

-	NB-IoT Operation Mode set to standalone.

-	System information combination 1 as defined in TS 36.508[18] clause 8.1.4.3.1.1.

UE:

None.

Preamble:

-	UE is in State 1-NB switched off.

##### 22.4.14a.3.2	Test procedure sequence

Table 22.4.14a.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-2 | Steps 1-2 expected sequence defined in TS 36.508 Table 8.1.5.2.3-1 NB-IoT UE connected mode procedure (state 1-NB to state 2-NB). | - | - | - | - |
| 3 | The SS transmits an RRCConnectionSetup-NB message including IE CarrierConfigDedicated. | <-- | RRCConnectionSetup-NB | - | - |
| 4 | Check: Does the UE transmit an RRCConnectionSetupComplete-NB message to confirm the successful completion of the connection establishment on non-anchor carrier? | --> | RRCConnectionSetupComplete-NB | 1, 2 | P |
| 5-16 | Check: Does the UE complete initial registration by performing steps 4-14 of TS 36.508 Table 8.1.5.2.3-1 NB-IoT UE connected mode procedure (state 1-NB to state 2-NB) on the non-anchor carrier. | - | - | 2 | P |

##### 22.4.14a.3.3	Specific message contents

Table 22.4.14a.3.3-1: Void

Table 22.4.14a.3.3-2: Void

Table 22.4.14a.3.3-3: RRCConnectionSetup-NB (step 3, Table 22.4.14a.3.2-1)

| Derivation Path: 36.508 table 8.1.6.1-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionSetup-NB ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionSetup-r13 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated-r13 | RadioResourceConfigDedicated-NB-SRB |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.4.14a.3.3-4: RadioResourceConfigDedicated-NB-SRB (Table 22.4.14a.3.3-3)

| Derivation Path: 36.508 table 8.1.6.3-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-NB-SRB ::= SEQUENCE { |  |  |  |  |
| physicalConfigDedicated-r13 |  | PhysicalConfigDedicated-NB-DEFAULT |  |  |
| } |  |  |  |  |

Table 22.4.14a.3.3-5: PhysicalConfigDedicated-NB-DEFAULT (Table 22.4.14a.3.3-4)

| Derivation Path: 36.508 table 8.1.8.2.1.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| carrierConfigDedicated-r13 | CarrierConfigDedicated-NB | Non-anchor carrier |  |
| } |  |  |  |

Table 22.4.14a.3.3-6: CarrierConfigDedicated-NB (Table 22.4.14a.3.3-5)

| Derivation Path: 36.331 clause 6.7.3.2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| CarrierConfigDedicated-NB-r13 ::= SEQUENCE { |  |  |  |  |  |  |  |
| dl-CarrierConfig-r13 SEQUENCE { |  |  |  |  |  |  |  |
| dl-CarrierFreq-r13 SEQUENCE { |  |  |  |  |  |  |  |
| carrierFreq-r13 |  | Downlink EARFCN for non-anchor carrier under test. For Signalling test cases, see 36.508 clauses 8.1.3.1. Use the guard-band test frequencies. |  |  |  |  |  |
| carrierFreqOffset-r13 |  | v-0dot5 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| downlinkBitmapNonAnchor-r13 CHOICE { |  |  |  |  |  |  |  |
| useNoBitmap-r13 |  | NULL |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| dl-GapNonAnchor-r13 CHOICE { |  |  |  |  |  |  |  |
| useNoGap-r13 |  | NULL |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| inbandCarrierInfo-r13 SEQUENCE {} |  | Not present |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| ul-CarrierConfig-r13 |  | Not present |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

[End of change]