3GPP TSG-RAN5 Meeting #96-e	R5-223946

# Electronic Meeting, 15th – 26th August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0267 | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of clause 5.5.3.1 - SDP Message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2022-07-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - When condition INITIAL_SDP_OFFER is specified in the specific message content of an SDP message there shall be no need to specify explicitly condition SDP_OFFER but SDP_OFFER shall be applied implicitly - In most cases conditions FIRST_SDP_FROM_UE/SS are not specified for specific SDP message content being the first SDP message of the session. On the other hand in general this can be handled implicitly by the test implementation so that there is no need for explicit specification - In Table 5.5.3.1.2-3 (SDP Message from the SS for MCData) there is no need to specify an optional c= line at session level when there are c= for all media descriptions; NOTE: In TTCN there is no such c= line already. - Editorial issues |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarifications added to Table 5.5.3.1.0-1 that INITIAL_SDP_OFFER implies SDP_OFFER - Clarifications added to Table 5.5.3.1.0-1 that FIRST_SDP_FROM_UE/SS are determined implicitly and therefore in general do not need to be specified explicitly - c= line at session level removed from Table 5.5.3.1.2-3. - Editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect or inconsistent test specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | No TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.5.3.1	SDP Message

##### 5.5.3.1.0	Common conditions for SDP Message

The following conditions apply throughout clause 5.5.3.1:

Table 5.5.3.1.0-1: Conditions

| Condition | Explanation |
| --- | --- |
| INITIAL_SDP_OFFER | SDP message is an initial offer |
| SDP_OFFER | SDP message is an offer; INITIAL_SDP_OFFER implies SDP_OFFER, i.e. when a test case or test procedure specifies INITIAL_SDP_OFFER then SDP_OFFER shall be applied too, even when not explicitly specified. |
| SDP_ANSWER | SDP message is an Answer |
| FIRST_SDP_FROM_UE | First SDP message sent by the UE within the session; FIRST_SDP_FROM_UE shall be applied implicitly for an SDP message sent by the UE when the SDP message is the first SDP message sent by the UE for a session. In general FIRST_SDP_FROM_UE does not need to be specified for a specific message content. |
| FIRST_SDP_FROM_SS | First SDP message sent by the SS within the session; FIRST_SDP_FROM_SS shall be applied implicitly for an SDP message sent by the SS when the SDP message is the first SDP message sent by the UE for a session. In general FIRST_SDP_FROM_SS does not need to be specified for a specific message content; nevertheless FIRST_SDP_FROM_SS may be specified for a specific message content when the SDP message is for a new session (e.g. when a new dialog replaces a pre-established session) |
| IMPLICIT_GRANT_REQUESTED | An implicit grant is requested by the user |
| IMPLICIT_FLOOR_GRANTED | An implicit grant shall be granted by the SS |
| PRE_ESTABLISHED_SESSION | SDP message during establishment or modification of a pre-established session NOTE: The condition shall be applied for all SDP messages of preestablished session test cases and it is not explicitly mentioned in specific message content for these test cases |
| WITHOUT_FLOORCONTROL | SDP message for MCPTT call without floor control: In general when this condition is applied for an on-demand call the SDP message does not contain a media description for media plane control at all, whereas for call establishment using pre-established session the SDP message still contains a media description for media plane control but without any floor control related fmtp attributes (see TS 24.380 [10] clauses 14.2.6 and 14.3.7). |
| WITHOUT_TRANSMISSIONCONTROL | SDP message for MCVideo call without transmission control Editor's note: In contrast to MCPTT there is no "mc_no_floor_ctrl" (or similar) fmtp parameter for MCVideo yet. |
| WITHOUT_SECURITY | In case of private call: SDP message shall not contain any "a=key-mgmt" attribute for end-to-end security |
| WITH_SECURITY | End-to-end security to be applied independent from other conditions like PRIVATE-CALL, SDP_OFFER (e.g. for first-to-answer call) |
| SDS_SESSION | SDP message for establishment of an SDS session according to TS 24.282 [87] clause 9.2.4. |

##### 5.5.3.1.1	SDP Message from the UE

##### -	MCPTT

Table 5.5.3.1.1-1: SDP Message from the UE for MCPTT

| Derivation Path: RFC 4566 [27] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Session description: |  |  |  |  |
| Protocol Version | "0" | v= line |  |  |
| Origin | Same o=line as in the previous SDP message sent by the UE except that sess-version is incremented by one | o= line |  |  |
| Origin |  | o= line |  | FIRST_SDP_FROM_UE |
| username | any allowed value |  |  |  |
| sess-id | any allowed value | A numeric string such that the tuple of <username>, <sess-id>, <nettype>, <addrtype>, and <unicast-address> forms a globally unique identifier for the session |  |  |
| sess-version | any allowed value |  |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address |  |  |  |
| unicast-address | IP address of the UE | IP address assigned at initial registration |  |  |
| Session Name | at least one UTF-8-encoded character, or if no name is given, a single empty space | s= line |  |  |
| Connection Data | not required if included in all media | c= line |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address |  |  |  |
| connection-address | IP address of the UE |  |  |  |
| Bandwidth |  | b= line |  |  |
| "AS" | any allowed value |  | TS 26.114 [64] Table K.6 |  |
| Time description |  |  |  |  |
| Timing |  | t= line |  |  |
| start-time | "0" |  |  |  |
| stop-time | "0" |  |  |  |
| Session attribute | present only if there is no key-mgmt media attribute in the media description for audio | a= line attribute = key-mgmt (NOTE 2) |  | WITH_SECURITY OR (PRIVATE-CALL AND SDP_OFFER AND NOT WITHOUT_SECURITY) |
| key-mgmt |  |  | TS 24.379 [9] clause 6.2.1 |  |
| mikey | MIKEY-SAKKE I_MESSAGE as specified in Table 5.5.9.1-2A |  | RFC 4567 [44] |  |
| Session attribute | optional (NOTE 3) | a=line attribute=”ice-lite” | RFC 5245 [115] | PRE_ESTABLISHED_SESSION |
| ice-lite |  |  |  |  |
| Media description[1] |  | Media description for audio |  |  |
| media description |  | m= line media = audio | RFC 4867 [59] |  |
| media | "audio" |  |  |  |
| port | any allowed value | The transport port to which the media stream is sent |  |  |
| proto | "RTP/SAVP" |  |  |  |
| fmt | any allowed value(s) | Indicating RTP payload type numbers |  |  |
| media title | "speech" | i= line |  |  |
| Connection Data | present if session description does not contain a c=line; optional otherwise | c= line |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address" |  |  |  |
| connection-address | IP address of the UE |  |  |  |
| Bandwidth |  | b= line |  |  |
| "AS" | any allowed value |  | TS 26.114 [64] Table K.6 |  |
| "RS" | any allowed value if present |  | RFC 3556 [113] |  |
| "RR" | any allowed value if present |  | RFC 3556 [113] |  |
| media attribute |  | a= line attribute = rtpmap |  |  |
| rtpmap | "rtpmap" |  |  |  |
| payload type | same value as format parameter of the "fmtp" attribute |  |  |  |
| encoding name | "AMR-WB" |  |  |  |
| clock rate | 16000 |  | RFC 4867 [59] clause 8.3 |  |
| encoding parameter | "1" if present | Channel number |  |  |
| media attribute |  | a= line attribute = fmtp |  |  |
| fmtp | "fmtp" |  |  |  |
| format | a value given in fmt in the audio media description |  |  |  |
| format specific parameters |  | Parameters of WB-AMR codec NOTE: In addition to the parameters below the UE may provide further parameters |  |  |
| mode-change-capability | "2" | To be able to interoperate fully with gateways to circuit switched networks | RFC 4867 [59] clause 8.2 |  |
| max-red | "0" | No redundancy will be used | RFC 4867 [59] clause 8.2 |  |
| media attribute |  | a= line attribute =ptime |  |  |
| ptime | any allowed value | packet time |  |  |
| media attribute |  | a= line attribute =maxptime |  |  |
| maxptime | any allowed value | maximum packet time |  |  |
| media attribute | optional | a= line attribute =sendrecv Indicates send and receive mode being activated |  |  |
| sendrecv |  | Parameter has no value |  |  |
| media attribute | one or several attribute lines if present | a=line attribute=ssrc | RFC 5576 [116] |  |
| ssrc |  |  |  |  |
| ssrc-id | any allowed value but all the same if there is more than one ssrc attribute for audio |  |  |  |
| attribute | any source attribute according to RFC 5576 [116] (NOTE 1) |  |  |  |
| media attribute |  | a=line attribute=”candidate” | RFC 5245 [115] | PRE_ESTABLISHED_SESSION |
| candidate |  | candidate for RTP |  |  |
| foundation | any value |  |  |  |
| component-id | 1 | according to RFC 5245 [115] clause 4.1.1.1 |  |  |
| transport | "UDP" |  |  |  |
| priority | any value |  |  |  |
| connection-address | same IP address as in speech media's c= line or in the session's c= line if the speech media does not have a c= line | default candidate |  |  |
| port | same port number as in the m= line for speech |  |  |  |
| cand-type | "host" |  |  |  |
| media attribute |  | a=line attribute=”candidate” | RFC 5245 [115] | PRE_ESTABLISHED_SESSION |
| candidate |  | candidate for RTCP |  |  |
| foundation | any value |  |  |  |
| component-id | 2 | according to RFC 5245 [115] clause 4.1.1.1 |  |  |
| transport | "UDP" |  |  |  |
| priority | any value |  |  |  |
| connection-address | same IP address as in speech media's c= line or in the session's c= line if the speech media does not have a c= line | default candidate |  |  |
| port | same port number as in the m= line for speech incremented by 1 |  |  |  |
| cand-type | "host" |  |  |  |
| media attribute | present only if there is no key-mgmt attribute at session level | a= line attribute = key-mgmt |  | WITH_SECURITY OR (PRIVATE-CALL AND SDP_OFFER AND NOT WITHOUT_SECURITY) |
| key-mgmt |  |  | TS 24.379 [9] clause 6.2.1 |  |
| mikey | MIKEY-SAKKE I_MESSAGE as specified in Table 5.5.9.1-2A |  | RFC 4567 [44] |  |
| Media description[2] |  | Media description for media control |  | NOT WITHOUT_FLOORCONTROL OR PRE_ESTABLISHED_SESSION |
| media description |  | m= line media = application SDP media-level section for a media-control entity (NOTE 2) |  |  |
| media | "application" |  |  |  |
| port | any allowed value | The port for the media-control entity |  |  |
| proto | "udp" |  |  |  |
| fmt | "MCPTT" |  |  |  |
| Connection Data | present if session description does not contain a c=line; optional otherwise | c= line |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address" |  |  |  |
| connection-address | IP address of the UE |  |  |  |
| media attribute |  | a= line attribute = fmtp |  |  |
| fmtp |  |  |  |  |
| format | "MCPTT" |  |  |  |
| format specific parameters |  |  |  | SDP_OFFER AND NOT WITHOUT_FLOORCONTROL |
| mc_queueing | not present |  | TS 24.380 [10] cl. 12.1.2.3 |  |
|  | present | Parameter has no value |  | pc_MCPTT_FloorRequestQueueing |
| mc_priority | any allowed value | Any integer value in the range of 1..255 | TS 24.380 [10] cl. 12.1.2.3 |  |
| mc_granted | not present |  |  |  |
|  | present | Parameter has no value | TS 24.380 [10] cl. 12.1.2.3 | INITIAL_SDP_OFFER |
| mc_implicit_request | not present |  |  |  |
|  | present | Parameter has no value | TS 24.380 [10] cl. 12.1.2.3 | IMPLICIT_GRANT_REQUESTED |
| mc_ssrc | any value if present |  | TS 24.380 [10] cl. 12.1.2.3 |  |
| mc_no_floor_ctrl | not present |  | TS 24.380 [10] cl. 12.1.2.3 |  |
| format specific parameters |  |  |  | SDP_ANSWER AND NOT WITHOUT_FLOORCONTROL |
| mc_queueing | not present |  | TS 24.380 [10] cl. 12.1.2.3 |  |
|  | present | Parameter has no value |  | pc_MCPTT_FloorRequestQueueing |
| mc_priority | same value as in the offer |  | TS 24.380 [10] cl. 12.1.2.3 |  |
| mc_granted | not present |  | TS 24.380 [10] cl. 12.1.2.3 |  |
| mc_implicit_request | not present |  | TS 24.380 [10] cl. 12.1.2.3 |  |
| mc_ssrc | not present |  | TS 24.380 [10] cl. 12.1.2.3 |  |
| format specific parameters |  |  |  | WITHOUT_FLOORCONTROL |
| mc_queueing | not present |  | TS 24.380 [10] cl. 12.1.2.3 |  |
| mc_priority | not present |  | TS 24.380 [10] cl. 12.1.2.3 and cl. 14.3.3 |  |
| mc_granted | not present |  | TS 24.380 [10] cl. 12.1.2.3 |  |
| mc_implicit_request | not present |  | TS 24.380 [10] cl. 12.1.2.3 |  |
| mc_ssrc | not present |  | TS 24.380 [10] cl. 12.1.2.3 |  |
| mc_no_floor_ctrl | present | Parameter has no value | TS 24.380 [10] cl. 12.1.2.3 |  |
| media attribute |  | a=line attribute=”candidate” | RFC 5245 [115] | PRE_ESTABLISHED_SESSION |
| candidate |  | candidate for Media Control messages |  |  |
| foundation | any value |  |  |  |
| component-id | 1 | according to RFC 5245 [115] clause 4.1.1.1 |  |  |
| transport | "UDP" |  |  |  |
| priority | any value |  |  |  |
| connection-address | same IP address as in application media's c= line or in the session's c= line if the application media does not have a c= line | default candidate |  |  |
| port | same port number as in the m= line for application |  |  |  |
| cand-type | "host" |  |  |  |
| NOTE 1: If "ssrc" media attributes are included, then at least one "ssrc=" line shall contain a "cname" source attribute according to RFC 5576 [116] clause 6.1. NOTE 2: Even though there is no clarity in core specs it is assumed that a key-mgmt attribute at session level does not affect the media control security, i.e. the key-mgmt attribute is not applicable for the "application" media description for which still the CSK is used as security key. This is in contrast to RFC 4566 [27] clause 5 saying "In general, session-level values are the default for all media unless overridden by an equivalent media-level value." NOTE 3: If the UE is configured as lite implementation according to RFC 5245 [115], it shall include "a=ice-lite" session-level attribute; nevertheless this is not a test requirement unless specified otherwise in a test case. |  |  |  |  |

##### -	MCVideo

Table 5.5.3.1.1-2: SDP Message from the UE for MCVideo

| Derivation Path: RFC 4566 [27] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Session description: |  |  |  |  |
| Protocol Version | "0" | v= line |  |  |
| Origin | Same o=line as in the previous SDP message sent by the UE except that sess-version is incremented by one | o= line |  |  |
| Origin |  | o= line |  | FIRST_SDP_FROM_UE |
| username | any allowed value |  |  |  |
| sess-id | any allowed value | A numeric string such that the tuple of <username>, <sess-id>, <nettype>, <addrtype>, and <unicast-address> forms a globally unique identifier for the session. |  |  |
| sess-version | any allowed value |  |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address |  |  |  |
| unicast-address | IP address of the UE | IP address assigned at initial registration |  |  |
| Session Name | at least one UTF-8-encoded character, or if no name is given, a single empty space | s= line |  |  |
| Connection Data | not required if included in all media | c= line |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address |  |  |  |
| connection-address | IP address of the UE |  |  |  |
| Bandwidth |  | b= line |  |  |
| "AS" | any allowed value |  |  |  |
| Time description |  |  |  |  |
| Timing |  | t= line |  |  |
| start-time | "0" |  |  |  |
| stop-time | "0" |  |  |  |
| Session attribute | present only if there is no key-mgmt media attribute in the media descriptions for audio and video | a= line attribute = key-mgmt (NOTE 2) |  | WITH_SECURITY OR (PRIVATE-CALL AND SDP_OFFER AND NOT WITHOUT_SECURITY) |
| key-mgmt |  |  | TS 24.379 [9] clause 6.2.1 |  |
| mikey | MIKEY-SAKKE I_MESSAGE as specified in Table 5.5.9.1-2A |  | RFC 4567 [44] |  |
| Session attribute | optional (NOTE 3) | a=line attribute=”ice-lite” | RFC 5245 [115] | PRE_ESTABLISHED_SESSION |
| ice-lite |  |  |  |  |
| Media description[1] |  | Media description for audio |  |  |
| media description |  | m= line media = audio | RFC 4867 [59] |  |
| media | "audio" |  |  |  |
| port | any allowed value | The transport port to which the media stream is sent |  |  |
| proto | "RTP/SAVP" |  |  |  |
| fmt | any allowed value(s) | Indicating RTP payload type numbers |  |  |
| media title | "audio component of MCVideo" | i= line |  |  |
| Connection Data | present if session description does not contain a c=line; optional otherwise | c= line |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address" |  |  |  |
| connection-address | IP address of the UE |  |  |  |
| Bandwidth |  | b= line |  |  |
| "AS" | any allowed value |  |  |  |
| "RS" | any allowed value if present |  | RFC 3556 [113] |  |
| "RR" | any allowed value if present |  | RFC 3556 [113] |  |
| media attribute |  | a= line attribute = rtpmap |  |  |
| rtpmap | "rtpmap" |  |  |  |
| payload type | same value as format parameter of the "fmtp" attribute |  |  |  |
| encoding name | "AMR-WB" |  |  |  |
| clock rate | 16000 |  | RFC 4867 [59] clause 8.3 |  |
| encoding parameter | "1" if present | Channel number |  |  |
| media attribute |  | a= line attribute = fmtp |  |  |
| fmtp | "fmtp" |  |  |  |
| format | a value given in fmt in the audio media description |  |  |  |
| format specific parameters |  | Parameters of WB-AMR codec NOTE: In addition to the parameters below the UE may provide further parameters |  |  |
| mode-change-capability | "2" | To be able to interoperate fully with gateways to circuit switched networks | RFC 4867 [59] clause 8.2 |  |
| max-red | "0" | No redundancy will be used | RFC 4867 [59] clause 8.2 |  |
| media attribute |  | a= line attribute =ptime |  |  |
| ptime | any allowed value | packet time |  |  |
| media attribute |  | a= line attribute =maxptime |  |  |
| maxptime | any allowed value | maximum packet time |  |  |
| media attribute | optional | a= line attribute =sendrecv Indicates send and receive mode being activated |  |  |
| sendrecv |  | Parameter has no value |  |  |
| media attribute | one or several attribute lines if present | a=line attribute=ssrc | RFC 5576 [116] |  |
| ssrc |  |  |  |  |
| ssrc-id | any allowed value but all the same if there is more than one ssrc attribute for audio |  |  |  |
| attribute | any source attribute according to RFC 5576 [116] (NOTE 1) |  |  |  |
| media attribute |  | a=line attribute=”candidate” | RFC 5245 [115] | PRE_ESTABLISHED_SESSION |
| candidate |  | candidate for RTP |  |  |
| foundation | any value |  |  |  |
| component-id | 1 | according to RFC 5245 [115] clause 4.1.1.1 |  |  |
| transport | "UDP" |  |  |  |
| priority | any value |  |  |  |
| connection-address | same IP address as in audio media's c= line or in the session's c= line if the audio media does not have a c= line | default candidate |  |  |
| port | same port number as in the m= line for audio |  |  |  |
| cand-type | "host" |  |  |  |
| media attribute |  | a=line attribute=”candidate” | RFC 5245 [115] | PRE_ESTABLISHED_SESSION |
| candidate |  | candidate for RTCP |  |  |
| foundation | any value |  |  |  |
| component-id | 2 | according to RFC 5245 [115] clause 4.1.1.1 |  |  |
| transport | "UDP" |  |  |  |
| priority | any value |  |  |  |
| connection-address | same IP address as in audio media's c= line or in the session's c= line if the audio media does not have a c= line | default candidate |  |  |
| port | same port number as in the m= line for audio incremented by 1 |  |  |  |
| cand-type | "host" |  |  |  |
| media attribute | present only if there is no key-mgmt attribute at session level | a= line attribute = key-mgmt |  | WITH_SECURITY OR (PRIVATE-CALL AND SDP_OFFER AND NOT WITHOUT_SECURITY) |
| key-mgmt |  |  | TS 24.281 [86] clause 6.2.1 |  |
| mikey | MIKEY-SAKKE I_MESSAGE as specified in Table 5.5.9.1-2A (NOTE 4) | Use condition MCVIDEO | RFC 4567 [44] |  |
| Media description[2] |  | Media description for video |  |  |
| media description |  | m= line media = video SDP media-level section for a media-transmission control entity |  |  |
| media | "video" |  |  |  |
| port | any allowed value | The port for the media-transmission control entity |  |  |
| proto | "RTP/SAVPF" or "RTP/SAVP" |  |  |  |
| fmt | any allowed value(s) |  |  |  |
| media title | "video component of MCVideo" | i= line |  |  |
| Connection Data | present if session description does not contain a c=line; optional otherwise | c= line |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address" |  |  |  |
| connection-address | IP address of the UE |  |  |  |
| Bandwidth |  | b= line |  |  |
| "AS" | any allowed value |  |  |  |
| "RS" | any allowed value if present |  | RFC 3556 [113] |  |
| "RR" | any allowed value if present |  | RFC 3556 [113] |  |
| media attribute |  | a= line attribute = rtpmap |  |  |
| rtpmap | "rtpmap" |  |  |  |
| payload type | same value as format parameter of the "fmtp" attribute |  |  |  |
| encoding name | "H264" |  |  |  |
| clock rate | 90000 |  | RFC 4867 [59] clause 8.3 |  |
| media attribute |  | a= line attribute = fmtp |  |  |
| fmtp | "fmtp" |  |  |  |
| format | a value given in fmt in the audio media description |  |  |  |
| format specific parameters |  | Parameters of H264 codec NOTE: In addition to the parameters below the UE may provide further parameters | RFC 6184 [129] |  |
| profile-level-id | any allowed value |  |  |  |
| packetization-mode | 0 |  |  | SDP_ANSWER |
| media attribute | present if proto="RTP/AVP" in the m=line | a= line attribute = tcap | RFC 5939 [128] TS 26.114 [64] clause 6.2.1a.2 | SDP_OFFER |
| tcap |  |  |  |  |
| trpr-cap-num | 1 |  |  |  |
| proto-list | RTP/AVPF |  |  |  |
| media attribute | present if proto="RTP/AVP" in the m=line | a= line attribute = pcfg | RFC 5939 [128] TS 26.114 [64] clause 6.2.1a.2 | SDP_OFFER |
| pcfg |  |  |  |  |
| config-number | 1 |  |  |  |
| pot-cfg-list | t=1 |  |  |  |
| media attribute | one or several attribute lines if present | a=line attribute=ssrc | RFC 5576 [116] |  |
| ssrc |  |  |  |  |
| ssrc-id | any allowed value but all the same if there is more than one ssrc attribute for audio |  |  |  |
| attribute | any source attribute according to RFC 5576 [116] (NOTE 1) |  |  |  |
| media attribute |  | a=line attribute=”candidate” | RFC 5245 [115] | PRE_ESTABLISHED_SESSION |
| candidate |  | candidate for RTP |  |  |
| foundation | any value |  |  |  |
| component-id | 1 | according to RFC 5245 [115] clause 4.1.1.1 |  |  |
| transport | "UDP" |  |  |  |
| priority | any value |  |  |  |
| connection-address | same IP address as in video media's c= line or in the session's c= line if the video media does not have a c= line | default candidate |  |  |
| port | same port number as in the m= line for video |  |  |  |
| cand-type | "host" |  |  |  |
| media attribute |  | a=line attribute=”candidate” | RFC 5245 [115] | PRE_ESTABLISHED_SESSION |
| candidate |  | candidate for RTCP |  |  |
| foundation | any value |  |  |  |
| component-id | 2 | according to RFC 5245 [115] clause 4.1.1.1 |  |  |
| transport | "UDP" |  |  |  |
| priority | any value |  |  |  |
| connection-address | same IP address as in video media's c= line or in the session's c= line if the video media does not have a c= line | default candidate |  |  |
| port | same port number as in the m= line for video incremented by 1 |  |  |  |
| cand-type | "host" |  |  |  |
| media attribute | present only if there is no key-mgmt attribute at session level | a= line attribute = key-mgmt |  | WITH_SECURITY OR (PRIVATE-CALL AND SDP_OFFER AND NOT WITHOUT_SECURITY) |
| key-mgmt |  |  | TS 24.281 [86] clause 6.2.1 |  |
| mikey | MIKEY-SAKKE I_MESSAGE as specified in Table 5.5.9.1-2A (NOTE 4) | Use condition MCVIDEO | RFC 4567 [44] |  |
| Media description[3] |  | Media description for media control |  | NOT WITHOUT_TRANSMISSIONCONTROL OR PRE_ESTABLISHED_SESSION |
| media description |  | m= line media = application SDP media-level section for a media-control entity (NOTE 2) |  |  |
| media | "application" |  | 3GPP TS 24.581 [88] clause 12 |  |
| port | any allowed value | The port for the media-control entity |  |  |
| proto | "udp" |  |  |  |
| fmt | "MCVideo" |  |  |  |
| Connection Data | present if session description does not contain a c=line; optional otherwise | c= line |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address" |  |  |  |
| connection-address | IP address of the UE |  |  |  |
| media attribute |  | a= line attribute = fmtp |  |  |
| fmtp |  |  | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| format | "MCVideo" |  |  |  |
| format specific parameters |  |  |  | SDP_OFFER AND NOT WITHOUT_TRANSMISSIONCONTROL |
| mc_queueing | not present |  | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
|  | present | Parameter has no value. |  | pc_MCVideo_TransmissionRequestQueueing |
| mc_priority | any allowed value if present | Any integer value in the range of 1..255 Shall be present when priority other than the default priority is required | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| mc_reception_priority | any allowed value if present | Any integer value in the range of 0…255 Shall be present when priority other than the default reception priority is required | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| mc_granted | not present |  |  |  |
|  | present | Parameter has no value | 3GPP TS 24.581 [88] clause 12, clause 14 | INITIAL_SDP_OFFER |
| mc_implicit_request | not present |  |  |  |
|  | present | Parameter has no value | 3GPP TS 24.581 [88] clause 12, clause 14 | IMPLICIT_GRANT_REQUESTED |
| format specific parameters |  |  |  | SDP_ANSWER AND NOT WITHOUT_TRANSMISSIONCONTROL |
| mc_queueing | not present |  | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
|  | present | Parameter has no value |  | pc_MCVideo_TransmissionRequestQueueing |
| mc_priority | same value as in the SDP offer if present, not present otherwise |  | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| mc_reception_priority | same value as in the SDP offer if present, not present otherwise |  | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| mc_granted | not present |  | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| mc_implicit_request | not present |  | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| media attribute |  | a=line attribute=”candidate” | RFC 5245 [115] | PRE_ESTABLISHED_SESSION |
| candidate |  | candidate for Transmission Control Messages |  |  |
| foundation | any value |  |  |  |
| component-id | 1 | according to RFC 5245 [115] clause 4.1.1.1 |  |  |
| transport | "UDP" |  |  |  |
| priority | any value |  |  |  |
| connection-address | same IP address as in application media's c= line or in the session's c= line if the application media does not have a c= line | default candidate |  |  |
| port | same port number as in the m= line for application |  |  |  |
| cand-type | "host" |  |  |  |
| NOTE 1: If "ssrc" media attributes are included, then at least one "ssrc=" line shall contain a "cname" source attribute according to RFC 5576 [116] clause 6.1. NOTE 2: Even though there is no clarity in core specs it is assumed that a key-mgmt attribute at session level does not affect the media control security, i.e. the key-mgmt attribute is not applicable for the "application" media description for which still the CSK is used as security key. This is in contrast to RFC 4566 [27] clause 5 saying "In general, session-level values are the default for all media unless overridden by an equivalent media-level value." NOTE 3: If the UE is configured as lite implementation according to RFC 5245 [115], it shall include "a=ice-lite" session-level attribute; nevertheless this is not a test requirement unless specified otherwise in a test case. NOTE 4: If present the a=key-mgmt attributes for audio and video carry the same keys. |  |  |  |  |

##### -	MCData

Table 5.5.3.1.1-3: SDP Message from the UE for MCData

| Derivation Path: RFC 4566 [27] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Session description: |  |  |  |  |
| Protocol Version | "0" | v= line |  |  |
| Origin | Same o=line as in the previous SDP message sent by the UE except that sess-version is incremented by one | o= line |  |  |
| Origin |  | o= line |  | FIRST_SDP_FROM_UE |
| username | any allowed value |  |  |  |
| sess-id | any allowed value | A numeric string such that the tuple of <username>, <sess-id>, <nettype>, <addrtype>, and <unicast-address> forms a globally unique identifier for the session. |  |  |
| sess-version | any allowed value |  |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address" |  |  |  |
| unicast-address | IP address of the UE | IP address assigned at initial registration |  |  |
| Session Name | at least one UTF-8-encoded character, or if no name is given, a single empty space | s= line |  |  |
| Connection Data | not required if included in all media | c= line |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address" |  |  |  |
| connection-address | IP address of the UE |  |  |  |
| Time description |  |  |  |  |
| Timing |  | t= line |  |  |
| start-time | "0" |  |  |  |
| stop-time | "0" |  |  |  |
| Session attribute | present only if there is no key-mgmt media attribute in the media description for audio | a= line attribute = key-mgmt |  | SDP_OFFER AND MCD_1to1 |
| key-mgmt |  |  | TS 24.379 [9] clause 6.2.1 |  |
| mikey | MIKEY-SAKKE I_MESSAGE as specified in Table 5.5.9.1-2A |  | RFC 4567 [44] |  |
| Media description[1] |  | Media description for data |  |  |
| media description |  | m= line media = message | RFC 4867 [59] TS 24.282 [31] |  |
| media | "message" |  |  |  |
| port | any allowed value | The transport port to which the media stream is sent |  |  |
| proto | "TCP/MSRP" |  |  |  |
| fmt | “*” |  |  |  |
| Connection Data | present if session description does not contain a c=line; optional otherwise | c= line |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address" |  |  |  |
| connection-address | IP address of the UE |  |  |  |
| media attribute |  | a= line attribute = sendonly |  | SDP_OFFER AND NOT SDS_SESSION |
| sendonly |  | No parameters associated with this line |  |  |
| media attribute |  | a= line attribute = recvonly |  | SDP_ANSWER AND NOT SDS_SESSION |
| recvonly |  | No parameters associated with this line |  |  |
| media attribute |  | a= line attribute = sendrecv |  | SDS_SESSION |
| sendrecv |  | No parameters associated with this line |  |  |
| media attribute |  | a= line attribute = path |  |  |
| path | MSRP URI according to RFC 4975 [120] clause 6 and 9 | attribute containing its own MSRP URI. An example: msrp://mcdata.example.com:7654/abcde1; tcp | TS 24.282 [31] |  |
| scheme | "msrp" |  |  |  |
| authority |  |  | RFC 3986 [123] clause 3.2 |  |
| userinfo | any value if present |  |  |  |
| host | any allowed value | domain name or IP address of the UE |  |  |
| port | same value as in the media line if present | port at which the UE may be connected to for MSRP; mandatory when hostname is an IP address |  |  |
| session id | any allowed value if present |  |  |  |
| transport | "tcp" | mandatory for MSRP according to RFC 4975 [120] clause 6 |  |  |
| URI-parameter | not present |  |  |  |
| media attribute |  | a= line attribute = accept-types | RFC 4975 [120] |  |
| accept-types |  |  |  |  |
| format-entry[1] | "application/vnd.3gpp.mcdata-signalling" |  |  |  |
| format-entry[2] | "application/vnd.3gpp.mcdata-payload" |  |  | MCDATA_SDS |
| media attribute |  | a= line attribute = setup | RFC 4145 [119] |  |
| setup | “actpass” |  |  | SDP_OFFER |
|  | "active" or "passive" |  |  | SDP_ANSWER |
| media attribute |  | a= line attribute = file-transfer-id | RFC 5547 [124] | MCDATA_FD |
| file-transfer-id | any allowed value |  | RFC 5547 [124] clause 8.2.1 | SDP_OFFER |
|  | same value as in the sdp offer |  | RFC 5547 [124] clause 8.2.2 | SDP_ANSWER |
| media attribute |  | a= line attribute = file-selector | RFC 5547 [124] | MCDATA_FD |
| file-selector |  |  |  | SDP_OFFER |
| selector[1] |  |  |  |  |
| filename | any allowed value | e.g. "TestFile.txt" |  |  |
| filesize | size of the file to be transferred |  |  |  |
| filetype | any allowed value | e.g. "text/plain" |  |  |
| hash |  |  |  |  |
| algorithm | "sha-1" |  |  |  |
| value | hash value of the file to be transferred |  |  |  |
| file-selector | same value as in the sdp offer |  |  | SDP_ANSWER |
| media attribute |  | a= line attribute = file-date | RFC 5547 [124] | MCDATA_FD AND SDP_OFFER |
| file-date |  |  |  |  |
| date-param | at least one entry with an allowed value |  |  |  |
| media attribute | present only if there is no key-mgmt attribute at session level | a= line attribute = key-mgmt |  | SDP_OFFER AND MCD_1to1 |
| key-mgmt |  |  | TS 24.379 [9] clause 6.2.1 |  |
| mikey | MIKEY-SAKKE I_MESSAGE as specified in Table 5.5.9.1-2A | Use condition MCDATA | RFC 4567 [44] |  |

##### 5.5.3.1.2	SDP Message from the SS

##### -	MCPTT

Table 5.5.3.1.2-1: SDP Message from the SS for MCPTT

| Derivation Path: RFC 4566 [27] |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment |  | Reference | Condition |
| Session description: |  |  |  |  |  |  |
| Protocol Version |  | "0" | v= line |  |  |  |
| Origin |  | Same o=line as in the previous SDP message sent by the SS except that sess-version is incremented by one | o= line |  |  |  |
| Origin |  |  | o= line |  |  | FIRST_SDP_FROM_SS |
| username |  | "-" | "-" indicating the concept of user IDs not being supported |  |  |  |
| sess-id |  | "11111111" | A numeric string such that the tuple of <username>, <sess-id>, <nettype>, <addrtype>, and <unicast-address> forms a globally unique identifier for the session. |  |  |  |
| sess-version |  | "11111111" |  |  |  |  |
| nettype |  | "IN" |  |  |  |  |
| Addrtype |  | "IP4" or "IP6" depending on IP address" | This depends on the unicast address of the UE |  |  |  |
| unicast-address |  | IP address of the SS |  |  |  |  |
| Session Name |  | " " | s= line single empty space indicating no session name |  |  |  |
| Bandwidth |  |  | b= line |  |  |  |
| "AS" |  | 38 |  |  | TS 26.114 [64] Table K.6 |  |
| Time description |  |  |  |  |  |  |
| Timing |  |  | t= line |  |  |  |
| start-time |  | "0" |  |  |  |  |
| stop-time |  | "0" |  |  |  |  |
| Session attribute |  |  | a=line attribute=”ice-lite” |  | RFC 5245 [115] | PRE_ESTABLISHED_SESSION |
| ice-lite |  |  |  |  |  |  |
| Media description[1] |  |  | Media description for audio |  |  |  |
| media description |  |  | m= line media = audio |  | RFC 4867 [59] |  |
| media |  | "audio" |  |  |  |  |
| port |  | port number assigned by the SS (even integer) | The transport port to which the media stream is sent |  | RFC 6335 [63] clause 6 |  |
| proto |  | "RTP/SAVP" |  |  |  |  |
| fmt |  | "99" | RTP/SAVP payload type for AMR-WB is dynamic |  |  | INITIAL_SDP_OFFER |
|  |  | value for AMR-WB as used in initial offer |  |  |  |  |
| media title |  | "speech" | i= line |  |  |  |
| Connection Data |  |  | c= line |  |  |  |
| nettype |  | "IN" |  |  |  |  |
| Addrtype |  | "IP4" or "IP6" depending on IP address" | This depends on the connection address |  |  |  |
| connection-address |  | IP address of the SS |  |  |  |  |
| Bandwidth |  |  | b= line |  |  |  |
| "AS" |  | 38 |  |  | TS 26.114 [64] Table K.6 |  |
| "RS" |  | 0 |  |  | RFC 3556 [113] |  |
| "RR" |  | 2000 |  |  | RFC 3556 [113] |  |
| media attribute |  |  | a= line attribute = rtpmap |  |  |  |
| rtpmap |  | "rtpmap" |  |  |  |  |
| payload type |  | "99" |  |  |  | INITIAL_SDP_OFFER |
|  |  | value for AMR-WB as used in initial offer |  |  |  |  |
| encoding name |  | "AMR-WB" |  |  |  |  |
| clock rate |  | 16000 |  |  | RFC 4867 [59] clause 8.3 |  |
| encoding parameter |  | "1" | Channel number |  |  |  |
| media attribute |  |  | a= line attribute = fmtp |  |  |  |
| fmtp |  |  |  |  |  |  |
| format |  | "99" |  |  |  | INITIAL_SDP_OFFER |
|  |  | value for AMR-WB as used in initial offer |  |  |  |  |
| format specific parameters |  |  | Parameters of WB-AMR codec |  |  |  |
| mode-change-capability |  | "2" | To be able to interoperate fully with gateways to circuit switched networks |  | RFC 4867 [59] clause 8.2 |  |
| max-red |  | "0" | No redundancy will be used |  | RFC 4867 [59] clause 8.2 |  |
| media attribute |  |  | a= line attribute =ptime |  |  |  |
| ptime |  | "20" | packet time |  |  |  |
| media attribute |  |  | a= line attribute =maxptime |  |  |  |
| maxptime |  | "240" | maximum packet time |  |  |  |
| media attribute |  |  | a= line attribute = key-mgmt |  |  | WITH_SECURITY OR (PRIVATE-CALL AND SDP_OFFER AND NOT WITHOUT_SECURITY) |
| key-mgmt |  |  |  |  | TS 24.379 [9] clause 6.2.1 |  |
| mikey |  | MIKEY-SAKKE I_MESSAGE as specified in Table 5.5.9.1-2 |  |  | RFC 4567 [44] |  |
| media attribute |  |  | a=line attribute=”candidate” |  | RFC 5245 [115] | PRE_ESTABLISHED_SESSION |
| candidate |  |  | candidate for RTP |  |  |  |
| foundation |  | 1234 | arbitrarily selected |  |  |  |
| component-id |  | 1 | according to RFC 5245 [115] clause 4.1.1.1 |  |  |  |
| transport |  | "UDP" |  |  |  |  |
| priority |  | 2130706431 | RFC 5245 [115] clause 4.2: 224 * 126 + 28 * 65535 + 256 - component id |  |  |  |
| connection-address |  | IP address of the SS (same IP address as in the c=line for speech) | default candidate |  |  |  |
| port |  | same port number as in the m= line for speech |  |  |  |  |
| cand-type |  | "host" |  |  |  |  |
| media attribute |  |  | a=line attribute=”candidate” |  | RFC 5245 [115] | PRE_ESTABLISHED_SESSION |
| candidate |  |  | candidate for RTCP |  |  |  |
| foundation |  | 1234 | same as for RTP |  |  |  |
| component-id |  | 2 | according to RFC 5245 [115] clause 4.1.1.1 |  |  |  |
| transport |  | "UDP" |  |  |  |  |
| priority |  | 2130706430 | RFC 5245 [115] clause 4.2: 224 * 126 + 28 * 65535 + 256 - component id |  |  |  |
| connection-address |  | IP address of the SS (same IP address as in the c=line for speech) | default candidate |  |  |  |
| port |  | same port number as in the m= line for speech incremented by 1 |  |  |  |  |
| cand-type |  | "host" |  |  |  |  |
| Media description[2] |  |  | Media description for media control |  |  | NOT WITHOUT_FLOORCONTROL OR PRE_ESTABLISHED_SESSION |
| media description |  |  | m= line media = application SDP media-level section for a media control entity |  |  |  |
| media |  | "application" |  |  |  |  |
| port |  | port number assigned by the SS being different than the port number of the audio channel (RTP) and its associated control channel (RTCP)" | The port for the media control entity |  |  |  |
| proto |  | "udp" |  |  |  |  |
| fmt |  | "MCPTT" |  |  |  |  |
| Connection Data |  |  | c= line |  |  |  |
| nettype |  | "IN" |  |  |  |  |
| Addrtype |  | "IP4" or "IP6" depending on IP address | This depends on the connection address |  |  |  |
| connection-address |  | IP address of the SS |  |  |  |  |
| media attribute |  |  | a= line attribute = fmtp |  |  |  |
| fmtp |  |  |  |  |  |  |
| format |  | "MCPTT" |  |  |  |  |
| format specific parameters |  |  |  |  |  | SDP_OFFER AND NOT WITHOUT_FLOORCONTROL |
| mc_queueing |  | Present | Parameter has no value |  | TS 24.380 [10] cl. 12.1.2.3 |  |
| mc_priority |  | "3" | "3" is the value of the <user-priority> element for user A in the MCPTT Group Configuration (Table 5.5.7.1-1) |  | TS 24.380 [10] cl. 12.1.2.3 and cl. 14.3.3 |  |
| mc_granted |  | not present |  |  | TS 24.380 [10] cl. 12.1.2.3 |  |
| mc_implicit_request |  | not present |  |  | TS 24.380 [10] cl. 12.1.2.3 |  |
| mc_ssrc |  | not present |  |  | TS 24.380 [10] cl. 12.1.2.3 |  |
| mc_no_floor_ctrl |  | not present |  |  | TS 24.380 [10] cl. 12.1.2.3 |  |
| format specific parameters |  |  |  |  |  | SDP_ANSWER AND NOT WITHOUT_FLOORCONTROL |
| mc_queueing |  | present if included in the offer | Parameter has no value |  | TS 24.380 [10] cl. 12.1.2.3 |  |
| mc_priority |  | if a value is provided in the offer: "3" or the value provided in the offer, whichever is the lower value; otherwise not present | "3" is the value of the <user-priority> element for user A in the MCPTT Group Configuration (Table 5.5.7.1-1) NOTE: <num-levels-priority-hierarchy> has a value of 10 for on-network i.e. it is greater than 3 |  | TS 24.380 [10] cl. 12.1.2.3 and cl. 14.3.3 |  |
| mc_granted |  | not present |  |  | TS 24.380 [10] cl. 12.1.2.3 |  |
|  |  | present | Parameter has no value |  | TS 24.380 [10] cl. 12.1.2.3 | IMPLICIT_FLOOR_GRANTED |
| mc_implicit_request |  | not present |  |  | TS 24.380 [10] cl. 12.1.2.3 |  |
|  |  | present | Parameter has no value |  | TS 24.380 [10] cl. 12.1.2.3 | IMPLICIT_GRANT_REQUESTED |
| mc_ssrc |  | not present |  |  | TS 24.380 [10] cl. 12.1.2.3 |  |
|  |  | same value as in the offer if provided in the offer and there is no collision with the value used by the SS; otherwise value assigned by the SS |  |  | TS 24.380 [10] cl. 12.1.2.3 | IMPLICIT_GRANT_REQUESTED |
| mc_no_floor_ctrl |  | not present |  |  | TS 24.380 [10] cl. 12.1.2.3 |  |
| format specific parameters |  |  |  |  |  | WITHOUT_FLOORCONTROL |
| mc_queueing |  | not present |  |  | TS 24.380 [10] cl. 12.1.2.3 |  |
| mc_priority |  | not present |  |  | TS 24.380 [10] cl. 12.1.2.3 and cl. 14.3.3 |  |
| mc_granted |  | not present |  |  | TS 24.380 [10] cl. 12.1.2.3 |  |
| mc_implicit_request |  | not present |  |  | TS 24.380 [10] cl. 12.1.2.3 |  |
| mc_ssrc |  | not present |  |  | TS 24.380 [10] cl. 12.1.2.3 |  |
| mc_no_floor_ctrl |  | present | Parameter has no value |  | TS 24.380 [10] cl. 12.1.2.3 |  |
| media attribute |  | a=line attribute=”candidate” | RFC 5245 [115] |  | PRE_ESTABLISHED_SESSION |
| candidate |  | candidate for Media Control messages |  |  |  |
| foundation | 4321 | arbitrarily selected; different than for RTP/RTCP |  |  |  |
| component-id | 1 | according to RFC 5245 [115] clause 4.1.1.1 |  |  |  |
| transport | "UDP" |  |  |  |  |
| priority | 2130706431 | RFC 5245 [115] clause 4.2: 224 * 126 + 28 * 65535 + 256 - component id |  |  |  |
| connection-address | IP address of the SS (same IP address as in the c=line for media control) | default candidate |  |  |  |
| port | same port number as in the m= line for application |  |  |  |  |
| cand-type | "host" |  |  |  |  |

##### -	MCVideo

Table 5.5.3.1.2-2: SDP Message from the SS for MCVideo

| Derivation Path: RFC 4566 [27] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Session description: |  |  |  |  |
| Protocol Version | "0" | v= line |  |  |
| Origin | Same o=line as in the previous SDP message sent by the SS except that sess-version is incremented by one | o= line |  |  |
| Origin |  | o= line |  | FIRST_SDP_FROM_SS |
| username | "-" | "-" indicating the concept of user IDs not being supported |  |  |
| sess-id | "11111111" | A numeric string such that the tuple of <username>, <sess-id>, <nettype>, <addrtype>, and <unicast-address> forms a globally unique identifier for the session. |  |  |
| sess-version | "11111111" |  |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address | This depends on the unicast address of the UE |  |  |
| unicast-address | IP address of the SS |  |  |  |
| Session Name | " " | s= line single empty space indicating no session name |  |  |
| Bandwidth |  | b= line |  |  |
| "AS" | 352 |  |  |  |
| Time description |  |  |  |  |
| Timing |  | t= line |  |  |
| start-time | "0" |  |  |  |
| stop-time | "0" |  |  |  |
| Session attribute |  | a=line attribute=”ice-lite” | RFC 5245 [115] | PRE_ESTABLISHED_SESSION |
| ice-lite |  |  |  |  |
| Media description[1] |  | Media description for audio |  |  |
| media description |  | m= line media = audio | RFC 4867 [59] |  |
| media | "audio" |  |  |  |
| port | port number assigned by the SS (even integer) | The transport port to which the media stream is sent | RFC 6335 [63] clause 6 |  |
| proto | "RTP/SAVP" |  |  |  |
| fmt | "99" | RTP/SAVP payload type for AMR-WB is dynamic |  | INITIAL_SDP_OFFER |
|  | value for AMR-WB as used in initial offer |  |  |  |
| media title | "audio component of MCVideo" | i= line |  |  |
| Connection Data |  | c= line |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address | This depends on the connection address |  |  |
| connection-address | IP address of the SS |  |  |  |
| Bandwidth |  | b= line |  |  |
| "AS" | 37 |  |  |  |
| "RS" | 0 |  | RFC 3556 [113] |  |
| "RR" | 2000 |  | RFC 3556 [113] |  |
| media attribute |  | a= line attribute = rtpmap |  |  |
| rtpmap | "rtpmap" |  |  |  |
| payload type | "99" |  |  | INITIAL_SDP_OFFER |
|  | value for AMR-WB as used in initial offer |  |  |  |
| encoding name | "AMR-WB" |  |  |  |
| clock rate | 16000 |  | RFC 4867 [59] clause 8.3 |  |
| encoding parameter | "1" | Channel number |  |  |
| media attribute |  | a= line attribute = fmtp |  |  |
| fmtp |  |  |  |  |
| format | "99" |  |  | INITIAL_SDP_OFFER |
|  | value for AMR-WB as used in initial offer |  |  |  |
| format specific parameters |  | Parameters of WB-AMR codec |  |  |
| mode-change-capability | "2" | To be able to interoperate fully with gateways to circuit switched networks | RFC 4867 [59] clause 8.2 |  |
| max-red | "0" | No redundancy will be used | RFC 4867 [59] clause 8.2 |  |
| media attribute |  | a= line attribute =ptime |  |  |
| ptime | "20" | packet time |  |  |
| media attribute |  | a= line attribute =maxptime |  |  |
| maxptime | "240" | maximum packet time |  |  |
| media attribute |  | a= line attribute = key-mgmt |  | WITH_SECURITY OR (PRIVATE-CALL AND SDP_OFFER AND NOT WITHOUT_SECURITY) |
| key-mgmt |  |  | TS 24.281 [86] clause 6.2.1 |  |
| mikey | MIKEY-SAKKE I_MESSAGE as specified in Table 5.5.9.1-2 | Use condition MCVIDEO | RFC 4567 [44] |  |
| media attribute |  | a=line attribute=”candidate” | RFC 5245 [115] | PRE_ESTABLISHED_SESSION |
| candidate |  | candidate for RTP |  |  |
| foundation | 1234 | arbitrarily selected |  |  |
| component-id | 1 | according to RFC 5245 [115] clause 4.1.1.1 |  |  |
| transport | "UDP" |  |  |  |
| priority | 2130706431 | RFC 5245 [115] clause 4.2: 224 * 126 + 28 * 65535 + 256 - component id |  |  |
| connection-address | IP address of the SS (same IP address as in the c=line for audio) | default candidate |  |  |
| port | same port number as in the m= line for audio |  |  |  |
| cand-type | "host" |  |  |  |
| media attribute |  | a=line attribute=”candidate” | RFC 5245 [115] | PRE_ESTABLISHED_SESSION |
| candidate |  | candidate for RTCP |  |  |
| foundation | 1234 | same as for RTP |  |  |
| component-id | 2 | according to RFC 5245 [115] clause 4.1.1.1 |  |  |
| transport | "UDP" |  |  |  |
| priority | 2130706430 | RFC 5245 [115] clause 4.2: 224 * 126 + 28 * 65535 + 256 - component id |  |  |
| connection-address | IP address of the SS (same IP address as in the c=line for audio) | default candidate |  |  |
| port | same port number as in the m= line for audio incremented by 1 |  |  |  |
| cand-type | "host" |  |  |  |
| Media description[2] |  | Media description for video |  |  |
| media description |  | m= line media = video SDP media-level section for a media-transmission control entity |  |  |
| media | "video" |  |  |  |
| port | port number of the audio stream incremented by 2 (resulting in even integer) | The port for the media-transmission control entity |  |  |
| proto | "RTP/SAVPF" |  |  |  |
| fmt | "100" |  |  | INITIAL_SDP_OFFER |
|  | value for H264 as used in initial offer |  |  |  |
| media title | "video component of MCVideo" | i= line |  |  |
| Connection Data |  | c= line |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address |  |  |  |
| connection-address | IP address of the SS |  |  |  |
| Bandwidth |  | b= line |  |  |
| "AS" | 315 |  |  |  |
| "RS" | 0 |  | RFC 3556 [113] |  |
| "RR" | 2500 |  | RFC 3556 [113] |  |
| media attribute |  | a= line attribute = rtpmap |  |  |
| rtpmap | "rtpmap" |  |  |  |
| payload type | "100" |  |  | INITIAL_SDP_OFFER |
|  | value for H264 as used in initial offer |  |  |  |
| encoding name | "H264" |  |  |  |
| clock rate | 90000 |  | RFC 6184 [129] |  |
| media attribute |  | a= line attribute = fmtp |  |  |
| fmtp |  |  |  |  |
| format | "100" |  |  | INITIAL_SDP_OFFER |
|  | value for H264 as used in initial offer |  |  |  |
| format specific parameters |  | Parameters the H264 codec | RFC 6184 [129] | SDP_OFFER |
| packetization-mode | "0" |  |  |  |
| profile-level-id | "42e00c" |  |  |  |
| sprop-parameter-sets | "J0LgDJWgUH6Af1A=,KM46gA==" |  |  |  |
| format specific parameters | same parameters and values as sent by the UE in the corresponding SDP offer | Parameters the H264 codec |  | SDP_ANSWER |
| media attribute |  | a= line attribute = rtcp-fb | RFC 4585 [130] | SDP_OFFER |
| rtcp-fb |  |  |  |  |
| rtcp-fb-pt | "*" |  |  |  |
| rtcp-fb-val | "trr-int 5000" |  |  |  |
| media attribute |  | a= line attribute = rtcp-fb | RFC 4585 [130] | SDP_OFFER |
| rtcp-fb |  |  |  |  |
| rtcp-fb-pt | "*" |  |  |  |
| rtcp-fb-val | "nack" |  |  |  |
| media attribute |  | a= line attribute = rtcp-fb | RFC 4585 [130] | SDP_OFFER |
| rtcp-fb |  |  |  |  |
| rtcp-fb-pt | "*" |  |  |  |
| rtcp-fb-val | "nack pli" |  |  |  |
| media attribute |  | a= line attribute = rtcp-fb | RFC 4585 [130] | SDP_OFFER |
| rtcp-fb |  |  |  |  |
| rtcp-fb-pt | "*" |  |  |  |
| rtcp-fb-val | "ccm fir" |  |  |  |
| media attribute |  | a= line attribute = rtcp-fb | RFC 4585 [130] | SDP_OFFER |
| rtcp-fb |  |  |  |  |
| rtcp-fb-pt | "*" |  |  |  |
| rtcp-fb-val | "ccm tmmbr" |  |  |  |
| media attribute | present if there have been a=tcap and a=pcfg attributes in the corresponding SDP offer | a= line attribute = acfg | RFC 5939 [128] TS 26.114 [64] clause 6.2.1a.3 | SDP_ANSWER |
| acfg |  |  |  |  |
| config-number | 1 |  |  |  |
| sel-cfg-list | "t=1" |  |  |  |
| media attribute |  | a= line attribute = key-mgmt |  | WITH_SECURITY OR (PRIVATE-CALL AND SDP_OFFER AND NOT WITHOUT_SECURITY) |
| key-mgmt |  |  | TS 24.281 [86] clause 6.2.1 |  |
| mikey | MIKEY-SAKKE I_MESSAGE as specified in Table 5.5.9.1-2 | Use condition MCVIDEO | RFC 4567 [44] |  |
| media attribute |  | a=line attribute=”candidate” | RFC 5245 [115] | PRE_ESTABLISHED_SESSION |
| candidate |  | candidate for RTP |  |  |
| foundation | 2345 | arbitrarily selected; different than audio |  |  |
| component-id | 1 | according to RFC 5245 [115] clause 4.1.1.1 |  |  |
| transport | "UDP" |  |  |  |
| priority | 2130706431 | RFC 5245 [115] clause 4.2: 224 * 126 + 28 * 65535 + 256 - component id |  |  |
| connection-address | IP address of the SS (same IP address as in the c=line for video) | default candidate |  |  |
| port | same port number as in the m= line for video |  |  |  |
| cand-type | "host" |  |  |  |
| media attribute |  | a=line attribute=”candidate” | RFC 5245 [115] | PRE_ESTABLISHED_SESSION |
| candidate |  | candidate for RTCP |  |  |
| foundation | 22345 | same as for RTP |  |  |
| component-id | 2 | according to RFC 5245 [115] clause 4.1.1.1 |  |  |
| transport | "UDP" |  |  |  |
| priority | 2130706430 | RFC 5245 [115] clause 4.2: 224 * 126 + 28 * 65535 + 256 - component id |  |  |
| connection-address | IP address of the SS (same IP address as in the c=line for video) | default candidate |  |  |
| port | same port number as in the m= line for video incremented by 1 |  |  |  |
| Media description[3] |  | Media description for media control |  | NOT WITHOUT_TRANSMISSIONCONTROL OR PRE_ESTABLISHED_SESSION |
| media description |  | m= line media = application SDP media-level section for a media control entity |  |  |
| media | "application" |  |  |  |
| port | port number assigned by the SS being different than the port number of the audio and video channels (RTP) and their associated control channels (RTCP)" | The port for the media control entity |  |  |
| proto | "udp" |  |  |  |
| fmt | "MCVideo" |  |  |  |
| Connection Data |  | c= line |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address | This depends on the connection address |  |  |
| connection-address | IP address of the SS |  |  |  |
| media attribute |  | a= line attribute = fmtp |  |  |
| fmtp |  |  |  |  |
| format | "MCVideo" |  |  |  |
| format specific parameters |  |  |  | SDP_OFFER AND NOT WITHOUT_TRANSMISSIONCONTROL |
| mc_queueing | Present | Parameter has no value | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| mc_priority | "5" | Any integer value in the range of 1..255 | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| mc_granted | not present |  | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| mc_implicit_request | not present |  | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| mc_reception_priority | not present |  | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| format specific parameters |  |  |  | SDP_ANSWER AND NOT WITHOUT_TRANSMISSIONCONTROL |
| mc_queueing | present if included in the offer | Parameter has no value | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| mc_priority | if a value is provided in the offer: "3" or the value provided in the offer, whichever is the lower value; otherwise not present | "3" is the value of the <user-priority> element for user A in the MCVideo Group Configuration (Table 5.5.7.2-1) | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| mc_granted | not present |  |  |  |
|  | present | Parameter has no value | 3GPP TS 24.581 [88] clause 12, clause 14 | IMPLICIT_FLOOR_GRANTED |
| mc_implicit_request | not present |  |  |  |
|  | present | Parameter has no value | 3GPP TS 24.581 [88] clause 12, clause 14 | IMPLICIT_GRANT_REQUESTED |
| mc_reception_priority | same value as in the SDP offer if present, not present otherwise |  | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| media attribute |  | a=line attribute=”candidate” | RFC 5245 [115] | PRE_ESTABLISHED_SESSION |
| candidate |  | candidate for Media Control messages |  |  |
| foundation | 4321 | arbitrarily selected; different than for RTP/RTCP (audio, video) |  |  |
| component-id | 1 | according to RFC 5245 [115] clause 4.1.1.1 |  |  |
| transport | "UDP" |  |  |  |
| priority | 2130706431 | RFC 5245 [115] clause 4.2: 224 * 126 + 28 * 65535 + 256 - component id |  |  |
| connection-address | IP address of the SS (same IP address as in the c=line for media control) | default candidate |  |  |
| port | same port number as in the m= line for application |  |  |  |
| cand-type | "host" |  |  |  |

##### -	MCData

Table 5.5.3.1.2-3: SDP Message from the SS for MCData

| Derivation Path: RFC 4566 [27] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Session description: |  |  |  |  |
| Protocol Version | "0" | v= line |  |  |
| Origin | Same o=line as in the previous SDP message sent by the SS except that sess-version is incremented by one | o= line |  |  |
| Origin |  | o= line |  | FIRST_SDP_FROM_SS |
| username | "-" | "-" indicating the concept of user IDs not being supported |  |  |
| sess-id | "11111111" | A numeric string such that the tuple of <username>, <sess-id>, <nettype>, <addrtype>, and <unicast-address> forms a globally unique identifier for the session. |  |  |
| sess-version | "11111111" |  |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address |  |  |  |
| unicast-address | IP address of the SS |  |  |  |
| Session Name | " " | s= line |  |  |
| Time description |  |  |  |  |
| Timing |  | t= line |  |  |
| start-time | "0" |  |  |  |
| stop-time | "0" |  |  |  |
| Media description[1] |  | Media description for data |  |  |
| media description |  | m= line media = message | RFC 4867 [59] TS 24.282 [31] |  |
| media | "message" |  |  |  |
| port | port number assigned by the SS | The transport port to which the media stream is sent |  |  |
| proto | "TCP/MSRP" |  |  |  |
| fmt | “*” |  |  |  |
| Connection Data |  | c= line |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address |  |  |  |
| connection-address | IP address of the SS |  |  |  |
| media attribute |  | a= line attribute = sendonly |  | SDP_OFFER AND NOT SDS_SESSION |
| sendonly |  | No parameters associated with this line |  |  |
| media attribute |  | a= line attribute = recvonly |  | SDP_ANSWER AND NOT SDS_SESSION |
| recvonly |  | No parameters associated with this line |  |  |
| media attribute |  | a= line attribute = sendrecv |  | SDS_SESSION |
| sendrecv |  | No parameters associated with this line |  |  |
| media attribute |  | a= line attribute = path |  |  |
| path | MSRP URI according to RFC 4975 [120] clause 6 and 9 |  | TS 24.282 [31] |  |
| scheme | "msrp" |  |  |  |
| authority |  |  | RFC 3986 [123] clause 3.2 |  |
| userinfo | not present |  |  |  |
| host | IP address of the SS |  |  |  |
| port | same value as in the media line |  |  |  |
| session id | assigned by the SS |  |  |  |
| transport | "tcp" |  |  |  |
| URI-parameter | not present |  |  |  |
| media attribute |  | a= line attribute = accept-types | RFC 4975 [120] |  |
| accept-types |  |  |  |  |
| format-entry[1] | "application/vnd.3gpp.mcdata-signalling" |  |  |  |
| format-entry[2] | "application/vnd.3gpp.mcdata-payload" |  |  | MCDATA_SDS |
| media attribute |  | a= line attribute = setup | RFC 4145 [119] |  |
| setup | “actpass” |  |  | SDP_OFFER |
|  | "passive" |  |  | SDP_ANSWER |
| media attribute |  | a= line attribute = file-transfer-id | RFC 5547 [124] | MCDATA_FD |
| file-transfer-id | value assigned by the SS | randomly chosen globally unique identification (RFC 5547 [124]) |  | SDP_OFFER |
|  | same value as in the sdp offer |  |  | SDP_ANSWER |
| media attribute |  | a= line attribute = file-selector | RFC 5547 [124] | MCDATA_FD |
| file-selector |  |  |  | SDP_OFFER |
| selector[1] |  |  |  |  |
| filename | name of the file to be transferred | e.g. "TestFile.txt" |  |  |
| filesize | size of the file to be transferred |  |  |  |
| filetype | type of the file to be transferred | e.g. "text/plain" |  |  |
| hash |  |  |  |  |
| algorithm | "sha-1" |  |  |  |
| value | hash value of the file to be transferred |  |  |  |
| file-selector | same value as in the sdp offer |  |  | SDP_ANSWER |
| media attribute |  | a= line attribute = file-date | RFC 5547 [124] | MCDATA_FD AND SDP_OFFER |
| file-date |  |  |  |  |
| date-param[1] |  |  |  |  |
| type | "creation" |  |  |  |
| date-time | date and time when the file has been created | e.g. "Mon, 20 Dec 2021 15:01:31 +0100" | RFC 5322 [109] |  |
| media attribute |  | a= line attribute = key-mgmt |  | SDP_OFFER AND MCD_1to1 |
| key-mgmt |  |  | TS 24.379 [9] clause 6.2.1 |  |
| mikey | MIKEY-SAKKE I_MESSAGE as specified in Table 5.5.9.1-2 | Use condition MCDATA | RFC 4567 [44] |  |

##### 5.5.3.1.3	SDP Message from the UE - Off-network

##### -	MCPTT

Table 5.5.3.1.3-1: SDP Message from the UE - Off-network for MCPTT

| Derivation Path: RFC 4566 [27] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Session description: |  |  |  |  |
| Protocol Version | "0" | v= line |  |  |
| Origin |  | o= line |  |  |
| username | "-" |  |  |  |
| sess-id | any allowed value | A numeric string such that the tuple of <username>, <sess-id>, <nettype>, <addrtype>, and <unicast-address> forms a globally unique identifier for the session. |  |  |
| sess-version | any allowed value |  |  |  |
| nettype | "IN" |  |  |  |
| addrtype | "IP4" | "IP4" or "IP6" |  |  |
| unicast-address | px_MCPTT_IP_ConnectionAddressAll |  |  |  |
| Session Name | "-" | s= line |  |  |
| Connection Data |  | c= line |  |  |
| nettype | "IN" |  |  |  |
| addrtype | "IP4" | "IP4" or "IP6" |  |  |
| connection-address | px_MCPTT_IP_ConnectionAddressAll | Set to the multicast IP address of the MCPTT group |  |  |
| Bandwidth |  | b= line |  |  |
| bwtype | "AS:" | bwtype:bandwidth |  |  |
| bandwidth | any allowed value |  |  |  |
| Time description |  |  |  |  |
| Timing |  | t= line |  |  |
| start-time | "0" |  |  |  |
| stop-time | "0" |  |  |  |
| Media descriptions |  |  |  |  |
| media description |  | m= line media = audio |  |  |
| media | "audio" |  |  |  |
| port | any allowed value | Set to a port number for MCPTT speech of the MCPTT group |  |  |
| proto | "RTP/AVP" |  |  |  |
| fmt | any allowed value(s) | Indicating RTP payload type numbers |  |  |
| media title | "speech" | i= line |  |  |
| media attribute |  | a= line attribute = rtpmap |  |  |
| rtpmap | "rtpmap" |  |  |  |
| payload type | "99" |  |  |  |
| encoding name | "AMR-WB" |  |  |  |
| clock rate | 16000 |  |  |  |
| encoding parameter | "1" if present | Channel number |  |  |
| media attribute |  | a= line attribute = fmtp |  |  |
| fmtp | "fmtp" |  |  |  |
| format | the value given in fmt in the audio media description |  |  |  |
| format specific parameters |  | Parameters of WB-AMR codec |  |  |
| mode-change-capability | "2" | To be able to interoperate fully with gateways to circuit switched networks |  |  |
| max-red | "0" | No redundancy will be used |  |  |
| media attribute |  | a= line attribute =ptime |  |  |
| ptime | any allowed value | packet time |  |  |
| media attribute |  | a= line attribute =maxptime |  |  |
| maxptime | any allowed value | maximum packet time |  |  |
| media description |  | m= line media = application |  |  |
| media | "application" |  |  |  |
| port | any allowed value | Set to a port number for media-floor control entity of the MCPTT group |  |  |
| proto | "udp" |  |  |  |
| fmt | "MCPTT" |  |  |  |
| media attribute |  | a= line attribute = fmtp |  |  |
| fmtp |  |  |  |  |
| format | "MCPTT" |  |  |  |
| format specific parameters |  |  |  |  |
| mc_queueing | optional | Parameter has no value |  |  |
| mc_priority | not present or any allowed value | Any integer value in the range of 1..255 |  |  |
| mc_granted | present | Parameter has no value |  |  |
| mc_implicit_request | present | Parameter has no value |  |  |
| media attribute |  | a= line attribute = key-mgmt |  |  |
| key-mgmt |  |  |  |  |
| mikey | MIKEY-SAKKE I_MESSAGE as specified in Table 5.5.9.1-2 |  |  |  |

##### -	MCVideo

Table 5.5.3.1.3-2: SDP Message from the UE - Off-network for MCVideo

| Derivation Path: RFC 4566 [27] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Session description: |  |  |  |  |
| Protocol Version | "0" | v= line |  |  |
| Origin |  | o= line |  |  |
| username | "-" |  |  |  |
| sess-id | any allowed value | A numeric string such that the tuple of <username>, <sess-id>, <nettype>, <addrtype>, and <unicast-address> forms a globally unique identifier for the session. |  |  |
| sess-version | any allowed value |  |  |  |
| nettype | "IN" |  |  |  |
| addrtype | "IP4" | "IP4" or "IP6" |  |  |
| unicast-address | px_MCVideo_IP_ConnectionAddressAll |  |  |  |
| Session Name | "-" | s= line |  |  |
| Connection Data |  | c= line |  |  |
| nettype | "IN" |  |  |  |
| addrtype | "IP4" | "IP4" or "IP6" |  |  |
| connection-address | px_MCVideo_IP_ConnectionAddressAll | Set to the multicast IP address of the MCVideo group |  |  |
| Bandwidth |  | b= line |  |  |
| bwtype | "AS:" | bwtype:bandwidth |  |  |
| bandwidth | any allowed value |  |  |  |
| Time description |  |  |  |  |
| Timing |  | t= line |  |  |
| start-time | "0" |  |  |  |
| stop-time | "0" |  |  |  |
| Media descriptions |  |  |  |  |
| media description |  | m= line media = audio |  |  |
| media | "audio" |  |  |  |
| port | any allowed value | Set to a port number for MCVideo speech of the MCVideo group |  |  |
| proto | "RTP/AVP" |  |  |  |
| fmt | any allowed value(s) | Indicating RTP payload type numbers |  |  |
| media title | "speech" | i= line |  |  |
| media attribute |  | a= line attribute = rtpmap |  |  |
| rtpmap | "rtpmap" |  |  |  |
| payload type | "99" |  |  |  |
| encoding name | "AMR-WB" |  |  |  |
| clock rate | 16000 |  |  |  |
| encoding parameter | "1" if present | Channel number |  |  |
| media attribute |  | a= line attribute = fmtp |  |  |
| fmtp | "fmtp" |  |  |  |
| format | the value given in fmt in the audio media description |  |  |  |
| format specific parameters |  | Parameters of WB-AMR codec |  |  |
| mode-change-capability | "2" | To be able to interoperate fully with gateways to circuit switched networks |  |  |
| max-red | "0" | No redundancy will be used |  |  |
| media attribute |  | a= line attribute =ptime |  |  |
| ptime | any allowed value | packet time |  |  |
| media attribute |  | a= line attribute =maxptime |  |  |
| maxptime | any allowed value | maximum packet time |  |  |
| media description |  | m= line media = video SDP media-level section for a media-transmission control entity |  |  |
| media | "video" |  |  |  |
| port | any allowed value | The port for the media-transmission control entity |  |  |
| proto | "udp" | User Datagram Protocol. With UDP, computer applications can send messages to other hosts on an Internet Protocol (IP) network. Time-sensitive applications often use UDP because dropping packets is preferable to waiting for packets delayed due to retransmission, which may not be an option in a real-time system. |  |  |
| fmt | "MCVideo" |  |  |  |
| Connection Data |  | c= line Included if the media plane control channel uses a different IP address than other media described in the SDP |  |  |
| nettype | "IN" |  |  |  |
| addrtype | "IP4" |  |  |  |
| connection-address | px_MCVideo_IP_ConnectionAddressApp |  |  |  |
| media attribute |  | a= line attribute = rtpmap |  |  |
| rtpmap | "rtpmap" |  |  |  |
| payload type | "" |  |  |  |
| encoding name | "H.264" |  |  |  |
| clock rate |  |  | RFC 4867 [59] clause 8.3 |  |
| encoding parameter | "" if present | Channel number |  |  |
| media attribute |  | a= line attribute = fmtp |  |  |
| fmtp |  |  | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| format | "MCVideo" |  |  |  |
| format specific parameters |  |  |  |  |
| mc_queueing | optional | Parameter has no value. Shall include the "mc_queueing" fmtp attribute in SDP offers when queueing of Transmission request is supported. | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| mc_priority | not present or any allowed value | Any integer value in the range of 1..255 Shall include the "mc_priority" fmtp attribute when a transmission priority different than the default priority is required. | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| mc_reception_priority | not present or any allowed value | Any integer value in the range of 0…255 Shall include the "mc_reception_priority" fmtp attribute when a reception priority different than the default reception priority is required. | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| mc_granted | present | Parameter has no value Shall include the "mc_granted" fmtp attribute in the SDP offer of an initial SIP INVITE request when it is acceptable for the MCVideo client to receive a granted indication in the SIP 200 (OK) response to an initial INVITE request. | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| mc_implicit_request | present | Parameter has no value Shall include the "mc_implicit_request" fmtp attribute when a SIP request shall be interpreted as an implicit Transmission request. If not explicitly stated in procedures in the present document or in procedures in 3GPP TS 24.281 [2] that the "mc_implicit_request" fmtp attribute shall be included, the decision to include the "mc_implicit_request" fmtp attribute or not, is an implementation option. | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| media attribute |  | a= line attribute = key-mgmt |  | PRIVATE-CALL |
| key-mgmt |  | Key Management attribute field in the media and session level. | TS 24.281 [86] clause 6.2.1 |  |
| mikey | MIKEY-SAKKE I_MESSAGE as specified in Table 6.1.1.1.3.3-3 | MIKEY carries the security parameters needed for setting up the security protocol. It is a protocol designed for government and relevant enterprises to enable secure, cross-platform multimedia communications. | RFC 4567 [44] |  |
| media description |  | m= line media = application |  |  |
| media | "application" |  |  |  |
| port | any allowed value | Set to a port number for media-floor control entity of the MCVideo group |  |  |
| proto | "udp" |  |  |  |
| fmt | "MCVideo" |  |  |  |
| media attribute |  | a= line attribute = fmtp |  |  |
| fmtp |  |  |  |  |
| format | "MCVideo" |  |  |  |
| format specific parameters |  |  |  |  |
| mc_queueing | optional | Parameter has no value |  |  |
| mc_priority | not present or any allowed value | Any integer value in the range of 1..255 |  |  |
| mc_granted | present | Parameter has no value |  |  |
| mc_implicit_request | present | Parameter has no value |  |  |
| media attribute |  | a= line attribute = key-mgmt |  |  |
| key-mgmt |  |  |  |  |
| mikey | MIKEY-SAKKE I_MESSAGE as specified in Table 5.5.9.1-2A |  |  |  |

##### -	MCData

Table 5.5.3.1.3-3: SDP Message from the UE - Off-network for MCData

FFS

##### 5.5.3.1.4	SDP Message from the SS - Off-network

##### -	MCPTT

Table 5.5.3.1.4-1: SDP Message from the SS - Off-network for MCPTT

| Derivation Path: RFC 4566 [27] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Session description: |  |  |  |  |
| Protocol Version | "0" | v= line |  |  |
| Origin |  | o= line |  |  |
| username | "-" |  |  |  |
| sess-id | "12345678" | A numeric string such that the tuple of <username>, <sess-id>, <nettype>, <addrtype>, and <unicast-address> forms a globally unique identifier for the session. |  |  |
| sess-version | "12345678" |  |  |  |
| nettype | "IN" |  |  |  |
| addrtype | "IP4" |  |  |  |
| unicast-address | px_MCPTT_IP_ConnectionAddressAll |  |  |  |
| Session Name | "-" | s= line |  |  |
| Connection Data |  | c= line |  |  |
| nettype | "IN" |  |  |  |
| addrtype | "IP4" | "IP4" or "IP6" |  |  |
| connection-address | px_MCPTT_IP_ConnectionAddressAll | Set to the multicast IP address of the MCPTT group |  |  |
| Bandwidth |  | b= line |  |  |
| bwtype | "AS:" | bwtype:bandwidth |  |  |
| bandwidth | any allowed value |  |  |  |
| Time description |  |  |  |  |
| Timing |  | t= line |  |  |
| start-time | "0" |  |  |  |
| stop-time | "0" |  |  |  |
| Media descriptions |  |  |  |  |
| media description |  | m= line media = audio |  |  |
| media | "audio" |  |  |  |
| port | "49152" | Set to a port number for MCPTT speech of the MCPTT group |  |  |
| proto | "RTP/AVP" |  |  |  |
| fmt | "99" | Indicating RTP payload type numbers |  |  |
| media title | "speech" | i= line |  |  |
| media attribute |  | a= line attribute = rtpmap |  |  |
| rtpmap | "rtpmap" |  |  |  |
| payload type | "99" |  |  |  |
| encoding name | "AMR-WB" |  |  |  |
| clock rate | 16000 |  |  |  |
| encoding parameter | "1" if present | Channel number |  |  |
| media attribute |  | a= line attribute = fmtp |  |  |
| fmtp | "fmtp" |  |  |  |
| format | "99" |  |  |  |
| format specific parameters |  | Parameters of WB-AMR codec |  |  |
| mode-change-capability | "2" | To be able to interoperate fully with gateways to circuit switched networks |  |  |
| max-red | "0" | No redundancy will be used |  |  |
| media attribute |  | a= line attribute =ptime |  |  |
| ptime | "20" | packet time |  |  |
| media attribute |  | a= line attribute =maxptime |  |  |
| maxptime | "240" | maximum packet time |  |  |
| media description |  | m= line media = application |  |  |
| media | "application" |  |  |  |
| port | "49153" | Set to a port number for media-floor control entity of the MCPTT group |  |  |
| proto | "udp" |  |  |  |
| fmt | "MCPTT" |  |  |  |
| media attribute |  | a= line attribute = fmtp |  |  |
| fmtp |  |  |  |  |
| format | "MCPTT" |  |  |  |
| format specific parameters |  |  |  |  |
| mc_queueing | Present | Parameter has no value |  |  |
| mc_priority | "5" | Any integer value in the range of 1..255 |  |  |
| mc_granted | Present | Parameter has no value |  |  |
| mc_implicit_request | Present | Parameter has no value |  |  |
| media attribute |  | a= line attribute = key-mgmt |  |  |
| key-mgmt |  |  |  |  |
| mikey | MIKEY-SAKKE I_MESSAGE as specified in Table 5.5.9.1-2 |  |  |  |

##### -	MCVideo

Table 5.5.3.1.4-2: SDP Message from the SS - Off-network for MCVideo

| Derivation Path: RFC 4566 [27] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Session description: |  |  |  |  |
| Protocol Version | "0" | v= line |  |  |
| Origin |  | o= line |  |  |
| username | "-" |  |  |  |
| sess-id | "12345678" | A numeric string such that the tuple of <username>, <sess-id>, <nettype>, <addrtype>, and <unicast-address> forms a globally unique identifier for the session. |  |  |
| sess-version | "12345678" |  |  |  |
| nettype | "IN" |  |  |  |
| addrtype | "IP4" |  |  |  |
| unicast-address | px_MCVideo_IP_ConnectionAddressAll |  |  |  |
| Session Name | "-" | s= line |  |  |
| Connection Data |  | c= line |  |  |
| nettype | "IN" |  |  |  |
| addrtype | "IP4" | "IP4" or "IP6" |  |  |
| connection-address | px_MCVideo_IP_ConnectionAddressAll | Set to the multicast IP address of the MCVideo group |  |  |
| Bandwidth |  | b= line |  |  |
| bwtype | "AS:" | bwtype:bandwidth |  |  |
| bandwidth | any allowed value |  |  |  |
| Time description |  |  |  |  |
| Timing |  | t= line |  |  |
| start-time | "0" |  |  |  |
| stop-time | "0" |  |  |  |
| Media descriptions |  |  |  |  |
| media description |  | m= line media = audio |  |  |
| media | "audio" |  |  |  |
| port | "49152" | Set to a port number for MCVideo speech of the MCVideo group |  |  |
| proto | "RTP/AVP" |  |  |  |
| fmt | "99" | Indicating RTP payload type numbers |  |  |
| media title | "speech" | i= line |  |  |
| media attribute |  | a= line attribute = rtpmap |  |  |
| rtpmap | "rtpmap" |  |  |  |
| payload type | "99" |  |  |  |
| encoding name | "AMR-WB" |  |  |  |
| clock rate | 16000 |  |  |  |
| encoding parameter | "1" if present | Channel number |  |  |
| media attribute |  | a= line attribute = fmtp |  |  |
| fmtp | "fmtp" |  |  |  |
| format | "99" |  |  |  |
| format specific parameters |  | Parameters of WB-AMR codec |  |  |
| mode-change-capability | "2" | To be able to interoperate fully with gateways to circuit switched networks |  |  |
| max-red | "0" | No redundancy will be used |  |  |
| media attribute |  | a= line attribute =ptime |  |  |
| ptime | "20" | packet time |  |  |
| media attribute |  | a= line attribute =maxptime |  |  |
| maxptime | "240" | maximum packet time |  |  |
| media description |  | m= line media = video SDP media-level section for a media-transmission control entity |  |  |
| media | "video" |  |  |  |
| port | any allowed value | The port for the media-transmission control entity |  |  |
| proto | "udp" | User Datagram Protocol. With UDP, computer applications can send messages to other hosts on an Internet Protocol (IP) network. Time-sensitive applications often use UDP because dropping packets is preferable to waiting for packets delayed due to retransmission, which may not be an option in a real-time system. |  |  |
| fmt | "MCVideo" |  |  |  |
| Connection Data |  | c= line Included if the media plane control channel uses a different IP address than other media described in the SDP |  |  |
| nettype | "IN" |  |  |  |
| addrtype | "IP4" |  |  |  |
| connection-address | px_MCVideo_IP_ConnectionAddressApp |  |  |  |
| media attribute |  | a= line attribute = rtpmap |  |  |
| rtpmap | "rtpmap" |  |  |  |
| payload type | "" |  |  |  |
| encoding name | "H.264" |  |  |  |
| clock rate |  |  | RFC 4867 [59] clause 8.3 |  |
| encoding parameter | "" if present | Channel number |  |  |
| media attribute |  | a= line attribute = fmtp |  |  |
| fmtp |  |  | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| format | "MCVideo" |  |  |  |
| format specific parameters |  |  |  |  |
| mc_queueing | optional | Parameter has no value. Shall include the "mc_queueing" fmtp attribute in SDP offers when queueing of Transmission request is supported. | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| mc_priority | not present or any allowed value | Any integer value in the range of 1..255 Shall include the "mc_priority" fmtp attribute when a transmission priority different than the default priority is required. | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| mc_reception_priority | not present or any allowed value | Any integer value in the range of 0…255 Shall include the "mc_reception_priority" fmtp attribute when a reception priority different than the default reception priority is required. | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| mc_granted | present | Parameter has no value Shall include the "mc_granted" fmtp attribute in the SDP offer of an initial SIP INVITE request when it is acceptable for the MCVideo client to receive a granted indication in the SIP 200 (OK) response to an initial INVITE request. | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| mc_implicit_request | present | Parameter has no value Shall include the "mc_implicit_request" fmtp attribute when a SIP request shall be interpreted as an implicit Transmission request. If not explicitly stated in procedures in the present document or in procedures in 3GPP TS 24.281 [2] that the "mc_implicit_request" fmtp attribute shall be included, the decision to include the "mc_implicit_request" fmtp attribute or not, is an implementation option. | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| media attribute |  | a= line attribute = key-mgmt |  | PRIVATE-CALL |
| key-mgmt |  | Key Management attribute field in the media and session level. | TS 24.281 [86] clause 6.2.1 |  |
| mikey | MIKEY-SAKKE I_MESSAGE as specified in Table 6.1.1.1.3.3-3 | MIKEY carries the security parameters needed for setting up the security protocol. It is a protocol designed for government and relevant enterprises to enable secure, cross-platform multimedia communications. | RFC 4567 [44] |  |
| media description |  | m= line media = application |  |  |
| media | "application" |  |  |  |
| port | "49153" | Set to a port number for media-floor control entity of the MCVideo group |  |  |
| proto | "udp" |  |  |  |
| fmt | "MCVideo" |  |  |  |
| media attribute |  | a= line attribute = fmtp |  |  |
| fmtp |  |  |  |  |
| format | "MCVideo" |  |  |  |
| format specific parameters |  |  |  |  |
| mc_queueing | Present | Parameter has no value |  |  |
| mc_priority | "5" | Any integer value in the range of 1..255 |  |  |
| mc_granted | Present | Parameter has no value |  |  |
| mc_implicit_request | Present | Parameter has no value |  |  |
| media attribute |  | a= line attribute = key-mgmt |  |  |
| key-mgmt |  |  |  |  |
| mikey | MIKEY-SAKKE I_MESSAGE as specified in Table 5.5.9.1-2 |  |  |  |

##### -	MCData

Table 5.5.3.1.4-3: SDP Message from the SS - Off-network for MCData

FFS