3GPP TSG-RAN5 Meeting #96-e	R5-223951

# Electronic Meeting, 15th – 26th August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-2 | CR | 0303 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of test case 5.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, MCenhUEConTest |  |  |  |  |  | Date: |  |  | 2022-07-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - According to 24.379 clause 9.2.2.3.10 the MCPTT server shall include the "expires" attribute in the <affiliation> element of the application/pidf+xml MIME body contained in the NOTIFY for notification of change of group dynamic data - Editorial issues |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Expires attributes added to affiliation elements of Table 5.7.3.3-5 with dateTime value indication one hour in advance. - Editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect test specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 5.7	Configuration / Subscription to group dynamic data / De-subscribe

5.7.1	Test Purpose (TP)

(1)

with { UE (MCPTT Client) having established an MCPTT On-demand Pre-arranged Group Call with Automatic Commencement Mode }

ensure that {

when { MCPTT User requests to subscribe to group dynamic data for group call ongoing information }

then { UE (MCPTT Client) sends a SIP SUBSCRIBE message to subscribe to group dynamic data for group call ongoing information }

}

(2)

with { UE (MCPTT Client) having established an MCPTT On-demand Pre-arranged Group Call with Automatic Commencement Mode }

ensure that {

when { UE (MCPTT Client) receives a SIP NOTIFY message indicating the group call ongoing information }

then { UE (MCPTT Client) responds with a SIP 200 (OK) message }

}

(3)

with { UE (MCPTT Client) having established an MCPTT On-demand Pre-arranged Group Call with Automatic Commencement Mode }

ensure that {

when { MCPTT User requests to de-subscribe from group dynamic data for group call ongoing information }

then { UE (MCPTT Client) sends a SIP SUBSCRIBE message to de-subscribe from group dynamic data for group call ongoing information }

}

5.7.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.379 clause 9.2.1.6. Unless otherwise stated these are Rel-15 requirements.

[TS 24.379, clause 9.2.1.6]

In order to subscribe to changes in per-group dynamic data, the MCPTT client shall generate an initial SIP SUBSCRIBE request specified in 3GPP TS 24.229 [4], IETF RFC 3856 [51], and IETF RFC 6665 [26].

In the SIP SUBSCRIBE request, the MCPTT client:

1)	shall set the Request-URI to the public service identity identifying the originating participating MCPTT function serving the MCPTT user;

2)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body. In the application/vnd.3gpp.mcptt-info+xml MIME body, the MCPTT client shall include the <mcptt-request-uri> element set to the MCPTT group ID of the targeted MCPTT group;

3)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Preferred-Service header field specified in IETF RFC 6050 [9];

4)	if the MCPTT client wants to receive the current status and later notification, shall set the Expires header field specified in IETF RFC 6665 [26], to 4294967295;

NOTE 1:	4294967295, which is equal to 232-1, is the highest value defined for Expires header field in IETF RFC 3261 [24].

5)	if the MCPTT client wants to fetch the current state only, shall set the Expires header field specified in IETF RFC 6665 [26], to zero;

6)	shall include an Accept header field containing the application/pidf+xml MIME type; and

7)	shall include an application/simple-filter+xml MIME body indicating per-group restrictions of presence event package notification information specified in clause 9.3.2, indicating the MCPTT group ID.

In order to re-subscribe or de-subscribe, the MCPTT client shall generate an in-dialog SIP SUBSCRIBE request specified in 3GPP TS 24.229 [4], IETF RFC 3856 [51], and IETF RFC 6665 [26]. In the SIP SUBSCRIBE request, the MCPTT client:

1)	if the MCPTT client wants to receive the current status and later notification, shall set the Expires header field specified in IETF RFC 6665 [26], to 4294967295;

NOTE 2:	4294967295, which is equal to 232-1, is the highest value defined for Expires header field in IETF RFC 3261 [24].

2)	if the MCPTT client wants to de-subscribe, shall set the Expires header field specified in IETF RFC 6665 [26], to zero; and

3)	shall include an Accept header field containing the application/pidf+xml MIME type.

Upon receiving a SIP NOTIFY request specified in 3GPP TS 24.229 [4], IETF RFC 3856 [51], and IETF RFC 6665 [26], if the SIP NOTIFY request contains an application/pidf+xml MIME body indicating per-group dynamic data information constructed according to clause 9.3.1, then the MCPTT client shall determine per-group dynamic data of the MCPTT group in the MIME body.

5.7.3	Test description

5.7.3.1		Pre-test conditions

System Simulator:

-	SS (MCPTT server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCPTT operation in the MCPTT configuration document).

IUT:

-	UE (MCPTT client)

-	The test USIM set as defined in TS 36.579-1 [2], subclause 5.5.10 is inserted.

Preamble:

-	The UE has performed the Generic Test Procedure for MCPTT UE registration as specified in TS 36.579-1 [2], subclause 5.4.2.

-	The MCPTT User performs the Generic Test Procedure for MCPTT Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCPTT Client Application has been activated and User has registered-in as the MCPTT User with the Server as active user at the Client.

5.7.3.2	Test procedure sequence

Table 5.7.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the MCPTT User request the establishment of an MCPTT on-demand pre-arranged group call using Group A, automatic commencement mode, without implicit floor request. (NOTE 1) | - | - | - | - |
| 2 | The UE (MCPTT Client) performs Generic Test Procedure for MCPTT CO session establishment/modification without provisional responses other than 100 Trying as described in TS 36.579-1 [2] Table 5.3.7.3-1 to establish an MCPTT on-demand pre-arranged group call, automatic commencement mode, applying End-to-end communication security without implicit floor control according to option a of NOTE 1 in TS 36.579.1 [2] Table 5.3.7.3-1. | - | - | - | - |
| 3 | Void | - | - | - | - |
| 4 | Make the MCPTT User request to subscribe to group dynamic data for group call ongoing information. (NOTE 1) | - | - | - | - |
| 5-8 | Check: Does the UE (MCPTT Client) correctly perform steps 2 to 5 of Generic Test Procedure for MCPTT Subscription and Notification as described in TS 36.579-1 [2] Table 5.3.29.3-1 to subscribe to group dynamic data for group call ongoing information? | - | - | 1,2 | P |
| 9 | Make the MCPTT User request to de-subscribe to group dynamic data. (NOTE 1) | - | - | - | - |
| 10-11 | Check: Does the UE (MCPTT Client) correctly perform steps 2 to 3 of Generic Test Procedure for MCPTT Subscription and Notification as described in TS 36.579-1 [2] Table 5.3.29.3-1 to subscribe to group dynamic data for group call ongoing information? | - | - | 3 | P |
| 12-13 | Void | - | - | - | - |
| 14 | Make the MCPTT User end the on-demand group call. (NOTE 1) | - | - | - | - |
| 15 | The UE (MCPTT client) performs Generic Test Procedure for MCX CO call release as described in TS 36.579-1 [2] Table 5.3.10.3-1 to end the on-demand group call. | - | - |  |  |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

5.7.3.3		Specific message contents

Table 5.7.3.3-1: SIP SUBSCRIBE from the UE (step 5, Table 5.7.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3.29.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.14-1 with conditions MCPTT, PRESENCE-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Expires |  |  | TS 24.379 [9] clause 9.2.1.6 |  |
| value | "4294967295" |  |  |  |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT Info |  |  |
| MIME-part-headers |  |  |  |  |
| MIME-part-body | MCPTT-Info as described in Table 5.7.3.3-2 |  |  |  |
| MIME body part |  | SIMPLE-FILTER |  |  |
| MIME-part-headers |  |  |  |  |
| MIME-part-body | SIMPLE-FILTER as described in Table 5.7.3.3-3 |  |  |  |

Table 5.7.3.3-2: MCPTT-Info in SIP SUBSCRIBE (Table 5.7.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-request-uri | Encrypted <mcptt-request-uri> with mcpttURI set to px_MCPTT_Group_A_ID | Encrypted element as described in TS 36.579-1 [2] Table 5.5.3.2.1-1A |  |  |

Table 5.7.3.3-3: SIMPLE-FILTER in SIP SUBSCRIBE (Table 5.7.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.6-1, condition PER-GROUP

Table 5.7.3.3-4: SIP NOTIFY from the SS (step 7, Table 5.8.3.2-1;
step 4, TS 36.579-1 [2] Table 5.3.29.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.8-1 with condition PRESENCE-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | PIDF |  |  |
| MIME-part-body | PIDF as described in Table 5.7.3.3-5 |  |  |  |

Table 5.7.3.3-5: PIDF in SIP NOTIFY (Table 5.7.3.3-4)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.5.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| presence |  |  | TS 24.379 [9] clause 9.3.1 |  |
| entity attribute | Encrypted URI (NOTE 1) with value set to px_MCPTT_Group_A_ID |  | TS 24.379 [9] clause 9.3.1 |  |
| tuple |  |  | TS 24.379 [9] clause 9.3.1 |  |
| id attribute | Encrypted URI (NOTE 1) with value set to px_MCPTT_ID_User_A |  | TS 24.379 [9] clause 9.3.1 |  |
| status |  |  | TS 24.379 [9] clause 9.3.1 |  |
| affiliation |  |  | TS 24.379 [9] clause 9.3.1 |  |
| group | not present |  |  |  |
| client | Encrypted URI (NOTE 1) with value set to the mcptt-client-id as provided by the UE at registration |  | TS 24.379 [9] clause 9.3.1 |  |
| status | not present |  |  |  |
| expires | Current time plus one hour |  | TS 24.379 [9] clause 9.2.2.3.10 |  |
| tuple |  |  | TS 24.379 [9] clause 9.3.1 |  |
| id attribute | Encrypted URI (NOTE 1) with value set to px_MCPTT_ID_User_B |  | TS 24.379 [9] clause 9.3.1 |  |
| status |  |  | TS 24.379 [9] clause 9.3.1 |  |
| affiliation |  |  | TS 24.379 [9] clause 9.3.1 |  |
| group | not present |  |  |  |
| client | Encrypted URI (NOTE 1) with value set to px_MCPTT_Client_B_ID |  | TS 24.379 [9] clause 9.3.1 |  |
| status | not present |  |  |  |
| expires | Current time plus one hour |  | TS 24.379 [9] clause 9.2.2.3.10 |  |
| additionalData |  |  | TS 24.379 [9] clause 9.3.1 |  |
| groupCallOngoing | "true" |  | TS 24.379 [9] clause 9.3.1 |  |
| NOTE 1: Encrypted attribute as described in TS 36.579-1 [2], Table 5.5.13.3-1 |  |  |  |  |

Table 5.7.3.3-6: SIP SUBSCRIBE from the UE (step 10, Table 5.7.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3.29.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.14-1 with conditions MCPTT, PRESENCE-EVENT, re_SUBSCRIBE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Expires |  |  | TS 24.379 [9] clause 9.2.1.6 |  |
| value | "0" |  |  |  |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT Info |  |  |
| MIME-part-headers |  |  |  |  |
| MIME-part-body | MCPTT-Info as described in Table 5.7.3.3-2 |  |  |  |
| MIME body part |  | SIMPLE-FILTER |  |  |
| MIME-part-headers |  |  |  |  |
| MIME-part-body | SIMPLE-FILTER as described in Table 5.7.3.3-3 |  |  |  |