3GPP TSG-RAN5 Meeting #96-e	R5-223960

# Electronic Meeting, 15th – 26th August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-5 | CR | 0076 | rev | - | Current version: | 14.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Routine maintenance for TS 36.579-5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2022-07-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - At RAN5 TTCN Workshop #57 (28/04/2022) document R5w220111 has been presented and it has been agreed introduce tsc_MCPTT_SessionId and tsc_MCVideo_SessionId to be used as Contact URI in the MCPTT and MCVideo calls - tsc_MCData_SessionId has a path what is not a valid SIP URI according to the BNF of RFC 3261 - px_MCX_SIP_PublicUserId_C is missing but used in some test cases - tsc_MCX_SessionID_A is not used in any test case - tsc_MCVideo_PublicServiceId_PF_A and tsc_MCData_PublicServiceId_PF_A are missing - According to RFC 2616 clause 5.1.2 and as discussed at RAN5 TTCN Workshop #58 (06/07/2022, R5w220214) requests to a proxy require the Request-URI to be an absolute URI - Editorial issues |  |  |  |  |  |  |  |  |
| Summary of change: |  | - tsc_MCPTT_SessionId and tsc_MCVideo_SessionId added to Table 9.2.3-1 - tsc_MCData_SessionId changed to a valid SIP URI (according to agreed proposal 2.3-8 in R5w220205) - px_MCX_SIP_PublicUserId_C added to Table 9.2.1-1 - tsc_MCX_SessionID_A removed from Table 9.2.3-1 - tsc_MCVideo_PublicServiceId_PF_A and tsc_MCData_PublicServiceId_PF_A added - Clarification added to clause 7.2.1 that HTTP requests to KMS, CMS and GMS need absolute URIs as Request URI - Editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Invalid test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7.2.1, 9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modification>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document unless the context in which the reference is made suggests a different Release is relevant (information on the applicable release in a particular context can be found in e.g. test case title, description or applicability, message description or content).

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.579-1: "Mission Critical (MC) services over LTE; Part 1: Common test environment".

[3]	3GPP TS 36.579-2: "Mission Critical (MC) services over LTE; Part 2: Mission Critical Push To Talk (MCPTT) User Equipment (UE) Protocol conformance specification".

[4]	3GPP TS 36.579-3: "Mission Critical (MC) services over LTE; Part 3: Mission Critical Push To Talk (MCPTT) Server Application conformance specification".

[5]	3GPP TS 36.579-4: "Mission Critical (MC) services over LTE; Part 4: Test Applicability and Implementation Conformance Statement (ICS) proforma specification".

[6]	3GPP TS 36.523-1: "User Equipment (UE) conformance specification; Part 1: Protocol conformance specification"

[7]	3GPP TS 22.179: "Mission Critical Push To Talk (MCPTT) over LTE; Stage 1".

[8]	Void.

[9]	3GPP TS 24.379: "Mission Critical Push To Talk (MCPTT) call control; Protocol specification".

[10]	3GPP TS 24.380: "Mission Critical Push To Talk (MCPTT) floor control; Protocol specification".

[11]	3GPP TS 24.481: "Mission Critical Services (MCS) group management; Protocol specification".

[12]	3GPP TS 24.482: "Mission Critical Services (MCS) identity management; Protocol specification".

[13]	3GPP TS 24.483: "Mission Critical Services (MCS) Management Object (MO)".

[14]	3GPP TS 24.484: "Mission Critical Services (MCS) configuration management; Protocol specification".

[15]	3GPP TS 33.179: "Security of Mission Critical Push-To-Talk (MCPTT)".

[16]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[17]	3GPP TS 24.237: "IP Multimedia Subsystem (IMS) Service Continuity; Stage 3".

[18]	3GPP TS 29.468: "Group Communication System Enablers for LTE (GCSE_LTE); MB2 Reference Point; Stage 3".

[19]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[20]	3GPP TS 24.008: "Mobile Radio Interface Layer 3 specification; Core Network Protocols; Stage 3".

[21]	3GPP TS 23.003: "Numbering, addressing and identification".

[22]	ISO/IEC 9646-1: "Information technology - Open Systems Interconnection - Conformance testing methodology and framework - Part 1: General concepts".

[23]	ISO/IEC 9646-7: "Information technology - Open systems interconnection - Conformance testing methodology and framework - Part 7: Implementation Conformance Statements".

[24]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2".

[25]	IETF RFC 4566 (July 2006): "SDP: Session Description Protocol".

[26]	3GPP TS 26.171: "Speech codec speech processing functions; Adaptive Multi-Rate - Wideband (AMR-WB) speech codec; General description".

[27]	3GPP TS 36.523-3: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification; Part 3: Test suites".

[28]	3GPP TS 34.229-3: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 3: Abstract Test Suites (ATS)".

[29]	ISO/IEC 9646-1: "Information technology - Open Systems Interconnection - Conformance testing methodology and framework - Part 1: General concepts".

[30]	ISO/IEC 9646-7: "Information technology - Open systems interconnection - Conformance testing methodology and framework - Part 7: Implementation Conformance Statements".

[31]	ETSI ES 201 873: "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3".

[32]	IETF RFC 3711: "The Secure Real-time Transport Protocol (SRTP)".

[33]	3GPP TS 27.007: "AT command set for User Equipment (UE)".

[34]	IETF RFC 4661: "An Extensible Markup Language (XML)-Based Format for Event Notification Filtering".

[34]	IETF RFC 4826: "Extensible Markup Language (XML) Formats for Representing Resource Lists".

[35]	W3C: "XML Encryption Syntax and Processing Version 1.1", https://www.w3.org/TR/xmlenc-core1/.

[36]	W3C: "XML Signature Syntax and Processing (Second Edition)", http://www.w3.org/TR/xmldsig-core/.

[37]	OMA - poc_listService-v1_0: "List Service".

[40]	OMA - xdm_commonPolicy-V1_0: "XDM - Common Policy".

[39]	OMA - xdm_extensions-v1_0: "XDM - XDM2 - Extensions".

[40]	OMA - xdm_rsrclst_uriusage-v1_0: "Resource List - URI usage".

[41]	W3C: "XML Encryption Syntax and Processing Version 1.1", https://www.w3.org/TR/xmlenc-core1/.

[42]	W3C: "XML Signature Syntax and Processing (Second Edition)", http://www.w3.org/TR/xmldsig-core/.

[43]	3GPP TS 33.180: "Security of the mission critical service".

[44]	IETF RFC 6507: "Elliptic Curve-Based Certificateless Signatures for Identity-Based Encryption (ECCSI)".

[45]	IETF RFC 6508: "Sakai-Kasahara Key Encryption (SAKKE)".

[46]	IETF RFC 6509 (February 2012): ''MIKEY-SAKKE: Sakai-Kasahara Key Encryption in Multimedia Internet KEYing (MIKEY)''.

[47]	IETF RFC 3394: "Advanced Encryption Standard (AES) Key Wrap Algorithm".

[48]	W3C: "XML Signature Syntax and Processing (Second Edition)", http://www.w3.org/TR/xmldsig-core/.

[49]	IETF RFC 7515: "JSON Web Signature (JWS)".

[50]	IETF RFC 5261: "An Extensible Markup Language (XML) Patch Operations Framework Utilizing XML Path Language (XPath) Selectors".

[51]	IETF RFC 5874: "An Extensible Markup Language (XML) Document Format for Indicating a Change in XML Configuration Access Protocol (XCAP) Resources".

[52]	IETF RFC 4354: "A Session Initiation Protocol (SIP) Event Package and Data Format for Various Settings in Support for the Push-to-Talk over Cellular (PoC) Service".

[53]	IETF RFC 3986: "Uniform Resource Identifier (URI): Generic Syntax".

[54]	3GPP TS 23.280: "Common functional architecture to support mission critical services".

[55]	3GPP TS 24.281: "Mission Critical Video (MCVideo) signalling control; Protocol specification"

[56]	3GPP TS 24.581: "Mission Critical Video (MCVideo) media plane control; Protocol specification"

[57]	3GPP TS 24.282: "Mission Critical Data (MCData) signalling control; Protocol specification"

[58]	3GPP TS 24.582: "Mission Critical Data (MCData) media plane control; Protocol specification"

[59]	3GPP TS 36.579-6: "Mission Critical (MC) services over LTE; Part 6: Mission Critical Video (MCVideo) User Equipment (UE) Protocol conformance specification"

[60]	3GPP TS 36.579-7: "Mission Critical (MC) services over LTE; Part 7: Mission Critical Data (MCData) User Equipment (UE) Protocol conformance specification"

[61]	3GPP TS 36.508: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); Common Test Environments for User Equipment (UE) Conformance Testing".

[62]	IETF RFC 4122: "A Universally Unique IDentifier (UUID) URN Namespace"

[63]	IETF RFC 5245: "Interactive Connectivity Establishment (ICE): A Protocol for Network Address Translator (NAT) Traversal for Offer/Answer Protocols"

[64]	IETF RFC 3830: "MIKEY: Multimedia Internet KEYing".

[65]	IETF RFC 6509 (February 2012): ''MIKEY-SAKKE: Sakai-Kasahara Key Encryption in Multimedia Internet KEYing (MIKEY)''.

[66]	IETF RFC 6043: "MIKEY-TICKET: Ticket-Based Modes of Key Distribution in Multimedia Internet KEYing (MIKEY)".

[67]	IETF RFC 3550: "RTP: A Transport Protocol for Real-Time Applications".

[68]	IETF RFC 4975: "The Message Session Relay Protocol (MSRP)".

[69]	IETF RFC 2616: "Hypertext Transfer Protocol -- HTTP/1.1".

<End of modification>

<Start of modification>

### 7.2.1	HTTP-Signalling

Figure 7.2.1-1 shows the functional architecture for HTTP-Signalling between the MCX-capable UE and different network entities as emulated by the test models.

Figure 7.2.1-1: Functional architecture for HTTP-Signalling

The UE is expected to establish HTTP connections to the following entities:

-	Initial Configuration Server: 
The UE gets configured with IP address (px_MCX_InitialConfigServer_IPAddress) and port number (px_MCX_InitialConfigServer_Port) of the server from which it retrieves the MCS UE initial configuration document (TS 24.484 [14] clause 7.2).
It depends on UE implementation how the IP address and port are configured at the UE but the UE is expected to support this to be configured. 
As there is no requirement by the core specs the HTTP connection is not security protected.

-	IdMS AUTH Endpoint:
The UE retrieves IP address (px_MCX_IdMS_auth_IPAddress) and port number (px_MCX_IdMS_auth_Port) from the MCS UE initial configuration document (TS 24.484 [14] clause 7.2).
The connection shall be TLS protected.

-	IdMS TOKEN Endpoint:
The UE retrieves IP address (px_MCX_IdMS_token_IPAddress) and port number (px_MCX_IdMS_token_Port) from the MCS UE initial configuration document (TS 24.484 [14] clause 7.2).
The connection shall be TLS protected.

-	HTTP Proxy:
The UE retrieves IP address (px_MCX_HTTP_Proxy_IPAddress) and port number (px_MCX_HTTP_Proxy_Port) from the MCS UE initial configuration document (TS 24.484 [14] clause 7.2).
The connection shall be TLS protected.

Design considerations and working assumptions:

-	Domain Name Server 
DNS is not needed for address resolution as in the URIs of the network entities in the MCS UE initial configuration document the IP address of the respective entity can be used as host name.

-	Initial UE configuration
The UE is able to retrieve the MCS UE initial configuration document (TS 24.484 [14] clause 7.2) via an HTTP connection to an Initial Configuration Server. Other means of initial configuration are out of scope for conformance testing.

-	HTTP signalling between UE and KMS
The UE is expected to support HTTP signalling with the KMS via the HTTP proxy using a transport key (TrK) according to TS 33.180 [43] clause 4.3.3 and clause 5.3.2.

-	Use of TLS
All HTTP traffic between the UE and the HTTP proxy and between the UE and the IdMS is protected using HTTPS (TS 33.180 [43] clause 5.1.3.1, clause 6.1.1 and B.12).
NOTE: Even though NOTE 2 of TS 24.482 [12] clause 6.2.1 might be interpreted as if the HTTP signalling to the authorization endpoint may not be TLS secured, 33.180 [43] mandates TLS to be used.

-	IP Addresses and port numbers
IP addresses and port numbers of the network entities as shown in figure 7.2.1-1 are given as PIXIT (see clause 9). As only restriction the tuple {IP address, port number} needs to be unique for each entity, but there is no restriction no to use one and the same IP address for all entities as long as they use different port numbers.

-	According to TS 24.282 [57] clause 10.2.2.1 the HTTP signalling between the MCData client and the media storage function (MSF) happens via an HTTP proxy according to annex A of TS 24.482 [12]. It is assumed that this is the same proxy as used for signalling with the KMS, CMS and GMS.

-	According to RFC 2616 [69] clause 5.1.2 HTTP requests via an HTTP proxy require the Request-URI to be an absolute URI  HTTP requests to KMS, CMS and GMS shall have an absolute URI as Request-URI.

<End of modification>

<Start of modification>

# 9	IXIT Proforma

## 9.1	General

This partial IXIT proforma contained in the present document is provided for completion, when the related Abstract Test Suite is to be used against the Implementation Under Test (IUT).

Text in italics is a comment for guidance for the production of an IXIT, and is not to be included in the actual IXIT.

The completed partial IXIT will normally be used in conjunction with the completed ICS, as it adds precision to the information provided by the ICS.

## 9.2	MCX

### 9.2.1	MCX Client PIXIT

Table 9.2.1-1: MCX Client Common PIXIT

| Parameter Name |  |  | Parameter Type |  |  | Default Value |  |  | Supported Values |  |  | Description |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Client relevant IXIT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| px_MCX_Client_B_ID |  |  | charstring |  |  | "urn:uuid:cdcd34fb-c2f1-4488-a03a-6b5a5d360c47" |  |  |  |  |  | MCX client ID for a remote user: As defined in TS 24.379 [9] clause 4.10 it is a UUID URN according to RFC 4122 [62], i.e. has the format "urn:uuid:XXXXXXXX-YYYY-ZZZZ-yyyy-zzzzzzzzzzzz" |  |  |
| Users relevant IXIT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| px_MCPTT_ID_User_A |  |  | charstring |  |  | "mcptt-user-A-id@ mcptt-op.gov" |  |  | NOTE 1 |  |  | MCPTT user identity (MCPTT ID) which is a globally unique identifier within the MCPTT service that represents the MCPTT user. Ref. TS 24.483 [13]. |  |  |
| px_MCVideo_ID_User_A |  |  | charstring |  |  |  |  |  | NOTE 1 |  |  | MCVideo user identity (MCVideo ID) which is a globally unique identifier within the MCVideo service that represents the MCVideo user. Ref. TS 24.483 [13]. |  |  |
| px_MCData_ID_User_A |  |  | charstring |  |  |  |  |  | NOTE 1 |  |  | MCData user identity (MCData ID) which is a globally unique identifier within the MCData service that represents the MCData user. Ref. TS 24.483 [13]. |  |  |
| px_MCPTT_User_A_Alias |  |  | charstring |  |  | "mcptt-user-A-alias" |  |  |  |  |  | Alphanumeric alias of MCPTT user. Ref. TS 24.483 [13]. |  |  |
| px_MCVideo_User_A_Alias |  |  | charstring |  |  | "mcvideo-user-A-alias" |  |  |  |  |  | Alphanumeric alias of MCVideo user. Ref. TS 24.483 [13]. |  |  |
| px_MCData_User_A_Alias |  |  | charstring |  |  | "mcdata-user-A-alias" |  |  |  |  |  | Alphanumeric alias of MCData user. Ref. TS 24.483 [13]. |  |  |
| px_MCPTT_ID_FA_A |  |  | charstring |  |  | "mcptt-FA-A-id@ mcptt-op.gov" |  |  |  |  |  | The value is a "uri" attribute. TS 24.483 [13] clause 5.2.48W6 |  |  |
| px_MCX_User_A_ParticipantType |  |  | charstring |  |  | "first responder" |  |  |  |  |  | Participant type of the MCPTT user. Ref. TS 24.483 [13]. |  |  |
| px_MCX_DomainName_Organization_A |  |  | charstring |  |  | "mcptt-op.gov" |  |  |  |  |  | Domain name of the organization the user belongs to. |  |  |
| px_MCX_User_A_username |  |  | charstring |  |  | "MCPTT#U01" |  |  |  |  |  | UE's User username used for user authentication |  |  |
| px_MCX_User_A_password |  |  | charstring |  |  | "psw@MCPTT&7" |  |  |  |  |  | UE's User password used for user authentication |  |  |
| px_MCX_TokenRsaPrivateKey |  |  | octetstring |  |  |  |  |  |  |  |  | Private key being used to create signature for ID and Access Token |  |  |
| px_MCX_TrK |  |  | bitstring |  |  |  |  |  |  |  |  | Transport key for XML confidentiality and between the KMS and the MC KM client according to TS 33.180 [43] clause 9.3.3. |  |  |
| px_MCX_TrK_ID |  |  | B32_Type |  |  |  |  |  |  |  |  | Transport key ID for XML confidentiality and between the KMS and the MC KM client according to TS 33.180 [43] clause 9.3.3. |  |  |
| px_MCX_InK |  |  | bitstring |  |  |  |  |  |  |  |  | Integrity protection key for XML confidentiality and between the KMS and the MC KM client according to TS 33.180 [43] clause 9.3.3. |  |  |
| px_MCX_InK_ID |  |  | B32_Type |  |  |  |  |  |  |  |  | Integrity protection key ID for XML confidentiality and between the KMS and the MC KM client according to TS 33.180 [43] clause 9.3.3. |  |  |
| px_MCPTT_ID_User_B |  |  | charstring |  |  | "mcptt-user-B-id@ mcptt-op.gov" |  |  |  |  |  | MCPTT user identity (MCPTT ID) which is a globally unique identifier within the MCPTT service that represents the MCPTT user. Ref. TS 24.483 [13]. |  |  |
| px_MCVideo_ID_User_B |  |  | charstring |  |  | "mcvideo-user-B-id@ mcptt-op.gov" |  |  |  |  |  | MCVideo user identity (MCVideo ID) which is a globally unique identifier within the MCVideo service that represents the MCVideo user. Ref. TS 24.483 [13]. |  |  |
| px_MCData_ID_User_B |  |  | charstring |  |  | "mcdata-user-B-id@ mcptt-op.gov" |  |  |  |  |  | MCData user identity (MCData ID) which is a globally unique identifier within the MCData service that represents the MCData user. Ref. TS 24.483 [13]. |  |  |
| px_MCX_User_B_ParticipantType |  |  | charstring |  |  | "first responder" |  |  |  |  |  | Participant type of the MCX user. Ref. TS 24.483 [13]. |  |  |
| px_MCPTT_ID_User_C |  |  | charstring |  |  | "mcptt-user-C-id@ mcptt-op.gov" |  |  |  |  |  | MCPTT user identity (MCPTT ID) which is a globally unique identifier within the MCPTT service that represents the MCPTT user. Ref. TS 24.483 [13]. |  |  |
| px_MCVideo_ID_User_C |  |  | charstring |  |  | "mcvideo-user-C-id@ mcptt-op.gov" |  |  |  |  |  | MCVideo user identity (MCVideo ID) which is a globally unique identifier within the MCVideo service that represents the MCVideo user. Ref. TS 24.483 [13]. |  |  |
| px_MCData_ID_User_C |  |  | charstring |  |  | "mcdata-user-C-id@ mcptt-op.gov" |  |  |  |  |  | MCData user identity (MCData ID) which is a globally unique identifier within the MCData service that represents the MCData user. Ref. TS 24.483 [13]. |  |  |
| px_MCX_User_C_ParticipantType |  |  | charstring |  |  | "first responder" |  |  |  |  |  | Participant type of the MCX user. Ref. TS 24.483 [13]. |  |  |
| px_MCPTT_ID_User_D |  |  | charstring |  |  | "mcptt-user-D-id@ mcptt-op.gov" |  |  |  |  |  | MCPTT user identity (MCPTT ID) which is a globally unique identifier within the MCPTT service that represents the MCPTT user. Ref. TS 24.483 [13]. |  |  |
| px_MCVideo_ID_User_D |  |  | charstring |  |  | "mcvideo-user-D-id@ mcptt-op.gov" |  |  |  |  |  | MCVideo user identity (MCVideo ID) which is a globally unique identifier within the MCVideo service that represents the MCVideo user. Ref. TS 24.483 [13]. |  |  |
| px_MCData_ID_User_D |  |  | charstring |  |  | "mcdata-user-D-id@ mcptt-op.gov" |  |  |  |  |  | MCData user identity (MCData ID) which is a globally unique identifier within the MCData service that represents the MCData user. Ref. TS 24.483 [13]. |  |  |
| px_MCX_SIP_PublicUserId_A_1 |  |  | charstring |  |  |  |  |  |  |  |  | (First) public user id of the SIP subscriber (user A) as stored in the HSS and on the ISIM, if the UE has an ISIM; the parameter shall be a SIP URI |  |  |
| px_MCX_SIP_PrivateUserId_A |  |  | charstring |  |  |  |  |  |  |  |  | Private user id of the SIP subscriber (user A) as stored in the HSS and on the ISIM, if the UE has an ISIM |  |  |
| px_MCX_SIP_HomeDomain_A |  |  | charstring |  |  |  |  |  |  |  |  | Home domain name of the SIP subscriber (user A) as stored in the HSS and on the ISIM, if the UE has an ISIM |  |  |
| px_MCX_SIP_PublicUserId_B |  |  | charstring |  |  |  |  |  |  |  |  | Public user id of user B |  |  |
| px_MCX_SIP_PublicUserId_C |  |  | charstring |  |  |  |  |  |  |  |  | Public user id of user C |  |  |
| px_MCX_SIP_RegistrationWithTemporaryIdentities |  |  | boolean |  |  |  |  |  |  |  |  | If true the UE shall derive the public user id, private user id and home domain name from the IMSI for SIP registration |  |  |
| Groups relevant IXIT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| px_MCPTT_Group_A_ID |  |  | charstring |  |  | "mcptt-group-A@mcptt-op.gov" |  |  |  |  |  | Group ID for a group. Value is an "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 that indicates the group id. Ref. TS 24.483 [13]. |  |  |
| px_MCVideo_Group_A_ID |  |  | charstring |  |  | "mcvideo-group-A@mcptt-op.gov" |  |  |  |  |  | Group ID for a group. Value is an "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 that indicates the group id. Ref. TS 24.483 [13]. |  |  |
| px_MCData_Group_A_ID |  |  | charstring |  |  | "mcdata-group-A@mcptt-op.gov" |  |  |  |  |  | Group ID for a group. Value is an "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 that indicates the group id. Ref. TS 24.483 [13]. |  |  |
| px_MCPTT_Group_A_Name |  |  | charstring |  |  | "mcptt-group-A-name" |  |  |  |  |  | A human readable Group name for the group |  |  |
| px_MCVideo_Group_A_Name |  |  | charstring |  |  | "mcvideo-group-A-name" |  |  |  |  |  | A human readable Group name for the group |  |  |
| px_MCData_Group_A_Name |  |  | charstring |  |  | "mcdata-group-A-name" |  |  |  |  |  | A human readable Group name for the group |  |  |
| px_MCX_Group_A_Owner_Organization |  |  | charstring |  |  | "mcx-op.gov" |  |  |  |  |  | Indicates the group's owner organization the group belongs to. Ref. TS 24.483 [13]. |  |  |
| px_MCPTT_Group_A_preferred_VCodec |  |  | charstring |  |  | "AMR-WB" |  |  |  |  |  | Preferred voice codec for the group (a RTP payload). MCPTT clients shall support the AMR-WB codec. RFC 4566 [25] TS 26.171 [26] |  |  |
| px_MCPTT_Group_B_ID |  |  | charstring |  |  | "mcptt-group-B@mcptt-op.gov" |  |  |  |  |  | Group ID for a group. Value is an "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 that indicates the group id. Ref. TS 24.483 [13]. |  |  |
| px_MCVideo_Group_B_ID |  |  | charstring |  |  | "mcvideo-group-B@mcptt-op.gov" |  |  |  |  |  | Group ID for a group. Value is an "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 that indicates the group id. Ref. TS 24.483 [13]. |  |  |
| px_MCData_Group_B_ID |  |  | charstring |  |  | "mcdata-group-B@mcptt-op.gov" |  |  |  |  |  | Group ID for a group. Value is an "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 that indicates the group id. Ref. TS 24.483 [13]. |  |  |
| px_MCPTT_Group_T_ID |  |  | charstring |  |  | "mcptt-group-T@mcptt-op.gov" |  |  |  |  |  | Group ID for a temporary group. Value is an "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 that indicates the group id. Ref. TS 24.483 [13]. |  |  |
| px_MCVideo_Group_T_ID |  |  | charstring |  |  | "mcvideo-group-T@mcptt-op.gov" |  |  |  |  |  | Group ID for a temporary group. Value is an "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 that indicates the group id. Ref. TS 24.483 [13]. |  |  |
| px_MCData_Group_T_ID |  |  | charstring |  |  | "mcdata-group-T@mcptt-op.gov" |  |  |  |  |  | Group ID for a temporary group. Value is an "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 that indicates the group id. Ref. TS 24.483 [13]. |  |  |
| Miscellaneous IXIT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| px_MCX_APN |  |  | charstring |  |  | "mcx-apn" |  |  |  |  |  | A single APN which the UE shall use to access each and all MCX relevant services. The APN is provided in the initial UE configuration as specified in TS 36.579-1 [2] Table 5.5.8.1-1. |  |  |
| px_MCX_InitialRegistration_TypeOfPDN1 |  |  | MCX_Registration_PDN_Type |  |  | mcx |  |  | ims, internet, mcx |  |  | First PDN registered during initial registration (either ‘ims’ or ‘internet’ or ‘mcx’; ‘none’ is not applicable as first PDN) |  |  |
| px_MCX_InitialRegistration_TypeOfPDN2 |  |  | MCX_Registration_PDN_Type |  |  | none |  |  | ims, internet, mcx, none |  |  | Second PDN registered during initial registration; in addition to ‘ims’ or ‘internet’ or ‘mcx’ it may be ‘none’ to indicate that there is no second PDN connectivity requested by the UE during initial registration |  |  |
| px_MCX_InitialRegistration_TypeOfPDN3 |  |  | MCX_Registration_PDN_Type |  |  | none |  |  | ims, internet, mcx, none |  |  | Third PDN registered during initial registration; in addition to ‘ims’ or ‘internet’ or ‘mcx’ it may be ‘none’ to indicate that there is no third PDN connectivity requested by the UE during initial registration |  |  |
| px_MCX_CoordinateLatitude_Client_B |  |  | float |  |  | none |  |  |  |  |  | Simulated latitude value sent to the UE to inform about Client_B location |  |  |
| px_MCX_CoordinateLongitude_Client_B |  |  | float |  |  | none |  |  |  |  |  | Simulated longitude value sent to the UE to inform about Client_B location |  |  |
| NOTE 1: According to TS 23.280 [54] clause 8.1.2 a MC service ID shall be a URI; nevertheless in context of this specification only URIs consisting of userinfo, host and path elements (see RFC 3986 [53]) are supported. Editor's note: A similar note may need to be added to other PIXITs too |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 9.2.2	MCX Server PIXIT

Table 9.2.2-1: MCX Server Common PIXIT

| Parameter Name | Parameter Type | Default Value | Supported Values | Description |
| --- | --- | --- | --- | --- |
| px_MCX_GroupCreationXUI | charstring | "mcx-gms@mcptt-op.gov" |  | Indicates the group creation XUI information for creation of groups. Ref. TS 23.003 [21]. |
| px_MCX_IdMS_auth_IPAddress | charstring |  |  | IPv4/IPv6 address of the IdMS authorization endpoint |
| px_MCX_IdMS_auth_Port | integer |  |  | Port number of the IdMS authorization endpoint |
| px_MCX_IdMS_auth_Certificate | charstring |  |  | Identifier or file name of the certificate to be used during establishment of the TLS tunnel to the IdMS authorization endpoint |
| px_MCX_IdMS_token_IPAddress | charstring |  |  | IPv4/IPv6 address of the IdMS token endpoint |
| px_MCX_IdMS_token_Port | integer |  |  | Port number of the IdMS token endpoint |
| px_MCX_IdMS_token_Certificate | charstring |  |  | Identifier or file name of the certificate to be used during establishment of the TLS tunnel to the IdMS token endpoint |
| px_MCX_HTTP_Proxy_IPAddress | charstring |  |  | IPv4/IPv6 address of the HTTP proxy |
| px_MCX_HTTP_Proxy_Port | integer |  |  | Port number of the HTTP proxy |
| px_MCX_HTTP_Proxy_Certificate | charstring |  |  | Identifier or file name of the certificate to be used during establishment of the TLS tunnel to the HTTP proxy |
| px_MCX_InitialConfigServer_IPAddress | charstring |  |  | IPv4/IPv6 address of the server optionally being used to download the Initial UE Configuration document |
| px_MCX_InitialConfigServer_Port | integer |  |  | Port number of the server optionally being used to download the Initial UE Configuration document |
| px_MCX_InitialConfigServer_UriPath | charstring | "/cms/initial-ue-config" |  | URI Path component: Absolute path used for HTTP requests addressing a server to download the UE initial configuration document |
| px_MCX_TLS_CipherSuite | TLS_CIPHER_Type | TLS_RSA_WITH_NULL_SHA |  | Cipher suite to be used for TLS connections |
| px_MCX_OAuth_ClientId_A | charstring |  |  | Client ID of the UE's MCX application as used in OAuth signalling with the IdMS |
| px_MCX_OAuth_RedirectURI_A | charstring |  |  | Redirect URI used by the UE implementation (user agent, MCX client and OS) to redirect the OAuth authentication response to the MCX client application |

### 9.2.3	MCX Constant PIXIT Definitions

Several parameters for MCX conformance testing can be defined as constants as they are neither preconfigured at the UE nor at the SS. Table 9.2.3-1 lists these constants.

Table 9.2.3-1: MCX Constant PIXIT Definitions

| Constant Name |  | Constant Type |  | Value |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| tsc_MCX_KMS_Hostname |  | charstring |  | "kms." & px_MCX_DomainName_Organization_A |  | FQDN of the KMS; used in initial UE configuration as domain name for the 'kms' URI in the App-Server-Info. |  |
| tsc_MCX_CMS_Hostname |  | charstring |  | "cms." & px_MCX_DomainName_Organization_A |  | FQDN of the CMS; used in initial UE configuration as domain name for the 'cms' URI in the App-Server-Info. |  |
| tsc_MCX_GMS_Hostname |  | charstring |  | "gms." & px_MCX_DomainName_Organization_A |  | FQDN of the GMS; used in initial UE configuration as domain name for the 'gms' URI in the App-Server-Info. |  |
| tsc_MCX_KMS_CertUri |  | charstring |  | "certificate1." & tsc_MCX_KMS_Hostname |  | Name of the KMS certificate sent to the UE during MCX user authentication |  |
| tsc_MCX_IdMS_auth_UriPath |  | charstring |  | "/idms/auth" |  | URI path to address the IdMS authorization endpoint |  |
| tsc_MCX_IdMS_token_UriPath |  | charstring |  | "/idms/token" |  | URI path to address the IdMS token endpoint |  |
| tsc_MCX_IdMS_userauth_UriPath |  | charstring |  | "/idms/userauth" |  | relative URI at IdMS used for user authentication |  |
| tsc_MCX_KMS_init_UriPath |  | charstring |  | "/keymanagement/identity/v1/init" |  | Path of the Request-URI for "KMS Initialize" request according to TS 33.180 [43] D.2.3 |  |
| tsc_MCX_KMS_keyprov_UriPath |  | charstring |  | "/keymanagement/identity/v1/keyprov" |  | Path of the Request-URI for "KMS KeyProvision" request according to TS 33.180 [43] D.2.4 |  |
| tsc_MCX_KMS_ClientReqUrl_init |  | charstring |  | "https://" & tsc_MCX_KMS_Hostname & tsc_MCX_KMS_init_UriPath |  | used as <ClientReqUrl> entry in the KMS Key Set and as Request-URI for "KMS Initialize" request according to TS 33.180 [43] D.2.3 (NOTE 1) |  |
| tsc_MCX_KMS_ClientReqUrl |  | charstring |  | "https://" & tsc_MCX_KMS_Hostname & tsc_MCX_KMS_keyprov_UriPath |  | used as <ClientReqUrl> entry in the KMS Key Set and as Request-URI for "KMS KeyProvision" request according to TS 33.180 [43] D.2.4 (NOTE 1) |  |
| tsc_MCX_GMSURI |  | charstring |  | "subscription-proxy." & tsc_MCX_GMS_Hostname |  | used for <GMS-URI> element in the MCS UE initial configuration document according to TS 24.484 [14] clause 7.2.2.7 and TS 24.483 [13] clause 8.2.9; sub-domain of the GMS's hostname |  |
| tsc_MCX_CMSXCAPRootURI |  | charstring |  | "http://xcap." & tsc_MCX_CMS_Hostname |  | used for <CMS-XCAP-root-URI> element in the MCS UE initial configuration document according to TS 24.484 [14] clause 7.2.2.7 and TS 24.483 [13] clause 8.2.9C |  |
| tsc_MCX_GMSXCAPRootURI |  | charstring |  | "http://xcap." & tsc_MCX_GMS_Hostname |  | used for <GMS-XCAP-root-URI> element in the MCS UE initial configuration document according to TS 24.484 [14] clause 7.2.2.7 and TS 24.483 [13] clause 8.2.9B |  |
| tsc_MCX_MC_ID_User_A |  | charstring |  | px_MCX_User_A_username |  | According to TS 24.482 [12] clause 6.3.1 the MC ID set to the user name |  |
| tsc_MCPTT_PublicServiceId_A |  | charstring |  | "sip:" & "mcptt-server-A@" & px_MCX_DomainName_Organization_A |  | The URI of the MCPTT Server which is simulated by the SS |  |
| tsc_MCVideo_PublicServiceId_A |  | charstring |  | "sip:" & "mcvideo-server-A@" & px_MCX_DomainName_Organization_A |  | The URI of the MCVideo Server which is simulated by the SS |  |
| tsc_MCData_PublicServiceId_A |  | charstring |  | "sip:" & "mcdata-server-A@" & px_MCX_DomainName_Organization_A |  | The URI of the MCData Server which is simulated by the SS |  |
| tsc_MCPTT_PublicServiceId_B |  | charstring |  | "sip:" & "mcptt-server-B@" & px_MCX_DomainName_Organization_A |  | The URI of a second MCPTT Server which is implemented in the DUT used in MCPTT Server testing. |  |
| tsc_MCPTT_PublicServiceId_PF_A |  | charstring |  | "sip:" & "participating-mcptt-function-A@" & px_MCX_DomainName_Organization_A |  | The URI of the participating MCPTT function which configures the location reporting at the UE according to TS 24.379 [9] clause 13.2.2. |  |
| tsc_MCVideo_PublicServiceId_PF_A |  | charstring |  | "sip:" & "participating-mcvideo-function-A@" & px_MCX_DomainName_Organization_A |  | The URI of the participating MCVideo function which configures the location reporting at the UE according to TS 24.281 [55] clause 18.2.2. |  |
| tsc_MCData_PublicServiceId_PF_A |  | charstring |  | "sip:" & "participating-mcdata-function-A@" & px_MCX_DomainName_Organization_A |  | The URI of the participating MCData function which configures the location reporting at the UE according to TS 24.282 [57] clause 17.2.2. |  |
| tsc_MCX_SessionID_B |  | charstring |  | "sip:" & "sessionB@cf." & px_MCX_DomainName_Organization_A |  | The URI of the MCPTT session B identity. Ref. TS 24.483 [13]. SIP URI according to TS 24.379 [9] clause 4.5. |  |
| tsc_MCPTT_SessionId |  | charstring |  | "sip:" & "mcptt-session-A@cf." & px_MCX_DomainName_Organization_A |  | Service Id for MCPTT calls as used as Contact URI of the MCPTT server |  |
| tsc_MCVideo_SessionId |  | charstring |  | "sip:" & "mcvideo-session-A@cf." & px_MCX_DomainName_Organization_A |  | Service Id for MCVideo calls as used as Contact URI of the MCVideo server |  |
| tsc_MCData_SessionId |  | charstring |  | "sip:" & "mcdata-session-A@cf." & px_MCX_DomainName_Organization_A |  | Service Id for MCData calls as used as Contact URI of the MCData server |  |
| tsc_MCData_MSF_Hostname |  | charstring |  | "msf." & px_MCX_DomainName_Organization_A |  | Hostname of the message storage function (MSF) |  |
| tsc_MCData_MSF_URI |  | charstring |  | "http://" & tsc_MCData_MSF_Hostname & "/userA/files" |  | Absolute URI of the message storage function (MSF) |  |
| tsc_MCPTT_Group_A_ProSeLayer2GroupID |  | octetstring |  | '00000A'O |  | Indicates the Prose layer-2 group ID for the group. Ref. TS 23.303 [24]. |  |
| tsc_MCPTT_Group_D_ProSeLayer2GroupID |  | octetstring |  | '00000D'O |  | Indicates the Prose layer-2 group ID for the group. Ref. TS 23.303 [24]. |  |
| NOTE 1: According to the NOTE at step 0 in clause 5.2.3 of TS 33.180 [43] the connection between the KMS and the HTTP Proxy is secured and therefore the URIs of the KMS are assumed to have “https” rather than “http” as scheme. |  |  |  |  |  |  |  |

## 9.3	MCX IPCAN test model

Table 9.3-1: MCX IPCAN test model specific PIXIT

| Parameter Name |  | Parameter Type |  | Default Value |  | Supported Values |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| px_MCX_IPCAN_EpsBearerId_IMS |  | MCX_IPCAN_EpsBearerId_Type |  | 5 |  | 5..13 |  | Default EPS bearer for IMS PDN (if any) |  |
| px_MCX_IPCAN_EpsBearerId_INTERNET |  | MCX_IPCAN_EpsBearerId_Type |  | 6 |  | 5..13 |  | Default EPS bearer for Internet PDN (if any) |  |
| px_MCX_IPCAN_EpsBearerId_MCX |  | MCX_IPCAN_EpsBearerId_Type |  | 7 |  | 5..13 |  | Default EPS bearer for MCX PDN |  |
| px_MCX_IPCAN_DedicatedEpsBearerId_MCPTT |  | MCX_IPCAN_EpsBearerId_Type |  | 8 |  | 5..13 |  | Dedicated EPS bearer for MCPTT |  |
| px_MCX_IPCAN_DedicatedEpsBearerId_MCVideo |  | MCX_IPCAN_EpsBearerId_Type |  | 9 |  | 5..13 |  | Dedicated EPS bearer for MCVideo |  |
| px_MCX_IPCAN_DedicatedEpsBearerId_MCData |  | MCX_IPCAN_EpsBearerId_Type |  | 10 |  | 5..13 |  | Dedicated EPS bearer for MCData |  |

<End of modification>