3GPP TSG-RAN5 Meeting #96-e	R5-224029

# Online, , 15th Aug 2022 - 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3019 | rev | - | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to 5GC test case 9.1.6.1.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, TDIA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-07-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | "Non-suitable cell" may cause calibration issue on FR2, which can be avoided by replacing with "Non-Suitable “Off” cell". |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Replacing "Non-suitable cell" with "Non-Suitable “Off” cell". 2. Editorial updating |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.6.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

9.1.6.1.3	UE-initiated de-registration / Abnormal / Change of cell into a new tracking area

9.1.6.1.3.1	Test Purpose (TP)

(1)

with { UE in 5GMM-DEREGISTERED-INITIATED state and de-registration request is not due to switch off }

ensure that {

when { UE changes into a new tracking area that is not in the stored TAI list }

then { UE aborts the de-registration procedure and initiates a mobility registration procedure }

}

(2)

with { UE in 5GMM-REGISTERED-INITIATED state for a mobility registration procedure due to change of cell into a new tracking area list during UE-initiated de-registration procedure }

ensure that {

when { UE receives REGISTRATION ACCEPT message }

then { UE re-initiates the de-registration procedure after completing the mobility registration procedure }

}

(3)

Void

9.1.6.1.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.501, clause 5.2.2.6. Unless otherwise stated these are Rel-15 requirements.

[TS 24.501 clause 5.2.2.6]

The following abnormal cases can be identified:

…

f)	Change of cell into a new tracking area.

If a cell change into a new tracking area that is not in the stored TAI list occurs before the UE-initiated de-registration procedure is completed, the de-registration procedure shall be aborted and re-initiated after successfully performing a registration procedure for mobility or periodic update used for mobility (i.e. the 5GS registration type IE set to "mobility registration updating" in the REGISTRATION REQUEST message). If the de-registration procedure was initiated due to removal of the USIM or the UE is to be switched off, the UE shall abort the de-registration procedure and enter the state 5GMM-DEREGISTERED.

9.1.6.1.3.3	Test description

9.1.6.1.3.3.1	Pre-test conditions

System Simulator:

-	NGC Cell A belongs to Home PLMN and TAI-1 and set as serving cell;

-	NGC Cell B belongs to Home PLMN and TAI-2 and set as Non-suitable “Off” cell.

UE:

-	None;

Preamble:

-	The UE is in state 3N-A on NGC cell A according to TS 38.508-1 [4].

9.1.6.1.3.3.2	Test procedure sequence

Table 9.1.6.1.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | The following messages are to be observed on NGC Cell A unless explicitly stated otherwise. | - | - | - | - |
| 1 | Cause UE to de-register for non Switch off reason using MMI or AT commands. | - | - | - | - |
| 2 | The UE transmits DEREGISTRATION REQUEST message. (The UE starts timer T3521) | --> | DEREGISTRATION REQUEST |  |  |
| 3 | The SS does not respond to DEREGISTRATION REQUEST message. | - | - | - | - |
| 4 | The SS configures NGC Cell A as the " Suitable neighbour intra-frequency cell " and NGC Cell B as the "Serving cell". Note: T3521 value is specified as 15s in TS 24.501 [22] and it is assumed that SS can configure cells within this time. | - | - | - | - |
| 5 | The SS transmits an RRCReconfiguration message on NGC Cell A to order the UE to perform intra-frequency handover to NGC Cell B. | - | - | - | - |
| 6 | The UE transmits a RRCReconfigurationComplete message on NGC Cell B to confirm the successful completion of the intra frequency handover. | - | - | - | - |
| - | The following messages are to be observed on NGC Cell B unless explicitly stated otherwise. | - | - | - | - |
| 7 | Check: Does the UE transmit a REGISTRATION REQUEST message with registration type value set to “mobility registration updating” | --> | REGISTRATION REQUEST | 1 | P |
| 8 -9 | Steps 4 to 5 of the generic procedure in TS 38.508-1 [4] subclause 4.9.5.2.2 | - | - | - | - |
| 10-22 | Void |  |  |  |  |
| 23 | Check: Does the UE transmit DEREGISTRATION REQUEST message? | --> | DEREGISTRATION REQUEST | 2 | P |
| 24 | The SS transmits DEREGISTRATION ACCEPT message. | <-- | DEREGISTRATION ACCEPT | - | - |
| 25 | The SS transmits an RRCRelease message. | - | - | - | - |
| 26 | Check: Does the test result of the generic procedure in TS 38.508-1 [4] subclause 4.9.13 indicates that the UE doesn’t responds to paging when paged with NG-5G-S-TMSI? | - | - | 2 | P |
| 27-35 | Void | - | - | - | - |

9.1.6.1.3.3.3	Specific message contents

Table 9.1.6.1.3.3.3-1: DEREGISTRATION REQUEST (steps 2 and 23, Table 9.1.6.1.3.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| De-registration type |  |  |  |  |
| Switch off |  | ‘0’B | Normal de-registration |  |
| Re-registration required |  | ‘0’B | Spare bit |  |
| Access type |  | ‘01’B | 3GPP access |  |
| 5GS mobile identity |  | 5G-GUTI | Same value as assigned in REGISTRATION ACCEPT |  |

Table 9.1.6.1.3.3.3-2: RRCReconfiguration-HO (step 5, Table 9.1.6.1.3.3.2-1)

Derivation Path: TS 38.508-1 [4],Table 4.8.1-1A with condition RBConfig_KeyChange

Table 9.1.6.1.3.3.3-3: Void

Table 9.1.6.1.3.3.3-4: Void

Table 9.1.6.1.3.3.3-5: Void

Table 9.1.6.1.3.3.3-6: Void)

Table 9.1.6.1.3.3.3-7: REGISTRATION REQUEST (step 7, Table 9.1.6.1.3.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration type value |  | ‘010’B |  | Mobility |

Table 9.1.6.1.3.3.3-8: Void