3GPP TSG-RAN5 Meeting #96-e	R5-224063

# Online, , 15th Aug 2022 - 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2424 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of 4.9.12A Test procedure for IMS MO Emergency call release |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-07-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | If the UE has only 1 PDU session for emergency service, after the emergency call is released, the UE has no reason to keep this PDU Session active and UEs may transmit a PDU Session Release REQ as soon as the emergency call is released. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added optional Step 3Aa1-3Aa3 in Table 4.9.12A.2.2-1 to handle the PDU Session Release REQ from UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9.12A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

4.9.12A	Test procedure for IMS MO Emergency call release

4.9.12A.1	Scope

This procedure aims at verifying the UE initiated release of an ongoing IMS Emergency call in 5GC

4.9.12A.2	Procedure description

4.9.12A.2.1	Initial conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

User Equipment:

-	The Test UICC shall be inserted. It shall provide Emergency Numbers.

The procedure assumes that the UE is in test state 3N-A, subclause 4.4A.2 on the NR Cell with an active IMS emergency call. All necessary details required shall be explicitly specified in the TC which calls the procedure in its entirety or refers to parts of it.

4.9.12A.2.2	Procedure sequence

Table 4.9.12A.2.2-1: Test procedure sequence IMS MO Emergency call release

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| 1-2 | Generic test procedure for MO Release of Voice Call / 5GS, as defined in Annex A.7 of TS 34.229-5 [47], is performed. | - | - | - | - |
| 3 | Void | - | - | - | - |
|  | EXCEPTION: Steps 3Aa1 to 3Aa3 describe a behaviour that depends on the UE implementation. |  |  |  |  |
| 3Aa1 | SS waits for timer T1 = 5 seconds. | - | - | - | - |
| 3Aa2 | The UE transmits a PDU SESSION RELEASE REQUEST message. | --> | PDU SESSION RELEASE REQUEST | - | - |
| 3Aa3 | Timer T1 expires. | - | - | - | - |
| 4 | The SS transmits a RRCReconfiguration message and a PDU SESSION RELEASE COMMAND message. | <-- | NR RRC: RRCReconfiguration NR NAS: PDU SESSION RELEASE COMMAND | - | - |
| 4A | The UE transmits a RRCReconfigurationComplete | - | NR RRC: RRCReconfigurationComplete | - | - |
| 5 | Check: Does the UE transmit a PDU SESSION RELEASE COMPLETE message? | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION RELEASE COMPLETE | - | P |

4.9.12A.2.3	Specific Message content

All specific message contents shall be according subclause 4.6 and 4.7 with the exceptions below.

Table 4.9.12A.2.3-1: RRCReconfiguration (step 4, Table 4.9.12A.2.2-1)

| Derivation Path: Table 4.6.1-13. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| radioBearerConfig SEQUENCE { |  |  |  |  |
| drb-ToReleaseList SEQUENCE (SIZE (1..maxDRB)) OF DRB-Identity { |  | 2 entries |  |  |
| DRB-Identity[1] |  | DRB-Identity linked to the IMS emergency signalling bearer | entry 1 |  |
| DRB-Identity[2] |  | DRB-Identity linked to the IMS emergency speech bearer | entry 2 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup SEQUENCE { |  |  |  |  |
| rlc-BearerToReleaseList SEQUENCE (SIZE (1..maxLC-ID)) OF LogicalChannelIdentity { |  | 2 entries |  |  |
| logicalChannelIdentity[1] |  | Same value as drb-Identity[1] above | entry 1 |  |
| logicalChannelIdentity[2] |  | Same value as drb-Identity[2] above | entry 2 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 4.9.12A.2.3-2: PDU SESSION RELEASE COMMAND (step 4, Table 4.9.12A.2.2-1)

| Derivation Path: Table 4.7.1-14. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PDU session ID |  | Set according to the Emergency session ID. |  |  |
| 5GSM cause |  | ‘0010 0100’B | Regular deactivation |  |

Table 4.9.12A.2.3-3: PDU SESSION RELEASE REQUEST (step 3Aa2, Table 4.9.12A.2.2-1)

| Derivation Path: Table 4.7.1-14. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PDU session ID |  | Set according to the Emergency session ID. |  |  |
| PTI |  | Any value from 1 to 254 |  |  |