3GPP TSG-RAN5 Meeting #96-e	R5-224099

# Online, , 15th Aug 2022 - 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1391 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of LTE Multi-SIM default NAS messages and IEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom,Datang LinkTester Technology |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_MUSIM_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2022-07-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Update to default NAS messages and IEs for LTE Multi-SIM devices for the protocol test. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Several NAS IEs are updated to add MUSIM features |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The WP of MUSIM is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

-	ATTACH ACCEPT

This message is sent by the SS to the UE.

Table 4.7.2-1: ATTACH ACCEPT

| Derivation Path: 24.301 clause 8.2.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | EMM |  |  |
| Security header type |  | '0000'B | Plain NAS message, not security protected |  |
| Attach accept message identity |  | '0100 0010'B | Attach accept |  |
| EPS attach result |  | '0001'B | EPS only | EPS_only |
|  |  | '0010'B | combined EPS/IMSI attach | combined_EPS_IMSI |
| Spare half octet |  | '0000'B |  |  |
| T3412 value |  |  |  |  |
| Timer value |  | '0 0000'B |  |  |
| Unit |  | '111'B | value indicates that the timer is deactivated. |  |
| TAI list |  |  |  |  |
| Length of tracking area identity list contents |  | '0000 0110'B | 6 octets |  |
| Partial tracking area identity list 1 |  |  |  |  |
| Number of elements |  | '0 0000'B | 1 element |  |
| Type of list |  | '00'B | list of TACs belonging to one PLMN, with non-consecutive TAC values |  |
| MCC |  | See table 4.4.2-2 in this document | For NAS test cases, see table 6.3.2.2-1. |  |
| MNC |  | See table 4.4.2-2 in this document | For NAS test cases, see table 6.3.2.2-1. |  |
| TAC 1 |  | See table 4.4.2-2 in this document | For NAS test cases, see table 6.3.2.2-1. |  |
| ESM message container |  | ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message to activate the default bearer |  |  |
| ESM message container |  | ESM DUMMY MESSAGE |  | CIoT_Attach_WithoutPDN |
| GUTI |  |  |  |  |
| Length of EPS mobile identity contents |  | '0000 1011'B | 11 octets |  |
| Type of identity |  | '110'B | GUTI |  |
| Odd/even indication |  | '0'B | even number of identity digits and also when the GUTI is used |  |
| MCC |  | See table 4.4.2-2 in this document | For NAS test cases, see table 6.3.2.2-1. |  |
| MNC |  | See table 4.4.2-2 in this document | For NAS test cases, see table 6.3.2.2-1. |  |
| MME Group ID |  | See table 4.4.2-2 in this document | For NAS test cases, see table 6.3.2.2-1. |  |
| MME Code |  | See table 4.4.2-2 in this document | For NAS test cases, see table 6.3.2.2-1. |  |
| M-TMSI |  | See table 4.4.2-2 in this document | For NAS test cases, see table 6.3.2.2-1. |  |
| Location area identification |  | Not present |  | EPS_only |
| Location area identification |  |  |  | combined_EPS_IMSI |
| MCC |  | MCC of the EUTRA cell from which this message is sent |  |  |
| MNC |  | MCC of the EUTRA cell from which this message is sent |  |  |
| LAC |  | 1 |  |  |
| MS identity |  | Not present |  | EPS_only |
| MS identity |  |  |  | combined_EPS_IMSI |
| Length of mobile identity contents |  | '0000 0101'B | 5 octets |  |
| Type of identity |  | '100'B | TMSI/P-TMSI/M-TMSI |  |
| Odd/even indication |  | '0'B | even number of identity digits and also when the TMSI/P-TMSI or TMGI and optional MBMS Session Identity is used |  |
| TMSI |  | TMSI-1 |  |  |
| EMM cause |  | Not present |  |  |
| T3402 value |  | Not present |  |  |
| T3423 value |  | Not present |  |  |
| Equivalent PLMNs |  | Not present |  |  |
| Emergency number list |  | Not present |  |  |
| EPS network feature support |  | '0000 0011'B | IMS voice over PS session in S1 mode supported, emergency bearer services in S1 mode supported |  |
| EPS network feature support |  | '0000 0001'B | IMS voice over PS session in S1 mode supported | Rel-8 |
| EPS network feature support |  | ‘1000 0011 0000 1111’B | IMS voice over PS session in S1 mode, emergency bearer services in S1 mode, CP CIoT, ePCO, HC-CP CIoT, S1-U data and UP CIoT supported | CIoT |
| EPS network feature support |  | ‘1100 0000 0000 1100’B | CIoT control plane optimization indicated, Attach Without PDN supported, ePCO supported, Header Compression for CIoT control plane supported | CIoT_CP AND pc_NB |
| EPS network feature support |  | ‘1000 0000 0000 1011’B | CP CIoT supported, ePCO supported, S1-U data transfer supported, CIoT user plane optimization indicated | CIoT_UP AND pc_NB |
| EPS network feature support |  | ‘1000 0000 0000 1010’B | CP CIoT supported, ePCO supported, S1-U data transfer only indicated | S1-U Only AND pc_NB |
| EPS network feature support |  | ‘0010 0000 0000 0000’B | EXTENDED SERVICE REQUEST for packet services indicated | ESRPS |
| EPS network feature support |  | '0000 0011 0000 1000'B | - IMS voice over PS session in S1 mode supported - emergency bearer services in S1 mode supported - ePCO supported - Interworking without N26 interface not supported | NR |
| EPS network feature support(octet 3 to 4) |  |  |  | pc_EPC_MUSIM |
| All octets with the exception of octet 5, bit 1 to bit 5 |  | Any allowed value |  |  |
| NAS signalling connection release (NCR) (octet 5, bit 1) |  | ‘1’B | NAS signalling connection release supported | pc_EPC_MUSIM_NCR |
| Paging indication for voice services (PIV) (octet 5, bit 2) |  | ‘1’B | Paging indication for voice services supported | pc_EPC_MUSIM_PIV |
| Reject paging request (RPR) (octet 5, bit 3) |  | ‘1’B | Reject paging request supported | pc_EPC_MUSIM_RPR |
| Paging restriction (PR) (octet 5, bit 4) |  | ‘1’B | Paging restriction supported | pc_EPC_MUSIM_PR |
| Paging timing collision control (PTCC) (octet 5, bit 5) |  | ‘1’B | Paging timing collision control supported | pc_EPC_MUSIM_PTCC |
| Additional update result |  | Not present |  |  |
| Additional update result |  | ‘0010’ B | “SMS only” | additional_update_result_SMS OR CIoT_SMSOnly |
| T3412 extended value |  | Not present |  | Rel-10 |
| T3324 value |  | Not present |  |  |
| Extended DRX parameters |  | Not present |  |  |
| DCN-ID |  | Not present |  |  |
| SMS services status |  | Not present |  |  |
| Non-3GPP NW provided policies |  | Not present |  |  |
| T3448 value |  | Not present |  |  |
| Network policy |  | Not present |  |  |
| T3447 value |  | Not present |  |  |
| Extended emergency number list |  | Not present |  |  |
| Ciphering key data |  | Not present |  |  |
| UE radio capability ID |  | The same value as received in UE radio capability ID; if any of the SECURITY MODE COMPLETE message |  | RACS |
| UE radio capability ID deletion indication |  | Not present |  |  |
| Negotiated IMSI offset |  | Not present or set according to specific message content |  | pc_EPC_MUSIM |

| Condition | Explanation |
| --- | --- |
| EPS_only | This condition applies if the UE is configured to initiate EPS attach or if explicitly specified. |
| combined_EPS_IMSI | This condition applies if the UE is configured to initiate combined EPS/IMSI attach or if explicitly specified. This condition does not apply for NB-S1. |
| additional_update_result_SMS | If the UE requested “SMS only” in the Additional update type IE and combined_EPS_IMSI condition applies |
| CIoT_Attach_WithoutPDN | If the UE wants to attach without PDN for CIoT optimization |
| CIoT_CP | If test case requires control plane CIoT optimization |
| CIoT_SMSOnly | If SMS Only is indicated in the ATTACH REQ received earlier |
| CIoT_UP | If test case requires user plane CIoT optimization |
| S1-U data Only | If S1-U data transfer Only is indicated |
| CIoT | If in the last ATTACH REQUEST, the UE network capability was included with the “CP CIoT” (octet 8, bit 3) or the “UP CIoT” and “S1-U data” (octet 8, bit 4 and bit 5). Note 1: This condition is only applicable for E-UTRA UEs (TS 36.523-2 A.4.1-1/1 or A.4.1-1/2). |
| ESRPS | If the test case requires the network to indicate support of EXTENDED SERVICE REQUEST for packet services (ESRPS) |
| NR | Used in NR test cases (TS 38.523-1 [71]) |
| RACS | Used in RACS test cases (TS 36.523-1 [18]) If pc_EPC_RACS |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

-	ATTACH REQUEST

This message is sent by the UE to the SS.

Table 4.7.2-4: ATTACH REQUEST

| Derivation Path: 24.301 clause 8.2.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | EMM |  |  |
| Security header type |  | '0000'B | Plain NAS message, not security protected |  |
| Attach request message identity |  | '0100 0001'B | Attach request |  |
| EPS attach type |  | '0001'B | EPS attach | EPS_only |
|  |  | '0010'B | combined EPS/IMSI attach | combined_EPS_IMSI |
| NAS key set identifier |  | Any allowed value |  |  |
| EPS mobile identity |  | Any allowed value |  |  |
| UE network capability |  | Any allowed value |  |  |
| UE network capability |  |  |  | EN-DC |
| All octets with the exception of octet 9, bit 5 |  | Any allowed value |  |  |
| Dual connectivity with NR (DCNR) (octet 9, bit 5) |  | 1 | dual connectivity with NR supported |  |
| UE network capability |  |  |  | NR |
| All octets with the exception of octet 8, bit 8 and octet 9, bit 6 |  | Any allowed value |  |  |
| Extended protocol configuration options (ePCO) (octet 8, bit 8) |  | '1' | Extended protocol configuration options IE supported |  |
| N1 mode supported (N1mode) (octet 9, bit 6) |  | 1 | N1 mode supported |  |
| UE network capability |  |  |  | pc_EPC_RACS |
| All octets with the exception of octet 10, bit 1 |  | Any allowed value |  |  |
| Radio capability signalling optimisation (RACS) capability (octet 10, bit 1) |  | 1 |  |  |
| UE network capability |  |  |  | pc_EPC_MUSIM |
| All octets with the exception of octet 10, bit 6 to bit 8 and octet 11,bit 1 to 2 |  | Any allowed value |  |  |
| NAS signalling connection release (NCR) (octet 10, bit 6) |  | 1 | NAS signalling connection release supported | pc_EPC_MUSIM_NCR |
| Paging indication for voice services (PIV) (octet 10, bit 7) |  | 1 | Paging indication for voice services supported | pc_EPC_MUSIM_PIV |
| Reject paging request (RPR) (octet 10, bit 8) |  | 1 | Reject paging request supported | pc_EPC_MUSIM_RPR |
| Paging restriction (PR) (octet 11, bit 1) |  | 1 | paging restriction supported | pc_EPC_MUSIM_PR |
| Paging timing collision control (PTCC) (octet 11, bit 2) |  | 1 | paging timing collision control supported | pc_EPC_MUSIM_PTCC |
| ESM message container |  | PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN |  |  |
| ESM message container |  | ESM DUMMY MESSAGE |  | CIoT_Attach_WithoutPDN |
| Old P-TMSI signature |  | Not present or any allowed value |  |  |
| Additional GUTI |  | Not present or any allowed value |  |  |
| Last visited registered TAI |  | Not present or any allowed value |  |  |
| DRX parameter |  | Not present or any allowed value |  |  |
| MS network capability |  | Not present or any allowed value |  |  |
| Old location area identification |  | Not present or any allowed value |  |  |
| TMSI status |  | Not present or any allowed value |  |  |
| Mobile station classmark 2 |  | Not present or any allowed value |  |  |
| Mobile station classmark 3 |  | Not present or any allowed value |  |  |
| Supported Codecs |  | Not present or any allowed value |  |  |
| Additional update type |  | Not present |  | EPS_only |
| Additional update type |  | Any allowed value |  | EPS_only_with_SMS |
| Additional update type |  | Not present or any allowed value |  | combined_EPS_IMSI or CIoT |
| Voice domain preference and UE's usage setting |  | Not present or any allowed value |  |  |
| Device properties |  | Not present or any allowed value |  |  |
| Old GUTI type |  | Not present or any allowed value |  |  |
| MS network feature support |  | Not present or any allowed value |  |  |
| TMSI based NRI container |  | Not present or any allowed value |  |  |
| T3324 value |  | Not present or any allowed value |  |  |
| T3412 extended value |  | Not present or any allowed value |  |  |
| Extended DRX parameters |  | Not present or any allowed value |  |  |
| UE additional security capability |  | Any allowed value (NOTE 1, NOTE 2) |  | EN-DC OR NR |
| UE status |  | Not present or any allowed value |  |  |
| Additional information requested |  | Not present or any allowed value |  |  |
| UE radio capability ID availability |  | Not present or any allowed value |  | pc_EPC_RACS |
| Requested IMSI offset |  | Not present or any allowed value |  | pc_EPC_MUSIM |
| NOTE 1: For a UE supporting dual connectivity with NR, if the UE supports one of the encryption algorithms for 5GS in bits 8 to 5 of octet 3, it shall support the same algorithms for E-UTRAN as specified in 3GPP TS 33.401 [41]. NOTE 2: For a UE supporting dual connectivity with NR, if the UE supports one of the integrity protection algorithms for 5GS in bits 8 to 5 of octet 5, it shall support the same algorithms for E-UTRAN as specified in 3GPP TS 33.401 [41]. |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| EPS_only | See the definition below table 4.7.2-1. |
| Combined_EPS_IMSI | See the definition below table 4.7.2-1. |
| CioT_Attach_WithoutPDN | If the UE wants to attach without PDN for CioT optimization |
| EPS_only_with_SMS | If the UE wants to perform EPS attach with SMS only for CioT optimization |
| CioT | If UE supports CioT Optimisation |
| EN-DC | Used in E-UTRA-NR dual connectivity test cases (TS 38.523-1 [71]) |
| NR | Used in NR test cases (TS 38.523-1 [71]) |

NOTE:	This message is sent integrity protected when a valid security context exists and without integrity protection otherwise.

-	TRACKING AREA UPDATE ACCEPT

This message is sent by the SS to the UE.

Table 4.7.2-24: TRACKING AREA UPDATE ACCEPT

| Derivation Path: 24.301 clause 8.2.26 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | EMM |  |  |
| Security header type |  | '0000'B | Plain NAS message, not security protected |  |
| Tracking area update accept message identity |  | '0100 1001'B | Tracking area update accept |  |
| EPS update result |  | '0000'B | TA updated | TA_only |
|  |  | '0001'B | combined TA/LA updated | combined_TA_LA |
| Spare half octet |  | '0000'B |  |  |
| T3412 value |  | Not present |  | Periodic |
| T3412 value |  |  |  |  |
| Timer value |  | '0 0000'B |  |  |
| Unit |  | '111'B | value indicates that the timer is deactivated. |  |
| GUTI |  |  |  |  |
| Length of EPS mobile identity contents |  | '0000 1011'B | 11 octets |  |
| Type of identity |  | '110'B | GUTI |  |
| Odd/even indication |  | '0'B | even number of identity digits and also when the GUTI is used |  |
| MCC |  | See table 4.4.2-2 in this document | For NAS test cases, see table 6.3.2.2-1. |  |
| MNC |  | See table 4.4.2-2 in this document | For NAS test cases, see table 6.3.2.2-1. |  |
| MME Group ID |  | See table 4.4.2-2 in this document | For NAS test cases, see table 6.3.2.2-1. |  |
| MME Code |  | See table 4.4.2-2 in this document | For NAS test cases, see table 6.3.2.2-1. |  |
| M-TMSI |  | See table 4.4.2-2 in this document | For NAS test cases, see table 6.3.2.2-1. |  |
| TAI list |  |  |  |  |
| Length of tracking area identity list contents |  | '0000 0110'B | 6 octets |  |
| Partial tracking area identity list 1 |  |  |  |  |
| Number of elements |  | '0 0000'B | 1 element |  |
| Type of list |  | '00'B | list of TACs belonging to one PLMN, with non-consecutive TAC values |  |
| MCC |  | See table 4.4.2-2 in this document | For NAS test cases, see table 6.3.2.2-1. |  |
| MNC |  | See table 4.4.2-2 in this document | For NAS test cases, see table 6.3.2.2-1. |  |
| TAC 1 |  | See table 4.4.2-2 in this document | For NAS test cases, see table 6.3.2.2-1. |  |
| EPS bearer context status |  | The same value as the value set in TRACKING AREA UPDATE REQUEST message |  |  |
| Location area identification |  | Not present |  | TA_only |
| Location area identification |  |  |  | combined_TA_LA |
| MCC |  | MCC of the EUTRA cell from which this message is sent |  |  |
| MNC |  | MCC of the EUTRA cell from which this message is sent |  |  |
| LAC |  | 1 |  |  |
| MS identity |  | Not present |  | TA_only |
| MS identity |  |  |  | combined_TA_LA |
| Length of mobile identity contents |  | '0000 0101'B | 5 octets |  |
| Type of identity |  | '100'B | TMSI/P-TMSI/M-TMSI |  |
| Odd/even indication |  | '0'B | even number of identity digits and also when the TMSI/P-TMSI or TMGI and optional MBMS Session Identity is used |  |
| TMSI |  | TMSI-1 |  |  |
| EMM cause |  | Not present |  |  |
| T3402 value |  | Not present |  |  |
| T3423 value |  | Not present |  |  |
| Equivalent PLMNs |  | Not present |  |  |
| Emergency number list |  | Not present |  |  |
| EPS network feature support |  | 0000 0001'B | IMS voice over PS session in S1 mode supported |  |
| EPS network feature support |  | ‘1100 0000 0000 1100’B | CIoT control plane optimization indicated, Attach Without PDN supported, ePCO supported, Header Compression for CIoT control plane supported | (CIoT_CP OR CIoT_CP_SMSOnly) AND pc_NB |
| EPS network feature support |  | ‘1000 0000 0000 1011’B | CP CIoT supported, ePCO supported, S1-U data transfer supported, CIoT user plane optimization indicated | CIoT_UP AND pc_NB |
| EPS network feature support |  | ‘1000 0000 0000 1010’B | CP CIoT supported, ePCO supported, S1-U data transfer Only indicated | S1-U Only AND pc_NB |
| EPS network feature support |  | '0000 0011 0000 1000'B | - IMS voice over PS session in S1 mode supported - emergency bearer services in S1 mode supported - ePCO supported - Interworking without N26 interface not supported | NR |
| EPS network feature support |  |  |  | pc_EPC_MUSIM |
| All octets with the exception of octet 5, bit 1 to bit 5 |  | Any allowed value |  |  |
| NAS signalling connection release (NCR) (octet 5, bit 1) |  | ‘1’B | NAS signalling connection release supported | pc_EPC_MUSIM_NCR |
| Paging indication for voice services (PIV) (octet 5, bit 2) |  | ‘1’B | Paging indication for voice services supported | pc_EPC_MUSIM_PIV |
| Reject paging request (RPR) (octet 5, bit 3) |  | ‘1’B | Reject paging request supported | pc_EPC_MUSIM_RPR |
| Paging restriction (PR) (octet 5, bit 4) |  | ‘1’B | Paging restriction supported | pc_EPC_MUSIM_PR |
| Paging timing collision control (PTCC) (octet 5, bit 5) |  | ‘1’B | Paging timing collision control supported | pc_EPC_MUSIM_PTCC |
| Additional update result |  | Not present |  |  |
| Additional update result |  | ‘0010’ B | “SMS only” | TAU_additional_update_result_SMS OR CIoT_CP_SMSOnly |
| T3412 extended value |  | Not present |  | Rel-10 |
| T3324 value |  | Not present |  |  |
| Extended DRX parameters |  | Not present |  |  |
| Header compression configuration status |  | Not present or set according to specific message content |  |  |
| DCN-ID |  | Not present |  |  |
| SMS services status |  | Not present |  |  |
| Non-3GPP NW provided policies |  | Not present |  |  |
| T3448 value |  | Not present |  |  |
| Network policy |  | Not present |  |  |
| T3447 value |  | Not present |  |  |
| Extended emergency number list |  | Not present |  |  |
| Ciphering key data |  | Not present |  |  |
| UE radio capability ID |  | The same value as received in UE radio capability ID; if any of the SECURITY MODE COMPLETE message |  | RACS |
| UE radio capability ID deletion indication |  | Not present |  |  |
| Negotiated IMSI offset IE |  | Not present or set according to specific message content |  | pc_EPC_MUSIM |
| EPS additional request result |  | Not present or set according to specific message content |  | pc_EPC_MUSIM |

| Condition | Explanation |
| --- | --- |
| TA_only | This condition applies if the UE is configured to initiate EPS attach or if explicitly specified. |
| combined_TA_LA | This condition applies if the UE is configured to initiate combined EPS/IMSI attach or if explicitly specified. This condition does not apply for NB-S1. |
| Periodic | This condition applies if in the last TRACKING AREA UPDATE REQUEST sent prior to this message, the EPS update type Value = '011'B (periodic updating). |
| TAU_additional_update_result_SMS | If the UE requested "SMS only" in the Additional update type IE and combined_TA_LA. |
| CIoT_CP | If test case requires control plane CIoT optimization |
| CIoT_CP_SMSOnly | If test case requires control plane CIoT optimization and SMS Only is indicated |
| CIoT_UP | If test case requires user plane CIoT optimization |
| S1-U data Only | If S1-U data transfer Only is indicated |
| NR | Used in NR test cases (TS 38.523-1 [71]) |
| RACS | Used in RACS test cases (TS 36.523-1 [18]) If pc_EPC_RACS |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

-	TRACKING AREA UPDATE REQUEST

This message is sent by the UE to the SS.

Table 4.7.2-27: TRACKING AREA UPDATE REQUEST

| Derivation Path: 24.301 clause 8.2.29 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | EMM |  |  |
| Security header type |  | '0000'B | Plain NAS message, not security protected |  |
| Tracking area update request message identity |  | '0100 1000'B | Tracking area update request |  |
| EPS update type |  |  |  |  |
| EPS update type Value |  | '000'B | TA updating | TA_only |
|  |  | '001'B | Combined TA/LA updating | combined_TA_LA |
| "Active" flag |  | '0'B | No bearer establishment requested |  |
| NAS key set identifier |  |  |  |  |
| NAS key set identifier |  | The valid NAS key set identifier of the UE |  |  |
| TSC |  | '0'B | native security context (for KSIASME) |  |
| Old GUTI |  | Any allowed value |  |  |
| Non-current native NAS key set identifier |  | Not present |  |  |
| GPRS ciphering key sequence number |  | Not present |  |  |
| Old P-TMSI signature |  | Not present |  |  |
| Additional GUTI |  | Not present |  |  |
| NonceUE |  | Not present |  |  |
| UE network capability |  | Not present or any allowed value |  |  |
| UE network capability |  |  |  | EN-DC |
| All octets with the exception of octet 9, bit 5 |  | Any allowed value |  |  |
| Dual connectivity with NR (DCNR) (octet 9, bit 5) |  | 1 | dual connectivity with NR supported |  |
| UE network capability |  |  |  | NR |
| All octets with the exception of octet 8, bit 8 and octet 9, bit 6 |  | Any allowed value |  |  |
| Extended protocol configuration options (ePCO) (octet 8, bit 8) |  | 1 | ePCO supported |  |
| N1 mode supported (N1mode) (octet 9, bit 6) |  | 1 | N1 mode supported |  |
| UE network capability |  |  |  | pc_EPC_RACS |
| All octets with the exception of octet 10, bit 1 |  | Any allowed value |  |  |
| Radio capability signalling optimisation (RACS) capability (octet 10, bit 1) |  | 1 |  |  |
| UE network capability |  |  |  | Pc_EPC_MUSIM |
| All octets with the exception of octet 10, bit 6 to bit 8 and octet 11,bit 1 to 2 |  | Any allowed value |  |  |
| NAS signalling connection release (NCR) (octet 10, bit 6) |  | 1 | NAS signalling connection release supported | pc_EPC_MUSIM_NCR |
| Paging indication for voice services (PIV) (octet 10, bit 7) |  | 1 | Paging indication for voice services supported | pc_EPC_MUSIM_PIV |
| Reject paging request (RPR) (octet 10, bit 8) |  | 1 | Reject paging request supported | pc_EPC_MUSIM_PRP |
| Paging restriction (PR) (octet 11, bit 1) |  | 1 | Paging restriction supported | pc_EPC_MUSIM_PR |
| Paging timing collision control (PTCC) (octet 11, bit 2) |  | 1 | Paging timing collision control supported | pc_EPC_MUSIM_PTCC |
| Last visited registered TAI |  | Not present or any allowed value |  |  |
| DRX parameter |  | Not present or any allowed value |  |  |
| UE radio capability information update needed |  | Not present or any allowed value |  |  |
| EPS bearer context status |  | Not present or (octet 3 = '00100000'B and octet 4 = '00000000'B) |  |  |
| MS network capability |  | Not present or any allowed value |  |  |
| Old location area identification |  | Not present or any allowed value |  |  |
| TMSI status |  | Not present or any allowed value |  |  |
| Mobile station classmark 2 |  | Not present or any allowed value |  |  |
| Mobile station classmark 3 |  | Not present or any allowed value |  |  |
| Supported Codecs |  | Not present or any allowed value |  |  |
| Additional update type |  | Not present |  | TA_only |
| Additional update type |  | Not present or any allowed value |  | combined_TA_LA |
| Additional update type |  | Any allowed value |  | TA_only_with_SMS |
| Additional update type |  | Not present or any allowed value |  | CIoT |
| Voice domain preference and UE's usage setting |  | Not present or any allowed value |  |  |
| Old GUTI type |  | Not present or any allowed value |  |  |
| Device properties |  | Not present or any allowed value |  |  |
| MS network feature support |  | Not present or any allowed value |  |  |
| TMSI based NRI container |  | Not present or any allowed value |  |  |
| T3324 value |  | Not present or any allowed value |  |  |
| T3412 extended value |  | Not present or any allowed value |  |  |
| Extended DRX parameters |  | Not present or any allowed value |  |  |
| UE additional security capability |  | Any allowed value (NOTE 1, NOTE 2) |  | EN-DC OR NR |
| UE status |  | Not present or any allowed value |  |  |
| Additional information requested |  | Not present or any allowed value |  |  |
| UE radio capability ID availability |  | Not present or any allowed value |  | pc_EPC_RACS |
| Requested IMSI offset |  | Not present or any allowed value |  | pc_EPC_MUSIM |
| UE request type |  | Not present or any allowed value |  | pc_EPC_MUSIM |
| Paging restriction |  | Not present or any allowed value |  | pc_EPC_MUSIM |
| NOTE 1: For a UE supporting dual connectivity with NR, if the UE supports one of the encryption algorithms for 5GS in bits 8 to 5 of octet 3, it shall support the same algorithms for E-UTRAN as specified in 3GPP TS 33.401 [41]. NOTE 2: For a UE supporting dual connectivity with NR, if the UE supports one of the integrity protection algorithms for 5GS in bits 8 to 5 of octet 5, it shall support the same algorithms for E-UTRAN as specified in 3GPP TS 33.401 [41]. |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| TA_only | See the definition below table 4.7.2-24. |
| combined_TA_LA | See the definition below table 4.7.2-24. |
| EPS_only_with_SMS | If the UE wants to update for EPS attach with SMS only for CIoT optimization |
| CIoT | If UE supports CIoT Optimisation |
| EN-DC | Used in E-UTRA-NR dual connectivity test cases (TS 38.523-1 [71]) |
| NR | Used in NR test cases (TS 38.523-1 [71]) |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

-	EXTENDED SERVICE REQUEST

This message is sent by the UE to the SS.

Table 4.7.2-14A: EXTENDED SERVICE REQUEST

| Derivation Path: 24.301 clause 8.2.15 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | EMM |  |  |
| Security header type |  | '0000'B | Plain NAS message, not security protected |  |
| Extended service request message identity |  | '0100 1100'B | Extended service request |  |
| Service type |  | '0001'B | mobile terminating CS fallback or 1xCS fallback |  |
| NAS key set identifier |  |  |  |  |
| NAS key set identifier |  | The valid NAS key set identifier. |  |  |
| TSC |  | '0'B | native security context (for KSIASME) |  |
| M-TMSI |  | If the UE has a valid M-TMSI, set to the M-TMSI, otherwise set to the IMSI of the UE. |  |  |
| CSFB response |  | ’001’B | CS fallback accepted by the UE |  |
| UE request type |  | Not present or any allowed value |  | pc_EPC_MUSIM |
| Paging restriction |  | Not present or any allowed value |  | pc_EPC_MUSIM_PR |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

-	SERVICE ACCEPT

This message is sent by the SS to the UE.

Table 4.7.2-21A: SERVICE ACCEPT

| Derivation Path: 24.301 clause 8.2.34 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | EMM |  |  |
| Security header type |  | '0000'B | Plain NAS message, not security protected |  |
| Service accept message identity |  | '0100 1111'B | Service accept |  |
| EPS bearer context status |  | The same value as the value set in CONTROL PLANE SERVICE REQUEST |  |  |
| EPS additional request result |  | Not present or set according to specific message content |  | pc_EPC_MUSIM |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

<End of modified section 1>