3GPP TSG-RAN5 Meeting #96-e	R5-224112
Electronic Meeting, 15th August – 26th August 2022

Title: 	Discussion on TPMI Test method applicability

Source:	        Keysight Technologies UK Ltd

Agenda Item:	5.3.50.1

Document for:	Discussion and Endorsement

# 1.	Introduction

According to the internal work plan approved in last RAN5 meeting ([1]) for UE Conformance Test Aspects for NR RF FR2 Enhanced Test Methods, the introduction of TPMI test method as defined in TR 38.884 [2] is required.

This document analyses the applicability of TPMI test methods from three points of view (UE Release, UE capabilities and test cases) and makes some proposals to add this enhanced test method to the specification.

# 2.	Discussion

2.1 UE Release

There is misleading information in 3GPP TR 38.884 [2] regarding the UE Release applicability of the TPMI test method:

- While in section 1, one can find the following statement (added in TP R4-2100530 [3]):

- In section 5.2.2.2, the following statement can be found (added in TP R4-2103967 [4]):

Observation 1: There is misleading information in 3GPP TR 38.884 [2] regarding the UE Release applicability of the TPMI test method.

However, based on the background provided in R4-2010202, it seems that the TPMI test method seems to be applicable for Rel-15 Coherent UEs. Hence, Keysight’s understanding is that the correct applicability is the one defined in section 5.2.2.2 in 3GPP TR 38.884 [2].

Proposal 1: TPMI test method is applicable from Rel-15 and onwards.

2.2 UE Capabilities

According to 3GPP TR 38.884 ([2]) section 5.2.2.2 (refer to snapshot in section 2.1 in this document), TPMI test method is applicable to coherent UEs as well as those Rel-16 supporting UL full power transmission mode1.

- According to Keysight’s understanding, the capability in 3GPP TS 38.306 [6] that indicates the UE coherency is the following one:

Possible values for this capability are defined in 3GPP TS 38.331 [7] as nonCoherent, partialCoherent, fullCoherent:

The network will indicate the UE which codebook is used through codebookSubset parameter in PUSCH-Config (refer to below snapshot from 3GG TS 38.331 [7]):

To be noticed that partialAndNonCoherent is not allowed for 2 layers according to 3GPP TS 38.214 [8]:

- UE capability indicating the support of UL full power transmission mode 1 is:

Possible value for this capability is defined in 3GPP TS 38.331 [7] as supported:

Observation 2: Either pusch-TransCoherence as well as ul-FullPwrMode1-r16 are optional UE capabilities.

Proposal 2: TPMI test method is applicable to devices with UE capabilities pusch-TransCoherence=fullCoherent and/or ul-FullPwrMode1-r16=supported.

To highlight that current test specification ([9]) is not covering the testing UE Coherency, as the default configuration of PUSCH-Config in 3GPP TS 38.508-1 ([10]) is codebookSubset=NonCoherent:

Observation 3: current 3GPP 38.521-2 test specification ([9]) is not covering the testing of UE Coherency, as the default configuration of PUSCH-Config in 3GPP TS 38.508-1 ([10]) is codebookSubset=NonCoherent.

Observation 3 opens the discussion on whether coherent UEs need to be tested using 2 different network configurations:

- Configuration A: codebookSubset= nonCoherent

- Configuration B: codebookSubset= FullyAndPartialAndNonCoherent

Or whether only worst case is needed. As most of core requirements are tested at UE maximum power and the output power is supposed to be higher under Configuration B, it would make sense to test coherent devices only under network Configuration B.

Proposal 3: Test coherent UEs using only codebookSubset = fullyAndPartialAndNonCoherent in PUSCH-Config.

2.3 Test Cases

Regarding the test cases where TPMI can be used, 3GPP 38.884 [2] indicates the following:

Highlighted text above indicates that TPMI test method is applicable to all Transmitter requirements defined through EIRP or TRP metric. However, in all Receiver requirements, there are also measurements of the UL power. Hence, TPMI test method will also be applicable to all Receiver core requirements.

Proposal 4: TPMI test method is applicable to all transmitter and receiver test cases.

To be noticed that applicability of TPMI test method does not exclude any type of dual connectivity nor carrier aggregation as far as TPMI index 2 can be forced for UE single layer transmission using two antenna ports in NR component carriers. Hence it is assumed that the TPMI test method is applicable to carrier aggregation, EN-DC, NE-DC and NR DC.

Proposal 5: TPMI test method is applicable to carrier aggregation, EN-DC, NE-DC and NR DC test cases.

# 3. 	Conclusion

This document analyses the applicability of TPMI test methods from three points of view (UE Release, UE capabilities and test cases) and makes some proposals to add this enhanced test method to the specification.

The following observations and proposals are done:

Observation 1: There is misleading information in 3GPP TR 38.884 [2] regarding the UE Release applicability of the TPMI test method.

Proposal 1: TPMI test method is applicable from Rel-15 and onwards.

Observation 2: Either pusch-TransCoherence as well as ul-FullPwrMode1-r16 are optional UE capabilities.

Proposal 2: TPMI test method is applicable to devices with UE capabilities pusch-TransCoherence=fullCoherent and/or ul-FullPwrMode1-r16=supported.

Observation 3: current 3GPP 38.521-2 test specification ([9]) is not covering the testing of UE Coherency, as the default configuration of PUSCH-Config in 3GPP TS 38.508-1 ([10]) is codebookSubset=NonCoherent.

Proposal 3: Test coherent UEs using only codebookSubset = fullyAndPartialAndNonCoherent in PUSCH-Config.

Proposal 4: TPMI test method is applicable to all transmitter and receiver test cases.

Proposal 5: TPMI test method is applicable to carrier aggregation, EN-DC, NE-DC and NR DC test cases.

# 4.	References

- R5-223028, “Draft internal Work Plan for FR2 Enhanced Test Methods”, Apple Portugal, RAN5#95-e

- 3GPP TR 38.884, “Study on enhanced test methods for FR2 NR UEs”

- R4-2100530, “TP to TR38.884 on structure updates related to band n262”, Apple Inc., RAN4#98-e

- R4-2103967, “TP to TR38.884 v0.1.0 on polarization basis mismatch”, vivo, Apple, RAN4#98-e

- R4-2010202, “Discussion on dual polarization transmission for UL TX test”, Samsung, RAN4#96-e

- 3GPP TS 38.306, “User Equipment (UE) radio access capabilities”

- 3GPP TS 38.331, “Radio Resource Control (RRC) protocol specification”

- 3GPP TS 38.214, “Physical layer procedures for data”

- 3GPP TS 38.521-2, “User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone”

- 3GPP TS 38.508-1, “User Equipment (UE) conformance specification; Part 1: Common test environment”