3GPP TSG-RAN5 Meeting #96-e	R5-224113
Electronic Meeting, 15th August – 26th August 2022

Title: 	Discussion on TPMI Test method implementation

Source:	        Keysight Technologies UK Ltd

Agenda Item:	5.3.50.1

Document for:	Discussion and Endorsement

# 1.	Introduction

According to the internal work plan approved in last RAN5 meeting ([1]) for UE Conformance Test Aspects for NR RF FR2 Enhanced Test Methods, the introduction of TPMI test method as defined in TR 38.884 [2] is required.

This document analyses how to introduce TPMI test method in 3GPP TS 38.521-2 test specification [3] based on the applicability analysis in R5-22xxxx [4].

# 2.	Discussion

2.1 Decoupled TPMI test method implementation for coherent UEs and for UEs supporting UL Full power transmission mode 1

According to 3GPP TS 38.214 ([5]) section 6.1.1.1, even when the UE supports pusch-TransCoherence=fullCoherent and ul-FullPwrMode1-r16 capabilities, the UE shall not expect to be configured with higher layer parameter ul-FullPowerTransmission set to ‘fullpowerMode1’ and codebookSubset or codebookSubsetDCI-0-2 set to ‘fullAndPartialAndNonCoherent’ simultaneously.

Observation 1: ul-FullPowerTransmission = fullpowerMode1 and codebookSubset=fullAndPartialAndNonCoherent can not be configured in the network simultaneously.

Proposal 1: TPMI test method implementation needs to be decoupled for coherent UEs and for UEs supporting ul-FullPwrMode1-r16 capability.

Proposal 2: Test TPMI test method implementation for coherent UEs in non-UL-MIMO test cases and TPMI test method implementation for UEs supporting ul-FullPwerMode1-r16 in UL-MIMO test cases.

2.2 TPMI test method implementation for coherent UEs

2.2.1 PUSCH-Config

According to 3GPP TS 38.508-1 [6], PUSCH-Config for RF Test cases is defined as:

Table 5.4.1-4: PUSCH-Config

| Derivation Path: Clause 4.6.3 Table 4.6.3-118 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| maxRank | 2 |  | 2TX_UL_MIMO |
|  | 1 |  | ULFPTx_Mode1 or ULFPTx_Mode2 or ULFPTx_ModeFull |
| ul-FullPowerTransmission-r16 | Not present |  | 2TX_UL_MIMO |
|  | fullpowerMode1 |  | ULFPTx_Mode1 |
|  | fullpowerMode2 |  | ULFPTx_Mode2 |
|  | fullpower |  | ULFPTx_ModeFull |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| 2TX_UL_MIMO | UL-MIMO test cases with 2 Tx antenna ports |
| ULFPTx_Mode1 | UL-MIMO test cases with UEs supporting UL full power transmission Mode-1 |
| ULFPTx_Mode2 | UL-MIMO test cases with UEs supporting UL full power transmission Mode-2 |
| ULFPTx_ModeFull | UL-MIMO test cases with UEs supporting UL full power transmission Mode-full power |

While other parameters for PUSCH-Config are defined in table 4.6.3-118 in same test specification:

–	PUSCH-Config

Table 4.6.3-118: PUSCH-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| dataScramblingIdentityPUSCH | Not present |  |  |
| txConfig | Not Present |  | Short_DCI |
|  | codebook |  |  |
| dmrs-UplinkForPUSCH-MappingTypeA CHOICE { |  |  |  |
| setup | DMRS-UplinkConfig |  |  |
| } |  |  |  |
| dmrs-UplinkForPUSCH-MappingTypeB | Not present |  |  |
| pusch-PowerControl | PUSCH-PowerControl |  |  |
| frequencyHopping | Not present |  |  |
| frequencyHoppingOffsetLists | Not present |  |  |
| resourceAllocation | resourceAllocationType1 |  |  |
| pusch-TimeDomainAllocationList | Not present |  |  |
| pusch-AggregationFactor | Not present |  |  |
| mcs-Table | Not present |  |  |
| mcs-TableTransformPrecoder | Not present |  |  |
| transformPrecoder | enabled |  | TRANSFORM_PRECODER_ENABLED |
|  | Not present | TRANSFORM_PRECODER_DISABLED |  |
| codebookSubset | Not present |  | Short_DCI |
|  | nonCoherent |  |  |
| maxRank | Not present |  | Short_DCI |
|  | 1 |  |  |
|  | 2 |  | 2TX_UL_MIMO |
| rbg-Size | Not present |  |  |
| uci-OnPUSCH CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| betaOffsets CHOICE { |  |  |  |
| semiStatic | BetaOffsets |  |  |
| } |  |  |  |
| scaling | f1 |  |  |
| } |  |  |  |
| } |  |  |  |
| tp-pi2BPSK | Not present |  |  |
| minimumSchedulingOffsetK2-r16 | Not present |  |  |
| ul-AccessConfigListDCI-0-1-r16 | Not present |  |  |
| harq-ProcessNumberSizeDCI-0-2-r16 | Not present |  |  |
| dmrs-SequenceInitializationDCI-0-2-r1 | Not present |  |  |
| numberOfBitsForRV-DCI-0-2-r16 | Not present |  |  |
| antennaPortsFieldPresenceDCI-0-2-r16 | Not present |  |  |
| dmrs-UplinkForPUSCH-MappingTypeA-DCI-0-2-r16 | Not present |  |  |
| dmrs-UplinkForPUSCH-MappingTypeB-DCI-0-2-r16 | Not present |  |  |
| frequencyHoppingDCI-0-2-r16 | Not present |  |  |
| frequencyHoppingOffsetListsDCI-0-2-r16 | Not present |  |  |
| codebookSubsetDCI-0-2-r16 | Not present |  |  |
| invalidSymbolPatternIndicatorDCI-0-2-r16 | Not present |  |  |
| maxRankDCI-0-2-r16 | Not present |  |  |
| mcs-TableDCI-0-2-r16 | Not present |  |  |
| mcs-TableTransformPrecoderDCI-0-2-r16 | Not present |  |  |
| priorityIndicatorDCI-0-2-r16 | Not present |  |  |
| pusch-RepTypeIndicatorDCI-0-2-r16 | Not present |  |  |
| resourceAllocationDCI-0-2-r16 | Not present |  |  |
| resourceAllocationType1GranularityDCI-0-2-r16 | Not present |  |  |
| uci-OnPUSCH-ListDCI-0-2-r16 | Not present |  |  |
| pusch-TimeDomainAllocationListDCI-0-2-r16 | Not present |  |  |
| pusch-TimeDomainAllocationListDCI-0-1-r16 | Not present |  |  |
| invalidSymbolPatternIndicatorDCI-0-1-r16 | Not present |  |  |
| priorityIndicatorDCI-0-1-r16 | Not present |  |  |
| pusch-RepTypeIndicatorDCI-0-1-r16 | Not present |  |  |
| frequencyHoppingDCI-0-1-r16 | Not present |  |  |
| uci-OnPUSCH-ListDCI-0-1-r16 | Not present |  |  |
| invalidSymbolPattern-r16 | Not present |  |  |
| pusch-PowerControl-v1610 | Not present |  |  |
| ul-FullPowerTransmission-r16 | Not present |  |  |
| pusch-TimeDomainAllocationListForMultiPUSCH-r16 | Not present |  |  |
| numberOfInvalidSymbolsForDL-UL-Switching-r16 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| TRANSFORM_PRECODER_ENABLED | Transform precoding is enabled (DFT-s-OFDM UL waveform is configured) |
| 2TX_UL_MIMO | UL-MIMO test cases with 2 Tx antenna ports |
| Short_DCI | Used in test scenarios requiring DCI formats 0-0 and 1-0 on USS |

In order to have the proper network configuration for Coherent UEs, codebookSubset needs to be set to ‘fullyAndPartialAndNonCoherent’ and maxRank needs to be set to ‘1’.

Proposal 3: Extend PUSCH-Config for RF Test cases in 3GPP TS 38.508-1 [6] as shown below:

Table 5.4.1-4: PUSCH-Config

| Derivation Path: Clause 4.6.3 Table 4.6.3-118 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| codebookSubset | nonCoherent |  |  |
|  | fullyAndPartialAndNonCoherent |  | Coherent_Not_UL_MIMO |
| maxRank | 2 |  | 2TX_UL_MIMO |
|  | 1 |  | ULFPTx_Mode1 or ULFPTx_Mode2 or ULFPTx_ModeFull or Coherent_Not_UL_MIMO |
| ul-FullPowerTransmission-r16 | Not present |  | 2TX_UL_MIMO |
|  | fullpowerMode1 |  | ULFPTx_Mode1 |
|  | fullpowerMode2 |  | ULFPTx_Mode2 |
|  | fullpower |  | ULFPTx_ModeFull |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| 2TX_UL_MIMO | UL-MIMO test cases with 2 Tx antenna ports |
| Coherent_Not_UL_MIMO | Non UL-MIMO RF test cases for UEs with puschTransCoherence=FullCoherent |
| ULFPTx_Mode1 | UL-MIMO test cases with UEs supporting UL full power transmission Mode-1 |
| ULFPTx_Mode2 | UL-MIMO test cases with UEs supporting UL full power transmission Mode-2 |
| ULFPTx_ModeFull | UL-MIMO test cases with UEs supporting UL full power transmission Mode-full power |

2.2.2 SRS-Config

According to 3GPP TS38.214 [5] section 6.1.1:

As shown in clause 2.2.1 in PUSCH-Config txConfig is set to ‘codebook’. Hence no update is required.

When checking SRS-Config, it can be seen that SRS-ResourceSet usage is already set to ‘codebook’ and SRS- Resource nrofports is already set to ‘ports2’ if 2TX_UL_MIMO and set to ‘ports1’ otherwise. Condition for SRS-ResourceSet nrofports needs to be extended to Coherent devices for Non UL-MIMO test cases

Proposal 4: Condition for SRS-ResourceSet nrofports needs to be extended to Coherent devices for Non UL-MIMO test cases:

SRS-Config

Table 4.6.3-182: SRS-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SRS-Config ::= SEQUENCE { |  |  |  |
| srs-ResourceSetToReleaseList | Not present |  |  |
| srs-ResourceSetToAddModList SEQUENCE (SIZE(0..maxNrofSRS-ResourceSets)) OF SRS-ResourceSet { | 1 entry |  |  |
| SRS-ResourceSet[1] SEQUENCE { |  | entry 1 |  |
| srs-ResourceSetId | 0 |  |  |
| srs-ResourceIdList SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-ResourceId { | 1 entry |  |  |
| SRS-ResourceId[1] | 0 | entry 1 |  |
| } |  |  |  |
| resourceType CHOICE { |  |  |  |
| aperiodic SEQUENCE { |  |  |  |
| aperiodicSRS-ResourceTrigger | 1 |  |  |
| csi-RS | Not present |  |  |
| slotOffset | 7 |  | FR1 |
|  | 4 |  | FR2 |
| } |  |  |  |
| } |  |  |  |
| usage | codebook |  |  |
| alpha | Alpha |  |  |
| p0 | 0 |  |  |
| pathlossReferenceRS CHOICE { |  |  |  |
| ssb-Index | SSB-Index |  |  |
| } |  |  |  |
| srs-PowerControlAdjustmentStates | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| srs-ResourceToReleaseList | Not present |  |  |
| srs-ResourceToAddModList SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-Resource { | 1 entry |  |  |
| SRS-Resource[1] SEQUENCE { |  | entry 1 |  |
| srs-ResourceId | 0 |  |  |
| nrofSRS-Ports | ports2 |  | 2TX_UL_MIMO or Coherent_Not_UL_MIMO |
|  | port1 |  |  |
| ptrs-PortIndex | Not present |  |  |
| transmissionComb CHOICE { |  |  |  |
| n2 SEQUENCE { |  |  |  |
| combOffset-n2 | 0 |  |  |
| cyclicShift-n2 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| resourceMapping SEQUENCE { |  |  |  |
| startPosition | 0 |  |  |
| nrofSymbols | n1 |  |  |
| repetitionFactor | n1 |  |  |
| } |  |  |  |
| freqDomainPosition | 0 |  |  |
| freqDomainShift | 0 |  |  |
| freqHopping SEQUENCE { |  |  |  |
| c-SRS | 0 |  |  |
|  | 63 |  | FR1_100MHz |
|  | 17 |  | FR2_100MHz |
| b-SRS | 0 |  |  |
| b-hop | 0 |  |  |
| } |  |  |  |
| groupOrSequenceHopping | groupHopping |  |  |
| resourceType CHOICE { |  |  |  |
| aperiodic SEQUENCE { |  |  |  |
| } |  |  |  |
| } |  |  |  |
| sequenceId | 0 |  |  |
| spatialRelationInfo SEQUENCE { | SRS-SpatialRelationInfo |  |  |
| servingCellId | Not present |  |  |
| referenceSignal CHOICE { |  |  |  |
| ssb-Index | SSB-Index |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| tpc-Accumulation | Not present |  |  |
| srs-RequestDCI-1-2-r16 | Not present |  |  |
| srs-RequestDCI-0-2-r16 | Not present |  |  |
| srs-ResourceSetToAddModListDCI-0-2-r16 | Not present |  |  |
| srs-ResourceSetToReleaseListDCI-0-2-r16 | Not present |  |  |
| srs-PosResourceSetToReleaseList-r16 | Not present |  |  |
| srs-PosResourceSetToAddModList-r16 | Not present |  |  |
| srs-PosResourceToReleaseList-r16 | Not present |  |  |
| srs-PosResourceToAddModList-r16 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| 2TX_UL_MIMO | UL-MIMO test cases with 2 Tx antenna ports |
| Coherent_Not_UL_MIMO | Non UL-MIMO RF test cases for UEs with puschTransCoherence=FullCoherent |
| FR1_100MHz | FR1 is used under the test. CBW is set to 100MHz. |
| FR2_100MHz | FR2 is used under the test. CBW is set to 100MHz. |

2.2.3 TPMI configuration

Specific value of TPMI configuration is defined in 3GPP TS 38.508-1 [6] table 4.3.6.1.12-1:

4.3.6.1.1.2	Physical layer parameters for DCI format 0_1

DCI format 0_1 is used for the scheduling of PUSCH in one cell.

Default physical layer parameters for DCI format 0_1 are specified in table 4.3.6.1.1.2-1.

Table 4.3.6.1.1.2-1: Physical layer parameters for DCI format 0_1

| Parameter | Value | Value in binary | Condition |
| --- | --- | --- | --- |
| Carrier indicator | Not present | - |  |
| UL/SUL indicator | Not present (0 bit for UEs not configured with SUL in the cell) | - |  |
|  | 1 | 1 | SUL |
| Identifier for DCI formats | Indicating an UL DCI format | “0” |  |
| Bandwidth part indicator | Not present (indicating active BWP, not present in case of only one BWP-Id as per Table 4.6.3-8) | - |  |
| Frequency domain resource assignment | Dependent on test parameters | - |  |
| Time domain resource assignment | Indicating the first entry of PUSCH-TimeDomainResourceAllocationList to be used | “0000” |  |
| Frequency hopping flag | Not present | - |  |
| Modulation and coding scheme | Dependent on test parameters | - |  |
| New data indicator | Set for every data transmission / retransmission according to the rules specified in TS 38.321 [20] | - |  |
| Redundancy version | Dependent on test parameters | - |  |
| HARQ process number | Dependent on test parameters | - |  |
| 1st downlink assignment index | Dependent on test parameters |  |  |
| 2nd downlink assignment index | Not present (0 bit if one HARQ-ACK sub-codebook) | - |  |
| TPC command for scheduled PUSCH | 0 dB (accumulated TPC) as per Table 7.1.1-1 in TS 38.213 [22] | “01” |  |
| SRS resource indicator | Not present | - |  |
| Precoding information and number of layers | Not present (0 bits for 1 antenna port and if the higher layer parameter txConfig = Codebook as per clause 7.3.1.1.2 TS 38.212 [27]) | - |  |
|  | 2 | “10” | 2TX_UL_MIMO or ULFPTx_Mode1 |
|  | Dependent on UE reported full power PMI | 1 bit as per Table 7.3.1.1.2-5 in TS 38.212 | ULFPTx_Mode2 |
|  | 0 or 1 | 1 bit | ULFPTx_ModeFull |
| Antenna ports | Port 0 (NOTE 2) | “000” |  |
|  |  | “00” | TRANSFORM _PRECODER _ENABLED |
| SRS request | No aperiodic SRS resource set triggered as per Table 7.3.1.1.2-24 in TS 38.212 (no SUL configured) | “00” |  |
| CSI request | Not present | - |  |
| CBG transmission information | Not present | - |  |
| PTRS-DMRS association | DMRS port 0 | “00” | PTRS_UL_CONFIG |
|  | Not present | - |  |
| beta_offset indicator | Not present (0 bit if the higher layer parameter dynamic in uci-on-PUSCH is not configured) | - |  |
| DMRS sequence initialization | = 0 (ScramblingID0 is not present as per Table 4.6.3-50) | “0” |  |
|  | Not present | - | TRANSFORM _PRECODER _ENABLED |
| UL-SCH indicator | Dependent on test parameters 1 bit. A value of "1" indicates UL-SCH shall be transmitted on the PUSCH and a value of "0" indicates UL-SCH shall not be transmitted on the PUSCH. | - |  |
| NOTE 1: codebookSubset = nonCoherent, 2 layers,TPMI = 0 as specified in TS 38.212 [27] Table 7.3.1.1.2-4 NOTE 2: Bitsize depends on transform precoder being enabled/disabled (PUSCH_Config, Table 4.6.3-118) and on dmrs-Type and maxLength (DMRS-UplinkConfig, Table 4.6.3-51); 3 bits (transform precoder disabled) or 2 bits (transform precoder enabled) for DMRS type 1 and len1 |  |  |  |

| Condition | Explanation |
| --- | --- |
| 2TX_UL_MIMO | UL-MIMO test cases with 2 Tx antenna ports |
| PTRS_UL_CONFIG | When PTRS Uplink is configured |
| TRANSFORM_PRECODER_ENABLED | Transform precoding is enabled (PUSCH_Config, Table 4.6.3-118) |
| SUL | On the SUL carrier when supplementary carrier is configured |
| ULFPTx_Mode1 | UL-MIMO test cases with UEs supporting UL full power transmission Mode-1 |
| ULFPTx_Mode2 | UL-MIMO test cases with UEs supporting UL full power transmission Mode-2 |
| ULFPTx_ModeFull | UL-MIMO test cases with UEs supporting UL full power transmission Mode-full power |

It can be observed that Precoding information and number of layers is set to ‘2’ only for 2TX-UL-MIMO or ULFPTx_Mode1. These conditions need to be extended also for testing of coherent devices.

Proposal 5: Update Physical layer parameters for DCI format 0_1 in 3GPP TS 38.508-1 [6] as follows to implement TPMI Test method for coherent devices:

Table 4.3.6.1.1.2-1: Physical layer parameters for DCI format 0_1

| Parameter | Value | Value in binary | Condition |
| --- | --- | --- | --- |
| Carrier indicator | Not present | - |  |
| UL/SUL indicator | Not present (0 bit for UEs not configured with SUL in the cell) | - |  |
|  | 1 | 1 | SUL |
| Identifier for DCI formats | Indicating an UL DCI format | “0” |  |
| Bandwidth part indicator | Not present (indicating active BWP, not present in case of only one BWP-Id as per Table 4.6.3-8) | - |  |
| Frequency domain resource assignment | Dependent on test parameters | - |  |
| Time domain resource assignment | Indicating the first entry of PUSCH-TimeDomainResourceAllocationList to be used | “0000” |  |
| Frequency hopping flag | Not present | - |  |
| Modulation and coding scheme | Dependent on test parameters | - |  |
| New data indicator | Set for every data transmission / retransmission according to the rules specified in TS 38.321 [20] | - |  |
| Redundancy version | Dependent on test parameters | - |  |
| HARQ process number | Dependent on test parameters | - |  |
| 1st downlink assignment index | Dependent on test parameters |  |  |
| 2nd downlink assignment index | Not present (0 bit if one HARQ-ACK sub-codebook) | - |  |
| TPC command for scheduled PUSCH | 0 dB (accumulated TPC) as per Table 7.1.1-1 in TS 38.213 [22] | “01” |  |
| SRS resource indicator | Not present | - |  |
| Precoding information and number of layers | Not present (0 bits for 1 antenna port and if the higher layer parameter txConfig = Codebook as per clause 7.3.1.1.2 TS 38.212 [27]) | - |  |
|  | 2 | “10” | 2TX_UL_MIMO or ULFPTx_Mode1 or Coherent_Not_UL_MIMO |
|  | Dependent on UE reported full power PMI | 1 bit as per Table 7.3.1.1.2-5 in TS 38.212 | ULFPTx_Mode2 |
|  | 0 or 1 | 1 bit | ULFPTx_ModeFull |
| Antenna ports | Port 0 (NOTE 2) | “000” |  |
|  |  | “00” | TRANSFORM _PRECODER _ENABLED |
| SRS request | No aperiodic SRS resource set triggered as per Table 7.3.1.1.2-24 in TS 38.212 (no SUL configured) | “00” |  |
| CSI request | Not present | - |  |
| CBG transmission information | Not present | - |  |
| PTRS-DMRS association | DMRS port 0 | “00” | PTRS_UL_CONFIG |
|  | Not present | - |  |
| beta_offset indicator | Not present (0 bit if the higher layer parameter dynamic in uci-on-PUSCH is not configured) | - |  |
| DMRS sequence initialization | = 0 (ScramblingID0 is not present as per Table 4.6.3-50) | “0” |  |
|  | Not present | - | TRANSFORM _PRECODER _ENABLED |
| UL-SCH indicator | Dependent on test parameters 1 bit. A value of "1" indicates UL-SCH shall be transmitted on the PUSCH and a value of "0" indicates UL-SCH shall not be transmitted on the PUSCH. | - |  |
| NOTE 1: codebookSubset = nonCoherent, 2 layers,TPMI = 0 as specified in TS 38.212 [27] Table 7.3.1.1.2-4 NOTE 2: Bitsize depends on transform precoder being enabled/disabled (PUSCH_Config, Table 4.6.3-118) and on dmrs-Type and maxLength (DMRS-UplinkConfig, Table 4.6.3-51); 3 bits (transform precoder disabled) or 2 bits (transform precoder enabled) for DMRS type 1 and len1 |  |  |  |

| Condition | Explanation |
| --- | --- |
| 2TX_UL_MIMO | UL-MIMO test cases with 2 Tx antenna ports |
| Coherent_Not_UL_MIMO | Non UL-MIMO RF test cases for UEs with puschTransCoherence=FullCoherent |
| PTRS_UL_CONFIG | When PTRS Uplink is configured |
| TRANSFORM_PRECODER_ENABLED | Transform precoding is enabled (PUSCH_Config, Table 4.6.3-118) |
| SUL | On the SUL carrier when supplementary carrier is configured |
| ULFPTx_Mode1 | UL-MIMO test cases with UEs supporting UL full power transmission Mode-1 |
| ULFPTx_Mode2 | UL-MIMO test cases with UEs supporting UL full power transmission Mode-2 |
| ULFPTx_ModeFull | UL-MIMO test cases with UEs supporting UL full power transmission Mode-full power |

2.3 TPMI test method implementation for UEs supporting ul-FullPwrMode1-r16 capability

2.3.1 PUSCH-Config

As shown in clause 2.2.1, PUSCH-Config configuration already considers ul-FullPowerTransmission-r16 set to ‘fullpowerMode1’ and maxRank is already set to ‘1’ if ULFPTX_Mode1, i.e. in UL-MIMO test cases with UEs supporting UL full power transmission Mode-1.

Observation 2: No update is required in PUSCH-Config in 3GPP TS 38.508-1 [6] to implement TPMI test method for UEs supporting ul-FullPwrMode1-r16 capability.

2.3.2 SRS-Config

When checking SRS-Config, it can be seen that SRS-ResourceSet usage is already set to ‘codebook’ and SRS- Resource nrofports is already set to ‘ports2’ if 2TX_UL_MIMO and set to ‘ports1’ otherwise. Hence no update is required on SRS-Config.

Observation 3: No update is required on SRS-Config in 3GPP TS 38.508-1 [6] for TPMI test method implementation for UEs supporting ul-FullPwrMode1-r16 capability.

2.3.3 TPMI Configuration

As shown in clause 2.2.3, Precoding information and number of layers are already set to ‘2’ in Physical layer parameters for DCI format 0_1 if ULFPTxMode1, i.e. in UL-MIMO test cases with UEs supporting UL full power transmission Mode-1.

Observation 4: No update is required in Physical layer parameters for DCI format 0_1 in 3GPP TS 38.508-1 [6] for TPMI test method implementation for UEs supporting ul-FullPwrMode1-r16 capability.

# 3. 	Conclusion

This document analyses how to introduce TPMI test method in 3GPP TS 38.521-2 test specification [3] based on the applicability analysis in R5-22xxxx [4]. The following observation and proposals are done:

Observation 1: ul-FullPowerTransmission = fullpowerMode1 and codebookSubset=fullAndPartialAndNonCoherent can not be configured in the network simultaneously.

Proposal 1: TPMI test method implementation needs to be decoupled for coherent UEs and for UEs supporting ul-FullPwrMode1-r16 capability.

Proposal 2: Test TPMI test method implementation for coherent UEs in non-UL-MIMO test cases and TPMI test method implementation for UEs supporting ul-FullPwerMode1-r16 in UL-MIMO test cases.

Proposal 3: Extend PUSCH-Config for RF Test cases in 3GPP TS 38.508-1 [6] as shown below:

Table 5.4.1-4: PUSCH-Config

| Derivation Path: Clause 4.6.3 Table 4.6.3-118 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| codebookSubset | nonCoherent |  |  |
|  | fullyAndPartialAndNonCoherent |  | Coherent_Not_UL_MIMO |
| maxRank | 2 |  | 2TX_UL_MIMO |
|  | 1 |  | ULFPTx_Mode1 or ULFPTx_Mode2 or ULFPTx_ModeFull or Coherent_Not_UL_MIMO |
| ul-FullPowerTransmission-r16 | Not present |  | 2TX_UL_MIMO |
|  | fullpowerMode1 |  | ULFPTx_Mode1 |
|  | fullpowerMode2 |  | ULFPTx_Mode2 |
|  | fullpower |  | ULFPTx_ModeFull |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| 2TX_UL_MIMO | UL-MIMO test cases with 2 Tx antenna ports |
| Coherent_Not_UL_MIMO | Non UL-MIMO RF test cases for UEs with puschTransCoherence=FullCoherent |
| ULFPTx_Mode1 | UL-MIMO test cases with UEs supporting UL full power transmission Mode-1 |
| ULFPTx_Mode2 | UL-MIMO test cases with UEs supporting UL full power transmission Mode-2 |
| ULFPTx_ModeFull | UL-MIMO test cases with UEs supporting UL full power transmission Mode-full power |

Proposal 4: Condition for SRS-ResourceSet nrofports needs to be extended to Coherent devices for Non UL-MIMO test cases:

SRS-Config

Table 4.6.3-182: SRS-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SRS-Config ::= SEQUENCE { |  |  |  |
| srs-ResourceSetToReleaseList | Not present |  |  |
| srs-ResourceSetToAddModList SEQUENCE (SIZE(0..maxNrofSRS-ResourceSets)) OF SRS-ResourceSet { | 1 entry |  |  |
| SRS-ResourceSet[1] SEQUENCE { |  | entry 1 |  |
| srs-ResourceSetId | 0 |  |  |
| srs-ResourceIdList SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-ResourceId { | 1 entry |  |  |
| SRS-ResourceId[1] | 0 | entry 1 |  |
| } |  |  |  |
| resourceType CHOICE { |  |  |  |
| aperiodic SEQUENCE { |  |  |  |
| aperiodicSRS-ResourceTrigger | 1 |  |  |
| csi-RS | Not present |  |  |
| slotOffset | 7 |  | FR1 |
|  | 4 |  | FR2 |
| } |  |  |  |
| } |  |  |  |
| usage | codebook |  |  |
| alpha | Alpha |  |  |
| p0 | 0 |  |  |
| pathlossReferenceRS CHOICE { |  |  |  |
| ssb-Index | SSB-Index |  |  |
| } |  |  |  |
| srs-PowerControlAdjustmentStates | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| srs-ResourceToReleaseList | Not present |  |  |
| srs-ResourceToAddModList SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-Resource { | 1 entry |  |  |
| SRS-Resource[1] SEQUENCE { |  | entry 1 |  |
| srs-ResourceId | 0 |  |  |
| nrofSRS-Ports | ports2 |  | 2TX_UL_MIMO or Coherent_Not_UL:MIMO |
|  | port1 |  |  |
| ptrs-PortIndex | Not present |  |  |
| transmissionComb CHOICE { |  |  |  |
| n2 SEQUENCE { |  |  |  |
| combOffset-n2 | 0 |  |  |
| cyclicShift-n2 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| resourceMapping SEQUENCE { |  |  |  |
| startPosition | 0 |  |  |
| nrofSymbols | n1 |  |  |
| repetitionFactor | n1 |  |  |
| } |  |  |  |
| freqDomainPosition | 0 |  |  |
| freqDomainShift | 0 |  |  |
| freqHopping SEQUENCE { |  |  |  |
| c-SRS | 0 |  |  |
|  | 63 |  | FR1_100MHz |
|  | 17 |  | FR2_100MHz |
| b-SRS | 0 |  |  |
| b-hop | 0 |  |  |
| } |  |  |  |
| groupOrSequenceHopping | groupHopping |  |  |
| resourceType CHOICE { |  |  |  |
| aperiodic SEQUENCE { |  |  |  |
| } |  |  |  |
| } |  |  |  |
| sequenceId | 0 |  |  |
| spatialRelationInfo SEQUENCE { | SRS-SpatialRelationInfo |  |  |
| servingCellId | Not present |  |  |
| referenceSignal CHOICE { |  |  |  |
| ssb-Index | SSB-Index |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| tpc-Accumulation | Not present |  |  |
| srs-RequestDCI-1-2-r16 | Not present |  |  |
| srs-RequestDCI-0-2-r16 | Not present |  |  |
| srs-ResourceSetToAddModListDCI-0-2-r16 | Not present |  |  |
| srs-ResourceSetToReleaseListDCI-0-2-r16 | Not present |  |  |
| srs-PosResourceSetToReleaseList-r16 | Not present |  |  |
| srs-PosResourceSetToAddModList-r16 | Not present |  |  |
| srs-PosResourceToReleaseList-r16 | Not present |  |  |
| srs-PosResourceToAddModList-r16 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| 2TX_UL_MIMO | UL-MIMO test cases with 2 Tx antenna ports |
| Coherent_Not_UL_MIMO | Non UL-MIMO RF test cases for UEs with puschTransCoherence=FullCoherent |
| FR1_100MHz | FR1 is used under the test. CBW is set to 100MHz. |
| FR2_100MHz | FR2 is used under the test. CBW is set to 100MHz. |

Proposal 5: Update Physical layer parameters for DCI format 0_1 in 3GPP TS 38.508-1 [6] as follows to implement TPMI Test method for coherent devices:

Table 4.3.6.1.1.2-1: Physical layer parameters for DCI format 0_1

| Parameter | Value | Value in binary | Condition |
| --- | --- | --- | --- |
| Carrier indicator | Not present | - |  |
| UL/SUL indicator | Not present (0 bit for UEs not configured with SUL in the cell) | - |  |
|  | 1 | 1 | SUL |
| Identifier for DCI formats | Indicating an UL DCI format | “0” |  |
| Bandwidth part indicator | Not present (indicating active BWP, not present in case of only one BWP-Id as per Table 4.6.3-8) | - |  |
| Frequency domain resource assignment | Dependent on test parameters | - |  |
| Time domain resource assignment | Indicating the first entry of PUSCH-TimeDomainResourceAllocationList to be used | “0000” |  |
| Frequency hopping flag | Not present | - |  |
| Modulation and coding scheme | Dependent on test parameters | - |  |
| New data indicator | Set for every data transmission / retransmission according to the rules specified in TS 38.321 [20] | - |  |
| Redundancy version | Dependent on test parameters | - |  |
| HARQ process number | Dependent on test parameters | - |  |
| 1st downlink assignment index | Dependent on test parameters |  |  |
| 2nd downlink assignment index | Not present (0 bit if one HARQ-ACK sub-codebook) | - |  |
| TPC command for scheduled PUSCH | 0 dB (accumulated TPC) as per Table 7.1.1-1 in TS 38.213 [22] | “01” |  |
| SRS resource indicator | Not present | - |  |
| Precoding information and number of layers | Not present (0 bits for 1 antenna port and if the higher layer parameter txConfig = Codebook as per clause 7.3.1.1.2 TS 38.212 [27]) | - |  |
|  | 2 | “10” | 2TX_UL_MIMO or ULFPTx_Mode1 or Coherent_Not_UL_MIMO |
|  | Dependent on UE reported full power PMI | 1 bit as per Table 7.3.1.1.2-5 in TS 38.212 | ULFPTx_Mode2 |
|  | 0 or 1 | 1 bit | ULFPTx_ModeFull |
| Antenna ports | Port 0 (NOTE 2) | “000” |  |
|  |  | “00” | TRANSFORM _PRECODER _ENABLED |
| SRS request | No aperiodic SRS resource set triggered as per Table 7.3.1.1.2-24 in TS 38.212 (no SUL configured) | “00” |  |
| CSI request | Not present | - |  |
| CBG transmission information | Not present | - |  |
| PTRS-DMRS association | DMRS port 0 | “00” | PTRS_UL_CONFIG |
|  | Not present | - |  |
| beta_offset indicator | Not present (0 bit if the higher layer parameter dynamic in uci-on-PUSCH is not configured) | - |  |
| DMRS sequence initialization | = 0 (ScramblingID0 is not present as per Table 4.6.3-50) | “0” |  |
|  | Not present | - | TRANSFORM _PRECODER _ENABLED |
| UL-SCH indicator | Dependent on test parameters 1 bit. A value of "1" indicates UL-SCH shall be transmitted on the PUSCH and a value of "0" indicates UL-SCH shall not be transmitted on the PUSCH. | - |  |
| NOTE 1: codebookSubset = nonCoherent, 2 layers,TPMI = 0 as specified in TS 38.212 [27] Table 7.3.1.1.2-4 NOTE 2: Bitsize depends on transform precoder being enabled/disabled (PUSCH_Config, Table 4.6.3-118) and on dmrs-Type and maxLength (DMRS-UplinkConfig, Table 4.6.3-51); 3 bits (transform precoder disabled) or 2 bits (transform precoder enabled) for DMRS type 1 and len1 |  |  |  |

| Condition | Explanation |
| --- | --- |
| 2TX_UL_MIMO | UL-MIMO test cases with 2 Tx antenna ports |
| Coherent_Not_UL_MIMO | Non UL-MIMO RF test cases for UEs with puschTransCoherence=FullCoherent |
| PTRS_UL_CONFIG | When PTRS Uplink is configured |
| TRANSFORM_PRECODER_ENABLED | Transform precoding is enabled (PUSCH_Config, Table 4.6.3-118) |
| SUL | On the SUL carrier when supplementary carrier is configured |
| ULFPTx_Mode1 | UL-MIMO test cases with UEs supporting UL full power transmission Mode-1 |
| ULFPTx_Mode2 | UL-MIMO test cases with UEs supporting UL full power transmission Mode-2 |
| ULFPTx_ModeFull | UL-MIMO test cases with UEs supporting UL full power transmission Mode-full power |

Observation 2: No update is required in PUSCH-Config in 3GPP TS 38.508-1 [6] to implement TPMI test method for UEs supporting ul-FullPwrMode1-r16 capability.

Observation 3: No update is required on SRS-Config in 3GPP TS 38.508-1 [6] for TPMI test method implementation for UEs supporting ul-FullPwrMode1-r16 capability.

Observation 4: No update is required in Physical layer parameters for DCI format 0_1 in 3GPP TS 38.508-1 [6] for TPMI test method implementation for UEs supporting ul-FullPwrMode1-r16 capability.

Required changes according to proposals 1 to 4 have been implemented in [7] and submitted to RAN5#96-6 for agreement.

# 4.	References

- R5-223028, “Draft internal Work Plan for FR2 Enhanced Test Methods”, Apple Portugal, RAN5#95-e

- 3GPP TR 38.884, “Study on enhanced test methods for FR2 NR UEs”

- 3GPP TS 38.521-2, “User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone”

- R5-224112, “Discussion on TPMI Test method applicability”, Keysight Technologies UK Ltd, RAN5#96-e

- 3GPP TS 38.214, “Physical layer procedures for data”

- 3GPP TS 38.508-1, “User Equipment (UE) conformance specification; Part 1: Common test environment”

- R5-224118, “38.508-1 Updates implementing TPMI test method for coherent UEs”, Keysight Technologies UK Ltd., RAN5#96-e