3GPP TSG-RAN5 Meeting #96-e	R5-224292
Electronic Meeting, 15th August – 26th August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1794 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NS_15 minimum requirements missing in test case 6.5.3.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_bands_BW_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In additional spurious emission test case 6.5.3.3, everything is defined to test NS_15 (test requirements, message exceptions, test tolerances…) except for the minimum requirements that are TBD. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Defined minimum requirements for NS_15 in clause 6.5.3.3.3.20. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

#### 6.5.3.3	Additional spurious emissions

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined

Initial conditions for NS_37, NS_38, NS_39, NS_40, NS_41, NS_42 and NS_45 are incomplete.

The requirements of this test case for NS_44, NS_46, NS_47, NS_48, and NS_49 apply to all types of NR UE release 16 forward, and release 15 if the corresponding channel bandwidths are supported.

6.5.3.3.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions under the deployment scenarios where additional requirements are specified.

6.5.3.3.2	Test applicability

This test case applies to all types of NR UE release 15 and forward.

6.5.3.3.3	Minimum conformance requirements

These requirements are specified in terms of an additional spectrum emission requirement. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

6.5.3.3.3.1	Minimum conformance requirements (network signalling value "NS_04")

When "NS 04" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.3.1-1.  This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1.3-1 from the edge of the channel bandwidth.

Table 6.5.3.3.3.1-1: Additional requirements for “NS_04”

| Frequency band (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| 2495 ≤ f < 2496 | -13 | 1% of Channel BW |
| 2490.5 ≤ f < 2495 | -13 | 1 MHz |
| 0.009 < f < 2490.5 | -25 | 1 MHz |

The normative reference for this requirement is TS 38.101-1 [2] subclause 6.5.3.3.1.

6.5.3.3.3.2	Minimum conformance requirements (network signalling value "NS_17")

When “NS_17” is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.3.2-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1.3-1 from the edge of the channel bandwidth.

Table 6.5.3.3.3.2-1: Additional requirements for “NS_17”

| Frequency band (MHz) | Channel bandwidth (MHz)/ Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5, 10 |  |  |
| 470 ≤ f ≤ 710 | -26.2 | 6 MHz | 1 |
| NOTE 1: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth is 5 or 10 MHz. |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] subclause 6.5.3.3.2.

## <<< Skip unchanged sections >>>

6.5.3.3.3.19	Requirement for network signalling value "NS_14"

When "NS_14" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.3.19-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1.3-1 from the edge of the channel bandwidth.

Table 6.5.3.3.3.19-1: Additional requirements for “NS_14”

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 10 MHz, 15 MHz, 20MHz |  |
| 806 ≤ f ≤ 816 | -42 | 6.25 kHz |
| NOTE 1: The requirement applies for NR carriers with lower channel edge at or above 824 MHz. NOTE 2: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

6.5.3.3.3.20	Requirement for network signalling value "NS_15"

When "NS_15" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.20-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1.3-1 from the edge of the channel bandwidth.

Table 6.5.3.3.3.20-1: Additional requirements NS_15

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |
| 851 ≤ f ≤ 859 | -53 | 6.25 kHz |
| NOTE 1: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

6.5.3.3.3.21	Requirement for network signalling value "NS_45"

When "NS_45" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.3.21-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1.3-1 from the edge of the channel bandwidth.

Table 6.5.3.3.3.21-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) |  | Measurement bandwidth |
| --- | --- | --- | --- |
|  | 5 MHz | 10 MHz |  |
| 0.009 < f ≤ 2473.5 | -25 | -25 | 1 MHz |
| 2473.5 < f ≤ 2477.5 | -25 | -13 | 1 MHz |
| 2477.5 < f ≤ 2478.5 | -13 | -13 | 1 MHz |
| 2478.5< f ≤ 2483.5 | -10 | -10 | 1 MHz |
| 2495 ≤ f < 2496 | -13 | -13 | 1% of Channel Bandwidth |
| 2496 ≤ f < 2501 | -13 | -13 | 1 MHz |
| 2501 < f ≤ 2505 | -25 | -13 | 1 MHz |
| 2505 ≤ f ≤ 5th harmonic of the upper frequency edge of the UL operating band | -25 | -25 | 1 MHz |

## <<< END OF CHANGES >>>