3GPP TSG-RAN WG5#96-e		R5-224315

# Electronic Meeting, 15 – 26 August 2022

Source: 	Ericsson, Huawei, Apple

Title:	Draft ITU-R recommendation on Unwanted Emissions for IMT-2020

Agenda Item:	4.2.2

Document for:	Discussion and Endorsement

This document will be attached to R5-224316, LS on work towards two new recommendations "Generic unwanted emission characteristics of base stations/mobile stations using the terrestrial radio interfaces of IMT-2020"

Annex 1

3GPP 5G

The present Annex includes unwanted emission requirements from long-term evolution (LTE) and new radio (NR) Releases for Mobile Stations. In 3GPP terminology, the term Evolved-UMTS Terrestrial Radio Access (E-UTRA) is also used to signify the LTE radio interface.

This Annex is divided into four parts:

–	Chapter 1 contains the E-UTRA MS unwanted emission requirements.

–	Chapter 2 contains the NR range 1 standalone unwanted emission requirements.

–	Chapter 3 contains the NR range 2 standalone unwanted emission requirements.

–	Chapter 4 contains the NR range 1 and range 2 interworking operation with other radios

Values specified in the present Annex incorporate test tolerances defined in Recommendation ITU R M.1545 [10].

# Generic unwanted emission characteristics for E-UTRA

## 1.1	Operating bands and channel arrangements

### 1.1.1	Operating bands

E-UTRA is designed to operate in the FR1 operating bands defined in Table 1.1.1-1.

TABLE  1.1.1-1

E-UTRA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high |  |  | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
| 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
| 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| 4 | 1710 MHz | – | 1755 MHz | 2110 MHz | – | 2155 MHz | FDD |
| 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894MHz | FDD |
| 61 | 830 MHz | – | 840 MHz | 875 MHz | – | 885 MHz | FDD |
| 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
| 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
| 9 | 1749.9 MHz | – | 1784.9 MHz | 1844.9 MHz | – | 1879.9 MHz | FDD |
| 10 | 1710 MHz | – | 1770 MHz | 2110 MHz | – | 2170 MHz | FDD |
| 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
| 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
| 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
| 14 | 788 MHz | – | 798 MHz | 758 MHz | – | 768 MHz | FDD |
| 15 | Reserved |  |  | Reserved |  |  | FDD |
| 16 | Reserved |  |  | Reserved |  |  | FDD |
| 17 | 704 MHz | – | 716 MHz | 734 MHz | – | 746 MHz | FDD |
| 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
| 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
| 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
| 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
| 22 | 3410 MHz | – | 3490 MHz | 3510 MHz | – | 3590 MHz | FDD |
| 231 | 2000 MHz | – | 2020 MHz | 2180 MHz | – | 2200 MHz | FDD |
| 2417 | 1626.5 MHz | – | 1660.5 MHz | 1525 MHz | – | 1559 MHz | FDD |
| 25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
| 26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
| 27 | 807 MHz | – | 824 MHz | 852 MHz | – | 869 MHz | FDD |
| 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
| 29 | NA |  |  | 717 MHz | – | 728 MHz | FDD2 |
| 30 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz | FDD15 |
| 31 | 452.5 MHz | – | 457.5 MHz | 462.5 MHz | – | 467.5 MHz | FDD |
| 32 | NA |  |  | 1452 MHz | – | 1496 MHz | FDD2 |
| 33 | 1900 MHz | – | 1920 MHz | 1900 MHz | – | 1920 MHz | TDD |
| 34 | 2010 MHz | – | 2025 MHz | 2010 MHz | – | 2025 MHz | TDD |
| 35 | 1850 MHz | – | 1910 MHz | 1850 MHz | – | 1910 MHz | TDD |
| 36 | 1930 MHz | – | 1990 MHz | 1930 MHz | – | 1990 MHz | TDD |
| 37 | 1910 MHz | – | 1930 MHz | 1910 MHz | – | 1930 MHz | TDD |
| 38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
| 39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |
| 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
| 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
| 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
| 43 | 3600 MHz | – | 3800 MHz | 3600 MHz | – | 3800 MHz | TDD |
| 44 | 703 MHz | – | 803 MHz | 703 MHz | – | 803 MHz | TDD |
| 45 | 1447 MHz | – | 1467 MHz | 1447 MHz | – | 1467 MHz | TDD |
| 46 | 5150 MHz | – | 5925 MHz | 5150 MHz | – | 5925 MHz | TDD8,9 |
| 47 | 5855 MHz | – | 5925 MHz | 5855 MHz | – | 5925 MHz | TDD11 |
| 48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |
| 50 | 1432 MHz | - | 1517 MHz | 1432 MHz | - | 1517 MHz | TDD13 |
| 51 | 1427 MHz | - | 1432 MHz | 1427 MHz | - | 1432 MHz | TDD13 |
| … |  |  |  |  |  |  |  |
| 53 | 2483.5 MHz | - | 2495 MHz | 2483.5 MHz | - | 2495 MHz | TDD |
| … |  |  |  |  |  |  |  |
| 64 | Reserved |  |  | Reserved |  |  | FDD |
| 65 | 1920 MHz | – | 2010 MHz | 2110 MHz | – | 2200 MHz | FDD |
| 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD4 |
| 67 | NA |  |  | 738 MHz | – | 758 MHz | FDD2 |
| 68 | 698 MHz | – | 728 MHz | 753 MHz | – | 783 MHz | FDD |
| 69 | NA |  |  | 2570 MHz | – | 2620 MHz | FDD2 |
| 70 | 1695 MHz | – | 1710 MHz | 1995 MHz | – | 2020 MHz | FDD10 |
| 71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
| 72 | 451 MHz | – | 456 MHz | 461 MHz | – | 466 MHz | FDD |
| 73 | 450MHz | – | 455 MHz | 460MHz | – | 465 MHz | FDD |
| 74 | 1427 MHz | – | 1470 MHz | 1475 MHz | – | 1518 MHz | FDD |
| 75 | NA |  |  | 1432 MHz | – | 1517 MHz | FDD2 |
| 76 | NA |  |  | 1427 MHz | – | 1432 MHz | FDD2 |
| 85 | 698 MHz | – | 716 MHz | 728 MHz | – | 746 MHz | FDD |
| 87 | 410 MHz | – | 415 MHz | 420 MHz | – | 425 MHz | FDD |
| 88 | 412 MHz | – | 417 MHz | 422 MHz | – | 427 MHz | FDD |

NOTE 1 – Band 6, 23 is not applicable.

NOTE 2 – Restricted to E-UTRA operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell.

NOTE 3 – A UE that complies with the E-UTRA Band 65 minimum requirements in this specification shall also comply with the E-UTRA Band 1 minimum requirements.

NOTE 4 – The range 2180-2200 MHz of the DL operating band is restricted to E-UTRA operation when carrier aggregation is configured.

NOTE 5 – A UE that supports E-UTRA Band 66 shall receive in the entire DL operating band

NOTE 6 – A UE that supports E-UTRA Band 66 and CA operation in any CA band shall also comply with the minimum requirements specified for the DL CA configurations CA_66B, CA_66C and CA_66A-66A.

NOTE 7 – A UE that complies with the E-UTRA Band 66 minimum requirements in this specification shall also comply with the E-UTRA Band 4 minimum requirements.

NOTE 8 – This band is an unlicensed band restricted to licensed-assisted operation using Frame Structure Type 3

NOTE 9 – In this version of the specification, restricted to E-UTRA DL operation when carrier aggregation is configured.

NOTE 10 – The range 2010-2020 MHz of the DL operating band is restricted to E-UTRA operation when carrier aggregation is configured and TX-RX separation is 300 MHz The range 2005-2020 MHz of the DL operating band is restricted to E-UTRA operation when carrier aggregation is configured and TX-RX separation is 295 MHz.

NOTE 11 This band is unlicensed band used for V2X communication. There is no expected network deployment in this band so Frame Structure Type 1 is used.

NOTE 12 – A UE that complies with the E-UTRA Band 74 minimum requirements in this specification shall also comply with the E-UTRA Band 11 and Band 21 minimum requirements.

NOTE 13 – 	UE that complies with the E-UTRA Band 50 minimum requirements in this specification shall also comply with the E-UTRA Band 51 minimum requirements.

NOTE 14 – A UE that complies with the E-UTRA Band 75 minimum requirements in this specification shall also comply with the E-UTRA Band 76 minimum requirements.

NOTE 15 – Uplink transmission is not allowed at this band for UE with external vehicle-mounted antennas.

NOTE 16 – This band is restricted to licensed-assisted operation using Frame Structure Type 3

NOTE 17 – DL operation in this band is restricted to 1526 – 1536 MHz and UL operation is restricted to 1627.5 – 1637.5 MHz and 1646.5 – 1656.5 MHz.

The UE operating bands for E-UTRA carrier aggregation are defined in TS 36.101 [1] clause 5.5A.

The UE operating bands for E-UTRA UL-MIMO are defined in TS 36.101 [1] clause 5.5B.

The UE operating bands for E-UTRA dual connectivity are defined in TS 36.101 [1] clause 5.5C.

The UE operating bands for E-UTRA ProSe are defined in TS 36.101 [1] clause 5.5D.

The UE operating bands for UE category 0, UE category M1 and M2 and UE category 1bis are defined in TS 36.101 [1] clause 5.5E.

The UE operating bands for category NB1 and NB2 are defined in TS 36.101 [1] clause 5.5F.

The UE operating bands for V2X Communication are defined in TS 36.101 [1] clause 5.5G.

### 1.1.2	UE Channel bandwidth

The UE channel bandwidth for different E-UTRA bands are defined in TS 36.101-1 [1] clause 5.6.

The UE CA configurations and bandwidth combination sets are defined in TS 36.101-1 [1] clause 5.6A.

The UE channel bandwidth for UL-MIMO are defined in TS 36.101-1 [1] clause 5.6B.

The UE channel bandwidth for Dual Connectivity are defined in TS 36.101-1 [1] clause 5.6C.

The UE channel bandwidth for ProSe.are defined in TS 36.101-1 [1] clause 5.6D.

The UE channel bandwidth for category NB1 and NB2 are defined in TS 36.101-1 [1] clause 5.6F.

The UE V2X configurations and bandwidth combination sets channel bandwidth are defined in TS 36.101-1 [1] clause 5.6G.

### 1.1.3	Channel arrangement

The UE channel spacing, channel raster, channel number and TX–RX frequency separation are defined in TS 36.101-1 [1] clause 5.7.1, 5.7.2, 5.7.3 and 5.7.4 respectively.

The UE channel spacing for CA, channel raster for CA and TX–RX frequency separation for CA are defined in TS 36.101-1 [1] clause 5.7.1A, 5.7.2A and 5.7.4A respectively.

The UE channel spacing for category NB1 and NB2, channel raster for category NB1 and NB2, channel number for category NB1 and NB2, and TX–RX frequency separation for category NB1 and NB2 are defined in TS 36.101-1 [1] clause 5.7.1F, 5.7.2F, 5.7.3F and 5.7.4A respectively.

The UE TX–RX frequency separation for category M1 and M2 are defined in TS 36.101-1 [1] clause 5.7.4E.

### 1.1.4	Void

### 1.1.5	Definitions, symbols and abbreviations

#### 1.1.5.1	Definitions

Carrier aggregation: Aggregation of two or more component carriers in order to support wider transmission bandwidths.

Carrier aggregation configuration: A combination of CA operating band(s) and CA bandwidth class(es) supported by a UE.

Channel bandwidth: The RF bandwidth supporting a single E-UTRA RF carrier with the transmission bandwidth configured in the uplink or downlink of a cell. The channel bandwidth is measured in MHz and is used as a reference for transmitter and receiver RF requirements.

Inter-band carrier aggregation: Carrier aggregation of component carriers in different operating bands.

Intra-band contiguous carrier aggregation: Contiguous carriers aggregated in the same operating band.

Intra-band non-contiguous carrier aggregation: Non-contiguous carriers aggregated in the same operating band.

#### 1.1.5.2	Symbols

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

#### 1.1.5.3	Abbreviations

ACLR	Adjacent Channel Leakage Ratio

CA	Carrier Aggregation

DC	Dual Connectivity

DL	Downlink

E-UTRA	Evolved UTRA

FDD	Frequency division duplex

FR	Frequency Range

NR	New Radio

NS	Network Signalling

TDD	Time division duplex

UE	User equipment

UL	Uplink

UL MIMO	Uplink Multiple Antenna transmission

V2X	Vehicle to Everything

# 1.2	RF spectrum emissions

# 1.2.1	Out of band emission

#### 1.2.1.1	Spectrum emission mask

The general E-UTRA spectrum emission mask limits are defined in TS 36.521-1 [5] clause 6.6.2.1.5.

The E-UTRA spectrum emission mask limits for CA are defined in TS 36.521-1 [5] clause 6.6.2.1A.1.5.

The E-UTRA spectrum emission mask limits for UL-MIMO  are defined in TS 36.521-1 [5] clause 6.6.2.1B.5.

The E-UTRA spectrum emission mask limits for UE category 0 are defined in TS 36.521-1 [5] clause 6.6.2.1E.5.

The E-UTRA spectrum emission mask limits for UE category M1 are defined in TS 36.521-1 [5] clause 6.6.2.1EA.5.

The E-UTRA spectrum emission mask limits for UE 1bis are defined in TS 36.521-1 [5] clause 6.6.2.1EB.5.

The E-UTRA spectrum emission mask limits for UE M2 are defined in TS 36.521-1 [5] clause 6.6.2.1EC.5.

The E-UTRA spectrum emission mask limits for UE NB1 and NB2  are defined in TS 36.521-1 [5] clause 6.6.2.1EF.5.

The E-UTRA spectrum emission mask limits for V2X are defined in TS 36.521-1 [5] clause 6.6.2.1G.1.5.

The E-UTRA spectrum emission mask limits for V2X Communication /Sidelink simultaneous with E-UTRA uplink transmissions are defined in TS 36.521-1 [5] clause 6.6.2.1G.2.5.

The E-UTRA spectrum emission mask limits for V2X Communication / Intra-band contiguous MCC operation are defined in TS 36.521-1 [5] clause 6.6.2.1G.3.5.

The E-UTRA spectrum emission mask limits for for subslot/slot TTI are defined in TS 36.521-1 [5] clause 6.6.2.1_s.5.

#### 1.2.1.2	Additional spectrum emission mask

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

The additional spectrum emission requirements for different network signalling labels are defined in TS 36.521-1 [5] clause 6.6.2.2.5.

The UL 64QAM additional spectrum emission requirements for different network signalling labels are defined in TS 36.521-1 [5] clause 6.6.2.2_1.5.

The UL 256QAM additional spectrum emission requirements for different network signalling labels are defined in TS 36.521-1 [5] clause  6.6.2.2.5.

The PUSCH frequency hopping additional spectrum emission requirements for different network signalling labels are defined in TS 36.521-1 [5] clause 6.6.2.2_3.5.

The CA additional spectrum emission requirements for different network signalling labels are defined in TS 36.521-1 [5] clause 6.6.2.2A.1.5.

The UL-MIMO additional spectrum emission requirements for different network signalling labels are defined in TS 36.521-1 [5] clause 6.6.2.2.5.

The UE category 0 additional spectrum emission requirements for different network signalling labels are defined in TS 36.521-1 [5] clause 6.6.2.2E.5.

The UE category M1 additional spectrum emission requirements for different network signalling labels are defined in TS 36.521-1 [5] clause 6.6.2.2EA.5.

The UE category 1bis additional spectrum emission requirements for different network signalling labels are defined in TS 36.521-1 [5] clause 6.6.2.2EB.5.

The UE category M2 additional spectrum emission requirements for different network signalling labels are defined in TS 36.521-1 [5] clause 6.6.2.2EC.5.

The UE category V2X additional spectrum emission requirements for different network signalling labels are defined in TS 36.521-1 [5] clause 6.6.2.2G.1.5.

The UE category V2X Communication / Sidelink simultaneous with E-UTRA uplink transmissions additional spectrum emission requirements for different network signalling labels are defined in TS 36.521-1 [5] clause 6.6.2.2G.2.5.

The UE subslot/slot TTI additional spectrum emission requirements for different network signalling labels are defined in TS 36.521-1 [5] clause 6.6.2.2_s.5.

#### 1.2.1.3	Adjacent channel leakage ratio

Adjacent channel leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency.

The E-UTRA and UTRA ACLR requirements are defined in TS 36.521-1 [5]. clause 6.6.2.3.5.1.

The HPUE ACLR requirements are defined in TS 36.521-1 [5]. clause 6.6.2.3_1.5.

The Multi-Cluster PUSCH ACLR requirements are defined in TS 36.521-1 [5]. clause 6.6.2.3_2.5.

The UL 64QAM ACLR requirements are defined in TS 36.521-1 [5]. clause 6.6.2.3_3.5.

The Multi-Cluster PUSCH with UL 64QAM ACLR requirements are defined in TS 36.521-1 [5]. clause 6.6.2.3_4.5.

The UL 256QAM ACLR requirements are defined in TS 36.521-1 [5]. clause 6.6.2.3_5.5.

The Multi-Cluster PUSCH with UL 256QAM ACLR requirements are defined in TS 36.521-1 [5]. clause 6.6.2.3_6.5.

The CA ACLR requirements are defined in TS 36.521-1 [5]. clause 6.6.2.3A.1.5.

The CA (intra-band contiguous DL CA and UL CA) for UL 64QAM ACLR requirements are defined in TS 36.521-1 [5]. clause 6.6.2.3A.1_1.5.

The CA (intra-band contiguous DL CA and UL CA) for UL 256QAM ACLR requirements are defined in TS 36.521-1 [5]. clause 6.6.2.3A.1_2.5.

The CA (intra-band contiguous DL CA and UL CA) for HPUE ACLR requirements are defined in TS 36.521-1 [5]. clause 6.6.2.3A.1_3.5.

The CA (inter-band DL CA and UL CA) ACLR requirements are defined in TS 36.521-1 [5]. clause 6.6.2.3A.2.5.

The CA (inter-band DL CA and UL CA) for UL 64QAM ACLR requirements are defined in TS 36.521-1 [5]. clause 6.6.2.3A.2_1.5

The CA (inter-band DL CA and UL CA) for UL 256QAM ACLR requirements are defined in TS 36.521-1 [5]. clause 6.6.2.3A.3_1.5.

The CA (intra-band non-contiguous DL CA and UL CA) ACLR requirements are defined in TS 36.521-1 [5]. clause 6.6.2.3A.3.5.

The CA (intra-band non-contiguous DL CA and UL CA) for UL 64QAM ACLR requirements are defined in TS 36.521-1 [5]. clause 6.6.2.3A.3_1.5.

The CA (intra-band non-contiguous DL CA and UL CA) for UL 256QAM ACLR requirements are defined in TS 36.521-1 [5]. clause 6.6.2.3A.3_2.5.

The CA (3UL CA) ACLR requirements are defined in TS 36.521-1 [5]. clause 6.6.2.3A.4.5.

The CA (4UL CA) ACLR requirements are defined in TS 36.521-1 [5]. clause 6.6.2.3A.5.5.

The UL-MIMO ACLR requirements are defined in TS 36.521-1 [5]. clause 6.6.2.3B.5.

The UE category 0 ACLR requirements are defined in TS 36.521-1 [5]. clause 6.6.2.3E.5.

The UE category M1 ACLR requirements are defined in TS 36.521-1 [5]. clause 6.6.2.3EA.5.

The UE category 1bis ACLR requirements are defined in TS 36.521-1 [5]. clause 6.6.2.3EB.5.

The UE category M2 ACLR requirements are defined in TS 36.521-1 [5]. clause 6.6.2.3EC.5.

The UE category NB1 and NB2 ACLR requirements are defined in TS 36.521-1 [5]. clause 6.6.2.3F.5.

The NB1 and NB2/Power Class 6 ACLR requirements are defined in TS 36.521-1 [5]. clause 6.6.2.3FA.5.

The V2X Communication ACLR requirements are defined in TS 36.521-1 [5]. clause 6.6.2.3G.1.5.

The V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions ACLR requirements are defined in TS 36.521-1 [5]. clause 6.6.2.3G.2.5.

The V2X Communication / Intra-band contiguous multi-carrier operation ACLR requirements are defined in TS 36.521-1 [5]. clause 6.6.2.3G.3.5.

The subslot/slot TTI ACLR requirements are defined in TS 36.521-1 [5]. clause 6.6.2.3_s.5.

### 1.2.2	Transmitter spurious emissions

Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products, but exclude out of band emissions unless otherwise stated. The spurious emission limits are specified in terms of general requirements in line with SM.329 [9] and NR operating band requirement to address UE co-existence.

To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

#### 1.2.2.1	Spurious emissions requirements

The general E-UTRA spurious emission limits are defined in TS 36.521-1 [5] clause 6.6.3.1.5.

The Multi-Cluster PUSCH spurious emission limits are defined in TS 36.521-1 [5] clause 6.6.3.1_1.5.

The CA (intra-band contiguous DL CA and UL CA) spurious emission limits are defined in TS 36.521-1 [5] clause 6.6.3.1A.1.5.

The CA (inter-band DL CA and UL CA) spurious emission limits are defined in TS 36.521-1 [5] clause 6.6.3.1A.2.5.

The CA (intra-band non-contiguous DL CA and UL CA) spurious emission limits are defined in TS 36.521-1 [5] clause 6.6.3.1A.3.5.

The CA (3UL CA) spurious emission limits are defined in TS 36.521-1 [5] clause 6.6.3.1A.4.1.5.

The CA (inter-band 3DL CA and 3UL CA) spurious emission limits are defined in TS 36.521-1 [5] clause 6.6.3.1A.4.2.5.

The CA (3UL CA) (intra-band non-contiguous 3DL CA and 3UL CA) spurious emission limits are defined in TS 36.521-1 [5] clause 6.6.3.1A.4.3.5.

The CA (4UL CA) spurious emission limits are defined in TS 36.521-1 [5] clause 6.6.3.1A.5.5.

#### 1.2.2.2	Spurious emissions for UE co-existence

The co-existence requirements for the UE are release specific.

The general co-existence requirements up to Rel-16 can found in TS 36.521-1 [5]. clause 6.6.3.2.5.

The co-existence requirements for CA (intra-band contiguous DL CA and UL CA) can found in TS 36.521-1 [5]. clause 6.6.3.2A.1.5.

The co-existence requirements for CA (inter-band DL CA and UL CA) can found in TS 36.521-1 [5]. clause 6.6.3.2A.2.5.

The co-existence requirements for CA (intra-band non-contiguous DL CA and UL CA) can found in TS 36.521-1 [5]. clause 6.6.3.2A.3.5.

The co-existence requirements for CA (3UL CA) can found in TS 36.521-1 [5]. clause 6.6.3.2A.4.5.

The co-existence requirements for CA (4UL CA) can found in TS 36.521-1 [5]. clause 6.6.3.2A.5.5.

#### 1.2.2.3	Additional spurious emissions

These requirements are specified in terms of an additional spectrum emission requirement. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

The additional spurious emission requirements for network signalling label "NS_04" are defined in TS 36.521-1 [5] Table 6.6.3.3.5.1.

The additional spurious emission requirements for network signalling label "NS_05" are defined in TS 36.521-1 [5] clause 6.6.3.3.5.2.

The additional spurious emission requirements for network signalling label "NS_07" are defined in TS 36.521-1 [5] Table 6.6.3.3.5.3-1.

The additional spurious emission requirements for network signalling label "NS_08" are defined in TS 36.521-1 [5] Table 6.6.3.3.5.4-1.

The additional spurious emission requirements for network signalling label "NS_09" are defined in TS 36.521-1 [5] Table 6.6.3.3.5.5-1.

The additional spurious emission requirements for network signalling label "NS_11" are defined in TS 36.521-1 [5] Table 6.6.3.3.5.6-1.

The additional spurious emission requirements for network signalling label "NS_12" are defined in TS 36.521-1 [5] Table 6.6.3.3.5.7-1.

The additional spurious emission requirements for network signalling label "NS_13" are defined in TS 36.521-1 [5] Table 6.6.3.3.5.8-1.

The additional spurious emission requirements for network signalling label "NS_14" are defined in TS 36.521-1 [5] Table 6.6.3.3.5.9-1.

The additional spurious emission requirements for network signalling label "NS_15" are defined in TS 36.521-1 [5] Table 6.6.3.3.5.10-1.

The additional spurious emission requirements for network signalling label "NS_16" are defined in TS 36.521-1 [5] Table 6.6.3.3.5.11-1.

The additional spurious emission requirements for network signalling label "NS_17" are defined in TS 36.521-1 [5] Table 6.6.3.3.5.12-1.

The additional spurious emission requirements for network signalling label "NS_18" are defined in TS 36.521-1 [5] Table 6.6.3.3.5.13-1.

The additional spurious emission requirements for network signalling label "NS_19" are defined in TS 36.521-1 [5] Table 6.6.3.3.5.14-1.

The additional spurious emission requirements for network signalling label "NS_20" are defined in TS 36.521-1 [5] Table 6.6.3.3.5.15-1.

The additional spurious emission requirements for network signalling label "NS_21" are defined in TS 36.521-1 [5] Table 6.6.3.3.5.16-1.

The additional spurious emission requirements for network signalling label "NS_22" are defined in TS 36.521-1 [5] Table 6.6.3.3.5.17-1.

The additional spurious emission requirements for network signalling label "NS_23" are defined in TS 36.521-1 [5] Table 6.6.3.3.5.18-1.

The additional spurious emission requirements for network signalling label "NS_24" are defined in TS 36.521-1 [5] Table 6.6.3.3.5.19-1.

The additional spurious emission requirements for network signalling label "NS_25" are defined in TS 36.521-1 [5] Table 6.6.3.3.5.20-1.

The additional spurious emission requirements for network signalling label "NS_27" are defined in TS 36.521-1 [5] Table 6.6.3.3.5.21-1.

The additional spurious emission requirements for network signalling label "NS_36" are defined in TS 36.521-1 [5] Table 6.6.3.3.5.22-1.

The additional spurious emission requirements for network signalling label "NS_38" are defined in TS 36.521-1 [5] Table 6.6.3.3.5.23-1.

The additional spurious emission requirements for network signalling label "NS_39" are defined in TS 36.521-1 [5] Table 6.6.3.3.5.24-1.

The additional spurious emission requirements for network signalling label "NS_45" are defined in TS 36.521-1 [5] clause 6.6.3.3.5.25

The additional spurious emission requirements for network signalling label "NS_44" are defined in TS 36.521-1 [5] Table 6.6.3.3.5.26-1.

The additional spurious emission requirements for network signalling label "NS_56" are defined in TS 36.521-1 [5] Table 6.6.3.3.5.27-1.

#### 1.2.2.4	Additional spurious emissions for UL 64QAM

The additional spurious emission requirements for UL 64QAM are defined in TS 36.521-1 [5] clause 6.6.3.3_1.5.

#### 1.2.2.5	Additional spurious emissions for UL 256QAM

The additional spurious emission requirements for UL 256QAM are defined in TS 36.521-1 [5] clause 6.6.3.3_2.5.

#### 1.2.2.6	Additional spurious emissions for CA

The additional spurious emission requirements for CA (intra-band contiguous DL CA and UL CA) are defined in TS 36.521-1 [5] clause 6.6.3.3A.1.5.

The additional spurious emission requirements for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM are defined in TS 36.521-1 [5] clause 6.2.4A.1_1.5.

The additional spurious emission requirements for CA (intra-band contiguous DL CA and UL CA) for UL 256QAM are defined in TS 36.521-1 [5] clause 6.2.4A.1_2.5.

The additional spurious emission requirements for CA (inter-band DL CA and UL CA) are defined in TS 36.521-1 [5] clause 6.6.3.3A.2.5.

The additional spurious emission requirements for CA (inter-band DL CA and UL CA) for UL 64QAM are defined in TS 36.521-1 [5] clause 6.6.3.3A.2_1.5.

The additional spurious emission requirements for CA (inter-band DL CA and UL CA) for UL 256QAM are defined in TS 36.521-1 [5] clause 6.6.3.3A.2_2.5.

The additional spurious emission requirements for CA (3UL CA) are defined in TS 36.521-1 [5] clause 6.6.3.3A.4.5.

The additional spurious emission requirements for CA (4UL CA) are defined in TS 36.521-1 [5] clause 6.6.3.3A.5.5.

### 1.2.3	Transmitter spurious emissions for UL-MIMO

The E-UTRA spurious emissions for UL-MIMO  are defined in TS 36.521-1 [5] clause 6.6.3B.1.5.

The co-existence requirements for UL-MIMO are defined in TS 36.521-1 [5]. clause 6.6.3B.2.5.

The additional spurious emissions requirements for UL-MIMO are defined in TS 36.521-1 [5]. clause 6.6.3B.3.5.

### 1.2.4	Transmitter spurious emissions for UE category 0

The E-UTRA spurious emissions for UE category 0 are defined in TS 36.521-1 [5] clause 6.6.3E.1.5.

The co-existence requirements for UE category 0 are defined in TS 36.521-1 [5]. clause 6.6.3E.2.5.

The additional spurious emissions requirements for UE category 0 are defined in TS 36.521-1 [5]. clause 6.6.3E.3.5.

### 1.2.5	Transmitter spurious emissions for UE category M1

The E-UTRA spurious emissions for UE category M1 are defined in TS 36.521-1 [5] clause 6.6.3EA.1.5.

The co-existence requirements for UE category M1 are defined in TS 36.521-1 [5]. clause 6.6.3EA.2.5.

The additional spurious emissions requirements for UE category M1 are defined in TS 36.521-1 [5]. clause 6.6.3EA.3.5.

### 1.2.6	Transmitter spurious emissions for UE category 1bis

The E-UTRA spurious emissions for UE category 1bis are defined in TS 36.521-1 [5] clause 6.6.3EB.1.5.

The co-existence requirements for UE category 1bis are defined in TS 36.521-1 [5]. clause 6.6.3EB.2.5.

The additional spurious emissions requirements for UE category 1bis are defined in TS 36.521-1 [5]. clause 6.6.3EB.3.5.

### 1.2.7	Transmitter spurious emissions for UE category M2

The E-UTRA spurious emissions for UE category M2 are defined in TS 36.521-1 [5] clause 6.6.3EC.1.5.

The co-existence requirements for UE category M2 are defined in TS 36.521-1 [5]. clause 6.6.3EC.2.5.

The additional spurious emissions requirements for UE category M2 are defined in TS 36.521-1 [5]. clause 6.6.3EC.3.5.

### 1.2.8	Transmitter spurious emissions for UE category NB1 and NB2

The E-UTRA spurious emissions for UE category NB1 and NB2 are defined in TS 36.521-1 [5] clause 6.6.3EF.1.5.

The co-existence requirements for UE category NB1 and NB2 are defined in TS 36.521-1 [5]. clause 6.6.3EF.2.5.

### 1.2.9	Transmitter spurious emissions for V2X

The spurious emissions for V2X Communication / Non-concurrent with E-UTRA uplink transmissions are defined in TS 36.521-1 [5]. clause 6.6.3G.1.5.

The spurious emission band UE co-existence for V2X Communication / Non-concurrent with E-UTRA uplink are defined in TS 36.521-1 [5]. clause 6.6.3G.1_1.5.

The spurious emission for V2X Communication / Sidelink simultaneous with E-UTRA uplink transmissions are defined in TS 36.521-1 [5]. clause 6.6.3G.2.5.

The spurious emission band UE co-existence for V2X Communication / Sidelink simultaneous with E-UTRA uplink transmissions are defined in TS 36.521-1 [5]. clause 6.6.3G.2_1.5.

The spurious emission for V2X Communication / Intra-band contiguous MCC operation are defined in TS 36.521-1 [5]. clause 6.6.3G.3.5.

The spurious emission band UE co-existence for V2X Communication / Intra-band contiguous MCC operationare defined in TS 36.521-1 [5]. clause 6.6.3G.3_1.5.

## 1.3	Receiver spurious emissions

The general receiver spurious emission requirements are defined in TS 36.521-1 [5]. clause 7.7.5.

The general receiver spurious emission requirements with 4 Rx antenna ports are defined in TS 36.521-1 [5] clause 7.7_1.5.

The receiver spurious emission requirements for CA (intra-band contiguous DL CA and UL CA) are defined in TS 36.521-1 [5] clause 7.7A.1.5.

The receiver spurious emission requirements for CA (intra-band contiguous DL CA without UL CA) are defined in TS 36.521-1 [5] clause 7.7A.2.5.

The receiver spurious emission requirements for CA (inter-band DL CA without UL CA) are defined in TS 36.521-1 [5] clause 7.7A.3.5.

The receiver spurious emission requirements for CA (intra-band non-contiguous DL CA without UL CA) are defined in TS 36.521-1 [5] clause 7.7A.4.5.

The receiver spurious emission requirements for 3DL CA are defined in TS 36.521-1 [5] clause 7.7A.5.5.

The receiver spurious emission requirements for 4DL CA are defined in TS 36.521-1 [5] clause 7.7A.7.5.

The receiver spurious emission requirements for 5DL CA are defined in TS 36.521-1 [5] clause 7.7A.8.5.

The receiver spurious emission requirements for 6DL CA are defined in TS 36.521-1 [5] clause 7.7A.9.5.

The receiver spurious emission requirements for 7DL CA are defined in TS 36.521-1 [5] clause 7.7A10.5.

The receiver spurious emission requirements for UL-MIMO are defined in TS 36.521-1 [5] clause 7.7B.5.

The receiver spurious emission requirements for ProSe Direct Discovery are defined in TS 36.521-1 [5] clause 7.7D.1.5.

The receiver spurious emission requirements for ProSe Direct Communication are defined in TS 36.521-1 [5] clause 7.7D.2.5.

The receiver spurious emission requirements for UE category 0 are defined in TS 36.521-1 [5] clause 7.7E.5.

The receiver spurious emission requirements for UE category M1 are defined in TS 36.521-1 [5] clause 7.7EA.5.

The receiver spurious emission requirements for UE category 1bis are defined in TS 36.521-1 [5] clause 7.7EB.5.

The receiver spurious emission requirements for UE category M2 are defined in TS 36.521-1 [5] clause 7.7EC.5.

The receiver spurious emission requirements for NB1 and NB2 are defined in TS 36.521-1 [5] clause 7.7F.5.

The receiver spurious emission requirements for V2X Communication / Non-concurrent with E-UTRA uplink transmissions are defined in TS 36.521-1 [5] clause 7.7G.1.5.

The receiver spurious emission requirements for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions are defined in TS 36.521-1 [5] clause 7.7G.2.5.

The receiver spurious emission requirements for V2X Communication / Intra-band contiguous multi-carrier operation are defined in TS 36.521-1 [5] clause 7.7G.3.5.

# Generic unwanted emission characteristics for NR range 1 standalone

## 2.1	Operating bands and channel arrangements

Requirements throughout the RF specifications are in many cases defined separately for different frequency ranges (FR). The frequency ranges in which NR can operate according to this version of the specification are identified as described in Table 2.1-1.

TABLE  2.1-1

Definition of frequency ranges

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1 | 410 MHz – 7125 MHz |
| FR2 | 24250 MHz – 52600 MHz |

### 2.1.1	Operating bands

NR is designed to operate in the FR1 operating bands defined in Table 2.1.1-1.

TABLE  2.1.1-1

NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD |
| n2 | 1850 MHz – 1910 MHz | 1930 MHz – 1990 MHz | FDD |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | FDD |
| n5 | 824 MHz – 849 MHz | 869 MHz – 894 MHz | FDD |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD |
| n8 | 880 MHz – 915 MHz | 925 MHz – 960 MHz | FDD |
| n12 | 699 MHz – 716 MHz | 729 MHz – 746 MHz | FDD |
| n14 | 788 MHz – 798 MHz | 758 MHz – 768 MHz | FDD |
| n20 | 832 MHz – 862 MHz | 791 MHz – 821 MHz | FDD |
| n2416 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| n25 | 1850 MHz – 1915 MHz | 1930 MHz – 1995 MHz | FDD |
| n26 | 814 MHz – 849 MHz | 859 MHz – 894 MHz | FDD |
| n28 | 703 MHz – 748 MHz | 758 MHz – 803 MHz | FDD |
| n29 | N/A | 717 MHz – 728 MHz | SDL |
| n303 | 2305 Mhz – 2315 MHz | 2350 MHz – 2360 MHz | FDD |
| n34 | 2010 MHz – 2025 MHz | 2010 MHz – 2025 MHz | TDD |
| n3810 | 2570 MHz – 2620 MHz | 2570 MHz – 2620 MHz | TDD |
| n39 | 1880 MHz – 1920 MHz | 1880 MHz – 1920 MHz | TDD |
| n40 | 2300 MHz – 2400 MHz | 2300 MHz – 2400 MHz | TDD |
| n41 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD |
| n46 | 5150 MHz – 5925 MHz | 5150 MHz – 5925 MHz | TDD13 |
| n4711 | 5855 MHz – 5925 MHz | 5855 MHz – 5925 MHz | TDD |
| n48 | 3550 MHz – 3700 MHz | 3550 MHz – 3700 MHz | TDD |
| n51 | 1427 MHz – 1432 MHz | 1427 MHz – 1432 MHz | TDD |
| n53 | 2483.5 MHz – 2495 MHz | 2483.5 MHz – 2495 MHz | TDD |
| n65 | 1920 MHz – 2010 MHz | 2110 MHz – 2200 MHz | FDD4 |
| n66 | 1710 MHz – 1780 MHz | 2110 MHz – 2200 MHz | FDD |
| n70 | 1695 MHz – 1710 MHz | 1995 MHz – 2020 MHz | FDD |
| n71 | 663 MHz – 698 MHz | 617 MHz – 652 MHz | FDD |
| n74 | 1427 MHz – 1470 MHz | 1475 MHz – 1518 MHz | FDD |
| n75 | N/A | 1432 MHz – 1517 MHz | SDL |
| n76 | N/A | 1427 MHz – 1432 MHz | SDL |
| n7712 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |
| n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD |
| n79 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |
| n81 | 880 MHz – 915 MHz | N/A | SUL |
| n82 | 832 MHz – 862 MHz | N/A | SUL |
| n84 | 1920 MHz – 1980 MHz | N/A | SUL |
| n958 | 2010 MHz – 2025 MHz | N/A | SUL |
| n9614 | 5925 MHz – 7125 MHz | 5925 MHz – 7125 MHz | TDD13 |
| n9715 | 2300 MHz – 2400 MHz | N/A | SUL |
| n9815 | 1880 MHz – 1920 MHz | N/A | SUL |
| n9916 | 1626.5 MHz – 1660.5 MHz | N/A | SUL |
| n100 | 874.4 MHz – 880 MHz | 919.4 MHz – 925 MHz | FDD |
| n101 | 1900 MHz – 1910 MHz | 1900 MHz – 1910 MHz | TDD |
| n10214 | 5925 MHz – 6425 MHz | 5925 MHz – 6425 MHz | TDD13 |
| n10417,18 | 6425 MHz – 7125 MHz | 6425 MHz – 7125 MHz | TDD |

NOTE 1 – UE that complies with the NR Band n50 minimum requirements in this specification. Shall also comply with the NR Band n51 minimum requirements.

NOTE 2 – UE that complies with the NR Band n75 minimum requirements in this specification. Shall also comply with the NR Band n76 minimum requirements.

NOTE 3 – Uplink transmission is not allowed at this band for UE with external vehicle-mounted antennas.

NOTE 4 – A UE that complies with the NR Band n65 minimum requirements in this specification shall also comply with the NR Band n1 minimum requirements.

NOTE 5 – FFS.

NOTE 6 – A UE that supports NR Band n66 shall receive in the entire DL operating band.

NOTE 7 – A UE that supports NR Band n66 and CA operation in any CA band shall also comply with the minimum requirements specified for the DL CA configurations CA_n66B and CA_n66(2A) in the current version of the specification.

NOTE 8 – This band is applicable in China only.

NOTE 9 – FFS.

NOTE 10 – When this band is used for V2X SL service, the band is exclusively used for NR V2X in particular regions.

NOTE 11 – This band is unlicensed band used for V2X service. There is no expected network deployment in this band.

NOTE 12 – In the USA this band is restricted to 3450 – 3550 MHz and 3700 – 3980 MHz.

NOTE 13 – This band is restricted to operation with shared spectrum channel access as defined in TS 37.213.

NOTE 14 – This band is applicable in the USA only subject to FCC Report and Order FCC 20-51

NOTE 15 – The requirements for this band are applicable only where no other NR or E-UTRA TDD operating band(s) are used within the frequency range of this band in the same geographical area. For scenarios where other NR or E-UTRA TDD operating band(s) are used within the frequency range of this band in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications.

NOTE 16 – DL operation in this band is restricted to 1526 – 1536 MHz and UL operation is restricted to 1627.5 – 1637.5 MHz and 1646.5 – 1656.5 MHz.

NOTE 17 – For this band, CORESET#0 values from Table 13-5 or Table 13-6 in TS 38.213 are applied regardless of the minimum channel bandwidth.

NOTE 18 – [This band is applicable only to RCC countries in accordance with RCC Recommendation 1/21]

The UE operating bands for CA are defined in TS 38.101-1 [1] clause 5.2A.

The UE operating bands for DC are defined in TS 38.101-1 [1] clause 5.2B.

The UE operating band combination for SUL are defined in TS 38.101-1 [1] clause 5.2C.

The UE operating band combination for UL MIMO are defined in TS 38.101-1 [1] clause 5.2D.

The UE operating band combination for V2X are defined in TS 38.101-1 [1] clause 5.2E.

### 2.1.2	UE Channel bandwidth

The UE channel bandwidth for different NR bands are defined in TS 38.101-1 [1] clause 5.3.

The UE channel bandwidth for CA are defined in TS 38.101-1 [1] clause 5.3A.

The UE channel bandwidth for V2X are defined in TS 38.101-1 [1] clause 5.3E.

### 2.1.3	Channel arrangement

The UE channel arrangement i.e. channel spacing, channel raster, synchronization raster and TX–RX frequency separation are defined in TS 38.101-1 [1] clause 5.4.

The UE channel arrangement for CA are defined in TS 38.101-1 [1] clause 5.4A.

The UE channel arrangement for V2X are defined in TS 38.101-1 [1] clause 5.4.

### 2.1.4	Configurations

The UE configurations for CA operating bands are defined in TS 38.101-1 [1] clause 5.5A.

The UE configurations for DC operating bands are defined in TS 38.101-1 [1] clause 5.5B.

The UE configurations for SUL operating bands are defined in TS 38.101-1 [1] clause 5.5C.

### 2.1.5	Definitions, symbols and abbreviations

# 2.1.5.1	Definitions

Carrier aggregation: Aggregation of two or more component carriers in order to support wider transmission bandwidths.

Channel bandwidth: The RF bandwidth supporting a single E-UTRA RF carrier with the transmission bandwidth configured in the uplink or downlink of a cell. The channel bandwidth is measured in MHz and is used as a reference for transmitter and receiver RF requirements.

#### 2.1.5.2	Symbols

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

#### 2.1.5.3	Abbreviations

ACLR	Adjacent Channel Leakage Ratio

CA	Carrier Aggregation

DC	Dual Connectivity

FDD	Frequency division duplex

FR	Frequency Range

NR	New Radio

NS	Network Signalling

SUL	Supplementary uplink

TDD	Time division duplex

UE	User equipment

V2X	Vehicle to Everything

# 2.2	RF spectrum emissions

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique NS label. A list of NS labels and their corresponding NR frequency bands are defined in TS 38.521-1 [1] Table 6.2.3.3.1-1.

### 2.2.1	Out of band emission

#### 2.2.1.1	Spectrum emission mask

The general NR spectrum emission mask limits are defined in TS 38.521-1 [1] Table 6.5.2.2.5-1 with test tolerances in Table 6.5.2.2.5-2.

#### 2.2.1.2	Additional spectrum emission mask

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

The additional spectrum emission requirements for network signalling label "NS_35" are defined in TS 38.521-1 [1] Table 6.5.2.3.5.1-1 with test tolerances in Table 6.5.2.3.5-1.

The additional spectrum emission requirements for network signalling label "NS_04" are defined in TS 38.521-1 [1] Table 6.5.2.3.5.2-1 with test tolerances in Table 6.5.2.3.5-1.

The additional spectrum emission requirements for network signalling label "NS_03" and “NS_21” are defined in TS 38.521-1 [1] Table 6.5.2.3.5.3-1 with test tolerances in Table 6.5.2.3.5-1.

The additional spectrum emission requirements for network signalling label "NS_06" are defined in TS 38.521-1 [1] Table 6.5.2.3.5.4-1 with test tolerances in Table 6.5.2.3.5-1.

#### 2.2.1.3	Adjacent channel leakage ratio

Adjacent channel leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency.

The NR ACLR requirements are defined in TS 38.521-1 [1] clause 6.5.2.4.1.5.

The UTRA ACLR requirements are defined in TS 38.521-1 [1] clause 6.5.2.4.2.5.

### 2.2.2	Transmitter spurious emissions

Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products, but exclude out of band emissions unless otherwise stated. The spurious emission limits are specified in terms of general requirements in line with SM.329 and NR operating band requirement to address UE co-existence.

To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

#### 2.2.2.1	Spurious emissions requirements

The general NR spurious emission limits are defined in TS 38.521-1 [1] Table 6.5.3.1.5-1.

#### 2.2.2.2	Spurious emissions for UE co-existence

The co-existence requirements for the UE are release specific. The Rel-15 and Rel-16 requirements can found in TS 38.521-1 [1] Table 6.5.3.2.5-1.

#### 2.2.2.3	Additional spurious emissions

These requirements are specified in terms of an additional spectrum emission requirement. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

The additional spurious emission requirements for network signalling label "NS_04" are defined in TS 38.521-1 [1] Table 6.5.3.3.5.1-1.

The additional spurious emission requirements for network signalling label "NS_17" are defined in TS 38.521-1 [1] Table 6.5.3.3.5.2-1.

The additional spurious emission requirements for network signalling label "NS_18" are defined in TS 38.521-1 [1] Table 6.5.3.3.5.3-1.

The additional spurious emission requirements for network signalling label "NS_05 are defined in TS 38.521-1 [1] Table 6.5.3.3.5.4-1.

The additional spurious emission requirements for network signalling label "NS_43 are defined in TS 38.521-1 [1] Table 6.5.3.3.5.5-1.

The additional spurious emission requirements for network signalling label "NS_37 are defined in TS 38.521-1 [1] Table 6.5.3.3.5.6-1.

The additional spurious emission requirements for network signalling label "NS_21 are defined in TS 38.521-1 [1] Table 6.5.3.3.5.12-1.

The additional spurious emission requirements for network signalling label "NS_24 are defined in TS 38.521-1 [1] Table 6.5.3.3.5.13-1.

The additional spurious emission requirements for network signalling label "NS_27 are defined in TS 38.521-1 [1] Table 6.5.3.3.5.14-1.

The additional spurious emission requirements for network signalling label "NS_47 are defined in TS 38.521-1 [1] Table 6.5.3.3.5.15-1.

The additional spurious emission requirements for network signalling label "NS_50 are defined in TS 38.521-1 [1] Table 6.5.3.3.5.16-1.

The additional spurious emission requirements for network signalling label "NS_12are defined in TS 38.521-1 [1] Table 6.5.3.3.5.17-1.

The additional spurious emission requirements for network signalling label "NS_13are defined in TS 38.521-1 [1] Table 6.5.3.3.5.18-1.

The additional spurious emission requirements for network signalling label "NS_14 are defined in TS 38.521-1 [1] Table 6.5.3.3.5.19-1.

The additional spurious emission requirements for network signalling label "NS_15 are defined in TS 38.521-1 [1] Table 6.5.3.3.5.20-1.

The additional spurious emission requirements for network signalling label "NS_45 are defined in TS 38.521-1 [1] Table 6.5.3.3.5.21-1.

The additional spurious emission requirements for network signalling label "NS_48 are defined in TS 38.521-1 [1] Table 6.5.3.3.5.22-1.

The additional spurious emission requirements for network signalling label "NS_49 are defined in TS 38.521-1 [1] Table 6.5.3.3.5.23-1.

The additional spurious emission requirements for network signalling label "NS_44 are defined in TS 38.521-1 [1] Table 6.5.3.3.5.24-1.

The additional spurious emission requirements for network signalling label "NS_46 are defined in TS 38.521-1 [1] Table 6.5.3.3.5.25-1.

The additional spurious emission requirements for network signalling label "NS_56 are defined in TS 38.521-1 [1] Table 6.5.3.3.5.27-1.

### 2.2.3	Receiver spurious emissions

The general receiver spurious emission requirements are defined in TS 38.521-1 [1] Table 7.9.5-1.

The receiver spurious emission requirements for CA are defined in TS 38.521-1 [1] Table 7.9.5-1.

## 2.2A	Output RF spectrum emissions for CA

### 2.2A.1	Out of band emission

For inter-band carrier aggregation with one uplink carrier assigned to one NR band, the output RF spectrum emissions requirements in clause 2.2 apply.

#### 2.2A.1.1	Spectrum emission mask

The general NR spectrum emission mask limits for intra-band CA are defined in TS 38.521-1 [1] clause 6.5A.2.2.1.5.

#### 2.2A.1.2	Void

#### 2.2A.1.3	Adjacent channel leakage ratio

The general NR ACLR limits for CA are defined in TS 38.521-1 [1] clause 6.5A.2.4.1.1.5.

The general UTRA spectrum emission mask limits for CA are defined in TS 38.521-1 [1] clause 6.5A.2.4.2.1.5

### 2.2A.2	Transmitter spurious emissions for CA

The general spurious emissions limits for CA are defined in TS 38.521-1 [1] clause 6.5A.3.1.1.5.

The surious emissions CA limits for UE co-existence are defined in TS 38.521-1 [1] clause in clause 6.5A.3.2.1.5

### 2.2B	Output RF spectrum emissions for NR-DC

#### 2.2B.1	Spectrum emission mask for NR-DC

For inter-band NR-DC with one uplink carrier assigned per NR band, the output RF spectrum emissions for the corresponding inter-band CA configuration as specified in clause 2.2A.1.1applies.

#### 2.2B.2	Adjacent channel leakage ratio for NR-DC

For inter-band dual connectivity, the spurious emissions for the corresponding inter-band CA configuration as specified in clause 2.2A.1.3 applies.

### 2.2C	Output RF spectrum emissions for SUL

## 2.2C.1 Spectrum emissions mask or SUL

The spectrum emission mask limits for SUL are defined in TS 38.521-1 [1] clause 6.5C.2.2.5

### 2.2C.2	Additional spectrum emission mask for SUL

The additional spectrum emission mask limits for SUL are defined in TS 38.521-1 [1] clause 6.5C.2.3.5.

### 2.2C.3	Adjacent channel leakage ratio for SUL

The NR ACLR limits for SUL are defined in TS 38.521-1 [1] clause 6.5C.2.4.1.5

The UTRA ACLR limits for SUL are defined in TS 38.521-1 [1] clause 6.5C.2.4.2.5.

### 2.2C.4 	Spurious emissions for SUL

#### 2.2C.4.1 	General spurious emissions for SUL

The general spurious emission limits for SUL are defined in TS 38.521-1 [1] clause 6.5C.3.1.5.

#### 2.2C.4.2 	Spurious emissions co-existence for SUL

The SUL spurious emission co-existence limits are defined in TS 38.521-1 [1] clause 6.5C.3.2.5.

#### 2.2C.4.2 	Additional spurious emissions for SUL

The additional spurious emission limits are defined in clause 2.2.2.3.

## 2.2D	 RF spectrum emissions for UL MIMO

## 2.2D.1	Out of band emission for UL MIMO

#### 2.2D.1.1	Spectrum emission mask for UL MIMO

The spectrum emission mask requirements for UL-MIMO are defined in TS 38.521-1 [1] clause 6.5D.2.2.5

#### 2.2D.1.2	Additional spectrum emission mask for UL MIMO

The additional spectrum emission requirements for UL-MIMO are defined in TS 38.521-1 [1] clause 6.5D.2.3.5.

#### 2.2D.1.3	Adjacent channel leakage ratio for UL MIMO

The NR ACLR requirements for UL-MIMO are defined in TS 38.521-1 [1] clause 6.5D.2.4.1.5.

The UTRA ACLR requirements for UL-MIMO are defined in TS 38.521-1 [1] clause 6.5D.2.4.2.5.

### 2.2D.2	Spurious emissions for UL MIMO

#### 2.2D.2.1	Spurious emissions requirements

The general spurious emission limits for UL MIMO are defined in TS 38.521-1 [1] clause 6.5D.3.1.5.

The general spurious emission limits for UL MIMO Rel-16 onward are defined in TS 38.521-1 [1] clause 6.5D.3_1.1.5.

#### 2.2D.2.2	Spurious emissions for UE co-existence

The UL MIMO co-existence limits are defined in clause 2.2.2.2.

#### 2.2D.2.3	Additional spurious emissions

The UL MIMO additional spurious emission limits are defined in clause 2.2.2.3 for diferent NS values.

## 2.2E	 RF spectrum emissions for V2X

#### 2.2E.1.1	Spectrum emission mask for V2X

The spectrum emission mask requirements for V2X are defined in TS 38.521-1 [1] clause 6.5E.2.2.1.5 and 6.5E.2.2.1D.5.

#### 2.2E.1.2	Additional spectrum emission mask for V2X

The additional spectrum emission requirements for V2X are defined in TS 38.521-1 [1] clause 6.5E.2.3.1.5 and 6.5E.2.3.1D.5

#### 2.2E.1.3	Adjacent channel leakage ratio for V2X

The ACLR requirements for V2X are defined in TS 38.521-1 [1] clause 6.5E.2.4.1.5 and 6.5E.2.4.1D.5.

### 2.2E.2	Spurious emissions for V2X

#### 2.2E.2.1	Spurious emissions requirements

The general spurious emission limits for V2X are defined in clause 2.2.2.1.

#### 2.2E.2.2	Spurious emissions for UE co-existence

The V2X co-existence limits are defined in clause 2.2.2.2.

#### 2.2E.2.3	Additional spurious emissions

The V2X additional spurious emission limits are defined in TS 38.521-1 [1] clause 6.5E.3.3.1.5 for diferent NS values.

## 2.2F	 RF spectrum emissions for shared spectrum channel access

#### 2.2F.1.1	Spectrum emission mask for shared spectrum channel access

The spectrum emission mask requirements for UL-MIMO are defined in TS 38.521-1 [1] clause 6.5F.2.2.5.

#### 2.2F.1.2	Adjacent channel leakage ratio for shared spectrum channel access

The ACLR requirements for shared spectrum channel access are defined in TS 38.521-1 [1] clause 6.5F.2.4.1.5.

### 2.2F.2	Spurious emissions for shared spectrum channel access

#### 2.2F.2.1	Spurious emissions requirements

The general spurious emission limits for shared spectrum channel access are defined in TS 38.521-1 [1] clause 6.5F.3.1.5.

#### 2.2F.2.2	Spurious emissions for UE co-existence

Spurious emissions requirements for UE coexistence are not applicable to bands restricted to stand-alone operation.

## 2.2G	 RF spectrum emissions for Tx Diversity

## 2.2G.1.1	Void

#### 2.2G.1.2	Adjacent channel leakage ratio for Tx Diversity

The ACLR requirements for Tx Diversity are defined in TS 38.521-1 [1] clause 6.5G.2.3.1.5

# 3	Generic unwanted emission characteristics for NR range 2 standalone

## 3.1	Operating bands and channel arrangements

Requirements throughout the RF specifications are in many cases defined separately for different frequency ranges (FR). The frequency ranges in which NR can operate according to this version of the specification are identified as described in Table 3.1-1.

TABLE  3.1-1

Definition of frequency ranges

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1 | 410 MHz – 7125 MHz |
| FR2 | 24250 MHz – 52600 MHz |

### 3.1.1	Operating bands

NR is designed to operate in the FR2 operating bands defined in Table 3.1.1-1.

TABLE  3.1.1-1

NR operating bands in FR2

| Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |  |  |
| n257 | 26500 MHz | – | 29500 MHz | 26500 MHz |  | – |  | 29500 MHz | TDD |
| n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – |  | 27500 MHz |  | TDD |  |
| n259 | 39500 MHz | – | 43500 MHz | 39500 MHz | – |  | 43500 MHz |  | TDD |  |
| n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – |  | 40000 MHz |  | TDD |  |
| n261 | 27500 MHz | – | 28350 MHz | 27500 MHz | – |  | 28350 MHz |  | TDD |  |

The UE operating bands for CA are defined in TS 38.101-2 [3] clause 5.2A.

The UE operating bands for DC are defined in TS 38.101-2 [3] clause 5.2B.

The UE operating band combination for UL MIMO are defined in TS 38.101-2 [3] clause 5.2D.

### 3.1.2	UE Channel bandwidth

The UE channel bandwidth for different NR bands are defined in TS 38.101-2 [3] clause 5.3.

The UE channel bandwidth for CA are defined in TS 38.101-2 [3] clause 5.3A.

### 3.1.3	Channel arrangement

The UE channel arrangement i.e. channel spacing, channel raster, synchronization raster and TX–RX frequency separation are defined in TS 38.101-2 [3] clause 5.4.

The UE channel arrangement for CA are defined in TS 38.101-2 [3] clause 5.4A.

### 3.1.4	Configurations

The UE configurations for CA operating bands are defined in TS 38.101-2 [3] clause 5.5A.

The UE configurations for UL-MIMO operating bands are defined in TS 38.101-2 [3] clause 5.5D.

### 3.1.5	Definitions, symbols and abbreviations

## 3.1.5.1	Definitions

Aggregated Channel Bandwidth: The RF bandwidth in which a UE is configured to transmit and receive multiple contiguously aggregated carriers.

Carrier aggregation: Aggregation of two or more component carriers in order to support wider transmission bandwidths.

Channel bandwidth: The RF bandwidth supporting a single E-UTRA RF carrier with the transmission bandwidth configured in the uplink or downlink of a cell. The channel bandwidth is measured in MHz and is used as a reference for transmitter and receiver RF requirements.

EIRP(Link=Link angle, Meas=Link angle): measurement of the UE such that the link angle is aligned with the measurement angle. EIRP (indicator to be measured) can be replaced by EIS, Frequency, EVM, carrier Leakage, In-band eission and OBW.

EIRP(Link=TX beam peak direction, Meas=Link angle): measurement of the EIRP of the UE such that the measurement angle is aligned with the beam peak direction within an acceptable measurement error uncertainty. EIRP (indicator to be measured) can be replaced by Frequency, EVM, carrier Leakage, In-band eission and OBW

EIRP(Link=Spherical coverage grid, Meas=Link  angle): measurement of the EIRP spherical coverage of the UE such that the EIRP link and measurement angles are aligned with the directions along the spherical coverage grid within an acceptable measurement error uncertainty. Alternatively, the spherical coverage grid can be replaced by the beam peak search grid as the results from the beam peak search can be re-used for spherical coverage.

EIS (effective isotropic sensitivity): sensitivity for an isotropic directivity device equivalent to the sensitivity of the discussed device exposed to an incoming wave from a defined AoA

EIS(Link=RX beam peak direction, Meas=Link angle): measurement of the EIS of the UE such that the measurement angle is aligned with the RX beam peak direction within an acceptable measurement error uncertainty.

NOTE 1:	The sensitivity is the minimum received power level at which specific requirement is met.

NOTE 2:	Isotropic directivity is equal in all directions (i.e. 0 dBi).

Fallback group: Group of carrier aggregation bandwidth classes for which it is mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration. It is not mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration that belong to a different fallback group.

Intra-band contiguous carrier aggregation: Contiguous carriers aggregated in the same operating band.

Intra-band non-contiguous carrier aggregation: Non-contiguous carriers aggregated in the same operating band.

Link angle: a DL-signal AoA from the view point of the UE, as described in Annex J. If the beam lock function is used to lock the UE beam(s), the link angle can become any arbitrary AoA once the beam lock has been activated.

Measurement angle: the angle of measurement of the desired metric from the view point of the UE, as described in Annex J

radiated interface boundary: operating band specific radiated requirements reference point where the radiated requirements apply

radiated requirements reference point: for the RF measurement setup, the radiated requirements reference point is located at the centre of the quiet zone. From the UE perspective the reference point is the input of the UE antenna array

RX beam peak direction: direction where the maximum total component of RSRP and thus best total component of EIS is found

Sub-block: This is one contiguous allocated block of spectrum for transmission and reception by the same UE. There may be multiple instances of sub-blocks within an RF bandwidth.

TX beam peak direction: direction where the maximum total component of  EIRP is found

TRP(Link=TX beam peak direction, Meas=TRP grid): measurement of the TRP of the UE such that the measurement angles are aligned with the directions of the TRP grid points within an acceptable measurement uncertainty while the link angle is aligned with the TX beam peak direction

#### 3.1.5.2	Symbols

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

#### 3.1.5.3	Abbreviations

ACLR	Adjacent Channel Leakage Ratio

CA	Carrier Aggregation

DC	Dual Connectivity

FDD	Frequency division duplex

FR	Frequency Range

NR	New Radio

NS	Network Signalling

SUL	Supplementary uplink

TDD	Time division duplex

UE	User equipment

EIRP	Effective Isotropic Radiated Power

EIS	Effective Isotropic Sensitivity

UL MIMO	Uplink Multiple Antenna transmission

## 3.2	RF spectrum emissions

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique NS label. A list of NS labels and their corresponding NR frequency bands are defined in TS 38.521-2 [6] Table 6.2.3.3.1-1 and Table 6.2.3.3.1-2.

### 3.2.1	Out of band emission

#### 3.2.1.1	Spectrum emission mask

The general NR spectrum emission mask limits are defined in TS 38.521-2 [6] Table 6.5.2.1.5-1 with test tolerances in Table 6.5.2.1.5-2.

#### 3.2.1.2	Additional spectrum emission mask

Additional spectrum emission requirements are currently not defined in TS 38.521-2 [6]

#### 3.2.1.3	Adjacent channel leakage ratio

Adjacent channel leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency.

The NR FR2 ACLR requirements are defined in TS 38.521-2 [6] clause 6.5.2.4.1.5.

### 3.2.2	Transmitter spurious emissions

Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products, but exclude out of band emissions unless otherwise stated. The spurious emission limits are specified in terms of general requirements in line with SM.329 and NR operating band requirement to address UE co-existence.

To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

#### 3.2.2.1	Spurious emissions requirements

The general NR spurious emission limits are defined in TS 38.521-2 [6] Table 6.5.3.1.5-1.

#### 3.2.2.2	Spurious emissions for UE co-existence

The co-existence requirements for the UE are found in TS 38.521-2 [6] Table 6.5.3.2.5-1.

#### 3.2.2.3	Additional spurious emissions

These requirements are specified in terms of an additional spectrum emission requirement. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

The additional spurious emission requirements for network signalling label "NS_202" are defined in TS 38.521-2 [6] Table 6.5.3.3.3-2.

The additional spurious emission requirements for network signalling label "NS_203" are defined in TS 38.521-2 [6] Table 6.5.3.3.3-3.

### 3.2.3	Receiver spurious emissions

The general receiver spurious emission requirements are currently not defined in TS 38.521-2 [6]

## 3.2A	Output RF spectrum emissions for CA

### 3.2A.1	Out of band emission

#### 3.2A.1.1	Spectrum emission mask

The general NR spectrum emission mask limits for intra-band contiguous CA are defined in TS 38.521-2 [6] clause 6.5A.2.1.1.5.

#### 3.2A.1.2	Void

#### 3.2A.1.3	Adjacent channel leakage ratio

The general NR ACLR limits for CA are defined in TS 38.521-2 [6] clause 6.5A.2.2.1.5-1.

### 3.2A.2	Transmitter spurious emissions for CA

The general spurious emissions limits for CA are defined in TS 38.521-2 [6] clause 6.5A.3.1.1.5.

The spurious emissions CA limits for UE co-existence are defined in TS 38.521-2 [6] clause in clause 6.5A.3.2.1.5

## 3.2D	 RF spectrum emissions for UL MIMO

### 3.2D.1	Out of band emission for UL MIMO

#### 3.2D.1.1	Spectrum emission mask for UL MIMO

The spectrum emission mask requirements for UL-MIMO are defined in TS 38.521-2 [6] clause 6.5D.2.1.5

#### 3.2D.1.2	Additional spectrum emission mask for UL MIMO

The additional spectrum emission requirements for UL-MIMO are currently not defined in TS 38.521-2 [6]

#### 3.2D.1.3	Adjacent channel leakage ratio for UL MIMO

The NR ACLR requirements for UL-MIMO are defined in TS 38.521-2 [6] clause 6.5D.3.3.

### 3.2D.2	Spurious emissions for UL MIMO

#### 3.2D.2.1	Spurious emissions requirements

The general spurious emission limits for UL MIMO are defined in TS 38.521-2 [6] clause 6.5D.3.1.5.

#### 3.2D.2.2	Spurious emissions for UE co-existence

The spurious emission for UE co-existence limits for UL MIMO are defined in TS 38.521-2 [6] clause 6.5D.3.2.

#### 3.2D.2.3	Additional spurious emissions

The spurious emission for UE co-existence limits for UL MIMO are defined in TS 38.521-2 [6] clause 6.5D.3.3.

# 4 	Generic unwanted emission characteristics for NR range 1 and range 2 interworking operation with other radios

## 4.1	Operating bands and channel arrangements

### 4.1.1	Operating bands

The UE operating bands for inter-band NR CA between FR1 and FR2 arrangements are defined in TS 38.521-3 [8] Table 5.2A.1-1.

The UE operating bands for DC arrangements are specified in TS 38.521-3 V16.7.0, subclause 5.5B

The UE operating bands for NR V2X operation concurrent with E-UTRA uplink/downlink or sidelink arrangements are defined in TS 38.521-3 V16.7.0, subclause 5.2E

### 4.1.2	UE Channel bandwidth

The UE channel bandwidth for inter-band NR CA between FR1 and FR2 arrangements are defined in TS 38.521-3 [8] clause 5.3A.

The UE channel bandwidth for EN-DC are defined in TS 38.521-3 [8] clause 5.3B.

The UE channel bandwidth for NR V2X operation concurrent with E-UTRA uplink/downlink or sidelink arrangements are defined in TS 38.521-3 [8] clause 5.3E.

### 4.1.3	Channel arrangement

The UE channel arrangement for inter-band NR CA between FR1 and FR2 arrangements are defined in TS 38.521-3 [8] clause 5.4A.

The UE channel arrangement for DC arrangements are defined in TS 38.521-3 [8] clause 5.4B.

### 4.1.4	Configurations

The supported inter-band CA configurations between FR1 and FR2 are specified in TS 38.521-3 [8] Table 5.5A.1-1.

The supported intra-band contiguous EN-DC configurations are specified in TS 38.521-3 [8] Table 5.5B.2-1.

The supported intra-band non-contiguous EN-DC configurations are specified in TS 38.521-3 [8] Table 5.5B.3-1.

The supported inter-band EN-DC configurations within FR1 are specified in TS 38.521-3 [8] Table 5.5B.4.1-1, Table 5.5B.4.2-1, Table 5.5B.4.3-1, Table 5.5B.4.4-1 and Table 5.5B.4.5-1.

The supported inter-band NE-DC configurations within FR1 are specified in TS 38.521-3 [8] Table 5.5B.4a.1-1.

The supported inter-band EN-DC configurations including FR2 are specified in TS 38.521-3 [8] Table 5.5B.5.1-1, Table 5.5B.5.2-1, Table 5.5B.5.3-1 and Table 5.5B.5.4-1.

The supported inter-band EN-DC configurations including FR1 and FR2 are specified in TS 38.521-3 [8] Table 5.5B.6.2-1, Table 5.5B.6.3-1, Table 5.5B.6.4-1 and Table 5.5B.6.5-1.

The supported inter-band NR-DC configurations between FR1 and FR2 are specified in TS 38.521-3 [8] Table 5.5B.7.1-1.

The supported V2X configurations between E-UTRA and NR are specified in TS 38.521-3 [8] Table 5.5E.2-1, Table 5.5E.3-1 and Table 5.5E.4.1-1.

### 4.1.5	Definitions, symbols and abbreviations

#### 4.1.5.1	Definitions

Con-current operation: The simultaneous transmission and reception of sidelink and Uu interfaces while operation is agnostic of the service used on each interface.

#### 4.1.5.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔRIB,c	Allowed reference sensitivity relaxation due to support for CA or DC operation, for serving cell c

ΔTIB,c	Allowed maximum configured output power relaxation due to support for CA or DC operation, for serving cell c

BWE-UTRA_Channel	Channel bandwidth of E-UTRA carrier

BWE-UTRA_Channel_CA	Channel bandwidth of E-UTRA sub-block which is composed of intra-band contiguous CA E-UTRA carriers

BWNR_Channel	Channel bandwidth of NR carrier

BWNR_Channel_CA	Channel bandwidth of NR sub-block which is composed of intra-band contiguous CA NR carriers

Ceil(x)	Rounding upwards; ceil(x) is the smallest integer such that ceil(x) ≥ x

EN-DCACLR	The ratio of the filtered mean power centred on the aggregated sub-block bandwidth ENBW to the filtered mean power centred on an adjacent bandwidth of the same size ENBW

E-UTRAACLR	E-UTRA ACLR

FC	RF reference frequency for the carrier centre on the channel raster

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

FOOB	The boundary between the NR out of band emission and spurious emission domains

LCRB	Transmission bandwidth which represents the length of a contiguous resource block allocation expressed in units of resources blocks

Max()	The largest of given numbers

Min()	The smallest of given numbers

NRACLR	NR ACLR

NRB	Transmission bandwidth configuration, expressed in units of resource blocks

NRB_agg	The number of the aggregated RBs within the fully allocated aggregated channel bandwidth
PCMAX	The configured maximum UE output power

RBstart	Indicates the lowest RB index of transmitted resource blocks

Wgap	The sub-block gap between the two sub-blocks

#### 4.1.5.3	Abbreviations

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

A-MPR	Additional Maximum Power Reduction

BCS	Bandwidth Combination Set

CA	Carrier Aggregation

CC	Component Carrier

DC	Dual Connectivity

EN-DC	E-UTRA/NR DC

EIRP	Equivalent Isotropically Radiated Power

EVM	Error Vector Magnitude

FR	Frequency Range

ENBW	The aggregated bandwidth of an E-UTRA sub-block and an adjacent NR sub-block

ITS	Intelligent Transportation System

ITU-R	Radio communication Sector of the International Telecommunication Union

MBW	Measurement bandwidth defined for the protected band

MPR	Allowed maximum power reduction

MSD	Maximum Sensitivity Degradation

MCG	Master Cell Group

NR	New Radio

NS	Network Signalling

NSA	Non-Standalone, a mode of operation where operation of a radio is assisted with another radio

OOB	Out-of-band

OOBE	Out-of-band emission

OTA	Over The Air

PRB	Physical Resource Block

PSCCH	Physical Sidelink Control CHannel

PSSCH	Physical Sidelink Shared CHannel

RE	Resource Element

REFSENS	Reference Sensitivity

RF	Radio Frequency

Rx	Receiver

SCG	Secondary Cell Group

SCS	Subcarrier spacing

SEM	Spectrum Emission Mask

SL	Sidelink

SUL	Supplementary uplink

TDM	Time Division Multiplex

Tx	Transmitter

UE	User Equipment

UL	Uplink

UL MIMO	Up Link Multiple Antenna transmission

ULSUP	Uplink sharing from UE perspective

V2X	Vehicle to Everything

### 4.1.6	Applicability of minimum requirements

a)	In TS 38.101-3 [4] the Minimum Requirements are specified as general requirements and additional requirements. Where the Requirement is specified as a general requirement, the requirement is mandated to be met in all scenarios

b)	For specific scenarios for which an additional requirement is specified, in addition to meeting the general requirement, the UE is mandated to meet the additional requirements.

c)	The spurious emissions power requirements are for the long-term average of the power. For the purpose of reducing measurement uncertainty, it is acceptable to average the measured power over a period of time sufficient to reduce the uncertainty due to the statistical nature of the signal.

d)	Terminal that supports EN-DC configuration shall meet E-UTRA requirements as specified in TS 36.101 [1] and NR requirements as in TS 38.101-1 [2] and TS 38.101-2 [3] unless otherwise specified in TS 38.101-3 [4].

e)	All the requirements for intra-band contiguous and non-contiguous EN-DC apply under the assumption of the same uplink-downlink and special subframe configurations in the E-UTRA and slot format indicated by UL-DL-configurationCommon and UL-DL-configurationDedicated in the NR for the EN-DC.

f)	For EN-DC combinations with CA configurations for E-UTRA and/or NR, all the requirements for E-UTRA and/or NR all the requirements for E-UTRA and/or NR intra-band contiguous and non-contiguous CA apply under the assumption of the same slot format indicated by UL-DL-configurationCommon and UL-DL-configurationDedicated in the PSCell and SCells for NR and the same uplink-downlink and special subframe configurations in PCell and SCells for E-UTRA.

A terminal which supports an EN-DC configuration shall support:

-	If any subsets of the EN-DC configuration do not specify its own bandwidth combination sets in TS 38.521-3 [8] clause 5.3B, then the terminal shall support the same E-UTRA bandwidth combination sets it signals the support for in E-UTRA CA configuration part of E-UTRA – NR DC and shall support the same NR bandwidth combination sets it signals the support for in NR CA configuration part of E-UTRA – NR DC.

-	Else if one of the subsets of the EN-DC configuration specify its own bandwidth combination sets in TS 38.521-3 [8] clause 5.3B, then the terminal shall support a product set of channel bandwidth for each band specified by E-UTRA bandwidth combination sets, NR bandwidth combination sets, and EN-DC bandwidth combination sets it signals the support.

A terminal which supports an inter-band EN-DC configuration with a certain UL configuration shall support the all lower order DL configurations of the lower order EN-DC combinations, which have this certain UL configuration and the fallbacks of this UL configuration.

A terminal which supports CA or DC configurations, which include FR2 intra-band CA combinations with multiple subblocks, where at least one of the subblocks consists of a contiguous CA combination, is not required to support all possible fallback combinations but can directly fall back to a single FR2 carrier. Deactivating carriers within the CA or DC combination is still possible.

Terminal that supports inter-band NR-DC between FR1 and FR2 configuration shall meet the requirements for corresponding CA configuration (suffix A), unless otherwise specified.

### 4.1.7	Applicability and test coverage rules

#### 4.1.7.1	General

(1) The applicability and test coverage rules for Non-Standalone (NSA) only capable devices shall include the following:

a) For each NR band in a device; test all the EN-DC exception test requirements as per test procedures in TS 38.521-3 [8].

b) Test all the EN-DC FR2 non-exception test requirements in this specification with test procedures which refer appropriately back to clause 3 for each NR band. Test only one EN-DC combination per FR2 band for each EN-DC configuration as defined in clause 4.1.4 using LTE anchor agnostic approach.

c) Test all the EN-DC FR1 non-exception test requirements in this specification with test procedures which refer appropriately back to clause 2 for each NR band. Test only one EN-DC combination per FR1 band for each EN-DC configuration as defined in clause 4.1.4 using LTE anchor agnostic approach.

(2) The applicability and test coverage rules for Standalone (SA) and NSA capable devices shall include the following:

a) For each NR band in a device, test all the EN-DC exception test requirements as per test procedures in TS 38.521-3 [8].

b) Test all the Standalone FR2 test requirements as per test procedures in clause 3for each NR band. This also fulfils coverage for all non-exception EN-DC FR2 test requirements for that NR band and need not be retested. If Standalone FR2 cannot be tested (due to test case not being complete), then test in EN-DC mode following (1)(b) above.

c) Test all the Standalone FR1 test requirements as per test procedures in clause 2 for each NR band. This also fulfils coverage for all non-exception EN-DC FR1 test requirements for that NR band and need not be retested. If Standalone FR1 cannot be tested (due to test case not being complete or if the UE does not support the band in SA mode), then test in EN-DC mode following (1)(c) above.

#### 4.1.7.2	Test coverage across 5G NR connectivity options

The test cases in this specification cover both Standalone (FR1+FR2 CA without DC) as well as Non-Standalone FR1 and FR2 (E-UTRA and 5G NR interworking) testing. Below shall be the understanding with respect to coverage across 5G NR architecture options:

1)	Unless otherwise stated within the test case, it shall be understood that test requirements are agnostic of the NSA architecture option configured within the test. The test coverage across NSA options shall be considered fulfilled by execution of the NSA test case in one NSA option. Subsequently the test execution and test results can be leveraged to other NSA options.

2)	Only one SA or NSA architecture option type is identified and utilized in the definition of each test case within this test specification. NSA test cases are configured using Connectivity EN-DC i.e. NSA Option 3 and Standalone (SA) test cases are configured using Connectivity NR i.e. SA Option 2 which shall be the default architecture options used for NSA and SA test execution respectively.

3)	If a UE does not support NSA Option 3, any other supported NSA option can be configured to execute the test. The leverage rule detailed in (1) would apply.

## 4.2	Output RF spectrum emissions

### 4.2.1	Output RF spectrum emissions for inter-band CA between FR1 and FR2

For inter-band NR CA between FR1 and FR2, out-of-band emissions specified in clause 2 and clause 3 apply for each frequency range respectively.

### 4.2.2	Output RF spectrum emissions for EN-DC

#### 4.2.2.1	Out of band emission

##### 4.2.2.1.1	General spectrum emission mask

For intra-band contiguous EN-DC, the general NR spectrum emission mask limits are defined in TS 38.521-3 [8] Table 6.5B.2.1.1.5-1 with test tolerances in Table 6.5B.2.1.1.5-2.

For intra-band non-contiguous EN-DC, the general NR spectrum emission mask limits are defined in TS 38.521-3 [8] clause 6.5B.2.2.1.5.

For inter-band EN-DC within FR1, the general NR spectrum emission mask limits are defined in TS 38.521-3 [8] clause 6.5B.2.3.1.5.

For inter-band EN-DC including FR2, the general NR spectrum emission mask limits are defined in TS 38.521-3 [8] clause 6.5B.2.4.1.5, 6.5B.2.4.1_1.1.5, 6.5B.2.4.1_1.2.5 and 6.5B.2.4.1_1.3.5.

For inter-band EN-DC including FR2 for UL-MIMO, the general NR spectrum emission mask limits are defined in TS 38.521-3 [8] clause 6.5B.2.4.1D.5.

##### 4.2.2.1.2	Additional spectrum emission mask

For intra-band contiguous EN-DC, the additional spectrum emission mask limits for NS_35 are defined in TS 38.521-3 [8] Table Table 6.5B.2.1.2.5.1-1 with test tolerances in Table 6.5B.2.1.2.5-1.

For intra-band contiguous EN-DC, the additional spectrum emission mask limits for NS_04 are defined in TS 38.521-3 [8] Table Table 6.5B.2.1.2.5.2-1 with test tolerances in Table 6.5B.2.1.2.5-1.

For intra-band non-contiguous EN-DC, the additional spectrum emission mask limits are defined in TS 38.521-3 [8] clause 6.5B.2.2.2.5.

For inter-band EN-DC within FR1, the additional spectrum emission mask limits are defined in TS 38.521-3 [8] clause 6.5B.2.3.2.5.

##### 4.2.2.1.3	Adjacent channel leakage ratio

For intra-band contiguous EN-DC, the adjacent channel leakage ratio limits are defined in TS 38.521-3 [8] Table 6.5B.2.1.3.5.1-1 with test tolerances in Table 6.5B.2.1.3.5-2.

For intra-band non-contiguous EN-DC, the adjacent channel leakage ratio limits are defined in TS 38.521-3 [8] clause 6.5B.2.2.3.5.

For inter-band EN-DC within FR1, the adjacent channel leakage ratio limits are defined in TS 38.521-3 [8] clause 6.5B.2.3.3.3.5.

For inter-band EN-DC including FR2, the adjacent channel leakage ratio limits are defined in TS 38.521-3 [8] clause 6.5B.2.4.3.5 and 6.5B.2.4.3_1.1.5.

For inter-band EN-DC including FR2 for UL-MIMO, the adjacent channel leakage ratio limits are defined in TS 38.521-3 [8] clause 6.5B.2.4D.3.5.

#### 4.2.2.2	Transmitter spurious emissions

##### 4.2.2.2.1	General spurious emissions requirements

For intra-band contiguous EN-DC, the general spurious emissions limits are defined in TS 38.521-3 [8] Table 6.5B.3.1.1.5-1.

For intra-band non-contiguous EN-DC, the general spurious emissions limits are defined in TS 38.521-3 [8] clause 6.5B.3.2.1.5.

For inter-band EN-DC within FR1, the general spurious emissions limits are defined in TS 38.521-3 [8] Table 6.5B.3.3.1.5-1.

For inter-band EN-DC including FR2, the general spurious emissions limits are defined in TS 38.521-3 [8] clause 6.5B.3.4.1.5 and 6.5B.3.4.1_1.1.5.

For inter-band EN-DC including FR2 for UL-MIMO, the general spurious emissions limits are defined in TS 38.521-3 [8] clause 6.5B.3.4.1D.5.

##### 4.2.2.2.2	Spurious emission band UE co-existence

For intra-band contiguous EN-DC, the general spurious emissions limits are defined in TS 38.521-3 [8] clause 6.5B.3.1.2.5.

For intra-band non-contiguous EN-DC, the general spurious emissions limits are defined in TS 38.521-3 [8] clause 6.5B.3.2.2.5.

For inter-band EN-DC within FR1, the general spurious emissions limits are defined in TS 38.521-3 [8] Table 6.5B.3.3.2.5-1 and 6.5B.3.3.2.5-2.

For inter-band EN-DC including FR2, the general spurious emissions limits are defined in TS 38.521-3 [8] clause 6.5B.3.4.2.5.

##### 4.2.2.2.3	Additional spurious emissions

For intra-band contiguous EN-DC, the additional spurious emissions limits are defined in TS 38.521-3 [8] clause 6.5B.4.1.5.

For intra-band non-contiguous EN-DC, the additional spurious emissions limits are defined in TS 38.521-3 [8] clause 6.5B.4.2.5.

For inter-band EN-DC within FR1, the additional spurious emissions limits are defined in TS 38.521-3 [8] clause 6.5B.4.3.5.

#### 4.2.2.3	Receiver spurious emissions

For intra-band contiguous EN-DC, the receiver spurious emissions limits are defined in TS 38.521-3 [8] clause 7.9B.1.5.

For intra-band non-contiguous EN-DC, the receiver spurious emissions limits are defined in TS 38.521-3 [8] clause 7.9B.2.5.

For inter-band EN-DC within FR1, the additional receiver emissions limits are defined in TS 38.521-3 [8] clause 7.9B.3.5 and 7.9B.3_1.1.5.

For inter-band EN-DC including FR2, the receiver spurious emissions limits are defined in TS 38.521-3 [8] clause 7.9B.4.5, 7.9B.4_1.1.5, 7.9B.4_1.2.5, 7.9B.4_1.3.5 and 7.9B.4_1.4.5.

# 5	References

[1]	GPP TS 36.101 [1] V17.6.0 (2022-06), “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio ransmission and reception (Release 17)”

[2]	3GPP TS 38.101-1 V17.6.0 (2022-06), “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 17)”

[3]	3GPP TS 38.101-2 V17.6.0 (2022-06), “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone (Release 17)”

[4]	3GPP TS 38.101-3 V17.6.0 (2022-06), “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios (Release 17)”

[5]	3GPP TS 36.521-1 [5] (2022-06), “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Conformance Testing (Release 17)”

[6]	3GPP 38.521-1 V17.5.0 (2022-06), “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone; (Release 17)”

[7]	3GPP 38.521-2 V16.12.0 (2022-06), “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone (Release 16)”

[8]	3GPP 38.521-3 V17.5.0 (2022-06), “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios (Release 17)”

[9]	ITU-R Recommendation SM.329: "Unwanted emissions in the spurious domain".

[10]	ITU-R Recommendation M.1545: "Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000".

Attachment 1 
to Annex 1

Definition of test tolerance

Test tolerance

With reference to Recommendation ITU-R M.1545 [10], “test tolerance” is the relaxation value referred to in recommends 2 of Recommendation ITU-R M.1545 [10], i.e. the difference between the core specification value and the test limit, evaluated applying the shared risk principle as per Figs 2 and 3 of Annex 1 of Recommendation ITU-R M.1545 [10]. In case the core specification value is equal to the test limit (Fig. 3 of Annex 1 of Recommendation ITU-R M.1545 [10]) the “test tolerances” are equal to 0.