3GPP TSG-RAN WG5 Meeting #96-e                                                       R5-224325

# Electronic Meeting, 15 – 26 August 2022

Source:	Qualcomm Incorporated

Title:	Discussion paper on TLB mode for SDT

# Document for:	Agreement

Agenda Item:	6.3.26.6

## 1.	Introduction

Small data transmission (SDT) is a procedure allowing data and/or signaling transmission while remaining in RRC_INACTIVE state (i.e., w/o transitioning to RRC_CONNECTED state). SDT can be configured and is enabled on a radio bearer (i.e., either DRBs or SRB) basis and is initiated by UE if certain criteria are met. The SDT is initiated with either a transmission over RACH (configured via system information) or over Type 1 CG resources (configured via dedicated signaling in RRCRelease message) by leveraging RRC resume procedure.

Release 17 WI on small data transmissions (SDT) includes transmissions using RA-SDT (RRC Based) and CG-SDT where the SDT data is generated in the UE while in RRC_INACTIVE state. Given this is MO Data, test methodology needs to be defined or reused from existing TLB modes

## 2.	Discussion

Given most of the SDT verification will involve NR Layer 2 and  NR layer 3 testing to verify data transfer continuation over RRC and RRM procedures, it may be possible to use TLB Mode B with below settings and call flow explained in below procedural steps –

Option 1:- Reuse of UE test loop mode B

Step 1:- NW configures Test loop B with T_delay_modeB

T_delay_modeB - timer value is configured by the ‘UE test loop mode B setup IE’ in the ‘CLOSE UE TEST LOOP’ message.

Step 2:- SS transmits one IP PDU

Step 3:- NW sends RRC release with “suspendConfig”; UE goes to RRC_INACTIVE state

UE treats this one IP PDU in its buffer as a candidate for the SDT transmission after the expiry of T_delay_modeB; test environment at this time shall ensure SDT transmission conditions are met (e.g. thresholds and other configurations)

Step 4:- UE loops back data in inactive state by triggering MO SDT  [RA based / CG based]

Step 5 – NW acks this data and this constitutes the first SDT successful transmission

Step 6:- NW sends subsequent DL data in inactive state

Step 7:- UE loops back subsequent data in inactive state

Step 8:- NW sends RRC release [SDT Session End]

Option 2:- Define a new Test Function and reuse TLB A or B

Step 1:- NW Sends New Test CMD to configure the first MO SDT mode[Ex- SET_SDT_COMMAND] with a delay timer

[ SET_SDT_COMMAND] - Consider any data PDU to be loop backed only in MO SDT mode with a delay criteria.

Step 2:- NW configures TLB A or TLB B and CLOSE UE Test Loop

Step 3:- NW sends RRC release with “suspendConfig”

Step 4 : UE starts MO SDT  [RA based / CG based] based on the criteria set forth in SET_SDT_COMMAND

Step 5 – NW acks this data and this constitutes the first SDT successful transmission

Step 6:- NW sends subsequent DL data in inactive state

Step 7:- UE loops back subsequent data in inactive state

Step 8:- NW sends RRC release [SDT Session End]

## 3.	Proposal/Way Forward

- Comparison: Option#1 involves minimal changes and might be best option to test SDT data in RRC Inactive; Option#2 involves defining a new Test Function.

- Proposal: Option#1 is proposed because there are no further changes are needed to TS36.509

## 4.	References

[1]	TS 36.509 - Special conformance testing functions for User Equipment (UE)