3GPP TSG-RAN5 Meeting #96-e	   R5-224327

# Electronic Meeting, 15th Aug – 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2449 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of Table 4.6.1-30 to add message contents values for SDT in UEAssistanceInformation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm CDMA Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-07-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SDT Information Elements values corresponding to UEAssistanceInformation message are needed for 3GPP SDT test cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | ##### Updated UEAssistanceInformation message Information elements values for SDT |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This aspect of the work plan will remain unfulfilled |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 4.6.1	Contents of RRC messages

<Contents skipped>

<Start of Changes>

#### –	UEAssistanceInformation

Table 4.6.1-30: UEAssistanceInformation

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEAssistanceInformation ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| ueAssistanceInformation SEQUENCE { |  |  |  |  |
| delayBudgetReport CHOICE { |  |  |  |  |
| type1 |  | Not checked |  |  |
| } |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | SIDELINK |
| overheatingAssistance |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| idc-Assistance-r16 |  | Not checked |  |  |
| drx-Preference-r16 |  | Not checked |  |  |
| maxBW-Preference-r16 |  | Not checked |  |  |
| maxCC-Preference-r16 |  | Not checked |  |  |
| maxMIMO-LayerPreference-r16 |  | Not checked |  |  |
| minSchedulingOffsetPreference-r16 |  | Not checked |  |  |
| releasePreference-r16 |  | Not checked |  |  |
| sl-UE-AssistanceInformationNR-r16 |  | Not checked |  |  |
| sl-UE-AssistanceInformationNR-r16 SEQUENCE (SIZE (1..maxNrofTrafficPattern-r16)) OF SL-TrafficPatternInfo-r16 { |  | 1 entry |  | SIDELINK |
| SL-TrafficPatternInfo-r16[1] SEQUENCE { |  |  | entry 1 |  |
| trafficPeriodicity-r16 |  | FFS |  |  |
| timingOffset-r16 |  | FFS |  |  |
| messageSize-r16 |  | FFS |  |  |
| sl-QoS-FlowIdentity-r16 |  | SL-QoS-FlowIdentity-r16 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| referenceTimeInfoPreference-r16 |  | Not checked |  |  |
| nonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | SDT |
| ul-GapFR2-Preference-r17 |  | Not checked |  |  |
| musim-Assistance-r17 |  | Not checked |  |  |
| overheatingAssistance-r17 |  | Not checked |  |  |
| maxBW-PreferenceFR2-2-r17 |  | Not checked |  |  |
| maxMIMO-LayerPreferenceFR2-2-r17 |  | Not checked |  |  |
| minSchedulingOffsetPreferenceExt-r17 |  | Not checked |  |  |
| rlm-MeasRelaxationState-r17 |  | Not checked |  |  |
| bfd-MeasRelaxationState-r17 |  | Not checked |  |  |
| nonSDT-DataIndication-r17 SEQUENCE { |  |  |  | SDT |
| resumeCause-r17 |  | mo-Data |  |  |
| } |  |  |  |  |
| scg-DeactivationPreference |  | Not checked |  |  |
| uplinkData-r17 |  | Not checked |  |  |
| rrm-MeasRelaxationFulfilment-r17 |  | Not checked |  |  |
| propagationDelayDifference-r17 |  | Not checked |  |  |
| nonCriticalExtension |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

<End of Changes>