3GPP TSG-RAN5 Meeting #96-e	R5-224382

# Online, , 15th Aug 2022 - 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-2 | CR | 0161 | rev | - | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LPP Provide Capabilities template for subclause 9 test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The CR R5-224379 for 3GPP 37.571-3 introduces 2 new pics parameters pc_ECID_onNR and pc_OTDOA_onNR for the support of ECID and OTDOA for NR test cases included in Subclause 9. These parameters replace the existing pc_ECID and pc_OTDOA for the aforementioned subclause. Therefore, the tables on subclause 9 have to be updated to replace the pics parameters mentioned above. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include the new pics parameters pc_ECID_onNR and pc_OTDOA_onNR in the LPP Provide Capabilities templates for Subclause 9 test cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail test cases defined in 3GPP TS 37.571-2 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.1, 9.3.1.2, 9.3.2.1, 9.3.2.2, 9.3.2.3, 9.3.4.1, 9.3.4.2, 9.3.4.3, 9.3.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

Table 9.3.1.1.3.3-5: LPP PROVIDE CAPABILITIES (step 2, Table 7.3.1.1.3.2-1)

| Derivation Path: Table 7.3.1.1.3.3-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| As defined in Table 7.3.1.1.3.3-6 with the following exception: |  |  |  |  |
| LPP-Message ::= SEQUENCE { |  |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideCapabilities SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideCapabilities-r9 SEQUENCE { |  |  |  |  |
| otdoa-ProvideCapabilities |  | Present or not present dependent on pc_OTDOA_onNR |  |  |
| ecid-ProvideCapabilities |  | Present or not present dependent on pc_ECID_onNR |  |  |
| nr-ECID-ProvideCapabilities-r16 |  | Present or not present dependent on pc_NR_ECID. Set according to Table 9.3.1.1.3.3-6. | Rel-16 onwards |  |
| nr-Multi-RTT-ProvideCapabilities-r16 SEQUENCE { |  | Present or not present dependent on pc_Multi_RTT. | Rel-16 onwards |  |
| nr-Multi-RTT-PRS-Capability-r16 |  | Set according to Table 9.3.1.1.3.3-7 |  |  |
| nr-Multi-RTT-MeasurementCapability-r16 |  | Set according to Table 9.3.1.1.3.3-8 |  |  |
| nr-DL-PRS-QCL-ProcessingCapability-r16 |  | Set according to Table 9.3.1.1.3.3-9 |  |  |
| nr-DL-PRS-ProcessingCapability-r16 |  | Set according to Table 9.3.1.1.3.3-10 |  |  |
| nr-UL-SRS-Capability-r16 |  | Set according to Table 9.3.1.1.3.3-11 |  |  |
| additionalPathsReport-r16 |  | Dependent on UE capabilities |  |  |
| periodicalReporting-r16 |  | Dependent on UE capabilities |  |  |
| } |  |  |  |  |
| nr-DL-AoD-ProvideCapabilities-r16 SEQUENCE { |  | Present or not present dependent on (pc_UEA_DL_AoD OR pc_UEB_DL_AoD). | Rel-16 onwards |  |
| nr-DL-AoD-Mode-r16 |  | Dependent on UE capabilities |  |  |
| nr-DL-AoD-PRS-Capability-r16 |  | Set according to Table 9.3.1.1.3.3-7. |  |  |
| nr-DL-AoD-MeasurementCapability-r16 |  | Set according to Table 9.3.1.1.3.3-12. |  |  |
| nr-DL-PRS-QCL-ProcessingCapability-r16 |  | Set according to Table 9.3.1.1.3.3-9. |  |  |
| nr-DL-PRS-ProcessingCapability-r16 |  | Set according to Table 9.3.1.1.3.3-10. |  |  |
| periodicalReporting-r16 |  | Dependent on UE capabilities |  |  |
| } |  |  |  |  |
| nr-DL-TDOA-ProvideCapabilities-r16 SEQUENCE { |  | Present or not present dependent on (pc_UEA_DL_TDOA OR pc_UEB_DL_TDOA) | Rel-16 onwards |  |
| nr-DL-TDOA-Mode-r16 |  | Dependent on UE capabilities |  |  |
| nr-DL-TDOA-PRS-Capability-r16 |  | Set according to Table 9.3.1.1.3.3-7. |  |  |
| nr-DL-TDOA-MeasurementCapability-r16 |  | Set according to Table 9.3.1.1.3.3-13. |  |  |
| nr-DL-PRS-QCL-ProcessingCapability-r16 |  | Set according to Table 9.3.1.1.3.3-9. |  |  |
| nr-DL-PRS-ProcessingCapability-r16 |  | Set according to Table 9.3.1.1.3.3-10. |  |  |
| additionalPathsReport-r16 |  | Dependent on UE capabilities |  |  |
| periodicalReporting-r16 |  | Dependent on UE capabilities |  |  |
| } |  |  |  |  |
| nr-UL-ProvideCapabilities-r16 SEQUENCE { |  | Present or not present and value dependent on UE capabilities. | Rel-16 onwards |  |
| nr-UL-SRS-Capability-r16 |  | Set according to Table 9.3.1.1.3.3-11 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

# < Unchanged sections omitted >

9.3.1.2.3.3	Specific message contents

As defined in clause 7.3.5.1.3.3, with the following exceptions:

Table 9.3.1.2.3.3-0 replaces Table 7.3.5.1.3.3-0, Table 9.3.1.2.3.3-1 replaces Table 7.3.5.1.3.3-1, Table 9.3.1.2.3.3-2 replaces Table 7.3.5.1.3.3-2, Table 9.3.1.2.3.3-2a replaces Table 7.3.5.1.3.3-2c, Table 9.3.1.2.3.3-2b replaces Table 7.3.5.1.3.3-2d, Table 9.3.1.2.3.3-3 replaces Table 7.3.5.1.3.3-5, Table 9.3.1.2.3.3-4 replaces Table 7.3.5.1.3.3-6, Table 9.3.1.2.3.3-5 replaces Table 7.3.5.1.3.3-3 and Table 9.3.1.2.3.3-6 replaces Table 7.3.5.1.3.3-7

# < Unchanged sections omitted >

Table 9.3.1.2.3.3-6: LPP Provide Capabilities (step 0a, Table 7.3.5.1.3.2-1)

| Derivation Path: Table 7.3.5.1.3.3-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| As defined in Table ith the following exceptions: |  |  |  |  |
| LPP-Message ::= SEQUENCE { |  |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideCapabilities SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideCapabilities-r9 SEQUENCE { |  |  |  |  |
| otdoa-ProvideCapabilities |  | Present or not present dependent on pc_OTDOA_onNR |  |  |
| ecid-ProvideCapabilities |  | Present or not present dependent on pc_ECID_onNR |  |  |
| nr-ECID-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| nr-Multi-RTT-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| nr-DL-AoD-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| nr-DL-TDOA-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| nr-UL-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

# < Unchanged sections omitted >

Table 9.3.2.1.3.3-5: LPP Provide Capabilities (step 3, Table 7.3.2.1.3.2-1)

| Derivation Path: Table 7.3.2.2.3.3-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| As defined in Table 7.3.2.2.3.3-6 with the following exceptions: |  |  |  |  |
| LPP-Message ::= SEQUENCE { |  |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideCapabilities SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideCapabilities-r9 SEQUENCE { |  |  |  |  |
| otdoa-ProvideCapabilities |  | Present or not present dependent on pc_OTDOA_onNR |  |  |
| ecid-ProvideCapabilities |  | Present or not present dependent on pc_ECID_onNR |  |  |
| nr-ECID-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| nr-Multi-RTT-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| nr-DL-AoD-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| nr-DL-TDOA-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| nr-UL-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

# < Unchanged sections omitted >

Table 9.3.2.2.3.3-5: LPP Provide Capabilities (step 2, Table 7.3.2.2.3.2-1)

| Derivation Path: Table 7.3.2.2.3.3-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| As defined in Table 7.3.2.2.3.3-6 with the following exceptions: |  |  |  |  |
| LPP-Message ::= SEQUENCE { |  |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideCapabilities SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideCapabilities-r9 SEQUENCE { |  |  |  |  |
| otdoa-ProvideCapabilities |  | Present or not present dependent on pc_OTDOA_onNR |  |  |
| ecid-ProvideCapabilities |  | Present or not present dependent on pc_ECID_onNR |  |  |
| nr-ECID-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| nr-Multi-RTT-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| nr-DL-AoD-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| nr-DL-TDOA-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| nr-UL-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

# < Unchanged sections omitted >

Table 9.3.2.3.3.3-5: LPP Provide Capabilities (steps 2, 4, 6, and 8, Table 7.3.2.3.3.2-1)

| Derivation Path: Table 7.3.2.3.3.3-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| As defined in Table 7.3.2.3.3.3-6 with the following exceptions: |  |  |  |  |
| LPP-Message ::= SEQUENCE { |  |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideCapabilities SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideCapabilities-r9 SEQUENCE { |  |  |  |  |
| otdoa-ProvideCapabilities |  | Present or not present dependent on pc_OTDOA_onNR |  |  |
| ecid-ProvideCapabilities |  | Present or not present dependent on pc_ECID_onNR |  |  |
| nr-ECID-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| nr-Multi-RTT-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| nr-DL-AoD-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| nr-DL-TDOA-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| nr-UL-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

# < Unchanged sections omitted >

Table 9.3.4.1.3.3-5: LPP Provide Capabilities. (step 1b, Table 7.3.4.1.3.2-1)

| Derivation Path: Table 7.3.4.1.3.3-7a |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| As defined in Table 7.3.4.1.3.3-7a with the following exceptions: |  |  |  |  |
| LPP-Message ::= SEQUENCE { |  |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideCapabilities SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideCapabilities-r9 SEQUENCE { |  |  |  |  |
| otdoa-ProvideCapabilities |  | Present or not present dependent on pc_OTDOA_onNR |  |  |
| ecid-ProvideCapabilities |  | Present or not present dependent on pc_ECID_onNR |  |  |
| nr-Multi-RTT-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| nr-DL-AoD-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| nr-DL-TDOA-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| nr-UL-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

# < Unchanged sections omitted >

Table 9.3.4.2.3.3-6: LPP Provide Capabilities. (step 1c, Table 7.3.4.2.3.2-1)

| Derivation Path: Table 7.3.4.2.3.3-8a |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| As defined in Table 7.3.4.2.3.3-8a with the following exceptions: |  |  |  |  |
| LPP-Message ::= SEQUENCE { |  |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideCapabilities SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideCapabilities-r9 SEQUENCE { |  |  |  |  |
| otdoa-ProvideCapabilities |  | Present or not present dependent on pc_OTDOA_onNR |  |  |
| ecid-ProvideCapabilities |  | Present or not present dependent on pc_ECID_onNR |  |  |
| nr-Multi-RTT-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| nr-DL-AoD-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| nr-DL-TDOA-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| nr-UL-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

# < Unchanged sections omitted >

Table 9.3.4.3.3.3-5: LPP Provide Capabilities. (step 1b, Table 7.3.4.3.3.2-1)

| Derivation Path: Table 7.3.4.3.3.3-7a |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| As defined in Table 7.3.4.3.3.3-7a with the following exceptions: |  |  |  |  |
| LPP-Message ::= SEQUENCE { |  |  |  |  |
| transactionID SEQUENCE { |  |  |  |  |
| initiator |  | locationServer |  |  |
| transactionNumber |  | (0..255) | Contains the same value as the corresponding field in the LPP Request Capabilities message in step 1a, Table 7.3.4.3.3.2-1 |  |
| } |  |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideCapabilities SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideCapabilities-r9 SEQUENCE { |  |  |  |  |
| otdoa-ProvideCapabilities |  | Present or not present dependent on pc_OTDOA_onNR |  |  |
| ecid-ProvideCapabilities |  | Present or not present dependent on pc_ECID_onNR |  |  |
| nr-DL-AoD-RequestCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| nr-DL-TDOA-RequestCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

# < Unchanged sections omitted >

Table 9.3.4.4.3.3-5: LPP Provide Capabilities. (step 1c, Table 7.3.4.4.3.2-1)

| Derivation Path: Table 7.3.4.4.3.3-8a |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| As defined in Table 7.3.4.4.3.3-8a with the following exceptions: |  |  |  |  |
| LPP-Message ::= SEQUENCE { |  |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideCapabilities SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideCapabilities-r9 SEQUENCE { |  |  |  |  |
| otdoa-ProvideCapabilities |  | Present or not present dependent on pc_OTDOA_onNR |  |  |
| ecid-ProvideCapabilities |  | Present or not present dependent on pc_ECID_onNR |  |  |
| nr-Multi-RTT-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| nr-DL-AoD-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| nr-DL-TDOA-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| nr-UL-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |