3GPP TSG-RAN5 Meeting #96-e	   R5-224392

# Electronic Meeting, 15th Aug – 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2461 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of message contents values for SDT in SDT-Config-r17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm CDMA Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-07-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SDT Information Elements values corresponding to SDT-Config-r17 message are needed for 3GPP SDT test cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | ##### Added SDT-Config-r17 and corresponding SDT-MAC-PHY-CG-Config-r17 message Information elements values for SDT |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This aspect of the work plan will remain unfulfilled |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Secretary to resolve the “xx” |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 4.6.3	Radio resource control information elements

<Contents skipped>

<Start of Changes>

#### –	SDT-Config-r17

Table 4.6.3-xxx: SDT-Config-r17

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SDT-Config-r17 ::= SEQUENCE { |  |  |  |
| sdt-DRB-List-r17 SEQUENCE (SIZE (0..maxDRB)) OF DRB-Identity | FFS |  |  |
| sdt-SRB2-Indication-r17 | Not present |  |  |
| sdt-MAC-PHY-CG-Config-r17 | SDT-MAC-PHY-CG-Config-r17 | OCTET STRING (CONTAINING SDT-MAC-PHY-CG-Config-r17) |  |
| sdt-DRB-ContinueROHC-r17 | Not present |  |  |
| } |  |  |  |

<End of Changes>

<Start of Changes>

#### –	SDT-MAC-PHY-CG-Config-r17

Table 4.6.3-xxx: SDT-MAC-PHY-CG-Config-r17

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SDT-MAC-PHY-CG-Config-r17 ::= SEQUENCE { |  |  |  |
| cg-SDT-Config-LCH-restrictionToAddModList-r17 SEQUENCE (SIZE(1..maxLC-ID)) OF CG-SDT-Config-LCH-restriction-r17 | FFS |  |  |
| cg-SDT-Config-LCH-restrictionToReleaseList-r17 SEQUENCE (SIZE(1..maxLC-ID)) OF LogicalChannelIdentity | FFS |  |  |
| cg-SDT-Config-Initial-BWP-NUL-r17 | FFS |  |  |
| cg-SDT-Config-Initial-BWP-SUL-r17 | FFS |  |  |
| cg-SDT-Config-Initial-BWP-DL-r17 | FFS |  |  |
| cg-SDT-TimeAlignmentTimer-r17 | FFS |  |  |
| cg-SDT-RSRP-ThresholdSSB-r17 | FFS |  |  |
| cg-SDT-TA-ValiditationConfig-r17 | FFS |  |  |
| } |  |  |  |

<End of Changes>