3GPP TSG-RAN5 Meeting #96-e	R5-224451

# Electronic Meeting, 15th - 26th August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5111 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to NB-IoT MAC test case 22.3.1.12 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, NB_IOTenh2-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current prose defines NPRACH resources only for a single enhanced coverage level. However, the test purpose requires to verify that the resource that corresponds to the actual UE enhanced coverage level is used. It is proposed to extend the configuration to include more coverage levels. Definition of fmt2-Parameters-r15 for SystemInformationBlockType2-NB is not strictly necessary since as per test purpose UE is supposed to use those defined in SystemInformationBlockType23-NB. It is proposed to remove the redundant configuration. Step 4 of the main procedure should also verify if the random access preamble was repeated numRepetitionsPerPreambleAttempt. In the absence of a declared requirement and in order to simplify the execution of the preamble, specific definitnon of nprach-ProbabilityAnchor-r14 may be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Extended configuration with two resources for further enhanced coverage level. Removed definition of fmt2-Parameters-r15 from SystemInformationBlockType2-NB. Updated definition of Step 4 in the main behaviour description. Removed definition of nprach-ProbabilityAnchor-r14 in SystemInformationBlockType22-NB for the preamble. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test purpose will not be achieved |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.3.1.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 22.3.1.12	NB-IoT / RACH Procedure / Non-anchor carrier / Preamble format 2

##### 22.3.1.12.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state and SystemInformationBlockType23-NB with IE nprach-ParametersListFmt2-r15 is broadcast }

ensure that {

when { UE has need to send a CCCH UL MAC PDU and UE is in enhanced coverage level 0}

then { UE transmits a random access preamble repeated numRepetitionsPerPreambleAttempt from Random Access Preambles group using the non-anchor NPRACH resource corresponding to enhanced coverage level 0}

}

##### 22.3.1.12.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.321, clauses 5.1.1 and TS 36.331, clauses 5.2.2.4.

[TS 36.321, clause 5.1.1]

…

-	if the UE is an NB-IoT UE:

-	the available set of PRACH resources supported in the Serving Cell on the anchor carrier, nprach-ParametersList, and on the non-anchor carriers, in ul-ConfigList.

-	for EDT, the available set of PRACH resources associated with EDT on anchor carrier, nprach-ParametersList-EDT, and on the non-anchor carriers, in ul-ConfigList.

-	for random access resource selection and preamble transmission:

-	a PRACH resource is mapped into an enhanced coverage level.

-	each PRACH resource contains a set of nprach-NumSubcarriers subcarriers which can be partitioned into one or two groups for single/multi-tone Msg3 transmission by nprach-SubcarrierMSG3-RangeStart and nprach-NumCBRA-StartSubcarriers as specified in TS 36.211 [7, 10.1.6.1]. Each group is referred to as a Random Access Preamble group below in the procedure text.

-	a subcarrier is identified by the subcarrier index in the range:
[nprach-SubcarrierOffset, nprach-SubcarrierOffset + nprach-NumSubcarriers -1]

-	each subcarrier of a Random Access Preamble group corresponds to a Random Access Preamble.

-	when the subcarrier index is explicitly sent from the eNB as part of a PDCCH order ra-PreambleIndex shall be set to the signalled subcarrier index.

-	the mapping of the PRACH resources into enhanced coverage levels is determined according to the following:

-	the number of enhanced coverage levels is equal to one plus the number of RSRP thresholds present in rsrp-ThresholdsPrachInfoList.

-	each enhanced coverage level has one anchor carrier PRACH resource present in nprach-ParametersList and zero or one PRACH resource for each non-anchor carrier signalled in ul-ConfigList.

-	for EDT, each enhanced coverage level has zero or one anchor carrier PRACH resource present in nprach-ParametersList-EDT and zero or one PRACH resource for each non-anchor carrier signalled in ul-ConfigList.

-	enhanced coverage levels are numbered from 0 and the mapping of PRACH resources to enhanced coverage levels are done in increasing numRepetitionsPerPreambleAttempt order.

-	when multiple carriers provide PRACH resources for the same enhanced coverage level, the UE will randomly select one of them using the following selection probabilities:

-	the selection probability of the anchor carrier PRACH resource for the given enhanced coverage level, nprach-ProbabilityAnchor, is given by the corresponding entry in nprach-ProbabilityAnchorList

-	the selection probability is equal for all non-anchor carrier PRACH resources and the probability of selecting one PRACH resource on a given non-anchor carrier is (1- nprach-ProbabilityAnchor)/(number of non-anchor NPRACH resources)

[TS 36.331, clauses 5.2.2.4]

…

1>	if the NB-IoT UE supports NPRACH resources using preamble format 2:

2>	if schedulingInfoList indicates that SystemInformationBlockType23-NB is present and the UE does not have stored a valid version of this system information block:

3>	acquire SystemInformationBlockType23-NB;

…

##### 22.3.1.12.3	Test description

22.3.1.12.3.1	Pre-test conditions

System Simulator:

-	Ncell 1.

NB-IoT Operation Mode set to standalone.

-	System information combination 7 as defined in TS 36.508[18] clause 8.1.4.3.1.1.

- 	SystemInformationBlockType2-NB with specific information element contents as defined in Tables 22.3.1.12.3.3-1, 22.3.1.12.3.3-2 and 22.3.1.12.3.3-3, SystemInformationBlockType22-NB as specified in Table 22.3.1.12.3.3-4, SystemInformationBlockType23-NB as specified in Table 22.3.1.12.3.3-7.

UE:

None.

Preamble:

-	UE is in state Registered, Idle Mode (state 3-NB) according to [18] in Ncell 1.

22.3.1.12.3.2	Test procedure sequence

Table 22.3.1.12.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts SystemInformationBlockType22-NB to indicate the NPRACH Probability Anchor is set to zero and notifies the UE of change of System Information. | <-- | Paging-NB | - | - |
| 2 | Wait for 90 seconds for UE to read updated SystemInformationBlockType22-NB | - | - | - | - |
| 3 | The SS transmits a Paging message including a matched identity. | <-- | Paging-NB | - | - |
| 4 | Check: Does the UE transmit a preamble numRepetitionsPerPreambleAttempt times on NPRACH on non-anchor carrier in ul-ConfigList-v1530 in SystemInformationBlockType23-NB? | --> | NPRACH Preamble | 1 | P |
| 5 | The SS transmits a MAC PDU addressed to UE RA-RNTI, containing a matching RAPID. | <-- | Random Access Response | - | - |
| 6 | UE sends an RRCConnectionRequest-NB message. | --> | MAC PDU (RRCConnectionRequest-NB) | - | - |
| 7 | The SS transmits a valid MAC PDU containing “UE Contention Resolution Identity” MAC control element with matching “Contention Resolution Identity” and CCCH message (RRCConnectionSetup-NB containing configuration of UE-specific search space). | <-- | MAC PDU (UE Contention Resolution Identity, RRCConnectionSetup-NB) | - | - |
| 8 | UE sends an RRCConnectionSetupComplete-NB message. | --> | RRCConnectionSetupComplete-NB | - | - |

22.3.1.12.3.3	Specific message contents

Table 22.3.1.12.3.3-1: NPRACH-ConfigSIB-NB-v1530-DEFAULT in SystemInformationBlockType2-NB (preamble)

| Derivation Path: 36.508 Table 8.1.6.3-17 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPRACH-ConfigSIB-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| rsrp-ThresholdsPrachInfoList-r13 SEQUENCE (SIZE(1..2)) OF RSRP-Range { | 2 entries |  |  |
| RSRP-Range[1] | 61 | entry 1 -79 dBm |  |
| RSRP-Range[2] | 41 | entry 2 -99 dBm |  |
| } |  |  |  |
| nprach-ParametersList-r13 SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF NPRACH-Parameters-NB-r13 { | 3 entries |  |  |
| NPRACH-Parameters-NB-r13[1] SEQUENCE { |  | entry 1 CE level 0 |  |
| nprach-Periodicity-r13 | ms640 |  |  |
| nprach-StartTime-r13 | ms8 |  |  |
| nprach-SubcarrierOffset-r13 | n12 |  |  |
| nprach-NumSubcarriers-r13 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r13 | oneThird |  |  |
| maxNumPreambleAttemptCE-r13 | n3 |  |  |
| numRepetitionsPerPreambleAttempt-r13 | n1 |  |  |
| npdcch-NumRepetitions-RA-r13 | r16 |  |  |
| npdcch-StartSF-CSS-RA-r13 | v4 |  |  |
| npdcch-Offset-RA-r13 | zero |  |  |
| } |  |  |  |
| NPRACH-Parameters-NB-r13[2] SEQUENCE { |  | entry 2 CE level 1 |  |
| nprach-Periodicity-r13 | ms640 |  |  |
| nprach-StartTime-r13 | ms32 |  |  |
| nprach-SubcarrierOffset-r13 | n12 |  |  |
| nprach-NumSubcarriers-r13 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r13 | oneThird |  |  |
| maxNumPreambleAttemptCE-r13 | n3 |  |  |
| numRepetitionsPerPreambleAttempt-r13 | n2 |  |  |
| npdcch-NumRepetitions-RA-r13 | r16 |  |  |
| npdcch-StartSF-CSS-RA-r13 | v4 |  |  |
| npdcch-Offset-RA-r13 | zero |  |  |
| } |  |  |  |
| NPRACH-Parameters-NB-r13[3] SEQUENCE { |  | entry 3 CE level 2 |  |
| nprach-Periodicity-r13 | ms640 |  |  |
| nprach-StartTime-r13 | ms128 |  |  |
| nprach-SubcarrierOffset-r13 | n12 |  |  |
| nprach-NumSubcarriers-r13[ | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r13 | oneThird |  |  |
| maxNumPreambleAttemptCE-r13 | n3 |  |  |
| numRepetitionsPerPreambleAttempt-r13 | n4 |  |  |
| npdcch-NumRepetitions-RA-r13 | r16 |  |  |
| npdcch-StartSF-CSS-RA-r13 | v4 |  |  |
| npdcch-Offset-RA-r13 | zero |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.3.1.12.3.3-2: RACH-ConfigCommon-NB-DEFAULT in SystemInformationBlockType2-NB  (preamble)

| Derivation Path: 36.508 Table 8.1.6.3-8 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RACH-ConfigCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| preambleTransMax-CE-r13 | n7 |  |  |
| rach-InfoList-r13 SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF RACH-Info-NB-r13 { | 3 entries |  |  |
| RACH-Info-NB-r13[1] SEQUENCE { |  | entry 1 CE level 0 |  |
| ra-ResponseWindowSize-r13 | pp10 |  |  |
| mac-ContentionResolutionTimer-r13 | pp8 |  |  |
| } |  |  |  |
| RACH-Info-NB-r13[2] SEQUENCE { |  | entry 2 CE level 1 |  |
| ra-ResponseWindowSize-r13 | pp10 |  |  |
| mac-ContentionResolutionTimer-r13 | pp8 |  |  |
| } |  |  |  |
| RACH-Info-NB-r13[3] SEQUENCE { |  | entry 3 CE level 2 |  |
| ra-ResponseWindowSize-r13 | pp10 |  |  |
| mac-ContentionResolutionTimer-r13 | pp8 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |  |

Table 22.3.1.12.3.3-3: NPUSCH-ConfigCommon-NB-DEFAULT in SystemInformationBlockType2-NB (preamble)

| Derivation Path: 36.508 Table 8.1.6.3-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPUSCH-ConfigCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| ack-NACK-NumRepetitions-Msg4-r13 SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF ACK-NACK-NumRepetitions-NB-r13 { | 3 entries |  |  |
| ACK-NACK-NumRepetitions-NB-r13[1] | r8 | entry 1 CE level 0 |  |
| ACK-NACK-NumRepetitions-NB-r13[2] | r8 | entry 2 CE level 1 |  |
| ACK-NACK-NumRepetitions-NB-r13[3] | r8 | entry 3 CE level 2 |  |
| } |  |  |  |
| } |  |  |  |

Table 22.3.1.12.3.3-4: SystemInformationBlockType22-NB (preamble)

| Derivation Path: 36.508 Table 8.1.4.3.3-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType22-NB-r14 ::= SEQUENCE { |  |  |  |
| dl-ConfigList-r14 SEQUENCE (SIZE (1.. maxNonAnchorCarriers-NB-r14)) OF DL-ConfigCommon-NB-r14 { | 1 entry |  |  |
| DL-ConfigCommon-NB-r14[1] SEQUENCE { |  | entry 1 |  |
| dl-CarrierConfig-r14 | DL-CarrierConfigCommon-NB-DEFAULT1 |  |  |
| pcch-Config-r14 SEQUENCE { |  |  |  |
| npdcch-NumRepetitionPaging-r14 | Not present |  |  |
| pagingWeight-r14 | w1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ul-ConfigList-r14 SEQUENCE (SIZE (1.. maxNonAnchorCarriers-NB-r14)) OF UL-ConfigCommon-NB-r14 { | 1 entry |  |  |
| UL-ConfigCommon-NB-r14[1] SEQUENCE { |  | entry 1 |  |
| ul-CarrierFreq-r14 | Uplink EARFCN for non-anchor carrier under test. For Signalling test cases, see 36.508 clauses 8.3.2.3. Use the anchor carrier EARFCN + 2. |  |  |
| nprach-ParametersList-r14 SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF NPRACH-Parameters-NB-r14 { | 3 entries |  |  |
| nprach-Parameters-r14[1] SEQUENCE { |  | entry 1 CE level 0 |  |
| nprach-Periodicity-r14 | ms640 |  |  |
| nprach-StartTime-r14 | ms8 |  |  |
| nprach-SubcarrierOffset-r14 | n12 |  |  |
| nprach-NumSubcarriers-r14 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r14 | oneThird |  |  |
| npdcch-NumRepetitions-RA-r14 | r16 |  |  |
| npdcch-StartSF-CSS-RA-r14 | v4 |  |  |
| npdcch-Offset-RA-r14 | zero |  |  |
| nprach-NumCBRA-StartSubcarriers-r14 | n8 |  |  |
| npdcch-CarrierIndex-r14 | 1 |  |  |
| } |  |  |  |
| nprach-Parameters-r14[2] SEQUENCE { |  | entry 2 CE level 1 |  |
| nprach-Periodicity-r14 | ms640 |  |  |
| nprach-StartTime-r14 | ms32 |  |  |
| nprach-SubcarrierOffset-r14 | n12 |  |  |
| nprach-NumSubcarriers-r14 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r14 | oneThird |  |  |
| npdcch-NumRepetitions-RA-r14 | r16 |  |  |
| npdcch-StartSF-CSS-RA-r14 | v4 |  |  |
| npdcch-Offset-RA-r14 | zero |  |  |
| nprach-NumCBRA-StartSubcarriers-r14 | n8 |  |  |
| npdcch-CarrierIndex-r14 | 2 |  |  |
| } |  |  |  |
| nprach-Parameters-r14[3] SEQUENCE { |  | entry 3 CE level 2 |  |
| nprach-Periodicity-r14 | ms640 |  |  |
| nprach-StartTime-r14 | ms128 |  |  |
| nprach-SubcarrierOffset-r14 | n12 |  |  |
| nprach-NumSubcarriers-r14 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r14 | oneThird |  |  |
| npdcch-NumRepetitions-RA-r14 | r16 |  |  |
| npdcch-StartSF-CSS-RA-r14 | v4 |  |  |
| npdcch-Offset-RA-r14 | zero |  |  |
| nprach-NumCBRA-StartSubcarriers-r14 | n8 |  |  |
| npdcch-CarrierIndex-r14 | 3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| pagingWeightAnchor-r14 | w1 |  |  |
| } |  |  |  |

Table 22.3.1.12.3.3-5: DL-CarrierConfigCommon-NB-DEFAULT1

| Derivation Path: 36.508 clause 8.1.6.3-1A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DL-CarrierConfigCommon-NB -DEFAULT::= SEQUENCE { |  |  |  |
| dl-CarrierFreq-r14 | Downlink EARFCN for non-anchor carrier under test. For Signalling test cases, see 36.508 clauses 8.3.2.3. Use the anchor carrier EARFCN + 2. |  |  |
| } |  |  |  |

Table 22.3.1.12.3.3-6: SystemInformationBlockType22-NB (Step 1)

| Derivation Path: 22.3.1.12.3.3-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType22-NB-r14 ::= SEQUENCE { |  |  |  |
| nprach-ProbabilityAnchorList-r14 SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF NPRACH-ProbabilityAnchor-NB-r14 { | 3 entries |  |  |
| nprach-ProbabilityAnchor-r14[1] | zero | entry 1 CE level 0 |  |
| nprach-ProbabilityAnchor-r14[2] | zero | entry 2 CE level 1 |  |
| nprach-ProbabilityAnchor-r14[3] | zero | entry 3 CE level 2 |  |
| } |  |  |  |
| } |  |  |  |

Table 22.3.1.12.3.3-7: SystemInformationBlockType23-NB

| Derivation Path: 36.508 Table 8.1.4.3.3-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType23-NB-r15 ::= SEQUENCE { |  |  |  |
| ul-ConfigList-v1530 SEQUENCE (SIZE (1.. maxNonAnchorCarriers-NB-r14)) OF UL-ConfigCommon-NB-v1530 { | 1 entry |  |  |
| UL-ConfigCommon-NB-v1530[1] SEQUENCE { |  | entry 1 |  |
| nprach-ParametersListFmt2-r15 SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF NPRACH-ParametersFmt2-NB-r15 { | 3 entries |  |  |
| NPRACH-ParametersFmt2-NB-r15[1] SEQUENCE { |  | entry 1 CE level 0 |  |
| nprach-Parameters-r15 SEQUENCE { |  |  |  |
| nprach-Periodicity-r15 | ms640 |  |  |
| nprach-StartTime-r15 | ms10 |  |  |
| nprach-SubcarrierOffset-r15 | n12 |  |  |
| nprach-NumSubcarriers-r15 | n32 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r15 | oneThird |  |  |
| npdcch-NumRepetitions-RA-r15 | r16 |  |  |
| npdcch-StartSF-CSS-RA-r15 | v4 |  |  |
| npdcch-Offset-RA-r15 | zero |  |  |
| nprach-NumCBRA-StartSubcarriers-r15 | n24 |  |  |
| npdcch-CarrierIndex-r15 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| NPRACH-ParametersFmt2-NB-r15[2] SEQUENCE { |  | entry 2 CE level 1 |  |
| nprach-Parameters-r15 SEQUENCE { |  |  |  |
| nprach-Periodicity-r15 | ms640 |  |  |
| nprach-StartTime-r15 | ms40 |  |  |
| nprach-SubcarrierOffset-r15 | n12 |  |  |
| nprach-NumSubcarriers-r15 | n32 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r15 | oneThird |  |  |
| npdcch-NumRepetitions-RA-r15 | r16 |  |  |
| npdcch-StartSF-CSS-RA-r15 | v4 |  |  |
| npdcch-Offset-RA-r15 | zero |  |  |
| nprach-NumCBRA-StartSubcarriers-r15 | n24 |  |  |
| npdcch-CarrierIndex-r15 | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| NPRACH-ParametersFmt2-NB-r15[3] SEQUENCE { |  | entry 3 CE level 2 |  |
| nprach-Parameters-r15 SEQUENCE { |  |  |  |
| nprach-Periodicity-r15 | ms640 |  |  |
| nprach-StartTime-r15 | ms160 |  |  |
| nprach-SubcarrierOffset-r15 | n12 |  |  |
| nprach-NumSubcarriers-r15 | n32 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r15 | oneThird |  |  |
| npdcch-NumRepetitions-RA-r15 | r16 |  |  |
| npdcch-StartSF-CSS-RA-r15 | v4 |  |  |
| npdcch-Offset-RA-r15 | zero |  |  |
| nprach-NumCBRA-StartSubcarriers-r15 | n24 |  |  |
| npdcch-CarrierIndex-r15 | 3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |