3GPP TSG-RAN5 Meeting #96-e	R5-224477

# Electronic Meeting, 15th – 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-5 | CR | 0217 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to the GNSS scenarios |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | GPS28 needs to be simulated, according to TS 37.571-5. However, it is not present in the assistance data file Almanac file for QZSS is missing QZSS rinex file is missing the UTC model |  |  |  |  |  |  |  |  |
| Summary of change: |  | Attached a new assistance data file with SVID28 present Added the missing almanac file and UTC model |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The GNSS scenarios cannot be properly simulated |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Note to the editor: Please remove: GNSS_Orbital_Data_Perf_V2.zip and GNSS_Orbital_Data_Sig_V2.zip from the attachments and add GNSS_Orbital_Data_Perf_V3.zip and GNSS_Orbital_Data_Sig_V3.zip |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Note to the editor:

# Please remove: GNSS_Orbital_Data_Perf_V2.zip and GNSS_Orbital_Data_Sig_V2.zip from the attachments and add GNSS_Orbital_Data_Perf_V3.zip and GNSS_Orbital_Data_Sig_V3.zip

# < Unchanged sections omitted >

###### 6.2.1.2.3.2	GNSS Scenario #3B

Almanac data: Sig GNSS NAVIC 2020_9_17 Almanac.txt

Ephemeris data: Ephemeris data: Sig GNSS GPS 2020_9_17 Rinex.rnx, Sig GNSS GLONASS 2020_9_17 Rinex.rnx, Sig GNSS GALILEO 2020_9_17 Rinex.rnx, Sig GNSS BDS 2020_9_17 Rinex.rnx, Sig GNSS NAVIC 2020_9_17 Rinex.rnx

UE location: the UE location is calculated as a random offset from the reference location using the method described in subclause 6.2.1.2.6. The reference location is: latitude: 10 degrees 44 minutes 0 seconds north, longitude: 79 degrees 40 minutes 0 seconds east, (Sigar India), height:= 300m.

Nominal start time: as for GNSS scenario #1.

Viable running time to maintain specified requirements: as for GNSS scenario #1.

Satellite meeting specified requirements to be used for simulation and for which Assistance Data (other than Almanac) shall be generated:

Navic:  PRN: 2, 3, 5, 6, 9.

GPS: PRN: 2, 19, 28

Galileo: tbd

Glonass: tbd

Beidou: tbd

###### 6.2.1.2.3.3	QZSS Scenario #1

Almanac data: Sig GNSS QZSS 2020_9_17 Almanac.txt.

Ephemeris data: Sig GNSS QZSS 2020_9_17 Rinex.rnx.

UE location: as for GNSS scenario #1.

Nominal start time: as for GNSS scenario #1.

Viable running time to maintain specified requirements: as for GNSS scenario #1.

Satellite meeting specified requirements to be used for simulation and for which Assistance Data (other than Almanac) shall be generated: PRN 193, 194, 195, 199.

###### 6.2.1.2.3.4	WAAS Scenario

Satellite positions: (PRN 135)133.0 degrees west, height: 35786037.417m, (PRN 138)107.3 degrees west, height: 35786037.417m.

UE location: as for related GNSS scenario.

Satellite used for simulation: PRN 135.

###### 6.2.1.2.3.5	EGNOS Scenario

Satellite positions: (PRN 120)15.5 degrees west, height: 35786037.417m, (PRN 124) 21.5 degrees west, height: 35786037.417m.

UE location: as for related GNSS scenario.

Satellite used for simulation: PRN 120.

###### 6.2.1.2.3.6	MSAS Scenario

Satellite positions: (PRN 129)140.0 degrees east, height: 35786037.417m, (PRN 137)145 degrees east, height: 35786037.417m

UE location: as for related GNSS scenario.

Satellite used for simulation: PRN 129.

###### 6.2.1.2.3.7	GAGAN Scenario

Satellite positions: (PRN 127)55.0 degrees east, height: 35786037.417m, (PRN 128)83 degrees east, height: 35786037.417m, (PRN 132)93.5 degrees east, height: 35786037.417m

UE location: as for related GNSS scenario.

Satellite used for simulation: PRN 128.

##### 6.2.1.2.4	GNSS Scenario #4

The following GNSS scenario #4 shall be used during the Nominal Accuracy test defined in TS 37.571-1 [6] subclauses 6, 7 and 13. The assistance data specified in the following subclauses for GNSS scenario #4 is consistent with this GNSS scenario.

The scenario used varies dependent on the SBAS supported by the UE and also whether QZSS is supported. The scenario to be used is defined below. Where more than one SBAS is supported use the scenario for MSAS if MSAS and QZSS are supported, otherwise use the scenario for the first supported SBAS in the list.

Table 6.2.1.2.4-1: Scenarios used for Scenario #4

| SBAS supported | Scenarios used |  |
| --- | --- | --- |
|  | UE supports QZSS | UE does not support QZSS |
| None | GNSS Scenario #4D with QZSS Scenario #2 | GNSS Scenario #2 |
| WAAS | [FFS] | GNSS Scenario #4C with WAAS |
| EGNOS | [FFS] | GNSS Scenario #4A with EGNOS |
| MSAS | GNSS Scenario #4D with QZSS Scenario #2 and MSAS | GNSS Scenario #4D with MSAS |
| GAGAN | [FFS] | GNSS Scenario #4B with GAGAN |

###### 6.2.1.2.4.1	GNSS Scenario #4A

[FFS]

###### 6.2.1.2.4.2	GNSS Scenario #4B

Almanac data: Sig GNSS NAVIC 2020_9_17 Almanac.txt

Ephemeris data: Ephemeris data: Sig GNSS GPS 2020_9_17 Rinex.rnx, Sig GNSS GLONASS 2020_9_17 Rinex.rnx, Sig GNSS GALILEO 2020_9_17 Rinex.rnx, Sig GNSS BDS 2020_9_17 Rinex.rnx, Sig GNSS NAVIC 2020_9_17 Rinex.rnx

UE location: the UE location is calculated as a random offset from the reference location using the method described in subclause 6.2.1.2.6. The reference location is: latitude: 3 degrees 56 minutes 31.668 seconds north, longitude: 73 degrees 29 minutes 26.0376 seconds east, (Maledives), height: 50m.

Nominal start time: as for GNSS scenario #1.

Viable running time to maintain specified requirements: as for GNSS scenario #1.

Satellite meeting specified requirements to be used for simulation and for which Assistance Data (other than Almanac) shall be generated:

Navic: PRN: 2, 3, 5, 6, 9

GPS: PRN: 2, 19, 28

Galileo: tbd

Glonass: tbd

Beidou: tbd

###### 6.2.1.2.4.3	GNSS Scenario #4C

[FFS]

###### 6.2.1.2.4.4		GNSS Scenario #4D

Almanac data: FFS.

Ephemeris data: Sig GNSS GPS 2020_9_17 Rinex.rnx, Sig GNSS GLONASS 2020_9_17 Rinex.rnx, Sig GNSS GALILEO 2020_9_17 Rinex.rnx, Sig GNSS BDS 2020_9_17 Rinex.rnx

UE location: the UE location is calculated as a random offset from the reference location using the method described in subclause 6.2.1.2.6. The reference location is: latitude: 34 degrees 59 minutes 20.1818 seconds north, longitude: 135 degrees 45 minutes 34.884 seconds east, (Kyoto, Japan), height: = 50m.

Nominal start time: as for GNSS scenario #1.

Visible satellites: as for GNSS scenario #1.

Simulated satellites: as for GNSS scenario #1.

###### 6.2.1.2.4.5	QZSS Scenario #2

Almanac data: Sig GNSS QZSS 2020_9_17 Almanac.txt.

Ephemeris data: Sig GNSS QZSS 2020_9_17 Rinex.rnx

UE location: as for GNSS scenario #4D.

Nominal start time: as for GNSS scenario #4D.

Viable running time to maintain specified requirements: as for GNSS scenario #4D.

Satellite meeting specified requirements to be used for simulation and for which Assistance Data (other than Almanac) shall be generated: PRN 193, 194, 195, 199

< End of changes >