3GPP TSG-RAN5 Meeting #96-e	R5-224522

# Electronic Meeting, 15th – 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.903 | CR | 0342 | rev | - | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TT analysis for NR SL RRM TC 9.1.1.2 - SyncRef UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To provide TT analysis for NR SL RRM TC 9.1.1.2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - TT analysis for NR SL RRM TC 9.1.1.2 is provided in attached .zip file. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 CR 1880, 1893 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

Add the following attached .zip file to TR 38.903:

"38.533 9.1.1.2 TT.zip "

<End of modified section 1>