# Title:	Test Tolerances for synchronization reference selection/reselection for GNSS configured as the highest priority synchronization reference source tests

Source:	Huawei, Hisilicon

# 1.	Introduction

The purpose of this document is to present the methodology and principles used for the development of Test Tolerances for synchronization reference selection/reselection for GNSS configured as the highest priority synchronization reference source tests. The test cases in question, from TS 38.533, are:

- 9.1.3.1 NR SA FR1 synchronization reference selection/reselection for GNSS configured as the highest priority synchronization reference source

# 2. 	Test case in TS 38.133

The test conditions are defined in the following extracts from TS 38.133 v16.12.0. Changes to be made in RAN4 #104 is yellow highlighted:

#### A.9.1.3.1	Test for GNSS configured as the highest priority

##### A.9.1.3.1.1	Test Purpose and Environment

The purpose of this test is to verify the requirements related to SyncRef UE selection / reselection defined in clause 12.4, when GNSS is configured as the highest priority. For this test, the UE is triggered by the test loop function or the upper layers to transmit for V2X Sidelink Communication.

The test parameters are given in Table A.9.1.3.1.1-1and A.9.1.3.1.1-2 below. There are no GNSS signals in this test. There are three active SyncRef UEs (SyncRef UE 1, SyncRef UE 2 and SyncRef UE 3) in this test. The test system shall emulate SyncRef UE 1, SyncRef UE 2 and SyncRef UE 3 to transmit S-SSB every S-SSB period.

The test system can verify the selection / reselection of SyncRef UE by monitoring the SLSS ID used by the V2X UE for its S-SSB transmissions. When the V2X UE is not synchronized to any SyncRef UE, then the V2X UE shall use the SLSS ID belonging to set id_oon. When the V2X UE is synchronized to a SyncRef UE, the V2X UE shall derive its SLSS ID from the SLSS ID of the SyncRef UE as per clause 5.8.5.3 of TS 38.331[2].

The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. SyncRef UE 1, SyncRef UE 2 and SyncRef UE 3 are all powered off before starting the test. During T1, SyncRef UE 1 is powered ON and the V2X UE will select SyncRef UE 1 as synchronization source. During T2, SyncRef UE 2 is powered ON and the V2X UE will select SyncRef UE 2 as the synchronization source. During T3, SyncRef UE 3 is powered ON and the V2X UE will reselect to SyncRef UE 3 as the synchronization source.

Table A.9.1.3.1.1-1: Test Parameters for V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| SCS |  | kHz | 30 |  |
| Initial condition | Active synchronization source |  | Sync Ref UE 1 | DUT transmits for V2X Sidelink Communication and S-SSB with SLSS ID = 30 and in-coverage set as FALSE in MIB-SL. |
| T2 end condition | Active synchronization source |  | Sync Ref UE 2 | DUT transmits for V2X Sidelink Communication and S-SSB with SLSS ID = 336 and in-coverage set as FALSE in MIB-SL. |
| Final condition | Active synchronization source |  | Sync Ref UE 3 | UE transmits for V2X Sidelink Communication and S-SSB with SLSS ID = 0 and in-coverage set as FALSE in MIB-SL. |
| Active SyncRef UEs |  |  | SyncRef UE 1 SyncRef UE 2 SyncRef UE 3 | Transmitting S-SSB on RF channel number 1 (HD carrier in Band n47 or n38) |
| Timing offset among SyncRef UEs |  | s | CP/2 | Synchronous |
| Frequency offset of SyncRef UE 1,2,3 |  | ppm | 0 |  |
| V2X sidelink Communication configuration |  |  | As specified in Table A. 3.21.2-2 | IE values unless specified otherwise in this test. |
| sl-SyncPriority |  |  | gnss |  |
| syncTxThreshOoC |  |  | +infinity |  |
| T1 |  | s | 24 |  |
| T2 |  | s | 16 |  |
| T3 |  | s | 3.2 |  |

Table A.9.1.3.1.1-2: SyncRef UE Specific Test Parameters for V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority

| Parameter | Unit | SyncRef UE 1 |  |  |  |  | SyncRef UE 2 |  |  |  |  |  | SyncRef UE 3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 |  | T3 |  | T1 |  | T2 |  | T3 |  | T1 |  | T2 |  | T3 |  |
| NR RF Channel Number |  | 1(HD carrier in Band n47 or n38) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Channel Bandwidth (BWchannel) Note 4 | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| V2X Sidelink Communication resource pool configuration |  | As specified in Table A.3.21.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| networkControlledSyncTx |  | ON |  |  |  |  | N/A |  |  |  |  |  | ON |  |  |  |  |  |
| syncTxThreshOoC | dBm/15 kHz | N/A |  |  |  |  | +infinity |  |  |  |  |  | N/A |  |  |  |  |  |
| SLSSID |  | 30 |  |  |  |  | 0 |  |  |  |  |  | 0 |  |  |  |  |  |
| inCoverage (in MIB-SL) |  | TRUE |  |  |  |  | FALSE |  |  |  |  |  | TRUE |  |  |  |  |  |
| Note1 | dBm/30 kHz | -95 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | dB | 0 |  | 0 |  | 0 |  | -infinity |  | 0 |  | 0 |  | -infinity |  | -infinity |  | 3 |
| Note2, | dB | 0 |  | 0 |  | -4.76 |  | -infinity |  | 0 |  | 0 |  | -infinity |  | -infinity |  | 0 |
| Io Note2, | dBm/18 MHz | -70.78 |  | -70.78 |  | -67.78 |  | -73.79 |  | -70.78 |  | -70.78 |  | -70.78 |  | -70.78 |  | -67.78 |
|  | dBm/36 MHz | -70.78 |  | -70.78 |  | -67.78 |  | -73.79 |  | -70.78 |  | -70.78 |  | -70.78 |  | -70.78 |  | -67.78 |
| PSBCH-RSRPNote2, Note 3 | dBm/30 kHz | -95 |  | -95 |  | -95 |  | -infinity |  | -95 |  | -95 |  | -infinity |  | -infinity |  | -92 |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: Es/Iot, Io, PSBCH-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: S-PSS Es/Iot and S-SSS Es/Iot are set the same as PSBCH Es/Iot. Note 4: The UE is only required to be tested in one of the supported test configurations. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### A.9.1.3.1.2	Test Requirements

During T1, SyncRef UE selection delay is defined as the time from the beginning of T1 to the time UE is synchronized to SyncRef UE 1, and changes its S-SSB transmissions timing and SLSS ID to follow SyncRef UE 1 as the synchronization source. For the test configuration, the SLSS ID will be changed to 30 (with in-coverage IE in MIB-SL set to FALSE) after SyncRef UE selection delay from start of T1.

The SyncRef UE selection delay shall be less than 8.8sec. The SyncRef UE selection delay can be expressed as:

SyncRef UE selection delay = Tdetect,SyncRef UE + Tevaluate,SLSS + S-SSB period

Where

-	Tdetect,SyncRef UE = 8sec (as specified in sub-clause 12.4)

-	Tevaluate,SLSS = 0.64 sec (as specified in sub-clause 12.3)

-	S-SSB period = 160ms

This gives a total of 8.8 seconds.

2) During T2, SyncRef UE reselection delay is defined as the time from the beginning of T2 to the time UE changes its synchronization source from SyncRef UE 1 to SyncRef UE 2 and changes its S-SSB transmissions timing and SLSS ID to follow SyncRef UE 2 as the synchronization source. For the test configuration, the SLSS ID will be changed to 336 (with in-coverage IE in MIB-SL set to FALSE) after SyncRef UE reselection delay from start of T2.

The SyncRef UE reselection delay shall be less than 8.8sec. The SyncRef UE reselection delay can be expressed as:

SyncRef UE reselection delay = Tdetect,SyncRef UE + Tevaluate,SLSS + S-SSB period

Where

-	Tdetect,SyncRef UE = 8sec (as specified in sub-clause 12.4)

-	Tevaluate,SLSS = 0.64 (as specified in sub-clause 12.3)

-	S-SSB period = 160ms

This gives a total of 8.8 seconds.

3) During T3, SyncRef UE reselection delay is defined as the time from the beginning of T3 to the time UE changes its synchronization source from SyncRef UE 2 to SyncRef UE 3, and changes its S-SSB transmissions timing and SLSS ID to follow SyncRef UE 3 as the synchronization source. For the test configuration, the SLSS ID will still be 0 (with in-coverage IE in MIB-SL set to FALSE) after SyncRef UE reselection delay from start of T3.

The SyncRef UE reselection delay shall be less than 2.4sec. The SyncRef UE reselection delay can be expressed as:

SyncRef UE reselection delay = Tdetect,SyncRef UE + Tevaluate,SLSS + S-SSB period

Where

-	Tdetect,SyncRef UE = 1.6sec (as specified in sub-clause 12.4)

-	Tevaluate,SLSS = 0.64 (as specified in sub-clause 12.3)

-	S-SSB period = 160 ms

This gives a total of 2.4 seconds.

The test system will verify that the V2X UE does not drop or delay more than 6% of its V2X data and S-SSB transmissions during the duration of T2, and does not drop or delay more than 30% of its S-SSB transmissions during the duration of T3.

The rate of correct SyncRef UE selection / reselection observed during repeated tests shall be at least 90%.

# 3. 	Discussion

Before the test formally begins, the UE under test is not sync-ed to any external synchronization reference source. Thus it shall use its internal clock as the synchronization reference source.

During T1, SyncRef UE 1 is powered on. The UE is expected to detect and select SyncRef UE 1 as the synchronization reference source. Then the PSBCH-RSRP measurement result of SyncRef UE 1 shall be higher than the minimum requirement given in 38.133 by sl-SyncRefMinHyst according to 38.331 clause 5.8.6.2:

2>	if the UE has not selected any synchronization reference:

3>	if the UE detects one or more SLSSIDs for which the PSBCH-RSRP exceeds the minimum requirement defined in TS 38.133 [14] by sl-SyncRefMinHyst and for which the UE received the corresponding MasterInformationBlockSidelink message (candidate SyncRef UEs), or if the UE detects GNSS that is reliable in accordance with TS 38.101-1 [15] and TS 38.133 [14], or if the UE detects a cell, select a synchronization reference according to the following priority group order:

...

The "minimum requirement defined in 38.133" referred by 38.331 is given in 38.133 Annex B.4.3:

## B.4.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE

This clause defines the S-SSB_RP and S-SSB Ês/Iot applicable for a corresponding operating band.

The conditions for selection/reselection to intra-frequency SyncRef UE are defined in Table B.4.3-1 for FR1.

Table B.4.3-1: V2X synchronization measurements in FR1

| Parameter | NR V2X operating band groups Note1 | Minimum S-SSB_RP |  |  | S-SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/SCSS-SSB |  |  | dB |
|  |  | SCSS-SSB = 15kHz | SCSS-SSB = 30kHz | SCSS-SSB = 60kHz |  |
|  | NR_TDD_FR1_B | -120.5 | -117.5 | -114.5 | 0 |
|  | NR_TDD_FR1_J | -116.5 | -113.5 | -110.5 | 0 |
| NOTE 1: NR V2X operating band groups are as defined in Section 3.5 for the corresponding NR operating bands. NOTE 2: The SyncRef UE transmission frequency shall be accurate to within ±5 PPM compared to the absolute frequency. |  |  |  |  |  |

During T2, SyncRef UE 2 is powered on. Because SyncRef UE 2 belongs to the higher priority group than SyncRef UE 1, the UE is expected to detect and reselect SyncRef UE 2 as the synchronization reference source. Then the PSBCH-RSRP measurement result of SyncRef UE 2 shall be higher than the minimum requirement given in 38.133 by sl-SyncRefMinHyst according to 38.331 clause 5.8.6.2.

Similarly, during T3 SyncRef UE 3 is powered on which belongs to the higher priority group than SyncRef UE 2. The UE is expected to detect and reselect SyncRef UE 3 as the synchronization reference source. Then the PSBCH-RSRP measurement result of SyncRef UE 3 shall be higher than the minimum requirement given in 38.133 by sl-SyncRefMinHyst according to 38.331 clause 5.8.6.2.

With nominal signal levels the condition for SyncRef UE (re)selection is easily met. but the effect of uncertainties on the test verdict needs to be checked. A decision can then be made on whether Test Tolerances need to be applied.

# 4. 	Calculation of Test Tolerances

Two test configurations are given in test case A.9.1.3.1 (20 MHz CBW and 40 MHz CBW). Analysis given in this clause are based on the test configuration of 20 MHz CBW.

General approach

The general approach is given in the steps below:

- Copy the originally specified key parameters from the core requirements

- Where relevant, calculate derived parameters from the core requirements

- Define uncertainties for a minimum set of parameters

- Define controlled parameters (critical to the test verdict), calculate sensitivity factors and uncertainty

- Determine which original or derived parameters to offset (apply Test Tolerances to) and by how much

- Recalculate original or derived parameters including Test Tolerances

- Check that the controlled parameters meet requirements to get the correct test verdict

Each step is explained below, and the calculations are given in the accompanying spreadsheet.

a) Original specified key parameters

The key parameters are copied from 38.133 Table A.9.1.3.1.1-1 and A.9.1.3.1.1-2. The key parameters are selected as the minimum set to define the cell power levels, and which are subject to a test system uncertainty which may affect the verdict of the test.

b) Derived parameters

A number of derived parameters are calculated, using the base information in a). The reason for deriving each additional parameter is given in the “Comment” column of section b) in the accompanying spreadsheet.

In this test case the UE is expected to select SyncRef UE belonging to higher priority group as the synchronization reference source. Hence UE makes PSBCH-RSRP measurements of SyncRef UEs and compares them with the minimum requirements given in 38.133. A further step is done to calculate PSBCH-RSRP power difference:

- (measured SyncRef UE PSBCH-RSRP - sl-SyncRefMinHyst - minimum requirement)

For sake of simplify, we set sl-SyncRefMinHyst to 0dB in message contents. Then the PSBCH-RSRP power difference is simplified as

- (measured SyncRef UE PSBCH-RSRP - minimum requirement)

c) Uncertainties

The SS provides AWGN, 3 SyncRef UEs to the UE. We propose to control the following parameters:

- AWGN Absolute power averaged over BWConfig, Noc ±1.5 dB

- AWGN Absolute power averaged over measurement PRB, Noc ±1.5 dB

- Ratio of SyncRef UE 1 signal to noise averaged over BWConfig, Ês1 / Noc ±0.3 dB

- Ratio of SyncRef UE 1 signal to noise averaged over measurement PRB, Ês1 / Noc ±0.3 dB

- Ratio of SyncRef UE 2 signal to noise averaged over BWConfig, Ês2 / Noc ±0.3 dB

- Ratio of SyncRef UE 2 signal to noise averaged over measurement PRB, Ês2 / Noc ±0.3 dB

- Ratio of SyncRef UE 3 signal to noise averaged over BWConfig, Ês3 / Noc ±0.3 dB

- Ratio of SyncRef UE 3 signal to noise averaged over measurement PRB, Ês3 / Noc ±0.3 dB

This choice forms a minimum set, so the superposition principle can be applied if necessary.

In addition, the UE PSBCH-RSRP absolute measurement accuracy needs to be considered, because when the UE compares SyncRef UE 1 PSBCH-RSRP against with SyncRef UE selection minimum requirement and syncTxThreshOoC, it has some inaccuracy which could affect the test verdict. The UE measurement accuracy is defined in the following extract from TS 38.133:

10.4.2.1	PSBCH-RSRP Absolute Accuracy

The requirements for absolute accuracy of PSBCH-RSRP in this clause apply to a V2X synchronization source on the same frequency as that of the own V2X UE performing the measurement in FR1.

The accuracy requirements in Table 10.4.2.1-1 are valid under the following conditions:

-	Demodulation reference signals are transmitted from one port.

-	Conditions defined in Clause 7.3E of TS38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for PSBCH-RSRP measurements are fulfilled according to Annex B.4.2 for a corresponding Band for each relevant PSBCH-DMRS.

Table 10.4.2.1-1: Intra-frequency PSBCH-RSRP absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR V2X operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSL |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSL = 15 kHz | SCSSL = 30 kHz | SCSSL = 60 kHz |  |  |
| 4.5 | 9 | -6 | NR_TDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_J | -116.5 | -113.5 | -110.5 | N/A | -70 |
| 8 | 11 | -6 | NR_TDD_FR1_B, NR_TDD_FR1_J | N/A | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR V2X operating band groups in FR1 are as defined in clause 3.5.2. NOTE 3: Ês/Iot for a SyncRef UE is the Ês/Iot of PSBCH-DMRS. |  |  |  |  |  |  |  |  |

Band group NR_TDD_FR1_J has the tightest Io requirement. which is no less than -113.5dBm/30kHz and no bigger than -50dBm/Ch BW. To ensure PSBCH-RSRP absolute measurement accuracy, the conditions of S-SSB_RP and S-SSB Es/Iot in Annex B.4.2 need to be also fulfilled.

## B.4.2	Conditions for PSBCH-RSRP Accuracy Requirements

This clause defines the following conditions for PSBCH-RSRP measurement accuracy requirements applicable for a corresponding operating band.

The conditions are defined in Table B.4.2-1 for FR1.

Table B.4.2-1: Conditions for PSBCH-RSRP measurements in FR1

| Parameter | NR V2X operating band groups Note1 | Minimum S-SSB_RP |  |  | S-SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/SCSS-SSB |  |  | dB |
|  |  | SCSS-SSB = 15kHz | SCSS-SSB = 30kHz | SCSS-SSB = 60kHz |  |
|  | NR_TDD_FR1_B | -126.5 | -123.5 | -120.5 | -6 |
|  | NR_TDD_FR1_J | -122.5 | -119.5 | -116.5 |  |
| NOTE 1: NR V2X operating band groups are as defined in Section 3.5 for the corresponding NR operating bands. |  |  |  |  |  |

In summary,

- Io shall be no less than -113.5dBm/30kHz and no bigger than -50dBm/Ch BW;

- S-SSB_RP of SyncRef UE 1 in T1 shall be no less than -113.5 dBm/30kHz;

- S-SSB_RP of SyncRef UE 2 in T2 shall be no less than -113.5 dBm/30kHz;

- S-SSB_RP of SyncRef UE 3 in T3 shall be no less than -113.5 dBm/30kHz;

- S-SSB Es/Iot of SyncRef UE 1 in T1 shall be no less than 0 dB.

- S-SSB Es/Iot of SyncRef UE 2 in T2 shall be no less than 0 dB.

- S-SSB Es/Iot of SyncRef UE 3 in T3 shall be no less than 0 dB.

- measured SyncRef UE 1 PSBCH-RSRP in T1 shall be no less than -113.5dBm/30kHz.

- measured SyncRef UE 2 PSBCH-RSRP in T2 shall be no less than -113.5dBm/30kHz.

- measured SyncRef UE 3 PSBCH-RSRP in T3 shall be no less than -113.5dBm/30kHz.

d) Controlled parameters critical to verdict

The controlled parameters listed in the accompanying spreadsheet have been derived by study of the test case and the relevant clauses in TS 38.133.

Having identified the parameters which need to be controlled, we now need to consider how they are affected by the parameters which can be set by the test equipment. This is done by working out “sensitivity factors”. A sensitivity factor is just the ratio (effect on a critical parameter y / a test equipment uncertainty x), and is usually in dB/dB.

Denoting critical parameter y as a function of test equipment uncertainties x1, x2, ..., i.e. y=fx1,x2,⋯. Then sensitivity factors of xi on y can be expressed as:

sensitivity factors of xi on y=∂y∂xi

Having filled in the matrix of sensitivity factors, the accompanying spreadsheet calculates the overall uncertainty for each controlled parameter, taking into account the uncertainties and sensitivity factors for each parameter that can be set by the test equipment, and the UE measurement accuracies where relevant. This process follows the superposition principle.

For the test system uncertainties the normal procedure of combining uncorrelated uncertainties root-sum-square is followed.

When the test system uncertainties and the UE measurement accuracies are combined, the (root-sum square of test system uncertainties) is added arithmetically to the UE measurement accuracies. If all the uncertainties were combined root-sum square, the resulting smaller test limits could case a conformant test system to fail a conformant UE, which would be unacceptable.

e) Determine parameters to offset

Nominal value of SyncRef UE 1 Es/Iot in T1 is 0dB, minimum Es/Iot requirement is satisfied with 0 dB margin. However, the overall uncertainty of Es/Iot is ±0.3 dB as calculated in step d) of the accompanying spreadsheet. After including uncertainty the minimum Es/Iot requirement may not be satisfied. We suppose to raise r SyncRef UE 1 Es/Iot in T1 by an amount of 0.35dB to ensure minimum Es/Iot requirement is satisfied.

Nominal value of SyncRef UE 2 Es/Iot in T2 is 0dB, minimum Es/Iot requirement is satisfied with 0 dB margin. However, the overall uncertainty of Es/Iot is ±0.3 dB as calculated in step d) of the accompanying spreadsheet. After including uncertainty the minimum Es/Iot requirement may not be satisfied. We suppose to raise r SyncRef UE 2 Es/Iot in T2 by an amount of 0.35dB to ensure minimum Es/Iot requirement is satisfied.

Nominal value of SyncRef UE 3 Es/Iot in T3 is -0.1dB, minimum Es/Iot requirement is not satisfied. Furthermore, The overall uncertainty of Es/Iot (±0.3 dB) as calculated in step d) of the accompanying spreadsheet should also be considered. We suppose to raise SyncRef UE 3 Es/Iot in T3 by an amount of 0.35dB to ensure minimum Es/Iot requirement is satisfied.

The check that all other controlled parameters meet their required range is done in step g). In theory it is possible for steps e) to g) to be iterative, or possibly even steps c) to g) to be iterative, but this test case can be done without such iteration.

f) Parameters modified by Test Tolerances

Based on the decision in e), the set of parameters in a) and b) is reproduced in section f) of the accompanying spreadsheet, but this time modified by the Test Tolerances (applied offsets).

Re-derived parameters are calculated using the same methods as were used in step b). In general all the values are listed for each time interval T1 and T2 but as NR Cell 3 is not present during T1 those entries are greyed out (the formulae are left in place for use by future test cases).

g) Check controlled parameters Min/Max

Using a format similar to that in step d), the nominal value of each controlled parameter is recalculated.

The minimum and maximum values, due to variability from uncertainties, of controlled parameters is then calculated and compared against the requirements. It is not necessary to calculate all parameters, so a selection is made of those critical to the test verdict. The critical requirement for each parameter is given briefly in the “Comment” column of section g) in the accompanying spreadsheet. The cases closest to limit are identified by turquoise cells in the spreadsheet. If all requirements are met, then the exercise is complete.

It can be seen that with the uncertainty values and Test Tolerances proposed, all the requirements are met.

# 5. Treatment of other configurations within the same test case

TC 9.1.3.1 also contains configurations with different channel bandwidths. However, difference on CBW has no impact on TT analysis since No PSCCH/PSSCH RMC is configured in this TC. So the analysis based on 20MHz CBW given in clause 4 also applies.