3GPP TSG-RAN5 Meeting #96-e	R5-224592

# Online, ,  15th Aug 2022 - 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2474 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to message contents for slice specific RACH configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_slice-UEConTest |  |  |  |  |  | Date: |  |  | 2022-07-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The information elements related to Rel-17 slice specific RACH configuration is missing in TS 38.508-1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added following new information elements for slice specific RACH configuration Table 4.6.3-56B: FeatureCombination Table 4.6.3-56C: FeatureCombinationPreambles Table 4.6.3-131A: RA-PrioritizationForSlicing-r17 - Updated following information elements to add slice specific RACH configuration - Table 4.6.3-14: BWP-UplinkCommon - Table 4.6.3-128: RACH-ConfigCommon - Table 4.6.3-131: RA-Prioritization |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

4.6.3	Radio resource control information elements

< Unchanged sections omitted >

–	BWP-UplinkCommon

Table 4.6.3-14: BWP-UplinkCommon

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkCommon ::= SEQUENCE { |  |  |  |
| genericParameters | BWP |  |  |
| rach-ConfigCommon CHOICE { |  |  |  |
| setup | RACH-ConfigCommon |  |  |
| } |  |  |  |
| rach-ConfigCommon | Not present |  | SUL_SUL AND (RF OR RRM) |
| pusch-ConfigCommon CHOICE { |  |  |  |
| setup | PUSCH-ConfigCommon |  |  |
| } |  |  |  |
| pucch-ConfigCommon CHOICE { |  |  |  |
| setup | PUCCH-ConfigCommon |  |  |
| } |  |  |  |
| enableRA-PrioritizationForSlicing-r17 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SUL_SUL | On the SUL carrier when supplementary carrier is configured |

< Unchanged sections omitted >

–	DRX-ConfigSecondaryGroup

Table 4.6.3-56A: DRX-ConfigSecondaryGroup

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DRX-ConfigSecondaryGroup ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

–	FeatureCombination

Table 4.6.3-56B: FeatureCombination

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FeatureCombination-r17 ::= SEQUENCE { |  |  |  |
| redCap | Not present |  |  |
| smallData | Not present |  |  |
| nsag ::= SEQUENCE (SIZE (1..maxSliceInfo-r17)) OF NSAG-ID-r17{ |  |  | Slice_RACH |
| NSAG-ID-r17[n] | Set to the corresponding NSAG value used in the test case | entry n |  |
| } |  |  |  |
| covEnh | Not present |  |  |
| laterThanRel17Features | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Slice_RACH | Slice specific RACH configuration |

–	FeatureCombinationPreambles

Table 4.6.3-56C: FeatureCombinationPreambles

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FeatureCombinationPreambles-r17 ::= SEQUENCE { |  |  |  |
| featureCombination-r17 | FeatureCombination-r17 |  |  |
| startPreambleForThisPartition-r17 | FFS |  |  |
| numberOfPreamblesForThisPartition-r17 | FFS |  |  |
| ssb-SharedRO-MaskIndex-r17 | Not present |  |  |
| numberOfRA-PreamblesGroupA-r17 | Not present |  |  |
| separateMsgA-PUSCH-Config-r17 | Not present |  |  |
| featureSpecificParameters-r17 | Not present |  |  |
| } |  |  |  |

–	FilterCoefficient

Table 4.6.3-57: FilterCoefficient

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FilterCoefficient | fc4 |  |  |

< Unchanged sections omitted >

–	RACH-ConfigCommon

Table 4.6.3-128: RACH-ConfigCommon

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon ::= SEQUENCE { |  |  |  |
| rach-ConfigGeneric | RACH-ConfigGeneric |  |  |
| totalNumberOfRA-Preambles | Not present |  |  |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB CHOICE { |  |  |  |
| one | n8 |  | FR1 |
|  | n4 |  | FR2 |
| } |  |  |  |
| groupBconfigured | Not present |  |  |
| ra-ContentionResolutionTimer | sf64 |  |  |
| rsrp-ThresholdSSB | RSRP-Range |  |  |
| rsrp-ThresholdSSB-SUL | Not present |  |  |
|  | RSRP-Range |  | SUL |
| prach-RootSequenceIndex CHOICE { |  |  |  |
| l139 | Set according to table 4.4.2-2 for the NR Cell. |  |  |
| } |  |  |  |
| msg1-SubcarrierSpacing | SubcarrierSpacing |  |  |
| restrictedSetConfig | unrestrictedSet |  |  |
| msg3-transformPrecoder | Not present | transform precoding is disabled for Msg3 PUSCH transmission and any PUSCH transmission scheduled with DCI format 0_0 |  |
| ra-PrioritizationForAccessIdentity-r16 | Not present |  |  |
| ra-PrioritizationForSlicing-r17 | Not present |  |  |
|  | RA-PrioritizationForSlicing-r17 |  | Slice_RACH |
| featureCombinationPreambles-r17 SEQUENCE (SIZE(1..maxFeatureCombPreambles-FFS-r17)) OF FeatureCombinationPreambles-r17{} | Not present |  |  |
|  | FeatureCombinationPreambles-r17 | n entry | Slice_RACH |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SUL | Supplementary uplink |
| Slice_RACH | Slice specific RACH configuration |

< Unchanged sections omitted >

–	RA-Prioritization

Table 4.6.3-131: RA-Prioritization

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RA-Prioritization::= SEQUENCE { |  |  |  |
| powerRampingStepHighPriority | dB0 |  |  |
|  | FFS |  | Slice_RACH |
| scalingFactorBI | zero | zero corresponds to 0 |  |
|  | FFS |  | Slice_RACH |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Slice_RACH | Slice specific RACH configuration |

–	RA-PrioritizationForSlicing

Table 4.6.3-131A: RA-PrioritizationForSlicing-r17

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RA-PrioritizationForSlicing-r17::= SEQUENCE { |  |  |  |
| ra-PrioritizationSliceInfoList-r17 SEQUENCE (SIZE (1..maxSliceInfo-r17)) OF RA-PrioritizationSliceInfo-r17 { |  |  |  |
| RA-PrioritizationSliceInfo-r17[n] SEQUENCE{ |  | entry n |  |
| nsagIDList-r17 SEQUENCE (SIZE (1..maxSliceInfo-r17)) OF NSAG-ID-r17 { |  |  |  |
| NSAG-ID-r17[n] | Set to the corresponding NSAG value used in the test case | entry n |  |
| } |  |  |  |
| ra-Prioritization-r17 | RA-Prioritization |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section>