3GPP TSG-RAN5 Meeting #96-e	R5-224611

# Online,  , 15th Aug 2022 - 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0780 | rev | - | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Definition of PC1 MU and relaxation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | MTSU and relaxation value are not defined for PC1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Relaxation value of OFF power for PC1 n257 was defined in Table 6.3.2.5-1. Relaxation value of ACLR for PC1 FR2a was defined in Table 6.5.2.3.5-1b and Table 6.5.2.3.5-1c. Editor’s note for 6.5.2.3 was corrected according to the above update. MTSU for the following Tx test cases were defined for PC1, FR2a in Table F.1.2-1: - MOP (EIRP) - MOP (TRP) - OFF power - OBW - ACLR - Tx Spurious MTSU for the following Rx test cases were defined for PC1, FR2a in Table F.1.3-1: - REFSENS - ACS - Rx Spurious MTSU for PC3 OFF power was corrected according to TR 38.903. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PC1 cannot be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.1.2, F.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Depending on the outcome of MU discussion R5-224610 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

6.3.2	Transmit OFF power

Editor's note:	Following aspects are either missing or not yet determined otherwise:

- Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2 and 4.

- Measurement grid for PC2/4 in Annex M.4 is TBD.

- The testability of this test case is pending further analysis on relaxation of the requirement for other than Band n257.

6.3.2.1	Test purpose

To verify that the UE transmit OFF power is lower than the value specified in the test requirement.

An excess transmit OFF power potentially increases the Rise Over Thermal (RoT) and therefore reduces the cell coverage area for other UEs.

6.3.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

NOTE:	Currently, this test case can only support Band n257 and PC3.

6.3.2.3	Minimum conformance requirements

The transmit OFF power is defined as the TRP in the channel bandwidth when the transmitter is OFF. The transmitter is considered OFF when the UE is not allowed to transmit on any of its ports.

The transmit OFF power shall not exceed the values specified in Table 6.3.2.3-1 for each operating band supported. The requirement is verified with the test metric of TRP (Link=TX beam peak direction, Meas=TRP grid).

Table 6.3.2.3-1: Transmit OFF power

| Operating band | Channel bandwidth / Transmit OFF power (dBm) / measurement bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257, n258, n259, n260, n261 | -35 | -35 | -35 | -35 |
|  | 47.58 MHz | 95.16 MHz | 190.20 MHz | 380.28 MHz |

The normative reference for this requirement is TS 38.101-2 [3] clause 6.3.2.

6.3.2.4	Test description

6.3.2.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.3.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  | Lowest |  |
| Test SCS as specified in Table 5.3.5-1. |  |  | Highest |  |
| Test Parameters |  |  |  |  |
|  | Downlink Configuration |  | Uplink Configuration |  |
| Test ID | Modulation | RB allocation | Modulation | RB allocation |
| 1 | - | - | - | - |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex G.

4.	The UL Reference Measurement Channels are set according to Table 6.3.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.3.2.4.3.

6.3.2.4.2	Test procedure

1.	Set the UE in the Tx beam peak direction found with a 3D EIRP scan as performed in Annex K.1.1. Allow at least BEAM_SELECT_WAIT_TIME (NOTE) for the UE Tx beam selection to complete.

2.	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.2 using condition Tx only.

3.	Measure UE TRP for the assigned NR channel with a rectangular measurement filter with bandwidths according to Table 6.3.2.5-1. Total radiated power is measured according to TRP measurement procedure defined in Annex K. TRP is calculated considering both polarizations, theta and phi.

NOTE:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.1.1.

6.3.2.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6.

6.3.2.5	Test requirement

The requirement for the transmit OFF power shall not exceed the values specified in Table 6.3.2.5-1.

Table 6.3.2.5-1: Transmit OFF power

| Operating band | Channel bandwidth / Transmit OFF power (dBm) / measurement bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n2572 | -35+21.4 | -35+24.4 | -35+27.4 | -35+30.4 |
|  | 47.58 MHz | 95.16 MHz | 190.20 MHz | 380.28 MHz |
| n258, n261 | -35+[21.4] | -35+[24.4] | -35+[27.4] | -35+[30.4] |
|  | 47.58 MHz | 95.16 MHz | 190.20 MHz | 380.28 MHz |
| n260 | -35+[24.1] | -35+[27.1] | -35+[30.1] | -35+[33.1] |
|  | 47.58 MHz | 95.16 MHz | 190.20 MHz | 380.28 MHz |
| NOTE 1: Core requirement cannot be tested due to testability issue and test requirement includes relaxation to achieve impact from test system noise to measurement result = 1.0 dB (Minimum requirement + relaxation). NOTE 2: Relaxed n257 test requirement is testable for PC3 and PC1. |  |  |  |  |

<<Unchanged sections skipped>>

6.5.2.3	Adjacent channel leakage ratio

Editor’s note: The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2, and 4.

- Testability for power class 2 and 4 are FFS.

- Testability for power class 1 and FR2b is FFS.

6.5.2.3.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR).

6.5.2.3.2	Test applicability

This test case applies to all types of NR UE release 15 and forward.

6.5.2.3.3	Minimum conformance requirements

Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency. ACLR requirement is specified for a scenario in which adjacent carrier is another NR channel.

NR Adjacent Channel Leakage power Ratio (NRACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency at nominal channel spacing. The assigned NR channel power and adjacent NR channel power are measured with rectangular filters with measurement bandwidths specified in Table 6.5.2.3.3-1.

If the measured adjacent channel power is greater than –35 dBm then the NRACLR shall be higher than the value specified in Table 6.5.2.3.3-1. The requirement is verified in beam locked mode with the test metric of TRP (Link=TX beam peak direction, Meas=TRP grid).

Table 6.5.2.3.3-1: General requirements for NRACLR

|  | Channel bandwidth / NRACLR / Measurement bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| NRACLR for band n257, n258, n261 | 17 dB | 17 dB | 17 dB | 17 dB |
| NRACLR for band n260 | 16 dB | 16 dB | 16 dB | 16 dB |
| NR channel Measurement bandwidth (MHz) | 47.58 | 95.16 | 190.20 | 380.28 |
| Adjacent channel centre frequency offset [MHz] | +50 / -50 | +100 / -100 | +200 / -200 | +400 / -400 |

The normative reference for this requirement is TS 38.101-2 [3] clause 6.5.2.3.

6.5.2.3.4	Test description

6.5.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and subcarrier spacing, are shown in Table 6.5.2.3.4.1-1 and Table 6.5.2.3.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2.3.4.1-1: Test Configuration Table (Power Class 1)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | - | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | DFT-s-OFDM PI/2 BPSK | 8@0 |
| 2 | High |  |  |  | DFT-s-OFDM PI/2 BPSK | 8@NRB-8 |
| 3 | Mid |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_Full |
| 4 | Low |  |  |  | DFT-s-OFDM QPSK | 8@0 |
| 5 | High |  |  |  | DFT-s-OFDM QPSK | 8@NRB-8 |
| 6 | Mid |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| 7 | Low |  |  |  | DFT-s-OFDM 16 QAM | 8@0 |
| 8 | High |  |  |  | DFT-s-OFDM 16 QAM | 8@NRB-8 |
| 9 | Mid |  |  |  | DFT-s-OFDM 16 QAM | Outer_Full |
| 10 | Low |  |  |  | DFT-s-OFDM 64 QAM | 8@0 |
| 11 | High |  |  |  | DFT-s-OFDM 64 QAM | 8@NRB-8 |
| 12 | Mid |  |  |  | DFT-s-OFDM 64 QAM | Outer_Full |
| 13 | Low |  |  |  | CP-OFDM QPSK | 8@0 |
| 14 | High |  |  |  | CP-OFDM QPSK | 8@NRB-8 |
| 15 | Mid |  |  |  | CP-OFDM QPSK | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-2 for PC1. NOTE 2: Following Test IDs shall be skipped for FR2b - FFS NOTE 3: All test points in this table must also exist in Table 6.2.2.4.1-1, Table 6.2.2.4.1-2, Table 6.2.2.4.1-3 (MPR). |  |  |  |  |  |  |

Table 6.5.2.3.4.1-2: Test Configuration Table (Power Class 2, 3 and 4)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | - | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_1RB_Left |
| 2 | High |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_1RB_Right |
| 3 | Mid |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_Full |
| 4 | Low |  |  |  | DFT-s-OFDM QPSK | Outer_1RB_Left |
| 5 | High |  |  |  | DFT-s-OFDM QPSK | Outer_1RB_Right |
| 6 | Mid |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| 7 | Low |  |  |  | DFT-s-OFDM 16 QAM | Outer_1RB_Left |
| 8 | High |  |  |  | DFT-s-OFDM 16 QAM | Outer_1RB_Right |
| 9 | Mid |  |  |  | DFT-s-OFDM 16 QAM | Outer_Full |
| 10 | Low |  |  |  | DFT-s-OFDM 64 QAM | Outer_1RB_Left |
| 11 | High |  |  |  | DFT-s-OFDM 64 QAM | Outer_1RB_Right |
| 12 | Mid |  |  |  | DFT-s-OFDM 64 QAM | Outer_Full |
| 13 | Low |  |  |  | CP-OFDM QPSK | Outer_1RB_Left |
| 14 | High |  |  |  | CP-OFDM QPSK | Outer_1RB_Right |
| 15 | Mid |  |  |  | CP-OFDM QPSK | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1 for PC2, PC3 and PC4. NOTE 2: Following Test IDs shall be skipped for FR2b and PC3 - All Test IDs for FR2b 400MHz Channel Bandwidth - Test ID 10-15 for FR2b 200MHz Channel Bandwidth - Test ID 10-12 for FR2b 100MHz Channel Bandwidth NOTE 3: All test points in this table must also exist in Table 6.2.2.4.1-7, Table 6.2.2.4.1-8, Table 6.2.2.4.1-9 (MPR). |  |  |  |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and section A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex G.

4.	The UL Reference Measurement channels are set according to Table 6.5.2.3.4.1-1 and Table 6.5.2.3.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.5.2.3.4.3

6.5.2.3.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5.2.3.4.1-1 and Table 6.5.2.3.4.1-2. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Set the UE in the Tx beam peak direction found with a 3D EIRP scan as performed in Annex K.1.1. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 2) for the UE Tx beam selection to complete.

3.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200 ms for the UE to reach maximum output power. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 2) for the UE Tx beam selection to complete.

4.	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.2 using condition Tx only.

5.	Measure EIRP of the transmitted signal in the Tx beam peak direction for the assigned NR channel with a rectangular measurement filter with bandwidths according to Table 6.5.2.3.5-1. EIRP measurement procedure defined in Annex K. EIRP is calculated considering both polarizations, theta and phi.

6.	Measure EIRP of the first NR adjacent channel on both lower and upper side of the assigned NR channel, respectively using a rectangular measurement filter with bandwidths according to Table 6.5.2.3.5-1. EIRP measurement procedure defined in Annex K. EIRP is calculated considering both polarizations, theta and phi.

7.	Calculate the ratios of the power between the values measured in step 5 over step 6 for lower and upper NR ACLR, respectively.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the Table 6.5.2.3.4.1-1 and Table 6.5.2.3.4.1-2, send an NR RRCReconfiguration message according to TS 38.508-1 [10] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

NOTE 2:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.1.1.

6.5.2.3.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6.

6.5.2.3.5	Test requirement

The measured NR ACLR, derived in step 7, shall be higher than the limits in Table 6.5.2.3.5-1.

Table 6.5.2.3.5-1: General requirements for NRACLR

|  | Channel bandwidth / NRACLR / Measurement bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| NRACLR for band n257, n258, n261 | 17 - TT – R dB | 17 - TT – R dB | 17 - TT – R dB | 17 - TT – R dB |
| NRACLR for band n260 | 16 - TT dB | 16 - TT dB | 16 - TT dB | 16 - TT dB |
| NR channel Measurement bandwidth (MHz) | 47.58 | 95.16 | 190.20 | 380.28 |
| Adjacent channel centre frequency offset [MHz] | +50 / -50 | +100 / -100 | +200 / -200 | +400 / -400 |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.5.2.3.5-1a NOTE 2: R for each frequency, channel bandwidth and test point is specified in Table 6.5.2.3.5-1b |  |  |  |  |

Table 6.5.2.3.5-1a: Test Tolerance (Adjacent channel leakage ratio)

|  |  | Channel bandwidth / NRACLR / Measurement bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Test ID | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| NRACLR for band n257, n258, n261 | 1-2, 4-5 | 4.10 | 4.49 | 4.66 | 5.06 |
|  | 3, 6 | 4.08 | 4.45 | 4.59 | 5.06 |
|  | 7-9 | 4.15 | 4.59 | 4.85 | 3.34 |
|  | 10-12 | 4.36 | 4.98 | 4.06 | 1.46 |
|  | 13-15 | 4.17 | 4.62 | 4.91 | 2.99 |
| NRACLR for band n260 | 1-2, 4-5 | 4.48 | 4.65 | 4.97 | - |
|  | 3, 6 | 4.45 | 4.58 | 4.84 | - |
|  | 7-9 | 4.58 | 4.84 | 5.31 | - |
|  | 10-12 | 4.97 | - | - | - |
|  | 13-15 | 4.62 | 4.90 | - | - |

Table 6.5.2.3.5-1b: Relaxation due to testability limit (Adjacent channel leakage ratio)

|  |  | Channel bandwidth / NRACLR / Measurement bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Test ID | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| NRACLR for band n257, n258, n261 | 1-6 | 0 | 0 | 0 | 0 |
|  | 7 | 0 | 0 | 0 | 2.5 |
|  | 8 | 0 | 0 | 0 | 2.5 |
|  | 9 | 0 | 0 | 0 | 2.5 |
|  | 10 | 0 | 0 | 1.5 | 5.5 |
|  | 11 | 0 | 0 | 1.5 | 5.5 |
|  | 12 | 0 | 0 | 1.5 | 5.5 |
|  | 13 | 0 | 0 | 0 | 3 |
|  | 14 | 0 | 0 | 0 | 3 |
|  | 15 | 0 | 0 | 0 | 3 |
| NOTE 1: Relaxation value is derived by Table 6.5.2.3.5-1c for FR2a. NOTE 2: Relaxation value is 0 for FR2b. NOTE 3: Relaxation value is 0 for PC1. |  |  |  |  |  |

Table 6.5.2.3.5-1c: Relaxation value for FR2a ACLR

|  | CA bandwidth class |  |  |
| --- | --- | --- | --- |
| MPR | 100 MHz | 200 MHz | 400 MHz |
| 0 | 0 | 0 | 0 |
| 0.5 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 |
| 1.5 | 0 | 0 | 0 |
| 2 | 0 | 0 | 0 |
| 2.5 | 0 | 0 | 0 |
| 3 | 0 | 0 | 0 |
| 3.5 | 0 | 0 | 0.5 |
| 4 | 0 | 0 | 1 |
| 4.5 | 0 | 0 | 2.5 |
| 5 | 0 | 0 | 3 |
| 5.5 | 0 | 1.5 | 4.5 |
| 6 | 0 | 2 | 5 |
| 6.5 | 0 | 2.5 | 5.5 |
| 7 | 0 | 3 | 6 |
| 7.5 | 0.5 | 3.5 | 6.5 |
| 8 | 1 | 4 | 7 |
| 8.5 | 1.5 | 4.5 | 7.5 |
| 9 | 2 | 5 | 8 |
| NOTE 1: Relaxation value is 0 for PC1. |  |  |  |

<<Unchanged sections skipped>>

F.1	Acceptable uncertainty of Test System (normative)

The maximum acceptable uncertainty of the Test System is specified below for each test, where appropriate. The Test System shall enable the stimulus signals in the test case to be adjusted to within the specified range, and the equipment under test to be measured with an uncertainty not exceeding the specified values. Care should be taken to ensure that each conformance test implementation including the OTA chamber aspects meets the specified measurement uncertainty for each test case by requiring the test laboratory to maintain a detailed measurement uncertainty test report showing compliance to all the measurement uncertainty requirements. The detailed measurement uncertainty report would contain the justification for each measurement uncertainty component and its value and distribution. The derivation of these values is based on the minimum conformance requirements plus relaxation, i.e., test tolerance is not to be considered. All ranges and uncertainties are absolute values, and are valid for a confidence level of 95 %, unless otherwise stated.

A confidence level of 95 % is the measurement uncertainty tolerance interval for a specific measurement that contains 95 % of the performance of a population of test equipment.

The downlink signal uncertainties apply at the defined quiet zone with the UE properly positioned in the quiet zone. The uplink signal uncertainties apply at the measurement equipment with the UE positioned properly in the quiet zone.

F.1.1	Measurement of test environments

Editor’s note: Various measurement accuracies for UE test environments, e.g., pressure, relative humidity, DC&AC voltage, vibration, and vibration frequency, are FFS:

The measurement accuracy of the UE test environments defined in TS 38.508-1 [5] subclause 4.1, Test environments shall be

-	Temperature			±4 degrees.

The above values shall apply unless the test environment is otherwise controlled and the specification for the control of the test environment specifies the uncertainty for the parameter.

F.1.2	Measurement of transmitter

Table F.1.2-1: Maximum Test System Uncertainty (MTSU) for transmitter tests

| Sub clause | Maximum Test System Uncertainty | Derivation of MTSU |
| --- | --- | --- |
| 6.2.1.1 UE maximum output power (EIRP) | PC3 Minimum peak EIRP, Max EIRP Max Device size ≤ 30 cm ±4.89 dB (FR2a, NTC testing) ±5.09 dB (FR2b, NTC testing) ±5.17 dB (FR2a, ETC testing) ±5.37 dB (FR2b, ETC testing) PC1 Minimum peak EIRP, Max EIRP Max Device size ≤ 30 cm ±5.02 dB (FR2a, NTC testing) | MTSU = 1.00 x MU (from Table B.3-1 in TR 38.903) |
| 6.2.1.1 UE maximum output power (TRP) | PC3 Max TRP Max Device size ≤ 30 cm ±4.42 dB (FR2a, NTC testing) ±4.62 dB (FR2b, NTC testing) ±4.70 dB (FR2a, ETC testing) ±4.90 dB (FR2b, ETC testing) PC1 Max TRP Max Device size ≤ 30 cm ±4.65 dB (FR2a, NTC testing) | MTSU = 1.00 x MU (from Table B.3-2 in TR 38.903) |
| <<Unchanged rows skipped>> |  |  |
| 6.3.2 Transmit OFF power | PC3, PC1: Max Device size ≤ 30 cm ±5.67 dB (FR2a) | MTSU = 1.00 x MU (from Table B.8-1 in TR 38.903) |
| <<Unchanged rows skipped>> |  |  |
| 6.5.1 Occupied bandwidth | Max Device size ≤ 30cm PC3: FR2a: ±0.4 [%CBW] (BW 50MHz) ±0.4 [%CBW] (BW 100MHz) ±1.2 [%CBW] (BW 200MHz) ±1.2 [%CBW] (BW 400MHz) FR2b: ±0.4 [%CBW] (BW 50MHz) ±0.4 [%CBW] (BW 100MHz) ±1.3 [%CBW] (BW 200MHz) ±1.3 [%CBW] (BW 400MHz) FR2c: TBD PC1: FR2a: ±0.4 [%CBW] (BW 50MHz) ±0.4 [%CBW] (BW 100MHz) ±1.2 [%CBW] (BW 200MHz) ±1.2 [%CBW] (BW 400MHz) |  |
| <<Unchanged rows skipped>> |  |  |
| 6.5.2.3 Adjacent Channel Leakage Ratio | Max Device size ≤ 30cm PC3: FR2a, NTC & ETC testing: ±5.63 dB (BW ≤ 50MHz) ±6.09 dB (50MHz < BW ≤ 100MHz) ±6.09 dB (100MHz < BW ≤ 200MHz) ±6.09 dB (200MHz < BW ≤ 400MHz) FR2b, NTC & ETC testing: ±6.09 dB (BW ≤ 50MHz) ±6.09 dB (50MHz < BW ≤ 100MHz) ±6.09 dB (100MHz < BW ≤ 200MHz) ±6.09 dB (200MHz < BW ≤ 400MHz) PC1: FR2a, NTC & ETC testing: ±5.59 dB (BW ≤ 400MHz) | MTSU = 1.00 x MU (from Table B.17-1B in TR 38.903) |
| 6.5.3.1 Transmitter Spurious emissions | Max Device size ≤ 30 cm Maximum in-band BW ≤ 400MHz PC3: ±5.14 dB (6GHz ≤ f ≤ 12.75GHz) ±5.11 dB (12.75GHz < f ≤ 23.45GHz) ±5.41 dB (23.45GHz < f < 40.8GHz) ±7.42 dB (40.8GHz ≤ f ≤ 66GHz) ±7.72 dB (66GHz ≤ f ≤ 80GHz) PC1: ±[5.24] dB (6GHz ≤ f ≤ 12.75GHz) ±[6.81] dB (12.75GHz < f ≤ 23.45GHz) ±[7.11] dB (23.45GHz < f < 40.8GHz) ±[9.09] dB (40.8GHz ≤ f ≤ 66GHz) | MTSU = 1.00 x MU (from Table B.18-1 in TR 38.903) |
| <<Unchanged rows skipped>> |  |  |
| NOTE 1: FR2a: 23.45GHz ≤ f ≤ 32.125GHz FR2b: 32.125GHz ≤ f ≤ 40.8GHz FR2c: 40.8GHz ≤ f ≤ 44.3GHz |  |  |

<<Unchanged tables skipped>>

F.1.3	Measurement of receiver

Table F.1.3-1: Maximum Test System Uncertainty (MTSU) for receiver tests

| Sub clause | Maximum Test System Uncertainty | Derivation of MTSU |
| --- | --- | --- |
| 7.3.2 Reference sensitivity power level | PC3 Max Device size ≤ 30 cm ±5.19 dB (FR2a, FR2b, NTC testing) ±5.45 dB (FR2a, FR2b, ETC testing) PC1 Max Device size ≤ 30 cm ±5.41 dB (FR2a, NTC testing) | MTSU = 1.00 x MU (from Table B.19-1 in TR 38.903) |
| <<Unchanged rows skipped>> |  |  |
| 7.5 Adjacent channel selectivity | PC3 ±7.84 dB (Max Device size ≤ 30 cm, FR2a, FR2b) PC1 ±8.06 dB (Max Device size ≤ 30 cm, FR2a) | MTSU = 1.00 x MU (from Table B.21-1 in TR 38.903) |
| <<Unchanged rows skipped>> |  |  |
| 7.9 Spurious emissions | Max Device size ≤ 30 cm Maximum in-band BW ≤ 400MHz PC3: For Band n257, n258, n260, n261: ±5.50dB (6GHz ≤ f ≤ 12.75GHz) ±5.46dB (12.75GHz < f ≤ 23.45GHz) ±6.11dB (23.45GHz < f < 40.8GHz) ±7.65dB (40.8GHz ≤ f ≤ 66GHz) ±7.95 dB (66GHz ≤ f ≤ 80GHz) PC1: For Band n257: ±[5.60]dB (6GHz ≤ f ≤ 12.75GHz) ±[5.56]dB (12.75GHz < f ≤ 23.45GHz) ±[6.20]dB (23.45GHz < f < 40.8GHz) ±[7.72]dB (40.8GHz ≤ f ≤ 66GHz) | MTSU = 1.00 x MU (from Table B.25-1 in TR 38.903) |
| NOTE 1: FR2a, FR2b and FR2c are specified in Table F.1.2-1. |  |  |

<<End of change>>