3GPP TSG-RAN5 Meeting #96-e	R5-224614

# Online,  , 15th Aug 2022 - 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0781 | rev | - | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update FR2 TRx MU in 38.521-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | MU value for FR2 EVM is not aligned with agreement of Proposal 1 in R5-221626. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Table F.1.2-2 and Table F.1.2-3 were corrected to be aligned with Proposal 1 in R5-221626. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Error of MU value will remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Will be updated to capture the outcome of MU discussion. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

F.1.2	Measurement of transmitter

<<Unchanged table skipped>>

Table F.1.2-2: EVM Measurement Uncertainty (MU) for PUSCH, PC3, FR2a (23.45GHz <= f <= 32.125GHz)

| Test ID | Modulation | RB alloc. | 50MHz | 100MHz | 200MHz | 400MHz |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 2.78% | 3.85% | 5.44% | 7.69% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 3.10% | 4.16% | 5.88% | 9.00% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 2.78% | 3.85% | 5.44% | 7.69% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 3.10% | 4.16% | 5.88% | 9.00% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 3.31% | 4.50% | 6.36% | 11.22% |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 3.60% | 4.73% | 6.68% | 11.22% |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 4.26% | 5.95% | 8.41% | 15.85% |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 5.00% | 7.07% | 9.99% | 15.85% |
| 9 | CP-OFDM QPSK | Inner_Full | 3.60% | 4.73% | 6.68% | 11.89% |
| 10 | CP-OFDM QPSK | Outer_Full | 3.71% | 4.99% | 7.06% | 11.89% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 4.26% | 5.95% | 8.41% | 15.85% |
| 12 | CP-OFDM 16 QAM | Outer_Full | 4.26% | 5.95% | 8.41% | 15.85% |
| 13 | CP-OFDM 64 QAM | Inner_Full | 6.29% | 8.89% | 12.58% | 21.13% |
| 14 | CP-OFDM 64 QAM | Outer_Full | 6.29% | 8.89% | 12.58% | 21.13% |

Table F.1.2-3: EVM Measurement Uncertainty (MU) for PUSCH, PC3, FR2b (32.125GHz < f <= 40.8GHz)

| Test ID | Modulation | RB alloc. | 50MHz | 100MHz | 200MHz | 400MHz |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 3.56% | 4.83% | 6.91% | 9.65% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 4.15% | 5.69% | 8.11% | 12.50% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 3.56% | 4.83% | 6.91% | 9.65% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 4.15% | 5.69% | 8.11% | 12.50% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 4.54% | 6.26% | 8.91% | 18.06% |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 5.08% | 7.18% | 10.15% | 18.06% |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 6.77% | 9.57% | 13.53% | 25.51% |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 8.04% | 11.37% | 16.08% | 25.51% |
| 9 | CP-OFDM QPSK | Inner_Full | 5.08% | 7.18% | 10.15% | 19.13% |
| 10 | CP-OFDM QPSK | Outer_Full | 5.38% | 7.60% | 10.75% | 19.13% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 6.77% | 9.57% | 13.53% | 25.51% |
| 12 | CP-OFDM 16 QAM | Outer_Full | 6.77% | 9.57% | 13.53% | 25.51% |
| 13 | CP-OFDM 64 QAM | Inner_Full | 10.12% | 14.31% | 20.24% | 34.02% |
| 14 | CP-OFDM 64 QAM | Outer_Full | 10.12% | 14.31% | 20.24% | 34.02% |

<<End of change>>