3GPP TSG-RAN5 Meeting #96-e	R5-224639

# Online,  , 15th Aug 2022 - 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2479 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to CSI-MeasConfig for RRM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on Table 7.3.1-6: CSI-MeasConfig for RRM, CSI-ReportConfig for BM should be configured – regardless of test case requirement – when CSI-RS for BM (CSI-RS.X.2 FDD/TDD RMCs in TS38.533) is configured in the test case. <TS38.508-1> |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated Table 7.3.1-6 such that CSI-ReportConfig is only configured when test case requires it. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformance test cannot be performed correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

7.3.1	Radio resource control information elements for RRM

As defined in clause 4.6.3 with the following exceptions:

<<Unchanged sections skipped>>

-	CSI-MeasConfig

Table 7.3.1-6: CSI-MeasConfig for RRM

| Derivation Path: Table 4.6.3-38 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| CSI-MeasConfig::= SEQUENCE { |  |  |  |  |  |  |  |
| nzp-CSI-RS-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource { |  | n1+n2+n3+n4+n5 entries |  | n1=1 if CSI-RS for CSI is configured in test case, n1=0 otherwise; n2=0 if CSI-RS for BM is not configured in the test case. If CSI-RS for BM is configured in the test case, n2 = 2 if 2SSB, n2 = 1 otherwise n3=4 if TRS is configured in test case, n3=0 otherwise; n4=4 if second resource set of TRS is configured in test case, n4=0 otherwise; n5=2 if aperiodic CSI-RS for BM is configured in test case, n5=0 otherwise; |  |  |  |
| NZP-CSI-RS-Resource[k, k=1..n1] |  | NZP-CSI-RS-Resource for CSI |  | entry 1 |  | n1>0 |  |
| NZP-CSI-RS-Resource[k, k=n1+1.. n1+n2] |  | NZP-CSI-RS-Resource for BM (k-n1-1) |  | entry ... |  | n2>0 |  |
| NZP-CSI-RS-Resource[k, k= n1+n2+1.. n1+n2+n3] |  | NZP-CSI-RS-Resource for TRS (k-n1-n2) |  | entry ... |  | n3>0 |  |
| NZP-CSI-RS-Resource[k, k= n1+n2+n3+1.. n1+n2+n3+n4] |  | NZP-CSI-RS-Resource for TRS (k-n1-n2-n3) with condition SECOND_SET |  | entry ... |  | n4>0 |  |
| NZP-CSI-RS-Resource[k, k= n1+n2+n3+n4+1.. n1+n2+n3+n4+n5] |  | NZP-CSI-RS-Resource for BM (k-n1-n2-n3-n4) with condition APERIODIC |  | entry ... |  | n5>0 |  |
| } |  |  |  |  |  |  |  |
| nzp-CSI-RS-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet { |  | m1+m2+m3+m4+m5 entries |  | mi=1 if ni>0, mi=0 otherwise |  |  |  |
| NZP-CSI-RS-ResourceSet[k, k=1..m1] |  | NZP-CSI-RS-ResourceSet for CSI |  | entry 1 |  | n1>0 |  |
| NZP-CSI-RS-ResourceSet[k, k=m1+1.. m1+m2] |  | NZP-CSI-RS-ResourceSet for BM |  | entry ... |  | n2>0 |  |
| NZP-CSI-RS-ResourceSet[k, k= m1+m2+1.. m1+m2+m3] |  | NZP-CSI-RS-ResourceSet for TRS |  | entry ... |  | n3>0 |  |
| NZP-CSI-RS-ResourceSet[k, k= m1+m2+m3+1.. m1+m2+m3+m4] |  | NZP-CSI-RS-ResourceSet for TRS with condition SECOND_SET |  | entry ... |  | n4>0 |  |
| NZP-CSI-RS-ResourceSet[k, k= m1+m2+m3+m4+1.. m1+m2+m3+m4+m5] |  | NZP-CSI-RS-ResourceSet for BM with condition APERIODIC |  | entry ... |  | n5>0 |  |
| } |  |  |  |  |  |  |  |
| csi-IM-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-Resource { |  | 1 entry |  |  |  | n1>0 |  |
| CSI-IM-Resource[1] |  | CSI-IM-Resource-RRM |  | entry 1 |  |  |  |
| } |  |  |  |  |  |  |  |
| csi-IM-ResourceToAddModList |  | Not present |  |  |  | n1=0 |  |
| csi-IM-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSet { |  | 1 entry |  |  |  | n1>0 |  |
| CSI-IM-ResourceSet[1] |  | CSI-IM-ResourceSet-RRM |  | entry 1 |  |  |  |
| } |  |  |  |  |  |  |  |
| csi-IM-ResourceSetToAddModList |  | Not present |  |  |  | n1=0 |  |
| csi-SSB-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSets)) OF CSI-SSB-ResourceSet { |  |  |  |  |  |  |  |
| CSI-SSB-ResourceSet[1] |  | CSI-SSB-ResourceSet-RRM |  | entry 1 |  |  |  |
| } |  |  |  |  |  |  |  |
| csi-ResourceConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfig { |  | p1+m2+m3+m5 entries |  | p1=m1+1 if n1>0, p1=0 otherwise. |  |  |  |
| CSI-ResourceConfig[k, k=1..m1] |  | CSI-ResourceConfig for CSI |  | entry 1 |  | n1>0 |  |
| CSI-ResourceConfig[k, k=p1] |  | CSI-ResourceConfig for CSI-IM |  | entry ... |  | n1>0 |  |
| CSI-ResourceConfig[k, k=p1+1.. p1+m2] |  | CSI-ResourceConfig for BM |  | entry ... |  | n2>0 |  |
| CSI-ResourceConfig[k, k= p1+m2+1.. p1+m2+m3] |  | CSI-ResourceConfig for TRS |  | entry ... |  | n3>0 and n4=0 |  |
|  |  | CSI-ResourceConfig for TRS with condition SECOND_SET |  |  |  | n4>0 |  |
| CSI-ResourceConfig[k, k= p1+m2+m3+1.. p1+m2+m3+m5] |  | CSI-ResourceConfig for BM with condition APERIODIC |  | entry ... |  | n5>0 |  |
| } |  |  |  |  |  |  |  |
| csi-ReportConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfig { |  | r1+r2+r5+s1 entries |  | r1 = 1 if CSI Reporting for CSI is configured in test case, r1 = 0 otherwise; r2 = 1 if CSI Reporting for BM is configured in test case, r2 = 0 otherwise; r5 = 1 if aperiodic CSI Reporting for BM is configured in test case, r5 = 0 otherwise; s1 = 1 if: - this CSI-MeasConfig is configured on NR SpCell, and, - CSI-RS for CSI is configured on SCell in TC, and, - SCell is not a PUCCH-SCell Otherwise s1 = 0. |  |  |  |
| CSI-ReportConfig[k, k=1..r1] |  | CSI-ReportConfig for CSI |  | entry 1 |  | r1>0 |  |
| CSI-ReportConfig[k, k=r1+1.. r1+r2] |  | CSI-ReportConfig for BM |  | entry ... |  | r2>0 |  |
| CSI-ReportConfig[k, k=r1+r2+1.. r1+r2+r5] |  | CSI-ReportConfig for BM with condition APERIODIC |  | entry ... |  | r5>0 |  |
| CSI-ReportConfig[k, k=r1+r2+r5+1.. r1+r2+r5+s1+1] |  | CSI-ReportConfig for CSI with condition SCELL_CSI_ON_SPCELL |  | entry ... |  | s1>0 |  |
| } |  |  |  |  |  |  |  |
| reportTriggerSize |  | Not present |  |  |  |  |  |
|  |  | 1 |  |  |  | n5>0 |  |
| aperiodicTriggerStateList CHOICE { |  | Not present |  |  |  |  |  |
| setup |  | CSI-AperiodicTriggerStateList |  |  |  | n5>0 |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| 2SSB | For configuration with 2 SS Blocks |

<<End of change>>