3GPP TSG-RAN5 Meeting #96-e	R5-224666

# Electronic Meeting, 15th – 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0439 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR IMS TC 6.3-Re-Registration Scenarios |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The initial registration procedure and IMS PDU establishment procedure specified in steps 2-19a1 of TS38.508-1 table 4.5.2.2-2 are missing and need added.But the corresponding behavior has been implemented in TTCN code. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add steps 1A-1V. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the TC or a non-conformant UE may pass the TC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

6.3	Re-Registration Scenarios / 5GS

6.3.1	Test Purpose (TP)

(1)

with { UE being registered to IMS with expiration interval at 120 seconds }

ensure that {

when { 60 seconds passed }

then {UE re-registers }

}

(2)

with { UE starting re-registration procedure by sending REGISTER }

ensure that {

when { UE receives 500 Server Internal Error response }

then {UE starts initial registration }

}

(3)

with { UE being registered to IMS with expiration interval at 360 seconds }

ensure that {

when { 180 seconds passed }

then {UE re-registers }

}

(4)

with { UE being registered to IMS with expiration interval at 1600 seconds }

ensure that {

when { 1000 seconds passed }

then {UE re-registers }

}

(5)

with { UE attempting re-registration }

ensure that {

when { UE receives 423 Interval Too Brief response }

then {UE sends another re-registration request with given expiration interval }

}

(6)

with { UE being registered }

ensure that {

when { UE receives notification about shortened expiration interval for one of its registered public user identities }

then {UE re-registers after half of the shorted expiration interval elapses }

}

6.3.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-15 requirements.

[TS 24.229, clause 5.1.1.4.1]:

The UE can perform the reregistration of a previously registered public user identity bound to any one of its contact addresses and the associated set of security associations or TLS sessions at any time after the initial registration has been completed.

...

Unless either the user or the application within the UE has determined that a continued registration is not required the UE shall reregister an already registered public user identity either 600 seconds before the expiration time if the previous registration was for greater than 1200 seconds, or when half of the time has expired if the previous registration was for 1200 seconds or less,

...

On receiving a 423 (Interval Too Brief) response to the REGISTER request, the UE shall:

-	send another REGISTER request populating the registration expiration interval value with an expiration timer of at least the value received in the Min-Expires header field of the 423 (Interval Too Brief) response.

On receiving a 408 (Request Timeout) response or 500 (Server Internal Error) response or 504 (Server Time-Out) response or 403 (Forbidden) response for a reregistration, the UE shall perform the procedures for initial registration as described in subclause 5.1.1.2.

[TS 24.229, clause 5.1.1.4.1]:

At any time, the UE can receive a NOTIFY request carrying information related to the reg event package (as described in subclause 5.1.1.3). If:

-	the state attribute in any of the <registration> elements is set to "active";

-	the value of the <uri> sub-element inside the <contact> sub-element is set to the Contact address that the UE registered; and

-	the event attribute of that <contact> sub-element(s) is set to "shortened";

the UE shall:

1)	use the expires attribute of the <contact> sub-element that the UE registered to adjust the expiration time for that public user identity; and

2)	start the re-authentication procedures at the appropriate time (as a result of the S-CSCF procedure described in subclause 5.4.1.6) by initiating a reregistration as described in subclause 5.1.1.4, if required.

NOTE:	When authenticating a given private user identity, the S-CSCF will only shorten the expiry time within the <contact> sub-element that the UE registered using its private user identity. The <contact> elements for the same public user identity, if registered by another UE using different private user identities remain unchanged. The UE will not initiate a reregistration procedure, if none of its <contact> sub-elements was modified.

6.3.3	Test description

6.3.3.1	Pre-test conditions

System Simulator:

-	SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE.

-	SS is listening to SIP default port 5060 for both UDP and TCP protocols.

-	SS is able to perform IMS AKA authentication for the IMPI, according to 3GPP TS 33.203 [16] clause 6.1.

-	1 NR Cell

UE:

-	The UE contains either ISIM and USIM applications or only USIM application on UICC.

-	The UE is configured to register for IMS after switch on.

-	The UE is switched off.

Preamble:

-	None

6.3.3.2	Test procedure sequence

Table 6.3.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is switched on. |  |  |  |  |
| 1A-1V | Steps 2-19a1 of generic procedure specified in Table 4.5.2.2-2 of TS 38.508-1 [21] are performed. | - |  | - | - |
| 2-9 | Steps 1-8 from clause A.2: initial IMS registration happens, with SS giving 120 seconds expiration interval. |  |  |  |  |
| 10 | UE re-registers 60 seconds later. | --> | REGISTER | 1 | P |
| 11 | SS declines re-registration attempt. | <-- | 500 Server Internal Error |  |  |
| 12 | Step 1 from clause A.2: UE sends initial IMS registration request | --> | REGISTER | 2 | P |
| 13-19 | Steps 2-8 from clause A.2, with SS giving 360 seconds expiration interval. |  |  |  |  |
| 20 | UE re-registers 180 seconds later. | --> | REGISTER | 3 | P |
| 21 | SS responds with 1600 seconds expiration interval | <-- | 200 OK |  |  |
| 22 | UE re-registers 1000 seconds later | --> | REGISTER | 4 | P |
| 23 | SS responds with 423 Interval Too Brief with Min-Expires value of 800000 seconds | <-- | 423 Interval Too Brief |  |  |
| 24 | UE sends a new another re-registration request using at least 800000 seconds expiration. | --> | REGISTER | 5 | P |
| 25 | SS responds with 200 OK. | <-- | 200 OK |  |  |
| 26 | SS notifies UE about shortened expiration time of 60 seconds for one of the registered public user identities. | <-- | NOTIFY |  |  |
| 27 | UE responds with 200 OK | --> | 200 OK |  |  |
| 28 | 30 seconds before new expiry time, UE re-registers | --> | REGISTER | 6 | P |
| 29 | SS responds with authentication challenge and security mechanism supported by the network | <-- | 401 Unauthorized |  |  |
| 30 | UE completes security procedures | --> | REGISTER |  |  |
| 31 | SS responds with 200 OK | <-- | 200 OK |  |  |

6.3.3.3	Specific message contents

Table 6.3.3.3-1: 200 OK for REGISTER (step 5, Table 6.3.3.2-1)

| Derivation path: TS 34.229-1 [2], Table in subclause A.1.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Contact |  |  |  |  |
| expires |  | 120 |  |  |

Table 6.3.3.3-2: REGISTER (step 10, Table 6.3.3.2-1)

| Derivation path: TS 34.229-1 [2], Table in subclause A.1.1, Conditions A2, A17, A32 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Security-Client |  |  |  |  |
| spi-c |  | new SPI number of the inbound SA at the protected client port, shall be different from previously used number |  |  |
| spi-s |  | new SPI number of the inbound SA at the protected server port, shall be different from previously used number |  |  |
| port-c |  | new protected client port, shall be different from previously used number |  |  |
| port-s |  | same value as in the previous REGISTER |  |  |
| Authorization |  |  |  | RFC 2617 [23] |
| nonce-count |  | 2 |  | TS 24.229 [7] |

Table 6.3.3.3-3: 500 Server Internal Error (step 11, Table 6.3.3.2-1)

Derivation path: TS 34.229-1 [2], Table in subclause A.4.7

Table 6.3.3.3-4: 200 OK for REGISTER (step 15, Table 6.3.3.2-1)

| Derivation path: TS 34.229-1 [2], Table in subclause A.1.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Contact |  |  |  |  |
| expires |  | 360 |  |  |

Table 6.3.3.3-5: REGISTER (step 20, Table 6.3.3.2-1)

| Derivation path: TS 34.229-1 [2], Table in subclause A.1.1, Conditions A2, A17, A32 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Security-Client |  |  |  |  |
| spi-c |  | new SPI number of the inbound SA at the protected client port, shall be different from previously used numbers |  |  |
| spi-s |  | new SPI number of the inbound SA at the protected server port, shall be different from previously used numbers |  |  |
| port-c |  | new protected client port, shall be different from previously used numbers |  |  |
| port-s |  | same value as in the previous REGISTER |  |  |
| Authorization |  |  |  | RFC 2617 [23] |
| nonce-count |  | 2 |  | TS 24.229 [7] |

Table 6.3.3.3-6: 200 OK for REGISTER (step 21, Table 6.3.3.2-1)

| Derivation path: TS 34.229-1 [2], Table in subclause A.1.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Contact |  |  |  |  |
| expires |  | 1600 |  |  |

Table 6.3.3.3-7: REGISTER (step 22, Table 6.3.3.2-1)

| Derivation path: TS 34.229-1 [2], Table in subclause A.1.1, Conditions A2, A17, A32 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Security-Client |  |  |  |  |
| spi-c |  | new SPI number of the inbound SA at the protected client port, shall be different from previously used numbers |  |  |
| spi-s |  | new SPI number of the inbound SA at the protected server port, shall be different from previously used numbers |  |  |
| port-c |  | new protected client port, shall be different from previously used numbers |  |  |
| port-s |  | same value as in the previous REGISTER |  |  |
| Authorization |  |  |  | RFC 2617 [23] |
| nonce-count |  | 3 |  | TS 24.229 [7] |

Table 6.3.3.3-8: 423 Interval Too Brief (step 23, Table 6.3.3.2-1)

| Derivation path: TS 34.229-1 [2], Table in subclause A.1.7 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Min-Expires |  |  |  |  |
| delta-seconds |  | 800000 |  |  |

Table 6.3.3.3-9: REGISTER (step 24, Table 6.3.3.2-1)

| Derivation path: TS 34.229-1 [2], Table in subclause A.1.1, Conditions A2, A17, A32 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Contact |  |  |  |  |
| expires |  | 800000 or more (Remark: either the Contact header contains such expires parameter or below Expires header is present. If both are present, Expires header is to be ignored) |  |  |
| Expires |  |  |  |  |
| delta-seconds |  | 800000 or more (Remark: either the Contact header contains above expires parameter or Expires header is present. If both are present, Expires header is to be ignored) |  |  |
| Authorization |  |  |  | RFC 2617 [23] |
| nonce-count |  | 4 |  | TS 24.229 [7] |

Table 6.3.3.3-9A: 200 OK for REGISTER (step 25, Table 6.3.3.2-1)

| Derivation path: TS 34.229-1 [2], Table in subclause A.1.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Contact |  |  |  |  |
| expires |  | 800000 |  |  |

Table 6.3.3.3-10: NOTIFY (step 26, Table 6.3.3.2-1)

| Derivation path: TS 34.229-1 [2], Table in subclause A.1.6, Conditions A1, and A3 OR A4 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Message-body |  |  |  |  |
|  | A3 | <?xml version="1.0" encoding="UTF-8"?> <reginfo xmlns="urn:ietf:params:xml:ns:reginfo" version="1" state="partial"> <registration aor=" PublicUserIdentity1 (NOTE 1)" id="a100" state="active"> <contact id="980" state="active" event="shortened" expires="60"> <uri>same value as in Contact header of REGISTER request</uri> </contact> </registration> </reginfo> |  |  |
|  | A4 | <?xml version="1.0" encoding="UTF-8"?> <reginfo xmlns="urn:ietf:params:xml:ns:reginfo" xmlns:gr="urn:ietf:params:xml:ns:gruuinfo" version="1" state="partial"> <registration aor=" PublicUserIdentity1 (NOTE 1)" id="a100" state="active"> <contact id="980" state="active" event="shortened" expires="60"> callid="Call-Id of most recent REGISTER" cseq="CSeq value of most recent REGISTER"> <uri>same value as in Contact header of REGISTER request</uri> <unknown-param name="+sip.instance"> "Instance ID of the UE;" </unknown-param> <gr:pub-gruu uri="public GRUU associated to this aor"/> <gr:temp-gruu uri="temporary GRUU associated to this aor" first-cseq="CSeq of the REGISTER request that caused the temporary GRUU to assigned for the UE"/> </contact> </registration> </reginfo> |  |  |

Table 6.3.3.3-11: 200 OK for other requests than REGISTER or SUBSCRIBE (step 27, Table 6.3.3.2-1)

Derivation path: TS 34.229-1 [2], Table in subclause A.3.1, Conditions A5, A11, A22

Table 6.3.3.3-12: REGISTER (step 28, Table 6.3.3.2-1)

| Derivation path: TS 34.229-1 [2], Table in subclause A.1.1, Conditions A2, A17, A32 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Header/param |  | Cond |  | Value/remark |  | Rel |  | Reference |  |
| Contact |  |  |  |  |  |  |  |  |  |
| expires |  |  |  | 800000 (if present) |  |  |  |  |  |
| Expires |  |  |  | present if no expires parameter in Contact header |  |  |  |  |  |
| delta-seconds |  |  |  | 800000 |  |  |  |  |  |
| Authorization |  |  |  |  |  |  |  |  |  |
| nonce-count |  |  |  | 5 |  |  |  |  |  |

Table 6.3.3.3-13: 401 Unauthorized (step 29, Table 6.3.3.2-1)

| Derivation path: TS 34.229-1 [2], Table in subclause A.1.2, Condition A1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| WWW-Authenticate |  |  |  | RFC 2617 [23] |
| nonce |  | Base 64 encoding of new RAND and new AUTN (different from the values used in step 3) |  | TS 24.229 [7] |

Table 6.3.3.3-14: REGISTER (step 30, Table 6.3.3.2-1)

| Derivation path: TS 34.229-1 [2], Table in subclause A.1.1, Conditions A2, A17, A32 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Header/param |  | Cond |  | Value/remark |  | Rel |  | Reference |  |
| Contact |  |  |  |  |  |  |  |  |  |
| expires |  |  |  | 800000 (if present) |  |  |  |  |  |
| Expires |  |  |  | present if no expires parameter in Contact header |  |  |  |  |  |
| delta-seconds |  |  |  | 800000 |  |  |  |  |  |
| Authorization |  |  |  |  |  |  |  |  |  |
| nonce-count |  |  |  | 1 |  |  |  |  |  |

Table 6.3.3.3-15: 200 OK for REGISTER (step 31, Table 6.3.3.2-1)

Derivation path: TS 34.229-1 [2], Table in subclause A.1.3, Condition A2

<End of modified section 1>