3GPP TSG-RAN5 Meeting #96-e	R5-224672

# Electronic Meeting, 15th – 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0445 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR IMS TC 6.8-Authentication with SQN out of range |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The presence of registration procedure and IMS PDU session establishment procedure as defined in steps 2-19a1 of TS38.508-1 table 4.5.2.2-2 are currently not reflected in test procedure sequence. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add steps 1A-1V. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the TC or a non-conformant UE may pass the TC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

6.8	Authentication / Security-Server missing / SQN out of range / 5GS

6.8.1	Test Purpose (TP)

(1)

with { UE starting registration procedure }

ensure that {

when { UE receiving a challenge response without Security-Server header }

then { UE abandons the authentication procedure and sends a new REGISTER request with new Call-ID }

}

(2)

with { UE having sent a new initial REGISTER request }

ensure that {

when { UE receiving a challenge response with SQN out of range }

then { UE sends another REGISTER request with challenge response AUTS and populates a new Security-Client header }

}

6.8.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-15 requirements.

[TS 24.229 clause 5.1.1.5.1]:

Authentication is performed during initial registration. A UE can be re-authenticated during subsequent reregistrations, deregistrations or registrations of additional public user identities. When the network requires authentication or re-authentication of the UE, the UE will receive a 401 (Unauthorized) response to the REGISTER request.

On receiving a 401 (Unauthorized) response to the REGISTER request, the UE shall:

1)	extract the RAND and AUTN parameters;

2)	check the validity of a received authentication challenge, as described in 3GPP TS 33.203 [19] i.e. the locally calculated XMAC must match the MAC parameter derived from the AUTN part of the challenge; and the SQN parameter derived from the AUTN part of the challenge must be within the correct range; and

3)	check the existence of the Security-Server header field as described in RFC 3329 [48]. If the Security-Server header field is not present or it does not contain the parameters required for the setup of the set of security associations (see annex H of 3GPP TS 33.203 [19]), the UE shall abandon the authentication procedure and send a new REGISTER request with a new Call-ID.

…

In the case that the 401 (Unauthorized) response is deemed to be invalid then the UE shall behave as defined in subclause 5.1.1.5.3.

[TS 24.229, clause 5.1.1.5.3]

If, in a 401 (Unauthorized) response, either the MAC or SQN is incorrect the UE shall respond with a further REGISTER indicating to the S-CSCF that the challenge has been deemed invalid as follows:

-	in the case where the UE deems the MAC parameter to be invalid the subsequent REGISTER request shall contain no "auts" Authorization header field parameter and an empty "response" Authorization header field parameter, i.e. no authentication challenge response;

-	in the case where the UE deems the SQN to be out of range, the subsequent REGISTER request shall contain the "auts" Authorization header field parameter (see 3GPP TS 33.102 [18]).

NOTE:	In the case of the SQN being out of range, a "response" Authorization header field parameter can be included by the UE, based on the procedures described in RFC 3310 [49].

Whenever the UE detects any of the above cases, the UE shall:

-	send the REGISTER request using an existing set of security associations, if available (see 3GPP TS 33.203 [19]);

-	populate a new Security-Client header field within the REGISTER request and associated contact address, set to specify the security mechanisms it supports, the IPsec layer algorithms for integrity and confidentiality protection it supports and the parameters needed for the new security association setup. These parameters shall contain new values for spi_uc, spi_us and port_uc; and

-	not create a temporary set of security associations.

6.8.3	Test description

6.8.3.1	Pre-test conditions

System Simulator:

-	SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE.

-	SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [16] clause 6.1 and RFC 3310 [15].

- 	SS is listening to SIP default port 5060 for both UDP and TCP protocols.

-	1 NR Cell

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC.

-	The UE is configured to register for IMS after switch on.

-	The UE is switched off.

Preamble:

-	None.

6.8.3.2	Test procedure sequence

Table 6.8.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. |  |  |  |  |
| 1A-1V | Steps 2-19a1 of generic procedure specified in Table 4.5.2.2-2 of TS 38.508-1 [21] are performed. |  |  |  |  |
| 2 | UE sends initial registration for IMS services. | --> | REGISTER |  |  |
| 3 | SS responds challenge response without Security-Server header. | <-- | 401 Unauthorized |  |  |
| 4 | Check: does the UE sends a new REGISTER request with new Call-ID | --> | REGISTER | 1 | P |
| 5 | SS responds with an invalid AKAv1-MD5 authentication challenge with SQN out of range. | <-- | 401 Unauthorized |  |  |
| 6 | Check: does the UE send another REGISTER request: - contains AUTS directive - UE populates a new Security-Client header set to specify the security mechanism it supports, the IPsec layer algorithms it supports and the parameters needed for the new security association setup. | --> | REGISTER | 2 | P |
| 7-13 | Continue with Annex A.2 step 2-8 in order to get the UE in a stable registered state. | - |  |  |  |

6.8.3.3	Specific message contents

Table 6.8.3.3-1: REGISTER (step 2, table 6.8.3.2-1)

Derivation Path: TS 34.229-1 [2], Table in subclause A.1.1, Condition A1

Table 6.8.3.3-2: 401 Unauthorized for REGISTER (step 3, table 6.8.3.2-1)

| Derivation path: TS 34.229-1 [2], Table in subclause A.1.2, Condition A1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Security-Server |  | not present. |  |  |

Table 6.8.3.3-3: REGISTER (step 4, table 6.8.3.2-1)

| Derivation path: TS 34.229-1 [2], Table in subclause A.1.1, Condition A1 and A32 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Call-ID |  |  |  |  |
| callid |  | Value differs from the one sent in in Step 2 of table 6.8.3.2-1. |  |  |

Table 6.8.3.3-4: 401 Unauthorized for REGISTER (step 5, table 6.8.3.2-1)

| Derivation Path: TS 34.229-1 [2], Table in subclause A.1.2, Condition A1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| WWW-Authenticate |  |  |  |  |
| nonce |  | Base 64 encoding of RAND and AUTN, generated with SQN out of range with the AMF information field set to AMFRESYNCH value to trigger SQN re-synchronisation procedure in test ISIM/USIM, see TS 34.108 clause 8.1.2.2. |  |  |

Table 6.8.3.3-5: REGISTER (step 6, table 6.8.3.2-1)

| Derivation Path: TS 34.229-1 [2], Table in subclause A.1.1, Conditions A1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| CSeq |  |  |  |  |
| value |  | must be incremented from previous REGISTER |  |  |
| Call-ID |  |  |  |  |
| callid |  | same value as in REGISTER at Step 4 |  |  |
| Authorization |  |  |  |  |
| nonce |  | same as in previous 401 UNAUTHORIZED message |  |  |
| opaque |  | same as in previous 401 UNAUTHORIZED message |  |  |
| auts |  | any value |  |  |
| Security-Client |  |  |  |  |
| spi-c |  | new SPI number of the inbound SA at the protected client port, shall be different from previously used number |  |  |
| spi-s |  | new SPI number of the inbound SA at the protected server port, shall be different from previously used number |  |  |
| port-c |  | new protected client port, shall be different from previously used number |  |  |

<End of modified section 1>