3GPP TSG-RAN5 Meeting #96-e	R5-224692

# Electronic Meeting, 15th – 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3148 | rev | - | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to NR TC 7.1.2.3.6 to test RedCap UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - RedCap UE use 12bit SN for AM RLC, so the RLC header is 2 bytes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The size of RLC SDUs during the test is added 1byte for RedCap UE. - Update Note 1 for RedCap UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the TC or a non-conformant UE may pass the TC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

7.1.2.3.6	AM RLC / Polling for status

7.1.2.3.6.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and using AM RLC }

ensure that {

when { last data in the UL buffer is being transmitted }

then { UE transmits a Poll }

}

(2)

with { UE in RRC_CONNECTED state and using AM RLC }

ensure that {

when { the t-PollRetransmit timer expires }

then { UE transmits a Poll }

}

(3)

with { UE in RRC_CONNECTED state and using AM RLC }

ensure that {

when { PDU_WITHOUT_POLL >= pollPDU }

then { UE transmits a Poll }

}

(4)

with { UE in RRC_CONNECTED state and using AM RLC }

ensure that {

when { BYTE_WITHOUT_POLL >= pollByte }

then { UE transmits a Poll }

}

7.1.2.3.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.322, clauses 5.3.3.2, 7.3 and 7.4. Unless otherwise stated these are Rel-15 requirements.

[TS 38.322, clause 5.3.3.2]

Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission such that the AMD PDU contains either a not previously transmitted RLC SDU or an RLC SDU segment containing not previously transmitted byte segment, the transmitting side of an AM RLC entity shall:

-	increment PDU_WITHOUT_POLL by one;

-	increment BYTE_WITHOUT_POLL by every new byte of Data field element that it maps to the Data field of the AMD PDU;

-	if PDU_WITHOUT_POLL >= pollPDU; or

-	if BYTE_WITHOUT_POLL >= pollByte:

-	include a poll in the AMD PDU as described below.

Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission, the transmitting side of an AM RLC entity shall:

-	if both the transmission buffer and the retransmission buffer becomes empty (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) after the transmission of the AMD PDU; or

-	if no new RLC SDU can be transmitted after the transmission of the AMD PDU (e.g. due to window stalling);

-	include a poll in the AMD PDU as described below.

NOTE:	Empty RLC buffer (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) should not lead to unnecessary polling when data awaits in the upper layer. Details are left up to UE implementation.

To include a poll in an AMD PDU, the transmitting side of an AM RLC entity shall:

-	set the P field of the AMD PDU to "1";

-	set PDU_WITHOUT_POLL to 0;

-	set BYTE_WITHOUT_POLL to 0.

After submitting an AMD PDU including a poll to lower layer and after incrementing of TX_Next if necessary, the transmitting side of an AM RLC entity shall:

-	set POLL_SN to TX_Next – 1;

-	if t-PollRetransmit is not running:

-	start t-PollRetransmit.

-	else:

-	restart t-PollRetransmit.

[TS 38.322, clause 5.3.3.4]

Upon expiry of t-PollRetransmit, the transmitting side of an AM RLC entity shall:

-	if both the transmission buffer and the retransmission buffer are empty (excluding transmitted RLC SDU or RLC SDU segment awaiting acknowledgements); or

-	if no new RLC SDU or RLC SDU segment can be transmitted (e.g. due to window stalling):

-	consider the RLC SDU with SN = TX_Next – 1 for retransmission; or

-	consider any RLC SDU which has not been positively acknowledged for retransmission.

-	include a poll in an AMD PDU as described in section 5.3.3.2.

[TS 38.322, clause 7.3]

a) t-PollRetransmit

This timer is used by the transmitting side of an AM RLC entity in order to retransmit a poll (see sub clause 5.3.3).

[TS 38.322, clause 7.4]

b) pollPDU

This parameter is used by the transmitting side of each AM RLC entity to trigger a poll for every pollPDU PDUs (see subclause 5.3.3).

c) pollByte

This parameter is used by the transmitting side of each AM RLC entity to trigger a poll for every pollByte bytes (see subclause 5.3.3).

7.1.2.3.6.3	Test description

7.1.2.3.6.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.2.1.1 with the exception that the AM DRB is configured according to Table 7.1.2.3.6.3.1-1.

Table 7.1.2.3.6.3.1-1: RLC parameters

| t-PollRetransmit | ms400 |
| --- | --- |
| pollPDU | p256 |
| pollByte | kB25 |

7.1.2.3.6.3.2	Test procedure sequence

Table 7.1.2.3.6.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | During the whole test sequence, the SS should not allocate UL grants unless when explicitly stated so in the procedure. | - | - | - | - |
| 2 | The SS transmits 4 AMD PDUs such that 1 AMD PDU is sent every two radio frame, each containing an RLC SDU of 976 bits for pc_RedCap_r17=false and 984 bits for pc_RedCap_r17=true. (Note 2) | <-- | AMD PDU (SN=0) AMD PDU (SN=1) AMD PDU (SN=2) AMD PDU (SN=3) | - | - |
| - | EXCEPTION: In parallel to the events described in step 3, the step specified in Table 7.1.2.3.6.3.2-2 should take place. | - | - | - | - |
| 3 | The SS waits for 100 ms after the first DL AMD PDU has been transmitted in step 2, then starts assigning UL grants in every second radio frame of size 1032 bits. (Note 1) (Note 2) | - | - | - | - |
| 4 | Check 1: Does the UE transmit an AMD PDU with a SN in range 0 to 3 and P=1? Record time TB. Check 2: Is (TB – TA) = t-PollRetransmit? | --> | AMD PDU | 2 | P |
| 5 | The SS starts the UL default grant transmission on reception of SR. | - | - | - | - |
| 6 | The SS transmits an RLC Status Report ACKing reception of PDU’s 0-3. | <-- | STATUS PDU | - | - |
| 7 | Check: Does the UE retransmit an AMD PDU within 1 sec? | --> | AMD PDU | 2 | F |
| 8 | The SS transmits NR RRCReconfiguration message changing pollPDU to p4. (Note 3) | <-- | RRCReconfiguration | - | - |
| 8A | The UE transmits a NR RRCReconfigurationcomplete message. (Note 4) | --> | RRCReconfigurationComplete | - | - |
| 9 | The SS stops allocating any UL grant. | - | - | - | - |
| 10 | The SS transmits 8 AMD PDUs such that 1 AMD PDU is sent every second radio frame, each containing an RLC SDU of 976 bits for pc_RedCap_r17=false and 984 bits for pc_RedCap_r17=true. (Note 2) | <-- | AMD PDU (SN=4) AMD PDU (SN=5) ... AMD PDU (SN=11) | - | - |
| - | EXCEPTION: In parallel to the events described in step 11, the step specified in Table 7.1.2.3.6.3.2-3 should take place. | - | - | - | - |
| 11 | The SS waits for 100 ms after the first DL AMD PDU has been transmitted in step 10, then starts assigning UL grants (UL grant allocation type 2) in every second radio frame of size 1032 bits. (Note 1) (Note 2) | - | - | - | - |
| 12 | The SS transmits a Status Report with ACK_SN=12, NACK_SN=4, NACK_SN=5, NACK_SN=6 (constructed by NACK_SN Range), NACK_SN=8 and NACK_SN=9 (constructed by NACK_SN Range). | <-- | STATUS PDU | - | - |
| 12A | Void. | - | - | - | - |
| 13 | Check: Does the UE transmit AMD PDUs with the following SN and P values? AMD PDU, SN=4, P=0 AMD PDU, SN=5, P=0 AMD PDU, SN=6, P=0 AMD PDU, SN=8, P=0 AMD PDU, SN=9, P=1 | --> | AMD PDU (SN=4, P=0) AMD PDU (SN=5, P=0) AMD PDU (SN=6, P=0) AMD PDU (SN=8, P=0) AMD PDU (SN=9, P=1) | 2 | P |
| 14 | The SS starts the UL default grant transmission on reception of SR. | - | - | - | - |
| 15 | The SS transmits a Status Report with ACK_SN=12 and no NACK_SN. | <-- | STATUS PDU | - | - |
| 16 | The SS transmits NR RRCReconfiguration message changing pollPDU to p256. (Note 3) | <-- | RRCReconfiguration | - | - |
| 16A | The UE transmits a NR RRCReconfigurationcomplete message. (Note 4) | --> | RRCReconfigurationComplete | - | - |
| 17 | The SS does not allocate any UL grant. | - | - | - | - |
| 18 | After 500 ms the SS transmits 412 AMD PDUs such that 1 AMD PDU is sent every second radio frame, each containing an RLC SDU of size 976 bits for pc_RedCap_r17=false and 984 bits for pc_RedCap_r17=true. (Note 2) | <-- | AMD PDU (SN=12) AMD PDU (SN=13) ... AMD PDU (SN=423) | - | - |
| - | EXCEPTION: In parallel to the events described in step 19, the steps specified in Table 7.1.2.3.6.3.2-4 should take place. | - | - | - | - |
| 19 | The SS waits for 100 ms after the first DL AMD PDU has been transmitted in step 10, then starts assigning UL grants (UL grant allocation type 2) in every second radio frame of size 1032 bits. (Note 1) (Note 2) | - | - | - | - |
| 20 | The SS starts the UL default grant transmission | - | - | - | - |
| Note 1: UL grant of 1032 bits (LRBs & IMCS as per 38.523-3[3] annex B) is chosen to allow the UE to loop back one SDU of size 976 bits and one short BSR (16 bits) into each MAC PDU sent in the uplink (1032 bits - 24 bit AMD PDU header - 16 bit MAC BSR CE- 16 bit MAC PDU subheader) for pc_RedCap_r17=false, or one SDU of size 984 bits and one short BSR (16 bits) into each MAC PDU sent in the uplink (1032 bits - 16 bit AMD PDU header - 16 bit MAC BSR CE- 16 bit MAC PDU subheader). Note 2: 20ms gap between transmissions both in DL and UL respectively allows TTCN to tolerate one HARQ retransmission (FDD/TDD) per transport block, if such happen (TS 38.523-3 [3]). Note 3: For EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration 36.508 [7] Table 4.6.1-8 using condition EN-DC_EmbedNR_RRCRecon. Note 4: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. |  |  |  |  |  |

Table 7.1.2.3.6.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE transmit 4 AMD PDUs, with only the last one having the poll bit set? Record time TA when the PDU with the poll bit set is received at the SS. | --> | AMD PDUs | 1 | P |

Table 7.1.2.3.6.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE transmit 8 AMD PDUs, with the poll bit set only in the 4th and the 8th PDUs? | --> | AMD PDUs | 3 | P |

Table 7.1.2.3.6.3.2-4: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE transmit 205 AMD PDUs, with the poll bit set only in the last (205th) one? (Note 1) | --> | AMD PDUs | 4 | P |
| 2 | The SS transmits an RLC Status Report. | <-- | STATUS PDU | - | - |
| 3 | Check: Does the UE transmit 205 AMD PDUs, with the poll bit set only in the last (410th) one? (Note 1) | --> | AMD PDUs | 4 | P |
| 4 | The SS transmits an RLC Status Report. | <-- | STATUS PDU | - | - |
| 5 | Check: Does the UE transmit 2 AMD PDUs, with the poll bit set only in the last (412th ) one? | --> | AMD PDUs | 1 | P |
| 6 | The SS transmits an RLC Status Report. | <-- | STATUS PDU | - | - |
| Note 1: (976 bits x 205PDUs) / 8 = 25010 > 25 KB, with 1 kB = 1000 bytes (TS 38.331 [12], clause 3.2) |  |  |  |  |  |

7.1.2.3.6.3.3	Specific message contents

Table 7.1.2.3.6.3.3-1: RRCReconfiguration (steps 8 and 16, Table 7.1.2.3.6.3.2-1)

| Derivation Path: 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| secondaryCellGroup | CellGroupConfig |  | EN-DC |
| nonCriticalExtension SEQUENCE { |  |  | NR |
| masterCellGroup | CellGroupConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.2.3.6.3.3-2: CellGroupConfig (Table 7.1.2.3.6.3.3-1)

| Derivation Path: 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 1 entry |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig | entry 1 |  |
| } |  |  |  |
| mac-CellGroupConfig | Not present |  |  |
| physicalCellGroupConfig | Not present |  |  |
| spCellConfig | Not present |  |  |
| } |  |  |  |

Table 7.1.2.3.6.3.3-3: RLC-BearerConfig (Table 7.1.2.3.6.3.3-2)

| Derivation Path: 38.508-1 [4], Table 4.6.3-148 with condition AM |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-BearerConfig ::= SEQUENCE { |  |  |  |
| logicalChannelIdentity | Set to LCID of the DRB under test |  |  |
| rlc-Config | RLC-Config |  |  |
| } |  |  |  |

Table 7.1.2.3.6.3.3-4: RLC-Config (Table 7.1.2.3.6.3.3-3)

| Derivation Path: 38.508-1 [4], Table 4.6.3-149 with condition AM |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-Config ::= CHOICE { |  |  |  |
| am SEQUENCE { |  |  |  |
| ul-AM-RLC SEQUENCE { |  |  |  |
| sn-FieldLength | Not present |  |  |
| t-PollRetransmit | ms400 |  |  |
| pollPDU | p4 |  | step 8 |
|  | p256 |  | step 16 |
| pollByte | kB25 |  |  |
| } |  |  |  |
| dl-AM-RLC SEQUENCE { |  |  |  |
| sn-FieldLength | Not present |  |  |
| } |  |  |  |

<End of modified section 1>