3GPP TSG-RAN5 Meeting #96-e	R5-224700

# Electronic Meeting, 15th – 26th Aug 2022

Source:	Huawei, HiSilicon

Title:	Discussion paper on legacy test cases adjustment for Redcap

Agenda item:	6.3.25.6

Document for:	Information and Discussion

Background

The Rel-17 NR_redcap_plus_ARCH-UEConTest WI was approved at RAN#95 [1]. In this WI, we will add several RedCap dedicated test cases and need to review all the legacy test cases. This document discusses how to handle the legacy SIG test cases for the RedCap UEs.

Discussion

What’s the scope of legacy SIG test cases for Redcap UEs

Considering Redcap is a Rel-17 WI, we need to at least review all the R15 NR test cases including IDLE, Layer 2, RRC, NAS (Mobility Management + Session management), Multi-Layer in TS 38.523-1 and IMS in TS 34.229-5 since they are the foundation of NR. IMS core spec does not make specific changes for Redcap, so test cases adjustment of this part seams easier than TS 38.523-1.

Besides Rel-15, some Rel-16 features would be optionally supported by UEs, such as power saving. If we could find a way forward on Rel-15 test cases adjustment for Redcap, I think Rel-16 part could follow. To simplify this discussion, we will only focus on the Rel-15 test cases in TS.38.523-1 now and postpone to handle Rel-16 test cases adjustment, maybe to next RAN5 meeting.

Observation 1: If we could find a way forward on Rel-15 test cases adjustment in TS 38.523-1 for Redcap, IMS (TS 34.229-5) and Rel-16 part could follow it.

What’s the default configuration of Redcap test cases

In order to ensure that Redcap UEs can access the NR successfully, we need to introduce some modifications on default configurations. We tried to do this by submitting [2]-[3] at the last RAN5 meeting. Finally, these prose CRs were only endorsed because of ASN.1 frozen issue. But they were also reviewed by RAN5 delegates. We now re-submit these prose CRs for discussion at this meeting. Please see [4] (R5-224724) - [5] (R5-224725).

Default cell configurations

Considering SIG test cases would not go through all the cell channel bandwidth, to simplify the test for redcap, the channel bandwidth of test cell could set 20MHz. Please see [7] (R5-224726) - [8] (R5-224727).

Observation 2: We could use 20MHz channel bandwidth of test cell for FR1 to meet the most test case requirements.

Default messages/IEs configurations

We will re-use almost legacy default configurations for Redcap UE including legacy initial BWP except the following necessary changes. The most changes are about the default configurations captured in TS 38.508-1 and only MSG3 identification will have effect on TS 38.523-3.

- SIB1: SIB1 is to check if Redcap is barred. We need to ensure the SIB1 broadcast the necessary IEs for Redcap UEs to access service. Please see [4] (R5-224724).

- RACH procedure: Msg 3 for UE identification will always be performed in RACH procedure. So we need to make sure SS could identify and process new LCID for Redcap. No related RRC message/IE need to modify but to update TS 38.523-3. Please see [6] R5-224967.

- RRCReconfiguration: SN-FieldLength of PDCP-Config and RLC-Config included by RRCReconfiguration need to be modified for RedCap UE test. For legacy UE, 18bit of SN field length is mandatory supported and 12bit is supported optionally. But Redcap UE is opposite. It mainly affects the Layer 2 test cases. Please see [5] (R5-224725).

In fact, the most of legacy test cases can be tested for Redcap UE if the changes above could be fixed in TS 38.508-1/38.523-3 as default configurations. There is no need to do any changes in prose, such as TC 6.1.1.1.

Observation 3: We could test most of legacy test cases for Redcap UE based on the default configurations in TS 38.508-1/38.523-3.

Generic procedure

In fact, the main thing would affect the generic procedure is the SN field length update for Redcap UE, such as initial access and registration with transmitting RRCReconfiguration message. But now RRCReconfiguration message content will associate with pc_RedCap_r17. Don’t need to do any changes in prose of generic procedure in TS 38.508-1. Please see [5] (R5-224725).

Observation 4: Based on the default cell configurations and messages/IEs values, the generic procedures specified in TS 38.508-1 could be tested normally.

Way forward on legacy test cases adjustment in TS.38.523-1(part 1) for Redcap UEs

In TS 38.523-1 clause 5.3, we introduce the general execution principle for redcap UE. Please see [9] R5-224966. When a Redcap UE want to run the test cases in TS 38.523-1, it should follow the indication in clause 5.3 to use default cell configurations and messages content.

Excluding the test cases that RedCap UE does not support, we can divide the remaining legacy test cases into two parts depending on whether it needs to be modified or not.

Layer 2 gets the most crush because of SN field length. So we will take Layer 2 for example. Now the number of R15 legacy test case in chapter 7 is 81, in which 69 are available for RedCap UE. Due to SN field length, we submit prose CRs R5-224690- R5-224695 and R5-224728- R5-224736, for total of 15. That means only 15 legacy test cases need to adjust for Redcap UE. As for the other chapters, I think the percentage would be smaller.

Observation 5: The percentage of legacy test cases that need to adjust for Redcap UE seems very small.

Proposal 1: For the majority of legacy test cases that do not require any changes, we can re-use them and keep the prose as it is.

Proposal 2: For the minority of legacy test cases that need to be adjusted for Redcap UE, we can also re-use them and directly modify the test cases.

Way forward on applicability in TS.38.523-2(part 2) for Redcap UEs

As for part 2, we still take Layer 2 for example. Please see [10] R5-224696

Proposal 3: We prefer to handle applicability (part 2) for Redcap UEs as [10] R5-224696 if we can reach agreement on Proposal 1-2.

Conclusion

Observation 1: If we could find a way forward on Rel-15 test cases adjustment in TS 38.523-1 for Redcap, IMS (TS 34.229-5) and Rel-16 part could follow it.

Observation 2: We could use 20MHz channel bandwidth of test cell for FR1 to meet the most test case requirements.

Observation 3: We could test most of legacy test cases for Redcap UE based on the default configurations in TS 38.508-1/38.523-3.

Observation 4: Based on the default cell configurations and messages/IEs values, the generic procedures specified in TS 38.508-1 could be tested normally.

Observation 5: The percentage of legacy test cases that need to adjust for Redcap UE seems very small.

Proposal 1: For the majority of legacy test cases that do not require any changes, we can re-use them and keep the prose as it is.

Proposal 2: For the minority of legacy test cases that need to be adjusted for Redcap UE, we can also re-use them and directly modify the test cases.

Proposal 3: We prefer to handle applicability (part 2) for Redcap UEs as [10] R5-224696 if we can reach agreement on Proposal 1-2.

Referring to the discussion on R5-223351 + R5-222804 at the last meeting, we reached agreement that the solutions for RedCap UE and SNPN only UE do not have to be the same. I think we shall figure it out case by case, feature by feature. For Redcap, we’ve reviewed and analyzed all R15 legacy test cases and found that the majority of legacy test cases do not require any changes.

Besides, Redcap would be compatible for vertical feature combos if we can directly adjust the few legacy test cases. Redcap UE which also support any other vertical feature can call the legacy test case with Redcap UE test configurations and environment.

- References

- RP-220830, “New WID on UE Conformance – Support of reduced capability NR devices”, China Unicom, Hisilicon, Ericsson, Huawei, Qualcomm, RAN #95-e, March 2022.

- R5-223411, “Update SIB1 for RedCap test”, Huawei, Hisilicon, RAN5 #95-e, May 2022.

- R5-223412, “Update the SN-FiledLengh of PDCP-Config and RLC-Config for RedCap test”, Huawei, Hisilicon, RAN5 #95-e, May 2022.

- R5-224724 , “Update SIB1 to test RedCap UE”, Huawei, Hisilicon, RAN5 #96-e, Aug 2022.

- R5-224725, “Update the SN-FieldLength of PDCP-Config and RLC-Config for RedCap test”, Huawei, Hisilicon, RAN5 #96-e, Aug 2022.

- R5-224967, “LCID of CCCH and CCCH1 update for RedCap UE”, Huawei, Hisilicon, RAN5 #96-e, Aug 2022.

- R5-224726, “Addition of RedCap default test channel bandwidth for signaling MFBI test”, Huawei, Hisilicon, RAN5 #96-e, Aug 2022.

- R5-224727, “Addition of RedCap default test channel bandwidth for signaling test”, Huawei, Hisilicon, RAN5 #96-e, Aug 2022.

- R5-224966, “Update TS 38.523-1 clause 5.3 for RedCap UE”, Huawei, Hisilicon, RAN5 #96-e, Aug 2022.

- R5-224696, “RedCap UE Test applicability for Legacy test cases”, Huawei, Hisilicon, RAN5 #96-e, Aug 2022.

- R5-223351, “Discussion paper for Rel-15 NR Tests Applicability on SNPN Only UE”, Qualcomm CDMA Technologies, RAN5 #95-e, May 2022.

- R5-222804, “Discussion paper on legacy test cases extension for Redcap”, Huawei, Hisilicon, RAN5 #95-e, May 2022.