3GPP TSG-RAN5 Meeting #96-e	R5-224723

# Electronic Meeting, 15th – 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5115 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE RACS test case 9.2.5.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, RACS-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. The value for the NAS IE in the table shall be shown in ‘xxx’B, otherwise, it’s incorrect for the TTCN implementation. 2. The UE radio capability ID is shown in wrong value. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Corrected the NAS IE Value in the table. - Corrected the UE radio capability ID. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the TC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

9.2.5.3	RACS / Handling of delete indication for NW assigned UE radio capability ID

9.2.5.3.1	Test Purpose (TP)

(1)

with { UE having an applicable UE radio capability ID for the current UE radio configuration for the selected PLMN performs Attach procedure }

ensure that {

when { UE receives ATTACH ACCEPT message with network assigned UE radio capability ID deletion indication }

then { UE deletes any stored Network-assigned UE radio capability IDs and performs Tracking Area update procedure and does not include UE radio capability ID availability indication in TRACKING AREA UPDATE REQUEST message }

}

(2)

with { UE in state EMM-REGISTERED and EMM-IDLE mode and having assigned a new UE Radio Capability ID, enters a tracking area that is not in the list of tracking areas that the UE previously registered in the MME }

ensure that {

when { UE receives TRACKING AREA UPDATE ACCEPT message with network assigned UE radio capability ID deletion indication }

then { UE deletes any stored Network-assigned UE radio capability IDs and performs Tracking Area update procedure and does not include UE radio capability ID availability indication in TRACKING AREA UPDATE REQUEST message }

}

9.2.5.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.3.20.

[TS 24.301, clause 5.3.20]

UE radio capability signalling optimisation (RACS) is a feature that is optional at both the UE and the network and which aims to optimise the transmission of UE radio capability over the radio interface (see 3GPP TS 23.401 [10]). RACS works by assigning an identifier to represent a set of UE radio capabilities. This identifier is called the UE radio capability ID. A UE radio capability ID can be either manufacturer-assigned or network-assigned. The UE radio capability ID is an alternative to the signalling of the radio capabilities container over the radio interface.

In this release of the specification, RACS is not applicable to NB-S1 mode.

If the UE supports RACS:

-	the UE shall indicate support for RACS by setting the RACS bit to "RACS supported" in the UE network capability IE of the ATTACH REQUEST and TRACKING AREA UPDATE REQUEST messages;

-	if the UE performs an attach procedure and the UE has an applicable UE radio capability ID for the current UE radio configuration in the selected network, the UE shall include the UE radio capability ID availability IE in the ATTACH REQUEST message and set the IE to "UE radio capability ID available";

-	if the UE performs a tracking area updating procedure and the UE has an applicable UE radio capability ID for the current UE radio configuration in the selected network, the UE shall include the UE radio capability ID availability IE in the TRACKING AREA UPDATE REQUEST message and set the IE to "UE radio capability ID available";

-	If the UE is requested to provide the UE radio capability ID by the network during a security mode control procedure, the UE shall include the UE radio capability ID in the UE radio capability ID IE of the SECURITY MODE COMPLETE message according to the rules in clause 5.4.3.3.;

…

-	upon receiving a UE radio capability ID deletion indication IE set to "Network-assigned UE radio capability IDs deletion requested" in the ATTACH ACCEPT message in the TRACKING AREA UPDATE ACCEPT message or in the GUTI REALLOCATION COMMAND message, the UE shall delete all network-assigned UE radio capability IDs stored at the UE for the serving network and initiate a tracking area updating procedure. If the UE has an applicable manufacturer-assigned UE radio capability ID for the current UE radio configuration in the selected network, the UE shall include a UE radio capability ID availability IE set to "UE radio capability ID available" in the TRACKING AREA UPDATE REQUEST message.

If the network supports RACS:

-	if the UE has included the UE radio capability ID availability IE in the ATTACH REQUEST message and set the IE to "UE radio capability ID available", the network shall initiate a security mode control procedure to retrieve the UE radio capability ID from the UE;

-	if the UE has included the UE radio capability ID availability IE in the TRACKING AREA UPDATE REQUEST message and set the IE to "UE radio capability ID available", the network may initiate a security mode control procedure to retrieve the UE radio capability ID from the UE;

-	if the UE has included the UE radio capability ID availability IE in the TRACKING AREA UPDATE REQUEST message, set the URCIDA bit to "UE radio capability ID available" in the UE radio capability ID availability IE and no UE radio capability ID is available in the UE context in the MME, the network shall initiate a security mode control procedure to retrieve the UE radio capability ID from the UE;

-	the network may assign a network-assigned UE radio capability ID to a UE which supports RACS by including a UE radio capability ID IE in the ATTACH ACCEPT message, in the TRACKING AREA UPDATE ACCEPT message or in the GUTI REALLOCATION COMMAND message; and

-	the network may trigger the UE to delete all network-assigned UE radio capability IDs stored at the UE for the serving network by including a UE radio capability ID deletion indication IE set to "Network-assigned UE radio capability IDs deletion requested" in the ATTACH ACCEPT message, in the TRACKING AREA UPDATE ACCEPT message or in the GUTI REALLOCATION COMMAND message.

9.2.5.3.3	Test description

9.2.5.3.3.1	Pre-test conditions

System Simulator:

-	cell A (belongs to TAI-1, home PLMN) is set to ''Serving cell'';

-	cell B (belongs to TAI-2, home PLMN) is set to ''Non- Suitable cell'';

UE:

- 	the UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18];

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.5.3.3.2	Test procedure sequence

Table 9.2.5.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | The UE transmits an ATTACH REQUEST message. | --> | ATTACH REQUEST | - | - |
| 3-11 | Steps 5-13 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 12 | SS transmits an ATTACH ACCEPT message including UE radio capability ID deletion indication. | <-- | ATTACH ACCEPT | - | - |
| 13-14 | Steps 15-16 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 15 | Check: Does the UE transmit TRACKING AREA UPDATE REQUEST message without UE radio capability availability indication? | --> | TRACKING AREA UPDATE REQUEST | 1 | P |
| 16 | The SS transmits a TRACKING AREA UPDATE ACCEPT message including a UE radio capability ID. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 17 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 18 | The SS releases the RRC connection. | - | - | - | - |
| - | The following messages are sent and shall be received on cell B. | - | - | - | - |
| 19 | The SS configures: - Cell A as the "Non-Suitable cell". - Cell B as a "Serving cell". | - | - | - | - |
| 20 | The UE transmits a TRACKING AREA UPDATE REQUEST message including the UE radio capability ID availability indication. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 21 | The SS transmits a TRACKING AREA UPDATE ACCEPT message including UE radio capability ID deletion indication. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 22 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 23 | Check: Does the UE transmit TRACKING AREA UPDATE REQUEST message without UE radio capability availability indication? | --> | TRACKING AREA UPDATE REQUEST | 2 | P |
| 24 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 25 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 26 | The SS releases the RRC connection. | - | - | - | - |

9.2.5.3.3.3	Specific message contents

Table 9.2.5.3.3.3-1: Message ATTACH REQUEST (preamble)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE network capability |  |  |  |
| Radio capability signalling optimisation (RACS) capability (octet 10, bit 1) | '1'B | RACS supported |  |

Table 9.2.5.3.3.3-2: Message ATTACH ACCEPT (preamble)

| Derivation path: 36.508 table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE radio capability ID | ‘01000000000010’H | Type Field (TF): 1 Version ID: 00 Radio Configuration Identifier (RCI): 000000000001 |  |

Table 9.2.5.3.3.3-3: Message ATTACH ACCEPT (step 12)

| Derivation path: 36.508 table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE radio capability ID deletion indication | '001'B | Network-assigned UE radio capability IDs deletion requested |  |

Table 9.2.5.3.3.3-4: Message TRACKING AREA UPDATE REQUEST (steps 15, 23)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE radio capability ID availability | '000'B | UE radio capability ID not available |  |

Table 9.2.5.3.3.3-5: Message TRACKING AREA UPDATE ACCEPT (step 16)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE radio capability ID | ‘01000000000020’H | Type Field (TF): 1 Version ID: 00 Radio Configuration Identifier (RCI): 000000000002 |  |

Table 9.2.5.3.3.3-6: Message TRACKING AREA UPDATE REQUEST (step 20)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type | '000'B | TA updating |  |
| UE radio capability ID availability | '001'B | UE radio capability ID available |  |

Table 9.2.5.3.3.3-7: Message TRACKING AREA UPDATE ACCEPT (step 21)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE radio capability ID deletion indication | '001'B | Network-assigned UE radio capability IDs deletion requested |  |

<End of modified section 1>