3GPP TSG-RAN5 Meeting #96-e	R5-224727

# Electronic Meeting, 15th – 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2493 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of RedCap default test channel bandwidth for signaling test |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1,Max bandwidth for RedCap UE is 20MHz for FR1 band, and set initial BWP for RedCap in SIB1 will impact TTCN implement so reducing the bandwidth of Cell is needed for RedCap signalling test. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Reduce the n41,n77,n78 CBW to 20Mhz only for RedCap UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RedCap UE could not test n41,n77,n78. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

Table 6.2.3.1-4A: Test frequencies for NR TDD FR1 bands using 60 MHz channel bandwidth for non-RedCap UE

| NR Band | SCS [kHz] | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA[ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n41 | 30 | 60 | 162 | Downlink & Uplink | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.1.1.41 for bandwidth=60 MHz and SCS=30 kHz. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.1-4B: Test frequencies for NR TDD FR1 bands using 20 MHz channel bandwidth for RedCap UE

| NR Band | SCS [kHz] | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA[ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n41 | 30 | 20 | 51 | Downlink & Uplink | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.1.1.41 for bandwidth=20 MHz and SCS=30 kHz. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.1-5: Test frequencies for NR TDD FR1 bands using 100 MHz channel bandwidth for non-RedCap UE

| NR Band | SCS [kHz] | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA[ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n77 | 30 | 100 | 273 | Downlink | Low, High | Same values as for Low and High range in clause 4.3.1.1.1.77 for bandwidth=100 MHz and SCS=30 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | & | Mid-Low | 3616.68 | 641112 | 3563.22 | 637548 | 12 | 30 | 7896 | 638112 | 12 | 0 | 1 (1) | 26 |
|  |  |  |  | Uplink | Mid-High | 3883.32 | 658888 | 3825.54 | 655036 | 24 |  | 8081 | 655872 | 20 | 0 | 0 (0) | 48 |
| n78 | 30 | 100 | 273 | Downlink | Low, High | Same values as for Low and High range in clause 4.3.1.1.1.78 for bandwidth=100 MHz and SCS=30 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | & | Mid-Low | 3483.33 | 632222 | 3429.87 | 628658 | 12 | 30 | 7804 | 629280 | 22 | 0 | 3 (3) | 30 |
|  |  |  |  | Uplink | Mid-High | 3616.68 | 641112 | 3558.9 | 637260 | 24 |  | 7896 | 638112 | 12 | 0 | 1 (1) | 50 |
| n79 | 30 | 100 | 273 | Downlink | Low, High | Same values as for Low and High range in clause 4.3.1.1.1.79 for bandwidth=100 MHz and SCS=30 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | & | Mid-Low | 4616.67 | 707778 | 4563.21 | 704214 | 12 | 30 | 8592 | 704928 | 18 | 6 | 1 (4) | 38 |
|  |  |  |  | Uplink | Mid-High | 4783.35 | 718890 | 4725.57 | 715038 | 24 |  | 8720 | 717216 | 18 | 54 | 1 (4) | 160 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22] for all bands in the table except for band n79 where Table 13-6 apply. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.1-5A: Test frequencies for NR TDD FR1 bands using 20 MHz channel bandwidth for RedCap UE

| NR Band | SCS [kHz] | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA[ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n77 | 30 | 20 | 51 | Downlink | Low, High | Same values as for Low and High range in clause 4.3.1.1.1.77 for bandwidth=20 MHz and SCS=30 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | & | Mid-Low | 3603.33 | 640222 | 3589.83 | 639322 | 12 | 30 | 7915 | 639936 | 14 | 0 | 3 (3) | 30 |
|  |  |  |  | Uplink | Mid-High | 3896.67 | 659778 | 3878.85 | 658590 | 24 |  | 8118 | 659424 | 18 | 0 | 0 (0) | 48 |
| n78 | 30 | 20 | 51 | Downlink | Low, High | Same values as for Low and High range in clause 4.3.1.1.1.78 for bandwidth=20 MHz and SCS=30 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | & | Mid-Low | 3470.01 | 631334 | 3456.51 | 630434 | 12 | 30 | 7822 | 631008 | 22 | 0 | 1 (1) | 26 |
|  |  |  |  | Uplink | Mid-High | 3630 | 642000 | 3612.18 | 640812 | 24 |  | 7933 | 641664 | 12 | 0 | 1 (1) | 50 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22] for all bands in the table except for band n79 where Table 13-6 apply. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<End of modified section 1>