3GPP TSG-RAN5 Meeting #96-e	R5-224741

# Electronic Meeting, 15th – 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2496 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add Default configuration for DCI format 4_2 scheduling MBS Multicast test |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS_5MBS_5MBUSA-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1, There is no default configuration for DCI format 4_2 scheduling MBS |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Add default configuration for DCI format 4_2 scheduling MBS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | DCI format 4_2 is not configured |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

4.3.6.1.X.3	Physical layer parameters for DCI format 4_2

DCI format 4_2 is used for the scheduling of PDSCH in DL cell.

Default physical layer parameters for DCI format 4_2 are specified in Table 4.3.6.1.X.3-1.

Table 4.3.6.1.X.3-1: Physical layer parameters for DCI format 4_2

| Parameter | Value | Value in binary |
| --- | --- | --- |
| Frequency domain resource assignment | Dependent on test parameters | - |
| Time domain resource assignment | Indicating the first entry of PDSCH-TimeDomainResourceAllocationList to be used | “0000” |
| VRB-to-PRB mapping | Non-interleaved | “0” |
| PRB bundling size indicator | Not present (semi-static PRB bundling) | - |
| Rate matching indicator | Not present | - |
| ZP CSI-RS trigger | Not present | - |
| Modulation and coding scheme (TB1) | Dependent on test parameters | - |
| New data indicator (TB1) | Set for every data transmission/retransmission according to the rules specified in TS 38.321 [20] | - |
| Redundancy version (TB1) | Dependent on test parameters | - |
| Modulation and coding scheme (TB2) | Dependent on test parameters | - |
| New data indicator (TB2) | Set for every data transmission/retransmission according to the rules specified in TS 38.321 [20] | - |
| Redundancy version (TB2) | Dependent on test parameters | - |
| HARQ process number | Dependent on test parameters | - |
| Downlink assignment index | Dependent on test parameters | - |
| PUCCH resource indicator | PUCCH-ResourceId[7] = 6 in pucch-ResourceSetID[1] or PUCCH-ResourceId[7] = 14 in pucch-ResourceSetID[2] as defined in Table 4.6.3-112 (Mapping as per Table 9.2.3-2 in TS 38.213 [22]) | “110” |
| PDSCH-to-HARQ_feedback timing indicator | corresponding to K1 slots as per Table 9.2.3-1 in TS 38.213 [22] and dl-DataToUL-ACK in Table 4.6.3-112 |  |
|  | µ=0 (SCS=15kHz): K1=7 | “101” |
|  | µ=1 (SCS=30kHz): K1=5 | “011” |
|  | µ=3 (SCS=120kHz): K1=8 | “110” |
| Antenna port(s) | DMRS port 0 as per Table 7.3.1.2.2-1 in TS 38.212 [27] (dmrs-Type = DMRS type 1 and maxLength = len1 as per Table 4.6.3-50) | “0000” |
| Transmission configuration indication | Not present (0 bit, tci-PresentInDCI in PDCCH-Config-Multicast is not enabled as per Table 4.6.3-FFS) | - |
| DMRS sequence initialization | Fix length of 1 bit (configured to physCellId as scramblingID0 is not present in Table 4.6.3-50. | physCellId |
| Priority indicator | Not present (0 bit, priorityIndicatorDCI-4-2-r17 is not configured in PDSCH-Config-Multicast as per Table 4.6.3-FFS) | - |
| Enabling/disabling HARQ-ACK feedback indication | Not present (0 bit, harq-FeedbackEnablerMulticast does not indicates dci-enable as per Table 4.6.3-FFS) | - |
| NOTE: The size of DCI format 4_2 is configurable by higher layer parameter sizeDCI-4-2 from 20 bits and up to 140 bits. |  |  |

<End of modified section 1>