3GPP TSG-RAN5 Meeting #96-e	R5-224764

# Online, , 15th Aug 2022 - 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.509 | CR | 0215 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Encoding of ellipsoidPointWithAltitude in UPDATE UE LOCATION INFORMATION |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI10_Test |  |  |  |  |  | Date: |  |  | 2022-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | DLO23..DLO0 = “degreesLongitude” value -8388608..8388607 (two’s complement binary coded as per ASN.1 encoding of an integer value, DLO23 is most significant bit and DLO0 is least significant bit). “degreesLongitude” is encoded as two’s complement binary coded as per ASN.1 encoding of integer value. In the LPP(TS36.355), “degreesLongitude” is encoded an ASN.1 integer value using unaligned PER encoding which does not use ‘two’s complement’ in this context. Therefore, to remove this ambiguity, there are two possibilities: - Two’s complement in a fixed length of 24 bits - As unaligned PER encoding of an INTEGER with the same range It is proposed to use ‘two’s complement in a fixed length of 24 bits’. Further to note, TTCN does not encode ellipsoidPointWithAltitude as the encoded octet string is provided using PIXIT(px_EllipsoidPointWithAltitude). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modified encoding of degreesLongitude to use ‘two’s complement in a fixed length of 24 bits’ |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguious specification leading to different UE implementation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | No TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <Start of Change>

## 6.12	UPDATE UE LOCATION INFORMATION

This message is only sent in the direction SS to UE.

| Information Element | Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- |
| Protocol discriminator | TS 24.007 [5], subclause 11.2.3.1.1 | M | V | ½ |
| Skip indicator | TS 24.007 [5], subclause 11.2.3.1.2 | M | V | ½ |
| Message type |  | M | V | 1 |
| ellipsoidPointWithAltitude | TS 36.355 [37], subclause 6.4.1. | M | V | 8 |
| horizontalVelocity | TS 36.355 [37], subclause 6.4.1. | M | V | 3 |
| gnss-TOD-msec | TS 36.355 [37], subclause 6.5.2.6. | M | V | 3 |

where message type is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 1 | 0 | 1 | 1 | octet 1 |

And where ellipsoidPointWithAltitude is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L1 | DLA22 | DLA21 | DLA20 | DLA19 | DLA18 | DLA17 | DLA16 | octet 1 |
| DLA15 | DLA14 | DLA13 | DLA12 | DLA11 | DLA10 | DLA9 | DLA8 | octet 2 |
| DLA7 | DLA6 | DLA5 | DLA4 | DLA3 | DLA2 | DLA1 | DLA0 | octet 3 |
| DLO23 | DLO22 | DLO21 | DLO20 | DLO19 | DLO18 | DLO17 | DLO16 | octet 4 |
| DLO15 | DLO14 | DLO13 | DLO12 | DLO11 | DLO10 | DLO9 | DLO8 | octet 5 |
| DLO7 | DLO6 | DLO5 | DLO4 | DLO3 | DLO2 | DLO1 | DLO0 | octet 6 |
| A1 | AD14 | AD13 | AD12 | AD11 | AD10 | AD9 | AD8 | octet 7 |
| AD7 | AD6 | AD5 | AD4 | AD3 | AD2 | AD1 | AD0 | octet 8 |

L1 = “latitudeSign” value 0=north and 1=south

DLA22..DLA0 = “degreesLatitude” value 0..8388607 (binary coded, DLA22 is most significant bit and DLA0 is least significant bit).

DLO23..DLO0 = “degreesLongitude” value -8388608..8388607 (two’s complement binary coded in a fixed length of 24 bits, DLO23 is most significant bit and DLO0 is least significant bit).

A1 = “altitudeDirection” value 0=height and 1=depth

AD14..AD0 = “altitude” value 0..32767 (binary coded, AD14 is most significant bit and AD0 is least significant bit).

And where horizontalVelocity is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B8 | B7 | B6 | B5 | B4 | B3 | B2 | B1 | Octet 1 |
| B0 | HS10 | HS9 | HS8 | HS7 | HS6 | HS5 | HS4 | octet 2 |
| HS3 | HS2 | HS1 | HS0 | Reserved |  |  |  | octet 3 |

B8..B0 = “bearing” value 0..359 (binary coded, B8 is most significant bit and B0 is least significant bit).

HS10..HS0 = “horizontalSpeed” value 0..2047 (binary coded, HS10 is most significant bit and HS0 is least significant bit).

And where nss-TOD-msec is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved |  | T21 | T20 | T19 | T18 | T17 | T16 | octet 1 |
| T15 | T14 | T13 | T12 | T11 | T10 | T9 | T8 | octet 2 |
| T7 | T6 | T5 | T4 | T3 | T2 | T1 | T0 | octet 3 |

T21..T0 = “gnss-TOD-msec” value 0.. 3599999 (binary coded, T21 is most significant bit and T0 is least significant bit).

<End of Change>