3GPP TSG-RAN5 Meeting #96-e                                                                 	R5-224806

# Electronic Meeting, 15th Aug 2022 - 26th Aug 2022

Agenda item:	5.4.17

Source: 	Qualcomm Inc

Title: 	Discussion on fading crest factor

Document for:	Discussion and Endorsement

## 1.	Introduction

In [7], fading crest factor for 64QAM scenarios were agreed which made all the 64QAM test points testable for 28GHz and 39 GHz bands. The fading crest factor for 256QAM test points are still pending.

This contribution resubmits the simulation results provided in [7] to discuss further the testability of 256QAM test points. This contribution also discusses the signal crest factor to consider for FR2 CSI test cases.

## 2.	Discussion

In [7], we provided link level simulation results for 64QAM and 256QAM test points

Table 7.1.1_1-1 of TS 38.521-4 [5] summarizes the current achievable max SNR for different bands and channel BW.

Table 7.1.1_1-2 and Table 8.1.1_1-2 summarizes the current testability of the various test points for different bands and channel BW.

## 3.	Simulation results and analysis

Re-pasting the Faded signal PAPR for all test points defined in 38.521-4 from [7].

Table 3-1: Faded signal PAPR for all test points defined in TS 38.521-4

|  | Test# | Fading | MCS | Test SNR | numSamples | Mean Signal Pwr (Rx0/Rx1) | Mean Faded signal Pwr (Rx0/Rx1) | Signal PAPR (Rx0/Rx1) | Faded Signal PAPR (Rx0/Rx1) | Faded Signal PAPR at 1e-6 clipping prob | Faded Signal PAPR at 1e-4 clipping prob | Faded Signal PAPR at 1e-3 clipping prob |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1-1 | TDLC60-300 Low | QPSK | 1.4 | 1.23E+08 | -4.44¦-4.45 | -1.39¦-1.37 | 12.89¦13.08 | 16.87¦17.40 |  |  |  |
| 2 | 1-2 | TDLA30-300 Low | 16QAM | 3.6 |  |  |  |  | leverage from 2-2 |  |  |  |
| 3 | 1-3 | TDLA30-300 XPL Med | 64QAM | 14.2 | 1.23E+08 | -4.44¦-4.45 | -1.52¦-1.39 | 13.04¦13.15 | 17.78¦18.20 | 15.72¦16.21 |  | 8.24¦8.4 |
| 4 | 1-4 | TDLD30-75 Low | 256QAM | 21.9 | 9.22E+07 | -4.58¦-4.59 | -1.72¦-1.60 | 13.31¦13.29 | 17.0¦16.24 | 15.08¦14.82 | 12.62¦12.5 | 10.96¦10.84 |
| 5 | 2-1 | TDLA30-75 Low | QPSK | 5.8 |  |  |  |  | leverage from 2-6 |  |  |  |
| 6 | 2-2 | TDLA30-300 Low | 16QAM | 16 | 1.23E+08 | -4.44¦-4.45 | -1.5¦-1.43 | 12.95¦12.96 | 16.37¦16.52 |  |  |  |
| 7 | 2-6 | TDLA30-75 Low | 64QAM | 20.3 | 3.98E+08 | -4.44 | -1.54 | 13.77¦13.23 | 17.77¦17.01 | 14.73¦14.54 |  |  |
| 8 | 3-1 | TDLA30-75 ULA Medium | 16QAM | 20.7 | 1.23E+08 | -4.44¦-4.45 | -1.71¦-1.78 | 12.95¦12.96 | 16.03¦16.48 | 14.19¦14.55 |  |  |
| 9 | 8.2.2.2.2 | TDLA30-35 ULA High | 256QAM | 6/7, 12/13, 7/8, 20/21 | 9.22E+07 | -4.59¦-4.59 | -1.70¦-1.90 | 12.86¦12.82 | 19.24¦20.72 | 17.69¦19.05 | 14.74¦15.36 | 12.64¦12.96 |
| 10 | 8.3.2.2.1 | TDLA30-35 Low | 16QAM |  | 1.23E+08 | -4.45¦-4.45 | -1.86¦-1.96 | 12.76¦12.85 | 17.80¦17.19 | 15.35¦15.45 |  |  |
| 11 | 8.4.2.2 Test1/2 | TDLA30-35 Low | 64QAM | 0,16 | 1.23E+08 | -4.45¦-4.45 | -1.86¦-2.0 | 13.46¦12.90 | 15.98¦15.86 |  |  |  |
| 12 | 8.4.2.2 Test3 | TDLA30-35 XP High | 64QAM | 16 | 1.23E+08 | -4.45¦-4.45 | -1.80¦-2.07 | 13.25¦13.27 | 16.99¦18.05 | 15.57¦15.54 |  |  |
| 13 | 8.4.2.2 Test3 | TDLA30-35 XP High | 64QAM | 16 | 1.23E+08 | -4.45¦-4.45 | -1.86¦-2.08 | 13.46¦12.90 | 15.42¦15.77 |  |  |  |
| 14 | 8.2.2.2.1.1 | AWGN only | 64QAM | 8/9, 14/15 |  | Signal PAPR already available as part of other cases |  |  |  |  |  |  |

Based on Table 3-1, we have the test points for {64QAM, 256QAM} * {Demod, CSI} cases which have the highest PAPR requirement.

For these test points we simulated the faded signal clipping that a UE under test will be subjected to and ran the throughput vs SNR curve for different faded signal clipping values.

The results are summarized in table 3-2

Table 3-2: Throughput values for key test points at different faded signal clipping probabilities

| Test# | Test SNR | Scenario | 1E-06 | 1E-04 | 1E-03 | 1E-02 |
| --- | --- | --- | --- | --- | --- | --- |
| 1-3 | 14.2 | 64QAM Demod | 100% | 100% | 100% | 100% |
| 2-6 | 20.3 | 64QAM Demod | 100% | 100% | 100% | 100% |
| 1-4 | 22 | 256QAM Demod | 100% | 100% | 100% | 83% |
| 8.2.2.2.2 | 13 | 64QAM CSI | 100% | 100% | 100% | 98% |
| 8.2.2.2.2 | 21 | 256QAM CSI | 100% | 100% | 97% | 85% |
| Note: All throughput values normalized to the no faded signal clipping case |  |  |  |  |  |  |

Observation 1: For 256QAM scenarios, Demod test point shows no throughput degradation for 1e-3 or better faded signal clipping probability and CSI test point shows no throughput degradation for 1e-4 or better faded signal clipping.

In Table 3-3, we summarize the different fading backoff values for 256QAM scenarios at different faded signal clipping probability. This is based on the simulation results summarized in table 3-1.

Table 3-3: Fading backoff values for 256QAM test points at different faded signal clipping probabilities

| Fading backoff (dB) | 256QAM Demod | 256QAM CSI |
| --- | --- | --- |
| current | TBD | TBD |
| 1E-6 clipping prob |  |  |
| 1E-4 clipping prob | 12.62 | 15.36 |
| 1E-3 clipping prob | 10.96 | 12.96 |
| 1E-2 clipping prob |  |  |

Proposal 1a: For 256QAM scenarios, consider fading backoff margin of 12.62 dB for Demod and 15.36 for CSI corresponding to 1e-4 fading signal clipping probability.

Proposal 1b: For 256QAM Demod, consider fading backoff margin of 10.96 dB corresponding to 1e-3 fading signal clipping probability. For 256QAM CSI scenarios, consider fading backoff margin of 15.36 dB corresponding to 1e-4 fading signal clipping probability.

Plugging the fading backoff margin values from prop1 in the TR 38.903 [6] Demod SNR range calculator spreadsheet, we get the updated testability of various points as summarized in table 3-6 and table 3-7.

Table 3-6: Testability of Demod test points based on prop1a

| Test Case | Test point | CHBW / MHz | Fading | SNR test requirement | Test Point Applicability |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | n257 | n258 | n259 | n260 | n261 |
| 7.2.2.2.1_1 | 1-1 | 100 | Yes | 1.4 | x | x | x | x | x |
|  | 1-2 | 100 | Yes | 3.6 | x | x | x | x | x |
|  | 1-3 | 100 | Yes | 14.2 | x | x | x | x | x |
|  | 2-1 | 100 | Yes | 5.8 | x | x | x | x | x |
|  | 2-2 | 100 | Yes | 16 | x | x | x | x | x |
|  | 2-3 | 50 | Yes | 15.7 | x | x | x | x | x |
|  | 2-4 | 200 | Yes | 15.8 | x | x | -1.7 dB | x | x |
|  | 2-5 | 50 | Yes | 16 | x | x | x | X | x |
|  | 2-6 | 100 | Yes | 20.3 | x | x | -3.1 dB | x | x |
| 7.2.2.2.1_2 | 3-1 | 100 | Yes | 20.7 | x | x | -3.5 dB | x | x |
| 7.2.2.2.1_3 | 1-4 | 50 | Yes | 22 | x | x | -3.2 dB | x | x |
| 7.3.2.2.1 | 1-1 | 100 | Yes | 7.7 | x | x | x | x | x |
|  | 1-2 | 100 | Yes | 4.3 | x | x | x | x | x |
| 7.3.2.2.2 | 2-1 | 100 | Yes | 3.2 | x | x | x | x | x |
|  | 2-2 | 100 | Yes | 0.2 | x | x | x | x | x |
| 7.3.2.2.3 | 1-1 | 100 | Yes | 4.7 | x | x | x | x | x |

Table 3-7: Testability of CSI test points based on prop1a

| Test Case | Test point | CHBW / MHz | Fading | SNR test requirement | Test Point Applicability |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | n257 | n258 | n259 | n260 | n261 |
| 8.2.2.2.1.1 | 1 | 100 | No | 9 | x | x | x | x | x |
|  | 2 | 100 | No | 15 | x | x | x | x | x |
| 8.2.2.2.2.1 | 1 | 100 | Yes | 7 | x | x | x | x | x |
|  | 2 | 100 | Yes | 13 | x | x | x | x | x |
| 3 | 50 | Yes | 8 | x | x | x | x | x |
|  | 4 | 50 | Yes | 21 | x | x | -5 dB | -0.9 dB | x |
| 8.4.2.2.1 | 1 | 100 | Yes | 0 | x | x | x | x | x |
|  | 2 | 100 | Yes | 16 | x | x | x | x | x |
|  | 3 | 100 | Yes | 16 | x | x | x | X | x |

Proposal 2: 256QAM test points for PDSCH Demod can be deemed testable for 28GHz and 39 GHz bands. 256QAM test points for CSI cases can be deemed testable for 28 GHz bands. Update the corresponding sections in TS 38.521-4 and TR 38.903 with the updated testability for all applicable bands.

The backoff from P1 dB for without fading cases is still a working assumption. All the test points currently defined for without fading cases are testable for currently defined FR2 bands. So we can either stay with the current assumption of 13 dB backoff from P1 dB or look at the signal backoff corresponding to the agreed faded signal backoff point in the link level simulation curve.

Proposal 3a: Go with current assumption of 13 dB backoff from P1 dB

Proposal 3b: Look at the signal backoff value corresponding to the agreed fading signal backoff point in the link level simulation curve and go with that value.

## 4.	Conclusion

Observation 1: For 256QAM scenarios, Demod test point shows no throughput degradation for 1e-3 or better faded signal clipping probability and CSI test point shows no throughput degradation for 1e-4 or better faded signal clipping.

Proposal 1a: For 256QAM scenarios, consider fading backoff margin of 12.62 dB for Demod and 15.36 for CSI corresponding to 1e-4 fading signal clipping probability.

Proposal 1b: For 256QAM Demod, consider fading backoff margin of 10.96 dB corresponding to 1e-3 fading signal clipping probability. For 256QAM CSI scenarios, consider fading backoff margin of 15.36 dB corresponding to 1e-4 fading signal clipping probability.

Proposal 2: 256QAM test points for PDSCH Demod can be deemed testable for 28GHz and 39 GHz bands. 256QAM test points for CSI cases can be deemed testable for 28 GHz bands. Update the corresponding sections in TS 38.521-4 and TR 38.903 with the updated testability for all applicable bands.

Proposal 3a: Go with current assumption of 13 dB backoff from P1 dB

Proposal 3b: Look at the signal backoff value corresponding to the agreed fading signal backoff point in the link level simulation curve and go with that value.

## 5.		References

- R5-205702, On Fading Impact on CREST and Demod Testability Analysis, Rohde & Schwarz

- R5-206614, On the achievable SNR for FR2 demod and perf test cases, Anritsu

[3]  R5-211083, Input on fading crest factor margin for FR2 Demodulation test cases, Qualcomm Inc

[4]  R5-215829, Discussion on max testable SNR for Demod scenarios, Qualcomm Inc

[5]  TS 38.521-4 v16.11.0

[6]  TR 38.903 v16.11

[7]  R5-221628, Discussion on fading crest factor, Qualcomm Inc