3GPP TSG-RAN5 Meeting #96-e                                                                 	R5-224808

# Electronic Meeting, 15th Aug 2022 - 26th Aug 2022

Agenda item:	5.4.17

Source: 	Qualcomm Inc

Title: 	Applicability of FWA devices for Demod and RRM test cases

Document for:	Discussion and Endorsement

## 1.	Introduction

DEMOD spec [1] and RRM spec [2] currently limit the applicability of test cases to UE power class 3. Also Additionally DEMOD spec [1] limits the test case applicability to IFF setup with QZ <=30cm. This contribution is to discuss extending the DEMOD and RRM test cases to other UE power classes and IFF with QZ >30cm.

## 2.	Discussion

DEMOD spec has below wordings in spec to limit test cases to PC3.

It is primarily due to 3 terms.

- UE REFSENS for PC3 device is used for SNR calculation

- Available DL power assumption for SNR calculation is for IFF 30cm QZ

- Test system DL setting uncertainty

In order to expand the applicability to other device power classes esp PC1, the Demod SNR calculator needs to be redone using PC1 UE REFSENS. PC1 UE REFSENS is better than PC3 UE REFSENS. So that would only make the testable SNR higher compared to existing values.

Observation1: In order to expand the applicability to other device power classes esp PC1, the Demod SNR calculator needs to be redone using PC1 UE REFSENS

Proposal1:  Revise 38.903 Demod SNR calculator to include PC1 and other power classes

For device size >= 55cm, the available DL power at CW 1 dB compression at QZ term needs to be analyzed by TE vendors.

Proposal2: In order to expand the applicability of DEMOD test cases for device size >= 55cm, go with the working assumption of existing available DL power at CW 1 dB compression at QZ.

Test System DL setting uncertainty term in the SNR calculator is based on the MU values specified in Annex B of TR 38.903 [3]. Some of the MU values are already specified for PC1. But some are still missing.

Proposal3: Identify the list of MU terms that are still missing for PC1 and are needed to make the DEMOD and RRM test case applicable.

RRM test cases are applicable for DFF based system. Hence QZ size is not relevant. RRM TT analysis uses UE REFSENS assumption for PC3.

The TT analysis needs to account for UE REFSENS for PC1 and other power classes.

Proposal4: Redo RRM TT analysis to include UE REFSENS for PC1 and other power class

Apart from the UE REFSENS assumptions, the overall Test System DL setting uncertainty also is only defined for PC3. As mentioned in prop3, looking for inputs from RAN5 group on all the missing aspects needed to make DEMOD and RRM test cases applicable for device power class other than PC3 and device sizes >= 55cm.

## 3.	Conclusion

Observation1: In order to expand the applicability to other device power classes esp PC1, the Demod SNR calculator needs to be redone using PC1 UE REFSENS

Proposal1:  Revise 38.903 Demod SNR calculator to include PC1 and other power classes

Proposal2: In order to expand the applicability of DEMOD test cases for device size >= 55cm, go with the working assumption of existing available DL power at CW 1 dB compression at QZ.

Proposal3: Identify the list of MU terms that are still missing for PC1 and are needed to make the DEMOD and RRM test case applicable.

Proposal4: Redo RRM TT analysis to include UE REFSENS for PC1 and other power class

## 4.		References

[1]  TS 38.521-4 v16.11.0

[2]  TS 38.533 v17.3.0

[3]  TR 38.903 v16.12