3GPP TSG-RAN5 Meeting #96-e	R5-224840

# Electronic Meeting, August 15 – 26, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-3 | CR | 2652 | rev | - | Current version: | 16.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to V2X test case 12.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_test, LTE_V2X-UEConTest |  |  |  |  |  | Date: |  |  | 2022-07-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | LTE V2X test case 12.1.2 verifes the transmission timing accuracy requirement when UE is synchonized with a syncRef UE. The minimum timing error requirement is 24*Ts as specified TS 36.133 [4] clause 11.2. However the existing test requirement is not aligned with the minimum requirement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correcting the test requirement. - Correcting the test procedure as UE is synchronized with syncRef UE rather than with E-UTRA cell. - Correcting the messages to adopt the pre-configuration messages for DUT. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case can’t be performed correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

12.1.2	V2X UE Transmission Timing Accuracy Test for SyncRef UE as Timing Reference

12.1.2.1	Test Purpose

The purpose of this test is to verify the timing requirements for V2X sidelink transmissions specified in TS 36.133 [4] clause 13.2.3, when SyncRef UE is used as timing reference. For this test, the UE is triggered by the test loop function to transmit for V2X sidelink communication.

12.1.2.2	Applicability of requirements

The requirements in this clause are applicable to all types of UE that support V2X Communication and Band 47 and SLSS transmission.

12.1.2.3	Minimum Conformance Requirements

For parameters specified in Tables 12.1.2.5.1-1, the timing accuracy for V2X sidelink transmission shall takes place  before the reception of the first detected path (in time) of the corresponding timing reference frame from the SyncRef UE, where   = 0 and . . The timing accuracy is verified by using PSSCH transmissions.

The transmission timing error for sidelink transmissions shall be less than or equal to Te where the timing error limit value Te is specified in Table 12.1.2.3-1.

Table 12.1.2.3-1: Te Timing Error Limit

| Sidelink Bandwidth (MHz) | Te |
| --- | --- |
| ≥1.4 | 24*TS |
| Note: TS is the basic timing unit defined in TS 36.211 |  |

The normative reference for this requirement is 3GPP TS 36.133 [4] clause 11.2, 13.2 and A.12.1.2.

12.1.2.4	Test Description

12.1.2.4.1	Initial Conditions

Resource pool configuration for PSCCH and PSSCH reference measurement channels are set according to Table 12.1.2.5.1-1-1 and Annex A.12 as appropriate.

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 4.1.

Frequencies to be tested: According to 3GPP TS 36.508 [7] clauses 4.4.2 and 4.3.1.

Channel Bandwidth to be tested: 10 MHz as defined in Table 12.1.2.5.1-1.

1.	Connect the SS and AWGN noise sources to the UE antenna connectors, connect the SS COM port to the UE COM port, as shown in 3GPP TS 36.508 [7] Annex A.92b.

2.	The parameter settings for the V2X sidelink transmission over PC5 are pre-configured according to 3GPP TS 36.508 [7] subclause 4.10.1. Message content exceptions are defined in clause 12.1.2.4.3.

3.	Void

4.	The V2X reference measurement channel is set according to Table 12.1.2.5.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in State 5A-V2X according to 3GPP TS 36.508 [7] clause 4.5.9.

Table 12.1.2.4.1-1: Void

12.1.2.4.2	Test Procedure

For this test, the UE is triggered by the test loop function to transmit for V2X sidelink Communication. There is one active SyncRef UE in this test without either serving cell or GNSS signals. Before the test starts, the UE has been synchronized to the SyncRef UE.  The transmit timing accuracy is verified by the UE transmitting PSSCH.

1.	UE transmits V2X sidelink communication data over the PC5 interface according to SL-V2X-Preconfiguration and the V2X RMC for PSCCH and PSSCH according to Table 12.1.2.5.1-1.

2.	After the UE is synchronized to the reference UE, the test system shall verify that the UE PSCCH transmission timing offset is within ± 27×TS with respect to the downlink EUTRA signal.

12.1.2.4.3	Message Contents

Message contents are according to 3GPP TS 36.508 [7] subclause 4.6, 4.7.I and 4.10 with the following exceptions:

Table 12.1.2.4.3-1: SL-V2X-PreconfigFreqInfo-r14-DEFAULT

| Derivation Path: 3GPP TS 36.508 Table 4.6.3-20K |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-V2X-PreconfigFreqInfo-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| v2x-CommPreconfigSync-r14 SEQUENCE { |  |  |  |
| syncOffsetIndicators-r14 SEQUENCE { | 0 |  |  |
| syncOffsetIndicator1-r14 | 0 | Indicates the offset of the first subframe of a resource pool within a SFN cycle |  |
| syncOffsetIndicator2-r14 | 2 | Indicates the offset of the first subframe of a resource pool within a SFN cycle |  |
| } |  |  |  |
| syncTxParametes-r14 | 31 |  |  |
| syncTxThreshOoc-r14 | 1 | -110dBm |  |
| filterCoefficient-r14 | fc0 |  |  |
| syncRefMinHyst-r14 | dB0 |  |  |
| syncRefDiffHyst-r14 | dB0 |  |  |
| } |  |  |  |
| v2x-CommRxPoolList-r14 SEQUENCE (SIZE (1..maxSL-V2X-RxPoolPreconf-r14)) OF SL-V2X-PreconfigCommPool-r14 { | 1 entry |  |  |
| SL-V2X-PreconfigCommPool-r14[1] | SL-V2X-PreconfigCommPool-r14 |  |  |
| } |  |  |  |
| v2x-CommTxPoolList-r14 SEQUENCE (SIZE (1..maxSL-V2X-TxPoolPreconf-r14)) OF SL-V2X-PreconfigCommPool-r14 { |  |  |  |
| SL-V2X-PreconfigCommPool-r14[1] | SL-V2X-PreconfigCommPool-r14 |  |  |
| } |  |  |  |
| v2x-ResourceSelectionConfig-r14 | SL-CommTxPoolSensingConfig-r14-DEFAULT |  |  |
| zoneConfig-r14 | Not present |  |  |
| syncPriority-r14 | enb |  |  |
| thresSL-TxPrioritization-r14 | Not present |  |  |
| offsetDFN-r14 | Not present |  |  |
| } |  |  |  |

Table 12.1.2.4.3-2: SL-V2X-PreconfigCommPool-r14 (Table 12.1.2.4.3-1)

| Derivation Path: 3GPP TS 36.508 Table 4.6.3-20J |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-V2X-PreconfigCommPool-r14 ::= SEQUENCE { |  |  |  |
| sl-OffsetIndicator-r14 | 0 |  |  |
| sl-Subframe-r14 CHOICE { |  |  |  |
| bs100-r14 | 11111111111111111111 11111111111111111111 11111111111111111111 11111111111111111111 11111111111111111111 |  |  |
| } |  |  |  |
| adjacencyPSCCH-PSSCH-r14 | true | BOOLEAN |  |
| sizeSubchannel-r14 | n5 |  |  |
| startRB-Subchannel-r14 | 0 |  |  |
| startRB-PSCCH-Pool-r14 | 0 |  |  |
| } |  |  |  |

Table 12.1.2.4.3-3: SL-V2X-InterFreqUE-Config-r14-DEFAULT (SyncRef UE)

| Derivation Path: 3GPP TS 36.508 Table 4.6.3-20I |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-V2X-InterFreqUE-Config-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| v2x-CommRxPool-r14 SEQUENCE (SIZE (1..maxSL-V2X-RxPool-r14)) OF SL-CommResourcePoolV2X-r14 { |  |  |  |
| SL-CommResourcePoolV2X-r14[1] SEQUENCE { |  |  |  |
| sl-OffsetIndicator-r14 | 0 |  |  |
| sl-Subframe-r14 CHOICE { |  |  |  |
| bs20-r14 | 11111111111111111111 |  |  |
| } |  |  |  |
| adjacencyPSCCH-PSSCH-r14 | true |  |  |
| sizeSubchannel-r14 | n50 |  |  |
| numSubchannel-r14 | n1 |  |  |
| startRB-Subchannel-r14 | 0 |  |  |
| startRB-PSCCH-Pool-r14 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| v2x-CommTxPoolNormal-r14 SEQUENCE (SIZE (1..maxSL-V2X-TxPool-r14)) OF SL-CommResourcePoolV2X-r14 { |  |  |  |
| SL-CommResourcePoolV2X-r14[1] SEQUENCE { |  |  |  |
| sl-OffsetIndicator-r14 | 0 |  |  |
| sl-Subframe-r14 CHOICE { |  |  |  |
| bs20-r14 | 11111111111111110000 |  |  |
| } |  |  |  |
| adjacencyPSCCH-PSSCH-r14 | true |  |  |
| sizeSubchannel-r14 | n50 |  |  |
| numSubchannel-r14 | n1 |  |  |
| startRB-Subchannel-r14 | 0 |  |  |
| startRB-PSCCH-Pool-r14 | 0 |  |  |
| dataTxParameters-r14 SEQUENCE { |  |  |  |
| alpha-r12 | al0 |  |  |
| p0-r12 | 31 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| v2x-CommTxPoolExceptional-r14 SEQUENCE { |  |  |  |
| sl-OffsetIndicator-r14 | 0 |  |  |
| sl-Subframe-r14 CHOICE { |  |  |  |
| bs20-r14 | 00000000000000001111 |  |  |
| } |  |  |  |
| adjacencyPSCCH-PSSCH-r14 | true |  |  |
| sizeSubchannel-r14 | n50 |  |  |
| numSubchannel-r14 | n1 |  |  |
| startRB-Subchannel-r14 | 0 |  |  |
| startRB-PSCCH-Pool-r14 | 0 |  |  |
| dataTxParameters-r14 SEQUENCE { |  |  |  |
| alpha-r12 | al0 |  |  |
| p0-r12 | 31 |  |  |
| } |  |  |  |
| } |  |  |  |
| v2x-ResourceSelectionConfig-r14 SEQUENCE { |  |  |  |
| pssch-TxConfigList-r14 | SL-PSSCH-TxConfig-r14-DEFAULT |  |  |
| restrictResourceReservationPeriod-r14 SEQUENCE (SIZE (1..maxReservationPeriod-r14)) OF SL-RestrictResourceReservationPeriod-r14 { |  |  |  |
| SL-RestrictResourceReservationPeriod-r14[1] | v1 |  |  |
| } |  |  |  |
| probResourceKeep-r14 | v0 |  |  |
| } |  |  |  |
| } |  |  |  |

12.1.2.5	Test Requirement

The reference point for the UE initial transmit timing control test requirement shall be the downlink timing minus place  where  = 0 and

The timing accuracy for V2V sidelink transmission shall be less than or equal to Te where the timing error limit value Te is specified as 27*Ts and 3GPP TS is the basic timing unit defined in 3GPP TS 36.211 [9]. The timing accuracy is verified by using PSSCH transmissions.

Table 12.1.2.5.1-1: Test parameters for V2X UE Transmit Timing Accuracy Test for SyncRef UE as Timing Reference

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | Band 47 TDD |
| Channel Bandwidth (BWchannel) |  | MHz | 10 |  |
| Active cell |  |  | None |  |
| Active SyncRef UE |  |  | SyncRef UE 1 | Transmitting SLSS+MIB-SL on uplink of RF channel number 1 |
| V2X sidelink communication configuration |  |  | As specified in Table A.12.2-2 (Configuration #1) | IE values unless specified otherwise in this test. |
| PSCCH Reference Measurement Channel |  |  | CC.1A | Defined in Table A.12.3-1 |
| PSSCH Reference Measurement Channel |  |  | CD.1A | Defined in Table A.12.3-2 |
|  |  | dBm/15 kHz | -98 |  |
| SyncRef UE 1 | syncCP-Len |  | Normal |  |
|  | syncOffsetIndicator |  | 3 |  |
|  | slssid |  | 30 |  |
|  | inCoverage |  | TRUE | In MIB-SL |
|  | networkControlledSyncTx |  | ON |  |
|  | V2X sidelink communication resource pool configuration |  | As specified in Table A.12.2-2 (Configuration #2) | IE values unless specified otherwise in this test; Note resource pool is same as Configuration #1 used by V2X UE. |
|  |  |  | 3.3 |  |
|  | S-RSRP Note1, Note 2 |  | -94.7 |  |
| Propagation condition |  |  | AWGN |  |
| NOTE 1: S-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 2: SSSS Es/Iot is set the same as PSSS/PSBCH Es/Iot. |  |  |  |  |

<End of Changes>