3GPP TSG-RAN5 Meeting #96-e	R5-224857

# Online, , 15th Aug 2022 - 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1843 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of n3 CBW 35MHz, 45MHz into refsens |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_lic_bands_BW_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test case not completely specified for NR band n3 and CBW 35 and 45 MHz. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added refsens requirements for NR band n3 and CBW 35 and 45 MHz to Table 7.3.2.3-1a, Table 7.3.2.3-3, Table 7.3.2.4.1-2, Table 7.3.2.4.1-3, Table 7.3.2.5-1a and Table 7.3.2.5-2a. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The refsens for NR band n3 CBW 35 and 45 MHz are still missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of changes>>

7.3.2	Reference sensitivity power level

7.3.2.1	Test purpose

The test purpose is to verify the ability of the UE to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise.

7.3.2.2	Test applicability

This test case applies to all types of NR UE release 15 and forward.

7.3.2.3	Minimum conformance requirements

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports for all UE categories, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A.2.3.2, A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.2.3-1a, 7.3.2.3-1b and Table 7.3.2.3-2.

Table 7.3.2.3-1a: Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n1 | 15 | -100.0 | -96.8 | -95.0 | -93.8 | -92.7 | -91.9 |  | -90.6 | -90.1 | -89.6 |
|  | 30 |  | -97.1 | -95.1 | -94.0 | -92.8 | -92.0 |  | -90.7 | -90.2 | -89.7 |
|  | 60 |  | -97.5 | -95.4 | -94.2 | -93.0 | -92.1 |  | -90.9 | -90.3 | -89.7 |
| n2 | 15 | -98 | -94.8 | -93 | -91.8 | -90.7 | -84.1 |  | -81.5 |  |  |
|  | 30 |  | -95.1 | -93.1 | -92 | -90.8 | -84.2 |  | -81.6 |  |  |
|  | 60 |  | -95.5 | -93.4 | -92.2 | -90.9 | -84.3 |  | -81.7 |  |  |
| n3 | 15 | -97.0 | -93.8 | -92.0 | -90.8 | -89.7 | -88.9 | -86.2 | -82.3 | -81.3 | -79.7 |
|  | 30 |  | -94.1 | -92.1 | -91.0 | -89.8 | -89.0 | -86.3 | -82.4 | -81.4 | -79.8 |
|  | 60 |  | -94.5 | -92.4 | -91.2 | -90.0 | -89.1 | -86.4 | -82.6 | -81.5 | -79.9 |
| n5 | 15 | -98.0 | -94.8 | -93.0 | -86.8 | -84.8 |  |  |  |  |  |
|  | 30 |  | -95.1 | -93.1 | -88.6 | -84.9 |  |  |  |  |  |
| n71 | 15 | -98.0 | -94.8 | -93.0 | -91.8 |  |  |  |  |  |  |
|  | 30 |  | -95.1 | -93.1 | -92.0 |  |  |  |  |  |  |
|  | 60 |  | -95.5 | -93.4 | -92.2 |  |  |  |  |  |  |
| n8 | 15 | -97.0 | -93.8 | -91.4 | -85.8 |  |  |  |  |  |  |
|  | 30 |  | -94.1 | -91.7 | -87.2 |  |  |  |  |  |  |
| n12 | 15 | -97.0 | -93.8 | -84.0 |  |  |  |  |  |  |  |
|  | 30 |  | -94.1 | -84.1 |  |  |  |  |  |  |  |
| n14 | 15 | -97.0 | -93.8 |  |  |  |  |  |  |  |  |
|  | 30 |  | -94.1 |  |  |  |  |  |  |  |  |
| n20 | 15 | -97.0 | -93.8 | -91.0 | -89.8 |  |  |  |  |  |  |
|  | 30 |  | -94.1 | -91.1 | -90.0 |  |  |  |  |  |  |
| n24 | 15 | -100.0 | -96.8 |  |  |  |  |  |  |  |  |
|  | 30 |  | -97.1 |  |  |  |  |  |  |  |  |
|  | 60 |  | -97.5 |  |  |  |  |  |  |  |  |
| n25 | 15 | -96.5 | -93.3 | -91.5 | -90.3 | -89.3 | -82.2 |  | -79.5 |  |  |
|  | 30 |  | -93.6 | -91.6 | -90.5 | -89.4 | -82.3 |  | -79.6 |  |  |
|  | 60 |  | -94.0 | -91.9 | -90.7 | -89.6 | -82.4 |  | -79.7 |  |  |
| n26 | 15 | -97.56 | -94.56 | -92.76 | -87.6 |  |  |  |  |  |  |
|  | 30 |  | -94.86 | -92.76 | -87.7 |  |  |  |  |  |  |
| n28 | 15 | -98.5 | -95.5 | -93.5 | -90.8 |  | -78.5 |  |  |  |  |
|  | 30 |  | -95.6 | -93.6 | -91.0 |  | -78.6 |  |  |  |  |
| n30 | 15 | -99.0 | -95.8 |  |  |  |  |  |  |  |  |
|  | 30 |  | -96.1 |  |  |  |  |  |  |  |  |
| n65 | 15 | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  | -89.2 |
|  | 30 |  | -96.6 | -94.6 | -93.5 |  |  |  |  |  | -89.3 |
|  | 60 |  | -97.0 | -94.9 | -93.7 |  |  |  |  |  | -89.4 |
| n66 | 15 | -99.5 | -96.3 | -94.5 | -93.3 | -92.2 | -91.4 |  | -90.1 | -89.6 |  |
|  | 30 |  | -96.6 | -94.6 | -93.5 | -92.3 | -91.5 |  | -90.2 | -89.7 |  |
|  | 60 |  | -97.0 | -94.9 | -93.7 | -92.5 | -91.6 |  | -90.4 | -89.8 |  |
|  | 15 | -100.0 | -96.8 | -95.0 | -93.8 | -92.7 |  |  |  |  |  |
| n70 | 30 |  | -97.1 | -95.1 | -94.0 | -92.8 |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 | -93.0 |  |  |  |  |  |
| n71 | 15 | -97.2 | -94.0 | -91.6 | -86.0 |  |  |  |  |  |  |
|  | 30 |  | -94.3 | -91.9 | -87.4 |  |  |  |  |  |  |
| n74 | 15 | -99.53 | -96.33 | -94.53 | -89.33 |  |  |  |  |  |  |
|  | 30 |  | -96.63 | -94.63 | -89.53 |  |  |  |  |  |  |
|  | 60 |  | -97.03 | -94.93 | -89.63 |  |  |  |  |  |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 3: The requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9 - 1510.9 MHz. NOTE 4: Void NOTE 5: Void NOTE 6: Values are modified by -0.5dB when carrier channel BW is between 865MHz and 894MHz. NOTE 7: Void. |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2.3-1b: Two antenna port reference sensitivity QPSK PREFSENS for TDD, SDL and FDD with variable duplex operation bands

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm)8 | Duplex Mode |
| n34 | 15 | 5, 10, 15 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15 | -97.5 + 10log10(NRB/11) |  |
| n381 | 15 | 5, 10, 15, 20, 25, 30, 40 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 40 | -97.5 + 10log10(NRB/11) |  |
| n39 | 15 | 5, 10, 15, 20, 25, 30, 40 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 40 | -97.5 + 10log10(NRB/11) |  |
| n40 | 15 | 5, 10, 15, 20, 25, 30, 40, 50 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | -97.5 + 10log10(NRB/11) |  |
| n411 | 15 | 10, 15, 20, 30, 40, 50 | -94.8 + 10log10(NRB/50) | TDD |
|  | 30 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -95.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -95.5 + 10log10(NRB/11) |  |
| n481 | 15 | 5, 10, 15, 20, 30, 40, 505 | -99 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 30, 40, 505, 605, 705, 805, 905, 1005 | -96.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 30, 40, 505, 605, 705, 805, 905, 1005 | -96.5 + 10log10(NRB/11) |  |
| n50 | 15 | 5, 10, 15, 20, 30, 40, 50 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 30, 40, 50, 60, 80 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 30, 40, 50, 60, 80 | -97.5 + 10log10(NRB/11) |  |
| n51 | 15 | 5 | -100 | TDD |
| n53 | 15 | 5, 10 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10 | -97.1 |  |
|  | 60 | 10 | -97.5 |  |
| n757 | 15 | 5,10,15,20 | -100 + 10log10(NRB/25) | SDL |
|  | 30 | 10,15,20 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10,15,20 | -97.5 + 10log10(NRB/11) |  |
| n767 | 15 | 5 | -100 | SDL |
| n771,4 | 15 | 10, 15, 20, 40, 50 | -95.3 + 10log10(NRB/50) | TDD |
|  | 30 | 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 | -95.6 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 | -96.0 + 10log10(NRB/11) |  |
| n781 | 15 | 10, 15, 20, 40, 50 | -95.8 + 10log10(NRB/50) | TDD |
|  | 30 | 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 | -96.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 | -96.5 + 10log10(NRB/11) |  |
| n791 | 15 | 40, 50 | -89.6 + 10log10(NRB/216) | TDD |
|  | 30 | 40, 50, 60, 80, 100 | -89.7 + 10log10(NRB/106) |  |
|  | 60 | 40, 50, 60, 80, 100 | -89.9 + 10log10(NRB/51) |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 3: Void NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. NOTE 6: Void NOTE 7: For SDL bands, the reference sensitivity requirements shall be verified by inter-band CA combinations with SDL band, which are supported by UE. NOTE 8: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. |  |  |  |  |

For UE(s) equipped with 4 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3.2.3-1a and in Table 7.3.2.3-1b shall be modified by the amount given in ΔRIB,4R in Table 7.3.2.3-2 for the applicable operating bands.

Table 7.3.2.3-2: Four antenna port reference sensitivity allowance ΔRIB,4R

| Operating band | ΔRIB,4R (dB) |
| --- | --- |
| n28, n71 | -2.71 |
| n1, n2, n3, n30, n40, n7, n34, n38, n39, n41, n66, n70 | -2.7 |
| n48, n77, n78, n79 | -2.2 |
| NOTE 1: 4 Rx operation is targeted for FWA form factor |  |

The reference sensitivity (REFSENS) requirement specified in Table 7.3.2.3-1a, Table 7.3.2.3-1b and Table 7.3.2.3-2 shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3.2.3-3.

Table 7.3.2.3-3: Uplink configuration for reference sensitivity

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 | Duplex Mode |
| n1 | 15 | 25 | 501 | 751 | 1001 | 1281 | 1281 |  | 1281 | 1281 | 1281 |  |  |  |  |  | FDD |
|  | 30 |  | 24 | 361 | 501 | 641 | 641 |  | 641 | 641 | 641 |  |  |  |  |  |  |
|  | 60 |  | 101 | 18 | 24 | 301 | 301 |  | 301 | 301 | 301 |  |  |  |  |  |  |
| n2 | 15 | 25 | 501 | 501 | 501 | 501 | 481 |  | 401 |  |  |  |  |  |  |  | FDD |
|  | 30 | 101 | 24 | 241 | 241 | 241 | 241 |  | 201 |  |  |  |  |  |  |  |  |
|  | 60 |  | 101 | 101 | 101 | 101 | 101 |  | 101 |  |  |  |  |  |  |  |  |
| n3 | 15 | 25 | 501 | 501 | 501 | 501 | 501 | 501 | 501 | 501 | 501 |  |  |  |  |  | FDD |
|  | 30 |  | 24 | 241 | 241 | 241 | 241 | 241 | 241 | 241 | 241 |  |  |  |  |  |  |
|  | 60 |  | 101 | 101 | 101 | 101 | 101 | 101 | 101 | 101 | 101 |  |  |  |  |  |  |
| n5 | 15 | 25 | 251 | 251 | 251 | Note 5 |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 101 | 101 | 101 | Note 5 |  |  |  |  |  |  |  |  |  |  |  |
| n7 | 15 | 25 | 501 | 751 | 751 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 24 | 361 | 361 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | 101 | 18 | 181 |  |  |  |  |  |  |  |  |  |  |  |  |
| n8 | 15 | 25 | 251 | 251 | 251 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 101 | 101 | 101 |  |  |  |  |  |  |  |  |  |  |  |  |
| n12 | 15 | 201 | 201 | 201 |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 101 | 101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n14 | 15 | 201 | 201 |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n20 | 15 | 25 | 201 | 202 | 202 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 101 | 102 | 102 |  |  |  |  |  |  |  |  |  |  |  |  |
| n24 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 24 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n25 | 15 | 25 | 501 | 501 | 501 | 501 | 481 |  | 401 |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 24 | 241 | 241 | 241 | 241 |  | 201 |  |  |  |  |  |  |  |  |
|  | 60 |  | 101 | 101 | 101 | 101 | 101 |  | 101 |  |  |  |  |  |  |  |  |
| n26 | 15 | 25 | 251 | 251 | 251 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 121 | 121 | 121 |  |  |  |  |  |  |  |  |  |  |  |  |
| n28 | 15 | 25 | 251 | 251 | 251 |  | 251 |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 101 | 101 | 101 |  | 101 |  |  |  |  |  |  |  |  |  |  |
| n30 | 15 | 201 | 201 |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n34 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |  |  |  |  | TDD |
|  | 30 |  | 24 | 36 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | 10 | 18 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n38 | 15 | 25 | 50 | 75 | 100 | 128 | 160 |  | 216 |  |  |  |  |  |  |  | TDD |
|  | 30 |  | 24 | 36 | 50 | 64 | 75 |  | 100 |  |  |  |  |  |  |  |  |
|  | 60 |  | 10 | 18 | 24 | 30 | 36 |  | 50 |  |  |  |  |  |  |  |  |
| n39 | 15 | 25 | 50 | 75 | 100 | 128 | 160 |  | 216 |  |  |  |  |  |  |  | TDD |
|  | 30 |  | 24 | 36 | 50 | 64 | 75 |  | 100 |  |  |  |  |  |  |  |  |
|  | 60 |  | 10 | 18 | 24 | 30 | 36 |  | 50 |  |  |  |  |  |  |  |  |
| n40 | 15 | 25 | 50 | 75 | 100 | 128 | 160 |  | 216 |  | 270 |  |  |  |  |  | TDD |
|  | 30 |  | 24 | 36 | 50 | 64 | 75 |  | 100 |  | 128 | 162 |  | 216 |  |  |  |
|  | 60 |  | 10 | 18 | 24 | 30 | 36 |  | 50 |  | 64 | 75 |  | 100 |  |  |  |
| n41 | 15 |  | 50 | 75 | 100 |  | 160 |  | 216 |  | 270 |  |  |  |  |  | TDD |
|  | 30 |  | 24 | 36 | 50 |  | 75 |  | 100 |  | 128 | 162 | 180 | 216f | 243 | 270 |  |
|  | 60 |  | 10 | 18 | 24 |  | 36 |  | 50 |  | 64 | 75 | 90 | 100 | 120 | 135 |  |
| n48 | 15 | 25 | 50 | 75 | 100 |  | 160 |  | 216 |  |  |  |  |  |  |  | TDD |
|  | 30 |  | 24 | 36 | 50 |  | 75 |  | 100 |  |  |  |  |  |  |  |  |
|  | 60 |  | 10 | 18 | 24 |  | 36 |  | 50 |  |  |  |  |  |  |  |  |
| n50 | 15 | 25 | 50 | 75 | 100 |  | 160 |  | 216 |  | 270 |  |  |  |  |  | TDD |
|  | 30 |  | 24 | 36 | 50 |  | 75 |  | 100 |  | 128 | 162 |  | Note 3 |  |  |  |
|  | 60 |  | 10 | 18 | 24 |  | 36 |  | 50 |  | 64 | 75 |  | Note 3 |  |  |  |
| n51 | 15 | 25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | TDD |
| n53 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |  |  |  |  | TDD |
|  | 30 |  | 24 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n65 | 15 | 25 | 501 | 751 | 1001 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 24 | 361 | 501 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | 101 | 18 | 24 |  |  |  |  |  |  |  |  |  |  |  |  |
| n66 | 15 | 25 | 501 | 751 | 1001 | 1281 | 160 |  | 216 |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 24 | 361 | 501 | 641 | 751 |  | 1001 |  |  |  |  |  |  |  |  |
|  | 60 |  | 101 | 18 | 24 | 301 | 361 |  | 501 |  |  |  |  |  |  |  |  |
| n70 | 15 | 25 | 501 | 751 | Note 3 | Note 3 |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 24 | 361 | Note 3 | Note 3 |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | 101 | 18 | Note 3 | Note 3 |  |  |  |  |  |  |  |  |  |  |  |
| n71 | 15 | 25 | 251 | 201 | 201 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 121 | 101 | 101 |  |  |  |  |  |  |  |  |  |  |  |  |
| n74 | 15 | 25 | 251 | 251 | 251 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 101 | 101 | 101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | 51 | 51 | 51 |  |  |  |  |  |  |  |  |  |  |  |  |
| n77 | 15 |  | 50 | 75 | 100 |  |  |  | 216 |  | 270 |  |  |  |  |  | TDD |
|  | 30 |  | 24 | 36 | 50 |  |  |  | 100 |  | 128 | 162 | 180 | 216 | 243 | 270 |  |
|  | 60 |  | 10 | 18 | 24 |  |  |  | 50 |  | 64 | 75 | 90 | 100 | 120 | 135 |  |
| n78 | 15 |  | 50 | 75 | 100 |  |  |  | 216 |  | 270 |  |  |  |  |  | TDD |
|  | 30 |  | 24 | 36 | 50 |  |  |  | 100 |  | 128 | 162 | 180 | 216 | 243 | 270 |  |
|  | 60 |  | 10 | 18 | 24 |  |  |  | 50 |  | 64 | 75 | 90 | 100 | 120 | 135 |  |
| n79 | 15 |  |  |  |  |  |  |  | 216 |  | 270 |  |  |  |  |  | TDD |
|  | 30 |  |  |  |  |  |  |  | 100 |  | 128 | 162 |  | 216 |  | 270 |  |
|  | 60 |  |  |  |  |  |  |  | 50 |  | 64 | 75 |  | 100 |  | 135 |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). NOTE 2: For Band 20; for 15kHz SCS, in the case of 15MHz channel bandwidth, the UL resource blocks shall be located at RBstart 11 and in the case of 20MHz channel bandwidth, the UL resource blocks shall be located at RBstart 16; for 30kHz SCS, in the case of 15MHz channel bandwidth, the UL resource blocks shall be located at RBstart 6 and in the case of 20MHz channel bandwidth, the UL resource blocks shall be located at RBstart 8; for 60kHz SCS, in the case of 15MHz channel bandwidth, the UL resource blocks shall be located at RBstart 3 and in the case of 20MHz channel bandwidth, the UL resource blocks shall be located at RBstart 4. NOTE 3: For DL channel bandwidths that do not have symmetric UL channel bandwidth, highest valid UL configuration with lowest duplex distance shall be used. NOTE 4: NOTE 5: For this DL channel bandwidth, the UL configuration of the highest UL channel bandwidth specified in Table 5.3.6-1 and the default Tx-Rx frequency separation specified in Table 5.4.4-1 shall be used. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Unless given by Table 7.3.2.3-4, the minimum requirements specified in Tables 7.3.2.3-1 and 7.3.2.3-2 shall be verified with the network signalling value NS_01 (Table 6.2.3.3-1) configured.

Table 7.3.2.3-4: Network signalling value for reference sensitivity

| Operating band | Network Signalling value |
| --- | --- |
| n2 | NS_03 |
| n12 | NS_06 |
| n14 | NS_06 |
| n24 | NS_56 |
| n25 | NS_03 |
| n30 | NS_21 |
| n48 | NS_27 |
| n53 | NS_45 |
| n66 | NS_03 |
| n70 | NS_03 |
| n71 | NS_35 |

For the UE which supports inter-band carrier aggregation, the minimum requirement for reference sensitivity in Table 7.3.2.3-1 shall be increased by the amount given in ΔRIB,c defined in subclause 7.3.3 for the applicable operating bands.

The normative reference for this requirement is TS 38.101-1 [2] clause 7.3.2.

7.3.2.4	Test description

7.3.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 7.3.2.4.1-1, Table 7.3.2.4.1-2, and Table 7.3.2.4.1-3 The details of the uplink reference measurement channels (RMCs) are specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.3.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1 |  |  | Low range, Mid range, High range (NOTE 4) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest (NOTE 4) Lowest UL / Lowest DL, Lowest UL / Highest DL (NOTE 3) |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration |  | Uplink Configuration |  |
|  | Modulation | RB allocation | Modulation | RB allocation |
| 1 | CP-OFDM QPSK | Full RB (NOTE 1) | DFT-s-OFDM QPSK | REFSENS (NOTE 2) |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. NOTE 3: Additional test points selected according to asymmetric channel bandwidths specified in clause 5.3.6. DL channel bandwidth shall be selected first. NOTE 4: For NR band n28, 30MHz test channel bandwidth is tested with Low range and High range test frequencies. NOTE 5: In a band where UE supports 4Rx, the test needs to be repeated with only 2Rx antennas connected and the other antennas terminated. |  |  |  |  |

Table 7.3.2.4.1-2: Downlink Configuration of each RB allocation

| Channel Bandwidth | SCS(kHz) | LCRBmax | Outer RB allocation / Normal RB allocation |
| --- | --- | --- | --- |
| 5MHz | 15 | 25 | 25@0 |
|  | 30 | 11 | 11@0 |
|  | 60 | N/A | N/A |
| 10MHz | 15 | 52 | 52@0 |
|  | 30 | 24 | 24@0 |
|  | 60 | 11 | 11@0 |
| 15MHz | 15 | 79 | 79@0 |
|  | 30 | 38 | 38@0 |
|  | 60 | 18 | 18@0 |
| 20MHz | 15 | 106 | 106@0 |
|  | 30 | 51 | 51@0 |
|  | 60 | 24 | 24@0 |
| 25MHz | 15 | 133 | 133@0 |
|  | 30 | 65 | 65@0 |
|  | 60 | 31 | 31@0 |
| 30MHz | 15 | 160 | 160@0 |
|  | 30 | 78 | 78@0 |
|  | 60 | 38 | 38@0 |
| 35MHz | 15 | 188 | 188@0 |
|  | 30 | 92 | 92@0 |
|  | 60 | 44 | 38@0 |
| 40MHz | 15 | 216 | 216@0 |
|  | 30 | 106 | 106@0 |
|  | 60 | 51 | 51@0 |
| 45MHz | 15 | 128 | 128@0 |
|  | 30 | 64 | 64@0 |
|  | 60 | 30 | 30@0 |
| 50MHz | 15 | 270 | 270@0 |
|  | 30 | 133 | 133@0 |
|  | 60 | 65 | 65@0 |
| 60MHz | 15 | N/A | N/A |
|  | 30 | 162 | 162@0 |
|  | 60 | 79 | 79@0 |
| 70MHz | 15 | N/A | N/A |
|  | 30 | 189 | 189@0 |
|  | 60 | 93 | 93@0 |
| 80MHz | 15 | N/A | N/A |
|  | 30 | 217 | 217@0 |
|  | 60 | 107 | 107@0 |
| 90MHz | 15 | N/A | N/A |
|  | 30 | 245 | 245@0 |
|  | 60 | 121 | 121@0 |
| 100MHz | 15 | N/A | N/A |
|  | 30 | 273 | 273@0 |
|  | 60 | 135 | 135@0 |
| NOTE 1: Test Channel Bandwidths are checked separately for each NR band, the applicable channel bandwidths are specified in Table 5.3.5-1. |  |  |  |

Table 7.3.2.4.1-3: Uplink configuration for reference sensitivity, LCRB @ RBstart format

| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 | 15 | 25@0 | 50@21 | 75@41 | 100@61 | 128@51 | 128@321 |  | 128@881 | 128@1141 | 128@1421 |  |  |  |  |  | FDD |
|  | 30 |  | 24@0 | 36@21 | 50@11 | 64@11 | 64@141 |  | 64@421 | 64@551 | 64@691 |  |  |  |  |  |  |
|  | 60 |  | 10@11 | 18@0 | 24@0 | 30@11 | 30@81 |  | 30@211 | 30@281 | 30@351 |  |  |  |  |  |  |
| n2 | 15 | 25@0 | 50@21 | 50@291 | 50@561 | 50@831 | 48@1121 |  | 40@1761 |  |  |  |  |  |  |  | FDD |
|  | 30 | 10@11 | 24@0 | 24@141 | 24@271 | 24@411 | 24@541 |  | 20@861 |  |  |  |  |  |  |  |  |
|  | 60 |  | 10@11 | 10@81 | 10@141 | 10@211 | 10@281 |  | 10@411 |  |  |  |  |  |  |  |  |
| n3 | 15 | 25@0 | 50@21 | 50@291 | 50@561 | 50@831 | 50@1101 | 50@1381 | 50@1661 | 50@1921 | 50@2201 |  |  |  |  |  | FDD |
|  | 30 |  | 24@0 | 24@141 | 24@271 | 24@411 | 24@541 | 24@681 | 24@821 | 24@951 | 24@1091 |  |  |  |  |  |  |
|  | 60 |  | 10@11 | 10@81 | 10@141 | 10@211 | 10@281 | 10@341 | 10@411 | 10@481 | 10@551 |  |  |  |  |  |  |
| n5 | 15 | 25@0 | 25@271 | 25@541 | 25@811 | Note 5 |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 10@141 | 10@281 | 10@411 | Note 5 |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n7 | 15 | 25@0 | 50@21 | 75@41 | 75@311 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 24@0 | 36@21 | 36@151 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | 10@11 | 18@0 | 18@61 |  |  |  |  |  |  |  |  |  |  |  |  |
| n8 | 15 | 25@0 | 25@271 | 25@541 | 25@811 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 10@141 | 10@281 | 10@411 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n12 | 15 | 20@51 | 20@321 | 20@591 |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 10@141 | 10@281 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n14 | 15 | 20@51 | 20@321 |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 10@141 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n20 | 15 | 25@0 | 20@01 | 20@112 | 20@162 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 10@01 | 10@62 | 10@82 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n24 | 15 | 25@0 | 50@0 |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 24@0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | 10@0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n25 | 15 | 25@0 | 50@0 | 50@291 | 50@561 | 50@831 | 48@1121 |  | 40@1761 |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 24@0 | 24@141 | 24@271 | 24@411 | 24@541 |  | 20@861 |  |  |  |  |  |  |  |  |
|  | 60 |  | 10@0 | 10@81 | 10@141 | 10@211 | 10@281 |  | 10@411 |  |  |  |  |  |  |  |  |
| n26 | 15 | 25@0 | 25@271 | 25@541 | 25@811 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 12@121 | 12@261 | 12@391 |  |  |  |  |  |  |  |  |  |  |  |  |
| n28 | 15 | 25@0 | 25@271 | 25@541 | 25@811 |  | 25@1351 |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 10@141 | 10@281 | 10@411 |  | 10@681 |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n30 | 15 | 20@51 | 20@321 |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 10@141 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n34 | 15 | 25@0 | 50@0 | 75@0 |  |  |  |  |  |  |  |  |  |  |  |  | TDD |
|  | 30 |  | 24@0 | 36@0 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | 10@0 | 18@0 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n38 | 15 | 25@0 | 50@0 | 75@0 | 100@0 | 128@0 | 160@0 |  | 216@0 |  |  |  |  |  |  |  | TDD |
|  | 30 |  | 24@0 | 36@0 | 50@0 | 64@0 | 75@0 |  | 100@0 |  |  |  |  |  |  |  |  |
|  | 60 |  | 10@0 | 18@0 | 24@0 | 30@0 | 36@0 |  | 50@0 |  |  |  |  |  |  |  |  |
| n39 | 15 | 25@0 | 50@0 | 75@0 | 100@0 | 128@0 | 160@0 |  | 216@0 |  |  |  |  |  |  |  | TDD |
|  | 30 |  | 24@0 | 36@0 | 50@0 | 64@0 | 75@0 |  | 100@0 |  |  |  |  |  |  |  |  |
|  | 60 |  | 10@0 | 18@0 | 24@0 | 30@0 | 36@0 |  | 50@0 |  |  |  |  |  |  |  |  |
| n40 | 15 | 25@0 | 50@0 | 75@0 | 100@0 | 128@0 | 160@0 |  | 216@0 |  | 270@0 |  |  |  |  |  | TDD |
|  | 30 |  | 24@0 | 36@0 | 50@0 | 64@0 | 75@0 |  | 100@0 |  | 128@0 | 162@0 |  | 216@0 |  |  |  |
|  | 60 |  | 10@0 | 18@0 | 24@0 | 30@0 | 36@0 |  | 50@0 |  | 64@0 | 75@0 |  | 100@0 |  |  |  |
| n41 | 15 |  | 50@0 | 75@0 | 100@0 |  | 160@0 |  | 216@0 |  | 270@0 |  |  |  |  |  | TDD |
|  | 30 |  | 24@0 | 36@0 | 50@0 |  | 75@0 |  | 100@0 |  | 128@0 | 162@0 | 180@0 | 216@0 | 243@0 | 270@0 |  |
|  | 60 |  | 10@0 | 18@0 | 24@0 |  | 36@0 |  | 50@0 |  | 64@0 | 75@0 | 90@0 | 100@0 | 120@0 | 135@0 |  |
| n48 | 15 | 25@0 | 50@0 | 75@0 | 100@0 |  | 160@0 |  | 216@0 |  |  |  |  |  |  |  | TDD |
|  | 30 |  | 24@0 | 36@0 | 50@0 |  | 75@0 |  | 100@0 |  |  |  |  |  |  |  |  |
|  | 60 |  | 10@0 | 18@0 | 24@0 |  | 36@0 |  | 50@0 |  |  |  |  |  |  |  |  |
| n50 | 15 | 25@0 | 50@0 | 75@0 | 100@0 |  |  |  | 216@0 |  | 270@0 |  |  |  |  |  | TDD |
|  | 30 |  | 24@0 | 36@0 | 50@0 |  |  |  | 100@0 |  | 128@0 | 162@0 |  | NOTE 3 |  |  |  |
|  | 60 |  | 10@0 | 18@0 | 24@0 |  |  |  | 50@0 |  | 64@0 | 75@0 |  | NOTE 3 |  |  |  |
| n51 | 15 | 25@0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | TDD |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n53 | 15 | 25@0 | 50@0 |  |  |  |  |  |  |  |  |  |  |  |  |  | TDD |
|  | 30 |  | 24@0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | 10@0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n65 | 15 | 25@0 | 50@21 | 75@41 | 100@61 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 24@0 | 36@21 | 50@11 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | 10@11 | 18@0 | 24@0 |  |  |  |  |  |  |  |  |  |  |  |  |
| n66 | 15 | 25@0 | 50@21 | 75@41 | 100@61 | 128@51 | 160@0 |  | 216@0 |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 24@0 | 36@21 | 50@11 | 64@11 | 75@31 |  | 100@61 |  |  |  |  |  |  |  |  |
|  | 60 |  | 10@11 | 18@0 | 24@0 | 30@11 | 36@21 |  | 50@11 |  |  |  |  |  |  |  |  |
| n70 | 15 | 25@0 | 50@21 | 75@41 | NOTE 3 | NOTE 3 |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 24@0 | 36@21 | NOTE 3 | NOTE 3 |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | 10@11 | 18@0 | NOTE 3 | NOTE 3 |  |  |  |  |  |  |  |  |  |  |  |
| n71 | 15 | 25@0 | 25@01 | 20@01 | 20@01 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 12@01 | 10@01 | 10@01 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n74 | 15 | 25@0 | 25@271 | 25@541 | 25@811 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 10@141 | 10@281 | 10@411 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | 5@61 | 5@131 | 5@191 |  |  |  |  |  |  |  |  |  |  |  |  |
| n77 | 15 |  | 50@0 | 75@0 | 100@0 |  |  |  | 216@0 |  | 270@0 |  |  |  |  |  | TDD |
|  | 30 |  | 24@0 | 36@0 | 50@0 |  |  |  | 100@0 |  | 128@0 | 162@0 | 180@0 | 216@0 | 243@0 | 270@0 |  |
|  | 60 | - | 10@0 | 18@0 | 24@0 |  |  |  | 50@0 |  | 64@0 | 75@0 | 90@0 | 100@0 | 120@0 | 135@0 |  |
| n78 | 15 |  | 50@0 | 75@0 | 100@0 |  |  |  | 216@0 |  | 270@0 |  |  |  |  |  | TDD |
|  | 30 |  | 24@0 | 36@0 | 50@0 |  |  |  | 100@0 |  | 128@0 | 162@0 | 180@0 | 216@0 | 243@0 | 270@0 |  |
|  | 60 |  | 10@0 | 18@0 | 24@0 |  |  |  | 50@0 |  | 64@0 | 75@0 | 90@0 | 100@0 | 120@0 | 135@0 |  |
| n79 | 15 |  |  |  |  |  |  |  | 216@0 |  | 270@0 |  |  |  |  |  | TDD |
|  | 30 |  |  |  |  |  |  |  | 100@0 |  | 128@0 | 162@0 |  | 216@0 |  | 270@0 |  |
|  | 60 |  |  |  |  |  |  |  | 50@0 |  | 64@0 | 75@0 |  | 100@0 |  | 135@0 |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). NOTE 2: For Band 20; for 15kHz SCS, in the case of 15MHz channel bandwidth, the UL resource blocks shall be located at RBstart 11 and in the case of 20MHz channel bandwidth, the UL resource blocks shall be located at RBstart 16; for 30kHz SCS, in the case of 15MHz channel bandwidth, the UL resource blocks shall be located at RBstart 6 and in the case of 20MHz channel bandwidth, the UL resource blocks shall be located at RBstart 8; for 60kHz SCS, in the case of 15MHz channel bandwidth, the UL resource blocks shall be located at RBstart 3 and in the case of 20MHz channel bandwidth, the UL resource blocks shall be located at RBstart 4. NOTE 3: For DL channel bandwidths that do not have symmetric UL channel bandwidth, highest valid UL configuration with lowest duplex distance shall be used. NOTE 4: Reserved. NOTE 5: For this DL channel bandwidth, the UL configuration of the highest UL channel bandwidth specified in Table 5.3.6-1 and the default Tx-Rx frequency separation specified in Table 5.4.4-1 shall be used. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and C.3.1, and uplink signals according to Annex G.0, G.1, G.2, and G.3.1.

4.	The UL and Reference Measurement Channel is set according to Table 7.3.2.4.1-1, Table 7.3.2.4.1-2, and Table 7.3.2.4.1-3.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 7.3.2.4.3.

7.3.2.4.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.3.2.4.1-1. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Tables 7.3.2.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level to the appropriate REFSENS value defined in Table 7.3.2.5-1 if 2Rx antennas connected or Table 7.3.2.5-2 if 4Rx antennas connected.  Send continuously uplink power control "up" commands in the uplink scheduling information to the UE to ensure the UE transmits PUMAX level for at least the duration of the Throughput measurement.

4.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex H.2

7.3.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] clause 4.6 ensuring Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED for NR band.

Message contents are according to TS 38.508-1[5] subclause 4.6 with the following exceptions for each network signalling value.

7.3.2.4.3.1	Message contents exceptions (network signalled value "NS_01")

Message contents according to TS 38.508-1 [5] subclause 4.6 can be used without exceptions.

7.3.2.4.3.2	Message contents exceptions (network signalled value "NS_03")

1. Information element additionalSpectrumEmission is set to NS_03. This can be set in SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 7.3.2.4.3.2-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_03" and NR band n2, n25 and n66

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_03) |  |  |

Table 7.3.2.4.3.2-2: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_03" and NR band n70

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_03) |  |  |

7.3.2.4.3.3	Message contents exceptions (network signalled value "NS_06")

1. Information element additionalSpectrumEmission is set to NS_06. This can be set in SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 7.3.2.4.3.3-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_06" and NR band n12 and n14

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_06) |  |  |

7.3.2.4.3.4	Message contents exceptions (network signalled value "NS_35")

1. Information element additionalSpectrumEmission is set to NS_35. This can be set in SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 7.3.2.4.3.4-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_35" and NR band n71

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_35) |  |  |

7.3.2.4.3.5	Message contents exceptions (network signalled value "NS_27")

1. Information element additionalSpectrumEmission is set to NS_27. This can be set in SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 7.3.2.4.3.5-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_27" and NR band n48

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_27) |  |  |

7.3.2.4.3.6	Message contents exceptions (network signalled value "NS_21")

1. Information element additionalSpectrumEmission is set to NS_21. This can be set in SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 7.3.2.4.3.6-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_21" and NR band n30

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_21) |  |  |

7.3.2.4.3.7	Message contents exceptions (network signalled value "NS_45")

1. Information element additionalSpectrumEmission is set to NS_45. This can be set in SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 7.3.2.4.3.7-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_45" and NR band n53

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_45) |  |  |

7.3.2.4.3.8	Message contents exceptions (network signalled value "NS_56")

1. Information element additionalSpectrumEmission is set to NS_56. This can be set in SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 7.3.2.4.3.8-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_56" and NR band n24

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_56) |  |  |

7.3.2.5	Test requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A3.2 with reference receive power level specified in Tables 7.3.2.5-1a and Tables 7.3.2.5-1b for 2 Rx antenna port, Tables 7.3.2.5-2 a and Tables 7.3.2.5-2b for 4 Rx antenna port, and parameters specified Tables 7.3.2.4.1-1, Tables 7.3.2.4.1-2 and Tables 7.3.2.4.1-3.

Table 7.3.2.5-1a: Two antenna port Reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) | Duplex Mode |
| n1 | 15 | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.8 +TT | -92.7 +TT | -91.9 +TT |  | -90.6 +TT | -90.1 +TT | -89.6 +TT | FDD |
|  | 30 |  | -97.1 +TT | -95.1 +TT | -94.0 +TT | -92.8 +TT | -92.0 +TT |  | -90.7 +TT | -90.2 +TT | -89.7 +TT |  |
|  | 60 |  | -97.5 +TT | -95.4 +TT | -94.2 +TT | -93.0 +TT | -92.1 +TT |  | -90.9 +TT | -90.3 +TT | -89.7 +TT |  |
| n2 | 15 | -98.0 +TT | -94.8 +TT | -93.0 +TT | -91.8 +TT | -90.7 +TT | -84.1 +TT |  | -81.5 +TT |  |  | FDD |
|  | 30 |  | -95.1 +TT | -93.1 +TT | -92.0 +TT | -90.8 +TT | -84.2 +TT |  | -81.6 +TT |  |  |  |
|  | 60 |  | -95.5 +TT | -93.4 +TT | -92.2 +TT | -90.9 +TT | -84.3 +TT |  | -81.7 +TT |  |  |  |
| n3 | 15 | -97.0 +TT | -93.8 +TT | -92.0 +TT | -90.8 +TT | -89.7 +TT | -88.9 +TT | -86.2+TT | -87.6 +TT | -81.3+TT | -79.7 +TT | FDD |
|  | 30 |  | -94.1 +TT | -92.1 +TT | -91.0 +TT | -89.8 +TT | -89.0 +TT | -86.3+TT | -87.7 +TT | -81.4+TT | -79.8 +TT |  |
|  | 60 |  | -94.5 +TT | -92.4 +TT | -91.2 +TT | -90.0 +TT | -89.1 +TT | -86.4+TT | -87.9 +TT | -81.5+TT | -79.9 +TT |  |
| n5 | 15 | -98.0 +TT | -94.8 +TT | -93.0 +TT | -90.8 +TT | -84.8 +TT |  |  |  |  |  | FDD |
|  | 30 |  | -95.1 +TT | -93.1 +TT | -91.0 +TT | -84.9 +TT |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n71 | 15 | -98.0 +TT | -94.8 +TT | -93.0 +TT | -91.8 +TT |  |  |  |  |  |  | FDD |
|  | 30 |  | -95.1 +TT | -93.1 +TT | -92.0 +TT |  |  |  |  |  |  |  |
|  | 60 |  | -95.5 +TT | -93.4 +TT | -92.2 +TT |  |  |  |  |  |  |  |
| n8 | 15 | -97.0 +TT | -93.8 +TT | -92.0 +TT | -90.0 +TT |  |  |  |  |  |  | FDD |
|  | 30 |  | -94.1 +TT | -92.1 +TT | -90.2 +TT |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n12 | 15 | -97.0 +TT | -93.8 +TT | -84.0 +TT |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -94.1 +TT | -84.1 +TT |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n14 | 15 | -97.0 +TT | -93.8 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -94.1 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n20 | 15 | -97.0 +TT | -93.8 +TT | -91.0 +TT | -89.8 +TT |  |  |  |  |  |  | FDD |
|  | 30 |  | -94.1 +TT | -91.1 +TT | -90.0 +TT |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n24 | 15 | -100.0 +TT | -96.8 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -97.1 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  | -97.5 +TT |  |  |  |  |  |  |  |  |  |
| n25 | 15 | -96.5 +TT | -93.3 +TT | -91.5 +TT | -90.3 +TT | -89.3 +TT | -82.2 +TT |  | -79.5 +TT |  |  | FDD |
|  | 30 |  | -93.6 +TT | -91.6 +TT | -90.5 +TT | -89.4 +TT | -82.3 +TT |  | -79.6 +TT |  |  |  |
|  | 60 |  | -94.0 +TT | -91.9 +TT | -90.7 +TT | -89.6 +TT | -82.4 +TT |  | -79.7 +TT |  |  |  |
| n26 | 15 | -97.5 +TT | -94.5 +TT | -92.7 +TT | -87.6 +TT |  |  |  |  |  |  |  |
|  | 30 |  | -94.8 +TT | -92.7 +TT | -87.7 +TT |  |  |  |  |  |  |  |
| n28 | 15 | -98.5 +TT | -95.5 +TT | -93.5 +TT | -90.8 +TT |  | -78.5 +TT |  |  |  |  | FDD |
|  | 30 |  | -95.6 +TT | -93.6 +TT | -91.0 +TT |  | -78.6 +TT |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n30 | 15 | -99.0 +TT | -95.8 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -96.1 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n65 | 15 | -99.5+TT | -96.3+TT | -94.5+TT | -93.3+TT |  |  |  |  |  |  | FDD |
|  | 30 |  | -96.6+TT | -94.6+TT | -93.5+TT |  |  |  |  |  |  |  |
|  | 60 |  | -97.0+TT | -94.9+TT | -93.7+TT |  |  |  |  |  |  |  |
| n66 | 15 | -99.5 +TT | -96.3 +TT | -94.5 +TT | -93.3 +TT | -92.2 +TT | -91.4 +TT |  | -90.1 +TT |  |  | FDD |
|  | 30 |  | -96.6 +TT | -94.6 +TT | -93.5 +TT | -92.3 +TT | -91.5 +TT |  | -90.2 +TT |  |  |  |
|  | 60 |  | -97.0 +TT | -94.9 +TT | -93.7 +TT | -92.5 +TT | -91.6 +TT |  | -90.4 +TT |  |  |  |
| n70 | 15 | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.8 +TT | -92.7 +TT |  |  |  |  |  | FDD |
|  | 30 |  | -97.1 +TT | -95.1 +TT | -94.0 +TT | -92.8 +TT |  |  |  |  |  |  |
|  | 60 |  | -97.5 +TT | -95.4 +TT | -94.2 +TT | -93.0 +TT |  |  |  |  |  |  |
| n71 | 15 | -97.2 +TT | -94.0 +TT | -91.6 +TT | -86.0 +TT |  |  |  |  |  |  | FDD |
|  | 30 |  | -94.3 +TT | -91.9 +TT | -87.4 +TT |  |  |  |  |  |  |  |
|  | 60 | - |  |  |  |  |  |  |  |  |  |  |
| n74 | 15 | -99.53 +TT | -96.33 +TT | -94.53 +TT | -93.33 +TT |  |  |  |  |  |  | FDD |
|  | 30 |  | -96.63 +TT | -94.63 +TT | -93.53 +TT |  |  |  |  |  |  |  |
|  | 60 |  | -97.03 +TT | -94.93 +TT | -93.73 +TT |  |  |  |  |  |  |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. NOTE 2: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 NOTE 3: 3 indicates that the requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9-1510.9 MHz. NOTE 4: Void NOTE 5: Void NOTE 6: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2.5-1b: Two antenna port reference sensitivity QPSK PREFSENS for TDD, SDL and FDD with variable duplex operation bands

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm)8 | Duplex Mode |
| n34 | 15 | 5, 10, 15 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15 | -97.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15 | -97.5 + 10log10(NRB/11)+TT |  |
| n381 | 15 | 5, 10, 15, 20, 25, 30, 40 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40 | -97.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20, 25, 30, 40 | -97.5 + 10log10(NRB/11)+TT |  |
| n39 | 15 | 5, 10, 15, 20, 25, 30, 40 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40 | -97.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20, 25, 30, 40 | -97.5 + 10log10(NRB/11)+TT |  |
| n40 | 15 | 5, 10, 15, 20, 25, 30, 40, 50 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | -97.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | -97.5 + 10log10(NRB/11)+TT |  |
| n411 | 15 | 10, 15, 20, 30, 40, 50 | -94.8 + 10log10(NRB/50)+TT | TDD |
|  | 30 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -95.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -95.5 + 10log10(NRB/11)+TT |  |
| n481 | 15 | 5, 10, 15, 20, 30, 40, 505 | -99 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15, 20, 30, 40, 505, 605, 705, 805, 905, 1005 | -96.1 + 10log10(NRB/24) +TT |  |
|  | 60 | 10, 15, 20, 30, 40, 505, 605, 705, 805, 905, 1005 | -96.5 + 10log10(NRB/11)+TT |  |
| n50 | 15 | 5, 10, 15, 20, 30, 40, 50 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15, 20, 30, 40, 50, 60, 80 | -97.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20, 30, 40, 50, 60, 80 | -97.5 + 10log10(NRB/11)+TT |  |
| n51 | 15 | 5 | -100+TT | TDD |
| n53 | 15 | 5, 10 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10 | -97.1+TT |  |
|  | 60 | 10 | -97.5+TT |  |
| n771,4 | 15 | 10, 15, 20, 40, 50 | -95.3 + 10log10(NRB/50)+TT | TDD |
|  | 30 | 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 | -95.6 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 | -96.0 + 10log10(NRB/11)+TT |  |
| n781 | 15 | 10, 15, 20, 40, 50 | -95.8 + 10log10(NRB/50) +TT | TDD |
|  | 30 | 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 | -96.1 + 10log10(NRB/24) +TT |  |
|  | 60 | 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 | -96.5 + 10log10(NRB/11)+TT |  |
| n791 | 15 | 40, 50 | -89.6 + 10log10(NRB/216)+TT | TDD |
|  | 30 | 40, 50, 60, 80, 100 | -89.7 + 10log10(NRB/106) +TT |  |
|  | 60 | 40, 50, 60, 80, 100 | -89.9 + 10log10(NRB/51) +TT |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 3: Void NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. NOTE 6: Void NOTE 7: For SDL bands, the reference sensitivity requirements shall be verified by inter-band CA combinations with SDL band, which are supported by UE. NOTE 8: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. NOTE 9: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. |  |  |  |  |

Table 7.3.2.5-2a: Four antenna port Reference sensitivity QPSK PREFSENS FDD bands

| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 | 15 | -102.7 +TT | -99.5 +TT | -97.7 +TT | -96.5 +TT | -95.4 +TT | -94.6 +TT |  | -93.3 +TT | -92.8 +TT | -92.3 +TT | FDD |
|  | 30 |  | -99.8 +TT | -97.8 +TT | -96.7 +TT | -95.5 +TT | -94.7 +TT |  | -93.4 +TT | -92.9 +TT | -92.4 +TT |  |
|  | 60 |  | -100.2 +TT | -98.1 +TT | -96.9 +TT | -95.7 +TT | -94.8 +TT |  | -93.6 +TT | -93 +TT | -92.4 +TT |  |
| n2 | 15 | -100.7 +TT | -97.5 +TT | -95.7 +TT | -94.5 +TT | -93.4 +TT | -86.8 +TT |  | -84.2 +TT |  |  | FDD |
|  | 30 |  | -97.8 +TT | -95.8 +TT | -94.7 +TT | -93.5 +TT | -86.9 +TT |  | -83.3 +TT |  |  |  |
|  | 60 |  | -98.2 +TT | -96.1 +TT | -94.9 +TT | -93.6 +TT | -87.0 +TT |  | -84.4 +TT |  |  |  |
| n3 | 15 | -99.7 +TT | -96.5 +TT | -94.7 +TT | -93.5 +TT | -92.4 +TT | -91.6 +TT | -88.9+TT | -90.3 +TT | -84.0+TT | -82.4 +TT | FDD |
|  | 30 |  | -96.8 +TT | -94.8 +TT | -93.7 +TT | -92.5 +TT | -91.7 +TT | -90.0+TT | -90.4 +TT | -84.1+TT | -82.5 +TT |  |
|  | 60 |  | -97.2 +TT | -95.1 +TT | -93.9 +TT | -92.7 +TT | -91.8 +TT | -90.1+TT | -90.6 +TT | -84.2+TT | -82.6 +TT |  |
| n7 | 15 | -100.7 +TT | -97.5 +TT | -95.7 +TT | -94.5 +TT |  |  |  |  |  |  | FDD |
|  | 30 |  | -97.8 +TT | -95.8 +TT | -94.7 +TT |  |  |  |  |  |  |  |
|  | 60 |  | -98.2 +TT | -97.1 +TT | -94.9 +TT |  |  |  |  |  |  |  |
| n30 | 15 | -101.7 +TT | -98.5 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -98.8 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n66 | 15 | -102.2 +TT | -99.0 +TT | -97.2 +TT | -96.0 +TT | -94.9 +TT | -94.1 +TT |  | -92.8 +TT |  |  | FDD |
|  | 30 |  | -99.3 +TT | -97.3 +TT | -96.2 +TT | -95.0 +TT | -94.2 +TT |  | -92.9 +TT |  |  |  |
|  | 60 |  | -99.7 +TT | -97.6 +TT | -96.4 +TT | -95.2 +TT | -94.3 +TT |  | -93.1 +TT |  |  |  |
| n70 | 15 | -102.7 +TT | -99.5 +TT | -97.7 +TT | -96.5 +TT | -95.4 +TT |  |  |  |  |  | FDD |
|  | 30 |  | -99.8 +TT | -97.8 +TT | -96.7 +TT | -95.5 +TT |  |  |  |  |  |  |
|  | 60 |  | -100.2 +TT | -98.1 +TT | -96.9 +TT | -95.7 +TT |  |  |  |  |  |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for above listed operating band except for two Rx vehicular UE. NOTE 2 The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 3: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. NOTE 4: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2.5-2b: Four antenna port Reference sensitivity QPSK PREFSENS for TDD, SDL and FDD with variable duplex operation bands

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm)8 | Duplex Mode |
| n34 | 15 | 5, 10, 15 | -100 + 10log10(NRB/25)-2.7 +TT | TDD |
|  | 30 | 10, 15 | -97.1 + 10log10(NRB/24)+-2.7 TT |  |
|  | 60 | 10, 15 | -97.5 + 10log10(NRB/11)-2.7 +TT |  |
| n381 | 15 | 5, 10, 15, 20, 25, 30, 40 | -100 + 10log10(NRB/25)-2.7 +TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40 | -97.1 + 10log10(NRB/24)-2.7 +TT |  |
|  | 60 | 10, 15, 20, 25, 30, 40 | -97.5 + 10log10(NRB/11)-2.7 +TT |  |
| n39 | 15 | 5, 10, 15, 20, 25, 30, 40 | -100 + 10log10(NRB/25)-2.7 +TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40 | -97.1 + 10log10(NRB/24)-2.7 +TT |  |
|  | 60 | 10, 15, 20, 25, 30, 40 | -97.5 + 10log10(NRB/11)-2.7 +TT |  |
| n40 | 15 | 5, 10, 15, 20, 25, 30, 40, 50 | -100 + 10log10(NRB/25) -2.7 +TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | -97.1 + 10log10(NRB/24)-2.7 +TT |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | -97.5 + 10log10(NRB/11)-2.7 +TT |  |
| n411 | 15 | 10, 15, 20, 30, 40, 50 | -94.8 + 10log10(NRB/50)-2.7 +TT | TDD |
|  | 30 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -95.1 + 10log10(NRB/24)-2.7 +TT |  |
|  | 60 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -95.5 + 10log10(NRB/11)-2.7 +TT |  |
| n481 | 15 | 5, 10, 15, 20, 30, 40, 505 | -99 + 10log10(NRB/25)-2.2 +TT | TDD |
|  | 30 | 10, 15, 20, 40, 505, 605, 705, 805, 905, 1005 | -96.1 + 10log10(NRB/24)-2.2 +TT |  |
|  | 60 | 10, 15, 20, 40, 505, 605, 705, 805, 905, 1005 | -96.5 + 10log10(NRB/11)-2.2+TT |  |
| n771,4 | 15 | 10, 15, 20, 40, 50 | -95.3 + 10log10(NRB/50)-2.2 +TT | TDD |
|  | 30 | 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 | -95.6 + 10log10(NRB/24)-2.2 +TT |  |
|  | 60 | 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 | -96.0 + 10log10(NRB/11)-2.2 +TT |  |
| n781 | 15 | 10, 15, 20, 40, 50 | -95.8 + 10log10(NRB/50)-2.2 +TT | TDD |
|  | 30 | 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 | -96.1 + 10log10(NRB/24)-2.2 +TT |  |
|  | 60 | 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 | -96.5 + 10log10(NRB/11)-2.2 +TT |  |
| n791 | 15 | 40, 50 | -89.6 + 10log10(NRB/216)-2.2 +TT | TDD |
|  | 30 | 40, 50, 60, 80, 100 | -89.7 + 10log10(NRB/106)-2.2 +TT |  |
|  | 60 | 40, 50, 60, 80, 100 | -89.9 + 10log10(NRB/51)-2.2 +TT |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 3: Void NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. NOTE 6: Void NOTE 7: For SDL bands, the reference sensitivity requirements shall be verified by inter-band CA combinations with SDL band, which are supported by UE. NOTE 8: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. NOTE 9: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. |  |  |  |  |

Table 7.3.2.5-3: Test Tolerance (TT) for RX sensitivity level

| f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0 GHz |
| --- | --- |
| 0.7 dB | 1.0 dB |

For the UE which supports inter-band carrier aggregation, the minimum requirement for reference sensitivity in Table 7.3.2.5-1 a and Table 7.3.2.5-1b shall be increased by the amount given in ΔRIB,c defined in subclause 7.3.3 for the applicable operating bands.

<<End of changes>>