Title: 	Discussion on test point selection for NS_56/NS_55 A-MPR for CA in FR1

Source: 	Ligado Networks

- Introduction

As agreed in RAN5#78, each test case would require specific analysis to conform its test configuration table based on the study for Frequency, Channel BW, SCS, RB allocation (including modulations).

The purpose of this contribution is to provide the complete analysis for each parameter included in the Test Configuration Table and propose test points selection for NS_56/NS_55 combination in Additional Maximum Power Reduction for CA test case in FR1.

In Rel-17 specification, only inter-band 2UL CA is defined in TS 38.101-1, but the analysis will be extended in the future for other configurations.

- Discussion

UE additional maximum output power reduction for CA FR1 is defined in Transmitter power for CA section in [1] where its scope is described as follows:

Unless otherwise stated, for inter-band carrier aggregation with uplink assigned to two NR bands, the requirements in clause 6.2.3 apply for each uplink component carrier.

For NS_56, A-MPR requirements in [2] are extracted below:

For 5 MHz channel centered on frequencies (FC) = 1630.0, 1630.3 MHz, A-MPR is defined as

if RBstart <= ceil{3/SCS/15 kHz)}and LCRB <= ceil{17/SCS/15 kHz)},

then

the A-MPR = 14 dB for SCS = 15 kHz and AMPR = 8 dB for SCS >= 30 kHz,

else,

if RBstart <= ceil{3/(SCS/15 kHz)} and LCRB > ceil{17/(SCS/15 kHz)},

then

the A-MPR = 6 dB,

else,

if RBstart <= ceil{8/(SCS/15 kHz)},

then

the A-MPR = 4 dB.

For 5 MHz channel centered on frequencies (Fc) = 1635.0, 1649.0, 1654.0 MHz, no A-MPR is needed.

For Channel 10 MHz with center frequency of 1632.5 MHz, A-MPR is defined as

if RBstart < ceil{3/(SCS/15 kHz)} and LCRB <= ceil{8/(SCS/15 kHz)},

then

the A-MPR = 12 dB for SCS = 15 kHz and AMPR = 8 dB for SCS >= 30 kHz,

else,

if RBstart < ceil{9/(SCS/15 kHz)}, and LCRB > ceil{8/(SCS/15 kHz)},

then

the A-MPR = 8 dB,

else,

if RBstart <= ceil{18/(SCS/15 kHz)},

then

the A-MPR = 6 dB,

else,

if RBstart >= floor{40/(SCS/15 kHz)}], and LCRB <= ceil{7/(SCS/15 kHz)},

then

the A-MPR = 5 dB,

else,

if RBstart >= floor{40/(SCS/15 kHz)} and LCRB > ceil{7/(SCS/15 kHz)},

then

the A-MPR = 3 dB,

else,

if RBstart >= floor{35/(SCS/15 kHz)} and LCRB <= ceil{7/(SCS/15 kHz)},

then

the A-MPR = 4 dB,

else,

if RBstart >= floor{35/(SCS/15 kHz)} and LCRB > ceil{7/(SCS/15 kHz)},

then

the A-MPR = 2 dB.

For 10 MHz channel centered on frequency of 1651.5 MHz, no A-MPR is needed.

For NS_55, there are no A-MPR requirements. It is only used to indicate the frequency range of operation for the n77 band in the US [2].

The MPR requirement for power class 3 is defined in 6.2.2 in [2], which is extracted as below:

Table 6.2.2.3-1: Maximum power reduction (MPR) for power class 3

| Modulation |  | MPR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Edge RB allocations |  | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.51 |  | ≤ 1.21 | ≤ 0.21 |
|  |  | ≤ 0.52 |  |  | 02 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.52 | 02 |  |  |
|  | QPSK | ≤ 1 |  |  | 0 |
|  | 16 QAM | ≤ 2 |  |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |  |
| CP-OFDM | QPSK | ≤ 3 |  |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |  |
| NOTE 1: Applicable for UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and if the IE powerBoostPi2BPSK is set to 1 and 40 % or less slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. The reference power of 0dB MPR is 26dBm. NOTE 2: Applicable for UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79 with Pi/2 BPSK modulation and if the IE powerBoostPi2BPSK is set to 0 and if more than 40% of slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. |  |  |  |  |  |

Following subclauses introduce study for test environment, test frequencies, test bandwidth, test subcarrier spacing and uplink configuration aspects.

  - Test Environment

Based on the baseline agreed in [3], test environment for UE additional maximum output power reduction for CA can be defined as in LTE regardless of number of CCs.

Proposal 1: Select Test Environment as Normal Conditions (NC) for NS_56/NS_55 for A-MPR for CA in FR1.

  - Test Frequencies

For single carrier test case 6.2.3 A-MPR for NS_56, low range is considered. For inter-band CA testing, it’s reasonable to select the same test frequencies for NS_56, which is PCC in this test case.

For SCC, NS_55 has no A-MPR requirement. Therefore, for NS_55, it is reasonable to select the low and high range.

Proposal 2: Select low range as test frequency for PCC.

Proposal 3: Select low and high range as test frequencies for SCC.

  - Test Channel Bandwidth

Low and High frequency range are selected for single carrier SEM test case for bandwidths for NS_56 and NS_55 carriers. In SEM for CA, low and high ranges are selected for PCC. It is reasonable to test the same channel bandwidths for NS_56/NS_55 A-MPR for CA testing.

Proposal 4: Select Lowest and Highest channel bandwidths for PCC and SCC for NS_56/NS_55 A-MPR for CA in FRI.

  - Test Subcarrier Spacing

The selection of test subcarrier spacing can be based on the agreed proposals for SEM, i.e., Lowest and Highest supported subcarrier spacing.

Proposal 5: Select Lowest and Highest SCS for PCC and SCC for NS_56/NS_55 A-MPR for CA testing.

  - Uplink Waveform, Modulation and RB allocation

According to the clause 4.1.1.2 in TR 38.905, three configurations should be tested in A-MPR for CA testing, which are:

-	Test the minimum Total power backoff value

-	Test the maximum Total power backoff value

-	Test the maximum unbalanced Total power backoff among CCs (max Pcmaxc difference).

Where the Total power backoff value means: MAX[MPR, A-MPR]

Based on the AMPR requirement for NS_56, NS_55 and MPR requirement, following configurations are derived for each CC:

The minimum Total power backoff configuration:

PCC:

For 5 MHz Channel: DFT-s-OFDM + QPSK + Inner_1RB_Right allocation for low and high SCSs (Backoff = 0)

For 10 MHz Channel: DFT-s-OFDM + QPSK + Inner RB allocation (18@20 for low SCS and 4@5 for high SCS) (Backoff = 0)

SCC:

DFT-s-OFDM + QPSK + Inner Full RB allocation (Backoff = 0)

Minimum Total power backoff = 0 dB

The maximum Total power backoff configuration:

PCC: DFT-s-OFDM + QPSK + Edge RB allocation (Backoff = 14)

SCC: CP-OFDM + 256 QAM + Outer Full RB allocation (Backoff = 6.5)

Maximum Total power backoff = 20.5 dB

The maximum unbalanced Total power backoff configuration:

PCC: DFT-s-OFDM + QPSK + Edge RB allocation (Backoff = 14)

SCC: DFT-s-OFDM + QPSK + Inner Full RB allocation (Backoff = 0)

Maximum unbalanced Total power backoff = 14 dB

Based on the above configuration analysis, the Uplink configurations specified in Table 1 should be selected. Specific LCRB@RBstart have been specified where common uplink configurations of Table 6.1-1 in TS 38.521-1 cannot be used. Moreover, separate test configs related to SCS have been defined since the A-MPR for NS_56 is different between the lowest and the highest SCSs.

Table 1 Test Parameters for NS_56/NS_04 AMPR

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  |  |  |  | Low range for PCC and Low and High range for SCC as specified in test parameters |  |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Lowest and Highest for PCC as specified in test parameters Lowest and Highest for SCC as specified in test parameters |  |  |  |  |  |
| Test SCS as specified in Table 5.5A.3-1 |  |  |  |  |  |  | Lowest, Highest as specified in test parameters |  |  |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | DL configuration | UL configuration |  |  |  |  |  |  |  |  |  |  |
|  |  | PCC |  |  |  |  |  | SCC |  |  |  |  |
|  |  | FC (MHz) | SCS | Ch BW (MHz) | Modulation | RB allocation |  | FC (MHz) | Ch BW (MHz) | Modulation | RB allocation |  |
|  |  |  |  |  |  |  |  |  |  |  | SCS (15 kHz) | SCS (60 kHz) |
| 1 |  | Low | 15 | 5 | DFT-s-OFDM QPSK | Inner 1RB Right |  | Low | 10 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 2 |  | Low | 15 | 5 | DFT-s-OFDM QPSK | Edge 1RB Left |  | Low | 10 | CP-OFDM 256QAM | Outer Full | Outer Full |
| 3 |  | Low | 15 | 5 | DFT-s-OFDM QPSK | Edge 1RB Left |  | Low | 10 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 4 | N/A | Low | 30 | 5 | DFT-s-OFDM QPSK | Inner 1RB Right |  | Low | 10 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 5 |  | Low | 30 | 5 | DFT-s-OFDM QPSK | Edge 1RB Left |  | Low | 10 | CP-OFDM 256QAM | Outer Full | Outer Full |
| 6 |  | Low | 30 | 5 | DFT-s-OFDM QPSK | Edge 1RB Left |  | Low | 10 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 7 |  | Low | 15 | 10 | DFT-s-OFDM QPSK | 18@20 |  | High | 100 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 8 |  | Low | 15 | 10 | DFT-s-OFDM QPSK | Edge 1RB Left |  | High | 100 | CP-OFDM 256QAM | Outer Full | Outer Full |
| 9 |  | Low | 15 | 10 | DFT-s-OFDM QPSK | Edge 1RB Left |  | High | 100 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 10 |  | Low | 60 | 10 | DFT-s-OFDM QPSK | 4@5 |  | High | 100 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 11 |  | Low | 60 | 10 | DFT-s-OFDM QPSK | Edge 1RB Left |  | High | 100 | CP-OFDM 256QAM | Outer Full | Outer Full |
| 12 |  | Low | 60 | 10 | DFT-s-OFDM QPSK | Edge 1RB Left |  | High | 100 | DFT-s-OFDM QPSK | Inner Full | Inner Full |

Proposal 6: Select Uplink Waveform, Modulation and RB allocation as shown in Table 1.

  - Number of Test Points

Based on above proposals, the total test points of NS_56/NS_55 A-MPR for CA are tabulated below.

Table 2 Total test points of NS_56/NS_55 A-MPR for CA

| Environment conditions | Test Frequencies | Test ChBW | Test SCS | Test Configs | Total Test Points |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 2 | 2 | 3 | 24 |

- Conclusion

Proposal 1: Select Test Environment as Normal Conditions (NC) for NS_56/NS_55 for A-MPR for CA in FR1.

Proposal 2: Select low range as test frequency for PCC.

Proposal 3: Select low and high range as test frequencies for SCC.

Proposal 4: Select Lowest and Highest channel bandwidths for PCC and SCC for NS_56/NS_55 A-MPR for CA in FRI.

Proposal 5: Select Lowest and Highest SCS for PCC and SCC for NS_56/NS_55 A-MPR for CA testing.

Proposal 6: Select Uplink Waveform, Modulation and RB allocation as shown in Table 1.

- References

[1] 3GPP TS 38.521-1 V17.5.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone; (Release 17)

[2] 	3GPP TS 38.101-1 v17.6.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 17)

[3]	3GPP TR 38.905 V17.5.0 Derivation of test points for radio transmission and reception User Equipment (UE) conformance test cases (Release 17)