Title: 	Discussion on test point selection for 38.521-1 6.5.3.3 TX_Additional_Spurious_Emission_NS_14

Source: 	Qualcomm

# Introduction

Test point selections for transmitter additional spurious emission for various NS values are chosen mostly the same test points defined for test case 6.2.3 Additional maximum power reduction.  This has been the case for many NS values already implemented for test case 6.5.3.3.

There are additional requirement are defined for some NS values for additional spurious emission testing. Additional consideration is needed when selection test points in order to comply with core requirement. Additional spurious emission for NS_14 (n26) is one of these cases.

The purpose of this contribution is to provide test point selection proposal for Additional spurious emission for NS_14 (n26) is one of these cases based on requirement in TS 38.101-1 [1] specification.

# Discussion

## Test points for UE additional maximum output power reduction

The test configuration for UE additional maximum output power reduction for NS_14 is defined in Table 6.2.3.4.1-23 of Ts 38.521-1 [2]:

Table 6.2.3.4.1-23: Test Configuration table for NS_14

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Low range, High range |  |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Refer to test parameters (10, 15, 20 MHz) |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest |  |  |  |  |  |
| A-MPR test parameters for NS_14 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation (NOTE 2, 3) |  | RB allocation (Note 1) |  |  |  |
| 1 | Default | 10 | Default | N/A for A-MPR testing. | DFT-s-OFDM | 64 QAM | Edge_1RB_Left |  |  |
| 2 | Default | 10 | Default |  |  | 16 QAM | Outer_Full |  |  |
| 3 | Default | 15 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 4 | Default | 15 | Default |  |  | 16 QAM | Outer_Full |  |  |
| 5-6 | Default | 20 | Default |  |  | 64 QAM 256 QAM | Edge_1RB_Left |  |  |
| 7 | Default | 20 | Default |  |  | 16 QAM | Outer_Full |  |  |
| 8 | Default | 10 | Default |  |  | QPSK | 8@2 |  |  |
| 9 | Default | 15 | Default |  |  | QPSK | 8@11 |  |  |
| 10 | Default | 20 | Default |  |  | QPSK | 8@20 |  |  |
| 11-13 | Default | 10 | Default |  | CP-OFDM | 16 QAM 64 QAM 256 QAM | Edge_1RB_Left |  |  |  |
| 14 | Default | 10 | Default |  |  | 16 QAM | Outer_Full |  |  |  |
| 15-17 | Default | 15 | Default |  |  | 16 QAM 64 QAM 256 QAM | Edge_1RB_Left |  |  |
| 18 | Default | 15 | Default |  |  | 16 QAM | Outer_Full |  |  |
| 19-21 | Default | 20 | Default |  |  | 16 QAM 64 QAM 256 QAM | Edge_1RB_Left |  |  |
| 22 | Default | 20 | Default |  |  | 16 QAM | Outer_Full |  |  |
| 23 | Default | 10 | Default |  |  | QPSK | 8@2 |  |  |
| 24 | Default | 15 | Default |  |  | QPSK | 8@11 |  |  |
| 25 | Default | 20 | Default |  |  | QPSK | 8@20 |  |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: In test IDs with multiple modulations, each UL Modulation shall be tested separately against RB allocation. |  |  |  |  |  |  |  |  |  |  |

The test frequencies for all test points are referring the default values which are Low range, High range. The values of Low range center frequencies for n26 is defined in TS 38.508-1 [3]:

## Minimum requirement for additional spurious emission for NS_14

The core requirement for additional spurious emission for NS_14 is defined in clause 6.5.3.3.19 of TS 38.101-1 [1]:

##### 6.5.3.3.19  Requirement for network signalling value "NS_14"

When "NS_14" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.19-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.19-1: Additional requirements NS_14

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 10 MHz, 15 MHz, 20MHz |  |
| 806 ≤ f ≤ 816 | -42 | 6.25 kHz |
| NOTE 1: The requirement applies for NR carriers with lower channel edge at or above 824 MHz. NOTE 2: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

Note 1 clearly indicating the test requirement for NR carrier with lower channel edge at or above 824 MHz.

Considering the center frequencies of low channels for 10 MHz, 15 MHz and 20 MHZ channel bandwidths are 819 MHz, 821.5 MHz and 824MHz, respectively, all their lower channel edges are below 824 MHz. Therefore, Low range test points shall not apply to additional spurious emission for NS_14.

Observation 1: 	For 10, 15, and 20 MHz channel bandwidths, the lower channel edge for low range are all below 824 MHz.

Proposal 1:  	The default Low range frequency test points defined in A-MPR test for NS_14 shall be removed for additional spurious emission for NS_14.

.

## All Other Test parameters

Following the general agreement, all other test parameters including Test Environment, Test Channel Bandwidth, Subcarrier Spacing and RB configurations shall be the same as defined for A-MPR test case.

Proposal 2: 	Test Environment, Test Channel Bandwidth, Subcarrier Spacing and RB configurations for additional spurious emission test shall be the same as defined for A-MPR test case for NS_14.

## Number of Test Points for each NR band

| Environmental conditions | Maximum Number of Frequencies with specific RB allocation | Maximum Number of ChBW | Number SCS | Number of RB allocations | Modulation | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | NA | 3 | 1 | 5 | 4 | 25 |

# Conclusion

Based on above discussion, it is proposed that Low range shall be removed for additional spurious emission testing for NS_14. Summarized as below:

Proposal 1: The default Low range frequency test points defined in A-MPR test for NS_14 shall be removed for additional spurious emission for NS_14.

Proposal 2: Test Environment, Test Channel Bandwidth, Subcarrier Spacing and RB configurations for additional spurious emission test shall be the same as defined for A-MPR test case for NS_14.

# References

[1]    3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[2]   3GPP TS 38.521-1:" User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone.

[3]    3GPP TS 38.508-1: "5GS; User Equipment (UE) conformance specification; Part 1: Common test environment".