3GPP TSG-RAN5 Meeting #96-e	R5-224922

# Electronic Meeting, 15th – 26th Aug. 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3175 | rev | - | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of NR V2X test case 12.1.6.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TBD’s need to be resolved |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added sequence to check UE has released PC5 connection Removed editors note The impact of continuous Uplink PDU transmission in every STTCH occasions is removed by ignoring such PDU’s. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.1.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 12.1.6.3	PC5-only operation / Sidelink failure / Sidelink radio link failure / Transmission side

12.1.6.3.1	Test Purpose (TP)

(1)

with { UE having established PC5 RRC connection with peer UE on unicast sidelink and has sent an RRCReconfigurationSidelink message to peer UE }

ensure that {

when { UE does not receive RRCReconfigurationCompleteSidelink or RRCReconfigurationFailure before T400 expires}

then { UE releases PC5-RRC connection and indicates the release to upper layer}

}

(2)

with { UE having established PC5 RRC connection with peer UE on unicast sidelink and has established a AM SL-DRB}

ensure that {

when { Retransmission number of the AM SL-DRB reaches the maximum number of retransmissions}

then { UE releases PC5-RRC connection and indicates the release to upper layer.}

}

(3)

with { UE having established PC5 RRC connection with peer UE on unicast sidelink }

ensure that {

when { MAC detects that maximum number of consecutive HARQ DTX has been reached }

then { UE releases PC5-RRC connection and indicates the release to upper layer }

}

12.1.6.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331 [22], subclause 5.8.9.3. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, subclause 5.8.9.3]

The UE shall:

1>	upon indication from sidelink RLC entity that the maximum number of retransmissions for a specific destination has been reached; or

1>	upon T400 expiry for a specific destination; or

1>	upon indication from MAC entity that the maximum number of consecutive HARQ DTX for a specific destination has been reached; or

1>	upon integrity check failure indication from sidelink PDCP entity concerning SL-SRB2 or SL-SRB3 for a specific destination:

2>	consider sidelink radio link failure to be detected for this destination;

2>	release the DRBs of this destination, in according to sub-clause 5.8.9.1a.1;

2>	release the SRBs of this destination, in according to sub-clause 5.8.9.1a.3;

2>	discard the NR sidelink communication related configuration of this destination;

2>	reset the sidelink specific MAC of this destination;

2>	consider the PC5-RRC connection is released for the destination;

2>	indicate the release of the PC5-RRC connection to the upper layers for this destination (i.e. PC5 is unavailable);

2>	if UE is in RRC_CONNECTED:

3>	perform the sidelink UE information for NR sidelink communication procedure, as specified in 5.8.3.3;

NOTE:	It is up to UE implementation on whether and how to indicate to upper layers to maintain the keep-alive procedure [55].

12.1.6.3.3	Test description

12.1.6.3.3.1	Pre-test conditions

System Simulator:

-	NR-SS-UE

-	NR-SS-UE 1 is as defined in TS 38.508-1 [4], configured for and operating as NR sidelink communication device on the resources (i.e. the frequency included in pre-configuration) that UE is expected to use for transmission and reception via PC5 interface.

- NR-SS-UE 1 is synchronised on GNSS.

-	GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

- UE is authorised to perform NR sidelink communication.

- The UE is equipped with a USIM containing default values as per TS 38.508-1 [4] clause 4.8.3.3.3.

- UE is synchronised on GNSS.

Preamble:

-	The UE is in state 4-A as defined in TS 38.508-1 [4], subclause 4.4A using generic procedure parameter Sidelink (On), Cast Type (Unicast), GNSS Sync (On) using UE initiated unicast mode NR sidelink communication procedure in subclause 4.9.23.

12.1.6.3.3.2	Test procedure sequence

Table 12.1.6.3.3.2-1: Specific Parameters

| Parameter | Value | Comment |
| --- | --- | --- |
| sl-MaxRetxThreshold-r16 | 1 |  |
| sl-MaxNumConsecutiveDTX-r16 | 1 |  |

Table 12.1.6.3.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is configured by upper layer to release SL-DRB to NR-SS-UE1. Note: This step is triggered by MMI or AT command. | - | - | - | - |
| 2 | UE sends an RRCReconfigurationSidelink message to NR-SS-UE1 to indicate SL-DRB release | --> | PC5 RRC: RRCReconfigurationSidelink | - | - |
| 3 | NR-SS-UE1 does not respond and waits for the expiration of t_400 (1 second). | - | - | - | - |
| 3A | 1 Second after step 3, the NR-SS-UE sends a DIRECT LINK RELEASE REQUEST message. | <-- | PC5-S: DIRECT LINK RELEASE REQUEST |  |  |
| 4 | Check: Does the UE sends a DIRECT LINK RELEASE ACCEPT message within the next 5 seconds. | --> | PC5-S: DIRECT LINK RELEASE ACCEPT | 1 | F |
| 5 | The UE is brought to state 4-A as defined in TS 38.508-1 [4], subclause 4.4A using generic procedure parameter Sidelink (On), Cast Type (Unicast), GNSS Sync (On), Test Mode = On using procedure in subclause 4.9.23. | - | - | - | - |
| 6 | The SS triggers UE to close UE test loop mode E (Transmission Mode). NOTE: Closing of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). | - | - | - | - |
| 7 | The UE transmits one AMD PDU#1 to NR-SS-UE1 on SL-DRB | --> | AMD PDU#1 (SN=0) | - | - |
| - | EXCEPTION: Steps 9-10 are repeated sl-maxRetxThreshold times | - | - | - | - |
| - | EXCEPTION: In parallel to seps 9-10 any additional AMD PDU’s received are ignored by SS | - | - | - | - |
| 8 | The NR-SS-UE1 transmits an RLC STATUS PDU. ACK_SN =1 and NACK_SN =0. | <-- | STATUS PDU | - | - |
| 9 | The UE transmits one AMD PDU#1 to NR-SS-UE1 | --> | AMD PDU#1 (SN=0) | - | - |
| 10 | The NR-SS-UE1 transmits an RLC STATUS PDU. ACK_SN =1 and NACK_SN =0. | <-- | STATUS PDU | - | - |
| 11 | Check: Does the UE has released PC5 RRC connection as per generic procedure TBD? | - | - | 2 | P |
| 12 | The UE is brought to state 4-A as defined in TS 38.508-1 [4], subclause 4.4A using generic procedure parameter Sidelink (On), Cast Type (Unicast), GNSS Sync (On), Test Mode = On using procedure in subclause 4.9.23. | - | - | - | - |
| 13 | The SS triggers UE to close UE test loop mode E (Transmission Mode). NOTE: Closing of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). | - | - | - | - |
| 14 | The NR-SS-UE1 MAC is configured to not send HARQ feedback | - | - | - | - |
| - | EXCEPTION: Step 14 is repeated sl-MaxNumConsecutiveDTX-r16 times | - | - | - | - |
| - | EXCEPTION: In parallel to seps 14 any additional MAC PDU’s received are ignored by SS | - | - | - | - |
| 15 | The UE transmits one MAC PDU to NR-SS-UE1 | --> | MAC PDU | - | - |
| 15A | 1 Second after step 15, the NR-SS-UE sends a DIRECT LINK RELEASE REQUEST message. | <-- | PC5-S: DIRECT LINK RELEASE REQUEST |  |  |
| 16 | Check: Does the UE sends a DIRECT LINK RELEASE ACCEPT message within the next 5 seconds. | --> | PC5-S: DIRECT LINK RELEASE ACCEPT | 3 | F |

12.1.6.3.3.3	Specific message contents

None