3GPP TSG-RAN WG5 Meeting # 96-e	R5-224958

# Electronic meeting, 15 – 26 August, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1861 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on spurious emission for UE co-existence limits for inter-band CA configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest, NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2022-07-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For the minimum conformance requirements and test requirements for spurious emission for UE co-existence for inter-band CA configuration within FR1, the requirements are specified in different tables in accordance with the different releases. It is proposed to merge the tables of different releases and add a new column of “Release” to indicate the starting release. For the minimum conformance requirements of single NR band, the UE co-existence tables for different releases should also be merged. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Merge the tables with different releases for the minimum conformance requirements for spurious emission for UE co-existence for inter-band CA configuration and add a new column of “Release” to indicate the starting release. - Merge the tables with different releases for the test requirements for spurious emission for UE co-existence for inter-band CA configuration and add a new column of “Release”. - Merge the tables with different releases for the minimum conformance requirements for spurious emission for UE co-existence for single NR band and add a new column of “Release”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spurious emission requirements for UE co-existence for inter-band CA and single NR band will be redundant. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.3.2.3, 6.5.3.2.4.2, 6.5.3.2.5, 6.5A.3.2.0.3, 6.5A.3.2.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

<< Unchanged sections omitted >>

6.5.3.2.3	Minimum conformance requirements

This clause specifies the requirements for the specified NR band for coexistence with protected bands as indicated in Table 6.5.3.2.3-4.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

Table 6.5.3.2.3-1: void

Table 6.5.3.2.3-2: Void

Table 6.5.3.2.3-3: Void

Table 6.5.3.2.3-4: Requirements for spurious emissions for UE co-existence

| NR Band | Release | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n1, n84 |  | Rel-15 |  | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 40, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76, NR Band n78, n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  | E-UTRA Band 3, 34 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 15 |  |
|  |  |  |  | Frequency range |  | 1880 |  | - |  | 1895 |  | -40 |  | 1 |  | 15, 27 |  |
|  |  |  |  | Frequency range |  | 1895 |  | - |  | 1915 |  | -15.5 |  | 5 |  | 15, 26, 27 |  |
|  |  |  |  | Frequency range |  | 1915 |  | - |  | 1920 |  | +1.6 |  | 5 |  | 15, 26, 27 |  |
| n2 |  | Rel-15 |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 50, 51, 53, 66, 70, 71, 74, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | E-UTRA Band 2, 25 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 15 |  |
|  |  |  |  | E-UTRA Band 43, NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
| n3, n80 |  | Rel-15 |  | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 39, 40, 41, 43, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73,74, 75, 76. NR Band n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | E-UTRA Band 3 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 15 |  |
|  |  |  |  | E-UTRA Band 11, 18, 19, 21 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | E-UTRA Band 22, 42, 52, NR Band n77, n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 8 |  |
| n5 |  | Rel-15 |  | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 18, 19, 24, 25, 26, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 73, 74, 85 NR Band n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | E-UTRA Band 41, 52, 53 NR Band n77, n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  | E-UTRA Band 11, 21 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 8 |  |
| n89 |  | Rel-16 |  | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 18, 19, 24, 25, 26, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 73, 74, 85 NR Band n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | E-UTRA Band 41, 52, 53 NR Band n77, n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  | E-UTRA Band 11, 21 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 8 |  |
| n7 |  | Rel-15 |  | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 20, 22, 26, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 66, 67, 68, 72, 74, 75, 76, 85, NR Band n77, n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 15, 21, 26 |  |
|  |  |  |  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 15, 21, 26 |  |
|  |  |  |  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 15, 21 |  |
| n8, n81 |  | Rel-15 |  | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | E-UTRA band 3, 7, 22, 41, 42, 43, 52, NR Band n77, n78, n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  | E-UTRA 8 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 15 |  |
|  |  |  |  | E-UTRA Band 11, 21 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 8 |  |
| n12 |  | Rel-15 |  | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 50, 53, 70, 71, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | E-UTRA Band 4, 48, 51, 66 NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  | E-UTRA Band 12, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 15 |  |
| n13 |  | Rel-17 |  | E-UTRA Band 2, 4, 5,12, 13, 17, 25, 26, 27, 29, 41, 48, 50, 51, 53, 66, 70, 71, 74, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | E-UTRA Band 14 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 15 |  |
|  |  |  |  | E-UTRA Band 24, 30 NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  | Frequency range |  | 769 |  | - |  | 775 |  | -35 |  | 0.00625 |  | 15 |  |
|  |  |  |  | Frequency range |  | 799 |  | - |  | 805 |  | -35 |  | 0.00625 |  | 11, 15 |  |
| n14 |  | Rel-16 |  | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 23, 24, 25, 26, 27, 29, 30, 41, 48, 53, 66, 70, 71, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  | Frequency range |  | 769 |  | - |  | 775 |  | -35 |  | 0.00625 |  | 12, 15 |  |
|  |  |  |  | Frequency range |  | 799 |  | - |  | 805 |  | -35 |  | 0.00625 |  | 11, 12, 15 |  |
| n18 |  | Rel-16 |  | E-UTRA Band 1, 3, 11, 21, 34, 40, 42, 65 NR Band n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | NR Band n77, n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  | Frequency range |  | 758 |  | - |  | 799 |  | -50 |  | 1 |  |  |  |
|  |  |  |  | Frequency range |  | 799 |  | - |  | 803 |  | -40 |  | 1 |  |  |  |
|  |  |  |  | Frequency range |  | 860 |  | - |  | 890 |  | -40 |  | 1 |  |  |  |
|  |  |  |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |  |
|  |  |  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 8 |  |
|  |  |  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |  |
|  |  |  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |  |
| n20, n82 |  | Rel-15 |  | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | E-UTRA Band 20 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 15 |  |
|  |  |  |  | E-UTRA Band 38, 42, 52, 69, NR Band n77, n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  | Frequency range |  | 758 |  | - |  | 788 |  | -50 |  | 1 |  |  |  |
| n24, n99 |  | Rel-17 |  | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 66, 70, 71, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
| n25 |  | Rel-15 |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 53, 66, 70, 71, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | E-UTRA Band 2 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 15 |  |
|  |  |  |  | E-UTRA Band 25 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 15 |  |
|  |  |  |  | E-UTRA Band 43, NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
| n26 |  | Rel-16 |  | E-UTRA Band 1, 2, 3, 4, 5, 11, 12, 13, 14, 17, 18,19, 21, 24, 25, 26, 29, 30, 31, 34, 39, 40, 42, 43, 48, 50, 51, 65, 66, 70, 71, 73,74, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | E-UTRA Band 41, 53 NR Band n77, n78, n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  | Frequency range |  | 703 |  | - |  | 799 |  | -50 |  | 1 |  |  |  |
|  |  |  |  | Frequency range |  | 799 |  | - |  | 803 |  | -40 |  | 1 |  | 15 |  |
|  |  |  |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |  |
|  |  |  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 8 |  |
| n28, n83 |  | Rel-15 |  | E-UTRA Band 1, 4, 22, 32, 42, 43, 50, 51, 65, 66, 74, 75, 76, NR Band n77, n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  | E-UTRA Band 1 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 19, 25 |  |
|  |  |  |  | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 20, 25, 26, 27, 31, 34, 38, 39, 40, 41, 52, 72, 73 NR Band n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | E-UTRA Band 11, 21 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 19, 24 |  |
|  |  |  |  | Frequency range |  | 470 |  | - |  | 694 |  | -42 |  | 8 |  | 15, 35 |  |
|  |  |  |  | Frequency range |  | 470 |  | - |  | 710 |  | -26.2 |  | 6 |  | 34 |  |
|  |  |  |  | Frequency range |  | 662 |  | - |  | 694 |  | -26.2 |  | 6 |  | 15 |  |
|  |  |  |  | Frequency range |  | 758 |  | - |  | 773 |  | -32 |  | 1 |  | 15 |  |
|  |  |  |  | Frequency range |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |  |
|  |  |  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 8, 19 |  |
| n30 |  | Rel-16 |  | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 38, 41, 48, 53, 66, 70, 71, 85, NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
| n34 |  | Rel-15 |  | E-UTRA Band 1, 3, 7, 8, 11, 18, 19, 20, 21, 22, 26, 28, 31, 32, 33, 38,39, 40, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 69, 72, 74, 75, 76, NR Band n78, n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |  |
|  |  |  |  | NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 8 |  |
| n38 |  | Rel-15 |  | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13, 14, 17, 20, 22, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 66, 67, 68, 72, 74, 75, 76, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | NR Band n77, n78, n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | Frequency range |  | 2620 |  | - |  | 2645 |  | -15.5 |  | 5 |  | 15, 22, 26 |  |
|  |  |  |  | Frequency range |  | 2645 |  | - |  | 2690 |  | -40 |  | 1 |  | 15, 22 |  |
| n39 |  | Rel-15 |  | E-UTRA Band 1, 8, 22, 26, 28, 34, 40, 41, 42, 44, 45, 50, 51, 52, 74, NR Band n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | NR Band n77, n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  | Frequency range |  | 1805 |  | - |  | 1855 |  | -40 |  | 1 |  | 33 |  |
|  |  |  |  | Frequency range |  | 1855 |  | - |  | 1880 |  | -15.5 |  | 5 |  | 15, 26, 33 |  |
| n98 |  | Rel-17 |  | E-UTRA Band 1, 8, 22, 26, 28, 34, 40, 41, 42, 44, 45, 50, 51, 52, 74, NR Band n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | NR Band n77, n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  | Frequency range |  | 1805 |  | - |  | 1855 |  | -40 |  | 1 |  | 33 |  |
|  |  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 15, 26, 33 |
| n40 |  | Rel-15 |  | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 74, 75, 76, NR Band n77, n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 44 |  |
|  |  |  |  | NR Band n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 8 |  |
| n97 |  | Rel-17 |  | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 74, 75, 76, NR Band n77, n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 44 |  |
|  |  |  |  | NR Band n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 8 |  |
| n41 |  | Rel-15 |  | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 42, 44, 45, 48, 50, 51, 52, 65, 66, 70, 71, 73, 74, 85, NR Band n77, n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | NR Band n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  | E-UTRA Band 11, 18, 19, 21 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 8 |
| n41 |  | Rel-17 |  | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 42, 44, 45, 48, 50, 51, 52, 65, 66, 70, 71, 73, 74, 85, NR Band n77, n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | E-UTRA Band 40 |  | FDL_low |  | - |  | FDL_high |  | -40 |  | 1 |  |  |  |
|  |  |  |  | NR Band n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  | E-UTRA Band 11, 18, 19, 21 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | Frequency range |  | 1884.5 |  |  |  | 1915.7 |  | -41 |  | 0.3 |  | 8 |  |
| n47 |  | Rel-16 |  | E-UTRA Band 1, 3, 5, 7, 8, 22, 26, 28, 34, 39, 40, 41, 42, 44, 45, 65, 68, 72, 73 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | NR Band n71, n77, n78, n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
| n48 | Rel-16 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| n50 | Rel-15 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 20, 26, 28, 29, 31, 34, 38, 39, 40, 41, 42, 43, 48, 65, 66, 67, 68 | FDL_low | - | FDL_high | -50 | 1 |  |
| n51 | Rel-15 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 20, 26, 28, 29, 31, 34, 38, 39, 40, 41, 42, 43, 48, 52, 65, 66, 67, 68, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| n53 |  | Rel-16 |  | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 48, 66, 70, 71, 85, NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
| n65 |  | Rel-16 |  | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 40, 41, 42, 43, 50, 51, 65, 68, 69, 72, 74, 75, 76, NR Band n78, n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  | E-UTRA Band 34 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 43 |  |
|  |  |  |  | Frequency range |  | 1900 |  | - |  | 1915 |  | -15.5 |  | 5 |  | 15, 26, 27 |  |
|  |  |  |  | Frequency range |  | 1915 |  | - |  | 1920 |  | +1.6 |  | 5 |  | 15, 26, 27 |  |
| n66, n86 |  | Rel-15 |  | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 25, 26, 27, 28, 29, 30, 38, 41, 43, 50, 51, 53, 66, 70, 71, 74, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | E-UTRA Band 42, 48, NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
| n70 |  | Rel-15 |  | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 66, 70, 71, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | NR Band n47, n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
| n71 |  | Rel-15 |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 53, 66, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | E-UTRA Band 2, 25, 41, 70, NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  | E-UTRA Band 29 |  | FDL_low |  | - |  | FDL_high |  | -38 |  | 1 |  | 15 |  |
|  |  |  |  | E-UTRA Band 71 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 15 |  |
| n74 |  | Rel-15 |  | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 18, 19, 20, 26, 28, 29, 31, 34, 38, 39, 40, 41, 42, 43, 48, 52, 65, 66, 67, 68, 85 NR Band n77, n78, 103 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | NR Band n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 8 |  |
|  |  |  |  | Frequency range |  | 1400 |  | - |  | 1427 |  | -32 |  | 27 |  | 15, 41 |  |
|  |  |  |  | Frequency range |  | 1475 |  | - |  | 1488 |  | -28 |  | 1 |  | 15, 42 |  |
|  |  |  |  | Frequency range |  | 1475 |  | - |  | 1488 |  | -50 |  | 1 |  | 15, 45 |  |
|  |  |  |  | Frequency range |  | 1475.9 |  | - |  | 1510.9 |  | -35 |  | 1 |  | 15, 46 |  |
|  |  |  |  | Frequency range |  | 1488 |  | - |  | 1518 |  | -50 |  | 1 |  | 15 |  |
| n77 |  | Rel-15 |  | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65, 74 |  | FDL_low |  | -- |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | Frequency range |  | 1884.5 |  | -- |  | 1915.7 |  | -41 |  | 0.3 |  | 8 |  |
| n77 |  | Rel-16 |  | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 11, 12, 13, 14, 17, 18, 19, 20, 21, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 41, 53, 65, 66, 70, 71, 74, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 8 |  |
| n77 |  | Rel-17 |  | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 11, 12, 13, 14, 17, 18, 19, 20, 21, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 41, 53, 65, 66, 70, 71, 74, 85, 103 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 8 |  |
| n78 |  | Rel-15 |  | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65, 75, 76 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 8 |  |
| n79 |  | Rel-15 |  | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 39, 40, 41, 42, 65, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 8 |  |
| n85 |  | Rel-17 |  | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 53, 70, 71, 74, 103 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | E-UTRA Band 4, 48, 51, 66 NR Band n77, n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  | E-UTRA Band 12, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 15 |  |
| n95 |  | Rel-16 |  | E-UTRA Band 1, 3 , 5, 8, 28, 39, 40, 41, NR Band n78, n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |  |
|  |  |  |  | NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 8 |  |
| n101 |  | Rel-17 |  | E-UTRA Band 1, 3, 8, 20, 22, 28, 31, 32, 33, 34, 38, 40, 43, 50, 51, 52, 65, 67, 68, 69, 72, 74, 75, 76 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | E-UTRA Band 7, 41, 42 NR Band n77, n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  | Frequency range |  | 758 |  | - |  | 788 |  | -50 |  | 1 |  |  |  |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 in TS 38.101-1 [2] or Table 5.5-1 in TS 36.101 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: 15 kHz SCS is assumed when RB is mentioned in the note when channel bandwidth is less than or equal to 50 MHz, lowest SCS is assumed when channel bandwidth is larger than 50 MHz. The transmission bandwidth in terms of RB position and range is not limited to 15 kHz SCS and shall scale with SCS accordingly. NOTE 4: Void NOTE 5: For non-synchronised TDD operation to meet these requirements some restriction will be needed for either the operating band or protected band NOTE 6: N/A NOTE 7: Void NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. NOTE 9: Void NOTE 10: Void NOTE 11: Void NOTE 12: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB NOTE 13: Void NOTE 14: Void NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 16: Void NOTE 17: Void NOTE 18: Void NOTE 19: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 20: Void NOTE 21: This requirement is applicable for any channel bandwidths up to 20MHz within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 22: This requirement is applicable for power class 3 UE for any channel bandwidths up to 20 MHz. For channel bandwidth within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when the carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when the carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. For carriers overlapping the frequency range 2615 - 2620 MHz the requirement applies with the maximum output power configured to +19 dBm in the IE P-Max. NOTE 23: Void NOTE 24: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned NR carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.3.1-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 25: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned NR carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.3.1-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 26: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 27: This requirement is applicable for channel bandwidths up to 20 MHz within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when the carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when the carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 28: Void NOTE 29: Void NOTE 30: Void NOTE 31: Void NOTE 32: Void NOTE 33: This requirement is only applicable for carriers with bandwidth up to 20MHz and confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier center frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier center frequency is within the range 1895 - 1903 MHz. NOTE 34: This requirement is applicable for 5 and 10 MHz NR channel bandwidth allocated within 718-728 MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart < 48. NOTE 35: This requirement is applicable in the case of a 10 MHz NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 36: Void NOTE 37: Void NOTE 38: Void NOTE 39: Void NOTE 40: Void NOTE 41: Applicable for cases and when the lower edge of the assigned NR UL channel bandwidth frequency is greater than or equal to 1427 MHz + the channel BW assigned for 5 and 10 MHz bandwidth, and when the lower edge of the assigned NR UL channel bandwidth frequency is greater than or equal to 1440 MHz for 15 and 20 MHz bandwidth. This requirement shall be verified with UE transmission power of 15 dBm. NOTE 42: Applicable when upper edge of the assigned NR UL channel bandwidth frequency is more than 1460MHz and more than 1460MHz and less than or equal to 1470MHz for 5 MHz bandwidth, and when the upper edge of the assigned NR UL channel bandwidth frequency is less than or equal to 1465 MHz for 10 MHz bandwidth. NOTE 43: This requirement is applicable for NR channel bandwidth allocated within 1920-1980 MHz. NOTE 44: As exceptions, for 90 and 100 MHz channel bandwidth, -40 dBm/MHz is applicable in the frequency range of 2496 – 2505 MHz. NOTE 45: Applicable when upper edge of the assigned NR UL channel bandwidth frequency is equal to or less than 1460MHz. NOTE 46: Applicable for 5MHz bandwidth and when the NR carrier is within 1447.9 – 1462.9 MHz. |  |  |  |  |  |  |  |  |

NOTE:	To simplify Table 6.5.3.2.3-4, E-UTRA band numbers are listed for bands which are specified only for E-UTRA operation or both E-UTRA and NR operation. NR band numbers are listed for bands which are specified only for NR operation.

The normative reference for this requirement is TS 38.101-1 [2] subclause 6.5.3.2. This test use minimum requirements from many releases of TS 38.101-1 [2] due to release independence defined in TS 38.307 [23].

6.5.3.2.4	Test description

6.5.3.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in Table 6.5.3.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.3.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1. |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1. |  | Low range, Mid range, High range (NOTE 2) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1. |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A | Modulation | RB allocation (NOTE 1) |
| 1 |  | CP-OFDM QPSK | Outer_Full |
| 2 |  | CP-OFDM QPSK | Edge_1RB_Left |
| 3 |  | CP-OFDM QPSK | Edge_1RB_Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 Common UL configuration. NOTE 2: For NR band n28, 30MHz test channel bandwidth is tested with Low range and High range test frequencies. |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.5.3.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5.  Message contents are defined in clause 6.5.3.2.4.3.

6.5.3.2.4.2	Test procedure

1	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5.3.2.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to Tables 6.5.3.2.3-4. The centre frequency of the filter shall be stepped in contiguous steps according to Tables 6.5.3.2.3-4. The measured power shall be verified for each step. The measurement period shall capture the active time slots.

6.5.3.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6.

6.5.3.2.5	Test requirement

Test requirements for Spurious Emissions UE Co-existence are the same as the minimum requirements and are not repeated in this section.

Unless otherwise stated, the spurious emission limits apply for the frequency ranges that are more than FOOB (MHz) in Tables 6.5.3.1.3-1 from the edge of the channel bandwidth. The spurious emission limits in Tables 6.5.3.2.3-4 apply for all transmitter band configurations (NRB) and channel bandwidths.

The measured average power of spurious emission, derived in step 3, shall not exceed the described value in Table 6.5.3.2.3-4 for difference releases.

The requirements for the UE are release specific and can be found in Tables 6.5.3.2.3-4. If the UE support a band, which is not defined in the table corresponding UE’s release, the requirements for this band are taken from the table of earliest release where requirements for this band are defined. This has been described in following Table 6.5.3.2.5-1.

Table 6.5.3.2.5-1: UE Requirements according to UE NR release and supported E-UTRA and NR band

| UE Requirements per release |  |  |  |
| --- | --- | --- | --- |
| NR Band | Rel-15 | Rel-16 | R17 |
| n1 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 |
| n2 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 |
| n3 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 |
| n5 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 |
| n7 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 |
| n8 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 |
| n12 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 |
| n14 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 |
| n18 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 |
| n20 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 |
| n24 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 |
| n25 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 |
| n26 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 |
| n28 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 |
| n30 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 |
| n34 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 |
| n38 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 |
| n39 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 |
| n40 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 |
| n41 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 |
| n48 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 |
| n50 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 |
| n51 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 |
| n53 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 |
| n65 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 |
| n66 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 |
| n70 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 |
| n71 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 |
| n74 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 |
| n77, n78 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 |
| n79 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 |
| NOTE 1: The frequency range applicable with network signalling values of NS_04, NS_17, NS_18, NS_05, NS_43, NS_37, NS_38, NS_39, NS_40, NS_41, NS_42, NS_45 and NS_56 are covered in subclause 6.5.3.3 Additional Spurious Emissions NOTE 2: The restriction on the maximum uplink transmission to 54 RB in Notes 21 and 22 of Tables 6.5.3.2.3-4 is intended for conformance testing and may be applied to network operation to facilitate coexistence when the aggressor and victim bands are deployed in the same geographical area. The applicable spurious emission requirement of -15.5 dBm/5MHz is a least restrictive technical condition for FDD/TDD coexistence and may have to be revised in the future. |  |  |  |

<< Unchanged sections omitted >>

6.5A.3.2.0.3	Spurious emissions for UE co-existence for Inter-band CA

For inter-band carrier aggregation with the uplink assigned to two NR bands, the requirements in Table 6.5A.3.2.0.3-4 apply on each component carrier with both component carriers are active.

NOTE:	For inter-band carrier aggregation with uplink assigned to two NR bands, the requirements in Table 6.5A.3.2.0.3-4 could be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur; in that case, the requirements for remaining applicable frequencies in Table 6.5A.3.2.0.3-4 would be considered to be verified by the measurements verifying the one uplink inter-band CA UE to UE co-existence requirements.

Table 6.5A.3.2.0.3-1: Void

Table 6.5A.3.2.0.3-2: Void

Table 6.5A.3.2.0.3-3: Void

Table 6.5A.3.2.0.3-4: Requirements for uplink inter-band carrier aggregation (two bands)

| NR CA Configuration |  |  |  | Release |  |  |  |  | Spurious emission |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Protected Band | Frequency range (Mhz) |  |  |  |  |  | Maximum Level (dBm) |  | MBW (MHz) |  | NOTE |  |
| CA_n1-n28 |  |  |  |  | Rel-15 |  | E-UTRA Band 5, 7, 8, 18, 19, 20, 26, 27, 31, 38, 40, 41, 72, 73 NR band n79 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  |  |  | E-UTRA Band 1, 22, 32, 42, 43, 50, 51, 52, 65, 74, 75, 76 NR band n77, n78 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  |  |  |  | E-UTRA Band 3, 34 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 4 |  |
|  |  |  |  |  |  |  | E-UTRA Band 11, 21 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 11, 12 |  |
|  |  |  |  |  |  |  | E-UTRA Band 1, 65 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 11, 15 |  |
|  |  |  |  |  |  |  | Frequency range |  |  | 470 |  | - |  | 694 |  | -42 |  | 8 |  | 4, 14 |  |
|  |  |  |  |  |  |  | Frequency range |  |  | 470 |  | - |  | 710 |  | -26.2 |  | 6 |  | 15 |  |
|  |  |  |  |  |  |  | Frequency range |  |  | 758 |  | - |  | 773 |  | -30 |  | 1 |  | 4 |  |
|  |  |  |  |  |  |  | Frequency range |  |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  |  |  | Frequency range |  |  | 662 |  | - |  | 694 |  | -26.2 |  | 6 |  | 4 |  |
|  |  |  |  |  |  |  | Frequency range |  |  | 1880 |  | - |  | 1895 |  | -40 |  | 1 |  | 4, 6 |  |
|  |  |  |  |  |  |  | Frequency range |  |  | 1895 |  | - |  | 1915 |  | -15.5 |  | 5 |  | 4, 6, 7 |  |
|  |  |  |  |  |  |  | Frequency range |  |  | 1915 |  | - |  | 1920 |  | +1.6 |  | 5 |  | 4, 6, 7 |  |
| CA_n1-n40 |  |  |  |  | Rel-15 |  | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76 NR band n78 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  |  |  | Band 3, 34 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 4 |  |
|  |  |  |  |  |  |  | NR band n77, n79 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  |  |  |  | Frequency range |  |  | 1880 |  |  |  | 1895 |  | -40 |  | 1 |  | 4, 14 |  |
|  |  |  |  |  |  |  | Frequency range |  |  | 1895 |  |  |  | 1915 |  | -15.5 |  | 5 |  | 4, 7, 14 |  |
|  |  |  |  |  |  |  | Frequency range |  |  | 1915 |  |  |  | 1920 |  | +1.6 |  | 5 |  | 4, 7, 14 |  |
|  |  |  |  |  |  |  | Frequency range |  |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |
| CA_n1-n78 |  |  | Rel-15 |  |  |  |  |  | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 40, 41, 65, 74 | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  |  |  |  |  | Frequency range | 1880 |  | - |  | 1895 |  | -40 |  | 1 |  | 4, 6 |  |
|  |  |  |  |  |  |  |  |  | Frequency range | 1895 |  | - |  | 1915 |  | -15.5 |  | 5 |  | 4, 6, 7 |  |
|  |  |  |  |  |  |  |  |  | Frequency range | 1915 |  | - |  | 1920 |  | +1.6 |  | 5 |  | 4, 6, 7 |  |
| CA_n1-n79 |  | Rel-15 |  |  |  |  |  |  | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 40, 41, 42, 65, 74 | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  |  |  |  |  | Frequency range | 1880 |  | - |  | 1895 |  | -40 |  | 1 |  | 4, 6 |  |
|  |  |  |  |  |  |  |  |  | Frequency range | 1895 |  | - |  | 1915 |  | -15.5 |  | 5 |  | 4, 6, 7 |  |
|  |  |  |  |  |  |  |  |  | Frequency range | 1915 |  | - |  | 1920 |  | +1.6 |  | 5 |  | 4, 6, 7 |  |
| CA_n3-n28 |  |  |  |  | Rel-15 |  | E-UTRA Band 5, 7, 8, 18, 19, 20, 26, 27, 31 38, 40, 41, 72 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  |  |  | E-UTRA Band 32, 42, 43, 50, 51, 74, 75, 76 NR band n77, n78, n79 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  |  |  |  | E-UTRA Band 3, 34 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 4 |  |
|  |  |  |  |  |  |  | E-UTRA Band 11, 21 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 11, 12 |  |
|  |  |  |  |  |  |  | E-UTRA Band 1, 65 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 11, 15 |  |
|  |  |  |  |  |  |  | Frequency range |  |  | 470 |  | - |  | 694 |  | -42 |  | 8 |  | 4, 14 |  |
|  |  |  |  |  |  |  | Frequency range |  |  | 470 |  | - |  | 710 |  | -26.2 |  | 6 |  | 15 |  |
|  |  |  |  |  |  |  | Frequency range |  |  | 758 |  | - |  | 773 |  | -30 |  | 1 |  | 4 |  |
|  |  |  |  |  |  |  | Frequency range |  |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  |  |  | Frequency range |  |  | 662 |  | - |  | 694 |  | -26.2 |  | 6 |  | 4 |  |
|  |  |  |  |  |  |  | Frequency range |  |  | 1880 |  | - |  | 1895 |  | -40 |  | 1 |  | 4, 6 |  |
|  |  |  |  |  |  |  | Frequency range |  |  | 1895 |  | - |  | 1915 |  | -15.5 |  | 5 |  | 4, 6, 7 |  |
|  |  |  |  |  |  |  | Frequency range |  |  | 1915 |  | - |  | 1920 |  | +1.6 |  | 5 |  | 4, 6, 7 |  |
|  |  |  |  |  |  |  | Frequency range |  |  | 1839.9 |  | - |  | 1879.9 |  | -50 |  | 1 |  | 4 |  |
|  |  |  |  |  |  |  | Frequency range |  |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 11 |  |
| CA_n3-n40 |  |  |  |  | Rel-15 |  | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 43, 44. 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  |  |  | E-UTRA Band 3 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 4 |  |
|  |  |  |  |  |  |  | UTRA Band 22, 42, 52 NR Band n77, n78, n79 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  |  |  |  | Frequency range |  |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |
| CA_n3-n41 |  |  |  |  | Rel-16 |  | E-UTRA Band 1, 5, 8, 11, 18, 19, 20, 21, 26, 27, 28, 34, 39, 44, 45, 50, 51, 65, 73, 74 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  |  |  | E-UTRA Band 40 |  |  | FDL_low |  | - |  | FDL_high |  | -40 |  | 1 |  |  |  |
|  |  |  |  |  |  |  | E-UTRA Band 3 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 4 |  |
|  |  |  |  |  |  |  | E-UTRA Band 42, NR Band n77, n78, n79 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  |  |  |  | Frequency range |  |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |
| CA_n3-n78 |  | Rel-15 |  |  |  |  |  |  | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65, 74 | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  |  |  |  |  | Frequency range | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |
| CA_n5-n77 |  |  |  |  | Rel-16 |  | E-UTRA Band 1, 2, 3, 4, 8, 11, 12, 13, 14, 17, 18, 19, 21, 25, 26, 28, 29, 30, 34, 40, 65, 66, 70, 71, 74 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  |  |  | E-UTRA Band 41 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  |  |  |  | Frequency range |  |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |
| CA_n8-n39 |  | Rel-16 |  |  |  |  |  |  | E-UTRA Band 1, 34, 40, 50, 51, 74 | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  |  |  |  |  | E-UTRA Band 22, 41, 42 NR Band n77, n78, n79 | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  |  |  |  |  |  | E-UTRA Band 8 | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 4 |  |
| CA_n8-n40 |  |  |  |  | Rel-16 |  | E-UTRA Bands 1, 5, 11, 18, 19, 20, 21, 26, 28, 31, 32, 33, 34, 38, 39, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  |  |  | E-UTRA Bands 3, 7, 22, 41, 42, 43, 52 NR Bands n77, n78, n79 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  |  |  |  | E-UTRA Band 8 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 4 |  |
|  |  |  |  |  |  |  | Frequency range |  |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |
| CA_n8-n78 |  | Rel-15 |  |  |  |  |  |  | E-UTRA Band 1, 8, 11, 20, 21, 28, 34, 39, 40, 65, 74 | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  |  |  |  |  | E-UTRA Band 3, 7, 41 | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  |  |  |  |  |  | Frequency range | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |
| CA_n24-n41 |  | Rel-17 |  |  |  |  |  |  | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 25, 26, 29, 30, 48, 66, 70, 71, 85 | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  |  |  |  |  | NR Band n77 | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
| CA_n24-n48 |  | Rel-17 |  |  |  |  |  |  | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 25, 26, 29, 30, 41, 66, 70, 71, 85 | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
| CA_n24-n77 |  | Rel-17 |  |  |  |  |  |  | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 25, 26, 29, 30, 41, 66, 70, 71, 85 | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
| CA_n26-n66 |  | Rel-17 |  |  |  |  |  |  | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 29, 30, 43, 47, 50, 51, 66, 70, 71, 74, 85 | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  |  |  |  |  | E-UTRA Band 41, 42, 48, 53 NR band 77 | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  |  |  |  |  |  | Frequency range | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |
| CA_n26-n70 |  | Rel-17 |  |  |  |  |  |  | E-UTRA Band 2, 5, 10, 12, 13, 14, 17, 24, 25, 29, 30, 48, 66, 70, 71, 85 | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  |  |  |  |  | E-UTRA Band 41, 53 | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  |  |  |  |  |  | Frequency range | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |
| CA_n28-n40 |  |  |  |  | Rel-16 |  | E-UTRA Band 1, 3, 5, 7, 8, 18, 19, 20, 26, 27, 28, 31, 34, 38, 41, 72 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  |  |  | E-UTRA Band 11, 21, 22, 32, 42, 43, 50, 51, 52, 65, 73, 74, 75, 76 NR band n77, n78, n79 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  |  |  |  | Frequency range |  |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |
| CA_n28-n41 |  | Rel-16 |  |  |  |  |  |  | E-UTRA Band 2, 3, 5, 8, 25, 26, 27, 34 | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  |  |  |  |  | E-UTRA Band 4, 42, 50, 51, 52, 65, 66, 73, 74 NR Band n77, n78, n79 | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  |  |  |  |  |  | E-UTRA Band 18, 19 | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 11 |  |
|  |  |  |  |  |  |  |  |  | E-UTRA Band 1 | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 11, 15 |  |
|  |  |  |  |  |  |  |  |  | E-UTRA Band 11, 21 | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 11, 12 |  |
|  |  |  |  |  |  |  |  |  | Frequency range | 470 |  | - |  | 694 |  | -42 |  | 8 |  | 4, 14 |  |
|  |  |  |  |  |  |  |  |  | Frequency range | 470 |  | - |  | 710 |  | -26.2 |  | 6 |  | 13 |  |
|  |  |  |  |  |  |  |  |  | Frequency range | 662 |  | - |  | 694 |  | -26.2 |  | 6 |  | 4 |  |
|  |  |  |  |  |  |  |  |  | Frequency range | 758 |  | - |  | 773 |  | -32 |  | 1 |  | 4 |  |
|  |  |  |  |  |  |  |  |  | Frequency range | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  |  |  |  |  | Frequency range | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 11 |  |
| CA_n39-n40 |  | Rel-16 |  |  |  |  |  |  | E-UTRA Band 1, 8, 22, 26, 28, 34, 41, 42, 44, 45, 50, 51, 52, 73, 74 | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  |  |  |  |  | NR Band n77, n78, n79 | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  |  |  |  |  |  | Frequency range | 1805 |  |  |  | 1855 |  | -40 |  | 1 |  | 8 |  |
|  |  |  |  |  |  |  |  |  | Frequency range | 1855 |  |  |  | 1880 |  | -15.5 |  | 5 |  | 4, 7, 8 |  |
| CA_n39-n41 |  | Rel-16 |  |  |  |  |  |  | E-UTRA Band 1, 8, 26, 28, 34, 40, 42, 44, 45, 50, 51, 74 | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  |  |  |  |  | NR Band n77, n78, n79 | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  |  |  |  |  |  | Frequency range | 1805 |  | - |  | 1855 |  | -40 |  | 1 |  | 4 |  |
|  |  |  |  |  |  |  |  |  | Frequency range | 1855 |  | - |  | 1880 |  | -15.5 |  | 5 |  | 4, 7, 8 |  |
| CA_n39-n79 |  | Rel-16 |  |  |  |  |  |  | E-UTRA Band 1, 8, 28, 34, 40, 41, 44, 45 | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  |  |  |  |  | NR Band n78 | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  |  |  |  |  |  | Frequency range | 1805 |  | - |  | 1855 |  | -40 |  | 1 |  | 4, 8 |  |
|  |  |  |  |  |  |  |  |  | Frequency range | 1855 |  | - |  | 1880 |  | -15.5 |  | 5 |  | 4, 7, 8 |  |
| CA_n40-n78 |  |  |  |  | Rel-15 |  | UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  |  |  | NR Band n79 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  |  |  |  | Frequency range |  |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |
| CA_n40-n41 |  |  |  |  | Rel-16 |  | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 26, 27, 28, 34, 39, 42, 44, 45, 50, 51, 65, 73, 74, NR Band n77, n78 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  |  |  | NR Band n79 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  |  |  |  | Frequency range |  |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |
| CA_n40-n78 |  |  |  |  | Rel-16 |  | UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  |  |  | NR Band n79 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n40-n79 |  |  |  |  | Rel-15 |  | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 26, 28, 34, 39, 41, 42, 65, 74, NR band n78 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  |  |  | Frequency range |  |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |
| CA_n41-n79 |  | Rel-16 |  |  |  |  |  |  | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 42, 44, 45, 65 | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  |  |  |  |  | E-UTRA Band 40 | FDL_low |  | - |  | FDL_high |  | -40 |  | 1 |  |  |  |
|  |  |  |  |  |  |  |  |  | Frequency range | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |
| CA_n48-n66 |  | Rel-16 |  |  |  |  |  |  | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
| CA_n48-n70 |  | Rel-17 |  |  |  |  |  |  | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 66, 70, 71, 85 | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 16 |  |
|  |  |  |  |  |  |  |  |  | E-UTRA Band 41 | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2, 16 |  |
| CA_n48-n71 |  | Rel-17 |  |  |  |  |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 50, 51, 53, 66, 74, 85 | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  |  |  |  |  | E-UTRA Band 2, 25, 41, 70 | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  |  |  |  |  |  | E-UTRA Band 29 | FDL_low |  | - |  | FDL_high |  | -38 |  | 1 |  | 15 |  |
|  |  |  |  |  |  |  |  |  | E-UTRA Band 71 | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 15 |  |
| CA_n66-n71 |  | Rel-16 |  |  |  |  |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 27, 30, 43, 50, 51, 53, 66, 74, 85 | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  |  |  |  |  | E-UTRA Band 2, 25, 41, 42, 48, 70 NR Band n77 | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  |  |  |  |  |  | E-UTRA Band 29 | FDL_low |  | - |  | FDL_high |  | -38 |  | 1 |  | 4 |  |
|  |  |  |  |  |  |  |  |  | E-UTRA Band 71 | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 4 |  |
| CA_n70-n71 |  | Rel-16 |  |  |  |  |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 27, 30, 48, 66, 74, 85 | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  |  |  |  |  | E-UTRA Band 2, 7, 25, 41, 70, NR Band n77 | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  |  |  |  |  |  | E-UTRA Band 29 | FDL_low |  | - |  | FDL_high |  | -38 |  | 1 |  | 4 |  |
|  |  |  |  |  |  |  |  |  | E-UTRA Band 71 | FDL_low |  | - |  | FDL_high |  | -38 |  | 1 |  | 4 |  |
| CA_n78-n79 |  |  |  |  | Rel-15 |  | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 40, 41, 65, 74 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  |  |  | Frequency range |  |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 or Table 5.5-1 in TS 36.101 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 and Table 6.5A.3.1-1 from the edge of the channel bandwidth. NOTE 5: Void. NOTE 6: This requirement is applicable for any channel bandwidths within the range 1920 – 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 – 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. NOTE 9: Void. NOTE 10: Void. NOTE 11: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 12: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned NR carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.3.1-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 13: This requirement is applicable for 5 and 10 MHz NR channel bandwidth allocated within 718 - 728 MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and Rbstart < 48. NOTE 14: This requirement is applicable in the case of a 10 MHz NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 15: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 16: The same configuration applies to corresponding NR-DC configuration in Table 4.3.1.1.7.1-1 in [5]. If UE supporting NR-DC configuration do not support the corresponding CA configuration, NR-DC configuration is used in this test. |  |  |  |  |  |  |  |  |

NOTE:	To simplify Table 6.5A.3.2.0.3-4, E-UTRA band numbers are listed for bands which are specified only for E-UTRA operation or both E-UTRA and NR operation. NR band numbers are listed for bands which are specified only for NR operation.

The normative reference for this requirement is TS 38.101-1 [2] subclause 6.5A.3.2.

6.5A.3.2.1	Spurious emissions for UE co-existence for CA (2UL CA)

6.5A.3.2.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to co-existing systems for the specified bands which has specific requirements in terms of transmitter spurious emissions for 2UL CA.

6.5A.3.2.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support 2UL CA.

6.5A.3.2.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in subclause 6.5A.3.2.0.

6.5A.3.2.1.4	Test description

6.5A.3.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in Table 6.5A.3.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5A.3.2.1.4.1-1: Inter band CA Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Normal |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Refer to “PCC NRB@SCS”and “SCC NRB@SCS ” columns |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test SCS as specified in Table 5.5A.3-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Lowest SCS for default test points. CA configuration specific test points: Refer to “PCC NRB@SCS”and “SCC NRB@SCS ” columns |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID |  |  |  | CA Configuration / NRB_agg (Note 4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DL Allocation |  |  |  |  |  |  |  |  | UL Allocation (Note 2,3) |  |  |  |  |  |  |
|  |  |  |  | CA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PCC NRB @SCS |  |  |  | SCC NRB@SCS |  |  | CC MOD |  |  |  | PCC & SCC RB allocation |  |  |  |  | CC MOD |  | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |  |
|  |  |  |  | PCC |  |  |  |  |  |  |  | SCC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PCC | SCC |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Band |  |  |  | Range |  |  |  | Band |  |  |  | Range |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a CA_XA-YA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | X |  |  |  | Low |  |  |  | Y |  |  |  | Low |  |  |  | Highest NRB@SCS |  |  | Highest NRB@SCS |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 2 |  | X |  |  |  | High |  |  |  | Y |  |  |  | High |  |  |  | Highest NRB@SCS |  |  | Highest NRB@SCS |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  |  | 1@RBmax |  |  |  | 1@RBmax |  |
| Test Settings for CA_n1A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 |  |  |  | High |  |  |  | n78 |  |  |  | High |  |  |  | 106@15kHz |  |  |  |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  |  | 1@105 |  |  | 1@272 |
| 2 | n1 |  |  |  | Low |  |  |  | n78 |  |  |  | 3455MHz |  |  |  | 106@15kHz |  |  |  |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  |  | 1@0 |  |  | 1@0 |
| 3 | n1 |  |  |  | Low |  |  |  | n78 |  |  |  | 3475MHz |  |  |  | 106@15kHz |  |  |  |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  |  | 1@0 |  |  | 1@0 |
| 4 | n1 |  |  |  | High |  |  |  | n78 |  |  |  | High |  |  |  | 106@15kHz |  |  |  |  |  | 106@30kHz |  |  | CP-OFDM QPSK |  |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  |  | 1@105 |  |  | 106@0 |
| 5 | n1 |  |  |  | Low |  |  |  | n78 |  |  |  | High |  |  |  | 106@15kHz |  |  |  |  |  | 106@30kHz |  |  | CP-OFDM QPSK |  |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  |  | 1@0 |  |  | 106@0 |
| Test Settings for CA_n3A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | n3 |  |  |  | Mid |  |  |  | n78 |  |  |  | Mid |  |  |  | 160@15kHz |  |  | 270@15kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@160 |  |  | 1@270 |  |  |
| 2 |  |  | n3 |  |  |  | Mid |  |  |  | n78 |  |  |  | Mid |  |  |  | 78@30KHz |  |  | 273@30KHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@78 |  |  | 1@273 |  |  |
| 3 |  |  | n3 |  |  |  | Mid |  |  |  | n78 |  |  |  | Mid |  |  |  | 38@60KHz |  |  | 135@60KHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@38 |  |  | 1@135 |  |  |
| Test Settings for CA_n24A-n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | n24 |  |  |  | High |  |  |  | n41 |  |  |  | Low |  |  |  | 52@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@136 |  |  |
| 2 |  |  | n24 |  |  |  | Low |  |  |  | n41 |  |  |  | Low |  |  |  | 52@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@52 |  |  | 1@108 |  |  |
| 3 |  |  | n24 |  |  |  | High |  |  |  | n41 |  |  |  | Low |  |  |  | 52@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@136 |  |  |
| 4 |  |  | n24 |  |  |  | Low |  |  |  | n41 |  |  |  | Low |  |  |  | 52@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@94 |  |  |
| 5 |  |  | n24 |  |  |  | High |  |  |  | n41 |  |  |  | High |  |  |  | 52@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@192 |  |  |
| Test Settings for CA_n24A-n48A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | n24 |  |  |  | High |  |  |  | n48 |  |  |  | Mid |  |  |  | 52@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@136 |  |  |
| 2 |  |  | n24 |  |  |  | High |  |  |  | n48 |  |  |  | High |  |  |  | 52@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@0 |  |  |
| Test Settings for CA_n24A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | n24 |  |  |  | Low |  |  |  | n77 |  |  |  | Mid |  |  |  | 52@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@136 |  |  |
| 2 |  |  | n24 |  |  |  | Low |  |  |  | n77 |  |  |  | 3930 MHz |  |  |  | 52@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@273 |  |  |
| 3 |  |  | n24 |  |  |  | Low |  |  |  | n77 |  |  |  | 3930 MHz |  |  |  | 52@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@262 |  |  |
| 4 |  |  | n24 |  |  |  | Low |  |  |  | n77 |  |  |  | 3930 MHz |  |  |  | 52@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@52 |  |  | 1@196 |  |  |
| Test Settings for CA_n8A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | n8 |  |  |  | Mid |  |  |  | n78 |  |  |  | Mid |  |  |  | 106@15kHz |  |  | 270@15kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@0 |  |  |
| 2 |  |  | n8 |  |  |  | Mid |  |  |  | n78 |  |  |  | Mid |  |  |  | 106@15kHz |  |  | 270@15kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@106 |  |  | 1@270 |  |  |
| 3 |  |  | n8 |  |  |  | Low |  |  |  | n78 |  |  |  | Low |  |  |  | 51@30KHz |  |  | 273@30KHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@51 |  |  | 1@273 |  |  |
| 4 |  |  | n8 |  |  |  | Mid |  |  |  | n78 |  |  |  | Mid |  |  |  | 51@30KHz |  |  | 273@30KHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@51 |  |  | 1@273 |  |  |
| Test Settings for CA_n28A-n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | n28 |  |  |  | High |  |  |  | n41 |  |  |  | High |  |  |  | 106@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@105 |  |  | 1@272 |  |  |
| 2 |  |  | n28 |  |  |  | Low |  |  |  | n41 |  |  |  | Mid |  |  |  | 106@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@0 |  |  |
| 3 |  |  | n28 |  |  |  | Low |  |  |  | n41 |  |  |  | High |  |  |  | 106@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@272 |  |  |
| 4 |  |  | n28 |  |  |  | Low |  |  |  | n41 |  |  |  | High |  |  |  | 106@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@0 |  |  |
| Test Settings for CA_n41A-n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | n41 |  |  |  | High |  |  |  | n79 |  |  |  | 4870MHz |  |  |  | 273@30KHz |  |  | 273@30KHz |  |  |  | QPSK |  |  | NA |  |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@272 |  |  | 1@0 |  |  |
| 2 |  |  | n41 |  |  |  | Mid |  |  |  | n79 |  |  |  | Low |  |  |  | 273@30KHz |  |  | 273@30KHz |  |  |  | QPSK |  |  | NA |  |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@136 |  |  | 1@136 |  |  |
| 3 |  |  | n41 |  |  |  | High |  |  |  | n79 |  |  |  | Low |  |  |  | 273@30KHz |  |  | 273@30KHz |  |  |  | QPSK |  |  | NA |  |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@272 |  |  | 1@272 |  |  |
| 4 |  |  | n41 |  |  |  | High |  |  |  | n79 |  |  |  | 4450MHz |  |  |  | 273@30KHz |  |  | 273@30KHz |  |  |  | QPSK |  |  | NA |  |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@272 |  |  | 1@136 |  |  |
| 5 |  |  | n41 |  |  |  | High |  |  |  | n79 |  |  |  | 4500MHz |  |  |  | 273@30KHz |  |  | 273@30KHz |  |  |  | QPSK |  |  | NA |  |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@272 |  |  | 1@136 |  |  |
| 6 |  |  | n41 |  |  |  | Mid |  |  |  | n79 |  |  |  | Low |  |  |  | 273@30KHz |  |  | 273@30KHz |  |  |  | QPSK |  |  | NA |  |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@136 |  |  | 1@0 |  |  |
| 7 |  |  | n41 |  |  |  | High |  |  |  | n79 |  |  |  | Mid |  |  |  | 273@30KHz |  |  | 273@30KHz |  |  |  | QPSK |  |  | NA |  |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@272 |  |  | 1@136 |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, in Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [6] subclause4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.5A.3.2.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5A.3.2.1.4.3.

6.5A.3.2.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, and Annex C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 6.5A.3.2.1.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause9.3).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5A.3.2.1.4.1-1 on both PCC and SCC. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to Table 6.5.3.2.3-4. The centre frequency of the filter shall be stepped in contiguous steps according to Table 6.5.3.2.3-4. The measured power shall be verified for each step. The measurement period shall capture the active time slots.

6.5A.3.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with following exception.

Table 6.5A.3.2.1.4.3-1: FrequencyInfoUL-SIB

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 16 |  | Power class 3 and Inter-band CA |

6.5A.3.2.1.5	Test requirement

Test requirements for Spurious Emissions UE Co-existence are the same as the minimum requirements. The measured average power of spurious emission, derived in step 6, shall not exceed the described value in Tables 6.5A.3.2.1.5-1c,. The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

Table 6.5A.3.2.1.5-1: Void

Table 6.5A.3.2.1.5-1a: Void

Table 6.5A.3.2.1.5-1b: Void

Table 6.5A.3.2.1.5-1c: Requirements for uplink inter-band carrier aggregation (two bands)

| NR CA Configuration |  | Release |  |  | Spurious emission |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Protected Band |  |  | Frequency range (Mhz) |  |  |  |  |  | Maximum Level (dBm) |  | MBW (MHz) |  | NOTE |  |
| CA_n1-n28 |  |  | Rel-15 |  |  | E-UTRA Band 5, 7, 8, 18, 19, 20, 26, 27, 31, 38, 40, 41, 72, 73 NR band n79 |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  |  |  |
|  |  |  |  |  |  | E-UTRA Band 1, 22, 32, 42, 43, 50, 51, 52, 65, 74, 75, 76 NR band n77, n78 |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  | 2 |  |
|  |  |  |  |  |  | E-UTRA Band 3, 34 |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  | 4 |  |
|  |  |  |  |  |  | E-UTRA Band 11, 21 |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  | 11, 12 |  |
|  |  |  |  |  |  | E-UTRA Band 1, 65 |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  | 11, 15 |  |
|  |  |  |  |  |  | Frequency range |  | 470 |  | - |  | 694 |  | -42+TT |  | 8 |  | 4, 14 |  |
|  |  |  |  |  |  | Frequency range |  | 470 |  | - |  | 710 |  | -26.2+TT |  | 6 |  | 15 |  |
|  |  |  |  |  |  | Frequency range |  | 758 |  | - |  | 773 |  | -30+TT |  | 1 |  | 4 |  |
|  |  |  |  |  |  | Frequency range |  | 773 |  | - |  | 803 |  | -50+TT |  | 1 |  |  |  |
|  |  |  |  |  |  | Frequency range |  | 662 |  | - |  | 694 |  | -26.2+TT |  | 6 |  | 4 |  |
|  |  |  |  |  |  | Frequency range |  | 1880 |  | - |  | 1895 |  | -40+TT |  | 1 |  | 4, 6 |  |
|  |  |  |  |  |  | Frequency range |  | 1895 |  | - |  | 1915 |  | -15.5+TT |  | 5 |  | 4, 6, 7 |  |
|  |  |  |  |  |  | Frequency range |  | 1915 |  | - |  | 1920 |  | +1.6+TT |  | 5 |  | 4, 6, 7 |  |
| CA_n1-n40 |  |  | Rel-15 |  |  | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76 NR band n78 |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  |  |  |
|  |  |  |  |  |  | Band 3, 34 |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  | 4 |  |
|  |  |  |  |  |  | NR band n77, n79 |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  | 2 |  |
|  |  |  |  |  |  | Frequency range |  | 1880 |  |  |  | 1895 |  | -40+TT |  | 1 |  | 4, 14 |  |
|  |  |  |  |  |  | Frequency range |  | 1895 |  |  |  | 1915 |  | -15.5+TT |  | 5 |  | 4, 7, 14 |  |
|  |  |  |  |  |  | Frequency range |  | 1915 |  |  |  | 1920 |  | +1.6+TT |  | 5 |  | 4, 7, 14 |  |
|  |  |  |  |  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41+TT |  | 0.3 |  | 3 |  |
| CA_n1-n78 |  | Rel-15 |  |  | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 40, 41, 65, 74 |  |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  |  |  |
|  |  |  |  |  | Frequency range |  |  | 1880 |  | - |  | 1895 |  | -40+TT |  | 1 |  | 4, 6 |  |
|  |  |  |  |  | Frequency range |  |  | 1895 |  | - |  | 1915 |  | -15.5+TT |  | 5 |  | 4, 6, 7 |  |
|  |  |  |  |  | Frequency range |  |  | 1915 |  | - |  | 1920 |  | +1.6+TT |  | 5 |  | 4, 6, 7 |  |
| CA_n1-n79 |  | Rel-15 |  |  | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 40, 41, 42, 65, 74 |  |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  |  |  |
|  |  |  |  |  | Frequency range |  |  | 1880 |  | - |  | 1895 |  | -40+TT |  | 1 |  | 4, 6 |  |
|  |  |  |  |  | Frequency range |  |  | 1895 |  | - |  | 1915 |  | -15.5+TT |  | 5 |  | 4, 6, 7 |  |
|  |  |  |  |  | Frequency range |  |  | 1915 |  | - |  | 1920 |  | +1.6+TT |  | 5 |  | 4, 6, 7 |  |
| CA_n3-n28 |  |  | Rel-15 |  |  | E-UTRA Band 5, 7, 8, 18, 19, 20, 26, 27, 31 38, 40, 41, 72 |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  |  |  |
|  |  |  |  |  |  | E-UTRA Band 32, 42, 43, 50, 51, 74, 75, 76 NR band n77, n78, n79 |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  | 2 |  |
|  |  |  |  |  |  | E-UTRA Band 3, 34 |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  | 4 |  |
|  |  |  |  |  |  | E-UTRA Band 11, 21 |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  | 11, 12 |  |
|  |  |  |  |  |  | E-UTRA Band 1, 65 |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  | 11, 15 |  |
|  |  |  |  |  |  | Frequency range |  | 470 |  | - |  | 694 |  | -42+TT |  | 8 |  | 4, 14 |  |
|  |  |  |  |  |  | Frequency range |  | 470 |  | - |  | 710 |  | -26.2+TT |  | 6 |  | 15 |  |
|  |  |  |  |  |  | Frequency range |  | 758 |  | - |  | 773 |  | -30+TT |  | 1 |  | 4 |  |
|  |  |  |  |  |  | Frequency range |  | 773 |  | - |  | 803 |  | -50+TT |  | 1 |  |  |  |
|  |  |  |  |  |  | Frequency range |  | 662 |  | - |  | 694 |  | -26.2+TT |  | 6 |  | 4 |  |
|  |  |  |  |  |  | Frequency range |  | 1880 |  | - |  | 1895 |  | -40+TT |  | 1 |  | 4, 6 |  |
|  |  |  |  |  |  | Frequency range |  | 1895 |  | - |  | 1915 |  | -15.5+TT |  | 5 |  | 4, 6, 7 |  |
|  |  |  |  |  |  | Frequency range |  | 1915 |  | - |  | 1920 |  | +1.6+TT |  | 5 |  | 4, 6, 7 |  |
|  |  |  |  |  |  | Frequency range |  | 1839.9 |  | - |  | 1879.9 |  | -50+TT |  | 1 |  | 4 |  |
|  |  |  |  |  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41+TT |  | 0.3 |  | 3, 11 |  |
| CA_n3-n40 |  |  | Rel-15 |  |  | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 43, 44. 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  |  |  |
|  |  |  |  |  |  | E-UTRA Band 3 |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  | 4 |  |
|  |  |  |  |  |  | UTRA Band 22, 42, 52 NR Band n77, n78, n79 |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  | 2 |  |
|  |  |  |  |  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41+TT |  | 0.3 |  | 3 |  |
| CA_n3-n78 |  |  | Rel-15 |  |  | E-UTRA Band 3, 34, 39 |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  |  |  |
|  |  | Frequency range | 1884.5 | - | 1915.7 | -41+TT | 0.3 | 3 |
| CA_n3-n78 |  | Rel-16 |  |  | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65, 74 |  |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  |  |  |
|  |  |  |  |  | Frequency range |  |  | 1884.5 |  | - |  | 1915.7 |  | -41+TT |  | 0.3 |  | 3 |  |
| CA_n5-n77 |  |  | Rel-16 |  |  | E-UTRA Band 1, 2, 3, 4, 8, 11, 12, 13, 14, 17, 18, 19, 21, 25, 26, 28, 29, 30, 34, 40, 65, 66, 70, 71, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  |  | E-UTRA Band 41 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  |  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |
| CA_n8-n39 |  | Rel-16 |  |  | E-UTRA Band 1, 34, 40, 50, 51, 74 |  |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  |  |  |
|  |  |  |  |  | E-UTRA Band 22, 41, 42 NR Band n77, n78, n79 |  |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  | 2 |  |
|  |  |  |  |  | E-UTRA Band 8 |  |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  | 4 |  |
| CA_n8-n40 |  |  | Rel-16 |  |  | E-UTRA Bands 1, 5, 11, 18, 19, 20, 21, 26, 28, 31, 32, 33, 34, 38, 39, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  |  | E-UTRA Bands 3, 7, 22, 41, 42, 43, 52 NR Bands n77, n78, n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  |  |  | E-UTRA Band 8 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 4 |  |
|  |  |  |  |  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |
| CA_n8-n78 |  |  | Rel-15 |  |  | E-UTRA Band 8, 20, 28, 34, 39, 40 |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  |  |  |
|  |  |  |  |  |  | E-UTRA Band 3, 7,41 |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  | 2 |  |
|  |  |  |  |  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41+TT |  | 0.3 |  | 3 |  |
| CA_n8-n78 |  | Rel-16 |  |  | E-UTRA Band 1, 8, 11, 20, 21, 28, 34, 39, 40, 65, 74 |  |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  |  |  |
|  |  |  |  |  | E-UTRA Band 3, 7,41 |  |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  | 2 |  |
|  |  |  |  |  | Frequency range |  |  | 1884.5 |  | - |  | 1915.7 |  | -41+TT |  | 0.3 |  | 3 |  |
| CA_n28-n40 |  |  | Rel-16 |  |  | E-UTRA Band 1, 3, 5, 7, 8, 18, 19, 20, 26, 27, 28, 31, 34, 38, 41, 72 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  |  | E-UTRA Band 11, 21, 22, 32, 42, 43, 50, 51, 52, 65, 73, 74, 75, 76 NR band n77, n78, n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  |  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |
| CA_n28-n41 |  |  | Rel-16 |  |  | E-UTRA Band 2, 3, 5, 8, 25, 26, 27, 34 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  |  | E-UTRA Band 4, 42, 50, 51, 52, 65, 66, 73, 74 NR Band n77, n78, n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  |  |  | E-UTRA Band 18, 19 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 11 |  |
|  |  |  |  |  |  | E-UTRA Band 1 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 11, 15 |  |
|  |  |  |  |  |  | E-UTRA Band 11, 21 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 11, 12 |  |
|  |  |  |  |  |  | Frequency range |  | 470 |  | - |  | 694 |  | -42 |  | 8 |  | 4, 14 |  |
|  |  |  |  |  |  | Frequency range |  | 470 |  | - |  | 710 |  | -26.2 |  | 6 |  | 13 |  |
|  |  |  |  |  |  | Frequency range |  | 662 |  | - |  | 694 |  | -26.2 |  | 6 |  | 4 |  |
|  |  |  |  |  |  | Frequency range |  | 758 |  | - |  | 773 |  | -32 |  | 1 |  | 4 |  |
|  |  |  |  |  |  | Frequency range |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 11 |  |
| CA_n24-n41 |  | Rel-17 |  |  | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 25, 26, 29, 30, 48, 66, 70, 71, 85 |  |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  |  |  |
|  |  |  |  |  | NR Band n77 |  |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  | 2 |  |
| CA_n24-n48 |  | Rel-17 |  |  | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 25, 26, 29, 30, 41, 66, 70, 71, 85 |  |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  |  |  |
| CA_n24-n77 |  | Rel-17 |  |  | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 25, 26, 29, 30, 41, 66, 70, 71, 85 |  |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  |  |  |
| CA_n26-n66 |  | Rel-17 |  |  | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 29, 30, 43, 47, 50, 51, 66, 70, 71, 74, 85 |  |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  |  |  |
|  |  |  |  |  | E-UTRA Band 41, 42, 48, 53 NR band 77 |  |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  | 2 |  |
|  |  |  |  |  | Frequency range |  |  | 1884.5 |  | - |  | 1915.7 |  | -41+TT |  | 0.3 |  | 3 |  |
| CA_n26-n70 |  | Rel-17 |  |  | E-UTRA Band 2, 5, 10, 12, 13, 14, 17, 24, 25, 29, 30, 48, 66, 70, 71, 85 |  |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  |  |  |
|  |  |  |  |  | E-UTRA Band 41, 53 |  |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  | 2 |  |
|  |  |  |  |  | Frequency range |  |  | 1884.5 |  | - |  | 1915.7 |  | -41+TT |  | 0.3 |  | 3 |  |
| CA_n28-n41 |  | Rel-17 |  |  | E-UTRA Band 2, 3, 5, 8, 25, 26, 27, 34 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  | E-UTRA Band 4, 42, 50, 51, 52, 65, 66, 73, 74 NR Band n77, n78, n79 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  |  | E-UTRA Band 18, 19 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 11 |  |
|  |  |  |  |  | E-UTRA Band 1 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 11, 15 |  |
|  |  |  |  |  | E-UTRA Band 11, 21 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 11, 12 |  |
|  |  |  |  |  | Frequency range |  |  | 470 |  | - |  | 694 |  | -42 |  | 8 |  | 4, 14 |  |
|  |  |  |  |  | Frequency range |  |  | 470 |  | - |  | 710 |  | -26.2 |  | 6 |  | 13 |  |
|  |  |  |  |  | Frequency range |  |  | 662 |  | - |  | 694 |  | -26.2 |  | 6 |  | 4 |  |
|  |  |  |  |  | Frequency range |  |  | 758 |  | - |  | 773 |  | -32 |  | 1 |  | 4 |  |
|  |  |  |  |  | Frequency range |  |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  | Frequency range |  |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 11 |  |
| CA_n39-n40 |  | Rel-16 |  |  | E-UTRA Band 1, 8, 22, 26, 28, 34, 41, 42, 44, 45, 50, 51, 52, 73, 74 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  | NR Band n77, n78, n79 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  |  | Frequency range |  |  | 1805 |  |  |  | 1855 |  | -40 |  | 1 |  | 8 |  |
|  |  |  |  |  | Frequency range |  |  | 1855 |  |  |  | 1880 |  | -15.5 |  | 5 |  | 4, 7, 8 |  |
| CA_n39-n41 |  | Rel-16 |  |  | E-UTRA Band 1, 8, 26, 28, 34, 40, 42, 44, 45, 50, 51, 74 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  | NR Band n77, n78, n79 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  |  | Frequency range |  |  | 1805 |  | - |  | 1855 |  | -40 |  | 1 |  | 4 |  |
|  |  |  |  |  | Frequency range |  |  | 1855 |  | - |  | 1880 |  | -15.5 |  | 5 |  | 4, 7, 8 |  |
| CA_n39-n79 |  | Rel-16 |  |  | E-UTRA Band 1, 8, 28, 34, 40, 41, 44, 45 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  | NR Band n78 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  |  | Frequency range |  |  | 1805 |  | - |  | 1855 |  | -40 |  | 1 |  | 4, 8 |  |
|  |  |  |  |  | Frequency range |  |  | 1855 |  | - |  | 1880 |  | -15.5 |  | 5 |  | 4, 7, 8 |  |
| CA_n40-n41 |  |  | Rel-16 |  |  | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 26, 27, 28, 34, 39, 42, 44, 45, 50, 51, 65, 73, 74, NR Band n77, n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  |  | NR Band n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  |  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |
| CA_n40-n78 |  |  | Rel-15 |  |  | UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  |  |  |
|  |  |  |  |  |  | NR Band n79 |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  | 2 |  |
|  |  |  |  |  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41+TT |  | 0.3 |  | 3 |  |
| CA_n40-n79 |  |  | Rel-15 |  |  | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 26, 28, 34, 39, 41, 42, 65, 74, NR band n78 |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  |  |  |
|  |  |  |  |  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41+TT |  | 0.3 |  | 3 |  |
| CA_n41-n79 |  | Rel-16 |  |  | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 42, 44, 45, 65 |  |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  | E-UTRA Band 40 |  |  | FDL_low |  | - |  | FDL_high |  | -40 |  | 1 |  |  |  |
|  |  |  |  |  | Frequency range |  |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |
| CA_n48-n66 |  | Rel-16 |  |  | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 |  |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  |  |  |
| CA_n48-n70 |  | Rel-17 |  |  | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 66, 70, 71, 85 |  |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  | 16 |  |
|  |  |  |  |  | E-UTRA Band 41 |  |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  | 2, 16 |  |
| CA_n48-n71 |  | Rel-17 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 50, 51, 53, 66, 74, 85 |  |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  |  |  |
|  |  |  |  |  | E-UTRA Band 2, 25, 41, 70 |  |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  | 2 |  |
|  |  |  |  |  | E-UTRA Band 29 |  |  | FDL_low |  | - |  | FDL_high |  | -38+TT |  | 1 |  | 15 |  |
|  |  |  |  |  | E-UTRA Band 71 |  |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  | 15 |  |
| CA_n66-n71 |  | Rel-16 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 27, 30, 43, 50, 51, 53, 66, 74, 85 |  |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  |  |  |
|  |  |  |  |  | E-UTRA Band 2, 25, 41, 42, 48, 70 NR Band n77 |  |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  | 2 |  |
|  |  |  |  |  | E-UTRA Band 29 |  |  | FDL_low |  | - |  | FDL_high |  | -38+TT |  | 1 |  | 4 |  |
|  |  |  |  |  | E-UTRA Band 71 |  |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  | 4 |  |
| CA_n70-n71 |  | Rel-16 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 27, 30, 48, 66, 74, 85 |  |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  |  |  |
|  |  |  |  |  | E-UTRA Band 2, 7, 25, 41, 70, NR Band n77 |  |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  | 2 |  |
|  |  |  |  |  | E-UTRA Band 29 |  |  | FDL_low |  | - |  | FDL_high |  | -38+TT |  | 1 |  | 4 |  |
|  |  |  |  |  | E-UTRA Band 71 |  |  | FDL_low |  | - |  | FDL_high |  | -38+TT |  | 1 |  | 4 |  |
| CA_n78-n79 |  |  | Rel-15 |  |  | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 40, 41, 65, 74 |  | FDL_low |  | - |  | FDL_high |  | -50+TT |  | 1 |  |  |  |
|  |  |  |  |  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41+TT |  | 0.3 |  | 3 |  |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 or Table 5.5-1 in TS 36.101 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 and Table 6.5A.3.1-1 from the edge of the channel bandwidth. NOTE 5: This requirement is applicable only for the following cases: - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth. - for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart> 3. NOTE 6: This requirement is applicable for any channel bandwidths within the range 1920 – 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 – 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. NOTE 9: Void. NOTE 10: Void. NOTE 11: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 12: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned NR carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.3.1-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 13: This requirement is applicable for 5 and 10 MHz NR channel bandwidth allocated within 718 - 728 MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and Rbstart < 48. NOTE 14: This requirement is applicable in the case of a 10 MHz NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 15: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 16: The same configuration applies to corresponding NR-DC configuration in Table 4.3.1.1.7.1-1 in [5]. If UE supporting NR-DC configuration do not support the corresponding CA configuration, NR-DC configuration is used in this test. |  |  |  |  |  |  |  |  |

Table 6.5A.3.2.1.5-2: Test Tolerance for uplink inter-band carrier aggregation (two bands)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0 | 0 |
| 40MHz < BW ≤ 100MHz | 0 | 0 |

Unless otherwise stated, the spurious emission limits apply for the frequency ranges that are more than ΔfOOB (MHz) in Table 6.5.3.2.3-4 from the edge of the channel bandwidth. The spurious emission limits in Table 6.5.3.2.3-4 apply for all transmitter band configurations (NRB) and channel bandwidths for all CC combinations.

==============================================================

<< End of changes >>