3GPP TSG-RAN5 Meeting #96-e	R5-225064

# Electronic Meeting, 15th – 26th Aug. 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1875 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating test configurations for SUL test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Tx test cases for SUL, the testing is focusing on the SUL band and the NUL band is an anchor. The testing SCS for NUL carrier is more reasonable to be set as lowest supported SCS but not mandatory 15kHz. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the test SCS for NUL band in SUL test cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SUL test cases can’t be correctly performed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2C.1, 6.2C.3, 6.2C.4. 6.2C.5, 6.3C.1, 6.3C.3, 6.3C.4.1, 6.3C.4.2, 6.3C.4.3, 6.4C.1, 6.4C.2, 6.5C.1, 6.5C.2, 6.5C.3, 6.5C.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.905 CR 0670 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TP in R5-225065 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.2C.1	Configured transmitted power for SUL

6.2C.1.1	Test purpose

Same test purpose as in clause 6.2.4.1

6.2C.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.2C.1.3	Minimum conformance requirements

When a UE is configured with both NR UL and NR SUL carriers in a serving cell with active transmission either on the UL carrier or SUL carrier, the configured transmit power requirements specified in subclause 6.2.4 are applicable for the UL carrier and the SUL carrier, respectively.

For the UE which supports SUL band combination, ΔTIB,c in Table 6.2C.2-1 applies.

The normative reference for this requirement is TS 38.101-1 [2] clauses 6.2C.

6.2C.1.4	Test description

Same test description as specified in clause 6.2.4.4 with following exceptions:

Instead of table 5.3.5-1  use Table 5.5C-1

Instead of table 6.2.4.4-1  use Table 6.2C.1.4-1

Table 6.2C.1.4-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid range for both SUL carrier and Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| Test ID | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
|  | N/A | N/A |  | Modulation | RB allocation (NOTE 2) |
| 1 |  |  |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 |  |  |  | DFT-s-OFDM QPSK | Inner Full |
| NOTE 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 3: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |

<Unchanged Text Skipped>

6.2C.3	UE maximum output power for SUL

6.2C.3.1	Test purpose

Same test purpose as in clause 6.2.1.1

6.2C.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.2C.3.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

The normative reference for this requirement is TS 38.101-1 [2] clauses 4.3 and 6.2.1.

6.2C.3.4	Test description

Same test description as specified in clause 6.2.1.4 with following exceptions:

Instead of table 5.3.5-1  use Table 5.5C-1.

Instead of table 6.2.1.4-1  use Table 6.2C.3.4-1

Table 6.2C.3.4-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid range for Non-SUL carrier. Low, Mid, High range for SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| Test ID | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
|  | N/A | N/A |  | Modulation | RB allocation (NOTE 2) |
| 1 |  |  |  | DFT-s-OFDM PI/2 BPSK | Inner Full |
| 2 |  |  |  | DFT-s-OFDM PI/2 BPSK | Inner 1RB Left |
| 3 |  |  |  | DFT-s-OFDM PI/2 BPSK | Inner 1RB Right |
| 4 |  |  |  | DFT-s-OFDM QPSK | Inner Full |
| 5 |  |  |  | DFT-s-OFDM QPSK | Inner 1RB Left |
| 6 |  |  |  | DFT-s-OFDM QPSK | Inner 1RB Right |
| NOTE 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 3: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |

<Unchanged Text Skipped>

6.2C.4	UE maximum output power reduction for SUL

6.2C.4.1	Test purpose

Same test purpose as in clause 6.2.2.1

6.2C.4.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

NOTE:	Test execution is not necessary if TS 38.521-1 6.5C.2.4.1 is executed.

6.2C.4.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

The normative reference for this requirement is TS 38.101-1 [2] clauses 4.3 and 6.2.2.

6.2C.4.4	Test description

Same test description as PC 3 with contiguous allocation specified in clause 6.2.2.4 with following exceptions:

Instead of table 5.3.5-1  use Table 5.5C-1.

Instead of table 6.2.2.4-1  use Table 6.2C.4.4-1.

Table 6.2C.4.4-1: Test Configuration Table for power class 3 contiguous allocation

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Low range and high range for SUL carrier Mid range for Non-SUL carrier. |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Lowest, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.5C-1 |  |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
| Test ID | Freq | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
|  |  | N/A | N/A |  | Modulation | RB allocation (NOTE 2) |
| 1 | Default |  |  |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 | Low |  |  |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 3 | High |  |  |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 4 | Default |  |  |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  |  |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  |  |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  |  |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  |  |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  |  |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  |  |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  |  |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  |  |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Low |  |  |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 14 | High |  |  |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 15 | Default |  |  |  | DFT-s-OFDM 64 QAM | Outer Full |
| 16 | Low |  |  |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 17 | High |  |  |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 18 | Default |  |  |  | DFT-s-OFDM 256 QAM | Outer Full |
| 19 | Default |  |  |  | CP-OFDM QPSK | Inner Full |
| 20 | Low |  |  |  | CP-OFDM QPSK | Edge_1RB_Left |
| 21 | High |  |  |  | CP-OFDM QPSK | Edge_1RB_Right |
| 22 | Default |  |  |  | CP-OFDM QPSK | Outer Full |
| 23 | Default |  |  |  | CP-OFDM 16 QAM | Inner Full |
| 24 | Low |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 25 | High |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 26 | Default |  |  |  | CP-OFDM 16 QAM | Outer Full |
| 27 | Low |  |  |  | OFDM 64 QAM | Edge_1RB_Left |
| 28 | High |  |  |  | OFDM 64 QAM | Edge_1RB_Right |
| 29 | Default |  |  |  | CP-OFDM 64 QAM | Outer Full |
| 30 | Low |  |  |  | OFDM 256 QAM | Edge_1RB_Left |
| 31 | High |  |  |  | OFDM 256 QAM | Edge_1RB_Right |
| 32 | Default |  |  |  | CP-OFDM 256 QAM | Outer Full |
| 334 | Low |  |  |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Left |
| 344 | High |  |  |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Right |
| 354 | Default |  |  |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Outer Full |
| NOTE 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 3: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 4: Applicable to UEs indicating support for UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16. |  |  |  |  |  |  |

<Unchanged Text Skipped>

6.2C.5	UE additional maximum output power reduction for SUL

Editor’s note: Tests for network signalling values NS_17, NS_43 not complete.

6.2C.5.1	Test purpose

Same test purpose as in clause 6.2.3.1

6.2C.5.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.2C.5.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

The normative reference for this requirement is TS 38.101-1 [2] clauses 4.3and 6.2.3.

6.2C.5.4	Test description

Same test description as specified in clause 6.2.3.4 with following exceptions:

Instead of table 5.3.5-1  use Table 5.5C-1.

For NS_03: Instead of table 6.2.3.4.1-3  use Table 6.2C.5.4-1

For NS_18: Instead of table 6.2.3.4.1-11  use Table 6.2C.5.4-2

For NS_48: Instead of table 6.2.3.4.1-19  use Table 6.2C.5.4-3

For NS_49: Instead of table 6.2.3.4.1-29  use Table 6.2C.5.4-4

For NS_05: Instead of table 6.2.3.4.1-4  use Table 6.2C.5.4-5

For NS_05U: Instead of table 6.2.3.4.1-5  use Table 6.2C.5.4-6

For NS_100: Instead of table 6.2.3.5-12  use Table 6.2C.5.4-7

For NS_56: Instead of table 6.2.3.4.1-30  use Table 6.2C.5.4-8

Table 6.2C.5.4-1: Test Configuration Table for NS_03

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Low range, High range for SUL carrier Mid range for Non-SUL carrier |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Lowest, Highest for SUL carrier Lowest for Non-SUL carrier |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |
| A-MPR test parameters for NS_03 |  |  |  |  |  |  |  |
| Test ID | Freq | Downlink Configuration | UL Configuration | SUL Configuration |  |  |  |
|  |  |  |  | Modulation |  |  | RB allocation (Note 2) |
| 1 | Low | N/A | N/A | DFT-s OFDM | PI/2 BPSK |  | Edge_1RB_Left |
| 2 | High |  |  |  | PI/2 BPSK |  | Edge_1RB_Right |
| 3 | Default |  |  |  | PI/2 BPSK |  | Outer_Full |
| 4 | Low |  |  |  | QPSK |  | Edge_1RB_Left |
| 5 | High |  |  |  | QPSK |  | Edge_1RB_Right |
| 6 | Default |  |  |  | QPSK |  | Outer_Full |
| 7 | Low |  |  |  | 16 QAM |  | Edge_1RB_Left |
| 8 | High |  |  |  | 16 QAM |  | Edge_1RB_Right |
| 9 | Default |  |  |  | 16 QAM |  | Outer_Full |
| 10 | Low |  |  |  | 64 QAM |  | Edge_1RB_Left |
| 11 | High |  |  |  | 64 QAM |  | Edge_1RB_Right |
| 12 | Default |  |  |  | 64 QAM |  | Outer_Full |
| 13 | Low |  |  |  | 256 QAM |  | Edge_1RB_Left |
| 14 | High |  |  |  | 256 QAM |  | Edge_1RB_Right |
| 15 | Default |  |  |  | 256 QAM |  | Outer_Full |
| 16 | Low |  |  | CP-s OFDM | QPSK |  | Edge_1RB_Left |
| 17 | High |  |  |  | QPSK |  | Edge_1RB_Right |
| 18 | Default |  |  |  | QPSK |  | Outer_Full |
| 19 | Low |  |  |  | 16 QAM |  | Edge_1RB_Left |
| 20 | High |  |  |  | 16 QAM |  | Edge_1RB_Right |
| 21 | Default |  |  |  | 16 QAM |  | Outer_Full |
| 22 | Low |  |  |  | 64 QAM |  | Edge_1RB_Left |
| 23 | High |  |  |  | 64 QAM |  | Edge_1RB_Right |

| A-MPR test parameters for NS_03 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Freq | Downlink Configuration | UL Configuration | SUL Configuration |  |  |
|  |  |  |  | Modulation |  | RB allocation (Note 2) |
| 24 | Default |  |  |  | 64 QAM | Outer_Full |
| 25 | Low |  |  |  | 256 QAM | Edge_1RB_Left |
| 26 | High |  |  |  | 256 QAM | Edge_1RB_Right |
| 27 | Default |  |  |  | 256 QAM | Outer_Full |
| NOTE 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 3: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |

Table 6.2C.5.4-2: Test Configuration Table for NS_18

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Low range for SUL carrier Mid range for Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | 5MHz 10MHz 20MHz 30MHz for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| A-MPR test parameters for NS_18 |  |  |  |  |  |  |
| Test ID | ChBw SCS | Downlink Configuration | SUL Configuration |  |  |  |
|  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) | Comment |
| 1 | 5MHz, 10MHz, 20MHz | N/A | DFT-s OFDM | QPSK | Edge_1RB_Left | A1, A2 |
| 2 |  |  |  | QPSK | Outer_Full | A1, A2 |
| 3 |  |  |  | 16 QAM | Edge_1RB_Left | A1, A2 |
| 4 |  |  |  | 16 QAM | Outer_Full | A1, A2 |
| 5 |  |  |  | 64 QAM | Edge_1RB_Left | A1, A2 |
| 6 |  |  |  | 64 QAM | Outer_Full | A1, A2 |
| 7 |  |  |  | 256 QAM | Edge_1RB_Left | A1, A2 |
| 8 |  |  |  | 256 QAM | Outer_Full | A1, A2 |
| 9 |  |  | CP-s OFDM | QPSK | Edge_1RB_Left | A1, A2 |
| 10 |  |  |  | QPSK | Outer_Full | A1, A2 |
| 11 |  |  |  | 16 QAM | Edge_1RB_Left | A1, A2 |
| 12 |  |  |  | 16 QAM | Outer_Full | A1, A2 |
| 13 |  |  |  | 64 QAM | Edge_1RB_Left | A1, A2 |
| 14 |  |  |  | 64 QAM | Outer_Full | A1, A2 |
| 15 |  |  |  | 256 QAM | Edge_1RB_Left | A1, A2 |
| 16 |  |  |  | 256 QAM | Outer_Full | A1, A2 |
| 17 | 30MHz |  | DFT-s OFDM | 256 QAM | 81@70 | A3 |
| 18 |  |  |  | 256 QAM | Outer_Full | A4 |
| 19 |  |  | CP-s OFDM | 256 QAM | 87@73 | A3 |
| 20 |  |  |  | 256 QAM | Outer_Full | A4 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |

Table 6.2C.5.4-3: Test Configuration Table for NS_48

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Low range, High range for SUL carrier Mid range for non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | 25 MHz, 30MHz, 40MHz, 50MHz for SUL carrier Lowest for non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| A-MPR test parameters for NS_48 |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | Uplink Configuration | Downlink Configuration | SUL Configuration |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  |  | RB allocation (Note 1) |
| 1 | Default | 25 | N/A for A-MPR testing. | N/A | DFT-s-OFDM |  | QPSK | Outer_Full (A3) |
| 2 | Default | 25 |  |  |  |  | QPSK | Edge_1RB_Right (A3) |
| 3 | Default | 30 |  |  |  |  | QPSK | Outer_Full (A3) |
| 4 | Default | 30 |  |  |  |  | QPSK | Edge_1RB_Right (A5) |
| 5 | Default | 40 |  |  |  |  | QPSK | 16@0 (A2) |
| 6 | Default | 40 |  |  |  |  | QPSK | 90@0 (A3) |
| 7 | Default | 40 |  |  |  |  | QPSK | 150@0 (A4) |
| 8 | Default | 40 |  |  |  |  | QPSK | 192@0 (A2) |
| 9 | Default | 40 |  |  |  |  | QPSK | 5@187 (A3) |
| 10 | Default | 40 |  |  |  |  | QPSK | Outer_Full (A1) |
| 11 | Default | 50 |  |  |  |  | QPSK | 32@0 (A2) |
| 12 | Default | 50 |  |  |  |  | QPSK | 108@0 (A4) |
| 13 | Default | 50 |  |  |  |  | QPSK | 225@0 (A2) |
| 14 | Default | 50 |  |  |  |  | QPSK | 5@223 (A5) |
| 15 | Default | 50 |  |  |  |  | QPSK | Outer_Full (A1) |
| 16 | Default | 25 |  |  |  |  | 256 QAM | Outer_Full (A3) |
| 17 | Default | 25 |  |  |  |  | 256 QAM | Edge_1RB_Right (A3) |
| 18 | Default | 30 |  |  |  |  | 256 QAM | Outer_Full (A3) |
| 19 | Default | 30 |  |  |  |  | 256 QAM | Edge_1RB_Right (A5) |
| 20 | Default | 40 |  |  |  |  | 256 QAM | 16@0 (A2) |
| 21 | Default | 40 |  |  |  |  | 256 QAM | 90@0 (A3) |
| 22 | Default | 40 |  |  |  |  | 256 QAM | 150@0 (A4) |
| 23 | Default | 40 |  |  |  |  | 256 QAM | 192@0 (A2) |
| 24 | Default | 40 |  |  |  |  | 256 QAM | 5@187 (A3) |
| 25 | Default | 40 |  |  |  |  | 256 QAM | Outer_Full (A1) |
| 26 | Default | 50 |  |  |  |  | 256 QAM | 32@0 (A2) |
| 27 | Default | 50 |  |  |  |  | 256 QAM | 108@0 (A4) |
| 28 | Default | 50 |  |  |  |  | 256 QAM | 225@0 (A2) |
| 29 | Default | 50 |  |  |  |  | 256 QAM | 5@223 (A5) |
| 30 | Default | 50 |  |  |  |  | 256 QAM | Outer_Full (A1) |
| 31 | Default | 25 |  |  | CP-OFDM |  | QPSK | Outer_Full (A3) |
| 32 | Default | 25 |  |  |  |  | QPSK | Edge_1RB_Right (A3) |
| 33 | Default | 30 |  |  |  |  | QPSK | Outer_Full (A3) |
| 34 | Default | 30 |  |  |  |  | QPSK | Edge_1RB_Right (A5) |
| 35 | Default | 40 |  |  |  |  | QPSK | 16@0 (A2) |
| 36 | Default | 40 |  |  |  |  | QPSK | 95@0 (A3) |
| 37 | Default | 40 |  |  |  |  | QPSK | 152@0 (A4) |
| 38 | Default | 40 |  |  |  |  | QPSK | 192@0 (A2) |
| 39 | Default | 40 |  |  |  |  | QPSK | 5@187 (A3) |
| 40 | Default | 40 |  |  |  |  | QPSK | Outer_Full (A1) |
| 41 | Default | 50 |  |  |  |  | QPSK | 34@0 (A2) |
| 42 | Default | 50 |  |  |  |  | QPSK | 115@0 (A4) |
| 43 | Default | 50 |  |  |  |  | QPSK | 228@0 (A2) |
| 44 | Default | 50 |  |  |  |  | QPSK | 5@223 (A5) |
| 45 | Default | 50 |  |  |  |  | QPSK | Outer_Full (A1) |
| 46 | Default | 25 |  |  |  |  | 256 QAM | Outer_Full (A3) |
| 47 | Default | 25 |  |  |  |  | 256 QAM | Edge_1RB_Right (A3) |
| 48 | Default | 30 |  |  |  |  | 256 QAM | Outer_Full (A3) |
| 49 | Default | 30 |  |  |  |  | 256 QAM | Edge_1RB_Right (A5) |

| A-MPR test parameters for NS_48 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | Ch BW (MHz) | Uplink Configuration | Downlink Configuration | SUL Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 50 | Default | 40 |  |  |  | 256 QAM | 16@0 (A2) |
| 51 | Default | 40 |  |  |  | 256 QAM | 95@0 (A3) |
| 52 | Default | 40 |  |  |  | 256 QAM | 152@0 (A4) |
| 53 | Default | 40 |  |  |  | 256 QAM | 192@0 (A2) |
| 54 | Default | 40 |  |  |  | 256 QAM | 5@187 (A3) |
| 55 | Default | 40 |  |  |  | 256 QAM | Outer_Full (A1) |
| 56 | Default | 50 |  |  |  | 256 QAM | 34@0 (A2) |
| 57 | Default | 50 |  |  |  | 256 QAM | 115@0 (A4) |
| 58 | Default | 50 |  |  |  | 256 QAM | 228@0 (A2) |
| 59 | Default | 50 |  |  |  | 256 QAM | 5@223 (A5) |
| 60 | Default | 50 |  |  |  | 256 QAM | Outer_Full (A1) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |

Table 6.2C.5.4-4: Test Configuration Table for NS_49

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Low range, High range for SUL carrier Mid range for non-SUL carrier |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | 25 MHz, 30MHz, 40MHz, 50MHz for SUL carrier Lowest for non-SUL carrier |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |  |
| A-MPR test parameters for NS_49 |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | Downlink Configuration | Uplink Configuration | SUL Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 1 | Default | 25 | N/A for A-MPR testing. | N/A | DFT-s-OFDM | QPSK | Outer_Full (A3) |
| 2 | Default | 25 |  |  |  | QPSK | Edge_1RB_Right (A3) |
| 3 | Default | 25 |  |  |  | QPSK | Edge_1RB_Left (A3) |
| 4 | Default | 30 |  |  |  | QPSK | 20@0 (A1) |
| 5 | Default | 30 |  |  |  | QPSK | 36@0 (A5) |
| 6 | Default | 30 |  |  |  | QPSK | 80@0 (A3) |
| 7 | Default | 30 |  |  |  | QPSK | 120@0 (A4) |
| 8 | Default | 30 |  |  |  | QPSK | Outer_Full (A2) |
| 9 | Default | 30 |  |  |  | QPSK | Edge_1RB_Right (A5) |
| 10 | Default | 40 |  |  |  | QPSK | 40@0 (A1) |
| 11 | Default | 40 |  |  |  | QPSK | 5@53 (A5) |
| 12 | Default | 40 |  |  |  | QPSK | 100@0 (A4) |
| 13 | Default | 40 |  |  |  | QPSK | 150@33 (A2) |
| 14 | Default | 40 |  |  |  | QPSK | 5@187 (A5) |
| 15 | Default | 40 |  |  |  | QPSK | 192@0 (A1) |
| 16 | Default | 40 |  |  |  | QPSK | Outer_Full (A1) |
| 17 | Default | 50 |  |  |  | QPSK | 5@75 (A5) |
| 18 | Default | 50 |  |  |  | QPSK | 5@215 (A5) |
| 19 | Default | 50 |  |  |  | QPSK | 175@45 (A2) |
| 20 | Default | 50 |  |  |  | QPSK | 216@0 (A1) |
| 21 | Default | 50 |  |  |  | QPSK | Outer_Full (A1) |

| A-MPR test parameters for NS_49 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | Ch BW (MHz) | Downlink Configuration | Uplink Configuration | SUL Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 22 | Default | 25 |  |  |  | 256 QAM | Outer_Full (A3) |
| 23 | Default | 25 |  |  |  | 256 QAM | Edge_1RB_Right (A3) |
| 24 | Default | 25 |  |  |  | 256 QAM | Edge_1RB_Left (A3) |
| 25 | Default | 30 |  |  |  | 256 QAM | 20@0 (A1) |
| 26 | Default | 30 |  |  |  | 256 QAM | 36@0 (A5) |
| 27 | Default | 30 |  |  |  | 256 QAM | 80@0 (A3) |
| 28 | Default | 30 |  |  |  | 256 QAM | 120@0 (A4) |
| 29 | Default | 30 |  |  |  | 256 QAM | Outer_Full (A2) |
| 30 | Default | 30 |  |  |  | 256 QAM | Edge_1RB_Right (A5) |
| 31 | Default | 40 |  |  |  | 256 QAM | 40@0 (A1) |
| 32 | Default | 40 |  |  |  | 256 QAM | 5@53 (A5) |
| 33 | Default | 40 |  |  |  | 256 QAM | 100@0 (A4) |
| 34 | Default | 40 |  |  |  | 256 QAM | 150@33 (A2) |
| 35 | Default | 40 |  |  |  | 256 QAM | 5@187 (A5) |
| 36 | Default | 40 |  |  |  | 256 QAM | 192@0 (A1) |
| 37 | Default | 40 |  |  |  | 256 QAM | Outer_Full (A1) |
| 38 | Default | 50 |  |  |  | 256 QAM | 5@75 (A5) |
| 39 | Default | 50 |  |  |  | 256 QAM | 5@215 (A5) |
| 40 | Default | 50 |  |  |  | 256 QAM | 175@45 (A2) |
| 41 | Default | 50 |  |  |  | 256 QAM | 216@0 (A1) |
| 42 | Default | 50 |  |  |  | 256 QAM | Outer_Full (A1) |
| 43 | Default | 25 |  |  | CP-OFDM | QPSK | Outer_Full (A3) |
| 44 | Default | 25 |  |  |  | QPSK | Edge_1RB_Right (A3) |
| 45 | Default | 25 |  |  |  | QPSK | Edge_1RB_Left (A3) |
| 46 | Default | 30 |  |  |  | QPSK | 20@0 (A1) |
| 47 | Default | 30 |  |  |  | QPSK | 36@0 (A5) |
| 48 | Default | 30 |  |  |  | QPSK | 80@0 (A3) |
| 49 | Default | 30 |  |  |  | QPSK | 120@0 (A4) |

| A-MPR test parameters for NS_49 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | Ch BW (MHz) | Downlink Configuration | Uplink Configuration | SUL Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 50 | Default | 30 |  |  |  | QPSK | Outer_Full (A2) |
| 51 | Default | 30 |  |  |  | QPSK | Edge_1RB_Right (A5) |
| 52 | Default | 40 |  |  |  | QPSK | 40@0 (A1) |
| 53 | Default | 40 |  |  |  | QPSK | 5@53 (A5) |
| 54 | Default | 40 |  |  |  | QPSK | 100@0 (A4) |
| 55 | Default | 40 |  |  |  | QPSK | 159@33 (A2) |
| 56 | Default | 40 |  |  |  | QPSK | 5@187 (A5) |
| 57 | Default | 40 |  |  |  | QPSK | 192@0 (A1) |
| 58 | Default | 40 |  |  |  | QPSK | Outer_Full (A1) |
| 59 | Default | 50 |  |  |  | QPSK | 5@75 (A5) |
| 60 | Default | 50 |  |  |  | QPSK | 5@215 (A5) |
| 61 | Default | 50 |  |  |  | QPSK | 175@45 (A2) |
| 62 | Default | 50 |  |  |  | QPSK | 220@0 (A1) |
| 63 | Default | 50 |  |  |  | QPSK | Outer_Full (A1) |
| 64 | Default | 25 |  |  |  | 256 QAM | Outer_Full (A3) |
| 65 | Default | 25 |  |  |  | 256 QAM | Edge_1RB_Right (A3) |
| 66 | Default | 25 |  |  |  | 256 QAM | Edge_1RB_Left (A3) |
| 67 | Default | 30 |  |  |  | 256 QAM | 20@0 (A1) |
| 68 | Default | 30 |  |  |  | 256 QAM | 36@0 (A5) |
| 69 | Default | 30 |  |  |  | 256 QAM | 80@0 (A3) |
| 70 | Default | 30 |  |  |  | 256 QAM | 120@0 (A4) |
| 71 | Default | 30 |  |  |  | 256 QAM | Outer_Full (A2) |
| 72 | Default | 30 |  |  |  | 256 QAM | Edge_1RB_Right (A5) |
| 73 | Default | 40 |  |  |  | 256 QAM | 40@0 (A1) |
| 74 | Default | 40 |  |  |  | 256 QAM | 5@53 (A5) |
| 75 | Default | 40 |  |  |  | 256 QAM | 100@0 (A4) |
| 76 | Default | 40 |  |  |  | 256 QAM | 159@33 (A2) |
| 77 | Default | 40 |  |  |  | 256 QAM | 5@187 (A5) |

| A-MPR test parameters for NS_49 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | Ch BW (MHz) | Downlink Configuration | Uplink Configuration | SUL Configuration |  |  |
| 78 | Default | 40 |  |  |  | 256 QAM | 192@0 (A1) |
| 79 | Default | 40 |  |  |  | 256 QAM | Outer_Full (A1) |
| 80 | Default | 50 |  |  |  | 256 QAM | 5@75 (A5) |
| 81 | Default | 50 |  |  |  | 256 QAM | 5@215 (A5) |
| 82 | Default | 50 |  |  |  | 256 QAM | 175@45 (A2) |
| 83 | Default | 50 |  |  |  | 256 QAM | 220@0 (A1) |
| 84 | Default | 50 |  |  |  | 256 QAM | Outer_Full (A1) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |

Table 6.2C.5.4-5: Test Configuration Table for NS_05

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | SUL carrier: use Fc as specified in test parameters NUL carrier: Mid range |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | SUL carrier: 5 MHz, 10 MHz, 15 MHz, 20 MHz NUL carrier: Highest |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 15kHz for SUL carrier and Lowest supported SCS for NUL carrier |  |  |  |
| A-MPR test parameters for NS_05 |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | Downlink Config. | Uplink Config | A-MPR | SUL Configuration |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |
| 1 | 1922.5 | 5 | N/A for A-MPR testing | N/A for A-MPR testing | A3 | DFT-s-OFDM | PI/2 BPSK | Outer_Full |
| 2 | 1925 | 10 |  |  | A1 |  | PI/2 BPSK | Outer_Full |
| 3 | 1925 | 10 |  |  | A7 |  | PI/2 BPSK | 40@10 |
| 4 | 1925 | 10 |  |  | A2 |  | PI/2 BPSK | 6@40 |
| 5 | 1935 | 10 |  |  | A4 |  | PI/2 BPSK | Outer_Full |
| 6 | 1927.5 | 15 |  |  | A1 |  | PI/2 BPSK | Outer_Full |
| 7 | 1927.5 | 15 |  |  | A7 |  | PI/2 BPSK | 60@19 |
| 8 | 1927.5 | 15 |  |  | A2 |  | PI/2 BPSK | 6@56 |
| 9 | 1932.5 | 15 |  |  | A1 |  | PI/2 BPSK | Outer_Full |
| 10 | 1932.5 | 15 |  |  | A2 |  | PI/2 BPSK | 6@68 |
| 11 | 1942.5 | 15 |  |  | A5 |  | PI/2 BPSK | Outer_Full |
| 12 | 1930 | 20 |  |  | A1 |  | PI/2 BPSK | Outer_Full |
| 13 | 1930 | 20 |  |  | A7 |  | PI/2 BPSK | 72@28 |
| 14 | 1930 | 20 |  |  | A2 |  | PI/2 BPSK | 6@76 |
| 15 | 1950 | 20 |  |  | A6 |  | PI/2 BPSK | Outer_Full |
| 16 | 1922.5 | 5 |  |  | A3 |  | QPSK | Outer_Full |
| 17 | 1925 | 10 |  |  | A1 |  | QPSK | Outer_Full |
| 18 | 1925 | 10 |  |  | A7 |  | QPSK | 40@10 |
| 19 | 1925 | 10 |  |  | A2 |  | QPSK | 6@40 |
| 20 | 1927.5 | 15 |  |  | A1 |  | QPSK | Outer_Full |
| 21 | 1927.5 | 15 |  |  | A7 |  | QPSK | 60@19 |
| 22 | 1927.5 | 15 |  |  | A2 |  | QPSK | 6@56 |
| 23 | 1932.5 | 15 |  |  | A1 |  | QPSK | Outer_Full |
| 24 | 1932.5 | 15 |  |  | A2 |  | QPSK | 6@68 |
| 25 | 1942.5 | 15 |  |  | A5 |  | QPSK | Outer_Full |
| 26 | 1930 | 20 |  |  | A1 |  | QPSK | Outer_Full |
| 27 | 1930 | 20 |  |  | A7 |  | QPSK | 72@28 |
| 28 | 1930 | 20 |  |  | A2 |  | QPSK | 6@76 |
| 29 | 1950 | 20 |  |  | A6 |  | QPSK | Outer_Full |
| 30 | 1922.5 | 5 |  |  | A3 |  | 16 QAM | Outer_Full |
| 31 | 1925 | 10 |  |  | A1 |  | 16 QAM | Outer_Full |
| 32 | 1925 | 10 |  |  | A7 |  | 16 QAM | 40@10 |
| 33 | 1925 | 10 |  |  | A2 |  | 16 QAM | 6@40 |
| 34 | 1927.5 | 15 |  |  | A1 |  | 16 QAM | Outer_Full |
| 35 | 1927.5 | 15 |  |  | A7 |  | 16 QAM | 60@19 |
| 36 | 1927.5 | 15 |  |  | A2 |  | 16 QAM | 6@56 |
| 37 | 1932.5 | 15 |  |  | A1 |  | 16 QAM | Outer_Full |
| 38 | 1932.5 | 15 |  |  | A2 |  | 16 QAM | 6@68 |
| 39 | 1930 | 20 |  |  | A1 |  | 16 QAM | Outer_Full |
| 40 | 1930 | 20 |  |  | A7 |  | 16 QAM | 72@28 |
| 41 | 1930 | 20 |  |  | A2 |  | 16 QAM | 6@76 |
| 42 | 1922.5 | 5 |  |  | A3 |  | 64 QAM | Outer_Full |
| 43 | 1925 | 10 |  |  | A1 |  | 64 QAM | Outer_Full |
| 44 | 1925 | 10 |  |  | A7 |  | 64 QAM | 40@10 |

| A-MPR test parameters for NS_05 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | ChBw (MHz) | Downlink Config. | Uplink Config | A-MPR | SUL Configuration |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |
| 45 | 1925 | 10 |  |  | A2 |  | 64 QAM | 6@40 |
| 46 | 1927.5 | 15 |  |  | A1 |  | 64 QAM | Outer_Full |
| 47 | 1927.5 | 15 |  |  | A7 |  | 64 QAM | 60@19 |
| 48 | 1927.5 | 15 |  |  | A2 |  | 64 QAM | 6@56 |
| 49 | 1932.5 | 15 |  |  | A1 |  | 64 QAM | Outer_Full |
| 50 | 1932.5 | 15 |  |  | A2 |  | 64 QAM | 6@68 |
| 51 | 1930 | 20 |  |  | A1 |  | 64 QAM | Outer_Full |
| 52 | 1930 | 20 |  |  | A7 |  | 64 QAM | 72@28 |
| 53 | 1930 | 20 |  |  | A2 |  | 64 QAM | 6@76 |
| 54 | 1922.5 | 5 |  |  | A3 |  | 256 QAM | Outer_Full |
| 55 | 1925 | 10 |  |  | A1 |  | 256 QAM | Outer_Full |
| 56 | 1925 | 10 |  |  | A7 |  | 256 QAM | 40@10 |
| 57 | 1925 | 10 |  |  | A2 |  | 256 QAM | 6@40 |
| 58 | 1927.5 | 15 |  |  | A1 |  | 256 QAM | Outer_Full |
| 59 | 1927.5 | 15 |  |  | A7 |  | 256 QAM | 60@19 |
| 60 | 1927.5 | 15 |  |  | A2 |  | 256 QAM | 6@56 |
| 61 | 1932.5 | 15 |  |  | A1 |  | 256 QAM | Outer_Full |
| 62 | 1932.5 | 15 |  |  | A2 |  | 256 QAM | 6@68 |
| 63 | 1930 | 20 |  |  | A1 |  | 256 QAM | Outer_Full |
| 64 | 1930 | 20 |  |  | A7 |  | 256 QAM | 72@28 |
| 65 | 1930 | 20 |  |  | A2 |  | 256 QAM | 6@76 |
| 66 | 1922.5 | 5 |  |  | A3 | CP-OFDM | QPSK | Outer_Full |
| 67 | 1925 | 10 |  |  | A1 |  | QPSK | Outer_Full |
| 68 | 1925 | 10 |  |  | A7 |  | QPSK | 42@10 |
| 69 | 1925 | 10 |  |  | A2 |  | QPSK | 6@40 |
| 70 | 1935 | 10 |  |  | A4 |  | QPSK | Outer_Full |
| 71 | 1927.5 | 15 |  |  | A1 |  | QPSK | Outer_Full |
| 72 | 1927.5 | 15 |  |  | A7 |  | QPSK | 60@19 |
| 73 | 1927.5 | 15 |  |  | A2 |  | QPSK | 6@56 |
| 74 | 1932.5 | 15 |  |  | A1 |  | QPSK | Outer_Full |
| 75 | 1932.5 | 15 |  |  | A2 |  | QPSK | 6@68 |
| 76 | 1942.5 | 15 |  |  | A5 |  | QPSK | Outer_Full |
| 77 | 1930 | 20 |  |  | A1 |  | QPSK | Outer_Full |
| 78 | 1930 | 20 |  |  | A7 |  | QPSK | 78@28 |
| 79 | 1930 | 20 |  |  | A2 |  | QPSK | 6@76 |
| 80 | 1950 | 20 |  |  | A6 |  | QPSK | Outer_Full |
| 81 | 1922.5 | 5 |  |  | A3 |  | 16 QAM | Outer_Full |
| 82 | 1925 | 10 |  |  | A1 |  | 16 QAM | Outer_Full |
| 83 | 1925 | 10 |  |  | A7 |  | 16 QAM | 42@10 |
| 84 | 1925 | 10 |  |  | A2 |  | 16 QAM | 6@40 |
| 85 | 1935 | 10 |  |  | A4 |  | 16 QAM | Outer_Full |
| 86 | 1927.5 | 15 |  |  | A1 |  | 16 QAM | Outer_Full |
| 87 | 1927.5 | 15 |  |  | A7 |  | 16 QAM | 60@19 |
| 88 | 1927.5 | 15 |  |  | A2 |  | 16 QAM | 6@56 |
| 89 | 1932.5 | 15 |  |  | A1 |  | 16 QAM | Outer_Full |
| 90 | 1932.5 | 15 |  |  | A2 |  | 16 QAM | 6@68 |
| 91 | 1942.5 | 15 |  |  | A5 |  | 16 QAM | Outer_Full |
| 92 | 1930 | 20 |  |  | A1 |  | 16 QAM | Outer_Full |
| 93 | 1930 | 20 |  |  | A7 |  | 16 QAM | 78@28 |
| 94 | 1930 | 20 |  |  | A2 |  | 16 QAM | 6@76 |
| 95 | 1950 | 20 |  |  | A6 |  | 16 QAM | Outer_Full |
| 96 | 1922.5 | 5 |  |  | A3 |  | 64 QAM | Outer_Full |
| 97 | 1925 | 10 |  |  | A1 |  | 64 QAM | Outer_Full |
| 98 | 1925 | 10 |  |  | A7 |  | 64 QAM | 42@10 |
| 99 | 1925 | 10 |  |  | A2 |  | 64 QAM | 6@40 |

| A-MPR test parameters for NS_05 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | ChBw (MHz) | Downlink Config. | Uplink Config | A-MPR | SUL Configuration |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |
| 100 | 1927.5 | 15 |  |  | A1 |  | 64 QAM | Outer_Full |
| 101 | 1927.5 | 15 |  |  | A7 |  | 64 QAM | 60@19 |
| 102 | 1927.5 | 15 |  |  | A2 |  | 64 QAM | 6@56 |
| 103 | 1932.5 | 15 |  |  | A1 |  | 64 QAM | Outer_Full |
| 104 | 1932.5 | 15 |  |  | A2 |  | 64 QAM | 6@68 |
| 105 | 1930 | 20 |  |  | A1 |  | 64 QAM | Outer_Full |
| 106 | 1930 | 20 |  |  | A7 |  | 64 QAM | 78@28 |
| 107 | 1930 | 20 |  |  | A2 |  | 64 QAM | 6@76 |
| 108 | 1922.5 | 5 |  |  | A3 |  | 256 QAM | Outer_Full |
| 109 | 1925 | 10 |  |  | A1 |  | 256 QAM | Outer_Full |
| 110 | 1925 | 10 |  |  | A7 |  | 256 QAM | 42@10 |
| 111 | 1927.5 | 15 |  |  | A1 |  | 256 QAM | Outer_Full |
| 112 | 1927.5 | 15 |  |  | A7 |  | 256 QAM | 60@19 |
| 113 | 1932.5 | 15 |  |  | A1 |  | 256 QAM | Outer_Full |
| 114 | 1930 | 20 |  |  | A1 |  | 256 QAM | Outer_Full |
| 115 | 1930 | 20 |  |  | A7 |  | 256 QAM | 78@28 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |

Table 6.2C.5.4-6: Test Configuration Table for NS_05U

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | SUL carrier: Use Fc as specified in test parameters NUL carrier: Mid range |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | SUL carrier: 5 MHz, 10 MHz, 15 MHz, 20 MHz NUL carrier: Highest |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 15kHz for SUL and Lowest supported SCS for NUL carrier |  |  |  |  |
| AMPR Test parameters for NS_05U |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | Downlink Config. | Uplink config |  | A-MPR | SUL Configuration |  |  |
|  |  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |
| 1 | 1922.5 | 5 | N/A | N/A |  | A3 | DFT-s-OFDM | PI/2 BPSK | Outer_Full |
| 2 | 1925 | 10 |  |  |  | A1 |  | PI/2 BPSK | Outer_Full |
| 3 | 1925 | 10 |  |  |  | A7 |  | PI/2 BPSK | 40@10 |
| 4 | 1925 | 10 |  |  |  | A2 |  | PI/2 BPSK | 6@40 |
| 5 | 1935 | 10 |  |  |  | A4 |  | PI/2 BPSK | Outer_Full |
| 6 | 1927.5 | 15 |  |  |  | A1 |  | PI/2 BPSK | Outer_Full |
| 7 | 1927.5 | 15 |  |  |  | A7 |  | PI/2 BPSK | 60@19 |
| 8 | 1927.5 | 15 |  |  |  | A2 |  | PI/2 BPSK | 6@56 |
| 9 | 1932.5 | 15 |  |  |  | A1 |  | PI/2 BPSK | Outer_Full |
| 10 | 1932.5 | 15 |  |  |  | A2 |  | PI/2 BPSK | 6@68 |
| 11 | 1942.5 | 15 |  |  |  | A5 |  | PI/2 BPSK | Outer_Full |
| 12 | 1930 | 20 |  |  |  | A1 |  | PI/2 BPSK | Outer_Full |
| 13 | 1930 | 20 |  |  |  | A7 |  | PI/2 BPSK | 72@28 |
| 14 | 1930 | 20 |  |  |  | A2 |  | PI/2 BPSK | 6@76 |
| 15 | 1950 | 20 |  |  |  | A6 |  | PI/2 BPSK | Outer_Full |
| 16 | 1922.5 | 5 |  |  |  | A3 |  | QPSK | Outer_Full |
| 17 | 1925 | 10 |  |  |  | A1 |  | QPSK | Outer_Full |
| 18 | 1925 | 10 |  |  |  | A7 |  | QPSK | 40@10 |
| 19 | 1925 | 10 |  |  |  | A2 |  | QPSK | 6@40 |
| 20 | 1935 | 10 |  |  |  | A4 |  | QPSK | Outer_Full |
| 21 | 1927.5 | 15 |  |  |  | A1 |  | QPSK | Outer_Full |
| 22 | 1927.5 | 15 |  |  |  | A7 |  | QPSK | 60@19 |
| 23 | 1927.5 | 15 |  |  |  | A2 |  | QPSK | 6@56 |
| 24 | 1932.5 | 15 |  |  |  | A1 |  | QPSK | Outer_Full |
| 25 | 1932.5 | 15 |  |  |  | A2 |  | QPSK | 6@68 |
| 26 | 1942.5 | 15 |  |  |  | A5 |  | QPSK | Outer_Full |
| 27 | 1930 | 20 |  |  |  | A1 |  | QPSK | Outer_Full |
| 28 | 1930 | 20 |  |  |  | A7 |  | QPSK | 72@28 |
| 29 | 1930 | 20 |  |  |  | A2 |  | QPSK | 6@76 |
| 30 | 1950 | 20 |  |  |  | A6 |  | QPSK | Outer_Full |
| 31 | 1922.5 | 5 |  |  |  | A3 |  | 16 QAM | Outer_Full |
| 32 | 1925 | 10 |  |  |  | A1 |  | 16 QAM | Outer_Full |
| 33 | 1925 | 10 |  |  |  | A7 |  | 16 QAM | 40@10 |
| 34 | 1925 | 10 |  |  |  | A2 |  | 16 QAM | 6@40 |
| 35 | 1935 | 10 |  |  |  | A4 |  | 16 QAM | Outer_Full |
| 36 | 1927.5 | 15 |  |  |  | A1 |  | 16 QAM | Outer_Full |
| 37 | 1927.5 | 15 |  |  |  | A7 |  | 16 QAM | 60@19 |
| 38 | 1927.5 | 15 |  |  |  | A2 |  | 16 QAM | 6@56 |
| 39 | 1932.5 | 15 |  |  |  | A1 |  | 16 QAM | Outer_Full |
| 40 | 1932.5 | 15 |  |  |  | A2 |  | 16 QAM | 6@68 |
| 41 | 1942.5 | 15 |  |  |  | A5 |  | 16 QAM | Outer_Full |
| 42 | 1930 | 20 |  |  |  | A1 |  | 16 QAM | Outer_Full |
| 43 | 1930 | 20 |  |  |  | A7 |  | 16 QAM | 72@28 |
| 44 | 1930 | 20 |  |  |  | A2 |  | 16 QAM | 6@76 |

| AMPR Test parameters for NS_05U |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | ChBw (MHz) | Downlink Config. | Uplink config | A-MPR | SUL Configuration |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |
| 45 | 1950 | 20 |  |  | A6 |  | 16 QAM | Outer_Full |
| 46 | 1922.5 | 5 |  |  | A3 |  | 64 QAM | Outer_Full |
| 47 | 1925 | 10 |  |  | A1 |  | 64 QAM | Outer_Full |
| 48 | 1925 | 10 |  |  | A7 |  | 64 QAM | 40@10 |
| 49 | 1925 | 10 |  |  | A2 |  | 64 QAM | 6@40 |
| 50 | 1935 | 10 |  |  | A4 |  | 64 QAM | Outer_Full |
| 51 | 1927.5 | 15 |  |  | A1 |  | 64 QAM | Outer_Full |
| 52 | 1927.5 | 15 |  |  | A7 |  | 64 QAM | 60@19 |
| 53 | 1927.5 | 15 |  |  | A2 |  | 64 QAM | 6@56 |
| 54 | 1932.5 | 15 |  |  | A1 |  | 64 QAM | Outer_Full |
| 55 | 1932.5 | 15 |  |  | A2 |  | 64 QAM | 6@68 |
| 56 | 1942.5 | 15 |  |  | A5 |  | 64 QAM | Outer_Full |
| 57 | 1930 | 20 |  |  | A1 |  | 64 QAM | Outer_Full |
| 58 | 1930 | 20 |  |  | A7 |  | 64 QAM | 72@28 |
| 59 | 1930 | 20 |  |  | A2 |  | 64 QAM | 6@76 |
| 60 | 1950 | 20 |  |  | A6 |  | 64 QAM | Outer_Full |
| 61 | 1922.5 | 5 |  |  | A3 |  | 256 QAM | Outer_Full |
| 62 | 1925 | 10 |  |  | A1 |  | 256 QAM | Outer_Full |
| 63 | 1925 | 10 |  |  | A7 |  | 256 QAM | 40@10 |
| 64 | 1925 | 10 |  |  | A2 |  | 256 QAM | 6@40 |
| 65 | 1935 | 10 |  |  | A4 |  | 256 QAM | Outer_Full |
| 66 | 1927.5 | 15 |  |  | A1 |  | 256 QAM | Outer_Full |
| 67 | 1927.5 | 15 |  |  | A7 |  | 256 QAM | 60@19 |
| 68 | 1927.5 | 15 |  |  | A2 |  | 256 QAM | 6@56 |
| 69 | 1932.5 | 15 |  |  | A1 |  | 256 QAM | Outer_Full |
| 70 | 1932.5 | 15 |  |  | A2 |  | 256 QAM | 6@68 |
| 71 | 1942.5 | 15 |  |  | A5 |  | 256 QAM | Outer_Full |
| 72 | 1930 | 20 |  |  | A1 |  | 256 QAM | Outer_Full |
| 73 | 1930 | 20 |  |  | A7 |  | 256 QAM | 72@28 |
| 74 | 1930 | 20 |  |  | A2 |  | 256 QAM | 6@76 |
| 75 | 1950 | 20 |  |  | A6 |  | 256 QAM | Outer_Full |
| 76 | 1922.5 | 5 |  |  | A3 | CP-OFDM | QPSK | Outer_Full |
| 77 | 1925 | 10 |  |  | A1 |  | QPSK | Outer_Full |
| 78 | 1925 | 10 |  |  | A7 |  | QPSK | 42@10 |
| 79 | 1925 | 10 |  |  | A2 |  | QPSK | 6@40 |
| 80 | 1935 | 10 |  |  | A4 |  | QPSK | Outer_Full |
| 81 | 1927.5 | 15 |  |  | A1 |  | QPSK | Outer_Full |
| 82 | 1927.5 | 15 |  |  | A7 |  | QPSK | 60@19 |
| 83 | 1927.5 | 15 |  |  | A2 |  | QPSK | 6@56 |
| 84 | 1932.5 | 15 |  |  | A1 |  | QPSK | Outer_Full |
| 85 | 1932.5 | 15 |  |  | A2 |  | QPSK | 6@68 |
| 86 | 1942.5 | 15 |  |  | A5 |  | QPSK | Outer_Full |
| 87 | 1930 | 20 |  |  | A1 |  | QPSK | Outer_Full |
| 88 | 1930 | 20 |  |  | A7 |  | QPSK | 78@28 |
| 89 | 1930 | 20 |  |  | A2 |  | QPSK | 6@76 |
| 90 | 1950 | 20 |  |  | A6 |  | QPSK | Outer_Full |
| 91 | 1922.5 | 5 |  |  | A3 |  | 16 QAM | Outer_Full |
| 92 | 1925 | 10 |  |  | A1 |  | 16 QAM | Outer_Full |
| 93 | 1925 | 10 |  |  | A7 |  | 16 QAM | 42@10 |
| 94 | 1925 | 10 |  |  | A2 |  | 16 QAM | 6@40 |
| 95 | 1935 | 10 |  |  | A4 |  | 16 QAM | Outer_Full |
| 96 | 1927.5 | 15 |  |  | A1 |  | 16 QAM | Outer_Full |
| 97 | 1927.5 | 15 |  |  | A7 |  | 16 QAM | 60@19 |
| 98 | 1927.5 | 15 |  |  | A2 |  | 16 QAM | 6@56 |
| 99 | 1932.5 | 15 |  |  | A1 |  | 16 QAM | Outer_Full |

| AMPR Test parameters for NS_05U |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | ChBw (MHz) | Downlink Config. | Uplink config | A-MPR | SUL Configuration |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |
| 100 | 1932.5 | 15 |  |  | A2 |  | 16 QAM | 6@68 |
| 101 | 1942.5 | 15 |  |  | A5 |  | 16 QAM | Outer_Full |
| 102 | 1930 | 20 |  |  | A1 |  | 16 QAM | Outer_Full |
| 103 | 1930 | 20 |  |  | A7 |  | 16 QAM | 78@28 |
| 104 | 1930 | 20 |  |  | A2 |  | 16 QAM | 6@76 |
| 105 | 1950 | 20 |  |  | A6 |  | 16 QAM | Outer_Full |
| 106 | 1922.5 | 5 |  |  | A3 |  | 64 QAM | Outer_Full |
| 107 | 1925 | 10 |  |  | A1 |  | 64 QAM | Outer_Full |
| 108 | 1925 | 10 |  |  | A7 |  | 64 QAM | 42@10 |
| 109 | 1925 | 10 |  |  | A2 |  | 64 QAM | 6@40 |
| 110 | 1935 | 10 |  |  | A4 |  | 64 QAM | Outer_Full |
| 111 | 1927.5 | 15 |  |  | A1 |  | 64 QAM | Outer_Full |
| 112 | 1927.5 | 15 |  |  | A7 |  | 64 QAM | 60@19 |
| 113 | 1927.5 | 15 |  |  | A2 |  | 64 QAM | 6@56 |
| 114 | 1932.5 | 15 |  |  | A1 |  | 64 QAM | Outer_Full |
| 115 | 1932.5 | 15 |  |  | A2 |  | 64 QAM | 6@68 |
| 116 | 1942.5 | 15 |  |  | A5 |  | 64 QAM | Outer_Full |
| 117 | 1930 | 20 |  |  | A1 |  | 64 QAM | Outer_Full |
| 118 | 1930 | 20 |  |  | A7 |  | 64 QAM | 78@28 |
| 119 | 1930 | 20 |  |  | A2 |  | 64 QAM | 6@76 |
| 120 | 1950 | 20 |  |  | A6 |  | 64 QAM | Outer_Full |
| 121 | 1922.5 | 5 |  |  | A3 |  | 256 QAM | Outer_Full |
| 122 | 1925 | 10 |  |  | A1 |  | 256 QAM | Outer_Full |
| 123 | 1925 | 10 |  |  | A7 |  | 256 QAM | 42@10 |
| 124 | 1935 | 10 |  |  | A4 |  | 256 QAM | Outer_Full |
| 125 | 1927.5 | 15 |  |  | A1 |  | 256 QAM | Outer_Full |
| 126 | 1927.5 | 15 |  |  | A7 |  | 256 QAM | 60@19 |
| 127 | 1932.5 | 15 |  |  | A1 |  | 256 QAM | Outer_Full |
| 128 | 1942.5 | 15 |  |  | A5 |  | 256 QAM | Outer_Full |
| 129 | 1930 | 20 |  |  | A1 |  | 256 QAM | Outer_Full |
| 130 | 1930 | 20 |  |  | A7 |  | 256 QAM | 78@28 |
| 131 | 1950 | 20 |  |  | A6 |  | 256 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |

Table 6.2C.5.4-7: Test Configuration Table for NS_100

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | SUL carrier:Low range, High range NUL carrier: Mid range |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | SUL carrier:Lowest, Highest NUL carrier: Highest |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | 15kHz for SUL and Lowest supported SCS for NUL carriers |
| Test parameters for NS_100 |  |  |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration | SUL Configuration |  |  |
|  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 1 | Low | N/A | N/A | DFT-s-OFDM | PI/2 BPSK | Edge_1RB_Left |
| 2 | High |  |  |  | PI/2 BPSK | Edge_1RB_Right |
| 3 | Default |  |  |  | PI/2 BPSK | Outer_Full |
| 4 | Low |  |  |  | QPSK | Edge_1RB_Left |
| 5 | High |  |  |  | QPSK | Edge_1RB_Right |
| 6 | Default |  |  |  | QPSK | Outer_Full |
| 7 | Low |  |  |  | 16 QAM | Edge_1RB_Left |
| 8 | High |  |  |  | 16 QAM | Edge_1RB_Right |
| 9 | Default |  |  |  | 16 QAM | Outer_Full |
| 10 | Low |  |  |  | 64 QAM | Edge_1RB_Left |
| 11 | High |  |  |  | 64 QAM | Edge_1RB_Right |
| 12 | Default |  |  |  | 64 QAM | Outer_Full |
| 13 | Low |  |  |  | 256 QAM | Edge_1RB_Left |
| 14 | High |  |  |  | 256 QAM | Edge_1RB_Right |
| 15 | Default |  |  |  | 256 QAM | Outer_Full |
| 16 | Low |  |  | CP-s OFDM | QPSK | Edge_1RB_Left |
| 17 | High |  |  |  | QPSK | Edge_1RB_Right |
| 18 | Default |  |  |  | QPSK | Outer_Full |
| 19 | Low |  |  |  | 16 QAM | Edge_1RB_Left |
| 20 | High |  |  |  | 16 QAM | Edge_1RB_Right |
| 21 | Default |  |  |  | 16 QAM | Outer_Full |
| 22 | Low |  |  |  | 64 QAM | Edge_1RB_Left |
| 23 | High |  |  |  | 64 QAM | Edge_1RB_Right |
| 24 | Default |  |  |  | 64 QAM | Outer_Full |
| 25 | Low |  |  |  | 256 QAM | Edge_1RB_Left |
| 26 | High |  |  |  | 256 QAM | Edge_1RB_Right |
| 27 | Default |  |  |  | 256 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Void. NOTE 4: Void |  |  |  |  |  |  |

Table 6.2C.5.4-8: Test Configuration Table for NS_56

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Use uplink carrier center frequency (Fc) as specified in test parameters |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 5 MHz, 10 MHz as specified in test parameters for SUL carrier Lowest for non-SUL carrier |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | 15kHz for SUL and Lowest supported SCS for NUL carriers |  |
| A-MPR test parameters for NS_56 |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | Downlink Configuration | Uplink Configuration | A-MPR | Uplink Configuration |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  |  | RB allocation (Note 1) |
|  |  |  |  |  |  |  |  |  | SCS 15 kHz |
| 1 | Low | 5 | N/A for A-MPR testing | N/A | 14 | DFT-s-OFDM | PI/2 BPSK |  | Edge_1RB_Left |
| 2 | Low | 5 |  |  | 6 |  | PI/2 BPSK |  | Outer_Full |
| 3 | Low | 5 |  |  | 4 |  | PI/2 BPSK |  | 20@4 |
| 4 | Low | 5 |  |  | 4 |  | PI/2 BPSK |  | 1@4 |
| 5 | Low | 5 |  |  | 14 |  | QPSK |  | Edge_1RB_Left |
| 6 | Low | 5 |  |  | 6 |  | QPSK |  | Outer_Full |
| 7 | Low | 5 |  |  | 4 |  | QPSK |  | 20@4 |
| 8 | Low | 5 |  |  | 4 |  | QPSK |  | 1@4 |
| 9 | Low | 5 |  |  | 14 |  | 16 QAM |  | Edge_1RB_Left |
| 10 | Low | 5 |  |  | 6 |  | 16 QAM |  | Outer_Full |
| 11 | Low | 5 |  |  | 4 |  | 16 QAM |  | 20@4 |
| 12 | Low | 5 |  |  | 4 |  | 16 QAM |  | 1@4 |
| 13 | Low | 5 |  |  | 14 |  | 64 QAM |  | Edge_1RB_Left |
| 14 | Low | 5 |  |  | 6 |  | 64 QAM |  | Outer_Full |
| 15 | Low | 5 |  |  | 4 |  | 64 QAM |  | 20@4 |
| 16 | Low | 5 |  |  | 4 |  | 64 QAM |  | 1@4 |
| 17 | Low | 5 |  |  | 14 |  | 256 QAM |  | Edge_1RB_Left |
| 18 | Low | 5 |  |  | 6 |  | 256 QAM |  | Outer_Full |
| 19 | Low | 5 |  |  | 4 |  | 256 QAM |  | 20@4 |
| 20 | Low | 5 |  |  | 4 |  | 256 QAM |  | 1@4 |
| 21 | Low | 10 |  |  | 12 |  | PI/2 BPSK |  | Edge_1RB_Left |
| 22 | Low | 10 |  |  | 8 |  | PI/2 BPSK |  | Outer_Full |

| 23 | Low | 10 |  |  | 6 |  | PI/2 BPSK | 1@3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24 | Low | 10 |  |  | 6 |  | PI/2 BPSK | 40@9 |
| 25 | Low | 10 |  |  | 4 |  | PI/2 BPSK | 1@35 |
| 26 | Low | 10 |  |  | 2 |  | PI/2 BPSK | 16@35 |
| 27 | Low | 10 |  |  | 5 |  | PI/2 BPSK | Edge_1RB_Right |
| 28 | Low | 10 |  |  | 5 |  | PI/2 BPSK | 1@40 |
| 29 | Low | 10 |  |  | 3 |  | PI/2 BPSK | 8@44 |
| 30 | Low | 10 |  |  | 12 |  | QPSK | Edge_1RB_Left |
| 31 | Low | 10 |  |  | 8 |  | QPSK | Outer_Full |
| 32 | Low | 10 |  |  | 6 |  | QPSK | 1@3 |
| 33 | Low | 10 |  |  | 6 |  | QPSK | 40@9 |
| 34 | Low | 10 |  |  | 4 |  | QPSK | 1@35 |
| 35 | Low | 10 |  |  | 2 |  | QPSK | 16@35 |
| 36 | Low | 10 |  |  | 5 |  | QPSK | Edge_1RB_Right |
| 37 | Low | 10 |  |  | 5 |  | QPSK | 1@40 |
| 38 | Low | 10 |  |  | 3 |  | QPSK | 8@44 |
| 39 | Low | 10 |  |  | 12 |  | 16 QAM | Edge_1RB_Left |
| 40 | Low | 10 |  |  | 8 |  | 16 QAM | Outer_Full |
| 41 | Low | 10 |  |  | 6 |  | 16 QAM | 1@3 |
| 42 | Low | 10 |  |  | 6 |  | 16 QAM | 40@9 |
| 43 | Low | 10 |  |  | 4 |  | 16 QAM | 1@35 |
| 44 | Low | 10 |  |  | 2 |  | 16 QAM | 16@35 |
| 45 | Low | 10 |  |  | 5 |  | 16 QAM | Edge_1RB_Right |
| 46 | Low | 10 |  |  | 5 |  | 16 QAM | 1@40 |
| 47 | Low | 10 |  |  | 3 |  | 16 QAM | 8@44 |
| 48 | Low | 10 |  |  | 12 |  | 64 QAM | Edge_1RB_Left Outer_Full |
| 49 | Low | 10 |  |  | 8 |  | 64 QAM |  |
| 50 | Low | 10 |  |  | 6 |  | 64 QAM | 1@3 40@9 |
| 51 | Low | 10 |  |  | 6 |  | 64 QAM |  |
| 52 | Low | 10 |  |  | 4 |  | 64 QAM | 1@35 |
| 53 | Low | 10 |  |  | 2 |  | 64 QAM | 16@35 |

| A-MPR test parameters for NS_56 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | ChBw (MHz) | Downlink Configuration | Uplink Configuration | A-MPR | Uplink Configuration |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |
|  |  |  |  |  |  |  |  | SCS 15 kHz |
| 54 | Low | 10 |  |  | 5 |  | 64 QAM | Edge_1RB_Right |
| 55 | Low | 10 |  |  | 5 |  | 64 QAM | 1@40 |
| 56 | Low | 10 |  |  | 3 |  | 64 QAM | 8@44 |
| 57 | Low | 10 |  |  | 12 |  | 256 QAM | Edge_1RB_Left |
| 58 | Low | 10 |  |  | 8 |  | 256 QAM | Outer_Full |
| 59 | Low | 10 |  |  | 6 |  | 256 QAM | 1@3 |
| 60 | Low | 10 |  |  | 6 |  | 256 QAM | 40@9 |
| 61 | Low | 10 |  |  | 4 |  | 256 QAM | 1@35 |
| 62 | Low | 10 |  |  | 2 |  | 256 QAM | 16@35 |
| 63 | Low | 10 |  |  | 5 |  | 256 QAM | Edge_1RB_Right |
| 64 | Low | 10 |  |  | 5 |  | 256 QAM | 1@40 |
| 65 | Low | 10 |  |  | 3 |  | 256 QAM | 8@44 |
| 66 | Low | 5 | N/A for A-MPR testing | N/A | 14 | CP-OFDM | QPSK | Edge_1RB_Left |
| 67 | Low | 5 |  |  | 6 |  | QPSK | Outer_Full |
| 68 | Low | 5 |  |  | 4 |  | QPSK | 21@4 |
| 69 | Low | 5 |  |  | 4 |  | QPSK | 1@4 |
| 70 | Low | 5 |  |  | 14 |  | 16 QAM | Edge_1RB_Left |
| 71 | Low | 5 |  |  | 6 |  | 16 QAM | Outer_Full |
| 72 | Low | 5 |  |  | 4 |  | 16 QAM | 21@4 |
| 73 | Low | 5 |  |  | 4 |  | 16 QAM | 1@4 |
| 74 | Low | 5 |  |  | 14 |  | 64 QAM | Edge_1RB_Left |
| 75 | Low | 5 |  |  | 6 |  | 64 QAM | Outer_Full |
| 76 | Low | 5 |  |  | 4 |  | 64 QAM | 21@4 |
| 77 | Low | 5 |  |  | 4 |  | 64 QAM | 1@4 |
| 78 | Low | 5 |  |  | 14 |  | 256 QAM | Edge_1RB_Left |
| 79 | Low | 5 |  |  | 6 |  | 256 QAM | Outer_Full |
| 80 | Low | 5 |  |  | 4 |  | 256 QAM | 21@4 |
| 81 | Low | 5 |  |  | 4 |  | 256 QAM | 1@4 |
| 82 | Low | 10 |  |  | 12 |  | QPSK | Edge_1RB_Left |
| 83 | Low | 10 |  |  | 8 |  | QPSK | Outer_Full |
| 84 | Low | 10 |  |  | 6 |  | QPSK | 1@3 |

| A-MPR test parameters for NS_56 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | ChBw (MHz) | Downlink Configuration | Uplink Configuration | A-MPR | Uplink Configuration |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |
|  |  |  |  |  |  |  |  | SCS 15 kHz |
| 85 | Low | 10 |  |  | 6 |  | QPSK | 43@9 |
| 86 | Low | 10 |  |  | 4 |  | QPSK | 1@35 |
| 87 | Low | 10 |  |  | 2 |  | QPSK | 17@35 |
| 88 | Low | 10 |  |  | 5 |  | QPSK | Edge_1RB_Right |
| 89 | Low | 10 |  |  | 5 |  | QPSK | 1@40 |
| 90 | Low | 10 |  |  | 3 |  | QPSK | 8@44 |
| 91 | Low | 10 |  |  | 12 |  | 16 QAM | Edge_1RB_Left |
| 92 | Low | 10 |  |  | 8 |  | 16 QAM | Outer_Full |
| 93 | Low | 10 |  |  | 6 |  | 16 QAM | 1@3 |
| 94 | Low | 10 |  |  | 6 |  | 16 QAM | 43@9 |
| 95 | Low | 10 |  |  | 4 |  | 16 QAM | 1@35 |
| 96 | Low | 10 |  |  | 2 |  | 16 QAM | 17@35 |
| 97 | Low | 10 |  |  | 5 |  | 16 QAM | Edge_1RB_Right |
| 98 | Low | 10 |  |  | 5 |  | 16 QAM | 1@40 |
| 99 | Low | 10 |  |  | 3 |  | 16 QAM | 8@44 |
| 100 | Low | 10 |  |  | 12 |  | 64 QAM | Edge_1RB_Left |
| 101 | Low | 10 |  |  | 8 |  | 64 QAM | Outer_Full |
| 102 | Low | 10 |  |  | 6 |  | 64 QAM | 1@3 |
| 103 | Low | 10 |  |  | 6 |  | 64 QAM | 43@9 |
| 104 | Low | 10 |  |  | 4 |  | 64 QAM | 1@35 |
| 105 | Low | 10 |  |  | 2 |  | 64 QAM | 17@35 |
| 106 | Low | 10 |  |  | 5 |  | 64 QAM | Edge_1RB_Right |
| 107 | Low | 10 |  |  | 5 |  | 64 QAM | 1@40 |
| 108 | Low | 10 |  |  | 3 |  | 64 QAM | 8@44 |
| 109 | Low | 10 |  |  | 12 |  | 256 QAM | Edge_1RB_Left |

| A-MPR test parameters for NS_56 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | ChBw (MHz) | Downlink Configuration | Uplink Configuration | A-MPR | Uplink Configuration |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |
|  |  |  |  |  |  |  |  | SCS 15 kHz |
| 110 | Low | 10 |  |  | 8 |  | 256 QAM | Outer_Full |
| 111 | Low | 10 |  |  | 6 |  | 256 QAM | 1@3 |
| 112 | Low | 10 |  |  | 6 |  | 256 QAM | 43@9 |
| 113 | Low | 10 |  |  | 4 |  | 256 QAM | 1@35 |
| 114 | Low | 10 |  |  | 2 |  | 256 QAM | 17@35 |
| 115 | Low | 10 |  |  | 5 |  | 256 QAM | Edge_1RB_Right |
| 116 | Low | 10 |  |  | 5 |  | 256 QAM | 1@40 |
| 117 | Low | 10 |  |  | 3 |  | 256 QAM | 8@44 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

6.3C.1	Minimum output power for SUL

6.3C.1.1	Test purpose

Same test purpose as in clause 6.3.1.1

6.3C.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.3C.1.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

The normative reference for this requirement is TS 38.101-1 [2] clauses 4.3 and 6.3.1.

6.3C.1.4	Test description

Same test description as specified in clause 6.3.1.4 with following exceptions:

Instead of table 5.3.5-1  use Table 5.5C-1.

Instead of table 6.3.1.4.1-1  use Table 6.3C.1.4-1

Table 6.3C.1.4-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low, Mid, High range for SUL carrier Mid-range for Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  | SUL Configuration |  |
|  | N/A | N/A |  | Modulation | RB allocation (NOTE 2) |
| 1 |  |  |  | DFT-s-OFDM QPSK | Outer Full |
| NOTE 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 3: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |

<Unchanged Text Skipped>

6.3C.3	Transmit ON/OFF time mask for SUL

6.3C.3.1	Test purpose

Same test purpose as in clause 6.3.3.2.1

6.3C.3.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.3C.3.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

The normative reference for this requirement is TS 38.101-1 [2] clauses 4.3 and 6.3.3.2

6.3C.3.4	Test description

Same test description as specified in clause 6.3.3.2.4 with following exceptions:

Instead of table 5.3.5-1  use Table 5.5C-1.

Instead of table 6.3.3.2.4.1-1  use Table 6.3C.3.4-1

Table 6.3C.3.4-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low, Mid, High range for SUL carrier Mid-range for Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  | SUL Configuration |  |
|  | N/A | N/A |  | Modulation | RB allocation (NOTE 2) |
| 1 |  |  |  | DFT-s-OFDM QPSK | Inner Full |
| NOTE 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |  |  |

<Unchanged Text Skipped>

6.3C.4.1	Absolute power tolerance for SUL

6.3C.4.1.1	Test purpose

Same test purpose as in clause 6.3.4.2.1

6.3C.4.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.3C.4.1.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

The normative reference for this requirement is TS 38.101-1 [2] clauses 4.3 and 6.3.4.2.

6.3C.4.1.4	Test description

Same test description as specified in clause 6.3.4.2.4 with following exceptions:

Instead of table 5.3.5-1  use Table 5.5C-1.

Instead of table 6.3.4.2.4.1-1  use Table 6.3C.4.1.4-1

Table 6.3C.4.1.4-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid-range for SUL and Non-SUL carrier |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest for SUL carrier Lowest for Non-SUL carrier |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration | SUL Configuration |  |
|  |  |  | Modulation | RB allocation (NOTE 2) |
| 1 | N/A | N/A | CP-OFDM QPSK | Outer_Full |
| NOTE 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 2: The specific configuration of each RF allocation is defined in Table 6.1-1. |  |  |  |  |

<Unchanged Text Skipped>

6.3C.4.2	Relative power tolerance for SUL

6.3C.4.2.1	Test purpose

Same test purpose as in clause 6.3.4.3.1

6.3C.4.2.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.3C.4.2.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

The normative reference for this requirement is TS 38.101-1 [2] clauses 4.3 and 6.3.4.3.

6.3C.4.2.4	Test description

Same test description as specified in clause 6.3.4.3.4 with following exceptions:

Instead of table 5.3.5-1  use Table 5.5C-1.

Instead of table 6.3.4.3.4.1-1  use Table 6.3C.4.2.4-1

Table 6.3C.4.2.4-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range for SUL carrier Mid-range for Non-SUL carrier |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest for SUL carrier Lowest for Non-SUL carrier |  |
| Test SCS as specified in table 5.3.5-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |
| Test Parameters |  |  |  |  |
| Ch BW | Downlink Configuration | Uplink Configuration | SUL Configuration |  |
|  |  |  | Modulation | RB allocation (NOTE 1) |
| 5MHz | N/A |  | DFT-s-OFDM QPSK | See Table 6.3C.4.2.5-1 See Table 6.3C.4.2.5-2 See Table 6.3C.4.2.5-5 |
| 10MHz |  |  | DFT-s-OFDM QPSK | See Table 6.3C.4.2.5-3 See Table 6.3C.4.2.5-4 See Table 6.3C.4.2.5-5 |
| 15MHz |  |  | DFT-s-OFDM QPSK | See Table 6.3C.4.2.5-3 See Table 6.3C.4.2.5-4 See Table 6.3C.4.2.5-5 |
| 20MHz |  |  | DFT-s-OFDM QPSK | See Table 6.3C.4.2.5-3 See Table 6.3C.4.2.5-4 See Table 6.3C.4.2.5-5 |
| 25MHz |  |  | DFT-s-OFDM QPSK | See Table 6.3C.4.2.5-3 See Table 6.3C.4.2.5-4 See Table 6.3C.4.2.5-5 |
| 30MHz |  |  | DFT-s-OFDM QPSK | See Table 6.3C.4.2.5-3 See Table 6.3C.4.2.5-4 See Table 6.3C.4.2.5-5 |
| 40MHz |  |  | DFT-s-OFDM QPSK | See Table 6.3C.4.2.5-3 See Table 6.3C.4.2.5-4 See Table 6.3C.4.2.5-5 |
| NOTE 1: The starting resource block shall be RB# 0 NOTE 2: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. |  |  |  |  |

<Unchanged Text Skipped>

6.3C.4.3	Aggregate power tolerance for SUL

6.3C.4.3.1	Test purpose

Same test purpose as in clause 6.3.4.3.1

6.3C.4.3.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.3C.4.3.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

The normative reference for this requirement is TS 38.101-1 [2] clauses 4.3 and 6.3.4.4.

6.3C.4.3.4	Test description

Same test description as specified in clause 6.3.4.4.4 with following exceptions:

Instead of table 5.3.5-1  use Table 5.5C-1.

Instead of table 6.3.4.4.4.1-1  use Table 6.3C.4.3.4-1

Instead of table 6.3.4.4.4.1-2  use Table 6.3C.4.3.4-2

Table 6.3C.4.3.4-1: Test Configuration Table: PUCCH sub-test

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid range for SUL and Non-SUL carrier |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest for SUL carrier Lowest for Non-SUL carrier |
| Test SCS as specified in Table 5.3.5-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |
| Test Parameters for Channel Bandwidths |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration | SUL Configuration |
|  |  |  | PUCCH format = Format 1 Length in OFDM symbols = 14 |
| 1 | N/A | N/A |  |

Table 6.3C.4.3.4-2: Test Configuration Table: PUSCH sub-test

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid range for SUL and Non-SUL carrier |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest for SUL carrier Lowest for Non-SUL carrier |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration | SUL Configuration |  |
|  |  |  | Modulation | RB allocation (NOTE 1) |
| 1 | N/A | N/A | CP-OFDM QPSK | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. |  |  |  |  |

<Unchanged Text Skipped>

6.4C.1	Frequency error for SUL

6.4C.1.1	Test purpose

Same test purpose as in clause 6.4.1.1

6.4C.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.4C.1.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

The normative reference for this requirement is TS 38.101-1 [2] clauses 4.3and 6.4.1.

6.4C.1.4	Test description

Same test description as specified in clause 6.4.1.4 with following exceptions:

Instead of table 5.3.5-1  use Table 5.5C-1.

Instead of table 6.4.1.4-1  use Table 6.4C.1.4-1

Table 6.4C.1.4-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid range for both SUL carrier and Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| Test ID | Downlink Configuration |  | UL Configuration | SUL Configuration |  |
|  | Modulation | RB allocation | N/A | Modulation | RB allocation |
| 1 | CP-OFDM QPSK | Full RB (NOTE 1) |  | DFT-s-OFDM QPSK | SUL REFSENS (NOTE 2) |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2 NOTE 2: SUL REFSENS refers to Table 7.3C.2.4.1-1a which defines SUL RB configuration and start RB location for each SCS, channel BW and NR band. |  |  |  |  |  |

<Unchanged Text Skipped>

6.4C.2	Transmit modulation quality for SUL

6.4C.2.1	Error Vector Magnitude for SUL

6.4C.2.1.1	Test purpose

Same test purpose as in clause 6.4.2.1

6.4C.2.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.4C.2.1.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

The normative reference for this requirement is TS 38.101-1 [2] clauses 4.3and 6.4.2.1.

6.4C.2.1.4	Test description

Same test description as specified in clause 6.4.2.1.4 with following exceptions:

Instead of table 5.3.5-1  use Table 5.5C-1.

Instead of table 6.4.2.1.4-1, table 6.4.2.1.4-2, table 6.4.2.1.4-3  use Table 6.4C.2.1.4-1, table 6.4C.2.1.4-2, table 6.4C.2.1.4-3

Table 6.4C.2.1.4-1: Test Configuration Table for PUSCH

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low, Mid, High range for SUL carrier Mid range for Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| Test ID | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
|  | N/A | N/A |  | Modulation | RB allocation (NOTE 2) |
| 13 |  |  |  | DFT-s-OFDM PI/2 BPSK | Inner Full |
| 23 |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer Full |
| 3 |  |  |  | DFT-s-OFDM QPSK | Inner Full |
| 4 |  |  |  | DFT-s-OFDM QPSK | Outer Full |
| 5 |  |  |  | DFT-s-OFDM 16 QAM | Inner Full |
| 6 |  |  |  | DFT-s-OFDM 16 QAM | Outer Full |
| 7 |  |  |  | DFT-s-OFDM 64 QAM | Outer Full |
| 8 |  |  |  | DFT-s-OFDM 256 QAM | Outer Full |
| 9 |  |  |  | CP-OFDM QPSK | Inner Full |
| 10 |  |  |  | CP-OFDM QPSK | Outer Full |
| 11 |  |  |  | CP-OFDM 16 QAM | Inner Full |
| 12 |  |  |  | CP-OFDM 16 QAM | Outer Full |
| 13 |  |  |  | CP-OFDM 64 QAM | Outer Full |
| 14 |  |  |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 3: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |

<Unchanged Text Skipped>

6.4C.2.2	Carrier leakage for SUL

6.4C.2.2.1	Test purpose

Same test purpose as in clause 6.4.2.2.1.

6.4C.2.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.4C.2.2.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

The normative reference for this requirement is TS 38.101-1 [2] clauses 4.3and 6.4.2.2.

6.4C.2.2.4	Test description

Same test description as specified in clause 6.4.2.2.4 with following exceptions:

Instead of table 5.3.5-1  use Table 5.5C-1.

Instead of table 6.4.2.2.4-1  use Table 6.4C.2.2.4-1

Table 6.4C.2.2.4-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low, Mid, High range for SUL carrier Mid range for Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| Test ID | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
|  | N/A | N/A |  | Modulation | RB allocation (NOTE 1) |
| 1 |  |  |  | DFT-s-OFDM QPSK | Inner_1RB_Left |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 3: When the signalled DC carrier position is at Inner_1RB_Left, use Inner_1RB_Right for UL RB allocation.. |  |  |  |  |  |

<Unchanged Text Skipped>

6.4C.2.3	In-band emissions for SUL

6.4C.2.3.1	Test purpose

Same test purpose as in clause 6.4.2.3.1.

6.4C.2.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.4C.2.3.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

The normative reference for this requirement is TS 38.101-1 [2] clauses 4.3and 6.4.2.3.

6.4C.2.3.4	Test description

Same test description as specified in clause 6.4.2.3.4 with following exceptions:

Instead of table 5.3.5-1  use Table 5.5C-1.

Instead of table 6.4.2.3.4-1  use Table 6.4C.2.3.4-1

Table 6.4C.2.3.4-1: Test Configuration Table for PUSCH

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, Mid range, High range for SUL carrier Mid range for Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| Test ID | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
|  | N/A | N/A |  | Modulation | RB allocation (NOTE 1) |
| 1 |  |  |  | DFT-s-OFDM QPSK | Inner_1RB_Left |
| 2 |  |  |  | DFT-s-OFDM QPSK | Inner_1RB_Right |
| 3 |  |  |  | CP-OFDM QPSK | Inner_1RB_Left |
| 4 |  |  |  | CP-OFDM QPSK | Inner_1RB_Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. |  |  |  |  |  |

<Unchanged Text Skipped>

6.4C.2.4	EVM equalizer spectrum flatness for SUL

6.4C.2.4.1	Test purpose

Same test purpose as in clause 6.4.2.4.1.

6.4C.2.4.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.4C.2.4.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

The normative reference for this requirement is TS 38.101-1 [2] clauses 4.3 and 6.4.2.4.

6.4C.2.4.4	Test description

Same test description as specified in clause 6.4.2.4.4 with following exceptions:

Instead of table 5.3.5-1  use Table 5.5C-1.

Instead of table 6.4.2.4.4-1  use Table 6.4C.2.4.4-1

Table 6.4C.2.4.4-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, Mid range, High range for SUL carrier Mid range for Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.5C-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| Test ID | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
|  | N/A | N/A |  | Modulation | RB allocation (NOTE 1) |
| 1 |  |  |  | DFT-s-OFDM QPSK | Outer Full |
| 2 |  |  |  | CP-OFDM QPSK | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. |  |  |  |  |  |

<Unchanged Text Skipped>

6.4C.2.5	EVM equalizer spectrum flatness for Pi/2 BPSK for SUL

6.4C.2.5.1	Test purpose

Same test purpose as in clause 6.4.2.5.1.

6.4C.2.5.2	Test applicability

This test applies to all types of NR UE release 16 and forward that support SUL operating on the SUL bands and indicate support for UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16.

6.4C.2.5.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

The normative reference for this requirement is TS 38.101-1 [2] clauses 4.3 and 6.4.2.4.1.

6.4C.2.5.4	Test description

6.4C.2.5.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.5C-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in table 6.4C.2.5.4.1-1. The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annexes A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.4C.2.5.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, Mid range, High range for SUL carrier Mid range for Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.5C-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| Test ID | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
|  | N/A | N/A |  | Modulation | RB allocation (NOTE 1) |
| 1 |  |  |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. |  |  |  |  |  |

<Unchanged Text Skipped>

6.5C.1	Occupied bandwidth for SUL

6.5C.1.1	Test purpose

To verify that the UE occupied bandwidth for all transmission bandwidth configurations supported by the UE supporting SUL are less than their specific limits when UE is configured using SUL transmission.

6.5C.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.5C.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.1.3

6.5C.1.4	Test description

Same test description as specified in clause 6.5.1.4 with following exceptions:

-	Instead of table 5.3.5-1  use Table 5.5C-1

-	Instead of table 6.5.1.4.1-1 use Table 6.5C.1.4-1

Table 6.5C.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low Range, Mid Range, High Range for SUL carrier Mid Range for Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | All for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
|  | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
| Test ID | N/A | N/A |  | Modulation | RB allocation |
| 1 |  |  |  | CP-OFDM QPSK | Outer_full |
| Note 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. Note 2: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |  |  |

<Unchanged Text Skipped>

6.5C.2.2	Spectrum emission mask for SUL

The spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the  edge of the assigned NR channel bandwidth. For frequencies greater than (ΔfOOB) the spurious requirements in subclause 6.5.3 are applicable.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

6.5C.2.2.1	Test purpose

To verify that the power of any UE emission shall not exceed specified level for the specified channel bandwidth.

6.5C.2.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.5C.2.2.3	Minimum conformance requirements

Same minimum conformance requirements as in the clause 6.5.2.2.3.

6.5C.2.2.4	Test description

Same test description as PC 3 with contiguous allocation specified in clause 6.5.2.2 with following exceptions:

-	Instead of table 5.3.5-1  use Table 5.5C-1

-	Instead of table 6.5.2.2.4.1-1 use Table 6.5C.2.2.4.1-1

Table 6.5C.2.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Low range, High range for SUL carrier Mid range for Non-SUL carrier |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Lowest, Highest for SUL carrier Lowest for Non-SUL carrier |  |
| Test SCS as specified in Table 5.5C-1 |  |  |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
| Test ID | Freq | Downlink Configuration | UL Configuration | SUL Configuration |  |  |
|  |  | N/A | N/A | Modulation |  | RB allocation |
| 1 | Low |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 2 | High |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Right |
| 3 | Default |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer_Full |
| 4 | Low |  |  | DFT-s-OFDM QPSK |  | Edger_1RB_Left |
| 5 | High |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Right |
| 6 | Default |  |  | DFT-s-OFDM QPSK |  | Outer_Full |
| 7 | Low |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Left |
| 8 | High |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Right |
| 9 | Default |  |  | DFT-s-OFDM 16 QAM |  | Outer_Full |
| 10 | Low |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Left |
| 11 | High |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Right |
| 12 | Default |  |  | DFT-s-OFDM 64 QAM |  | Outer_Full |
| 13 | Low |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Left |
| 14 | High |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Right |
| 15 | Default |  |  | DFT-s-OFDM 256 QAM |  | Outer_Full |
| 16 | Low |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 17 | High |  |  | CP-OFDM QPSK |  | Edge_1RB_Right |
| 18 | Default |  |  | CP-OFDM QPSK |  | Outer_Full |
| 19 | Low |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 20 | High |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Right |
| 21 | Default |  |  | CP-OFDM 16 QAM |  | Outer_Full |
| 22 | Low |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |
| 23 | High |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Right |
| 24 | Default |  |  | CP-OFDM 64 QAM |  | Outer_Full |
| 25 | Low |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Left |
| 26 | High |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Right |
| 27 | Default |  |  | CP-OFDM 256 QAM |  | Outer_Full |
| 284 | Low |  |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS |  | Edge_1RB_Left |
| 294 | High |  |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS |  | Edge_1RB_Right |
| 304 | Default |  |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS |  | Outer Full |
| Note 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. Note 2: The specific configuration of each RF allocation is defined in Table 6.1-1. Note 3: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. Note 4: Applicable to UEs indicating support for UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16. |  |  |  |  |  |  |

<Unchanged Text Skipped>

6.5C.3	Spurious emissions for SUL

6.5C.3.1	General spurious emissions for SUL

6.5C.3.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions.

6.5C.3.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.5C.3.1.3	Minimum conformance requirements

The general spurious emission requirement specified in clause 6.5.3.1.3 applies to the UE that support SUL operating on the SUL bands.

The normative reference for this requirement is TS 38.101-1 [2] subclauses 6.5.3.1

6.5C.3.1.4	Test description

Same test description as specified in clause 6.5.3.1.4 with following exceptions:

-	Instead of table 5.3.5-1  use Table 5.5C-1

-	Instead of table 6.5.3.1.4.1-1 use Table 6.5C.3.1.4-1

Table 6.5C.3.1.4-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, Mid range, High range for SUL carrier Mid range for Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| Test ID | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
|  | N/A | N/A |  | Modulation | RB allocation (NOTE 2) |
| 1 |  |  |  | CP-OFDM QPSK | OuterFull |
| 2 |  |  |  | CP-OFDM QPSK | Edge_1RB_Left |
| 3 |  |  |  | CP-OFDM QPSK | Edge_1RB_Right |
| NOTE 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 3: Void |  |  |  |  |  |

<Unchanged Text Skipped>

6.5C.3.2	Spurious emissions for UE co-existence for SUL

6.5C.3.2.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to co-existing systems for the specified bands which has specific requirements in terms of transmitter spurious emissions.

6.5C.3.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.5C.3.2.3	Minimum conformance requirements

The requirements for NR bands for coexistence with protected bands specified in subclause 6.5.3.2.3 apply to the UE that support SUL operating on the SUL bands

The normative reference for this requirement is TS 38.101-1 [2] subclause 6.5.3.2.

6.5C.3.2.4	Test description

Same test description as specified in clause 6.5.3.2.4 with following exceptions:

-	Instead of table 5.3.5-1  use Table 5.5C-1

-	Instead of table 6.5.3.2.4.1-1 use Table 6.5C.3.2.4-1

Table 6.5C.3.2.4-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, Mid range, High range for SUL carrier Mid range for Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| Test ID | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
|  | N/A | N/A |  | Modulation | RB allocation (NOTE 1) |
| 1 |  |  |  | CP-OFDM QPSK | OuterFull |
| 2 |  |  |  | CP-OFDM QPSK | Edge_1RB_Left |
| 3 |  |  |  | CP-OFDM QPSK | Edge_1RB_Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 Common UL configuration. NOTE 2: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. |  |  |  |  |  |

<Unchanged Text Skipped>

6.5C.3.3	Additional spurious emissions for SUL

6.5C.3.3.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions under the deployment scenarios where additional requirements are specified.

6.5C.3.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.5C.3.3.3	Minimum conformance requirements

The additional spurious emission requirements specified in 6.5.3.3.3 apply to the UE operating on SUL bands.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5.3.3

6.5C.3.3.4	Test description

Same test description as specified in clause 6.5.3.3.4 with following exceptions:

-	Instead of table 5.3.5-1  use Table 5.5C-1

-	For NS_05, instead of table 6.5.3.3.4.1-4 use Table 6.5C.3.3.4-1

-	For NS_43, instead of table 6.5.3.3.4.1-5 use Table 6.5C.3.3.4-2

-	For NS_18, instead of table 6.5.3.3.4.1-3 use Table 6.5C.3.3.4-3

-	For NS_48, instead of table 6.5.3.3.4.1-22 use Table 6.5C.3.3.4-4

-	For NS_49, instead of table 6.5.3.3.4.1-23 use Table 6.5C.3.3.4-5

-	For NS_56, instead of table 6.5.3.3.4.1-27 use Table 6.5C.3.3.4-6

Table 6.5C.3.3.4-1: Test Configuration Table (network signalling value “NS_05”)

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Use centre frequency (Fc) as specified in test parameters for SUL carrier Mid range for Non-SUL carrier |  |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | 5 MHz, 10 MHz, 15 MHz, 20 MHz for SUL carrier Lowest for Non-SUL carrier |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |  |  |  |
| Additional Spurious for SUL test parameters for NS_05 |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | SUL ChBw (MHz) | Downlink Config. |  | Uplink Config | A-MPR | SUL Configuration |  |  |
|  |  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |
| 1 | 1922.5 | 5 | N/A |  | N/A | A3 | DFT-s-OFDM | PI/2 BPSK | Outer_Full |
| 2 | 1925 | 10 |  |  |  | A1 |  | PI/2 BPSK | Outer_Full |
| 3 | 1925 | 10 |  |  |  | A7 |  | PI/2 BPSK | 40@10 |
| 4 | 1925 | 10 |  |  |  | A2 |  | PI/2 BPSK | 6@40 |
| 5 | 1935 | 10 |  |  |  | A4 |  | PI/2 BPSK | Outer_Full |
| 6 | 1927.5 | 15 |  |  |  | A1 |  | PI/2 BPSK | Outer_Full |
| 7 | 1927.5 | 15 |  |  |  | A7 |  | PI/2 BPSK | 60@19 |
| 8 | 1927.5 | 15 |  |  |  | A2 |  | PI/2 BPSK | 6@56 |
| 9 | 1932.5 | 15 |  |  |  | A1 |  | PI/2 BPSK | Outer_Full |
| 10 | 1932.5 | 15 |  |  |  | A2 |  | PI/2 BPSK | 6@68 |
| 11 | 1942.5 | 15 |  |  |  | A5 |  | PI/2 BPSK | Outer_Full |
| 12 | 1930 | 20 |  |  |  | A1 |  | PI/2 BPSK | Outer_Full |
| 13 | 1930 | 20 |  |  |  | A7 |  | PI/2 BPSK | 72@28 |
| 14 | 1930 | 20 |  |  |  | A2 |  | PI/2 BPSK | 6@76 |
| 15 | 1950 | 20 |  |  |  | A6 |  | PI/2 BPSK | Outer_Full |
| 16 | 1922.5 | 5 |  |  |  | A3 |  | QPSK | Outer_Full |
| 17 | 1925 | 10 |  |  |  | A1 |  | QPSK | Outer_Full |
| 18 | 1925 | 10 |  |  |  | A7 |  | QPSK | 40@10 |
| 19 | 1925 | 10 |  |  |  | A2 |  | QPSK | 6@40 |
| 20 | 1927.5 | 15 |  |  |  | A1 |  | QPSK | Outer_Full |
| 21 | 1927.5 | 15 |  |  |  | A7 |  | QPSK | 60@19 |
| 22 | 1927.5 | 15 |  |  |  | A2 |  | QPSK | 6@56 |
| 23 | 1932.5 | 15 |  |  |  | A1 |  | QPSK | Outer_Full |
| 24 | 1932.5 | 15 |  |  |  | A2 |  | QPSK | 6@68 |
| 25 | 1942.5 | 15 |  |  |  | A5 |  | QPSK | Outer_Full |
| 26 | 1930 | 20 |  |  |  | A1 |  | QPSK | Outer_Full |
| 27 | 1930 | 20 |  |  |  | A7 |  | QPSK | 72@28 |
| 28 | 1930 | 20 |  |  |  | A2 |  | QPSK | 6@76 |
| 29 | 1950 | 20 |  |  |  | A6 |  | QPSK | Outer_Full |
| 30 | 1922.5 | 5 |  |  |  | A3 |  | 16 QAM | Outer_Full |
| 31 | 1925 | 10 |  |  |  | A1 |  | 16 QAM | Outer_Full |
| 32 | 1925 | 10 |  |  |  | A7 |  | 16 QAM | 40@10 |
| 33 | 1925 | 10 |  |  |  | A2 |  | 16 QAM | 6@40 |
| 34 | 1927.5 | 15 |  |  |  | A1 |  | 16 QAM | Outer_Full |
| 35 | 1927.5 | 15 |  |  |  | A7 |  | 16 QAM | 60@19 |
| 36 | 1927.5 | 15 |  |  |  | A2 |  | 16 QAM | 6@56 |
| 37 | 1932.5 | 15 |  |  |  | A1 |  | 16 QAM | Outer_Full |
| 38 | 1932.5 | 15 |  |  |  | A2 |  | 16 QAM | 6@68 |
| 39 | 1930 | 20 |  |  |  | A1 |  | 16 QAM | Outer_Full |
| 40 | 1930 | 20 |  |  |  | A7 |  | 16 QAM | 72@28 |
| 41 | 1930 | 20 |  |  |  | A2 |  | 16 QAM | 6@76 |
| 42 | 1922.5 | 5 |  |  |  | A3 |  | 64 QAM | Outer_Full |
| 43 | 1925 | 10 |  |  |  | A1 |  | 64 QAM | Outer_Full |
| 44 | 1925 | 10 |  |  |  | A7 |  | 64 QAM | 40@10 |
| 45 | 1925 | 10 |  |  |  | A2 |  | 64 QAM | 6@40 |
| 46 | 1927.5 | 15 |  |  |  | A1 |  | 64 QAM | Outer_Full |
| 47 | 1927.5 | 15 |  |  |  | A7 |  | 64 QAM | 60@19 |
| 48 | 1927.5 | 15 |  |  |  | A2 |  | 64 QAM | 6@56 |
| 49 | 1932.5 | 15 |  |  |  | A1 |  | 64 QAM | Outer_Full |
| 50 | 1932.5 | 15 |  |  |  | A2 |  | 64 QAM | 6@68 |
| 51 | 1930 | 20 |  |  |  | A1 |  | 64 QAM | Outer_Full |
| 52 | 1930 | 20 |  |  |  | A7 |  | 64 QAM | 72@28 |
| 53 | 1930 | 20 |  |  |  | A2 |  | 64 QAM | 6@76 |
| 54 | 1922.5 | 5 |  |  |  | A3 |  | 256 QAM | Outer_Full |
| 55 | 1925 | 10 |  |  |  | A1 |  | 256 QAM | Outer_Full |
| 56 | 1925 | 10 |  |  |  | A7 |  | 256 QAM | 40@10 |
| 57 | 1925 | 10 |  |  |  | A2 |  | 256 QAM | 6@40 |
| 58 | 1927.5 | 15 |  |  |  | A1 |  | 256 QAM | Outer_Full |
| 59 | 1927.5 | 15 |  |  |  | A7 |  | 256 QAM | 60@19 |
| 60 | 1927.5 | 15 |  |  |  | A2 |  | 256 QAM | 6@56 |
| 61 | 1932.5 | 15 |  |  |  | A1 |  | 256 QAM | Outer_Full |
| 62 | 1932.5 | 15 |  |  |  | A2 |  | 256 QAM | 6@68 |
| 63 | 1930 | 20 |  |  |  | A1 |  | 256 QAM | Outer_Full |
| 64 | 1930 | 20 |  |  |  | A7 |  | 256 QAM | 72@28 |
| 65 | 1930 | 20 |  |  |  | A2 |  | 256 QAM | 6@76 |
| 66 | 1922.5 | 5 |  |  |  | A3 | CP-OFDM | QPSK | Outer_Full |
| 67 | 1925 | 10 |  |  |  | A1 |  | QPSK | Outer_Full |
| 68 | 1925 | 10 |  |  |  | A7 |  | QPSK | 42@10 |
| 69 | 1925 | 10 |  |  |  | A2 |  | QPSK | 6@40 |
| 70 | 1935 | 10 |  |  |  | A4 |  | QPSK | Outer_Full |
| 71 | 1927.5 | 15 |  |  |  | A1 |  | QPSK | Outer_Full |
| 72 | 1927.5 | 15 |  |  |  | A7 |  | QPSK | 60@19 |
| 73 | 1927.5 | 15 |  |  |  | A2 |  | QPSK | 6@56 |
| 74 | 1932.5 | 15 |  |  |  | A1 |  | QPSK | Outer_Full |
| 75 | 1932.5 | 15 |  |  |  | A2 |  | QPSK | 6@68 |
| 76 | 1942.5 | 15 |  |  |  | A5 |  | QPSK | Outer_Full |
| 77 | 1930 | 20 |  |  |  | A1 |  | QPSK | Outer_Full |
| 78 | 1930 | 20 |  |  |  | A7 |  | QPSK | 78@28 |
| 79 | 1930 | 20 |  |  |  | A2 |  | QPSK | 6@76 |
| 80 | 1950 | 20 |  |  |  | A6 |  | QPSK | Outer_Full |
| 81 | 1922.5 | 5 |  |  |  | A3 |  | 16 QAM | Outer_Full |
| 82 | 1925 | 10 |  |  |  | A1 |  | 16 QAM | Outer_Full |
| 83 | 1925 | 10 |  |  |  | A7 |  | 16 QAM | 42@10 |
| 84 | 1925 | 10 |  |  |  | A2 |  | 16 QAM | 6@40 |
| 85 | 1935 | 10 |  |  |  | A4 |  | 16 QAM | Outer_Full |
| 86 | 1927.5 | 15 |  |  |  | A1 |  | 16 QAM | Outer_Full |
| 87 | 1927.5 | 15 |  |  |  | A7 |  | 16 QAM | 60@19 |
| 88 | 1927.5 | 15 |  |  |  | A2 |  | 16 QAM | 6@56 |
| 89 | 1932.5 | 15 |  |  |  | A1 |  | 16 QAM | Outer_Full |
| 90 | 1932.5 | 15 |  |  |  | A2 |  | 16 QAM | 6@68 |
| 91 | 1942.5 | 15 |  |  |  | A5 |  | 16 QAM | Outer_Full |
| 92 | 1930 | 20 |  |  |  | A1 |  | 16 QAM | Outer_Full |
| 93 | 1930 | 20 |  |  |  | A7 |  | 16 QAM | 78@28 |
| 94 | 1930 | 20 |  |  |  | A2 |  | 16 QAM | 6@76 |
| 95 | 1950 | 20 |  |  |  | A6 |  | 16 QAM | Outer_Full |
| 96 | 1922.5 | 5 |  |  |  | A3 |  | 64 QAM | Outer_Full |
| 97 | 1925 | 10 |  |  |  | A1 |  | 64 QAM | Outer_Full |
| 98 | 1925 | 10 |  |  |  | A7 |  | 64 QAM | 42@10 |
| 99 | 1925 | 10 |  |  |  | A2 |  | 64 QAM | 6@40 |
| 100 | 1927.5 | 15 |  |  |  | A1 |  | 64 QAM | Outer_Full |
| 101 | 1927.5 | 15 |  |  |  | A7 |  | 64 QAM | 60@19 |
| 102 | 1927.5 | 15 |  |  |  | A2 |  | 64 QAM | 6@56 |
| 103 | 1932.5 | 15 |  |  |  | A1 |  | 64 QAM | Outer_Full |
| 104 | 1932.5 | 15 |  |  |  | A2 |  | 64 QAM | 6@68 |
| 105 | 1930 | 20 |  |  |  | A1 |  | 64 QAM | Outer_Full |
| 106 | 1930 | 20 |  |  |  | A7 |  | 64 QAM | 78@28 |
| 107 | 1930 | 20 |  |  |  | A2 |  | 64 QAM | 6@76 |
| 108 | 1922.5 | 5 |  |  |  | A3 |  | 256 QAM | Outer_Full |
| 109 | 1925 | 10 |  |  |  | A1 |  | 256 QAM | Outer_Full |
| 110 | 1925 | 10 |  |  |  | A7 |  | 256 QAM | 42@10 |
| 111 | 1927.5 | 15 |  |  |  | A1 |  | 256 QAM | Outer_Full |
| 112 | 1927.5 | 15 |  |  |  | A7 |  | 256 QAM | 60@19 |
| 113 | 1932.5 | 15 |  |  |  | A1 |  | 256 QAM | Outer_Full |
| 114 | 1930 | 20 |  |  |  | A1 |  | 256 QAM | Outer_Full |
| 115 | 1930 | 20 |  |  |  | A7 |  | 256 QAM | 78@28 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: AMPR requirement for NS_05 is specified in 6.2.3.3.4. |  |  |  |  |  |  |  |  |  |

Table 6.5C.3.3.4-2: Test Configuration Table (network signalling value “NS_43”)

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Use carrier centre frequency (Fc) as specified in test parameters for SUL carrier Mid range for Non-SUL carrier |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | 5 MHz, 10 MHz, 15 MHz for SUL carrier Lowest for Non-SUL carrier |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | 15 kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |
| Additional Spurious for SUL test parameters for NS_43 |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | SUL Ch BW (MHz) | Downlink Configuration | Uplink Configuration | SUL Configuration |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  |  | RB allocation (Note 1) |
| 1 | 910 | 10 | N/A | N/A | DFT-s--OFDM | PI/2 BPSK |  | Outer_Full (A2) |
| 2 | 907.5 | 15 |  |  |  | PI/2 BPSK |  | Edge_1RB_Left (A6) |
| 3 | 907.5 | 15 |  |  |  | PI/2 BPSK |  | Outer_Full (A6) |
| 4 | 902.5 | 5 |  |  |  | QPSK |  | Outer_Full (A1) |
| 5 | 910 | 10 |  |  |  | QPSK |  | Outer_Full (A4) |
| 6 | 907.5 | 15 |  |  |  | QPSK |  | Edge_1RB_Left (A6) |
| 7 | 907.5 | 15 |  |  |  | QPSK |  | Outer_Full (A6) |
| 8 | 910 | 10 |  |  |  | 16 QAM |  | Outer_Full (A5) |
| 9 | 907.5 | 15 |  |  |  | 16 QAM |  | Edge_1RB_Left (A6) |
| 10 | 907.5 | 15 |  |  |  | 16 QAM |  | Outer_Full (A6) |
| 11 | 910 | 10 |  |  |  | 64 QAM |  | Outer_Full (A3) |
| 12 | 907.5 | 15 |  |  |  | 64 QAM |  | Edge_1RB_Left (A6) |
| 13 | 907.5 | 15 |  |  |  | 64 QAM |  | Outer_Full (A6) |
| 14 | 907.5 | 15 |  |  |  | 256 QAM |  | Edge_1RB_Left (A6) |
| 15 | 907.5 | 15 |  |  |  | 256 QAM |  | Outer_Full (A6) |
| 16 | 902.5 | 5 |  |  | CP-OFDM | QPSK |  | Outer_Full (A1) |
| 17 | 910 | 10 |  |  |  | QPSK |  | Outer_Full (A5) |
| 18 | 907.5 | 15 |  |  |  | QPSK |  | Edge_1RB_Left (A6) |
| 19 | 907.5 | 15 |  |  |  | QPSK |  | Outer_Full (A6) |
| 20 | 902.5 | 5 |  |  |  | 16 QAM |  | Outer_Full (A1) |
| 21 | 910 | 10 |  |  |  | 16 QAM |  | Outer_Full (A5) |
| 22 | 907.5 | 15 |  |  |  | 16 QAM |  | Edge_1RB_Left (A6) |
| 23 | 907.5 | 15 |  |  |  | 16 QAM |  | Outer_Full (A6) |
| 24 | 910 | 10 |  |  |  | 64 QAM |  | Outer_Full (A3) |
| 25 | 907.5 | 15 |  |  |  | 64 QAM |  | Edge_1RB_Left (A6) |
| 26 | 907.5 | 15 |  |  |  | 64 QAM |  | Outer_Full (A6) |
| 27 | 907.5 | 15 |  |  |  | 256 QAM |  | Edge_1RB_Left (A6) |
| 28 | 907.5 | 15 |  |  |  | 256 QAM |  | Outer_Full (A6) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 2: AMPR requirement for NS_43 is specified in 6.2.3.3.6. |  |  |  |  |  |  |  |  |

Table 6.5C.3.3.4-3: Test Configuration Table (network signalling value “NS_18”)

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Low range for SUL carrier Mid range for Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | 5MHz 10MHz 20MHz 30MHz for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 15 kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| A-MPR test parameters for NS_18 |  |  |  |  |  |  |
| Test ID | ChBw SCS | Downlink Configuration | SUL Configuration |  |  |  |
|  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) | Comment |
| 1 | 5MHz, 10MHz, 20MHz | N/A | DFT-s OFDM | QPSK | Edge_1RB_Left | A1, A2 |
| 2 |  |  |  | QPSK | Outer_Full | A1, A2 |
| 3 |  |  |  | 16 QAM | Edge_1RB_Left | A1, A2 |
| 4 |  |  |  | 16 QAM | Outer_Full | A1, A2 |
| 5 |  |  |  | 64 QAM | Edge_1RB_Left | A1, A2 |
| 6 |  |  |  | 64 QAM | Outer_Full | A1, A2 |
| 7 |  |  |  | 256 QAM | Edge_1RB_Left | A1, A2 |
| 8 |  |  |  | 256 QAM | Outer_Full | A1, A2 |
| 9 |  |  | CP-s OFDM | QPSK | Edge_1RB_Left | A1, A2 |
| 10 |  |  |  | QPSK | Outer_Full | A1, A2 |
| 11 |  |  |  | 16 QAM | Edge_1RB_Left | A1, A2 |
| 12 |  |  |  | 16 QAM | Outer_Full | A1, A2 |
| 13 |  |  |  | 64 QAM | Edge_1RB_Left | A1, A2 |
| 14 |  |  |  | 64 QAM | Outer_Full | A1, A2 |
| 15 |  |  |  | 256 QAM | Edge_1RB_Left | A1, A2 |
| 16 |  |  |  | 256 QAM | Outer_Full | A1, A2 |
| 17 | 30MHz |  | DFT-s OFDM | 256 QAM | 81@70 | A3 |
| 18 |  |  |  | 256 QAM | Outer_Full | A4 |
| 19 |  |  | CP-s OFDM | 256 QAM | 87@73 | A3 |
| 20 |  |  |  | 256 QAM | Outer_Full | A4 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |

Table 6.5C.3.3.4-4: Test Configuration Table (network signalling value “NS_48”)

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Low range, High range for SUL carrier Mid range for non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | 25 MHz, 30MHz, 40MHz, 50MHz for SUL carrier Lowest for non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | 15 kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| A-MPR test parameters for NS_48 |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | Uplink Configuration | Downlink Configuration | SUL Configuration |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  |  | RB allocation (Note 1) |
| 1 | Default | 25 | N/A | N/A | DFT-s-OFDM |  | QPSK | Outer_Full (A3) |
| 2 | Default | 25 |  |  |  |  | QPSK | Edge_1RB_Right (A3) |
| 3 | Default | 30 |  |  |  |  | QPSK | Outer_Full (A3) |
| 4 | Default | 30 |  |  |  |  | QPSK | Edge_1RB_Right (A5) |
| 5 | Default | 40 |  |  |  |  | QPSK | 16@0 (A2) |
| 6 | Default | 40 |  |  |  |  | QPSK | 90@0 (A3) |
| 7 | Default | 40 |  |  |  |  | QPSK | 150@0 (A4) |
| 8 | Default | 40 |  |  |  |  | QPSK | 192@0 (A2) |
| 9 | Default | 40 |  |  |  |  | QPSK | 5@187 (A3) |
| 10 | Default | 40 |  |  |  |  | QPSK | Outer_Full (A1) |
| 11 | Default | 50 |  |  |  |  | QPSK | 32@0 (A2) |
| 12 | Default | 50 |  |  |  |  | QPSK | 108@0 (A4) |
| 13 | Default | 50 |  |  |  |  | QPSK | 225@0 (A2) |
| 14 | Default | 50 |  |  |  |  | QPSK | 5@223 (A5) |
| 15 | Default | 50 |  |  |  |  | QPSK | Outer_Full (A1) |
| 16 | Default | 25 |  |  |  |  | 256 QAM | Outer_Full (A3) |
| 17 | Default | 25 |  |  |  |  | 256 QAM | Edge_1RB_Right (A3) |
| 18 | Default | 30 |  |  |  |  | 256 QAM | Outer_Full (A3) |
| 19 | Default | 30 |  |  |  |  | 256 QAM | Edge_1RB_Right (A5) |
| 20 | Default | 40 |  |  |  |  | 256 QAM | 16@0 (A2) |
| 21 | Default | 40 |  |  |  |  | 256 QAM | 90@0 (A3) |
| 22 | Default | 40 |  |  |  |  | 256 QAM | 150@0 (A4) |
| 23 | Default | 40 |  |  |  |  | 256 QAM | 192@0 (A2) |
| 24 | Default | 40 |  |  |  |  | 256 QAM | 5@187 (A3) |
| 25 | Default | 40 |  |  |  |  | 256 QAM | Outer_Full (A1) |
| 26 | Default | 50 |  |  |  |  | 256 QAM | 32@0 (A2) |
| 27 | Default | 50 |  |  |  |  | 256 QAM | 108@0 (A4) |
| 28 | Default | 50 |  |  |  |  | 256 QAM | 225@0 (A2) |
| 29 | Default | 50 |  |  |  |  | 256 QAM | 5@223 (A5) |
| 30 | Default | 50 |  |  |  |  | 256 QAM | Outer_Full (A1) |
| 31 | Default | 25 |  |  | CP-OFDM |  | QPSK | Outer_Full (A3) |
| 32 | Default | 25 |  |  |  |  | QPSK | Edge_1RB_Right (A3) |
| 33 | Default | 30 |  |  |  |  | QPSK | Outer_Full (A3) |
| 34 | Default | 30 |  |  |  |  | QPSK | Edge_1RB_Right (A5) |
| 35 | Default | 40 |  |  |  |  | QPSK | 16@0 (A2) |
| 36 | Default | 40 |  |  |  |  | QPSK | 95@0 (A3) |
| 37 | Default | 40 |  |  |  |  | QPSK | 152@0 (A4) |
| 38 | Default | 40 |  |  |  |  | QPSK | 192@0 (A2) |
| 39 | Default | 40 |  |  |  |  | QPSK | 5@187 (A3) |
| 40 | Default | 40 |  |  |  |  | QPSK | Outer_Full (A1) |
| 41 | Default | 50 |  |  |  |  | QPSK | 34@0 (A2) |
| 42 | Default | 50 |  |  |  |  | QPSK | 115@0 (A4) |
| 43 | Default | 50 |  |  |  |  | QPSK | 228@0 (A2) |
| 44 | Default | 50 |  |  |  |  | QPSK | 5@223 (A5) |
| 45 | Default | 50 |  |  |  |  | QPSK | Outer_Full (A1) |
| 46 | Default | 25 |  |  |  |  | 256 QAM | Outer_Full (A3) |
| 47 | Default | 25 |  |  |  |  | 256 QAM | Edge_1RB_Right (A3) |
| 48 | Default | 30 |  |  |  |  | 256 QAM | Outer_Full (A3) |
| 49 | Default | 30 |  |  |  |  | 256 QAM | Edge_1RB_Right (A5) |
| 50 | Default | 40 |  |  |  |  | 256 QAM | 16@0 (A2) |
| 51 | Default | 40 |  |  |  |  | 256 QAM | 95@0 (A3) |
| 52 | Default | 40 |  |  |  |  | 256 QAM | 152@0 (A4) |
| 53 | Default | 40 |  |  |  |  | 256 QAM | 192@0 (A2) |
| 54 | Default | 40 |  |  |  |  | 256 QAM | 5@187 (A3) |
| 55 | Default | 40 |  |  |  |  | 256 QAM | Outer_Full (A1) |
| 56 | Default | 50 |  |  |  |  | 256 QAM | 34@0 (A2) |
| 57 | Default | 50 |  |  |  |  | 256 QAM | 115@0 (A4) |
| 58 | Default | 50 |  |  |  |  | 256 QAM | 228@0 (A2) |
| 59 | Default | 50 |  |  |  |  | 256 QAM | 5@223 (A5) |
| 60 | Default | 50 |  |  |  |  | 256 QAM | Outer_Full (A1) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |

Table 6.5C.3.3.4-5: Test Configuration Table (network signalling value “NS_49”)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Low range, High range for SUL carrier Mid range for non-SUL carrier |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | 25 MHz, 30MHz, 40MHz, 50MHz for SUL carrier Lowest for non-SUL carrier |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 15 kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |  |
| A-MPR test parameters for NS_49 |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | Downlink Configuration | Uplink Configuration | SUL Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 1 | Default | 25 | N/A | N/A | DFT-s-OFDM | QPSK | Outer_Full (A3) |
| 2 | Default | 25 |  |  |  | QPSK | Edge_1RB_Right (A3) |
| 3 | Default | 25 |  |  |  | QPSK | Edge_1RB_Left (A3) |
| 4 | Default | 30 |  |  |  | QPSK | 20@0 (A1) |
| 5 | Default | 30 |  |  |  | QPSK | 36@0 (A5) |
| 6 | Default | 30 |  |  |  | QPSK | 80@0 (A3) |
| 7 | Default | 30 |  |  |  | QPSK | 120@0 (A4) |
| 8 | Default | 30 |  |  |  | QPSK | Outer_Full (A2) |
| 9 | Default | 30 |  |  |  | QPSK | Edge_1RB_Right (A5) |
| 10 | Default | 40 |  |  |  | QPSK | 40@0 (A1) |
| 11 | Default | 40 |  |  |  | QPSK | 5@53 (A5) |
| 12 | Default | 40 |  |  |  | QPSK | 100@0 (A4) |
| 13 | Default | 40 |  |  |  | QPSK | 150@33 (A2) |
| 14 | Default | 40 |  |  |  | QPSK | 5@187 (A5) |
| 15 | Default | 40 |  |  |  | QPSK | 192@0 (A1) |
| 16 | Default | 40 |  |  |  | QPSK | Outer_Full (A1) |
| 17 | Default | 50 |  |  |  | QPSK | 5@75 (A5) |
| 18 | Default | 50 |  |  |  | QPSK | 5@215 (A5) |
| 19 | Default | 50 |  |  |  | QPSK | 162@45 (A2) |
| 20 | Default | 50 |  |  |  | QPSK | 220@0 (A1) |
| 21 | Default | 50 |  |  |  | QPSK | Outer_Full (A1) |
| 22 | Default | 25 |  |  |  | 256 QAM | Outer_Full (A3) |
| 23 | Default | 25 |  |  |  | 256 QAM | Edge_1RB_Right (A3) |
| 24 | Default | 25 |  |  |  | 256 QAM | Edge_1RB_Left (A3) |
| 25 | Default | 30 |  |  |  | 256 QAM | 20@0 (A1) |
| 26 | Default | 30 |  |  |  | 256 QAM | 36@0 (A5) |
| 27 | Default | 30 |  |  |  | 256 QAM | 80@0 (A3) |
| 28 | Default | 30 |  |  |  | 256 QAM | 120@0 (A4) |
| 29 | Default | 30 |  |  |  | 256 QAM | Outer_Full (A2) |
| 30 | Default | 30 |  |  |  | 256 QAM | Edge_1RB_Right (A5) |
| 31 | Default | 40 |  |  |  | 256 QAM | 40@0 (A1) |
| 32 | Default | 40 |  |  |  | 256 QAM | 5@53 (A5) |
| 33 | Default | 40 |  |  |  | 256 QAM | 100@0 (A4) |
| 34 | Default | 40 |  |  |  | 256 QAM | 150@33 (A2) |
| 35 | Default | 40 |  |  |  | 256 QAM | 5@187 (A5) |
| 36 | Default | 40 |  |  |  | 256 QAM | 192@0 (A1) |
| 37 | Default | 40 |  |  |  | 256 QAM | Outer_Full (A1) |
| 38 | Default | 50 |  |  |  | 256 QAM | 5@75 (A5) |
| 39 | Default | 50 |  |  |  | 256 QAM | 5@215 (A5) |
| 40 | Default | 50 |  |  |  | 256 QAM | 162@45 (A2) |
| 41 | Default | 50 |  |  |  | 256 QAM | 216@0 (A1) |
| 42 | Default | 50 |  |  |  | 256 QAM | Outer_Full (A1) |
| 43 | Default | 25 |  |  | CP-OFDM | QPSK | Outer_Full (A3) |
| 44 | Default | 25 |  |  |  | QPSK | Edge_1RB_Right (A3) |
| 45 | Default | 25 |  |  |  | QPSK | Edge_1RB_Left (A3) |
| 46 | Default | 30 |  |  |  | QPSK | 20@0 (A1) |
| 47 | Default | 30 |  |  |  | QPSK | 36@0 (A5) |
| 48 | Default | 30 |  |  |  | QPSK | 80@0 (A3) |
| 49 | Default | 30 |  |  |  | QPSK | 120@0 (A4) |
| 50 | Default | 30 |  |  |  | QPSK | Outer_Full (A2) |
| 51 | Default | 30 |  |  |  | QPSK | Edge_1RB_Right (A5) |
| 52 | Default | 40 |  |  |  | QPSK | 40@0 (A1) |
| 53 | Default | 40 |  |  |  | QPSK | 5@53 (A5) |
| 54 | Default | 40 |  |  |  | QPSK | 100@0 (A4) |
| 55 | Default | 40 |  |  |  | QPSK | 159@33 (A2) |
| 56 | Default | 40 |  |  |  | QPSK | 5@187 (A5) |
| 57 | Default | 40 |  |  |  | QPSK | 192@0 (A1) |
| 58 | Default | 40 |  |  |  | QPSK | Outer_Full (A1) |
| 59 | Default | 50 |  |  |  | QPSK | 5@75 (A5) |
| 60 | Default | 50 |  |  |  | QPSK | 5@215 (A5) |
| 61 | Default | 50 |  |  |  | QPSK | 175@45 (A2) |
| 62 | Default | 50 |  |  |  | QPSK | 216@0 (A1) |
| 63 | Default | 50 |  |  |  | QPSK | Outer_Full (A1) |
| 64 | Default | 25 |  |  |  | 256 QAM | Outer_Full (A3) |
| 65 | Default | 25 |  |  |  | 256 QAM | Edge_1RB_Right (A3) |
| 66 | Default | 25 |  |  |  | 256 QAM | Edge_1RB_Left (A3) |
| 67 | Default | 30 |  |  |  | 256 QAM | 20@0 (A1) |
| 68 | Default | 30 |  |  |  | 256 QAM | 36@0 (A5) |
| 69 | Default | 30 |  |  |  | 256 QAM | 80@0 (A3) |
| 70 | Default | 30 |  |  |  | 256 QAM | 120@0 (A4) |
| 71 | Default | 30 |  |  |  | 256 QAM | Outer_Full (A2) |
| 72 | Default | 30 |  |  |  | 256 QAM | Edge_1RB_Right (A5) |
| 73 | Default | 40 |  |  |  | 256 QAM | 40@0 (A1) |
| 74 | Default | 40 |  |  |  | 256 QAM | 5@53 (A5) |
| 75 | Default | 40 |  |  |  | 256 QAM | 100@0 (A4) |
| 76 | Default | 40 |  |  |  | 256 QAM | 150@33 (A2) |
| 77 | Default | 40 |  |  |  | 256 QAM | 5@187 (A5) |
| 78 | Default | 40 |  |  |  | 256 QAM | 192@0 (A1) |
| 79 | Default | 40 |  |  |  | 256 QAM | Outer_Full (A1) |
| 80 | Default | 50 |  |  |  | 256 QAM | 5@75 (A5) |
| 81 | Default | 50 |  |  |  | 256 QAM | 5@215 (A5) |
| 82 | Default | 50 |  |  |  | 256 QAM | 175@45 (A2) |
| 83 | Default | 50 |  |  |  | 256 QAM | 220@0 (A1) |
| 84 | Default | 50 |  |  |  | 256 QAM | Outer_Full (A1) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |

Table 6.5C.3.3.4-6: Test Configuration Table (network signalled value “NS_56”)

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Use uplink carrier centre frequency (Fc) as specified in test parameters |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 5 MHz, 10 MHz as specified in test parameters for SUL carrier Lowest for non-SUL carrier |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | 15 kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |
| A-MPR test parameters for NS_56 |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | Downlink Configuration | Uplink Configuration | A-MPR | Uplink Configuration |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  |  | RB allocation (Note 1) |
|  |  |  |  |  |  |  |  |  | SCS 15 kHz |
| 1 | Low | 5 | N/A for A-MPR testing | N/A | 14 | DFT-s-OFDM | PI/2 BPSK |  | Edge_1RB_Left |
| 2 | Low | 5 |  |  | 6 |  | PI/2 BPSK |  | Outer_Full |
| 3 | Low | 5 |  |  | 4 |  | PI/2 BPSK |  | 20@4 |
| 4 | Low | 5 |  |  | 4 |  | PI/2 BPSK |  | 1@4 |
| 5 | Low | 5 |  |  | 14 |  | QPSK |  | Edge_1RB_Left |
| 6 | Low | 5 |  |  | 6 |  | QPSK |  | Outer_Full |
| 7 | Low | 5 |  |  | 4 |  | QPSK |  | 20@4 |
| 8 | Low | 5 |  |  | 4 |  | QPSK |  | 1@4 |
| 9 | Low | 5 |  |  | 14 |  | 16 QAM |  | Edge_1RB_Left |
| 10 | Low | 5 |  |  | 6 |  | 16 QAM |  | Outer_Full |
| 11 | Low | 5 |  |  | 4 |  | 16 QAM |  | 20@4 |
| 12 | Low | 5 |  |  | 4 |  | 16 QAM |  | 1@4 |
| 13 | Low | 5 |  |  | 14 |  | 64 QAM |  | Edge_1RB_Left |
| 14 | Low | 5 |  |  | 6 |  | 64 QAM |  | Outer_Full |
| 15 | Low | 5 |  |  | 4 |  | 64 QAM |  | 20@4 |
| 16 | Low | 5 |  |  | 4 |  | 64 QAM |  | 1@4 |
| 17 | Low | 5 |  |  | 14 |  | 256 QAM |  | Edge_1RB_Left |
| 18 | Low | 5 |  |  | 6 |  | 256 QAM |  | Outer_Full |
| 19 | Low | 5 |  |  | 4 |  | 256 QAM |  | 20@4 |
| 20 | Low | 5 |  |  | 4 |  | 256 QAM |  | 1@4 |
| 21 | Low | 10 |  |  | 12 |  | PI/2 BPSK |  | Edge_1RB_Left |
| 22 | Low | 10 |  |  | 8 |  | PI/2 BPSK |  | Outer_Full |
| 23 | Low | 10 |  |  | 6 |  | PI/2 BPSK |  | 1@3 |
| 24 | Low | 10 |  |  | 6 |  | PI/2 BPSK |  | 40@9 |
| 25 | Low | 10 |  |  | 4 |  | PI/2 BPSK |  | 1@35 |
| 26 | Low | 10 |  |  | 2 |  | PI/2 BPSK |  | 16@35 |
| 27 | Low | 10 |  |  | 5 |  | PI/2 BPSK |  | Edge_1RB_Right |
| 28 | Low | 10 |  |  | 5 |  | PI/2 BPSK |  | 1@40 |
| 29 | Low | 10 |  |  | 3 |  | PI/2 BPSK |  | 8@44 |
| 30 | Low | 10 |  |  | 12 |  | QPSK |  | Edge_1RB_Left |
| 31 | Low | 10 |  |  | 8 |  | QPSK |  | Outer_Full |
| 32 | Low | 10 |  |  | 6 |  | QPSK |  | 1@3 |
| 33 | Low | 10 |  |  | 6 |  | QPSK |  | 40@9 |
| 34 | Low | 10 |  |  | 4 |  | QPSK |  | 1@35 |
| 35 | Low | 10 |  |  | 2 |  | QPSK |  | 16@35 |
| 36 | Low | 10 |  |  | 5 |  | QPSK |  | Edge_1RB_Right |
| 37 | Low | 10 |  |  | 5 |  | QPSK |  | 1@40 |
| 38 | Low | 10 |  |  | 3 |  | QPSK |  | 8@44 |
| 39 | Low | 10 |  |  | 12 |  | 16 QAM |  | Edge_1RB_Left |
| 40 | Low | 10 |  |  | 8 |  | 16 QAM |  | Outer_Full |
| 41 | Low | 10 |  |  | 6 |  | 16 QAM |  | 1@3 |
| 42 | Low | 10 |  |  | 6 |  | 16 QAM |  | 40@9 |
| 43 | Low | 10 |  |  | 4 |  | 16 QAM |  | 1@35 |
| 44 | Low | 10 |  |  | 2 |  | 16 QAM |  | 16@35 |
| 45 | Low | 10 |  |  | 5 |  | 16 QAM |  | Edge_1RB_Right |
| 46 | Low | 10 |  |  | 5 |  | 16 QAM |  | 1@40 |
| 47 | Low | 10 |  |  | 3 |  | 16 QAM |  | 8@44 |
| 48 | Low | 10 |  |  | 12 |  | 64 QAM |  | Edge_1RB_Left Outer_Full |
| 49 | Low | 10 |  |  | 8 |  | 64 QAM |  |  |
| 50 | Low | 10 |  |  | 6 |  | 64 QAM |  | 1@3 40@9 |
| 51 | Low | 10 |  |  | 6 |  | 64 QAM |  |  |
| 52 | Low | 10 |  |  | 4 |  | 64 QAM |  | 1@35 |
| 53 | Low | 10 |  |  | 2 |  | 64 QAM |  | 16@35 |
| 54 | Low | 10 |  |  | 5 |  | 64 QAM |  | Edge_1RB_Right |
| 55 | Low | 10 |  |  | 5 |  | 64 QAM |  | 1@40 |
| 56 | Low | 10 |  |  | 3 |  | 64 QAM |  | 8@44 |
| 57 | Low | 10 |  |  | 12 |  | 256 QAM |  | Edge_1RB_Left |
| 58 | Low | 10 |  |  | 8 |  | 256 QAM |  | Outer_Full |
| 59 | Low | 10 |  |  | 6 |  | 256 QAM |  | 1@3 |
| 60 | Low | 10 |  |  | 6 |  | 256 QAM |  | 40@9 |
| 61 | Low | 10 |  |  | 4 |  | 256 QAM |  | 1@35 |
| 62 | Low | 10 |  |  | 2 |  | 256 QAM |  | 16@35 |
| 63 | Low | 10 |  |  | 5 |  | 256 QAM |  | Edge_1RB_Right |
| 64 | Low | 10 |  |  | 5 |  | 256 QAM |  | 1@40 |
| 65 | Low | 10 |  |  | 3 |  | 256 QAM |  | 8@44 |
| 66 | Low | 5 | N/A for A-MPR testing | N/A | 14 | CP-OFDM | QPSK |  | Edge_1RB_Left |
| 67 | Low | 5 |  |  | 6 |  | QPSK |  | Outer_Full |
| 68 | Low | 5 |  |  | 4 |  | QPSK |  | 21@4 |
| 69 | Low | 5 |  |  | 4 |  | QPSK |  | 1@4 |
| 70 | Low | 5 |  |  | 14 |  | 16 QAM |  | Edge_1RB_Left |
| 71 | Low | 5 |  |  | 6 |  | 16 QAM |  | Outer_Full |
| 72 | Low | 5 |  |  | 4 |  | 16 QAM |  | 21@4 |
| 73 | Low | 5 |  |  | 4 |  | 16 QAM |  | 1@4 |
| 74 | Low | 5 |  |  | 14 |  | 64 QAM |  | Edge_1RB_Left |
| 75 | Low | 5 |  |  | 6 |  | 64 QAM |  | Outer_Full |
| 76 | Low | 5 |  |  | 4 |  | 64 QAM |  | 21@4 |
| 77 | Low | 5 |  |  | 4 |  | 64 QAM |  | 1@4 |
| 78 | Low | 5 |  |  | 14 |  | 256 QAM |  | Edge_1RB_Left |
| 79 | Low | 5 |  |  | 6 |  | 256 QAM |  | Outer_Full |
| 80 | Low | 5 |  |  | 4 |  | 256 QAM |  | 21@4 |
| 81 | Low | 5 |  |  | 4 |  | 256 QAM |  | 1@4 |
| 82 | Low | 10 |  |  | 12 |  | QPSK |  | Edge_1RB_Left |
| 83 | Low | 10 |  |  | 8 |  | QPSK |  | Outer_Full |
| 84 | Low | 10 |  |  | 6 |  | QPSK |  | 1@3 |
| 85 | Low | 10 |  |  | 6 |  | QPSK |  | 43@9 |
| 86 | Low | 10 |  |  | 4 |  | QPSK |  | 1@35 |
| 87 | Low | 10 |  |  | 2 |  | QPSK |  | 17@35 |
| 88 | Low | 10 |  |  | 5 |  | QPSK |  | Edge_1RB_Right |
| 89 | Low | 10 |  |  | 5 |  | QPSK |  | 1@40 |
| 90 | Low | 10 |  |  | 3 |  | QPSK |  | 8@44 |
| 91 | Low | 10 |  |  | 12 |  | 16 QAM |  | Edge_1RB_Left |
| 92 | Low | 10 |  |  | 8 |  | 16 QAM |  | Outer_Full |
| 93 | Low | 10 |  |  | 6 |  | 16 QAM |  | 1@3 |
| 94 | Low | 10 |  |  | 6 |  | 16 QAM |  | 43@9 |
| 95 | Low | 10 |  |  | 4 |  | 16 QAM |  | 1@35 |
| 96 | Low | 10 |  |  | 2 |  | 16 QAM |  | 17@35 |
| 97 | Low | 10 |  |  | 5 |  | 16 QAM |  | Edge_1RB_Right |
| 98 | Low | 10 |  |  | 5 |  | 16 QAM |  | 1@40 |
| 99 | Low | 10 |  |  | 3 |  | 16 QAM |  | 8@44 |
| 100 | Low | 10 |  |  | 12 |  | 64 QAM |  | Edge_1RB_Left |
| 101 | Low | 10 |  |  | 8 |  | 64 QAM |  | Outer_Full |
| 102 | Low | 10 |  |  | 6 |  | 64 QAM |  | 1@3 |
| 103 | Low | 10 |  |  | 6 |  | 64 QAM |  | 43@9 |
| 104 | Low | 10 |  |  | 4 |  | 64 QAM |  | 1@35 |
| 105 | Low | 10 |  |  | 2 |  | 64 QAM |  | 17@35 |
| 106 | Low | 10 |  |  | 5 |  | 64 QAM |  | Edge_1RB_Right |
| 107 | Low | 10 |  |  | 5 |  | 64 QAM |  | 1@40 |
| 108 | Low | 10 |  |  | 3 |  | 64 QAM |  | 8@44 |
| 109 | Low | 10 |  |  | 12 |  | 256 QAM |  | Edge_1RB_Left |
| 110 | Low | 10 |  |  | 8 |  | 256 QAM |  | Outer_Full |
| 111 | Low | 10 |  |  | 6 |  | 256 QAM |  | 1@3 |
| 112 | Low | 10 |  |  | 6 |  | 256 QAM |  | 43@9 |
| 113 | Low | 10 |  |  | 4 |  | 256 QAM |  | 1@35 |
| 114 | Low | 10 |  |  | 2 |  | 256 QAM |  | 17@35 |
| 115 | Low | 10 |  |  | 5 |  | 256 QAM |  | Edge_1RB_Right |
| 116 | Low | 10 |  |  | 5 |  | 256 QAM |  | 1@40 |
| 117 | Low | 10 |  |  | 3 |  | 256 QAM |  | 8@44 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

6.5C.4	Transmit intermodulation for SUL

6.5C.4.1	Test purpose

To verify that the UE transmit intermodulation does not exceed the described value in the test requirement.

The transmit intermodulation performance is a measure of the capability of the transmitter to inhibit the generation of signals in its non-linear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter via the antenna.

6.5C.4.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.5C.4.3	Minimum conformance requirements

The requirements in subclause 6.5.4 apply to the UE operating on SUL bands

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5.4.

6.5C.4.4	Test description

Same test description as specified in clause 6.2.4.4 with following exceptions:

-	Instead of table 5.3.5-1  use Table 5.5C-1

-	Instead of table 6.5.4.4.1-1 use Table 6.5C.4.4-1

Table 6.5C.4.4-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid range for both SUL carrier and Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| Test ID | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
|  | N/A | N/A |  | Modulation | RB allocation (NOTE 2) |
| 1 |  |  |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 |  |  |  | DFT-s-OFDM QPSK | Inner Full |
| NOTE 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 3: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |

<End of Changes>