3GPP TSG-RAN5 Meeting #96-e	R5-225074

# Electronic Meeting, 15th – 26th Aug. 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.905 | CR | 0671 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding TP analysis for new FR1 test case 6.3C.3.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | A new FR1 SUL test case 6.3C.3.2 - General transmit ON/OFF time mask for switching between two uplink carriers is introduced into TS 38.521-1 in R5-225073. This test case need specific TP analysis. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating table 4.1.1.2-1 to add a new row for 6.3C.3.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TP analysis for 6.3C.3.2 is absent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 CR 1882 |  |  |
| R1 |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Added file: 38.521-1_TPanalysis_6.3C.3.2_SULTxSwitch_M.zip |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

4.1.1.2	FR1 SUL test cases

This section contains information on test point selection for SUL test cases in [2]. The basic principle is following the same rules for test point selection in single carrier test cases. In these SUL test cases, there are default test points to be used unless SUL configuration specific test points are over-ruling.

Basic rules for Tx SUL test cases:

For Test environment: Adopt the same selection of test environment in corresponding single carrier test cases.

For Test frequency: Considering that Non-SUL carrier should have no impact on SUL carrier testing results, for any SUL configurations, Mid range is chosen as default for Non-SUL carrier. Select the same test frequency in corresponding single carrier test cases for SUL carrier.

For Test SCS: Considering only 15 kHz SCS is supported for SUL bands, it’s reasonable to select 15 kHz SCS for SUL carrier and Non-SUL carrier regardless of SUL configurations.

For Test channel bandwidths: Under the limit of 15 kHz SCS, only the lowest channel bandwidth is supported for current Non-SUL bands in SUL configurations, which are band n78 and n79. Select the lowest channel bandwidth that support 15kHz SCS for Non-SUL carrier. Select the same test channel bandwidths as in corresponding single carrier test cases for SUL carrier.

For waveform, modulation and RB allocations: Adopt the same selection of test configurations as in corresponding single carrier test cases for SUL carrier.

Basic rules for Rx SUL test cases:

In Rx testing for SUL, test point selection in clause 7.3C and 7.6C need to be defined. Considering the focus of Rx test cases is testing DL bands, the configuration of SUL carrier shall be selected to ensure the test coverage without costing too much testing time. The configuration of Non-SUL carrier shall be selected based on the same principle as single carrier test cases. The basic test point selection rule for Rx SUL test cases is specified as below:

For Test environment: Adopt the same selection of test environment in corresponding single carrier test cases.

For Test frequency: The Non-SUL carrier should select the same test frequency as corresponding single carrier test cases. Select Mid range as default for SUL carrier.

For Test SCS: Since the REFSENS requirement for SUL is specified for 15 kHz SCS for SUL band and the test point selection of clause 7.6C is also based on that of clause 7.3C, 15 kHz SCS should be selected for SUL carrier. For the Non-SUL carrier the SCS should be selected following the same rule as single carrier testing.

For test channel bandwidths: Highest channel bandwidth when SCS =15 kHz for SUL shall be selected for SUL carrier. For the Non-SUL carrier the channel bandwidth should be selected following the same rule as single carrier testing.

For waveform, modulation and RB allocations: Adopt the same selection of test configurations as single carrier test cases for Non-SUL carrier. SUL carrier select DFT-s-OFDM QPSK. The RB allocation of SUL carrier shall fulfill the requirement in clause 7.3C.0 in TS 38.521-1.

Number of test points for SUL test cases in FR1 are listed in table 4.1.1.2-1 and table 4.1.1.2-2.

Table 4.1.1.2-1: Number of test points for SUL test cases in FR1 (NR UE Transmitter test)

| Subclause | Number of test points | Comments |
| --- | --- | --- |
| 6.2C.1 Configured transmitted power for SUL | 30 | RAN5#86e |
| 6.2C.3 UE maximum output power for SUL | 270 | RAN5#86e |
| 6.2C.4 UE maximum output power reduction for SUL | 460 (5001) | RAN5#90e |
| 6.2C.5 UE additional maximum output power reduction for SUL | Table 4.1.1.1-1 | RAN5#87e |
| 6.3C.1 Minimum output power for SUL | 45 | RAN5#87e |
| 6.3C.3.1 Transmit ON/OFF time mask for SUL | 45 | RAN5#87e |
| 6.3C.3.2 General transmit ON/OFF time mask for switching between two uplink carriers | 1 | RAN5#96e |
| 6.3C.4.1 Absolute power tolerance for SUL | 3 | RAN5#87e |
| 6.3C.4.2 Power Control Relative power tolerance for SUL | TBD | RAN5#87e |
| 6.3C.4.3 Aggregate power tolerance for SUL | PUCCH: 3 PUSCH: 3 | RAN5#87e |
| 6.4C.1 Frequency error for SUL | 5 | RAN5#86e |
| 6.4C.2.1 Error Vector Magnitude for SUL | PUSCH: 84 PUCCH: 24 PRACH: 12 | RAN5#86e |
| 6.4C.2.2 Carrier leakage for SUL | 3 | RAN5#90e |
| 6.4C.2.3 In-band emissions for SUL | 36 | RAN5#90e |
| 6.4C.2.4 EVM equalizer spectrum flatness for SUL | 90 | RAN5#90e |
| 6.4C.2.5 EVM equalizer spectrum flatness for Pi/2 BPSK for SUL | 9 | RAN5#90e |
| 6.5C.1 Occupied bandwidth for SUL | 18 | RAN5#86e |
| 6.5C.2.2 Spectrum Emission Mask for SUL | 72 (801) | RAN5#90e |
| 6.5C.2.3 Additional spectrum emission mask for SUL | 27 | RAN5#86e |
| 6.5C.2.4.1 NR ACLR for SUL | 460 (5001) | RAN5#90e |
| 6.5C.2.4.2 UTRA ACLR for SUL | Table 4.1.1.1-1 | RAN5#86e |
| 6.5C.3.1 General spurious emissions for SUL | 27 | RAN5#86e |
| 6.5C.3.2 Spurious emission for UE co-existence for SUL | 27 | RAN5#86e |
| 6.5C.3.3 Additional spurious emissions for SUL | 115 for NS_05 28 for NS_43 | RAN5#87e |
| 6.5C.4 Transmit intermodulation for SUL | 4 | RAN5#86e |
| NOTE 1: UEs supporting pi/2 BPSK DMRS and the corresponding IE [DMRSPi2BPSK] is set to 1. |  |  |

Table 4.1.1.2-2: Number of test points for SUL test cases in FR1 (NR UE Receiver test)

| Subclause | Number of test points | Comments |
| --- | --- | --- |
| 7.3C.2 Reference sensitivity power level for SUL | General test points:45 SUL configuration specific test points: SUL_n78-n80: 2 | RAN5#90e |
| 7.6C.2 Inband Blocking for SUL | 3 | RAN5#87e |
| 7.6C.3 Out-of-band blocking for SUL | 3 | RAN5#87e |

<End of Changes>

Add attached .zip files to TR 38.905: “38.521-1_TPanalysis_6.3C.3.2_SULTxSwitch_M”