3GPP TSG-RAN5 Meeting #96-e	R5-225111

# Electronic Meeting, 15th – 26th August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1441 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of reference sensitivity test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following issues exist in tables of clause 7.3B.2.3.4.2.1: - There are some combination in which NR DL Modulation is QPSK, but QPSK is not supported type for NR Downlink Modulation. - For DC_13A_n77A, test frequencies in test IDs 2 and 2a are incorrect since for ID 2 the DL frequency is not a harmonic of the UL frequency and for ID2a the frequency is outside the band. - The frequencies in the range column are ambiguous since in several cases an UL/DL label is missing. - Several NR test frequencies do not match to the frequency raster |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changes to tables of clause 7.3B.2.3.4.2.1: - DL modulation has been changed to CP-OFDM QPSK. - For DC_13A_n77A, test frequencies in test IDs 2 and 2a have been corrected. - The frequencies have been labeled as DL or UL. - Test frequencies have been corrected. A generic note has been added to clarify the NR frequency selection (also added to Table 7.3B.2.3_1.1.4.1-1). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Refsens cannot be tested for DC_13A_n77A and the specification will remain ambiguous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3B.2.3.4.2.1, 7.3B.2.3_1.1.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

7.3B.2.3.4.2.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths based on EN-DC operating bands specified in clause 5.5B, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration, and are shown in Table 7.3B.2.3.4.2.1-0 to Table 7.3B.2.3.4.2.1-6.

The details of the uplink and downlink reference measurement channels (RMCs) are specified in TS 36.521-1 [10] Annex A.2 and A.3 for E-UTRA RMC, and in TS 38.521-1 [8] Annex A.2 and A.3 for NR RMC respectively. The details of the OCNG patterns used are specified in TS 36.521-1 [10] Annex A.5 and in TS 38.521-1 [8] Annex A.5 for E-UTRA CG and NR CG, respectively. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG, respectively.

For configurations without any reference sensitivity exception, in a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. For configurations with reference sensitivity exception, in an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected.

< Unchanged tables omitted >

Table 7.3B.2.3.4.2.1-3aa: Void

Table 7.3B.2.3.4.2.1-3b: Default test conditions for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1 for EN-DC 26_n41

| NR Band n41 |  |  |  | E-UTRA Band 26 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | FC (DL) (MHz) NDL | CBW (MHz) | UL allocation (LCRB) |
| 1 | 10 | 2593.005 MHz/ 518601 | 50@0 | 864.3 MHz/ 8743 | 5, 10, 15, 20 | REFSENS (NOTE 2) |
| 2 |  | 2660 MHz/ 532000 | 50@0 | 886.6 MHz/ 8966 | 5, 10, 15, 20 | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil Note 4 in Table 7.3B.2.0.3.2-1. NOTE 2: REFSENS refers to Table 7.3.4.1-1 in TS 36.521-1 [10] which defines uplink RB configuration and start RB location for each channel BW. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-3c: Void

< Unchanged tables omitted >

Table 7.3B.2.3.4.2.1-6: Test Configuration Table for EN-DC configurations affected by Reference sensitivity exceptions (two bands)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |
| Test EN-DC channel bandwidth as specified in TS 36.508 [11] clause 4.3.1 and TS 38.508-1 [6] clause 4.3.1. |  |  |  |  |  |  |  |  | Refer to "NR NRB"and "E-UTRA NRB " columns |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  |  |  |  | Lowest supported SCS |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  | NS_01 by default Unless given by Table 7.3.3-3 in TS 36.521-1 [10] for the E-UTRA band and Table 7.3.2.3-4 in TS 38.521-1 [8] for the NR band. |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  |  |  |  | SCG -NR |  |  |  |  |
|  | Band |  | Range | NRB |  |  |  | Band |  | Range | NRB |  |
|  | UL MOD |  | DL MOD | CH BW |  |  | DLalloc / UL alloc | UL MOD |  | DL MOD | UL/DL Ch BW | DLalloc / UL alloc |
| Test Settings for a DC_1A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 12,11 | 1 |  | Mid |  |  |  |  | n3 |  | Mid |  |  |
|  | QPSK |  | QPSK | 20 MHz |  |  | All RBs / REFSENS_ENDC_3 | N/A |  | CP-OFDM QPSK | 40 MHz | All RBs / 0 |
| 24,10 | 1 |  | UL 1950 / DL 2140 |  |  |  |  | n3 |  | UL 1760 / DL 1855 |  |  |
|  | QPSK |  | QPSK | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |
| 36 | 1 |  | Low |  |  |  |  | n3 |  | High |  |  |
|  | QPSK |  | QPSK | 20 MHz |  |  | All RBs / REFSENS_ENDC_3 | N/A |  | CP-OFDM QPSK | 40 MHz | All RBs / 0 |
| Test Settings for a DC_1A_n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 1 |  | UL 1965 / DL 2155 |  |  |  |  | n8 |  | UL 887.5 / DL 932.5 |  |  |
|  | QPSK |  | QPSK | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_1A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 1 |  | UL 1950 / DL 2140 |  |  |  |  | n78 |  | UL/DL 3709.995 |  |  |
|  | QPSK |  | QPSK | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_2A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  | 2 | UL 1855 / DL 1935 |  |  |  |  | n66 |  | DL 2175 |  |  |  |
|  |  | QPSK | QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  |
| 24 |  | 2 | UL 1883.3 /DL1963.3 |  |  |  |  | n66 |  | DL 2150 |  |  |  |
|  |  | QPSK | QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_2A_n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 2 |  | DL 1980 MHZ |  |  |  |  | n71 |  | UL 665.5 MHz |  |  |  |
|  | QPSK |  | QPSK | 20 MHz |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_1 |  |
| 23 | 2 |  | DL1980 MHZ |  |  |  |  | n71 |  | UL 673 MHz |  |  |  |
|  | QPSK |  | QPSK | 20 MHz |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_1 |  |
| 35 | 2 |  | UL 1881 MHZ |  |  |  |  | n71 |  | Low |  |  |  |
|  | QPSK |  | QPSK | 20 MHz |  |  | All RBs / REFSENS_ENDC_2 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 20 MHz | All RBs / 0 |  |
| Test Settings for a DC_2A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 2 |  | UL 1860 |  |  |  |  | n77 |  | UL/DL 3720 |  |  |  |
|  | QPSK |  | QPSK | 20 |  |  | All RBs / REFSENS_ENDC_1 | N/A |  | CP-OFDM QPSK | 100 MHz | All RBs / 0 |  |
| 23 | 2 |  | UL 1860 |  |  |  |  | n77 |  | UL/DL 3700.005 |  |  |  |
|  | QPSK |  | QPSK | 20 |  |  | All RBs / REFSENS_ENDC_1 | N/A |  | CP-OFDM QPSK | 20 MHz | All RBs / 0 |  |
| 3 8 | 2 |  | UL 1860 |  |  |  |  | n77 |  | UL/DL 3400.005 |  |  |  |
|  | QPSK |  | QPSK | 20 |  |  | All RBs / REFSENS_ENDC_1 | N/A |  | CP-OFDM QPSK | 20 MHz | All RBs / 0 |  |
| 45 | 2 |  | DL Mid |  |  |  |  | n77 |  | UL/DL 3919.995 |  |  |  |
|  | N/A |  | QPSK | 20 MHz |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 100 MHz | All RBs / REFSENS_ENDC_2 |  |
| 4a 9 | 2 |  | DL 1950 |  |  |  |  | n77 |  | UL/DL 4000.005 |  |  |  |
|  | N/A |  | QPSK | 20 MHz |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 100 MHz | All RBs / REFSENS_ENDC_2 |  |
| 54 | 2 |  | UL 1855/DL 1935 |  |  |  |  | n77 |  | UL/DL 3790.005 |  |  |  |
|  | QPSK |  | QPSK | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| 64 | 2 |  | UL 1900/DL 1980 |  |  |  |  | n77 |  | UL/DL 3720 |  |  |  |
|  | QPSK |  | QPSK | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| 74 | 2 |  | UL 1885/DL 1965 |  |  |  |  | n77 |  | UL/DL 3810 |  |  |  |
|  | QPSK |  | QPSK | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_2A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 2 |  | UL 1870 |  |  |  |  | n78 |  | UL/DL 3739.995 |  |  |  |
|  | QPSK |  | QPSK | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 | N/A |  | CP-OFDM QPSK | 100 MHz | All RBs / 0 |  |
| 23 | 2 |  | UL 1885 |  |  |  |  | n78 |  | UL/DL 3739.995 |  |  |  |
|  | QPSK |  | QPSK | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 | N/A |  | CP-OFDM QPSK | 20 MHz | All RBs / 0 |  |
| 34 | 2 |  | UL 1855/ DL 1935 |  |  |  |  | n78 |  | UL/DL 3790.005 |  |  |  |
|  | QPSK |  | QPSK | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| 44 | 2 |  | UL 1885/ DL 1965 |  |  |  |  | n78 |  | UL/DL 3690 |  |  |  |
|  | QPSK |  | QPSK | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_3A_n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1,11 | 3 |  | Mid |  |  |  |  | n1 |  | Mid |  |  |
|  | N/A |  | QPSK | 20 MHz |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 50 MHz | All RBs / REFSENS_ENDC_3 |
| 26 | 3 |  | High |  |  |  |  | n1 |  | Low |  |  |
|  | N/A |  | QPSK | 20 MHz |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 50 MHz | All RBs / REFSENS_ENDC_3 |
| 34 | 3 |  | UL 1760 / DL 1855 |  |  |  |  | n1 |  | UL 1950 / DL 2140 |  |  |
|  | QPSK |  | QPSK | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_3A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 3 |  | UL 1771 / DL 1866 |  |  |  |  | n5 |  | UL 838 / DL 883 |  |  |
|  | QPSK |  | QPSK |  | 10 MHz |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |
| 24 | 3 |  | UL 1721 / DL 1816 |  |  |  |  | n5 |  | UL 838 / DL 883 |  |  |
|  | QPSK |  | QPSK |  | 10 MHz |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_3A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 3 |  | High |  |  |  |  | n41 |  | Low |  |  |
|  | QPSK |  | QPSK | 20 MHz |  |  | All RBs / REFSENS_ENDC_3 | N/A |  | CP-OFDM QPSK | 100 MHz | All RBs / 0 |
| 26 | 3 |  | High |  |  |  |  | n41 |  | Low |  |  |
|  | N/A |  | QPSK | 20 MHz |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 50 MHz | All RBs / REFSENS_ENDC_3 |
| 34, 10 | 3 |  | UL 1740 / DL 1835 |  |  |  |  | n41 |  | UL/DL 2657.5 |  |  |
|  | QPSK |  | QPSK | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_3A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 3 |  | Mid |  |  |  |  | n78 |  | UL/DL 3495 |  |  |
|  | QPSK |  | QPSK | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 | N/A |  | CP-OFDM QPSK | 100 MHz | All RBs / 0 |
| 23 | 3 |  | Mid |  |  |  |  | n78 |  | UL/DL 3525 |  |  |
|  | QPSK |  | QPSK | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 | N/A |  | CP-OFDM QPSK | 20 MHz | All RBs / 0 |
| 35 | 3 |  | Mid |  |  |  |  | n78 |  | UL/DL 3685.005 |  |  |
|  | N/A |  | QPSK | 20 MHz |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 20 MHz | All RBs / REFSENS_ENDC_2 |
| 3a9 | 3 |  | Low |  |  |  |  | n78 |  | High |  |  |
|  | QPSK |  | QPSK | 20 |  |  | All RBs/0 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 20 | All RBs / REFSENS_ENDC_2 |
| 44 | 3 |  | UL 1740 / DL 1835 |  |  |  |  | n78 |  | UL/DL 3574.995 |  |  |
|  | QPSK |  | QPSK | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |
| 54 | 3 |  | UL 1765 / DL 1860 |  |  |  |  | n78 |  | UL/DL 3435 |  |  |
|  | QPSK |  | QPSK | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |

| Test Settings for a DC_5A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 5 |  |  | UL 838/ DL 883 |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  | UL 1721/ DL 2121 |  |  |  |  |  |  |  |
|  | QPSK |  |  | N/A |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | N/A |  |  |  | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_5A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 5 |  |  | UL 829 |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  | UL/DL 3316.005 |  |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  |  | 10 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |  |  |
| 23 | 5 |  |  | UL 829 |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  | UL/DL 4144.995 |  |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  |  | 10 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |  |  |
| 38 | 5 |  |  | UL 840 |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  | UL/DL 3600 |  |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  |  | 10 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |  |  |
| 44, 13 | 5 |  |  | UL 844/DL 889 |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  | UL/DL 3421.005 |  |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| 54, 13 | 5 |  |  | UL 826.5/DL 871.5 |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  | UL/DL 4177.5 |  |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_7A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 7 |  |  | DL 2655 |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  | DL 1825 |  |  |  |  |  |  |
|  | QPSK |  |  | N/A |  |  |  |  | 10 MHz |  |  |  | N/A |  |  | N/A |  |  |  | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_7A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 7 |  |  | DL 2667 |  |  |  |  |  |  |  |  |  |  |  | n5 |  |  |  | DL 879 |  |  |  |  |  |  |
|  | QPSK |  |  | N/A |  |  |  |  | 10 MHz |  |  |  | N/A |  |  | N/A |  |  |  | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_7A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 7 |  |  | High |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  | Low |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  |  | 20 MHz |  |  |  | All RBs / REFSENS_ENDC_3 |  |  | N/A |  |  |  | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |  |
| Test Settings for a DC_8A_n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 8 |  |  | DL 932.5 |  |  |  |  |  |  |  |  |  |  |  | n1 |  |  |  | DL 2155 |  |  |  |  |  |  |
|  | QPSK |  |  | N/A |  |  |  |  | 5 MHz |  |  |  | N/A |  |  | N/A |  |  |  | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_8A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 8 |  |  | DL 945 |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  | DL 1850 |  |  |  |  |  |  |
|  | QPSK |  |  | N/A |  |  |  |  | 5 MHz |  |  |  | N/A |  |  | N/A |  |  |  | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 24 | 8 |  |  | DL 942.5 |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  | DL 1842.5 |  |  |  |  |  |  |
|  | QPSK |  |  | N/A |  |  |  |  | 5 MHz |  |  |  | N/A |  |  | N/A |  |  |  | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_8A_n20A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 8 |  |  | UL 890.5 / DL 935.5 |  |  |  |  |  |  |  |  |  |  |  | n20 |  |  |  | UL 849.5 / DL 808.5 |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 24 | 8 |  |  | UL 892.5 / DL 937.5 |  |  |  |  |  |  |  |  |  |  |  | n20 |  |  |  | UL 847.5 / DL 806.5 |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_8A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 8 |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  | UL/DL 3589.995 |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  |  | 10 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |  |
| 2,8 | 8 |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  | UL/DL 3520.005 |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  |  | 10 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |  |
| 34 | 8 |  |  | UL 897.5 |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  | UL/DL 3634.995 |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_11A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 11 |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  | UL/DL 4457.705 |  |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 2,9 | 11 |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  | UL/DL 4512.705 |  |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| Test Settings for a DC_13A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 13 |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  | UL/DL 3910.005 |  |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  |  | 10 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |  |  |
| 1a8 | 13 |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  | UL/DL 3600 |  |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  |  | 10 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |  |  |
| 25 | 13 |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  | UL/DL 3910.005 |  |  |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |
| 2a9 | 13 |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  | UL/DL 3834 |  |  |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |
| 34 | 13 |  |  | UL 784.5/DL 753.5 |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  | UL/DL 3891.495 |  |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_19A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 19 |  |  | DL 884 |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  | Low |  |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  |  | 40 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 2,9 | 19 |  |  | DL 884 |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  | UL/DL 4445.025 |  |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  |  | 40 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| Test Settings for a DC_20A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 20 |  |  | UL 840 / DL 799 |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  | UL 1775 / DL 1870 |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 24 | 20 |  |  | UL 847 / DL 806 |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  | UL 1735 / DL 1830 |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_20A_n7A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 20 |  |  | UL 851 / DL 810 |  |  |  |  |  |  |  |  |  |  |  | n7 |  |  |  | UL 2512 / DL 2632 |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_20A_n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 20 |  |  | UL 849.5 / DL 808.5 |  |  |  |  |  |  |  |  |  |  |  | n8 |  |  |  | UL 892.5 / DL 937.5 |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_20A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1,8 | 20 |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  | Mid |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  |  | 20 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |  |
| 23 | 20 |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  | UL/DL 3388.005 |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  |  | 20 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |  |
| 34 | 20 |  |  | UL 850 / DL 809 |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  | UL/DL 3358.995 |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_26A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 26 |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  | UL/DL 2524.5 |  |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 15 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  | CP-OFDM QPSK |  |  | 50 MHz |  |  |  | All RBs / 0 |
| 2,8 | 26 |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  | UL/DL 2572 |  |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 15 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  | QPSK |  |  | 50 MHz |  |  |  | All RBs / 0 |
| 34 | 26 |  |  | UL 839 |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  | UL/DL 2562 |  |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_26A_n77A/DC_26A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 26 |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  | UL/DL 3366 |  |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 15 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  | CP-OFDM QPSK |  |  | 100 MHz |  |  |  | All RBs / 0 |
| 2,8 | 26 |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  | UL/DL 3445.995 |  |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 15 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  | CP-OFDM QPSK |  |  | 100 MHz |  |  |  | All RBs / 0 |
| 34 | 26 |  |  | UL 836.5 |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  | UL/DL 3391.005 |  |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_26A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 26 |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  | UL/DL 4432.5 |  |  |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  |  | 15 MHz |  |  |  | All RBs / 0 |  |  |  |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 60 MHz |  |  |  | All RBs / REFSENS_ENDC_2 |
| 22,9 | 26 |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  | UL/DL 4470.495 |  |  |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  |  | 15 MHz |  |  |  | All RBs / 0 |  |  |  |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 60 MHz |  |  |  | All RBs / REFSENS_ENDC_2 |
| Test Settings for a DC_28A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 28 |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  | n5 |  | Low |  |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 20MHz |  |  |  | All RBs / REFSENS_ENDC_3 |  |  |  |  | N/A |  | CP-OFDM QPSK |  |  | 20 MHz |  |  |  | All RBs / 0 |
| Test Settings for a DC_28A_n77A/DC_28A_n78A Configuration (Test ID 4 only apply to DC_28A_n77A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  | 28 |  | Low |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | UL/DL 3540 |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  | QPSK |  |  | 10 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  | CP-OFDM QPSK |  |  | 100 MHz |  |  |  | All RBs / 0 |  |  |
| 28 |  | 28 |  | Low |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | Low |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  | QPSK |  |  | 10 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  | CP-OFDM QPSK |  |  | 100 MHz |  |  |  | All RBs / 0 |  |  |
| 34 |  | 28 |  | UL 705.5 / DL 760.5 |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | UL/DL 3582.495 |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  | QPSK |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | CP-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |
| 45 |  | 28 |  | Low |  |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3814.995 |  |  |  |  |  |  |  |  |  |
|  |  | N/A |  | QPSK |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  |  | CP-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 100 MHz |  |  |  | All RBs / REFSENS_ENDC_2 |  |  |
| Test Settings for a DC_40A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 40 |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  | UL/DL 3525 |  |  |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  |  | 20 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK |  |  | 20 MHz |  |  |  | All RBs / REFSENS_ENDC_2 |
| 2,9 | 40 |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  | Mid |  |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 20 MHz |  |  |  | All RBs / REFSENS_ENDC_2 |  |  |  |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK |  |  | 100 MHz |  |  |  | All RBs / REFSENS_ENDC_2 |
| Test Settings for a DC_41A_n77A/DC_41A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 41 |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  | UL/DL 3750 |  |  |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  |  | 20 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK |  |  | 100 MHz |  |  |  | All RBs / REFSENS_ENDC_2 |
| 26 | 41 |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  | Low |  |  |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  |  | 20 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK |  |  | 100 MHz |  |  |  | All RBs / REFSENS_ENDC_3 |
| 36 | 41 |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  | Low |  |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 20MHz |  |  |  | All RBs / REFSENS_ENDC_3 |  |  |  |  | N/A |  | CP-OFDM QPSK |  |  | 100 MHz |  |  |  | All RBs / 0 |
| Test Settings for a DC_48A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  | 48 |  |  | UL/DL 3557.6 |  |  |  |  |  |  |  | n66 |  |  |  |  | UL 1778.8 |  |  |  |  |  |  |  |  |
|  |  |  | N/A |  |  | QPSK |  |  |  | 20 MHz |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  | 40 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |
| 23 |  |  | 48 |  |  | UL/DL 3581.0 |  |  |  |  |  |  |  | n66 |  |  |  |  | UL 1777.5 |  |  |  |  |  |  |  |  |
|  |  |  | N/A |  |  | QPSK |  |  |  | 20 MHz |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  | 40 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |
| 34 |  |  | 48 |  |  | UL/DL 3630 |  |  |  |  |  |  |  | n66 |  |  |  |  | UL 1715 / DL 2115 |  |  |  |  |  |  |  |  |
|  |  |  | QPSK |  |  | QPSK |  |  |  | 20 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_66A_n2A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14, 10 |  |  | 66 |  |  | UL 1775/ DL 2175 |  |  |  |  |  |  |  | n2 |  |  |  |  | UL 1855 / DL 1935 |  |  |  |  |  |  |  |  |
|  |  |  | QPSK |  |  | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |
| 24, 10 |  |  | 66 |  |  | UL 1750/ DL 2150 |  |  |  |  |  |  |  | n2 |  |  |  |  | UL 1883.3 / DL 1963.3 |  |  |  |  |  |  |  |  |
|  |  |  | QPSK |  |  | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_66A_n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  | 66 |  | UL 1750/ DL 2150 |  |  |  |  |  |  |  |  | n71 |  |  |  |  | UL 675/ DL 629 |  |  |  |  |  |  |  |  |
|  |  |  | QPSK |  | QPSK |  |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |

| Test Settings for a DC_66A_n77A Configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13 | 66 | UL 1720 |  |  | n77 | UL/DL 3439.995 |  |  |  |
|  | QPSK | QPSK | 20 | All RBs / REFSENS_ENDC_1 | N/A | CP-OFDM QPSK | 100 MHz | All RBs / 0 |  |
| 23 | 66 | UL 1720 |  |  | n77 | UL/DL 3420 |  |  |  |
|  | QPSK | QPSK | 20 | All RBs / REFSENS_ENDC_1 | N/A | CP-OFDM QPSK | 20 MHz | All RBs / 0 |  |
| 38 | 66 | UL 1720 |  |  | n77 | UL/DL 3600 |  |  |  |
|  | QPSK | QPSK | 20 | All RBs / REFSENS_ENDC_1 | N/A | CP-OFDM QPSK | 100 MHz | All RBs / 0 |  |
| 44 | 66 | UL 1775/ DL 2175 |  |  | n77 | UL/DL 3949.995 |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| 54 | 66 | UL 1760 / DL 2160 |  |  | n77 | UL/DL 3720 |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_66A_n78A Configuration |  |  |  |  |  |  |  |  |  |
| 13 | 66 | UL 1755 |  |  | n78 | UL/DL 3510 |  |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_1 | N/A | CP-OFDM QPSK | 100 MHz | All RBs / 0 |  |
| 23 | 66 | UL 1755 |  |  | n78 | UL/DL 3480 |  |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_1 | N/A | CP-OFDM QPSK | 20 MHz | All RBs / 0 |  |
| 34 | 66 | UL 1730/ DL 2130 |  |  | n78 | UL/DL 3660 |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| NOTE 1: REFSENS_LTE refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.3-2 of TS 36.521-1 [10]. REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. REFSENS_ENDC_1 refers to the Uplink RB allocation for reference sensitivity exceptions due to UL harmonic interference according to table 7.3B.2.0.3.1-2. REFSENS_ENDC_2 refers to the Uplink RB allocation for reference sensitivity exceptions due to receiver harmonic mixing according to table 7.3B.2.0.3.2-2. REFSENS_ENDC_3 refers to the Uplink RB allocation for reference sensitivity exceptions due to cross band isolation according to table 7.3B.2.0.3.4-2. REFSENS_ENDC_4 refers to the Uplink RB allocation for reference sensitivity exceptions due to dual uplink operation for ENDC according to table 7.3B.2.0.3.5.1-1 for PC3 and table 7.3B.2.0.3.5.1-1a for PC2. NOTE 2: Void NOTE 3: Test ID with UL harmonic exception NOTE 4: Test ID with 2UL intermodulation exception NOTE 5: Test ID with UL receiver harmonic mixing NOTE 6: Test ID with UL cross band isolation NOTE 7: Void NOTE 8: Test ID with UL harmonic exception avoided NOTE 9: Test ID with UL receiving harmonic mixing exception avoided NOTE 10: Only applicable to UEs not supporting UE capability singleUL-Transmission. NOTE 11: Test ID with Cross band isolation exception avoided, which is only applicable to DC_1_n3, DC_3_n1 and DC_3_n84. NOTE 12: In an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. NOTE 13: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 14: If the NR frequency does not match to a valid NR-ARFCN, apply the closest NR frequency with a valid NR-ARFCN. |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

7.3B.2.3_1.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.3B.2.0

Exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

7.3B.2.3_1.1.4	Test description

7.3B.2.3_1.1.4.1	Initial conditions

Same initial conditions as in clause 7.3B.2.3.4.1 with following exceptions:

The initial test configurations for E-UTRA band and NR band consist of environmental conditions, test frequencies, and channel bandwidths and RB allocations for exceptional test scenarios are specified in Table 7.3B.2.3_1.1.4.1-0 and 7.3B.2.3_1.1.4.1-1.

Table 7.3B.2.3_1.1.4.1-0: Test Configuration Table for all EN-DC configurations for FR1 non-exception requirements

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  |  | DC_XA_nYA-nZA Mid range for MCG and SCG DC_XA_nYC: Mid range for XA Low range, High Range for nXC DC_(n)XCA: Low range, High Range DC_XA_nY(2A): TBD |  |  |  |  |  |  |  |
| NR Test Channel Bandwidths as specified in TS 38.508-1 [6] clause 4.3.1 E-UTRA Test Channel Bandwidths as specified in TS 36.508 [11] clause 4.3.1. |  |  |  |  |  |  | Refer to "NR NRB"and "E-UTRA NRB" columns |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |  |  |  |
| Test SCS for the NR cell as specified in TS 38.521-1 [8] Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest SCS per Channel Bandwidth |  |  |  |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  | SCC1 – EUTRA/NR |  |  |  |  |  | SCG -NR |  |  |  |
|  | Band | Range | NRB |  | Band | Range |  |  | NRB |  | Band | Range | NRB |  |
|  | UL MOD | DL MOD | CH BW | DLalloc/UL alloc | UL MOD | DL MOD |  |  | UL/DL Ch BW | DLalloc/ ULalloc | UL MOD | DL MOD | UL/DL Ch BW | DLalloc/UL alloc |
| Test Settings for a DC_(n)XCA Configuration (Intra-band contiguous EN-DC with LTE CA)– Note 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | default |  |  | X | default |  |  |  |  | nX | default |  |  |
|  | QPSK | QPSK | Highest | All RBs / REFSENS_LTE | N/A | QPSK |  |  | Highest | All RBs / 0 | DFT-s-OFDM QPSK | CP-OFDM QPSK | Highest | All RBs / REFSENS_NR |
| Test Settings for a DC_XC_nXA Configuration (Intra-band non-contiguous EN-DC with LTE CA)– Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.2 |  |  |  |  |  |
| Test Settings for a DC_XA-XA_nXA/ DC_XA_(n)XAA Configuration (Intra-band non-contiguous EN-DC with LTE CA)– Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.2 |  |  |  |  |  |
| Test Settings for a DC_XA_nYC Configuration (Inter-band EN-DC with NR CA, 2 bands) – Note 1,5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | default |  |  | nY | default |  |  |  |  | nY | default |  |  |
|  | N/A | N/A | 5 MHz | 0/0 | N/A | CP-OFDM QPSK |  |  | Highest | All RBs / 0 | DFT-s-OFDM QPSK | CP-OFDM QPSK | Highest | All RBs / REFSENS_NR |
| Test Settings for a DC_XA_nY(2A) Configuration (Inter-band EN-DC with NR CA, 2 bands) – Note 1,5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | default |  |  | nY | default |  |  |  |  | nY | default |  |  |
|  | N/A | N/A | 5 MHz | 0/0 | N/A | CP-OFDM QPSK |  |  | Highest | All RBs / 0 | DFT-s-OFDM QPSK | CP-OFDM QPSK | Highest | All RBs / REFSENS_NR |
| Test Settings for a DC_XC_nYA, DC_XA-XA_nYA Configuration (Inter-band EN-DC with LTE CA, 2 bands) – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.3 |  |  |  |  |  |
| Default Test Settings for a DC_XA-YA_nZA Configuration (Inter-band EN-DC with LTE CA, 3 bands) – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.3 |  |  |  |  |  |
| Default Test Settings for a DC_XA_nYA-nZA Configuration (Inter-band EN-DC with NR CA, 3 bands) – Note 1,5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid |  |  | nY | default |  |  |  |  | nZ | default |  |  |
|  | N/A | N/A | 5 MHz | 0/0 | N/A | CP-OFDM QPSK |  |  | Highest | All RBs / 0 | DFT-s-OFDM QPSK | CP-OFDM QPSK | Highest | All RBs / REFSENS_NR |
| 2 | X | Mid |  |  | nY | default |  |  |  |  | nZ | default |  |  |
|  | N/A | N/A | 5 MHz | 0/0 | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | Highest | All RBs / REFSENS_NR | N/A | CP-OFDM QPSK | Highest | All RBs / 0 |
| Note 1: LTE anchor agnostic configuration Note 2: Not LTE anchor agnostic configuration due to exception requirement for intra-band contiguous CA in clause 7.4B, 7.5B, 7.6B.2, 7.6B.3, 7.6B.4, 7.7B, 7.8B, 7.9B test cases Note 3: LTE anchor agnostic configuration with LTE CA where LTE CA fallback to 1CC is sufficient to test Note 4: In a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. Note 5: For UE supporting multiple EN-DC configurations with the same NR CA combination, only one EN-DC configuration is tested. Note 6: REFSENS_LTE refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.3-2 of TS 36.521-1 [10]. REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3_1.1.4.1-1: Test Configuration Table for EN-DC configurations affected by Reference sensitivity exceptions

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | EN-DC configurations containing the following band combinations: 20-n78: Mid in band 20 and Mid in band 78. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR Test Channel Bandwidths as specified in TS 38.508-1 [6] clause 4.3.1 E-UTRA Test Channel Bandwidths as specified in TS 36.508 [11] clause 4.3.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Refer to "NR NRB" and "E-UTRA NRB" columns |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test SCS for the NR cell as specified in TS 38.521-1 [8] Table 5.3.5-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Lowest SCS per Channel Bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SCC1 – EUTRA/NR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CG -NR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  |  |  | Range | NRB |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Band |  |  | Range |  |  |  |  |  |  |  | NRB |  |  |  |  |  |  |  |  |  |  |  |  | Band |  |  |  |  | Range |  |  |  |  |  |  |  |  |  | NRB |  |  |  |  |  |  |  |
|  | UL MOD |  |  |  | DL MOD | CH BW |  |  |  |  |  | DLalloc/UL alloc |  |  |  |  |  |  |  |  | UL MOD |  |  | DL MOD |  |  |  |  |  |  |  | UL/DL Ch BW |  |  |  |  |  |  |  |  | DLalloc/ ULalloc |  |  |  | UL MOD |  |  |  |  | DL MOD |  |  |  |  |  |  |  |  |  | UL/DL Ch BW |  |  | DLalloc/UL alloc |  |  |  |  |
| Default Test Settings for a DC_XA_nYC, DC_XA_nY(2A) Configuration (Inter-band EN-DC with NR CA, 2 bands) – Note 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | No test required, NR 1CC fallback is tested in 7.3B.2.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XC_nYA, DC_XA-XA_nYA Configuration (Inter-band EN-DC with LTE CA, 2 bands) – Note 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XA-YA_nZA Configuration (Inter-band EN-DC with LTE CA, 3 bands) – Note 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XA_nYA-nZA Configuration (Inter-band EN-DC with NR CA, 3 bands) – Note 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | No test required, NR 1CC fallback is tested in 7.3B.2.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for DC_1A-3A_n28A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  |  |  | UL 1975 / DL 2165 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 |  |  | DL 1818.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  | UL 710.5 / DL 765.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  | QPSK | 5 MHz |  |  |  |  |  | All RBs / All RBs |  |  |  |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / All RBs |  |  |  |  |  |  |
| 2 | 3 |  |  |  | UL 1780 / DL 1875 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  | DL 2139 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  | UL 710.5 / DL 765.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  | QPSK | 5 MHz |  |  |  |  |  | All RBs / All RBs |  |  |  |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s- OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / All RBs |  |  |  |  |  |  |
| Test Settings for DC_1A-7A_n28A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  |  |  | UL 1935 / DL 2125 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  | DL 2653 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  | UL 718 / DL 773 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  | QPSK | 5 MHz |  |  |  |  |  | All RBs / All RBs |  |  |  |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  |  |  | 5 MHz |  |  | All RBs / All RBs |  |  |  |  |
| Test Settings for DC_1A-3A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  | UL 1950 / DL 2140 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 |  |  |  |  |  |  |  | DL 1807.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3757.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 1 |  | DL 2125 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 |  |  |  |  |  |  |  | UL 1775 / DL 1870 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3725 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_1A-5A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  | DL 2122 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 5 |  |  |  |  |  |  |  | UL 829 / DL 874 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3780 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 1 |  | UL 1975 / DL 2165 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 5 |  |  |  |  |  |  |  | DL 885 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3405 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_1A-7A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  | UL 1977.5 / DL 2167.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  | DL 2627.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3305 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 1 |  | DL 2140 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  | UL 2510 / DL 2630 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3580 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 10MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_1A-20A_n8A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  | UL 1925/ DL 2115 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  |  | DL 805 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n8 |  |  |  |  | UL 910 / DL 955 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_1A-20A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  | DL 2120 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  |  | UL 835 / DL 794 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3790 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 1 |  | UL 1950 / DL 2140 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  |  | DL 810 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3330 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_1A-28A_n3A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  | DL 2139 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 28 |  |  |  |  |  |  |  | UL 710.5 / DL 765.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  | UL 1780 / DL 1875 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_1A_n28A-n78A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  | UL 1950 / DL 2140 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  |  |  | 3416 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  |  |  |  |  | UL 733 / DL 788 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | N/A |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / All RBs |  |  |
| 2 | 1 |  | UL 1950 / DL 2140 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  |  |  |  |  | DL 790 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  |  |  | 3320 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | N/A |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / All RBs |  |  |
| Test Settings for DC_1A-28A_n78A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 28 |  | UL 740 / DL 795 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  | DL 2150 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3630 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 1 |  | UL 1970 / DL 2160 |  |  |  |  |  |  |  |  |  |  |  |  |  | 28 |  |  |  |  |  |  |  | DL 794 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3352 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_3A-7A_n5A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  | UL 1780/ DL 1875 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  | DL 2625 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n5 |  |  |  |  | UL 845 / DL 890 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_3A-7A_n28A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  |  |  | UL 1712.5 / DL 1807.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  | DL 2682 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  |  | UL 743 / DL 798 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  | QPSK |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  |  |  |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |
| 2 | 7 |  |  |  | UL 2543 / DL 2663 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 |  |  | DL 1832.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  |  | UL 710.5 / DL 765.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  | QPSK |  |  |  |  | 10 MHz |  |  |  | All RBs / All RBs |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  |  |  |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |
| Test Settings for DC_3A-7A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  | DL 1820 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  | UL 2565 / DL 2685 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3310 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 3 |  | DL 1820 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  | UL 2565 / DL 2685 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3475 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_3A-8A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  | DL 1820 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 8 |  |  |  |  |  |  |  | UL 910 / DL 955 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3640 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_3A-20A_n28A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  | DL 1828 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  |  | UL 852 / DL 811 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  | UL 728 / DL 783 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_3A-20A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  | DL 1820 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  |  | UL 845 / DL 804 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3510 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_3A-28A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 28 |  | UL 740 / DL 760 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 |  |  |  |  |  |  |  | UL 1775 / DL 1870 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3350 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_3A_n28A-n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  | UL 1750/ DL 1845 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  |  | 3764 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  | UL 743 / DL 798 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_3A-40A_n1A Configuration – Note 3, 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  | UL 1735 / DL 1830 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 |  |  |  |  |  |  |  | DL 2380 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n1A |  |  |  |  | UL 1950 / DL 2140 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_5A-7A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 5 |  | UL 844 / DL 889 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  | DL 2645 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3489 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 5 |  | DL 879 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  | UL 2550 / DL 2670 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3429 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 3 | 5 |  | DL 875 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  | UL 2525 / DL 2645 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3350 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_7A-8A_n3A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 |  | UL 2530 / DL 2650 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 8 |  |  |  |  |  |  |  | DL 940 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  | UL 1735 / DL 1830 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 7 |  | DL 2670 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 8 |  |  |  |  |  |  |  | UL 890 / DL 935 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  | UL 1780 / DL 1875 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_7A-20A_n1A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 |  | UL 2510 / DL 2630 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  |  | 800 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n1 |  |  |  |  | UL 1940 / DL 2130 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 10 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_7A-20A_n3A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 |  | UL 2543 / DL 2663 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  |  | DL 806 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  | UL 1737 / DL 1832 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 7 |  | DL 2630 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  |  | UL 855 / DL 896 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  | UL 1775 / DL 1870 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_7A-20A_n8A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 |  | UL 2565 / DL 2685 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  |  | DL 795 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n8 |  |  |  |  | UL 885 / DL 930 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 7 |  | DL 2640 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  |  | UL 840 / DL 799 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n8 |  |  |  |  | UL 900 / DL 945 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 3 | 7 |  | DL 2624 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  |  | UL 857 / DL 816 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n8 |  |  |  |  | UL 910 / DL 955 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_7A-20A_n28A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 20 |  | UL 842/ DL 801 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  | DL 2640 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  | UL 728 / DL 783 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  | N/A |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | All RBs / All RBs |  |
| Test Settings for DC_7A-20A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 |  | UL 2560 / DL 2680 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  |  | DL 810 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3370 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 7 |  | UL 2560 / DL 2680 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  |  | DL 810 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3435 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 3 | 7 |  | DL 2675 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  |  | UL 845 / DL 804 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3520 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_7A-28A_n3A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 |  | UL 2543 / DL 2663 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 28 |  |  |  |  |  |  |  | DL 796 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  | 1842 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 7 |  | DL 2685 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 28 |  |  |  |  |  |  |  | UL 745 / DL 800 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  | 1810 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_7A-28A_n5A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 |  | UL 2540 / DL 2660 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 28 |  |  |  |  |  |  |  | DL 776 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n5 |  |  |  |  | UL 829 / DL 874 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 7 |  | DL 2630 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 28 |  |  |  |  |  |  |  | UL 730 / DL 785 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n5 |  |  |  |  | UL 840 / DL 874 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_7A-28A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 |  | UL 2567.5 / DL 2687.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 28 |  |  |  |  |  |  |  | DL 782.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3350 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 7 |  | UL 2567.5 / DL 2687.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 28 |  |  |  |  |  |  |  | DL 782.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3460 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 3 | 7 |  | DL 2650 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 28 |  |  |  |  |  |  |  | UL 740 / DL 795 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3390 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_7A_n28A-n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 |  | UL 2565/ DL 2685 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  |  | 3310 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  | UL 745 / DL 800 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 7 |  | UL 2565/ DL 2685 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  |  |  |  | DL 800 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3365 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_2A-14A_n66A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 |  | DL 1954 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 14 |  |  |  |  |  |  |  | UL 793 / DL 763 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  | UL 1770 / DL 2170 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_14A-66A_n2A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 14 |  | DL 763 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 66 |  |  |  |  |  |  |  | UL 1762 / DL 2162 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n2 |  |  |  |  | UL 1874 / DL 1954 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_20A_n28A-n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 20 |  | UL 857 / DL 816 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  |  | 3314 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  | UL 743 / DL 798 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  |  | 10MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 20 |  | UL 837 / DL 796 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  |  |  |  | UL 744 / DL 799 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3310 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  |  | 5MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_20A_n28A-n75A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 20 |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n75 |  |  |  |  | DL 1476MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 20MHz |  |  |  |  |  |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  |  | 20MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 20MHz |  |  |  |  |  |  |  | All RBs / N/A |  |  |  |
| 2Note 8 | 20 |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n75 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 20MHz |  |  |  |  |  |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  |  | 20MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 20MHz |  |  |  |  |  |  |  | All RBs / N/A |  |  |  |
| Test Settings for DC_28A_n7A-n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 28 |  | UL 745 / DL 8005 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  |  | 3310 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n7 |  |  |  |  | UL 2565 / DL 2685 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 28 |  | UL 745 / DL 8005 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | n7 |  |  |  |  |  |  |  | DL 2650 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3390 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Note 1: Void Note 2: Void Note 3: EN-DC configuration affected by 2UL intermodulation exception. The exceptions always apply for a certain UL configuration. Note 4: LTE CA fallback to 1CC is sufficient to test, unless both LTE cells are part of the exception requirement in which case the configuration need to be tested (using configuration specific test settings and not default). Note 5: In an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. Note 6: Test only applicable for to UEs not supporting UE capability singleUL-Transmissionn for the correspondent uplink configuration. Note 7: NR CA fallback to 1CC is sufficient to test, unless both NR cells are part of the exception requirement in which case the configuration need to be tested (using configuration specific test settings and not default). Note 8: Test ID with UL harmonic exception avoided. Note 9: REFSENS_LTE refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.3-2 of TS 36.521-1 [10]. REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. REFSENS_ENDC_1 refers to the Uplink RB allocation for reference sensitivity exceptions due to UL harmonic interference according to table 7.3B.2.0.3.1-2. REFSENS_ENDC_2 refers to the Uplink RB allocation for reference sensitivity exceptions due to receiver harmonic mixing according to table 7.3B.2.0.3.2-2. REFSENS_ENDC_3 refers to the Uplink RB allocation for reference sensitivity exceptions due to cross band isolation according to table 7.3B.2.0.3.4-2. REFSENS_ENDC_4 refers to the Uplink RB allocation for reference sensitivity exceptions due to dual uplink operation for ENDC according to table 7.3B.2.0.3.5.1-1 for PC3 and table 7.3B.2.0.3.5.1-1a for PC2. NOTE 10: If the NR frequency does not match to a valid NR-ARFCN, apply the closest NR frequency with a valid NR-ARFCN. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

< End of changes >