3GPP TSG RAN Meeting #96-e	 	R5-225123

Electronic Meeting, 15th – 26th August 2022

Source:		Rohde & Schwarz

Title:	On the MU for FR2 PC1 TRx test cases

Agenda Item:		5.4.17

Document for:	Discussion and Endorsement

# Introduction

At previous meetings, estimation for the SNR for PC1 MOP TCs have been discussed [1, 2, 3]. In this discussion paper, we present the results of our analysis of for influence of noise and relaxation based on the common assumptions.

# Discussion

In this paper we focus our analysis on the requirements for MOP and Refsens in band n257.

The requirement for MOP from TS 38.101-2 [4] is as follows:

Table 6.2.1.1-2: UE maximum output power limits for power class 1

| Operating band | Max TRP (dBm) | Max EIRP (dBm) |
| --- | --- | --- |
| n257 | 35 | 55 |
| n258 | 35 | 55 |
| n260 | 35 | 55 |
| n261 | 35 | 55 |
| n262 | 35 | 55 |

We have analyzed the SNR for MOP for an UE carrier rms level of 55 dBm (as shown above) equivalent to 68 dBm PEP.

Given that this maximum power is higher than the 43 dBm max from Power Class 3, no relaxation of the requirement is needed, since this higher power can be compensated for by utilizing higher input attenuation in the measurement equipment. Thus no relaxation is needed on the requirement.

Proposal 1: No relaxation is needed for the MOP TC in PC1.

For the Refsens TC, the requirements from TS 38.101-2 [4] are shown in the following table:

Table 7.3.2.1-1: Reference sensitivity for power class 1

| Operating band | REFSENS (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -97.5 | -94.5 | -91.5 | -88.5 |
| n258 | -97.5 | -94.5 | -91.5 | -88.5 |
| n260 | -94.5 | -91.5 | -88.5 | -85.5 |
| n261 | -97.5 | -94.5 | -91.5 | -88.5 |
| n262 | -92.5 | -89.5 | -86.5 | -83.5 |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 |  |  |  |  |

Compared to the PC3 Refsens values, the PC1 values are 9.2 dB lower. These values can be achieved by either lowering the output power from the system simulator or by increasing the attenuation in the test system. This however has no impact on the testability of the requirement and no relaxation is required.

Proposal 2: No relaxation is needed for the Refsens TC in PC1.

# Assumptions

| ID | Description | Assumption |
| --- | --- | --- |
| #1 | Frequency ranges under consideration | Band n257 |
| #2 | Size of QZ for IFF | 30 cm |
| #3 | UE power class | PC1 |
| #4 | Max EIRP | EIRP MAX= +55 dBm (rms) |
| #5 | Wanted signal PAPR | 13 dB |
| #6 | Temperature range of the test equipment | 20°C – 35°C |
| #7 | Channel bandwidth | 400 MHz |

# Conclusion

In summary no relaxation is needed for the MOP and Refsens TCs in band n257 for PC1.

Proposal 1: No relaxation is needed for the MOP TC in PC1.

Proposal 2: No relaxation is needed for the Refsens TC in PC1.

# References

- R5-211275, Measurement uncertainties for PC1 devices, Keysight Technologies, GPP TSG RAN WG5 Meeting #90-e, February 2021

- R5-213314, On the SNR of FR2 TRx test cases for PC1, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #91-e, May 2021

- R5-215481, On the SNR of FR2 TRx test cases for PC1, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #92-e, August 2021

- TS 38.101-2, v17.6.0, 3GPP TSG RAN WG4, June 2022